MOUSER ver 1.0   This is a set of mouse interface routines.

These routines only read the mouse for input and return that information to

the calling procedure.  These routines DO NOT actually manipulate the screen

or screen cursor.  These are basic routines so that the user can manipulate

the mouse information however he or she like.  The MOUSER unit provides

these variable types, variables, and procedures:

MOUSEMOVE    : record with X and Y motion counters.

mousemove = record

xmouse : integer;

ymouse : integer;

end; {record mousemove}

MOUSEPRESENT : boolean variable set by mouseready function. Tells whether

mouse is present.

LEFTBUTTON   : boolean variable. True if left button has been pressed.

Important for calling program to reset to false after

using.

RIGHTBUTTON  : boolean variable. True if right button has been pressed.

Same usage note as LEFTBUTTON.

CENTERBUTTON : boolean variable. True if center button has been pressed.

Same usage note as LEFTBUTTON.  Not sure of response on

a two button mouse.  Can be deleted for two button mouse

if needed.

LASTMOUSE    : Last position of mouse. Set by QUESTIONMOUSE function.

MOUSEREADY   : Function testing whether mouse is present and ready.

MOUSEBUTTONPRESSED : Function testing if a button has been pressed on

the mouse.  It will set the appropriate button

variables if the a button has been pressed.

QUESTIONMOUSE : Function testing if mouse has input.  Works like

KEYPRESSED function in TP.

MOUSEMOVED : Procedure to read mouse motion counters. It will return

any movement since the last time it was called.

These procedures have been tested on LOGITECH mice (three button mouse) and

MICROSOFT Two-button mice, both using the MOUSE driver.  The following files

are in this set:

MOUSER.PAS : unit containing the mouse routines.

MOUSER.DOC : This file.

MOUSDEMO.PAS : Short demo program to demonstrate the mouse routines.

The program echos back the motion counters and the status

of the mouse button variables.  RIGHTBUTTON or CENTERBUTTON

terminates program.