Pascal Communications Library ( PCL )

for Turbo Pascal 4.0 through 6.0

USER REFERENCE MANUAL

Version 1.0

20 May 1991

Copyright (C) 1991

By MarshallSoft

All rights reserved

MarshallSoft

PO Box 4543

Hunstville, AL 35815

(205) 881-4630

Page 1

Table of Contents

Chapter                                     Page

Introduction.................................3

Registration.................................4

Serial COM Ports.............................5

RS232........................................6

National INS8250 UART........................7

Revision History.............................7

Warranty.....................................7

Using the Library............................8

Compiling.................................9

Problems.................................10

Library Reference...........................11

SioBaud..................................12

SioBrkKey................................13

SioBrkSig................................14

SioCrtWrite..............................15

SioDTR...................................16

SioDelay.................................17

SioDone..................................18

SioError.................................19

SioGetc..................................20

SioKeyPress..............................21

SioKeyRead...............................22

SioLine..................................23

SioModem.................................24

SioParms.................................25

SioPutc..................................26

SioRTS...................................27

SioReset.................................28

SioRxBuf.................................29

SioRxFlush...............................30

SioRxQue.................................31

SioTimer.................................32

SioUnGetc................................33

Function Summary............................34

Error Code Summary..........................35

Page 2

Introduction

The    Pascal  Communications  Library  ( PCL ) is an asynchronous

communications   library   designed   for   experienced   software

developers  programming in Turbo Pascal ( version 4.0 and up).  An

IBM PC/XT/AT or compatible is required.  The PCL features:

o  16 communications functions + 6 support functions.

o  Receiver is interrupt driven.

o  Runs from 300 baud to 115,200 baud.

o  Supports COM1, COM2, COM3, and COM4.

o  Adjustable receive queues from 8 bytes to 16 KB.

o  Control-BREAK error exit.

o  11 comm error conditions trapped.

o  Allows 2 ports to run concurrently.

o  Complete modem control & status.

o  Written in hand optimized assembly language.

o  Low overhead & very reliable !

A typical application  program  using  PCL  might  look  like  the

following code outline:

+----------------------------------------------------------------+

| program YourProgram                                            |

| uses PCL;                                                      |

| ...                                                            |

| ...                                                            |

| var Buffer : array[0..1023] of Char;                           |

| ...                                                            |

| begin (* YourProgram *)                                        |

|    RetCode := SioRxBuf(Port,Buffer,Size1024);                  |

|    RetCode := SioParms(Port,NoParity,OneStopBit,WordLength8);  |

|    RetCode := SioReset(Port,Baud2400);                         |

|    ...                                                         |

|    ... ( application code )                                    |

|    ...                                                         |

|    RetCode := SioDone(Port);                                   |

| end. (* YourProgram *)                                         |

+----------------------------------------------------------------+

A simple terminal emulator program CALLPGM.C is provided in source

code  form  as an example of the use of PCL functions. CALLPGM can

be used to call up bulletin board services or mainframe computers.

If you find any problems with the  Pascal  Communications  Library

or  have any suggestions for improvement, please call or write.  A

C language version of PCL ( called CCL )  is available.

Custom versions of PCL can be developed for special needs.  Custom

communication packages can also be developed. Call for prices.

Page 3

Registration

The  shareware  version  of  PCL.LIB  is  provided so that you may

personally determine the usefulness of the product  for  yourself.

If you can use PCL, please register your use with us.

MarshallSoft

PO Box 4543

Huntsville, AL

35815-4543

Please  pay  by  check  in  US  dollars.   Payment  must accompany

purchase orders. Print the file PCL.INV if an invoice  is  needed.

The registered package is $35 postpaid and includes:

o  PCL.PAS      PCL Unit.

o  PCL_LIB.ASM  PCL library source code.

( without the shareware screen )

o  PCL.DOC      PCL User Reference Manual.

o  Backbone bound printed User Reference Manual.

o  Telephone support for one year.

o  All future updates are $10

PCL_LIB.ASM is the source code for the library. The source code is

copyrighted  by MarshallSoft. The user is granted a license to use

the PCL object code in his own application only.   PCL_LIB.ASM  is

not shareware and may not be sold or given away to anyone.

The  registered  user  will  receive  the latest version of PCL by

return mail.  A 5.25" diskette is provided unless a 3.5"  diskette

is requested when ordering.

Page 4

Serial COM Ports

IBM PC compatible computers can have up to four serial ports.  The

BIOS   table   located  at  address  40:0000  has  room  for  four

communication port addresses: COM1 to COM4. During  boot  up,  the

COM1  and  COM2  addresses  are placed in the BIOS table providing

that the hardware is present. Unfortunately,  the  COM3  and  COM4

addresses  are not placed in the table in DOS through version 3.3.

This can be corrected by using DEBUG  to  assemble  the  following

program  SETCOM3  which  should be added to the AUTOEXEC.BAT file.

When executed, this program will  place  the  standard  COM3  port

address  03E8 in the BIOS table. For COM4, change 03E8 to 02E8 and

[0004] to [0006].

PUSH DS

MOV AX,0040

MOV DS,AX

MOV AX,03E8

MOV [0004],AX

POP DS

MOV AX,4C00

INT 21

Please note that you must have the physical hardware present !

Page 5

RS-232C

RS-232 is the name of the serial data interface standard  used  to

connect  computers  to  modems.  Most IBM compatible computers are

built with at least one serial port and use either DB9 ( 9  pin  )

or DB25 ( 25 pin ) connectors.

A  summary  of  these  pins  and their function follows.  For more

detailed information, refer to one of the many books dealing  with

RS-232 interfacing.

Signal Ground Pin 7 (DB25), Pin 5 (DB9)

The SG line is used as the common signal ground, and  must  always

be connected.

Transmit Data Pin 2 (DB25), Pin 3 (DB9)

The TX line is used to carry data from the computer to the modem.

Receive Data Pin 3 (DB25), Pin 2 (DB9)

The RX line is used to carry data from the modem to the computer.

Data Terminal Ready Pin 20 (DB25), Pin 4 (DB9)

The  DTR  line is used by the computer to signal the modem that it

is ready.

Data Set Ready Pin 6 (DB25), Pin 6 (DB9)

The DSR line is used by the modem to signal the computer  that  it

is ready.

Request to Send Pin 4 (DB25), Pin 7 (DB9)

The  RTS  line  is  used  to "turn the line around" in half duplex

modems, but is not necessary in full duplex modems.

Clear to Send Pin 5 (DB25), Pin 8 (DB9)

The  CTS  line, like the RTS line, in not necessary in full duplex

modems.

Data Carrier Detect Pin 8 (DB25), Pin 1 (DB9)

The DCD line is used by the modem to signal the  computer  that  a

data carrier signal is present.

Ring Indicator Pin 22 (DB25), Pin 9 (DB9)

The RI line is asserted when a 'ring' occurs.

Page 6

National INS8250

The   Pascal  Communications  Library  is  based  on  the standard

National INS8250 UART. The  8250  consists  of  6  register  ports

based at the following standard addresses:

COM1 = 3F8H  COM2 = 2F8H  COM3 = 3E8H  COM4 = 2E8H

If  you  are not familiar with the INS8250, several good books are

available.  Although a knowledge of the 8250 is not  necessary  to

use  PCL,  a  general  knowledge  of  the  theory  of operation of

Univeral  Asynchronous  Receiver  /  Transmitters  (  UARTs  )  is

recommended.

Offset R/W Register

0       R/W   Receiver ( read ) / Transmitter ( write )

1       R/W   Interrupt Enable

2       R     Interrupt Identification

3       R/W   Data Format ( Line Control )

4       R/W   RS-232 ( Modem ) Control

5       R/W   Line Status

6       R/W   RS-232 ( Modem ) Status

Revision History

Version 1.0 -- 20 May 1991 -- original release.

Warranty

The user of this software assumes all liability for its use. In no

case shall MarshallSoft be liable for any damages,  including  any

incidental or consequential damages.

Page 7

Using the Library

The  first  thing  to  do  is  to  copy all the files from the PCL

distribution disk to a working disk,  and  put  the  original  PCL

distribution disk in a safe place.

The PCL has been tested on a TANDY 1000 ( IBM PC clone ), a  TANDY

3000  (  IBM  AT  clone  ), a TANDY 1400LT ( IBM XT clone ), and a

Gateway 2000 Cache ( 25 MHZ 80386-DX ).

For  an  example of PCL use, examine the terminal emulator program

CALLPGM.PAS. The user should compile CALLPGM.PAS as a test of  the

library.

If you have two computers, then you can connect them together with

a  null modem cable and run CALLPGM on both machines.  Whatever is

typed on one machine should appear on the other, and  vice  versa.

Depending  on  the  design  of the null modem cable, CALLPGM.PAS may

need to be modified so that it does not  wait  for  DSR.  This  is

clearly documented in the CALLPGM.PAS code.

If you have a modem, then use CALLPGM  to  call  up  any  bulletin

board  system  (  BBS  ).   There  are  many  free BBSs around the

country. Look in any issue of "Computer Shopper"  (  available  in

bookstores,  computer  shops, and many grocery stores ) for a list

of current systems.

Page 8

Compiling

Registered  users  may  wish to assemble PCL_LIB.ASM.  Use the /MX

switch in order to disable automatic conversion from lower case to

to upper case. To assemble using the Microsoft assembler:

MASM PCL_LIB /MX;

To build the library TPU:

TPC PCL

To compile the sample program:

TPC CALLPGM

Page 9

Problems

If  you cannot get your application to run properly, first compile

and run the terminal emulator program  CALLPGM  provided  on  your

distribution disk.

If  CALLPGM  runs  correctly,  then  you  have  made a programming

mistake in  your  application.   MarshallSoft  cannot  debug  your

application,  especially  over  the  telephone!  However, consider

each of  the  following  when  searching  for  an  error  in  your

application.

1.  Did you include the "uses PCL" statement ?

2.  Is your receive buffer large enough ? If you are using 1K data

blocks in a file transfer  protocol  such  as  YMODEM,  then  your

receive buffer should be 1K or more.

3.   Have  you  selected too high a baud rate ?  Always start with

the slowest baud rate.  If only one COM port  is  being  run,  you

should be able to run at 38400 baud.

4.    Are  you  attempting  to  run  another  application  in  the

background ?  Try running without any other  programs  running  in

the background.

5.  If you are running two COM ports simultaneously, are you using

seperate receive buffers ? ( you should ).

6.  Did SioReset return a zero value ?  If not, then you must call

SioReset again. See CALLPGM.PAS for an example.

If  CALLPGM  does  not  run,  then  either  there  is  a  physical

connection problem or your computer isn't  as  compatible  as  you

thought!   Registered users can always call (205) 881 - 4630 after

5 PM CST Monday through Friday for help.

Page 10

Library Reference

The  remainder  of  this manual list all the PCL functions.  Every

library function will return a value as follows:

1.  Negative values for error conditions. See last  page  of  this

manual for a list of error values and their meanings.

2.  Non-negative values when returning data ( eg: SioLine ).

3.  Zero otherwise.

When  debugging an application, be sure to test all return values.

Use SioError to print the associated text for errors.

/*** example code segment ***/

RetCode := SioFunction();    (* any PCL function *)

if RetCode < 0 then begin

RetCode := SioError(RetCode);

(* ...do some stuff... *)

end;

Page 11

SioBaud

Function

Sets the baud rate of the selected port.

Syntax

function SioBaud(Port,BaudCode : Integer) : Integer;

Arguments

Port     : Port selected (COM1..COM4).

BaudCode : Baud code.

Remarks

The  SioBaud  function  sets  the  baud rate without resetting the

port. It is used to change the baud rate after calling SioReset.

Baud Code        Baud Rate        PCL Name

0                 300            Baud300

1                 600            Baud600

2                1200            Baud1200

3                2400            Baud2400

4                4800            Baud4800

5                9600            Baud9600

6               19200            Baud19200

7               38400            Baud38400

8               57600            Baud57600

9              115200            Baud115200

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad  port selected.  Value  must  be  0  to 3.

-11 : Bad baud rate code. See above code values.

Example

/* do auto baud detect */

for Code = 0 to 9 do begin

RetCode := SioBaud(Port,Code);

RetCode := SioPutc(Port,'A');

if SioGetc(Port,18) = Ord('A') then

writeln('Baud rate detected');

(*...do something here...*)

end

end;

Page 12

SioBrkKey

Function

Return non-zero if the Control-BREAK key was pressed.

Syntax

function SioBrkKey : Integer;

Remarks

The SioBrkKey function returns a TRUE value ( non-zero  )  if  the

Control-BREAK  key  was  pressed,  else  it  returns  a zero.  Use

SioBrkKey as a safety exit from a polling loop.   Don't  mix  this

function up with SioBrkSig.

Returns

-1 : Control-BREAK was pressed.

0 : Control-BREAK was NOT pressed.

Example

while TRUE do begin

if SioBrkKey then begin

writeln('User typed Contrl-BREAK');

RetCode := SioDone(Port);

halt;

end;

(* do some other stuff *)

end;

Page 13

SioBrkSig

Function

Asserts, cancels, or detects BREAK signal.

Syntax

function SioBrkSig(Port : Integer; Cmd : Char) : Boolean;

Arguments

Port : Port selected (COM1,COM2,COM3,or COM4)

Cmd  : ASSERT, CANCEL, or DETECT

Remarks

The SioBrkSig function controls the BREAK bit in the  line  status

register. The legal commands are:

ASSERT ('A') to assert BREAK

CANCEL ('C') to cancel BREAK

DETECT ('D') to detect BREAK

ASSERT,  CANCEL,  and  DETECT  are defined in PCL.PAS. See CALLPGM.PAS

for an example of the use of SioBrkSig.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-6 : Illegal command. Expected 'A', 'C', or 'D'.

>0 : BREAK signal detected ( DETECT only )

Example

(* Assert BREAK for 1 second *)

RetCode := SioBrkSig(Port,ASSERT);

RetCode := SioDelay(18);

RetCode := SioBrkSig(Port,CANCEL);

(* Detect BREAK *)

if SioBrkSig(Port,DETECT) then begin

writeln('BREAK signal detected');

(* ...do some more stuff... *)

end

Page 14

SioCrtWrite

Function

Write character to the screen.

Syntax

function SioCrtWrite(ch : Char) : Integer;

Arguments

ch = Character to write

Remarks

The SioCrtWrite function uses the BIOS to write a single character

to the screen at the current cursor location.

SioCrtWrite  is  faster  than a call to  the  Pascal  library  and

for this reason is included in PCL.

Returns

zero

Example

Ch := SioGetc(COM1,18);

if Ch <> -1 then begin

(* do some stuff *)

end

Page 15

SioDTR

Function

Set, clear, or read the Data Terminal Ready ( DTR ) bit.

Syntax

function SioDTR(Port,Cmd : Integer) : Integer;

Arguments

Port : Port selected (COM1,COM2,COM3,or COM4)

Cmd  : DTR command ( SET, CLEAR, or READ )

Remarks

The  SioDTR  function controls the Data Terminal Ready ( DTR ) bit

in the modem control register. Commands (  defined  in  PCL.PAS  )

are:

SET ('S')  to set DTR ( ON )

CLEAR ('C')  to clear DTR ( OFF )

READ ('R')  to read DTR

Returns

-2 : Port not enabled. Must call RetCode := SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-5 : Not one of 'S', 'C', or 'R'.

0 : DTR is OFF (READ Command).

>0 : DTR is ON (READ Command).

Example

(* turn DTR on for modem connected to COM4 *)

RetCode := SioDTR(COM4,SET);

Page 16

RetCode := SioDelay

Function

Delays one or more tics.

Syntax

function SioDelay(Tics : Integer) : Integer;

Arguments

Tics : Number of timer tics ( 18.2 per second )

Remarks

The  SioDelay function is used to delay one or  more  timer  tics,

where  each timer tic is approximately 55 milliseconds ( 18 to the

second ). See SioTimer also.

Returns

zero

Example

RetCode := SioDelay(5*18);               (* delay 5 seconds *)

Page 17

SioDone

Function

Terminates further serial processing.

Syntax

function SioDone(Port : Integer) : Integer;

Arguments

Port : Port selected (COM1,COM2,COM3,or COM4)

Remarks

The   SioDone   function  terminates  further  serial  processing.

SioDone  MUST  be  called  before exiting your application so that

interrupts can be restored to their original state. Failure to  do

this  can  crash  the  operating  system.   If  you forget to call

SioDone before exiting, be sure to re-boot your computer.

Returns

-2 : Port not enabled. Must call RetCode := SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

(* terminate processing for COM3 *)

RetCode := SioDone(COM3);

Page 18

SioError

Function

Displays error in text.

Syntax

function SioError(Code : Integer) : Integer;

Arguments

Code : Error code returned from a PCL function

Remarks

The SioError function displays the error in text corresponding  to

the   error   code.    During   development  of  a  communications

application, it is a good idea to always test  return  codes,  and

print out their descriptions with SioError.

Returns

none

Example

RetCode := SioReset(Port,Baud4800);

if RetCode < 0 then RetCode := SioError(RetCode);

Page 19

SioGetc

Function

Reads the next character from the serial line.

Syntax

function SioGetc(Port,Tics : Integer) : Integer;

Arguments

Port : COM1 to COM4

Tics : Number of timer tics

Remarks

The  SioGetc function reads the selected serial port. The function

will wait for the number  of  system  tics  given  by  the  'Tics'

argument  before  returning  'timed out'. There are 18 tics to the

second.

To specify no waiting, call SioGetc with Tics = 0.

Returns

-2 : Port not enabled. Must call RetCode := SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-1 : If timed out.

>0 : Character read.

Example

RetCode := SioGetc(COM1,1);

if RetCode <> -1 then  writeln('Character is ', chr(RetCode) );

else writeln('Timed out');

Page 20

SioKeyPress

Function

Detects if keyboard has been pressed.

Syntax

function SioKeyPress() : Boolean;

Remarks

The  SioKeyPress function uses the BIOS to test the keyboard for a

key press.

Returns

zero

Example

if SioKeyPress begin

(* ...do something... *)

end;

Page 21

SioKeyRead

Function

Reads the keyboard.

Syntax

function SioKeyRead : Integer;

Remarks

The SioKeyRead function uses the BIOS to read the keyboard.   Will

wait until a character is typed.

SioKeyRead is faster than using the Pascal library.

Returns

character typed.

Example

if SioKeyPress then begin

RetCode := SioKeyRead();

end

Page 22

SioLine

Function

Reads the line status register.

Syntax

function SioLine(Port : Integer) : Integer;

Arguments

Port : Port selected (COM1,COM2,COM3,or COM4)

Remarks

The  SioLine  function  reads  the  line  status  register.   The

individual bit masks are as follows:

$20  = Transmitter Buffer Empty.

$10  = Break detected.

$08  = Framming error.

$04  = Parity error.

$02  = Overrun error.

$01  = Data ready.

The above are documented in the file PCL.PAS.

Returns

-2 : Port not enabled. Must call RetCode := SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

>0 : Line status ( rightmost byte of word ).

Example

RetCode := SioLine(Port);

if(RetCode and (FramingError or ParityError or OverrunError)) <> 0

then

begin

if (RetCode and FramingError) <> 0 then writeln('Framing Error');

if (RetCode and ParityError) <> 0 then writeln('Parity Error');

if (RetCode and OverrunError) <> 0 then writeln('Overrun Error')

end

else writeln('No error');

Page 23

SioModem

Function

Reads the modem status register.

Syntax

function SioModem(Port : Integer;  Mask : Char) ; Integer;

Arguments

Port : Port selected (COM1,COM2,COM3,or COM4)

Mask : Modem function mask

Remarks

The SioModem   function  reads  the  modem  register.   The  bit

definitions for the function mask are as follows:

Bit    PCL.PAS Name   Function

7      DCD         Data Carrier Detect

6      RI          Ring Indicator

5      DSR         Data Set Ready

4      CTS         Clear To Send

3      DeltaDCD    Delta DCD   ( DCD has changed )

2      DeltaRI     Delta RI    ( RI has changed )

1      DeltaDSR    Delta DSR   ( DSR has changed )

0      DeltaCTS    Delta CTS   ( CTS has changed )

Bits  4 through 7 represent the absolute state of their respective

RS-232 inputs. Bits 0 through 3 repesent a change in the state  of

their respective RS-232 inputs since last read.

The above definitions are also in the PCL.PAS file for use  by  your

application program.

Returns

-2 : Port not enabled. Must call RetCode := SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

>0 : Modem status ( rightmost byte of word ).

Example

(* any change in DCD ? *)

Status := SioModem(Port,DeltaDCD);

if Status < 0 then

begin

RetCode := SioError(Status);

Halt;

end;

else writeln('DCD status = ', SioModem(Port,DCD) );

Page 24

SioParms

Function

Sets parity, stop bits, and word length.

Syntax

function SioParms(Port,ParityCode,StopBitsCode,WordLengthCode: Integer)

: Integer;

Arguments

Port           : Port selected (COM1,COM2,COM3,or COM4)

ParityCode     : Parity code [0,1,2]

StopBitsCode   : Stop bits code  [0,1]

WordLengthCode : Word length code [0,1,2,3]

Remarks

The SioParms function sets the parity, stop bits, and word length.

If the default parity ( none ), stop bits ( 1 ), or word length  (

8  )  is  not  acceptable,  then  they  can  be changed by calling

SioParms.  SioParms can be called either before or  after  calling

SioReset. See file PCL.PAS.

Value      Description    PCL.PAS Name

ParityCode:   *0        no parity      NoParity

1        odd parity     OddParity

2        even parity    EvenParity

StopBitsCode:  *0        1 stop bit     OneStopBit

1        2 stop bits    TwoStopBits

WordLengthCode: 0        5 data bits    WordLength5

1        6 data bits    WordLength6

2        7 data bits    WordLength7

*3        8 data bits    WordLength8

* = Default

Returns

-2 : Port not enabled. Must call RetCode := SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-7 : Bad parity code selected. Value must be 0 to 2.

-8 : Bad stop bits code. Value must be 0 or 1.

-9 : Bad word length code. Value must be 0 to 3.

Example

RetCode := SioParms(COM1,NoParity,OneStopBit,WordLength8);

Page 25

SioPutc

Function

Transmit a character over a serial line.

Syntax

function SioPutc(Port : Integer; Ch : Char) : Integer;

Arguments

Port : Port selected (COM1,COM2,COM3,or COM4)

Ch   : Character to send

Remarks

The  SioPutc  function  transmits  one character over the selected

serial line.

Returns

-2 : Port not enabled. Must call RetCode := SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

crc := 0;

for i = 0 to 127 do begin

crc := crcupdate( buffer[i], crc);

RetCode := SioPutc(Port, buffer[i]);

end;

RetCode := SioPutc(crc);

Page 26

SioRTS

Function

Sets, clears, or reads the Request  to  Send ( RTS ) line.

Syntax

function SioRTS(Port : Integer; Cmd : Char) : Integer;

Arguments

Port : COM1 to COM4

Cmd  : RTS command ( SET, CLEAR, or READ )

Remarks

The SioRTS function controls the Request to Send ( RTS  )  bit  in

the modem control register. Commands ( defined in PCL.PAS ) are:

SET ('S')  set RTS ( ON )

CLEAR ('C')  clear RTS ( OFF )

READ ('R')  read RTS

Returns

-2 : Port not enabled. Must call RetCode := SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-5 : Command is not one of 'S', 'C', or 'R'.

0 : RTS is OFF (READ Command).

>0 : RTS is ON  (READ Command).

Example

(* turn off RTS for modem *)

RetCode := SioRTS(Port,CLEAR);

Page 27

SioReset

Function

Initialize a serial port for processing.

Syntax

function SioReset(Port,BaudCode : Integer) : Integer;

Arguments

Port     : Port selected (COM1,COM2,COM3,or COM4)

BaudCode : baud code

Remarks

The  SioReset  function  initializes  the  selected  serial  port.

SioReset  should  be called after calling SioParm and SioRxBuf but

before making any other calls to PCL. SioReset  uses  the  parity,

stop  bits,  and  word  length value previously set if SioParm was

called, otherwise the default values ( see SioParm ) are used.

Recall that COM1 and COM3 share  the  same  interrupt  vector  and

therefore  cannot  operate  simultaneously.   Similiarly, COM2 and

COM4 cannot operate simultaneously. Any other combination  of  two

ports can be used.

See SioBaud for a list of the baud rate codes, or see "PCL.PAS".

Returns

-4 : Bad port selected. Value must be 0 to 3.

-11 : Bad baud rate code selected. Value must be 0 to 9.

Example

RetCode := SioRxBuf(COM1,Buffer,Size128);

RetCode := SioReset(Com1,Baud38400);

if RetCode = 0 then writeln('RESET ok');

else begin

RetCode := SioError(rc);

if (RetCode and OverrunError) <> 0 then writeln('Overrun Error');

if (RetCode and ParityError) <> 0 then writeln('Parity Error');

if (RetCode and FramingError) <> 0 then writeln('Framing Error');

if (RetCode and BreakDetected) <> 0 then writeln('Break Detected');

end;

Page 28

SioRxBuf

Function

Sets up receive buffers.

Syntax

function SioRxBuf(Port,BufferOfs,BufferSeg,SizeCode : Integer) : Integer;

Arguments

Port     : Port selected (COM1,COM2,COM3,or COM4)

Buffer   : Receive buffer

SizeCode : Buffer size code

Remarks

The  SioRxBuf  function passes the address and size of the receive

buffer to PCL. Recall that PCL requires a receive buffer for  each

port  in  simultaneous  operation  since  the  receive function is

interrupt driven.  SioRxBuf passes the receive buffer to  PCL  for

both  the primary ( COM1/COM3 ) and secondary ( COM2/COM4 ) ports.

It must be called before any incoming characters can be  received.

SioRxBuf  should  be called before SioReset. Buffer size codes are

listed in "PCL.PAS".

Size Code       Buffer Size    PCL.PAS Name

0              8 bytes        Size8

1             16 bytes        Size16

2             32 bytes        Size32

3             64 bytes        Size64

4            128 bytes        Size128

5            256 bytes        Size256

6            512 bytes        Size512

7           1024 bytes        Size1024

8           2048 bytes        Size2048

9           4096 bytes        Size4096

10           8192 bytes        Size8192

11          16384 bytes        Size16384

Returns

-2 : Port not enabled. Must call RetCode := SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-10 : Bad buffer size code. Value must be between 0 and 11.

Example

RetCode := SioRxBuf( COM1, RxBuf, 128);

Page 29

SioRxFlush

Function

To flush the receive buffer associated with the specified port.

Syntax

function SioRxFlush(Port: Integer) : Integer;

Arguments

Port : Port selected (COM1,COM2,COM3,or COM4)

Remarks

The  SioRxFlush function will delete any characters in the receive

buffer for the  specified  port.   After  execution,  the  receive

buffer  will  be  empty.   Call SioRxBuf after resetting a port in

order to delete any spurious characters.

Returns

-2 : Port not enabled. Must call RetCode := SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

(* setup receive buffer  and  reset port *)

RetCode := SioRxBuf(COM1,buffer,Size1024);

RetCode := SioReset(COM1,Baud115200);

(* flush any spurious character *)

RetCode := SioRxFlush(COM1);

(* ready for serial processing ! *)

Page 30

SioRxQue

Function

Returns the number of characters in the receive queue.

Syntax

function SioRxQue(Port : Integer) : Integer;

Arguments

Port : Port selected (COM1,COM2,COM3,or COM4)

Remarks

The SioRxQue function will return the number of characters in  the

receive queue. It can be used to implement XON/XOFF flow control.

Returns

-2 : Port not enabled. Must call RetCode := SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

RetCode := SioRxBuf(COM1,Buffer,Size128);

(* implement XON / XOFF *)

count := SioRxQue(COM1);

if (last=XON) and (count>120) then

begin

RetCode := SioPutc(COM1,char(XOFF));

last := XOFF;

end

if (last=XOFF) and (count<8) then

begin

RetCode := SioPutc(COM1,char(XON));

last := XON;

end

Page 31

SioTimer

Function

Returns the number of system clock tics since midnight.

Syntax

function SioTimer : Integer;

Remarks

The SioTimer function will return the number of system clock  tics

since  midnight,  at  18.2065  tics  per second.  This function is

usefull for timeing various functions. Also see SioDelay.

Returns

timer tics

Example

Time := SioTimer();

(* do some stuff *)

writeln('Elasped time ', SioTimer - Time );

Page 32

SioUnGetc

Function

"Un-gets" the last character read with SioGetc.

Syntax

function SioUnGetc(Port : Integer; Ch : Char) : Integer;

Arguments

Port : Port selected (COM1,COM2,COM3,or COM4)

Ch   : Character to unget

Remarks

The SioUnGetc function returns ( pushes ) the character back  into

the  serial  input  buffer.  The character pushed will be the next

character returned by SioGetc. Only one character  can  be  pushed

back. This function works just like the "ungetc" function in the C

language.

Returns

-2 : Port not enabled. Must call RetCode := SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

RetCode := SioUnGetc(Port,c);

Page 33

Function Sumary

+-------------+----------+----------+----------+----------+

| Function    |   Arg1   |   Arg2   |   Arg3   |   Arg4   |

+-------------+----------+----------+----------+----------+

|  SioBaud    | Port     | BaudCode |          |          |

+-------------+----------+----------+----------+----------+

|  SioBrkKey  |          |          |          |          |

+-------------+----------+----------+----------+----------+

|  SioBrkSig  | Port     | Cmd      |          |          |

+-------------+----------+----------+----------+----------+

|  SioCrtWrite| Ch       |          |          |          |

+-------------+----------+----------+----------+----------+

|  SioDTR     | Port     | Cmd      |          |          |

+-------------+----------+----------+----------+----------+

|  SioDelay   | Tics     |          |          |          |

+-------------+----------+----------+----------+----------+

|  SioDone    | Port     |          |          |          |

+-------------+----------+----------+----------+----------+

|  SioError   | Code     |          |          |          |

+-------------+----------+----------+----------+----------+

|  SioGetc    | Port     | Tics     |          |          |

+-------------+----------+----------+----------+----------+

|  SioKeyPress|          |          |          |          |

+-------------+----------+----------+----------+----------+

|  SioKeyRead |          |          |          |          |

+-------------+----------+----------+----------+----------+

|  SioLine    | Port     |          |          |          |

+-------------+----------+----------+----------+----------+

|  SioModem   | Port     | Mask     |          |          |

+-------------+----------+----------+----------+----------+

|  SioParms   | Port     | Parity   | StopBits |WordLength|

+-------------+----------+----------+----------+----------+

|  SioPutc    | Port     | Ch       |          |          |

+-------------+----------+----------+----------+----------+

|  SioRTS     | Port     | Cmd      |          |          |

+-------------+----------+----------+----------+----------+

|  SioReset   | Port     | BaudCode |          |          |

+-------------+----------+----------+----------+----------+

|  SioRxBuf   | Port     | Buffer   | SizeCode |          |

+-------------+----------+----------+----------+----------+

|  SioRxFlush | Port     |          |          |          |

+-------------+----------+----------+----------+----------+

|  SioRxQue   | Port     |          |          |          |

+-------------+----------+----------+----------+----------+

|  SioTimer   |          |          |          |          |

+-------------+----------+----------+----------+----------+

|  SioUnGetc  | Port     | Ch       |          |          |

+-------------+----------+----------+----------+----------+

Page 34

Error Code Summary

Code        Description

0          No error.

-1          Timeout waiting for input. Only returned by  SioGetc.

-2          Port not enabled. Must call  SioReset first.

-3          No buffer available. Must call  SioRxBuf before

calling SioReset.

-4          Bad port specified. Must be 0 to 3. Recall that COM1

is port 0, COM2 is port 1, etc.

-5          Expected 'S', 'C', or 'R' as second argument.

-6          Expected 'A', 'C', or 'D' as second argument.

-7          Bad parity code specified. Must be 0 to 7.

-8          Bad stop bits code specified. Must be 0 or 1.

-9          Bad wordlength code specified. Must be 0 to 3.

-10          Bad buffer size code specified. Must be 0 to 11.

-11          Bad baud rate code. Must be 0 to 9.

Page 35