Q U I C K  B A S I C

��� ��� ����� ����  �����

� � � �   �   �   �   �

�  �  �   �   �   �   �

�  �  � ����� ����  �����

QBMIDI(TM) Library Overview

S H A R E W A R E    V E R S I O N    1 . 0

SHAREWARE PRICE $20.00

Developed by:

AskUs! Technology Specialists

PO Box 737

Bountiful, UT 84011-0737

QBMIDI is a Trademark of AskUs! Technology Specialists

(c) 1990 AskUs.  All rights reserved.

Hello and welcome to QBMIDI.  If you use MIDI and you program using

Microsoft QuickBASIC or BASIC PDS, then we're confident you'll love the

QBMIDI library, since it gives you to access and control your MIDI'ed

instruments using simple QuickBASIC compatible calls.  QBMIDI V1.0 is a

fully functional library of utilities that provide you with access to

all of your MIDI instruments connected through a Roland MPU401 or

compatible MIDI interface.  You can play your instruments via QuickBASIC

as well as receive MIDI information and send MIDI commands to your MIDI'ed

instruments.  It's simple, fast, and best of all ... you'll be in

control of your MIDI instruments!

Included with this document file (QBMIDI.DOC), you'll find the following

related files:

QBM-DEM1.BAS     Initial demonstration program that features a few

simple MIDI commands and general starting place

to view source examples of QBMIDI's use.

QBM-DEM2.BAS     A demonstration program that generates music based

constraints you provide.  Not only does it show

working examples of your computer controlling your

MIDI instruments, it's interesting to hear the

sounds created.  We invite you to add functions and

modify both of the demo programs to suit your needs.

QBM-DEM3.BAS     A simple keyboard split program.

QBMIDI.QLB       A QuickBASIC 4.5 environment QuickLibrary that

provides you access to QBMIDI functions from within

the QuickBASIC programming environment (QB.EXE).

QBMIDI.LIB       A linkable library suitable for linking to your

result .EXE programs.  Once linked, the QBMIDI calls

you use become an integral part of your compiled

program.

QBMIDI.ORD       Order information for the advanced MUSICIANS QBMIDI

V2.0 which contains numerous advanced MIDI functions

that are not included with this Shareware version.

A QUICK OVERVIEW OF QBMIDI's V1.0 FUNCTIONS

-------------------------------------------

CALL STATEMENT                 WHAT IT DOES

--------------                 ---------------------------------------------

CALL SeeIfMPUExists(Found)     If Midi Controller (MPU401) or compatible

is found in your system

CALL ResetMpu                  Resets the MPU to power on state

CALL SetDataInStopMode         Turns off some of the MPU's intelligence

you'll be controlling it via QuickBASIC

CALL PlayNote(Note, Velocity)  Plays a Note, striking it as you would from

a keyboard with different strike velocities,

or turns off a note currently playing

CALL AllNotesOff               All notes off currently playing turned off

CALL ChangePatchTo (NewPatch)  Change the patch (voice) currently in use

to New Patch

CALL ReceiveMidiData(Value)    Receive any midi data that may be coming

in.  It listens to MIDI being sent back.

CALL OmniModeOn                Turn on Omni, where all MIDI data will be

played by all synthesizer channels.

CALL OmniModeOff               Set Omni Off, where MIDI channel data is

only received by the synthesizer's channel

CALL SetToPolyMode             Set multi-voice mode

CALL SetToMonoMode             Set to Mono-voice mode

The demo programs make use of all functions found in QBMIDI 1.0, and

describe usage within the source code of the demo programs.  We encourage

experimentation with the demo programs to gain a better understanding while

having some fun.

FOR NEW QUICKBASIC PROGRAMMERS

------------------------------

For new programmers, like we were at one time,we've tried to aim our examples

at your level.  QuickBASIC's library and sub functions can be somewhat tricky

at first, so we've listed what you should type to get the libraries loaded

and running.  If you're a pro at QuickBASIC please excuse the lengthy

descriptions, as we want everyone to get to make music as fast as possible

with the least amount of problems.

USING QBMIDI WITHIN THE QUICKBASIC PROGRAMMING ENVIRONMENT (QB.EXE)

-------------------------------------------------------------------

Two libraries are provided with QBMIDI, one called QBMIDI.QLB (the Quick

Library) and QBMIDI.LIB (the Linkable library).  When working within the

QuickBASIC programming environment (ie. using QB.EXE), you have the option

to load a QuickLibrary at the time you start QB.EXE.  We make use of this

feature to enables you to use all of QBMIDI call routines while debugging

your programs.

To start QuickBASIC with QBMIDI library functions available you would type

the following:

QB /L QBMIDI <Enter>

Note: The file QBMIDI.QLB should be placed in the same directory with your

other QB library files (typically the same directory where the file

QB.EXE is found).  You may also start QB.EXE with QBMIDI by adding

the pathname where to find QBMIDI.QLB (ie. QB /L Pathname\QBMIDI)

If when starting QB.EXE you receive this error message:

Cannot find file (QBMIDI.QLB). Input path:

you may type in a new path name, but we recommend control ^C and re-entering

a correct pathname from startup.  By doing this now, you'll save time when

compiling your programs to .EXE executable's.

If you are greeted with the Microsoft QuickBASIC environment, without seeing

any error messages, you can assume that QBMIDI.QLB was loaded and its

functions are available.  For some reason QuickBASIC doesn't tell you what

library is loaded of if the load went OK so we just say no news is good news.

WITHIN THE QUICKBASIC PROGRAMMING ENVIRONMENT

---------------------------------------------

To use any of the QBMIDI commands, you simply call the command as you would

a local SUB function.  Don't worry if you don't know how to use SUB's since

QBMIDI does not use the SUB capabilities, but is merely called like one.

For example:

Any good midi program should first check to see if an MPU401 controller

is present, connected, and ready for data.  To find out we use the

QBMIDI SeeIfMPUExists command.  The full QuickBASIC call syntax is:

CALL SeeIfMPUExists(Found)

When run, the QBMIDI.QLB library function named SeeIfMPUExists is called and

returns a value in the variable called Found.  SeeIfMPUExists returns the

value of zero (0) for no MPU found and -1 indicating an MPU was located.

NOTE: If you receive the message Subprogram Not Found, this indicates you've

not loaded QB using the /L option.  See the examples above and restart QB.EXE.

Each QBMIDI command functions in this same way, with one exception. Some

commands require that you set the variables values before calling.  For

example:

If playing a note, we us the PlayNote command with the following

syntax.

Note=60                           'Middle C

Velocity = 64                     'Moderate strike velocity

CALL PlayNote (Note,Velocity)     'Use QBMIDI playnote to play

In the above example, the values were set before calling the routine.  The

following table lists the QBMIDI command and a quick description of how it

is used.

CALL STATEMENT                 SEND/RETURNED

--------------                 ---------------------------------------------

CALL SeeIfMPUExists(Found)     Found = 0 if no MPU is found

Found = -1 if MPU is found and ready

CALL ResetMpu                  Resets the MPU to power on state. No variable

is given or returned. Use sparingly to reset

MPU if another program modifies it.  This will

not stop notes in play. Use AllNotesOff.

CALL SetDataInStopMode         Turns off some of the MPU's intelligence.

No variable is sent or returned.

Note = n                       Note to play. Range 0 to 127 where 60=Middle C

Velocity = n                   Strike velocity. Range 0 to 127 where 1=soft

127=hardest, and 0= turn note off.

CALL PlayNote(Note, Velocity)  Plays a Note, passes variables Note, Velocity

To turn note off, pass note number with the

velocity set at zero (0). Note and velocity

variables are left unchanged after call.

CALL AllNotesOff               All notes off currently playing turned off.

No variable is sent or returned.

NewPatch = n                   Patch number to change to, range 0 - 127

CALL ChangePatchTo (NewPatch)  Change the patch (voice) currently in use

to New Patch number given. NewPatch variable

is left unchanged after the call.

CALL ReceiveMidiData(Value)    Receive any midi data that may be coming

in from synthesizers or keyboards. Several

midi bytes may be sent in succession, so

your routines should be ready to catch all

that are sent.  See QBM-DEM2.BAS for more.

CALL OmniModeOn                Turn on Omni, where all MIDI data will be

played by all synthesizer channels. No

variables are passed or returned.

CALL OmniModeOff               Set Omni Off, where MIDI channel data is

only received by the synthesizer's channel. No

variables are passed or returned.

CALL SetToPolyMode             Set multi-voice mode. No variables are passed

or returned.

CALL SetToMonoMode             Set to Mono-voice mode. No variables are

passed or returned.

Call up the program titled QBM-DEM1.BAS by using the FILE OPEN command and

view it's contents.  This demo program shows a few of the basics and

illustrates how we call the routines from QuickBASIC.

COMPILING PROGRAMS FROM QUICKBASIC SO THEY CAN USE QBMIDI

---------------------------------------------------------

One nice feature within QuickBASIC is its ability to compile your program

to a standalone or runtime .EXE file and use the library you started with.

Since you started QB using the /L QBMIDI command, Quick-BASIC will

automatically use the QBMIDI.LIB (run version of QBMIDI) file when it

compiles your program.

To compile you're program, use Alt select, Run, Make EXE file.  Your screen

should resemble the following.  Select Make EXE and Exit and QuickBASIC will

compile your program and exit.  To run the example below after compile, you

type QBM-DEM1 <Enter>.

File  Edit  View  Search  <RUN>  Debug  Calls  Options            Help

�������������������������������� Untitled �������������������������Ĵ#�Ŀ

�   �������������������������� Make EXE File ����������������������Ŀ   #

�   �                ��������������������������������������������Ŀ �   �

�   � EXE File Name: �QBM-DEM1.EXE                                � �   �

�   �                ���������������������������������������������� �   �

�   �                                                               �   �

�   �   [ ] Produce Debug Code         Produce:                     �   �

�   �                                  ( ) EXE Requiring BRUN45.EXE �   �

�   �                                  (X) Stand-Alone EXE File     �   �

�   ���������������������������������������������������������������Ĵ   �

�   �  < Make EXE >   < Make EXE and Exit >   < Cancel >   < Help > �   �

�   �����������������������������������������������������������������   �

�������������������������������������������������������������������������

If you wish a further discussion of linking libraries, we refer you to your

Microsoft documentation.

PDS7 (BASIC 7.x) USERS

----------------------

Please write us about receiving PDS7.x compatible versions of QBMIDI.QLB and

QBMIDI.LIB.  We'll also supply QBMIDI.OBJ so you may add the functions to

other libraries you have.

CONCLUSION

-----------

We hope you, like us, enjoy making music and we feel making music with Quick-

BASIC is more fun than writing any other type of program.  As you experiment

you may find referring to the MIDI specification helpful or related MIDI

books helpful in understanding what's going on.

We also want you to know about our MUSICIANS VERSION of QBMIDI, that contains

added features such as:

o  Independent receive and Transmit over all 16 midi channels

o  Record and Playback using the MPU401

o  Facilities for MIDI dumps, voice dumps, etc.

o  Direct access to transmit and receive MIDI data

o  Metronome On/Off

o  Tempo adjustment

o  Customization to your synth's features

o  Superb Printed documentation

o  And much more ...

You may order MUSICIANS QBMIDI V2.0 by sending $45.00 cash or

check (we'll pay shipping to you by return mail) with your return address

info to:

AskUs!

QBMIDI 2.0

PO Box 737

Bountiful, UT 84011-0737

We thank you for trying QBMIDI and hope you find making QB Music as

enjoyable as we have.

Best Regards,

AskUs!