-----------------------------------------------------------------------------

*********      XLIB - Mode X graphics library           ****************

*********                                               ****************

********* Written By Themie Gouthas                     ****************

********* Aeronautical Research Laboratory              ****************

********* Defence Science and Technology Organisation   ****************

********* Australia                                     ****************

*********                                               ****************

********* egg@dstos3.dsto.gov.au                        ****************

********* teg@bart.dsto.gov.au                          ****************

Some of the code in this library has been contributed by :

Matthew MacKenzie - matm@eng.umd.edu

-----------------------------------------------------------------------------

DISCLAIMER

This library is distributed AS IS. The author/s specifically disclaim

any responsibility for any loss of profit or any incidental, consequen-

tial or other damages.

---------------------------------------------------------------------------

INTRODUCTION

---------------------------------------------------------------------------

XLIB is a "user supported freeware" graphics library specifically designed

with game programming in mind.

It has been placed in the public domain for the benefit of all, and

represents MANY hours of work so it is requested that all users comply

with the the wishes of the author/s as specified in the individual modules

and:

a) To leave the code in the public domain

b) Not distribute any modified versions of this library

New contribution and comments are welcome and hopefully there will be

more releases as the code evolves.

Finally, do not trust this excuse for a manual if in doubt. The place to get

the answers is in the code itself.

REQUIREMENTS

Minimum requirements

286 processor,

Turbo C 2.0 or higher, or BORLANDC

MAKE  2.0 or higher

TLIB  2.0 or higher

Turbo Assembler 1.01 or higher

GENERAL FEATURES

Support for a number of 256 colour tweaked graphics mode resolutions

320x200 320x240 360x200 360x240 320x400 320x480 360x400 360x480

Virtual screens larger than the physical screen (memory

permitting) that can be panned at pixel resolution in all directions

A split screen capability for status displays etc.

Text functions supporting 8x8 and 8x14 ROM fonts and user defined fonts

Support for page flipping

Graphics primitives such as line and rectangle drawing functions and

of course bit block manipulation functions

MODULES COMPRISING XLIB

XMAIN    - Main module containig mode setting code and basic functions

XPOINT   - Pixel functions

XRECT    - Filled Rectangle and VRAM to VRAM block move functions

XPAL     - Palette functions

XLINE    - Line Functions

XTEXT    - Text and Font Functions

XPRINTF  - Printf style string output

XPBITMAP - Planar Bitmap functions

XCBITMAP - Compiled Bitmap functions

-------------------------------------------------------------------------

BUILDING THE LIBRARIES

-------------------------------------------------------------------------

Building the library had been made simple through the use of make.

To build and examples for one of the two models:

a) edit the makefile for the apropriate model (see note in the makefile)

b) edit the makefile for the apropriate compiler (again see note in the

makefile)

c) type "make" at the dos prompt.

It should be as simple as that. If problems are encountered then check

to see if tasm, make, tlib, link and bcc (or tcc) are withinin your path.

If not either change your path or specify the full path for these programs

in the makefile. It is preferrable to have your path set correctly.

Individual Compilation

----------------------

each ASM module can be compiled with the following commandline:

tasm /ml /d<model> <asm module name>

where <model> is s or l. Note the small model library can be used with

compact model programs. Similarly the large model library can be used with

medium model programs.

The resulting libraries are:

xlib<version>s.lib    - small model library

xlib<version>l.lib    - large model library

To link the library with your programs just include the paropriate .lib

file in your project file or on the BCC or TCC command line.

Using the library with your programs

--------------------------------------

Using the XLIB library in your programs is simple. Knowing the particular

modules you require, just include the associated header files in your program

and link your program modules with the library. If you don't want to wory

about selecting the apropriate header file then just include "XLIB_ALL.H"

which automatically includes all XLIB header files in your program.

For example compilations see the supplied makefile.

--------------------------------------------------------------------------

GLOBAL DEFINES (xlib.inc)

--------------------------------------------------------------------------

Types

BYTE unsigned char

WORD unsigned int

Available X mode resolutions

X_MODE_320x200  0

X_MODE_320x240  1

X_MODE_360x200  2

X_MODE_360x240  3

X_MODE_320x400  4

X_MODE_320x480  5

X_MODE_360x400  6

X_MODE_360x480  7

Palette rotation direction directiion

BACKWARD 0

FORWARD  1

X_MODE_INVALID -1

ERROR           1

OK              0

--------------------------------------------------------------------------

MODULE XMAIN

--------------------------------------------------------------------------

The Xmain module is the base module of the XLIB library. It contains the

essential functions that initialize and customize the graphic environment.

ASM SOURCES

xmain.asm xmain.inc xlib.inc model.inc

C HEADER FILE

xlib.h

EXPORTED VARIABLES

NOTE: All variables are read only. I you modify them the results may

be unpredictable.

InGraphics -  BYTE -  Flag indicating that the xlib graphics system is

active. Set by function "x_set_mode".

CurrXMode - WORD - If the xlib graphics system is active, contains the id

of the x mode. Set by function "x_set_mode".

See also defines (ie X_MODE_320x200 ... )

ScrnPhysicalByteWidth - WORD - Physical screen width in bytes. Set by

function "x_set_mode"

ScrnPhysicalPixelWidth - WORD - Physical screen width in pixels. Set by

function "x_set_mode"

ScrnPhysicalPixelHeight - WORD - Physical screen height in pixels. Set by

function "x_set_mode".

ErrorValue - WORD - Contains error value. General use variable to

communicate the error status from several functions. The value

in this variable usually is only valid for the the last

function called that sets it.

SplitScrnOffs - WORD - Offset in video ram of split screen. Set by

function "x_set_splitscrn". The value is only valid if a split

screen is active. See also global variable "SplitScrnActive".

SplitScrnScanLine - WORD - Screen Scan Line the Split Screen starts at

initially when set by function "x_set_splitscrn". The value is only

valid if a split screen is active. See also global variable

"SplitScrnActive".This variable is not updated by "x_hide_splitscrn",

"x_adjust_splitscrn".

SplitScrnVisibleHeight - WORD - The number of rows of the initial split

screen which are currently displayed. Modified by "x_hide_splitscrn",

"x_adjust_splitscrn" and "x_show_splitscrn".

Page0_Offs - WORD - Offset in video ram of main virtual screen. Initially

set by function "x_set_mode" but is updated by functions

"x_set_splitscrn" and "x_set_doublebuffer".

Page1_Offs - WORD - Offset in video ram of second virtual screen. Set by

and only is valid after a call to "x_set_doublebuffer".

ScrnLogicalByteWidth - WORD - Virtual screen width in bytes. Set by

function "x_set_mode".

ScrnLogicalPixelWidth - WORD - Virtual screen width in pixels. Set

by function "x_set_mode".

ScrnLogicalHeight - WORD - Virtual screen height in pixels. Set

initially by function "x_set_mode" but is updated by functions

"x_set_splitscrn" and "x_set_doublebuffer".

MaxScrollX - WORD - Max X pixel position of physical screen within

virtual screen. Set by function "x_set_mode".

MaxScrollY - WORD - Max Y position of physical screen within virtual

screen. Set initially by function "x_set_mode" but is updated by

functions "x_set_splitscrn" and "x_set_doublebuffer".

DoubleBufferActive - WORD - Indicates whether double-buffering is on. Set

by function "x_set_doublebuffer".

VisiblePageIdx - WORD - Index number of current visible page. Initially

set by function "x_set_doublebuffer" but is updated by "x_page_flip".

This variable is only used while double buffering is on.

HiddenPageOffs - WORD - Offset of hidden page. Initially set by function

"x_set_doublebuffer" but is updated by "x_page_flip". This variable

is only used while double buffering is on.

VisiblePageOffs - WORD - Offset of visible page. Initially set by function

"x_set_doublebuffer" but is updated by "x_page_flip". This variable

is only used while double buffering is on.

NonVisual_Offs - WORD - Offset of first byte of non-visual ram, the ram

that is available for bitmap storage etc. Set initially by function

"x_set_mode" but is updated by functions "x_set_splitscrn" and

"x_set_doublebuffer".

EXPORTED FUNCTIONS

x_set_mode

----------

C Prototype:  extern WORD x_set_mode(WORD mode,WORD WidthInPixels);

mode          - The required mode as defined by the "Available X Mode

resolutions" set of defines in the xlib.h header file.

WidthInPixels - The required virtual screen width.

Returns       - The actual width in pixels of the allocated virtual

screen

This function initialises the graphics system, setting the apropriate

screen resolution and allocating a virtual screen. The virtual screen

allocated may not necessarily be of the same size as specified in the

"WidthInPixels" parameter as it is rounded down to the nearest

multiple of 4.

The function returns the actual width of the allocated virtual screen

in pixels if a valid mode was selected otherwise returns

X_MODE_INVALID.

Saves virtual screen pixel width in "ScrnLogicalPixelWidth".

Saves virtual screen byte  width in "ScrnLogicalByteWidth".

Physical screen dimensions are set in "ScrnPhysicalPixelWidth".

"ScrnPhysicalByteWidth" and "ScrnPhysicalHeight". Other global

variables set are "CurrXMode","MaxScrollX", "MaxScrollY",

"InGraphics". The variable "SplitScrnScanline" is also initialized

to zero.

See also:

Available X Mode resolutions

What is Mode X

x_select_default_plane

----------------------

C Prototype:  void x_select_default_plane(BYTE plane);

Enables default Read/Write access to a specified plane

x_set_splitscreen

-----------------

C Prototype:  extern void x_set_splitscreen(WORD line);

line - The starting scan line of the required split screen.

This function activates Mode X split screen and sets starting scan

line. The split screen resides on the bottom half of the screen and has

a starting address of A000:0000 in video RAM.

It also Updates Page0_Offs to reflect the existence of the split screen

region ie "MainScrnOffset" is set to the offset of the first pixel

beyond the split screen region. Other variable set are "Page1_Offs" which

is set to the same value as "Page0_Offs" (see graphics call sequence

below), "ScrnLogicalHeight","ScrnPhysicalHeight", "SplitScrnScanLine" and

"MaxScrollY".

This function cannot be called after double buffering has been activated,

it will return an error. To configure your graphics environment the

sequence of graphics calls is as follows although either or both steps b

and c may be omitted:

a) x_set_mode

b) x_set_splitscreen

c) x_set_doublebuffer

Thus when you call this function successfully, double buffering is not

active so "Page1_Offs" is set to the same address as "Page0_Offs".

WARNING: If you use one of the high resolution modes (360x480 as an

extreme example) you may not have enough video ram for split screen

and double buffering options since VGA video RAM is restricted to

64K.

See Also:

What is a Split Screen ?

What is double buffering ?

x_set_doublebuffer

------------------

C Prototype: extern WORD x_set_doublebuffer(WORD PageHeight);

PageHeight - The height of the two double buffering virtual screens.

Returns    - The closest possible height to the specified.

This function sets up two double buffering virtual pages. 'ErrorValue"

is set according to the success or failure of this command.

Other variables set are:

_Page1_Offs	       Offset of second virtual page

_NonVisual_Offs          Offset of first non visible video ram byte

_DoubleBufferActive      Flag

_PageAddrTable           Table of Double buffering pages start offsets

_ScrnLogicalHeight       Logical height of the double buffering pages

_MaxScrollY              Max vertical start address of physical screen

within the virtual screen

WARNING: If you use one of the high resolution modes (360x480 as an

extreme example) you may not have enough video ram for split screen

and double buffering options since VGA video RAM is restricted to

64K.

See Also:

What is double buffering ?

x_hide_splitscreen

------------------

C Prototype: extern void x_hide_splitscreen(void);

This function hides an existing split screen by setting its starting

scan line to the last physical screen scan line.

"ScreenPhysicalHeight" is adjusted but the "SplitScreenScanLine" is not

altered as it is required for restoring the split screen at a later stage.

WARNING: Only to be used if SplitScrnLine has been previously called

Disabled for mode 4-7 (320x400-360x480). The memory for

the initial split screen is reserved and the size limitations

of these modes means any change in the split screen scan line

will encroach on the split screen ram

Update: Now disabled for these modes

See Also:

What is a split screen ?

x_show_splitscreen

------------------

C Prototype: extern void x_show_splitscreen(void);

Restores split screen start scan line to the initial split screen

starting scan line as set by "SplitScrnScanLine".

"ScreenPhysicalHeight" is adjusted.

WARNING: Only to be used if SplitScrnLine has been previously called

Disabled for mode 4-7 (320x400-360x480). The memory for

the initial split screen is reserved and the size limitations

of these modes means any change in the split screen scan line

will encroach on the split screen ram

x_adjust_splitscreen

--------------------

C Prototype: extern void x_adjust_splitscreen(WORD line);

line - The scan line at which the split screen is to start.

Sets the split screen start scan line to a new scan line. Valid scan lines

are between the initial split screen starting scan line and the last

physical screen scan line. "ScreenPhysicalHeight" is also adjusted.

WARNING: Only to be used if SplitScrnLine has been previously called

Disabled for mode 4-7 (320x400-360x480). The memory for

the initial split screen is reserved and the size limitations

of these modes means any change in the split screen scan line

will encroach on the split screen ram

x_set_start_addr

----------------

C Prototype: extern void x_set_start_addr(WORD X,WORD Y);

X,Y - coordinates of top left corner of physical screen within current

virtual screen.

Set Mode X non split screen physical start address within current virtual

page.

X must not exceed (Logical screen width - Physical screen width)

ie "MaxScrollX" and Y must not exceed (Logical screen height -

Physical screen height) ie "MaxScrollY"

x_page_flip

-----------

C Prototype: extern void x_page_flip(WORD X,WORD Y);

X,Y - coordinates of top left corner of physical screen within the

the hidden virtual screen if double buffering is active, or

the current virtual screen otherwise.

Sets the physical screen start address within currently hidden virtual

page and then flips pages. If double buffering is not active then this

function is functionally equivalent to "x_set_start_addr".

X must not exceed (Logical screen width - Physical screen width)

ie "MaxScrollX" and Y must not exceed (Logical screen height -

Physical screen height) ie "MaxScrollY"

x_text_mode

-----------

C Prototype: extern void x_text_mode(void);

Disables graphics mode.

--------------------------------------------------------------------------

MODULE XPOINT

--------------------------------------------------------------------------

Point functions all MODE X 256 Color resolutions

ASM SOURCES

xpoint.asm xpoint.inc xlib.inc model.inc

C HEADER FILE

xpoint.h

EXPORTED FUNCTIONS

x_put_pix

---------

C Prototype: extern void x_put_pix(WORD X,WORD Y,WORD PageOffset,

WORD Color);

Draw a point of specified colour at coordinates X,Y

within the virtual page starting at offset PageOffset.

x_get_pix

---------

C Prototype: extern WORD x_get_pix(WORD X, WORD Y, WORD PageBase);

Read a point of at coordinates X,Y within the virtual page starting

at offset PageOffset.

--------------------------------------------------------------------------

MODULE XRECT

--------------------------------------------------------------------------

Screen rectangle display and manipulation functions

ASM SOURCES

xrect.asm xrect.inc xlib.inc model.inc

C HEADER FILE

xrect.h

EXPORTED FUNCTIONS

x_rect_pattern

--------------

C Prototype: extern void x_rect_pattern(WORD StartX, WORD StartY,

WORD EndX, WORD EndY,

WORD PageBase,BYTE far *Pattern);

StartX,StartY - Coordinates of upper left hand corner of rectangle

EndX,EndY     - Coordinates of lower right hand corner of rectangle

PageBase      - Offset of virtual screen

*Pattern      - Pointer to the user defined pattern (16 bytes)

Mode X rectangle 4x4 pattern fill routine.

Upper left corner of pattern is always aligned to a multiple-of-4

row and column. Works on all VGAs. Uses approach of copying the

pattern to off-screen display memory, then loading the latches with

the pattern for each scan line and filling each scan line four

pixels at a time. Fills up to but not including the column at EndX

and the row at EndY. No clipping is performed.

Based on code originally published in DDJ Mag by M. Abrash

Warning the VGA memory locations PATTERN_BUFFER (A000:FFFc) to

A000:FFFF are reserved for the pattern buffer

See Also:

Doctor Dobbs Journal references.

x_rect_fill

------------

C Prototype: extern void x_rect_fill(WORD StartX,WORD StartY,

WORD EndX,WORD EndY,

WORD PageBase,WORD color);

StartX,StartY - Coordinates of upper left hand corner of rectangle

EndX,EndY - Coordinates of lower right hand corner of rectangle

PageBase - Offset of virtual screen

Color -color to use for fill

Mode X rectangle solid color fill routine.

Based on code originally published in DDJ Mag by M. Abrash

See Also:

Doctor Dobbs Journal references.

x_cp_vid_rect

--------------

C Prototype: extern void x_cp_vid_rect(WORD SourceStartX,WORD SourceStartY,

WORD SourceEndX,WORD SourceEndY,

WORD DestStartX,WORD DestStartY,

WORD SourcePageBase,WORD DestPageBase,

WORD SourceBitmapWidth,WORD DestBitmapWidth);

StartX,StartY- Coordinates of upper left hand corner of source rectangle

EndX,EndY    - Coordinates of lower right hand corner of source rectangle

DestStartX,DestStartY - Coordinates of rectangle destination

SourcePageBase        - source rectangle page offset

DestPageBase          - destination rectangles page offset

SourceBitmapWidth     - width of bitmap within the source virtual screen

containing the source rectangle

DestBitmapWidth       - width of bitmap within the dest. virtual screen

containing the destination rectangle

Mode X display memory to display memory copy

routine. Left edge of source rectangle modulo 4 must equal left edge

of destination rectangle modulo 4. Works on all VGAs. Uses approach

of reading 4 pixels at a time from the source into the latches, then

writing the latches to the destination. Copies up to but not

including the column at SrcEndX and the row at SrcEndY. No

clipping is performed. Results are not guaranteed if the source and

destination overlap.

Based on code originally published in DDJ Mag by M. Abrash

See Also:

Doctor Dobbs Journal references.

---------------------------------------------------------------------------

MODULE XPAL

---------------------------------------------------------------------------

Palette functions for VGA 256 color modes.

All the functions in this module operate on two variations of the

pallete buffer, the raw and annotated buffers.

All those functions ending in "raw" operate on the following palette

structure:

BYTE:r0,g0,b0,r1,g1,b1,...rn,gn,bn

No reference to the starting colour index or number of colours stored

is contained in the structure.

All those functions ending in "struc" operate on the following palette

structure:

BYTE:c,BYTE:n,BYTE:r0,g0,b0,r1,g1,b1,...rn,gn,bn

where c is the starting colour and n is the number of colours stored

WARNING: There is no validity checking in these functions. The onus is

on the user to supply valid parameters to the functions.

ASM SOURCES

xpal.asm xpal.inc xlib.inc model.inc

C HEADER FILE:

xpal.h

EXPORTED FUNCTIONS

x_get_pal_raw

-------------

C Prototype: extern void x_get_pal_raw(BYTE far * pal,WORD num_colrs,

WORD start_index);

Read DAC palette into raw buffer with interrupts disabled

ie BYTE r1,g1,b1,r1,g2,b2...rn,gn,bn

WARNING: Memory for the palette buffers must all be pre-allocated.

x_get_pal_struc

---------------

C Prototype: extern void x_get_pal_struc(BYTE far * pal,WORD num_colrs,

WORD start_index);

Read DAC palette into annotated type buffer with interrupts disabled

ie BYTE colours to skip, BYTE colours to set, r1,g1,b1,r1,g2,b2...rn,gn,bn

WARNING: memory for the palette buffers must all be pre-allocated

x_put_pal_raw

-------------

C Prototype: extern void x_put_pal_raw(BYTE far * pal,WORD num_colrs,

WORD start_index);

Write DAC palette from raw buffer with interrupts disabled

ie BYTE r1,g1,b1,r1,g2,b2...rn,gn,bn

x_put_pal_struc

--------------

C Prototype: extern void x_put_pal_struc(BYTE far * pal);

Write DAC palette from annotated type buffer with interrupts disabled

ie BYTE colours to skip, BYTE colours to set, r1,g1,b1,r1,g2,b2...rn,gn,bn

x_set_rgb

---------

C Prototype: extern x_set_rgb(BYTE color,BYTE red_c,BYTE green_c,

BYTE blue_c);

Set the RGB components of a vga color

x_rot_pal_struc

---------------

C Prototype: extern void x_rot_pal_struc(BYTE far * pal,WORD direction);

Rotate annotated palette buffer entries. Direction 0 = backward,

1 = forward.

x_rot_pal_raw

-------------

C Prototype: extern x_rot_pal_raw(BYTE far * pal,WORD direction,

WORD num_colrs);

Rotate a raw palette buffer. Direction 0 = backward,

1 = forward.

x_put_contrast_pal_struc

------------------------

C Prototype: extern  void x_put_contrast_pal_struc(BYTE far * pal,

BYTE  intensity);

Write DAC palette from annotated type buffer with specified intensity

adjustment (ie palette entries are decremented where possible by

"intensity" units).

Designed for fading in or out a palette without using an intermediate

working palette buffer ! (Slow but memory efficient ... OK for small

pal strucs}

x_transpose_pal_struc

---------------------

C Prototype: extern void x_transpose_pal_struc(BYTE far * pal,

WORD StartColor);

Write DAC palette from annotated type buffer with interrupts disabled

starting at a new palette index.

x_cpcontrast_pal_struc

----------------------

C Prototype: extern WORD x_cpcontrast_pal_struc(BYTE far *src_pal,

BYTE far *dest_pal,BYTE Intensity);

Copy one annotated palette buffer to another making the intensity

adjustment. Used in fading in and out fast and smoothly.

---------------------------------------------------------------------------

MODULE XLINE

---------------------------------------------------------------------------

Line Drawing functions.

ASM SOURCES

xline.asm xline.inc xlib.inc model.inc

C HEADER FILE

xline.h

EXPORTED FUNCTIONS

x_line

------

C Prototype: extern void x_line(WORD x0,WORD y0,WORD x1,WORD y1,

WORD color,WORD PageBase);

Draw a line with the specified end points in the page starting at

offset "PageBase".

No Clipping is performed.

---------------------------------------------------------------------------

MODULE XTEXT

---------------------------------------------------------------------------

ASM SOURCES

xtext.asm xtext.inc xlib.inc model.inc

C HEADER FILE

xtext.h

DEFINES

FONT_8x8  0

FONT_8x15 1

FONT_USER 2

EXPORTED VARIABLES

NOTE: All variables are read only. I you modify them the results may

be unpredictable.

CharHeight - BYTE - Height of current inbuilt character set

CharWidth  - BYTE - Width of current inbuilt character set

FirstChar  - BYTE - First character of current inbuilt character set

UserCharHeight - BYTE - Height of current user character set

UserCharWidth - BYTE - Width of current user character set

UserFirstCh - BYTE - First character of current user character set

EXPORTED FUNCTIONS

x_text_init

-----------

C Prototype: extern WORD x_text_init(void);

Initializes the Mode X text driver and sets the default font (VGA ROM 8x8)

x_set_font

----------

C Prototype: extern void x_set_font(WORD FontId);

Select the working font where 0 = VGA ROM 8x8, 1 = VGA ROM 8x14

2 = User defined bitmapped font.

WARNING: A user font must be registered before setting FontID 2

See Also:

Defines for this module

x_register_userfont

-------------------

C Prototype: extern void x_register_userfont(char far *UserFontPtr);

Register a user font for later selection. Only one user font can be

registered at any given time. Registering a user font deregisters the

previous user font. User fonts may be at most 8 pixels wide.

USER FONT STRUCTURE

Word:  ascii code of first char in font

Byte:  Height of chars in font

Byte:  Width of chars in font

n*h*Byte: the font data where n = number of chars and h = height

of chars

WARNING: The onus is on the program to ensure that all characters

drawn whilst this font is active, are within the range of

characters defined.

x_put_char

----------

C Prototype: extern void x_put_char(char ch,WORD X,WORD Y,WORD PgOffs,

WORD Color);

Draw a text character at the specified location with the specified

color.

ch       -  char to draw

x,y      -  screen coords at which to draw ch

ScrnOffs -  Starting offset of page on whih to draw

Color    -  Color of the text

WARNING: InitTextDriver must be called before using this function

**** NOTE ****

The file "xprintf.c" implements a printf style formatted output function

x_printf

--------

C Prototype: void x_printf(int x,int y,unsigned ScrnOffs,int color,

char *ln,...);

x,y      -  screen coords at which to draw ch

ScrnOffs -  Starting offset of page on whih to draw

Color    -  Color of the text

Parameters beyond Color conform to the standard printf parameters.

x_bgprintf

----------

C Prototype: void x_bgprintf(int x,int y,unsigned ScrnOffs,int fgcolor,

int bgcolor, char *ln,...);

x,y      -  screen coords at which to draw ch

ScrnOffs -  Starting offset of page on whih to draw

fgcolor    -  Color of the text foreground

bgcolor    -  Color of the text background

Parameters beyond bgolor conform to the standard printf parameters.

--------------------------------------------------------------------------

MODULE XPBITMAP

--------------------------------------------------------------------------

This module implements a set of functions to operate on planar bitmaps.

Planar bitmaps as used by these functions have the following structure:

BYTE 0                 The bitmap width in bytes (4 pixel groups) range 1..255

BYTE 1                 The bitmap height in rows range 1..255

BYTE 2..n1             The plane 0 pixels width*height bytes

BYTE n1..n2            The plane 1 pixels width*height bytes

BYTE n2..n3            The plane 2 pixels width*height bytes

BYTE n3..n4            The plane 3 pixels width*height bytes

These functions provide the fastest possible bitmap blts from system ram to

to video and further, the single bitmap is applicable to all pixel

allignments. The masked functions do not need separate masks since all non

zero pixels are considered to be masking pixels, hence if a pixel is 0 the

corresponding screen destination pixel is left unchanged.

ASM SOURCES

xpbitmap.asm xpbitmap.inc xlib.inc model.inc

C HEADER FILE

xpbitmap.h

EXPORT FUNCTIONS

x_put_masked_pbm

----------------

C Prototype: extern void x_put_masked_pbm(WORD X,WORD Y,WORD ScrnOffs,

BYTE far * Bitmap);

Mask write a planar bitmap from system ram to video ram. All zero source

bitmap bytes indicate destination byte to be left unchanged.

Source Bitmap structure:

Width:byte, Height:byte, Bitmap data (plane 0)...Bitmap data (plane 1)..,

Bitmap data (plane 2)..,Bitmap data (plane 3)..

NOTE: width is in bytes ie lots of 4 pixels

LIMITATIONS: No clipping is supported

Only supports bitmaps with widths which are a multiple of

4 pixels

See Also: XBMTOOLS module for linear <-> planar bitmap conversion

functions.

x_put_pbm

---------

C Prototype: extern void x_put_pbm(WORD X, WORD Y, WORD ScrnOffs,

BYTE far * Bitmap);

Write a planar bitmap from system ram to video ram.

Source Bitmap structure:

Width:byte, Height:byte, Bitmap data (plane 0)...Bitmap data (plane 1)..,

Bitmap data (plane 2)..,Bitmap data (plane 3)..

NOTE: width is in bytes ie lots of 4 pixels

LIMITATIONS: No clipping is supported

Only supports bitmaps with widths which are a multiple of

4 pixels

FEATURES   : Automatically selects REP MOVSB or REP MOVSW  depending on

source bitmap width, by modifying opcode ;-).

See Also: XBMTOOLS module for linear <-> planar bitmap conversion

functions.

x_get_pbm

---------

C Prototype: extern void x_get_pbm(WORD X, WORD Y,BYTE Bw,BYTE Bh,

WORD ScrnOffs, BYTE far * Bitmap);

Read a planar bitmap to system ram from video ram.

Source Bitmap structure:

Width:byte, Height:byte, Bitmap data (plane 0)...Bitmap data (plane 1)..,

Bitmap data (plane 2)..,Bitmap data (plane 3)..

NOTE: width is in bytes ie lots of 4 pixels

LIMITATIONS: No clipping is supported

Only supports bitmaps with widths which are a multiple of

4 pixels

FEATURES   : Automatically selects REP MOVSB or REP MOVSW  depending on

source bitmap width, by modifying opcode ;-).

See Also: XBMTOOLS module for linear <-> planar bitmap conversion

functions.

--------------------------------------------------------------------------

MODULE  XCBITMAP

--------------------------------------------------------------------------

XCBITMAP:

The Care and Feeding of Compiled Masked Blits

by Matthew MacKenzie

This file describes the XCBITMAP module of the Xlib library, made up of

the files XCBITMAP.ASM, XCBITMAP.INC, XCBITMAP.H, and this file, XCBITMAP.DOC.

The XCBITMAP module is small, containing only two procedures:

o  x_compile_bitmap compiles your bitmap into native code which writes

to the VGA screen in an X mode.

o  x_put_cbitmap converts X and Y coordinates into a location on the

screen, sets up the necessary VGA registers, and executes the compiled

bitmap as a subroutine.

x_compile_bitmap scans through a source bitmap and generates 8086

instructions to plot every nonzero pixel.  It is designed to be used

before the action begins rather than on-the-fly.  The compiled bitmap

contains no branches, and no reference to the zero (transparent) pixels.

Where two pixels are exactly four columns apart they are plotted with a

single 16-bit store, and the VGA MAP_MASK register will be set at most

four times.  As a result your bitmap may run several times faster than a

traditional memory-to-VGA masked blit routine.

There is no way to perform clipping on these bitmaps, or to plot a

pixel of color zero.

x_compile_bitmap works with bitmaps in a very simple format, and does

not require the bitmap's width or height to be multiples of four, eight,

or even two.  While this makes it easier for us humans to design new

bitmaps, it does make the compilation relatively slow, on a time scale of

60 frames per second.  Since a compiled bitmap is relocatable you may

just want to have it saved to disk, and not include the source bitmap

in your program at all.

The source bitmap format is an array of bytes, like this:

char far * eye ={

13,7,

0, 0, 0, 0, 9, 1, 1, 1, 9, 0, 0, 0, 0,

0, 0, 9, 9, 1, 1, 1, 4, 4, 9, 9, 0, 0,

0, 9, 9, 1, 2, 0, 0, 4, 4, 1, 9, 9, 0,

9, 9, 9, 1, 0, 0, 0, 0, 1, 1, 9, 9, 9,

0, 9, 9, 1, 2, 0, 0, 2, 1, 1, 9, 9, 0,

0, 0, 9, 9, 1, 1, 1, 1, 1, 9, 9, 0, 0,

0, 0, 0, 0, 9, 1, 1, 1, 9, 0, 0, 0, 0};

To compile this image for a mode 360 pixels (90 byte columns) across:

EyeSize = x_compile_bitmap(90, eye, CompiledEye);

Both `eye' and `CompiledEye' are of type (char far *).  Notice that

both buffers must exist beforehand.  Since x_compile_bitmap returns the

size of the compiled code, in bytes, you can reallocate the bitmap

immediately to the right size.  The pointers are 32-bit because

compiled bitmaps take so much space: they are at one end of the speed-

versus-memory spectrum. You may, of course, still use a 16-bit memory

model, or your own allocation scheme, and cast the pointers to 32 bits

to compile or blit a bitmap.  A good rule of thumb is to allocate

(3.5 x buffer-height x buffer-width) + 25 bytes  (rounding up ;-),

then pare your bitmap down when you find out how much space you've

actually used.

Since the compiled bitmap has to fit within one segment of memory, it

cannot contain more than about 19,000 pixels.  This will not be a

limitation for most sane programmers.  If you are not a sane programmer try

splitting your huge, unwieldy image up into smaller parts -- you can use

the same gigantic bitmap if you divide it into horizontal slices for

compilation.  For that matter, dividing the source up that way will let

you use a source bitmap large than 64K, which is an even sicker idea...

Back to business.  A bitmap is compiled for only one width of screen.

If you are using a logical screen larger than your physical screen, call

the bitmap compiler with the logical width -- the important thing is the

number of bytes per line.  Notice that you do not have to be in a graphics

mode to use this routine.  This allows you to develop and compile bitmaps

separately, with whatever utility programs you might cook up.

The second function is x_put_cbitmap.  To plot our eye at (99,4), on

the page which starts at location 0:

x_put_cbitmap(99, 4, 0, CompiledEye);

This function depends on the global variable ScrnLogicalByteWidth from

the module XMAIN, which should be the same number as the column parameter

you used to compile your bitmap.

The XCBITMAP module supports memory-to-VGA blits only.  Xlib also

includes non-masking routines which can quickly save and restore the

background screen behind your bitmap, using fast string operations.

This module is part of the Xlib package, and is in the public domain.

If you write something which uses it, though, please send me a copy as a

courtesy -- if for no other reason so I can tilt my chair back and reflect

that it may have been worth the trouble after all.

The included program DEMO2.C demonstarates the performance difference

between planar bitmap masked blits and compiled bitmap blits.

ASM SOURCES

xcbitmap.asm xcbitmap.inc xlib.inc model.inc

C HEADER FILE

xcbitmap.h

EXPORT FUNCTIONS

x_compile_bitmap

----------------

C Prototype: extern int x_compile_bitmap (WORD scrn_logical_width,

char far * bitmap,char far * output);

Compile a linear bitmap to generate machine code to plot it

at any required screen coordinates FAST. Faster than blits

using planar bitmaps as in module XPBIITMAP

The screen logical width is in bytes rather than pixels.

The source linear bitmaps have the following structure:

BYTE 0                 The bitmap width in pixels  range 1..255

BYTE 1                 The bitmap height in rows   range 1..255

BYTE 2..n              The width*height bytes of the bitmap

All four main registers are totaled.

See Also: XBMTOOLS module for linear <-> planar bitmap conversion

functions.

x_put_cbitmap

-------------

C Prototype: extern void x_put_cbitmap (WORD XPos,WORD YPos,

WORD PageOffset,char far * CompiledBitmap);

XPos,YPos   -  screen coords at which to draw bitmap

PageOffs    -  Starting offset of page on whih to draw

CompiledBitmap - pointer to compiled bitmap

Displays a compiled bitmap generated by x_compile_bitmap at given

coordinates, on a given screen page.

See Also: XBMTOOLS module for linear <-> planar bitmap conversion

functions.

--------------------------------------------------------------------------

MODULE XBMTOOLS

--------------------------------------------------------------------------

This module implements a set of functions to convert between planar

bitmaps and linear bitmaps.

PLANAR BITMAPS

Planar bitmaps as used by these functions have the following structure:

BYTE 0                 The bitmap width in bytes (4 pixel groups) range 1..255

BYTE 1                 The bitmap height in rows range 1..255

BYTE 2..n1             The plane 0 pixels width*height bytes

BYTE n1..n2            The plane 1 pixels width*height bytes

BYTE n2..n3            The plane 2 pixels width*height bytes

BYTE n3..n4            The plane 3 pixels width*height bytes

as used by x_put_pbm, x_get_pbm, x_put_masked_pbm.

LINEAR BITMAPS

Linear bitmaps have the following structure:

BYTE 0                 The bitmap width in pixels  range 1..255

BYTE 1                 The bitmap height in rows   range 1..255

BYTE 2..n              The width*height bytes of the bitmap

ASM SOURCES

xbmtools.asm xpbmtools.inc model.inc

C HEADER FILE

xbmtools.h

EXPORT DEFINES

BM_WIDTH_ERROR

EXPORT FUNCTIONS

x_pbm_to_bm

------------

C Prototype:  extern int x_pbm_to_bm(char far * source_pbm,

char far * dest_bm);

This function converts a bitmap in the planar format to the linear format

as used by x_compile_bitmap.

WARNING: the source and destination bitmaps must be pre - allocated

NOTE: This function can only convert planar bitmaps that are suitable.

If the source planar bitmap's width (per plane) is >= 256/4

it cannot be converted. In this situation an error code

BM_WIDTH_ERROR. On successful conversion 0 is returned.

x_bm_to_pbm

------------

C Prototype:  extern int x_bm_to_pbm(char far * source_pbm,

char far * dest_bm);

This function converts a bitmap in the linear format as used by

x_compile_bitmap to the planar formap.

WARNING: the source and destination bitmaps must be pre - allocated

NOTE: This function can only convert linear bitmaps that are suitable.

If the source linear bitmap's width is not a multiple of 4

it cannot be converted. In this situation an error code

BM_WIDTH_ERROR. On successful conversion 0 is returned.

--------------------------------------------------------------------------

MODULE XDETECT

--------------------------------------------------------------------------

This module implements a set of functions to detect the PC's hardware

configuration.

ASM SOURCES

xdetect.asm xdetect.inc model.inc

C HEADER FILE

xdetect.h

EXPORTED DEFINES

I8086   0

I80186  1

I80286  2

I80386  3

NoGraphics 0

MDA        1

CGA        2

EGAMono    3

EGAColor   4

VGAMono    5

VGAColor   6

MCGAMono   7

MCGAColor  8

EXPORT FUNCTIONS

x_graphics_card

---------------

C Prototype: extern int x_graphics_card();

This function returns the type of graphics card installed. See defines

above.

x_processor

-----------

C Prototype: extern int x_processor();

This function returns the type of processor installed. A 486 registers

as a 386. See defines above.

--------------------------------------------------------------------------

MODULE XFILEIO

--------------------------------------------------------------------------

Handle based file I/O functions.

See any good DOS programming reference for more information on int 21h

DOS services.

ASM SOURCES

xfileio.asm xfileio.inc model.inc

C HEADER FILE

xfileio.h

EXPORTED DEFINES

file access modes

F_RDONLY

F_WRONLY

F_RDWR

seek codes

SEEK_START

SEEK_CURR

SEEK_END

file error value

FILE_ERR

EXPORT FUNCTIONS

f_open

------

C Prototype: extern int f_open(char * filename, char access);

Opens a file according to the access char:

F_RDONLY = read only   - If doesnt exist return error

F_WRONLY = write only  - If doesnt exist create it otherwise clear it

F_RDWR   = read/write  - If doesnt exist create it

Returns the file handle on success, FILE_ERR on failure

f_close

-------

C Prototype:  extern int f_close(int handle);

Closes the file associated with the specified handle

Returns 0 on success, FILE_ERR on failure

f_read

------

C Prototype:  extern int f_read(int handle, char far * buffer, int count);

Reads a block of count bytes from the file specified by the handle

into the buffer

Returns count on success, FILE_ERR on failure

f_write

-------

C Prototype: extern int f_write(int handle, char far * buffer, int count);

Writes a block of count bytes to the file specified by the handle

from the buffer

Returns count on success, FILE_ERR on failure

f_seek

------

C Prototype: extern long int f_seek(int handle, long int position,

char method_code)

Moves the file pointer according to the position and method code

Returns file pointer position on success, FILE_ERR on failure

f_filelength

------------

C Prototype:

extern long int f_filelength(int handle)

Returns the length of the file associated with the specified handle

Returns file length on success, FILE_ERR on failure

--------------------------------------------------------------------------

MODULE XRLETOOL

--------------------------------------------------------------------------

This module implements a number of functions comprising an RLE encoding

decoding system.

RLE stands for RUN LENGTH ENCODING. It is a quick simple data compression

scheme which is commonly used for image data compression or compression

of any data. Although not the most efficient system, it is fast, which is

why it is used in image storage systems like PCX. This implementation is

more efficient than the one used in PCX files because it uses 1 bit to

identify a Run Length byte as opposed to two in PCX files, but more on this

later.

This set of functions can be used to implement your own compressed image

file format or for example compress game mapse for various levels etc.

The uses are limited by your imagination.

I opted for trading off PCX RLE compatibility for the improved compression

efficiency.

Here is how the data is un-compressed to give an idea of its structure.

STEP 1 read a byte from the RLE compressed source buffer.

STEP 2 if has its high bit is set then the lower 7 bits represent the number

of times the next byte is to be repeated in the destination buffer.

if the count (lower 7 bits) is zero then

we have finished decoding goto STEP 5

else goto STEP 4

STEP 3 Read a data from the source buffer and copy it directly to the

destination buffer.

goto STEP 1

STEP 4 Read a data from the source buffer and copy it to the destination

buffer the number of times specified by step 2.

goto STEP 1

STEP 5 Stop, decoding done.

If the byte does not have the high bit set then the byte itself is transfered

to the destination buffer.

Data bytes that have the high bit already set and are unique in the input

stream are represented as a Run Length of 1 (ie 81 which includes high bit)

followed by the data byte.

If your original uncompressed data contains few consecutive bytes and most

have high bit set (ie have values > 127) then your so called

compressed data would require up to 2x the space of the uncompressed data,

so be aware that the compression ratio is extremely variable depending on the

type of data being compressed.

Apologies for this poor attempt at a description, but you can look up

RLE in any good text. Alternatively, any text that describes the PCX file

structure in any depth should have a section on RLE compression.

ASM SOURCES

xrletool.asm xrletool.inc model.inc

C HEADER FILE

xrletool.h

EXPORTED DEFINES

EXPORT FUNCTIONS

x_buff_RLDecode

---------------

Expands an RLE compresses source buffer to a destination buffer.

returns the size of the resultant uncompressed data.

C PROTOTYPE:

extern unsigned int x_buff_RLDecode(char far * source_buff,

char far * dest_buff);

source_buff   - The buffer to compress

dest_buff     - The destination buffer

WARNING: buffers must be pre allocated.

x_buff_RLEncode

---------------

RLE Compresses a source buffer to a destination buffer and returns

the size of the resultant compressed data.

C PROTOTYPE:

extern unsigned int x_buff_RLEncode(char far * source_buff,

char far * dest_buff,unsigned int count);

source_buff   - The buffer to compress

dest_buff     - The destination buffer

count         - The size of the source data in bytes

WARNING: buffers must be pre allocated.

x_buff_RLE_size

---------------

Returns the size the input data would compress to.

C PROTOTYPE:

extern unsigned int x_buff_RLE_size(char far * source_buff,

unsigned int count);

source_buff   - The uncompressed data buffer

count         - The size of the source data in bytes

x_file_RLEncode

---------------

RLE Compresses a source buffer to an output file returning

the size of the resultant compressed data or 0 if it fails.

C PROTOTYPE:

extern unsigned int x_file_RLEncode(int handle,

char far * source_buff,unsigned int count);

source_buff   - The buffer to compress

handle        - The file handler

count         - The size of the source data in bytes

x_file_RLDecode

---------------

Expands an RLE compresses file to a destination RAM buffer.

returns the size of the resultant uncompressed data.

C PROTOTYPE:

extern unsigned int x_buff_RLDecode(int handle,

char far * dest_buff);

handle        - Input file handle

dest_buff     - The destination buffer

--------------------------------------------------------------------

REFERENCE SECTION

--------------------------------------------------------------------

REFERENCES

----------

In my opinion Doctor Dobbs Journal is the best reference text for

VGA Mode X graphics:

Issue 178 Jul 1991 : First reference to Mode X

Article Abstract   : VGA's undocumented Mode X supports page flipping,

makes off screen memory available, has square pixels,

and increases performance by as muck as 4 times.

Issue 179 Aug 1991 : Continuation

Article Abstract   : Michael discusses latches and VGA's undoccumented

Mode X.

Issue 181 Sep 1991 : Continuation

Article Abstract   : Michael puts the moves on animation using VGA's 256

colors.

Issue 184 Oct 1991 : First of a continuing series covering 3-D animation

using VGA's Mode X. This series is still ongoing

(October 1992)

Article Abstract   : Michael moves into 3-D animation, starting with basic

polygon fills and page flips.

WHAT IS MODE X ?

----------------

Mode X is a derrivative of the VGA's standard mode 13h (320x200 256 color).

It is a (family) of undocumented video modes that are created by tweaking

the VGA's registers. The beauty of mode X is that it offers several

benefits to the programmer:

- Multiple graphice pages where mode 13h doesn't allowing for page flipping

(also known as double buffering) and storage of images and data in

offscreen video memory

- A planar video ram organization which although more difficult to program,

allows the VGA's plane-oriented hardware to be used to process pixels in

parallel, improving performance by up to 4 times over mode 13h

See issue 178-179 of D.D.J. for a full description of VGA's Mode X.

WHAT IS A SPLIT SCREEN ?

------------------------

A split screen is a neat hardware feature offered by the EGA and VGA video

cards. A split screen is a mode of graphics operationin which the Hardware

splits the visual graphics screen horizontally and treats both halves as

individual screens each starting at different locations in video RAM.

The bottom half (which is usually referred to as the split screen) always

starts at address A000:0000 but the top half's starting address is user

definable.

The most common application of split screens in games is the status display

in scrolling games. Split screens make this sort of game simpler to program

because when the top half window is scrolled the programmer does not have to

worry about redrawing the bottom half.

WHAT IS DOUBLE BUFFERING ?

--------------------------

Double buffering (also known as page flipping) is the technique most often

used to do animation. it requires hardware that is capable of displaying

multiple graphics pages (or at least 2). Animation is achieved by drawing

an image in the non visible screen and then displaying the non visible

screen. Once te page has been flipped the process starts again. The next

frame of the animation is drawn on the non visible screen, the page is

flipped again etc.

iMode X is a derrivative of the VGA's standard mode 13h (320x200 256 color).

It is a (family) of undocumented video modes that are created by tweaking

the VGA's registers. The beauty of mode X is that it offers several

benefits to the programmer:

- Multiple graphice pages where mode 13h doesn't allowing for page flipping

(also known as double buffering) and storage of images and data in

offscreen video memory

- A planar video ram organization which although more difficult to program,

allows the VGA's plane-oriented hardware to be used to process pixels in

parallel, improving performance by up to 4 times over mode 13h

Again see D.D.J. for an in depth discussion of animation using Mode X