Upgrading your Central Station Laptop_To_Ethernet link

for use with LANtastic V5.0

------------------------------------------------------

(C) Copyright 1993 Artisoft, Inc.

(Edition 1.00)

INTRODUCTION

============

This file provides information to enable you to upgrade your Central Station

Laptop_To_Ethernet (LTE) link for use with LANtasitic V5.0.

Do not perform this upgrade if your network is not running LANtastic V5.0.

How to Determine Your Version of LANtastic

==========================================

If you are running DOS-based LANtastic, look for the version number at the

top of any menu.  If you are using LANtastic from the command line, look

for the version number in the sign-on message when you load the network

software.  If you are running LANtastic Windows Version, pull down the Help

menu from the NET or NET_MGR Main Functions menu, then choose About Net or

About Net Manager.

Upgrading your Central Station

==============================

To upgrade your software:

1.  Connect the Central Station to a PC (if it is not already).

Instructions for doing this appear in your StationWare for

LANtastic User's Manual.

2.  Make sure the PC is not running CSPPORT.  At the DOS prompt,

type:  CSPPORT/REMOVE

3.  Using a diskette containing the upgrade software place it

into your PC's floppy drive.  Type:

A: <Enter>

CSCONFIG <Enter>

where "A:" is the floppy drive in which you placed the diskette.

If you connected the Central Station to the computer's LPT2

port, type:

A: <Enter>

CSCONFIG PORT=LPT2 <Enter>

You can also copy the files from the StationWare diskette to

your PC's hard drive and upgrade your software from there.

4.  You may be prompted to reset the Central Station.  If the

Central Station power is on, turn it off for at least four

seconds, then turn it back on.

5.  From the CSCONFIG Main Functions menu, select Central Station

System Software <Enter>.

6.  Select Update System Software <Enter>.

7.  When the upgrade process is finished press Esc to exit to

the Main Functions menu, then press Esc again.

8.  You are asked if you would like to save the changes.  Press

Enter to select YES.

Your Central Station LTE system software is now upgraded

and ready for use on your LANtastic V5.0 network.

1993 Artisoft, Inc.  All rights reserved.  Central Station and StationWare

are trademarks, and Artisoft and LANtastic are registered trademarks of

Artisoft, Inc.  Brand names and product names are trademarks of their

respective companies.

Artisoft, Inc.

691 E. River Road

Tucson, AZ 85704 USA

Sales/Customer Service:  1-800-TINY-RAM

Technical Support:  1-602-293-6363

Headquarters:  1-602-293-4000

Fax:  602-293-8065

Edition 1.00