AI-LANBIOS Low-level Driver for the Intel 82593 MLM hardware and derivatives

(INTEL593.DOC)

----------------------------------------------------------------------------

(C) Copyright 1992 ARTISOFT, Inc.

Portions Copyright 1992-93 Intel Corporation

Introduction

============

The INTEL593 low-level AI-LANBIOS driver for Intel's "82593-MLM hardware

and derivatives" was developed by Intel Corp. in conformance with the

specifications outlined in the Artisoft AI-LANBIOS Driver Development Program.

The low-level driver interfaces directly to the network hardware and

provides basic unreliable data transfer capabilities.  The AI-LANBIOS

high-level driver provides the reliable data transfer capabilities

including all NETBIOS functions.

The INTEL593 driver makes use of two system DMA channels for data transfer

operations. It also requires memory from the system memory space for its

transmit and receive buffers. This approach does not require any memory on

the board.

NOTE:   As two DMA channels are used by one "82593-MLM hardware",

it is not possible to have more than one INTEL593 driver installed

in the same system.

Compatibility Statement

=======================

Version 3.10 of INTEL593 is compatible with Artisoft's LANtastic/AI Network

Operating System v4.1, except for the following functional considerations:

-INTEL593 driver requires two additional command line switches for its use

of system DMA channels.

-If a NETID Block is found on the computer in which the 82593 is installed,

the parameter values obtained from NETID Block will override the command

line switch values.

Vendor Information

==================

Intel Connectivity Division. 1900, Prairie City Road, MS FM4-27.

Folsom, CA-95630.  Fax# 916-356-6289 (attn: ICD Product Marketing)

Running INTEL593

================

The INTEL593 low-level driver must be run before the AI-LANBIOS.  The

INTEL593 driver interfaces to the AI-LANBIOS using a DOS multiplex

interrupt number.  The default multiplex number is C7.  If you are only

running one low-level driver and one copy of AI-LANBIOS then you will

not have to worry about which multiplex number to assign.  Multiplex

numbers 00 through BF are reserved for DOS use so you must use a multiplex

number between C0 through FF.

One invocation of the INTEL593 driver supports one "Intel 82593-MLM hardware

or derivative".  Multiple copies can not be run as enough DMA Channels are

not available.

The INTEL593 low-level driver program name is INTEL593.EXE.  The syntax for

INTEL593 is

INTEL593[switches . . . ] [ ; comment]

where [switches . . . ] denotes zero or more optional command line switches.

Switches may be delimited by blanks or slashes (/).  If a switch takes a

value, the switch must be followed by an equal sign (=) or a colon (:).

Some values may be hexadecimal numbers while other values may be decimal.

All switch values are range checked and illegal values produce errors.

You may place a comment on the command line by preceding the comment with

a semicolon (;).  You must precede the comment character with a blank.

The valid switch formats are:

SWITCH

SWITCH:value

/SWITCH

@file

The valid INTEL593 switches are enumerated below.  The letters "ddd"

denote a decimal number.  The letters "hh" denote a hexadecimal number.

Values enclosed in [] indicate default values if the switch is omitted.

Values after [] denote the valid range of the switch.

----------

@switch-file

This option specifies that further switches are to be taken from an

switch file.  Any switches after the @switch-file name will be used,

and the switch settings take precedence over any settings in the switch

file.  You may invoke switch files from within switch files as often as

you wish.  The file should contain valid switches and may contain comment

characters (;) at the beginning of each line or after switches.

For example,

INTEL593 @setup

The file SETUP contains

; INTEL593 AI-LANBIOS driver setup file

IOBASE=300            ; Use I/O address 300

verbose               ; Display verbose information

----------

HELP or ?

This switch displays information about the valid command line switches,

but does not install the INTEL593 driver.  For example,

INTEL593/help

INTEL593 ?

See the messages section for a detailed description of the HELP display

output.

----------

XEROX  [OFF]

This switch specifies that support for XEROX style packets has to be

provided. Specifying this switch will force the driver to transmit only

XEROX style packets.

NOTE: This doesn't affect the receive operation. The driver will continue

to receive IEEE802.3 style packets.

----------

IOBASE=hhhh [300] 0000-03e0

This switch specifies the I/O base address configured on the hardware

containing Intel 82593.

The IOBASE switch value must match the value specified on hardware.

The default value is IOBASE of 300.

Unless you have changed the settings on the hardware, the IOBASE switch is

unnecessary, as the driver default is same as the factory default.

----------

TX_DMA=dd [6] 6-7

This switch specifies the DMA Channel used for Transmit Commands. You will

need to change this value only if the DMA Channel 6 is being used by some

other application in your system.

----------

RX_DMA=dd [7] 6-7

This switch specifies the DMA Channel used for Transmit Commands. You will

need to change this value only if the DMA Channel 7 is being used by some

other application in your system.

----------

IRQ=dd [10] 2-10

This switch specifies the IRQ level used by hardware to interrupt the CPU.

You will need to change this value only if IRQ 10 is being used by some

other application in your system. But make sure that it matches the jumpered

IRQ on the hardware.

----------

MPX=hh [C7] C0-FF

This switch specifies the multiplex (MPX) interrupt number to use for

communication to the high-level AI-LANBIOS.  You will need to change this

value only if the MPX number is in use by another application or low-level

driver.

----------

PACKET_SIZE=dddd [1500] 574-1500

This switch specifies the maximum size of transmitted IEEE 802.3 packets.

You may need to change this value when using certain bridges that do not

support the full 1500 byte packet size.  Even when you reduce the packet

size, the low-level driver can still receive large packets.

----------

REMOVE

This switch removes INTEL593.EXE from memory.  You must remove TSR programs

in the opposite order they were loaded.  For example, if you loaded

INTEL593.EXE and then AILANBIO, you must remove AILANBIO before INTEL593.EXE.

For example if you have loaded

INTEL593.EXE

AILANBIO

To unload these copies of INTEL593.EXE and AILANBIO:

AILANBIO/REMOVE

INTEL593.EXE/REMOVE

The REMOVE option will remove the most recently run copy of AILANBIO.

----------

VERBOSE

This switch causes the INTEL593 driver to be installed and detailed

information about the configuration of the INTEL593 driver to be displayed.

See the messages section for a detailed description of the verbose output.

INTEL593 messages

=================

The INTEL593 driver produces two types of messages: Informative and Error.

All error messages are preceded by the text "ERROR:" and cause the INTEL593

driver not to be installed.

--------

MESSAGE:

Intel MLM-82593(tm) AI-LANBIOS compatible driver Vn.nn

(C) Copyright 1991 by ARTISOFT Inc.

Portions (C) Copyright 1992 by Intel Corporation

This message appears each time the low-level driver is invoked.

--------

MESSAGE:

---- MLM-82593 AI-LANBIOS Compatible driver installed ----

This message appears after the low-level driver is successfully

installed.

--------

MESSAGE:

---- MLM-82593 AI-LANBIOS Compatible driver not installed ----

This message appears if the low-level driver is not installed.  The driver

will not be installed if help information is displayed or an error

occurred.

--------

MESSAGE:

Command line              verbose

IEEE 802.3 node address  00AA00032456   Network packet size      1500

MPX interface number     C7             IO base address          0300

Transmit DMA Channel     6              Receive  DMA Channel     7

Interrupt request (IRQ)  10             Packet type              IEEE 802.3

Bytes of memory used     19520

This message appears if the VERBOSE switch is specified.  The values

displayed specify how the low-level driver is configured.  Some values may

not agree with switch values you specified on the command line.  This will

happen when you specify an IOBASE or RAMBASE switch value that is not a

proper multiple of the allowed value.

--------

MESSAGE:

Valid command line switches:

HELP

XEROX

IOBASE= Range 0000 to 03E0 hex

TX_DMA= Range 6 to 7 decimal

RX_DMA= Range 6 to 7 decimal

IRQ= Range 2 to 10 decimal

MPX= Range C0 to FF hex

PACKET_SIZE= Range 574 to 1500 decimal

REMOVE

VERBOSE

?

@ Range 0 to 2048 byte switch file

This message appears when the HELP or ? switch is used.  Each valid switch

is listed with the acceptable range of values (if any) and the base (hex

or decimal) that the numbers must be entered in.  The acceptable file size

range for indirect files (@) is also listed.  The low-level driver is not

installed when this switch is used.

--------

MESSAGE:

---- MLM-82593 AI-LANBIOS Compatible driver removed ----

The INTEL593.EXE driver has been removed from memory.

--------

MESSAGE:

Reading Configuration Parameters from NETIDBLK

The NETID Block was found in the memory. The parameters read from the NETID

Block will override any other specified parameters.

--------

ERROR:   MLM-82593 Board is not responding to initial setup. Check

Configuration.

Meaning: The initial configuration of 82593 is not successful.

Remedy:  Check that the configuration parameters for the hardware are OK.

Wrong IOBASE, TX_DMA or RX_DMA may cause this message to appear.

--------

ERROR:   MLM-82593 Configuration failed

Meaning: The initial configure command issued to 82593 is not successful.

Remedy:  Contact your Network support.

--------

ERROR:   MLM-82593 Individual Address Setup failed

Meaning: The initial IASETUP command issued to 82593 is not successful.

Remedy:  Contact your Network support.

--------

ERROR:   MLM-82593 AI-LANBIOS Compatible driver is not loaded

Meaning: You have issued the INTEL593/REMOVE command and do not have

INTEL593.EXE loaded into memory.

Remedy:  Only issue this command after the program is loaded.

--------

ERROR:   Can't REMOVE -- interrupts re-hooked

Meaning: You have attempted to remove INTEL593.EXE and the interrupts are

in use by another TSR, or INTEL593.EXE was not the last TSR

loaded.

Remedy:  Remove any TSRs loaded after INTEL593.EXE.  For example, if you

load INTEL593.EXE then AILANBIO.EXE, you must first remove

AILANBIO.EXE, then you can remove INTEL593.EXE.

--------

ERROR:   Installed AI-LANBIOS Compatible driver is different version

Meaning: You have loaded a version of the INTEL593.EXE program and

attempted to remove it from memory with the /REMOVE switch.  The

version with which you are attempting to remove INTEL593.EXE is

not the same version that was loaded.

Remedy:  Remove INTEL593.EXE with the same version of the program that was

loaded.

--------

ERROR:   MPX number hh is already in use - Try another number

Meaning: Multiplex (MPX) interrupt number hh is being used by another

application or low-level driver.

Remedy:  Try another MPX number.

--------

ERROR:   No network hardware found at I/O base address ????H

Meaning: The MLM-82593 hardware could not be located.  This error message

appears displayed because the adapter card is not present or the

IOBASE specified on the command line is different from what is

jumpered on the adapter.

Remedy:  Check to see that the IOBASE switch specified on the

INTEL593.EXE command line matches the I/O Address setting on the

hardware.

--------

ERROR:   Illegal switch - ??????=nnnn

Meaning: The switch ??????=nnnn is not a recognized switch.

Remedy:  You may only specify legal switches on the command line.

--------

ERROR:   Illegal character after switch name - ??????xnnnn

Meaning: The switch ?????? is followed by a character other than "=" or

":" or the switch does not take a value.

Remedy:  Only use the characters "=" or ":" to specify switch values and

do not place characters after switches that do not take values.

--------

ERROR:   Switch value not in range - ??????=nnnn

Valid range is ssss to eeee

Meaning: The value nnnn is not in proper range.  The value ssss

represents the lowest acceptable value and the value eeee

represents the highest acceptable value.

Remedy:  Limit your range of values to the acceptable range.

--------

ERROR:   Illegal digit in switch value - ??????=nnnn

Meaning: An illegal digit (character) was encountered in the numeric

switch value.

Remedy:  You must restrict numeric decimal values to the numbers 0-9.

You must restrict hexadecimal values to the numbers 0-9 and the

letters A-F.

--------

ERROR:   Cannot open indirect file - @file

Meaning: The file "file" cannot be opened as an indirect file.

Remedy:  The file must exist before it can be used as an indirect file.

Testing INTEL593 return code (error level)

========================================

You may test the INTEL593 return code or error level in a batch file by

using the IF ERRORLEVEL command.  The following error levels are returned:

0     No error has encountered.  INTEL593 driver installed

1     MPX number is in use

2     Switch error

4     82593-MLM not installed.

5     Can't remove, different version loaded

6     Cannot remove, interrupts re-hooked

7     ERROR: Can't REMOVE -- AI-LANBIOS (R) still installed

-------------------