XR v1.14 - X10 Report Program

Copyright (c) 1991 by Adrienne Cousins

All Rights Reserved

VersaWare

30 Jennifer Street, Littleton, MA 01460

508-486-9351 (Voice 9am-4pm Eastern)

508-486-8507 (24 hour Support BBS)

This program provides a couple of simple functions for the X10 that I have

been unable to find in any other X10 program, and have found very useful

while connecting and reprogramming my X10 computer interface.

The most useful function is the STATUS report (-S) which requests the

X10 to return an actual list of currently programmed events.

XR will translate the data returned from the X10 into an english format

report using the same "tokens" as the XA program.  This report will

show ALL events that are programmed in the X10 (many X10 control programs

do not clear prior events before programming new ones), and will also

show actual times that were calculated by "tokens" such as SUNRISE and

SUNSET.

The other useful function is the WATCH function (-W) which monitors the

serial port for data received from the X10.  Anytime a rocker switch is

pressed, or a programmed event occurs, the X10 sends a data packet to

the PC describing the event that just occurred.  If XR is running in

WATCH mode when this happens, the event is described on screen using the

same english "tokens" used by the XA program.

All output from XR can be redirected to any standard output device or

disk file.

-----------------------------------------

XR command syntax & functions:

XR [<cmd>][,<cmd>][,<cmd>] [>>outfile]

Where <cmd> can be any of the following switches.

?      Print a short help text to the screen

-A     Echo all X10 I/O to the screen in ascii

Causes all data sent to or received from the X10 to be echoed to

the screen in ascii.  This can also be redirected to any standard

output device or file.

-Bn    Set baud rate of serial port (300-19200)

Causes the serial port to be set to the specified speed.

If this switch is not specified, XR will use the serial port at

the currently initialized speed.

This is so that XR can also be used with X10s attached via Lawns.

XR -B600,-W

-H     Echo all X10 I/O to the screen in hexadecimal

Causes all data sent to or received from the X10 to be echoed to

the screen as hexadecimal bytes.  This data can also be redirected

to any standard output device or file.

XR -H,-W

-Pn    Serial port that X10 is connected to (1-4)

Use this switch to specify a serial port other the default of COM1.

Multiple -P switches may appear on the line, to send commands to

multiple serial ports in a single command.

XR -P1,-S,-P2,-S

-S     Display status of all events in the X10

Causes the X10 to be queried for a list of all events currently

programmed.  The data will be translated into english "tokens"

and displayed on the screen.  This report can also be redirected

to any standard output device or file.

XR -S >X10STS.LOG

-Tf:l  Change X10 timeout tick counts (first:last)

The value called FIRST, is the timeout in ticks until the first

byte is received from the X10.  The value called LAST, is the

timeout in ticks after the last byte is received from the X10.

FIRST defaults to 36 (2 secs), and LAST defaults to 18 (1 sec).

-W     Watch for any data received from the X10

This monitors the serial port for any data received from the X10.

It may be used in conjunction with the -A and -H echo switches.

This mode is terminated on any keystroke.

XR -H,-W

-----------------------------------------------------------

A list of the tokens used by XR are as follows:

HOUSE, NORMAL, SECURITY

TODAY, TOMORROW, EVERYDAY, WEEKENDS, WEEKDAYS

MONDAY, TUESDAY, WEDNESDAY, THURSDAY, FRIDAY, SATURDAY, SUNDAY

TIME, UNIT, LEVEL, ON, OFF, DIM, An-Pn

A sample status report follows:

XR -S

Status:1 TIME 6:52 pm MONDAY HOUSE A

Event:0 NORMAL WEEKENDS TIME 4:00 am A8 Level:0 ON

Event:1 NORMAL WEEKDAYS TIME 7:00 am A8 Level:0 ON

Event:2 NORMAL EVERYDAY TIME 8:53 pm A2 Level:7 DIM

Event:3 NORMAL EVERYDAY TIME 9:00 pm A2 Level:3 DIM

Event:4 NORMAL EVERYDAY TIME 12:00 am A2 Level:0 OFF

Event:5 NORMAL EVERYDAY TIME 8:53 pm A3 Level:7 DIM

Event:6 NORMAL EVERYDAY TIME 10:00 pm A3 Level:11 DIM

Event:7 NORMAL EVERYDAY TIME 4:49 am A3 Level:0 OFF

A sample WATCH session with HEX echo enabled follows:

XR -H,-W

Press any key to exit Watch mode

<R:FF FF FF FF FF FF 01 62 00 08 60 CA

Status:1 ON A5

<R:FF FF FF FF FF FF 01 63 00 08 60 CB

Status:1 OFF A5