AI-LANBIOS Low-level Driver for the Intel EtherExpress 16(tm) Adapter

(EXP16LAN.DOC - Rev 0.1)

-----------------------------------------------------

(C) Copyright 1992, 1993 ARTISOFT, Inc.

Portions Coyright 1992, 1993 Intel Corporation

Introduction

============

The EXP16LAN low-level AI-LANBIOS driver for Intel's EtherExpress 16 ethernet

adapter was developed by Intel Corp. in conformance with the specifications

outlined in the Artisoft AI-LANBIOS Driver Development Program.  The

low-level driver interfaces directly to the network hardware and

provides basic unreliable data transfer capabilities.  The AI-LANBIOS

high-level driver provides the reliable data transfer capabilities

including all NETBIOS functions.

The EXP16LAN driver makes extensive use of the on-board memory of the

EtherExpress 16 board to improve network throughput.

NOTE:   For performance and compatibility reasons, the EXP16LAN driver

is an I/O mapped-only version for the EtherExpress 16 adapter.

This means that even if you have your adapter configured to run

in memory mapped mode, the driver will ignore those settings and

run in I/O mapped mode.

Compatibility Statement

=======================

* Version 2.01 of EXP16LAN is compatible with Artisoft's LANtastic/AI Network

Operating System v4.1, except for the following functional considerations:

- Frequent disconnects are encountered when trying to configure

multiple adapters on 1 node.

- Cannot exist on a network with traffic other than Lantastic 4.1

802.3 type packets.

- Not compatible with XT systems.

- Does not support XEROX type packets.

- Hardware support is limited to EtherExpress 16/16TP ISA adapters

* Version 2.03 of EXP16LAN is compatible with Artisoft's LANtastic/AI Network

Operating System v4.1.  This driver works with all EtherExpress 16/16TP/FlashC

adapters and removes the functional considerations above.

- System hangs can occur when trying to configure multiple adapters

on 1 node.

- Auto-connect feature can fail to detect attached network cable.

* Version 3.00 of EXP16LAN is compatible with Artisoft's LANtastic/AI Network

Operating System v5.0.  This driver works with all EtherExpress 16/16TP/FlashC

adapters and has the following functional considerations:

- System hangs can occur when trying to configure multiple adapters

on 1 node.

- Auto-connect feature can fail to detect attached network cable.

* Version 3.01 of EXP16LAN is compatible with Artisoft's LANtastic/AI Network

Operating System v5.0.  This driver works with all EtherExpress 16/16TP/FlashC

adapters and removes the functional considerations above.  It has a new auto-

config feature when using multiple adapters on 1 node.

- Automatically finds next available adapter and assigns a mutiplex

number if no IOBASE or MPX parameter is specified on command line.

- Detects an IRQ conflict if more than 1 adapter is configured for

the same interrupt request.

In addition, the EXP16LAN driver does not accept IRQ as a command line switch

because the settings on the card are controlled by Intel's SOFTSET(tm) program.

Vendor Information

==================

Intel Personal Computer Enhancement Division.  5200, N.E. Elam Young Pkwy.

Hillsboro  OR 97124-6497.  Tech Support Phone Number: 503-629-7576.

Running EXP16LAN

================

Before running the EXP16LAN driver, configure your EtherExpress board by

running the SOFTSET program that is contained on the driver disk that came

with your EtherExpress board.

The EXP16LAN low-level driver must be run before the AI-LANBIOS.  The

EXP16LAN driver interfaces to the AI-LANBIOS using a DOS multiplex (Int 2Fh)

interrupt number.  The default multiplex number is C7.  If you are only

running one low-level driver and one copy of AI-LANBIOS then you will

not have to worry about which multiplex number to assign.  Multiplex

numbers 00 through BF are reserved for DOS use so you must use a multiplex

number between C0 through FF.

* Warning: Testing results showed that frequent disconnects were encountered

when trying to configure multiple adapters (and invocations of EXP16LAN.EXE

v2.01) on a single node.  See Compatibility Statement above. *

Each invocation of the EXP16LAN driver supports one Intel EtherExpress 16

adapter board.  You must run multiple copies of EXP16LAN if you have multiple

cards installed in your computer.  For each invocation of EXP16LAN you must

run a separate invocation of the AI-LANBIOS program.  When running multiple

invocations of the EXP16LAN driver each invocation must have a unique

multiplex number and I/O base address (V2.01/2.03/3.00).  The multiplex number

and I/O base address can be assigned automatically by not specifying these

parameters on the command line (V3.01).

The EXP16LAN low-level driver program name is EXP16LAN.EXE.  The syntax for

EXP16LAN is

EXP16LAN[switches . . . ] [ ; comment]

where [switches . . . ] denotes zero or more optional command line switches.

Switches may be delimited by blanks or slashes (/).  If a switch takes a

value, the switch must be followed by an equal sign (=) or a colon (:).

Some values may be hexadecimal numbers while other values may be decimal.

All switch values are range checked and illegal values produce errors.

You may place a comment on the command line by preceding the comment with

a semicolon (;).  You must precede the comment character with a blank.

The valid switch formats are:

SWITCH

SWITCH:value

/SWITCH

/SWITCH=value

@file

The valid EXP16LAN switches are enumerated below.  The letters "ddd"

denote a decimal number.  The letters "hh" denote a hexadecimal number.

Values enclosed in [] indicate default values if the switch is omitted.

Values after [] denote the valid range of the switch.

----------

@switch-file

This option specifies that further switches are to be taken from an

switch file.  Any switches after the @switch-file name will be used,

and the switch settings take precedence over any settings in the switch

file.  You may invoke switch files from within switch files as often as

you wish.  The file should contain valid switches and may contain comment

characters (;) at the beginning of each line or after switches.

For example,

EXP16LAN @setup

The file SETUP contains

; EXP16LAN AI-LANBIOS driver setup file

IOBASE=310            ; Use I/O address 310

verbose               ; Display verbose information

----------

HELP or ?

This switch displays information about the valid command line switches,

but does not install the EXP16LAN driver.  For example,

EXP16LAN/help

EXP16LAN ?

See the messages section for a detailed description of the HELP display

output.

----------

IOBASE=hhhh [300] 200, 210, 220, 230, 240, 250, 260, 270,

300, 310, 320, 330, 340, 350, 360, 370

This switch specifies the I/O base address configured on the EtherExpress

card.  The IOBASE switch value must match the value stored in the board EEPROM.

The default value is the same as the default factory setting on the cards.

Unless you have more than one EtherExpress board installed in your computer,

the IOBASE switch is unnecessary, as the driver can find and initialize

the board - (V2.01/2.03/3.00)

The driver can find and initialize

To set the I/O base address on the card, run Intel's SOFTSET program.

----------

MPX=hh [C7] C0-FF

This switch specifies the multiplex (MPX) interrupt number to use for

communication to the high-level AI-LANBIOS.  You will need to change this

value only if the MPX number is in use by another application or low-level

driver.

----------

PACKET_SIZE=dddd [1500] (574-1500 - V2.01/2.03) (600-1500 - V3.00/3.01)

This switch specifies the maximum data size of transmitted packets.

You may need to change this value when using certain bridges that do not

support the full 1500 byte packet size.  Even when you reduce the packet

size, the low-level driver can still receive large packets.

----------

XEROX [XEROX] - V3.00/3.01

This switch specifies the XEROX format of the packet header.

----------

IEEE [IEEE] - V2.01/2.03

This switch specifies the IEEE 802.3 format of the packet header.

----------

REMOVE

This switch removes EXP16LAN.EXE from memory.  You must remove TSR programs

in the opposite order they were loaded.  For example, if you loaded

EXP16LAN.EXE and then AILANBIO, you must remove AILANBIO before EXP16LAN.EXE

In addition, if you have more than one invocation of EXP16LAN.EXE and

AILANBIO you must use the MPX= switch on the command line to specify which

copy of EXP16LAN.EXE but not (AILANBIO) that you want to unload.  For

example, if you load two copies of EXP16LAN.EXE and AILANBIO:

EXP16LAN.EXE/MPX=C7

AILANBIO/ADAPTER=0/MPX=C7

EXP16LAN.EXE/MPX=D7

AILANBIO/ADAPTER=1/MPX=D7

To unload these copies of EXP16LAN.EXE and AILANBIO:

AILANBIO/REMOVE

EXP16LAN.EXE/MPX=D7/REMOVE

AILANBIO/REMOVE

EXP16LAN.EXE/MPX=C7/REMOVE

The REMOVE option will remove the most recently run copy of AILANBIO.

----------

VERBOSE

This switch causes the EXP16LAN driver to be installed and detailed

information about the configuration of the EXP16LAN driver to be displayed.

See the messages section for a detailed description of the verbose output.

EXP16LAN messages

=================

The EXP16LAN driver produces two types of messages: Informative and Error.

All error messages are preceded by the text "ERROR:" and cause the EXP16LAN

driver not to be installed.

--------

MESSAGE:

AI-LANBIOS EXP16LAN driver Vn.nn - (C) Copyright 1992 ARTISOFT Inc.

This message appears each time the low-level driver is invoked.

--------

MESSAGE:

---- EXP16LAN driver installed ----

This message appears after the low-level driver is successfully

installed.

--------

MESSAGE:

---- EXP16LAN driver NOT installed ----

This message appears if the low-level driver is not installed.  The driver

will not be installed if help information is displayed or an error

occurred.

--------

MESSAGE:

Command line             /verbose

IEEE 802.3 node address  02608C4B10CC   Network packet size      1500

MPX interface number     C7             Machine bus type         PC/XT/AT

IO base address          0300           Interrupt request (IRQ)  2

Packet type              IEEE 802.3

Bytes of memory used     2160

This message appears if the VERBOSE switch is specified.  The values

displayed specify how the low-level driver is configured.  Some values may

not agree with switch values you specified on the command line.  This will

happen when you specify an IOBASE or RAMBASE switch value that is not a

proper multiple of the allowed value.

--------

MESSAGE:

Valid command line switches:

HELP

IOBASE= One of the following choices:

200 210 220 230 240 250 260 270

300 310 320 330 340 350 360 370

MPX= range C0 to FF hex

PACKET_SIZE= range 570 to 1500 decimal

VERBOSE

?

@ range 0 to 2048 byte file

This message appears when the HELP or ? switch is used.  Each valid switch

is listed with the acceptable range of values (if any) and the base (hex

or decimal) that the numbers must be entered in.  The acceptable file size

range for indirect files (@) is also listed.  The low-level driver is not

installed when this switch is used.

--------

MESSAGE:

---- EXP16LAN removed ----

The EXP16LAN.EXE driver has been removed from memory.

--------

ERROR:   EXP16LAN not loaded

Meaning: You have issued the EXP16LAN/REMOVE command and do not have

EXP16LAN.EXE loaded into memory.

Remedy:  Only issue this command after the program is loaded.

--------

ERROR:   Can't REMOVE -- EXP16LAN interrupts re-hooked

Meaning: You have attempted to remove EXP16LAN.EXE and the interrupts are

in use by another TSR, or EXP16LAN.EXE was not the last TSR

loaded.

Remedy:  Remove any TSRs loaded after EXP16LAN.EXE.  For example, if you

load EXP16LAN.EXE then AILANBIO.EXE, you must first remove

AILANBIO.EXE, then you can remove EXP16LAN.EXE.

--------

ERROR:   Installed EXP16LAN is different version

Meaning: You have loaded a version of the EXP16LAN.EXE program and

attempted to remove it from memory with the /REMOVE switch.  The

version with which you are attempting to remove EXP16LAN.EXE is

not the same version that was loaded.

Remedy:  Remove EXP16LAN.EXE with the same version of the program that was

loaded.

--------

ERROR:   MPX number hh is already in use - Try another number

Meaning: Multiplex (MPX) interrupt number hh is being used by another

application or low-level driver.

Remedy:  Try another MPX number.

--------

ERROR:   No network hardware found at I/O base address ????H

Meaning: The EtherExpress 16 hardware could not be located.  This error message

appears displayed because the adapter card is not present or the

IOBASE specified on the command line is different from what is

jumpered on the adapter.

Remedy:  Check to see that the IOBASE switch specified on the

EXP16LAN.EXE command line matches the I/O Address setting contained in

the EtherExpress board's EEPROM.  To do this, run Intel's SOFTSET

program and choose the VIEW CURRENT CONFIGURATION menu.

--------

ERROR:   Illegal switch - ??????=nnnn

Meaning: The switch ??????=nnnn is not a recognized switch.

Remedy:  You may only specify legal switches on the command line.

--------

ERROR:   Illegal character after switch name - ??????xnnnn

Meaning: The switch ?????? is followed by a character other than "=" or

":" or the switch does not take a value.

Remedy:  Only use the characters "=" or ":" to specify switch values and

do not place characters after switches that do not take values.

--------

ERROR:   Switch value not in range - ??????=nnnn

Valid range is ssss to eeee

Meaning: The value nnnn is not in proper range.  The value ssss

represents the lowest acceptable value and the value eeee

represents the highest acceptable value.

Remedy:  Limit your range of values to the acceptable range.

--------

ERROR:   Illegal digit in switch value - ??????=nnnn

Meaning: An illegal digit (character) was encountered in the numeric

switch value.

Remedy:  You must restrict numeric decimal values to the numbers 0-9.

You must restrict hexadecimal values to the numbers 0-9 and the

letters A-F.

--------

ERROR:   Cannot open indirect file - @file

Meaning: The file "file" cannot be opened as an indirect file.

Remedy:  The file must exist before it can be used as an indirect file.

Testing EXP16LAN return code (error level)

========================================

You may test the EXP16LAN return code or error level in a batch file by

using the IF ERRORLEVEL command.  The following error levels are returned:

0     No error has encountered.  EXP16LAN driver installed

1     MPX number is in use

2     Switch error

3     No memory found or memory error

4     No hardware found or hardware already in use

6     EXP16LAN not loaded

7     Can't remove, different version loaded

8     Cannot remove, interrupts re-hooked

9     ERROR: Can't REMOVE -- AI-LANBIOS (R) still installed

-------------------