FBIOS is a COM port handler that is configured with command line

specifications for interrupt, hardware type, and port addresses.

The command line contains groups of specifications delimited by blanks.

FBIOS spec1 spec2 spec3 etc.

Each specification has several fields, delimited by commas.

The first field is a single number giving the IRQ: 2,3,4 or 5

The second field is a single letter giving the hardware type:

C - Standard type COM port, no shared interrupt.

Q - QuadRAM Quadport board.

S - Shared interrupt board (modified MS-400)

The remaining fields give the COM numbers and associated port addresses.

Some examples.

1. Standard COM1 only

This creates only COM1

FBIOS 4,C,1,3F8

2. Standard COM2 only

This creates only COM2

FBIOS 3,C,2,2F8

3. Standard COM1 and COM2

This creates COM1 and COM2

FBIOS 4,C,1,3F8 3,C,2,2F8

4. Standard COM1 and COM2, with ports reversed.

This creates COM1 and COM2

FBIOS 4,C,2,3F8 3,C,1,2F8

5. COM1 plus Quadport

This creates COM1 through COM6

FBIOS 4,C,1,3F8 3,Q,2,280,3,290,4,2A0,5,2B0,6,2C0

6. MS-400 on IRQ2

This creates COM3 through COM6

FBIOS 2,S,3,3E8,4,3E0,5,2F0,6,2E8