AI-LANBIOS Low-Level Driver For KODIAK NotePort (NOTEPORT.DOC)

-----------------------------------------------------

(C) Copyright 1991 ARTISOFT, Inc.

(Revised 9/24/92)

I. INTRODUCTION

============

The NP_LAN_S.EXE / NP_LAN_E.EXE low-level AI-LANBIOS NotePort driver allows

you to use your Kodiak NotePort in a LANtastic network.

NP_LAN_S.EXE offers standard driver performance

and

NP_LAN_E.EXE offers enhanced performance but requires a bi-directional

printer port on the PC.

Please refer to the Kodiak NotePort Installation Guide prior to installing

the LANtastic software driver. Install the NotePort as instructed, then run

the supporting software (NP_LAN_S.EXE / NP_LAN_E.EXE and AILANBIO.EXE) as

detailed in the Guide.

NP_LAN_S.EXE / NP_LAN_E.EXE communicates with the NotePort, while AILANBIO.EXE

provides high-level communications across the local area network.  Because of

this, you cannot run network software until after you load NP_LAN_S.EXE /

NP_LAN_E.EXE and AILANBIO.EXE.  (Note: For information on running AILANBIO.EXE,

refer to AILANBIO.DOC on your LANtastic driver diskette.)

There are command line switches that you can use when you run NP_LAN_S.EXE /

NP_LAN_E.EXE and AILANBIO.EXE.  These switches allow you greater flexibility

in using these programs.  For information on the AILANBIO.EXE command line

options refer to AILANBIO.DOC on your LANtastic driver disk.  For information on

NP_LAN_S.EXE / NP_LAN_E.EXE command line switches, refer to "NP_LAN_S.EXE /

NP_LAN_E.EXE COMMAND LINE OPTIONS" in this file.

Compatibility Statement

-----------------------

v3.10 of NP_LAN_S.exe and NP_LAN_E.exe is compatible with Artisoft's

LANtastic/AI Network Operating System v4.1.  There are a few minor

anomalities of the adapter running on a LANtastic network, and they are

documented at the final section of this file.

Vendor Information

------------------

Kodiak Technology, 1338 Ridder Park Dr., San Jose CA 95131.  Tech. Support

800-777-7704.  BBS 408-452-0677.  Customer Service Hotline 800-888-8084.

II. RUNNING NP_LAN_S.EXE / NP_LAN_E.EXE

===================================

The syntax for running NP_LAN_S.EXE / NP_LAN_E.EXE is

NP_LAN_S[switches . . . ] [ ; comment]

or

NP_LAN_E[switches . . . ] [ ; comment]

where "switches" denotes any optional command line switches.  If you are

using more than one switch in a command line, use a space or a forward

slash (/) to separate the arguments.  If a switch takes a value (such as

when you designate the DOS multiplex number with the MPX= switch) the

switch must be followed by an equal sign (=) or a colon (:).  All switch

values are checked to make sure they are valid.  Illegal values produce

error messages.

These are valid switch formats:

SWITCH

/SWITCH

SWITCH:VALUE

/SWITCH=VALUE

You would use one of the first two formats for a switch that does not take

a value.  For example:

NP_LAN_S/HELP           or      NP_LAN_E/HELP

You would use the third or fourth formats for a switch that takes a value.

Such as:

NP_LAN_S/MPX=D7         or      NP_LAN_E/MPX=D7

III. NP_LAN_S.EXE / NP_LAN_E.EXE COMMAND LINE OPTIONS

================================================

The valid NP_LAN_S.EXE / NP_LAN_E.EXE switches appear below.

- The letters "ddd" denote that you must provide a decimal number.

- The letters "hh" denote a required hexadecimal value.

- Values enclosed in brackets "[]" indicate default values if the switch

is omitted.

- Values after the brackets denote the valid range of variables for the

switch.

----------

@switch-file

This option specifies that further switches are to be taken from an

switch file.  Any switches after the @switch-file name will be used,

and the switch settings take precedence over any settings in the switch

file.  You may invoke switch files from within switch files as often as

you wish.  The file should contain valid switches and may contain comment

characters (;) at the beginning of each line or after switches.

For example,

NP_LAN_S @setup            or      NP_LAN_E @setup

The file SETUP contains

; AI-LANBIOS NotePort Driver setup file

;

MPX=D7            ; Use Multiplex number D7

IRQ=5             ; Use interrupt request 5

VERBOSE           ; Display verbose information

----------

HELP or ?

This switch instructs NP_LAN_S.EXE / NP_LAN_E.EXE to display information about

the valid command line switches, but does not install the NotePort driver.

For example,

NP_LAN_S/help      or      NP_LAN_E/help

NP_LAN_S ?                 NP_LAN_E ?

See the messages section for a detailed description of the HELP display

output.

----------

IRQ=d [7] 5 or 7

This switch specifies the interrupt request (IRQ) level for the parallel port

adapter to which the NotePort is connected.  If you have more than one NotePort

installed, that means you should have more than one parallel port adapter

and each parallel port adapter should have its own unique IRQ. The IRQ switch

value must match the jumper setting on the parallel port adapter.

Consult the parallel port adapter installation guide for more information

on setting IRQ jumpers on the adapter.

----------

LPT=d [1] Range 1 to 3

This switch allows you to specify the parallel port where the NotePort is

installed.  You will need to use this switch if you've installed the NotePort

on LPT2 or LPT3.

----------

MPX=hh [C7] C0-FF

This switch specifies the DOS multiplex (MPX) number to use for

communication with AILANBIO.EXE.  You will need to change this value only

if the default MPX number is in use by another application or NETBIOS.

Multiplex numbers 00 through BF are reserved for DOS use; therefore, you

must use a multiplex number between C0 and FF.

----------

PACKET_SIZE=dddd [1500] 574-1500

This switch specifies the maximum size of transmitted IEEE 802.3 packets.

You may need to change this value when using certain bridges that do not

support the full 1500-byte packet size.  Even when you reduce the packet

size, the low-level driver can still receive larger packets.

----------

REMOVE

This switch removes NP_LAN_S.EXE / NP_LAN_E.EXE from memory.  You must remove

TSR programs in the opposite order they were loaded.

For example, if you loaded NP_LAN_S.EXE / NP_LAN_E.EXE and then AILANBIO, you

must remove AILANBIO before NP_LAN_S.EXE / NP_LAN_E.EXE.

In addition, if you have more than one invocation of NP_LAN_S.EXE / NP_LAN_E.EXE

and AILANBIO you must use the MPX= switch on the command line to specify which

copy of NP_LAN_S.EXE / NP_LAN_E.EXE but not (AILANBIO) that you want to unload.

For example, if you load two copies of NP_LAN_S.EXE / NP_LAN_E.EXE and AILANBIO:

NP_LAN_S/MPX=C7                 or      NP_LAN_E/MPX=C7

AILANBIO/ADAPTER=0/MPX=C7

NP_LAN_S/MPX=D7                 or      NP_LAN_E/MPX=D7

AILANBIO/ADAPTER=1/MPX=D7

To unload these copies of NP_LAN_S.EXE / NP_LAN_E.EXE and AILANBIO:

AILANBIO/REMOVE

NP_LAN_S/MPX=D7/REMOVE          or      NP_LAN_E/MPX=D7/REMOVE

AILANBIO/REMOVE

NP_LAN_S/MPX=C7/REMOVE          or      NP_LAN_E/MPX=C7/REMOVE

The REMOVE option will remove the most recently run copy of AILANBIO.

----------

VERBOSE

This switch causes the NP_LAN_S.EXE / NP_LAN_E.EXE driver to be installed and

detailed information about the configuration of the NP_LAN_S.EXE / NP_LAN_E.EXE

driver to be displayed.

See the messages section for a detailed description of the VERBOSE output.

---------

XEROX

This switch causes all transmitted packets to be in the XEROX format

instead of the IEEE 802.3 standard.  When using this switch NP_LAN_S.EXE /

NP_LAN_E.EXE will receive both XEROX and IEEE 802.3 packets.

IV. USING MULTIPLE NOTEPORTS

========================

NP_LAN_S.EXE / NP_LAN_E.EXE and AILANBIO.EXE communicate with each other using

a DOS multiplex (MPX) number.  Each NP_LAN_S.EXE / NP_LAN_E.EXE and AILANBIO.EXE

pair supports one NotePort.

You will need to run an additional copy of NP_LAN_S.EXE / NP_LAN_E.EXE and

AILANBIO.EXE for each additional NotePort installed in your computer.  Using

command line switches, you must assign each NP_LAN_S.EXE / NP_LAN_E.EXE and

AILANBIO.EXE pair a unique multiplex, adapter and IRQ number.  You must also

specify the LPT port to which each NotePort is attached.  For example:

NP_LAN_S        or      NP_LAN_E

AILANBIO

NP_LAN_S/IRQ=5/MPX=D7/LPT=2     or      NP_LAN_E/IRQ=5/MPX=D7/LPT=2

AILANBIO/ADAPTER=1/MPX=D7

The first invocation of NP_LAN_S.EXE / NP_LAN_E.EXE and AILANBIO.EXE loads into

the first NotePort using the default MPX number (C7) interrupt request number

(7) and adapter number (0).  This NotePort is attached to LPT1.

The command line switches in the second invocation instruct the NETBIOS to load

into the second NotePort, (adapter number 1) using interrupt request 5 and DOS

MPX number D7.  This NotePort is attached to LPT2.

V. NP_LAN_S.EXE / NP_LAN_E.EXE MESSAGES

====================================

The NP_LAN_S.EXE / NP_LAN_E.EXE driver produces two types of messages:

Informative and Error.

All error messages are preceded by the text "ERROR:" and in those cases

the NP_LAN_S.EXE / NP_LAN_E.EXE driver is not installed.

--------

MESSAGE:

AI-LANBIOS NotePort driver (Standard Version) Vn.nn

- (C) Copyright 1989, 1990 ARTISOFT Inc.

- Portions (C) Copyright 1992 Kodiak Technology, Inc.

or

AI-LANBIOS NotePort driver (Enhanced Version) Vn.nn

- (C) Copyright 1989, 1990 ARTISOFT Inc.

- Portions (C) Copyright 1992 Kodiak Technology, Inc.

This message appears each time the low-level driver is invoked.

--------

MESSAGE:

---- AI-LANBIOS NotePort Driver installed ----

This message appears after the low-level driver is successfully installed.

--------

MESSAGE:

---- AI-LANBIOS NotePort Driver NOT installed ----

This message appears if the low-level driver is not installed.  The driver

will not be installed if HELP information is displayed or an error

occurred.

--------

MESSAGE:

Command line                 /verbose

IEEE 802.3 node address      00801B0209D6   Network packet size  1500

MPX interface number         C7             LPT port number      1

Interrupt request (IRQ)      7              LPT type             UNI-DIRECTIONAL

Packet type                  IEEE 802.3     Cable type           THIN COAXIAL

Bytes of memory used         4000

This message appears if the VERBOSE switch is specified.  The values

displayed specify how the low-level driver is configured.

--------

MESSAGE:

Valid command line switches:

HELP

XEROX

LPT= Range 1 to 3 decimal

IRQ= One of the following choices:

5 7

MPX= Range C0 to FF hex

PACKET_SIZE= Range 574 to 1500 decimal

REMOVE

VERBOSE

?

@ range 0 to 2048 byte file

This message is displayed when the HELP or ? switch is used.  Each valid

switch is listed with the acceptable range of values (if any) and the base

(hex or decimal) that the numbers must be entered in.  The acceptable file

size range for indirect files (@) is also listed.  The low-level driver is

not installed when this switch is used.

-------

MESSAGE:

---- AI-LANBIOS NotePort Driver removed ----

The NP_LAN_S.EXE / NP_LAN_E.EXE driver has been removed from memory.

--------

ERROR:    AI-LANBIOS NotePort Driver not loaded

Meaning:  You have issued the NP_LAN_S/REMOVE (or NP_LAN_E/REMOVE) command and

do not have NP_LAN_S.EXE / NP_LAN_E.EXE loaded into memory.

Remedy:   Only issue this command after the program is loaded.

-------

ERROR:    Can't REMOVE -- interrupts re-hooked

Meaning:  You have attempted to remove NP_LAN_S.EXE / NP_LAN_E.EXE and the

interrupts are in use by another TSR, or NP_LAN_S.EXE / NP_LAN_E.EXE

was not the last TSR loaded.

Remedy:   Remove any TSRs loaded after NP_LAN_S.EXE / NP_LAN_E.EXE.

For example, if you load NP_LAN_S.EXE / NP_LAN_E.EXE then

AILANBIO.EXE, you must first remove AILANBIO.EXE, then you can

remove NP_LAN_S.EXE / NP_LAN_E.EXE.

-------

ERROR:    Installed AI-LANBIOS NotePort Driver is different version

Meaning:  You have loaded a version of the NP_LAN_S.EXE / NP_LAN_E.EXE program

and attempted to remove it from memory with the /REMOVE switch.

The version with which you are attempting to remove NP_LAN_S.EXE /

NP_LAN_E.EXE is not the same version that was loaded.

Remedy:   Remove NP_LAN_S.EXE / NP_LAN_E.EXE with the same version of the

program that was loaded.

------

ERROR:   Cannot open switch file - @file

Meaning: The file "file" cannot be opened as an switch file.

Remedy:  The file must exist before it can be used as an switch file.

------

ERROR:   Illegal digit in switch value - ??????=nnnn

Meaning: An illegal digit (character) occurred in the numeric

switch value.

Remedy:  You must restrict numeric decimal values to the numbers 0-9.

You must restrict hexadecimal values to the numbers 0-9 and the

letters A-F.

------

ERROR:   Illegal switch - ??????=nnnn

Meaning: The switch ??????=nnnn is not a recognized switch.

Remedy:  You may only specify legal switches on the command line.

------

ERROR:   MPX number hh is already in use - Try another number

Meaning: The multiplex (MPX) interrupt number hh is being used by another

application or low-level driver.

Remedy:  Try another MPX number.

------

ERROR:   AI-LANBIOS NotePort Driver already loaded

Meaning: You have attempted to load a second copy of NP_LAN_S.EXE / NP_LAN_E.EXE

into the same NotePort.

Remedy:  Load only one copy of NP_LAN_S.EXE / NP_LAN_E.EXE per NotePort.

If you use multiple NotePorts, make sure each copy of NP_LAN_S.EXE /

NP_LAN_E.EXE has a unique MPX, IRQ and LPT value.

------

ERROR:   Either NotePort is not found at LPTn or Self-Test fails.

Meaning: NP_LAN_S.EXE / NP_LAN_E.EXE was unable to load into the NotePort.

Remedy:  Check the NotePort connection to the computer's printer port and that

the green LEDs are lit.

------

ERROR:   Switch value not in range - ??????=nnnn

Valid range is ssss to eeee

Meaning: The value nnnn is not in proper range.  The value "ssss"

represents the lowest acceptable value and the value "eeee"

represents the highest acceptable value.

Remedy:  Limit your range of values to the acceptable range.

VI. TESTING NP_LAN_S.EXE / NP_LAN_E.EXE RETURN CODES (ERROR LEVEL)

==============================================================

You may test the NP_LAN_S.EXE / NP_LAN_E.EXE return code or error level in a

batch file by

using the IF ERRORLEVEL command.  The following error levels are returned:

0     No error has encountered.  NotePort Driver installed

1     MPX number is in use

2     Switch error

3     NotePort not found or Self-Test fails

4     NP_LAN_S.EXE / NP_LAN_E.EXE not loaded

5     Can't remove, different version loaded

6     Can't remove, interrupt vector re-hooked

7     ERROR: Can't REMOVE -- AI-LANBIOS (R) still installed

VII. WARNING

=======

With some printers, it is necessary to leave the printer power on when

the printer is attached to the NotePort printer connection. If you need to

power off the printer while using the NotePort, you should disconnect the

printer from the NotePort before turning it off.

The Kodiak drivers cannot be loaded on most models of microchannel machines

in our lab.

The enhanced driver (NP_LAN_E.exe) supports bi-directional communication on

the parallel port.  It does not support the new Xircom-Intel enhanced parallel

port specifications available on i386SL chips.

-------------------

NotePort is a trademark of Kodiak Technology, Inc.