Novell v3.11 PCN driver README File

With the release of NetWare v3.11, Novell has adopted and

standardized the IBM PCNII media frame type on the 3.11 server.

NetWare 2.x fileservers supported two PCNII media frame types - the

IBM PCNII and the Novell PCNII.

NOTE:       In order for your NetWare v3.x server to

communicate with other fileserver/bridges

or workstations on your PCNII network

segments, you must make sure that all

fileserver/bridges and workstations on each

segment are configured using the IBM PCNII

media frame type.

CHECKING MEDIA FRAME TYPE ON THE NETWARE 286 FILESERVER/BRIDGE

To determine what media frame type your NetWare 286

fileserver/bridge is currently using, type CONFIG at the

fileserver/bridge console.  If you see the message "PCN II &

Baseband Lansup Support Prg Compatible" listed, then you are running

the IBM media frame type and will not need to make any changes to

that server/bridge.  Otherwise, if you only see "PCN II & Baseband

(version)" then you must reconfigure that fileserver/bridge

UPDATING THE NETWARE 2.0X OR NETWARE 2.1X SERVER

If you have determined that you are NOT currently using the IBM

Media Frame Type (See section above), then you must re-create the

NetWare 286 operating system and use the PCNIIL lan driver that came

with your 286 NetWare.  This is done by using the Novell NETGEN

utility. Refer to your NetWare installation manual for additional

information on how to perform this function.

CHECKING MEDIA FRAME TYPE ON THE WORKSTATION

Locate the NetWare shell files (IPX.COM and Netx.COM) on your DOS

workstation.  From the directory where these files are located, type

IPX I.  If you see the message "LAN Support Pgm Driver", then you do

not need to make any changes to this workstation.  Otherwise, if you

only see "PCN II & Baseband (version)" then you will be required to

also update the NetWare shell files on that workstation.

UPDATING THE WORKSTATION NETWARE SHELL

If you determine that you need to update the NetWare shell on the

workstation, obtain the DOS ODI workstation kit.  This kit is

available as part of the NetWare  v3.11 product, from Novell as a

separate kit, or the files from this kit, contained in the file

called DOSODI.ZIP in NOVA forum 8,  may be downloaded from

Compuserve (normal Compuserve connect fees and telephone tolls will

apply).  Since the DOS ODI drivers are "modular" you do not need the

Novell SHGEN utility to create the driver.