Command-line diagnostic CHECKSYS.EXE

Dan Shearer, Computer Centre, University of South Australia

Comments and bug reports to:  ccdps@lux.levels.unisa.edu.au

Version 1.36 released to the public domain 20th May, 1992.

Overview

---------

CHECKSYS is a command-line diagnostic program which either gives a short

description of a whole PC system, or returns a one-line description and

gives an errorlevel when asked about one aspect of the system.

The primary intention is for it to be used to determine system configuration

and driver status from batch files. It can also be handy when wanting a

quick idea of what a machine has during a system debugging session

without starting up a complicated package and poking around menus.

Reasoning Behind CHECKSYS

-------------------------

We have a very mixed hardware environment at UniSA, but like to have

generic batch files on the LAN. In our batch file that starts up Windows,

for instance, we have several CHECKSYS calls to determine what mode

Windows should be run in and what hardware drivers are needed, and also

to check and see if Windows is already running.

How it Works and Acknowledgements

---------------------------------

It is a collection of routines that I have put together in Turbo Pascal

6.0, with assembler routines to check for CPU and FPU. All the detection

calls are in a self-contained TP unit, and I would be willing to release

this to the public domain as well if there was a call for it.

The commandline switch checking code in CHECKSYS is with the kind permission

of Rocky Stone, also of UniSA. Gary Oxton (once a UniSA student) wrote

the basis of the CPU-detection assembler I have used for his final-year

Computer Studies project. I have learnt about checking for memory types

from the InfoPlus source code, by Andrew Rossman and Steve Grant, and

used their LongCall assembler function, which overcomes a shonky feature

of Turbo Pascal 6.0.

Ralf Brown's interrupt list is, of course, absolutely essential.

Syntax

------

CHECKSYS [/switches]

where switches and corresponding actions are:

/ALL  Displays the messages for all tests and returns errorlevel 0

/CPU  returns errorlevel 0-8 in order for:

8088, 8086, V20, V30, 80188, 80186, Unknown, 80286, 80386

/EMM  returns errorlevel 1 if EMM driver installed, 0 if not

/FTP  returns 1 if packet driver responding else 0 if not installed

/KBD  returns errorlevel 1 if extended keyboard installed, 0 if not

/MOUSE  returns 0 for no mouse driver else number of buttons

/NETBIOS  returns errorlevel 1 if NetBIOS responding, 0 otherwise

/PAR  returns number of parallel ports recognised by BIOS

/SER  returns number of serial ports recognised by BIOS

/VID  returns 1-10 in order for:

CGA, MCGA, EGA, EGA64, EGAMon, 8514, HercMon, ATT400, VGA, PC3270

/WIN  returns 0-3 in order for:

No Windows, Win/386 2.x, Win 3.0 (/R or /S), Win 3.x (/3)

/XMS  returns 2 for XMS & HMA avail, 1 for XMS but no HMA, 0 for no XMS

/87   returns 0-3 in order for: None, 8087, 80287, 80387

Each test also displays a detailed message to standard output.

Switches can be abbreviated to 1 letter. Errorlevel 99 for syntax error.

Examples:

CHECKSYS             displays the above help message

CHECKSYS /ALL        displays detailed results of all tests

CHECKSYS /MOUSE      displays detailed info about the mouse, and

returns errorlevels 0 for not present, 2 for two

button mouse present or 3 for three button mouse.

Bugs

----

BIOS check for printer and comms ports is often not accurate. Could add

some better checks.

The FPU check gives a silly result when used on a NEC V20 chip with an

Intel 8087 installed

Doesn't test for Weitek FPU

Doesn't distinguish OS/2 dos-boxes from Windows dos-boxes

Doesn't test for various network cards

Doesn't test for game port

Doesn't give video chipset particulars (might be useful for running

Windows with specific screen drivers)

Doesn't detect sound cards

Could possibly have a TSR check, ie syntax like:

CHECKSYS <unique TSR signature> [<optional range of interrupts>]