CZCOMMOD version 1.1 beta

12/12/90

I got tired of all the clumsy commport drivers that were around.

Not a single one that *I* found would allow various mixes of

different commports or "skipping" (i.e. COM1,3,4). Sooooo...

I sat down one evening and wrote CZCOM.SYS and CZMOD.COM.

These two serial port utilities enable COM3 and COM4 to perform the

same functions as COM1 and COM2.  One is CZCOM.SYS, a driver.  The

other is CZMOD.COM, a utility program which replaces DOS MODE.COM/EXE.

1.   CZCOM

CZCOM is a device driver that initializes two devices,

COM3 and COM4, for serial ports on your I/O Card.  After

CZCOM has been executed, you can gain access to these

serial ports by simply entering the names COM3 and COM4.

Before installing CZCOM, please set your I/O Card's serial

ports to COM3 and COM4.  If the system fails to "see" these

COM ports, the following message will appear on the screen:

"Can not find device COMn"  (where "n" is comport number)

Installation is easy.  Simply include "DEVICE=[dr:\DIR\]CZCOM.SYS"

in your CONFIG.SYS file (without the quotes, of course).  You may

have to define a [path] depending on where you keep CZCOM.SYS.

If your boot diskette or current working drive does not have a

CONFIG.SYS device, create one which includes the DEVICE= line.

reboot your system by pressing the RESET buttom; simultaneously

pressing CTRL, ALT and DEL; or turning the power off and on. If

COM3 or COM4 ports exist, then the following messages will appear

on the screen:

Device COM3 is installed.

Parameters set to 2400,n,8,1

Device COM4 is installed.

Parameters set to 2400,n,8,1

NOTE:  CZCOM DEFAULT SETS COM3 AND COM4 COMMUNICATION PARAMETERS

TO BAUD RATE 2400, NO PARITY, 8 DATA BITS AND 1 STOP BIT.

These parameters can be changed using CZMOD.COM as will be

explained later.

If COM3 and COM4 ports do not exist, then the following

messages will appear on the screen:

Can not find device COM3.

Can not find device COM4.

2.   CZMOD COMMAND

The utility programm, CZMOD.COM, sets options for an

asynchronous communications adapter.  The CZMOD.COM also

enables printer output to be routed to an asynchronous

communications adapter or to another printer port (LPT).

2.1  Setting Parameters

The command format for setting the options of an asynchronous

communications adapter is listed below.

CZMOD COMn:baud[,parity[,data bits[,stop bits[,p]]]]

The variable "n" can be 1, 2, 3 or 4 designating the number

of the asynchronous communications adapter.

The protocol parameters used to initialize the asynchronous

communications adapter can be set by replacing the bracketed

items with specific characters.  These parameters and the

specific characters are listed below.

Baud Rate: Enter 50, 75, 110, 134.5, 150, 300, 600, 1200,

1800, 2000, 2400, 3600, 4800, 7200, 9600, or 19200.

(The default setting is 2400)

NOTE: ONLY THE FIRST TWO CHARACTERS ARE REQUIRED, SUBSEQUENT

CHARACTERS ARE IGNORED.  FOR EXAMPLE, IF THE COMMAND

FORMAT WAS "CZMOD COM3: 12,2", THE MODE OF OPERATION

WOULD BE 1200 BAUD RATE, EVEN PARITY (default), DATABIT

7 (default), AND STOPBIT 2. ANY PARAMETERS OMITTED WOULD

REVERT TO DEFAULT SINCE NO CHARACTERS WERE ENTERED.

Parity:		Enter N (None), O (Odd), E (Even), M (Mark),

or S (Space).

The default setting is E.

Databits:	Enter 5, 6, 7 or 8.

The default setting is 7.

Stopbits:	Enter either 1 or 2.

The default setting is 2 if the baud rate

is less than or equal to 110.

The default setting is 1 if the baud rate

is greater than 110.

The "P" option indicates that the asynchronous adapter is

being used for a serial interface printer.  If you enter the

"P", time-out errors are continously retried.  You can stop

the retry loop by pressing Ctrl-Break.  To stop the time-out

errors from being continously retried when you have entered

"P", you must reinitialize the asynchronous adapter without

entering the "P". Including the P parameter allows CTL-Break

to react immediately. Without P, "break" will not occur until

the printer buffer has emptied.

2.2  Routing Printer Output

The command formats for directing parllel printer output to

an asynchronous communications adapter (serial port) or to

another printer are listed below.

CZMOD LPT#: = COMn

CZMOD LPT#: = LPT$

There are three variables in this command format:

The variable "#" can be 1, 2, or 3 designating the printer port

(LPT) number.

The variable "n" can be 1, 2, 3 or 4 designating the asynchronous

communications adapter (COM).

The variable "$" can be 1, 2, or 3 designating another printer port.

All output direct to the printer LPT# is redirected to the asynchronous

adapter n (COMn), or to another printer $.

Another format, CZMOD LPT#: = LPT#, disables any redirection to the

printer designated by the "#".

Before you can use the CZMOD to redirect the parallel printer

output to a serial device, you must initialize the asynchronous

communications adapter by using the CZMOD COMn format described

in Section 2.1 Setting Parameters.  If that serial device is a

printer, your seial initialization command should also include

the "P" parameter.

NOTE: WHEN USED IN THE CZMOD COMn FORMAT WITH THE "P" OPTION, OR

IN THE CZMOD LPT#: = COMn OR CZMOD LPT#: = LPT$, THE CZMOD

COMMAND CAUSES THE PRINTER AND ASYNCHRONOUS COMMUNICATIONS

ADAPTER INTERCEPT CODE TO BE MADE RESIDENT IN MEMORY.

!! VERY IMPORTANT NOTE !!

On some machines less than 100% IBM compatable, CZMOD.COM parameters

may tend to "stick". If this occurs, reboot your CPU "COLD" (turn it

OFF and then ON with the power switch) to reset CZCOM.SYS defaults !

3.   DISCLAIMER

Both CZCOM.SYS and CZMOD.COM have been tested extensively with no

injurious results to hardware or neighboring software.  But they

are provided strictly "as is", and any damage resulting from their

installation and /or use is strictly the responsibility of the user.

The author makes no claims of guarantee or warranty of any kind.

4.   LEGALESE

The author hereby donates CZCOM.SYS & CZMOD.COM to the public domain.

Any problems, questions, suggestions may be deposited in the nearest

file-13.  Any compliments, thank-yous, or monetary gratuities may be

forwarded to:

Cap'n Chris

LIBERTY BBS

(313)792-4911