OPERATION INSTRUCTIONS

The enclosed ADI printer driver SKFUJI36.COM has been developed

for FUJITSU 24-wire printers of the DL series in DPL24C PLUS

emulation with a maximum resolution of 360 * 360 dots per inch.

ADI provides the same functionality and performance as the

standard device drivers used for AutoCAD and AutoSketch.

As opposed to other common AutoCAD drivers the ADI drivers

have to be loaded before starting AutoSketch.

They are residing in the main memory.

1. Printer Installation

Before starting with the printing set the printer to the following

values:

EMULATE:		DPL24C PLUS	for 8-color printers

COLOR:		AUTOSEL  (only for 8-color printers)

LF-Code:		LF only

CR-Code:		CR only

2. Driver Installation

Copy the driver via the following command onto your harddisk:

COPY A:SKFUJI36.COM C:	for AutoSketch

3. Usage of the Driver with AutoSketch

You can only use an ADI printer driver if you have an AutoSketch

version of 1.04 or higher. If the output of the data is not

leaded directly to the printer but at first to a file,

AutoSketch will not enable the information of the file name.

Therefore, exists a driver only for AutoSketch which will

determine the name to "OUTPUT.LST" if there is an output to a file.

Before starting AutoSketch you first have to load the driver

SKFUJIE.COM or SKFUJI36.COM for FUJITSU 24 pin dot matrix

printers, which are using the DPL 24C PLUS command set.

Proceed according to the following syntax:

C:\SKETCH>SKFUJI36 /M		- monochrome printout

C:\SKETCH>SKFUJI36 /C		- color printout

C:\SKETCH>SKFUJI36 /F		- output into a file

Filename: OUTPUT.LST

You have to enter SKFUJI36, if you want to delete the

residentially loaded driver from your main memory.

If the driver is to be loaded automatically then

SKFUJI36.COM has to be entered together with the right

parameter in the AUTOEXEC.BAT batch file.

The program must be configured, if ADI is used for the

first time together with AutoSketch. While starting

AutoSketch you have to specify the parameter "-R":

C:\SKETCH>SKETCH -R

Now, determine the input device and the monitor in accordance

with your hardware configuration. By selecting the

plotter/printer select option 3:

AUTODESK DEVICE INTERFACE PRINTER

If this option does not appear in the AutoSketch configuration,

you probably have an old AutoSketch version. In this case

please contact your dealer.

Programming Text                                       Your Choise

__________________________________________________________________

Hexadecimal interrupt number                                   7bh

Specify plot size in millimeter: <N>                        RETURN

Maximum horizontal (X) plot size in inches <11.0>           13.599

Dots per inch in horizontal (X) direction <100.0>              360

Maximum vertical (Y) plot size in inches <8.50>                 12

Dots per inch in vertical (Y) direction <360.0>                180

__________________________________________________________________

When the following text appears:

DOES THE PRINTER SUPPORT COLOR  <N>

enter <Y> if your printer supports color and the driver with the

parameter /C for color was selected. Otherwise select <N>.

4.1 Color Allocation with AutoSketch

AutoSketch supports 7 colors: red, yellow, green, cyan, blue,

magenta and black. In order to be able to use these colors

you have to determine them in the menu item pin information

as follows:

Color			Pinallocation

_________________________________

Red				1

Yellow				2

Green				3

Cyan				5

Blue				4

Magenta				6

Black				7

_________________________________

In case of blue the printer will use orange, because the color

blue is not supported by the printer. If you do not want to do

so, you can also select pin No. 5 (cyan). Then blue appears in

the same color as cyan and now you have only 6 colors at your

disposal.

5. General Hints for Usage

Due to the limited capacity of the main memory the color printers

can only be selected with 8 pins. This cannot be changed because

of programming reasons.

We hope you understand our situation. The result is a slower

printout than in the case of a white-and-black printout, but

the print quality is the same.

If you should have any problems with the configuration or the

work with AutoCAD respectively AutoSketch under the ADI-driver,

please call the hotline of your country specific subsidiary

of Autodesk or FUJITSU.

We hope that these operation instructions will help you and

hope you enjoy working with AutoSketch and your Fujitsu printer.