SAPI scanner driver

SAPI.EXE

November 1, 1991

The SAPI scanner driver is a revision 6 scanner driver (see REV6.DOC for

more information) providing Black and White, Gray Scale and COlor scanning for

the current ZSoft products.

The ZSoft SAPI scanner driver requires the presents of a manufacturer

supplied scanner driver (referred to in this document as SAPI.SYS since the

actual name varies from manufacturer to manufacturer.  Logitech's SAPI

driver is called HHSCAND.SYS).  SAPI.SYS should be loaded before the ZSoft

driver.

SAPI.EXE also REQUIRES that the scanner hardware be setup for interrupts.

This has NOT been the case for previous ZSoft scanner drivers.

The commands provided by the SAPI scanner driver are listed below in the

example of the /H parameter.

ZSoft Scanner Driver, version 6.00

Copyright (C) ZSoft Corp. 1991.  All rights reserved.

Valid commands:

/Bxxx  = interrupt Buffer size in K (default is 8k)

/Cx    = application Code (1-4)

/H     = display this Help text

/?     = display this help text

/Tx    = do diagnostic Testing (verify communication with scanner)

/V     = Verbose messages and status displays

/U     = Unload driver

/B

==

The interrupt buffer will hold the scan data until the application is able

to retrieve the data from the scanner driver.  If the application can not

request the data fast enough or often enough then "drop outs" or "hits" can

occur in the scanned picture.  Increasing the buffer size allots the

application more time to process the scan data before it must retrieve

additional data from the scanner driver.  The idea here is to use as small

an interrupt buffer as you can without getting any "drop outs" in your

pictures.

/C

==

In order to maximize the scanner functionality available to the application,

the /C parameter is used.  This command parameter is usually set by the

install program.  If the parameter is omitted, the application will still

work, but may be missing some features.  The codes are:

/C1 = Paintbrush Plus (DOS)

/C2 = Paintbrush Plus (WIN)

/C3 = Paintbrush 4 Plus (DOS)

/C4 = Publisher's Paintbrush (Win)

Manufacturer Notes

==================

At this time only the Logitech ScanMan and ScanMan 256 have been tested

with the SAPI.EXE scanner driver.  Any Black and White or Grayscale scanner

should work with this driver, however, no color support exists at this

time.  Do to the lack of software control over the Logitech scanners the

SAPI driver can sometimes return invalid data for a scan.  In order to

avoid this problem be sure that the switch settings on the hand scanner are

set to match the selections made in the application.  For the ScanMan 256

be sure that the mode switch is set to 256 BEFORE SAPI.EXE is loaded or

grayscale may not be available as an option within the application.

SAPI Messages

=============

'Supporting: ???????????'

This message is displayed upon successful initialization of the SAPI

scanner.  The name and model of the scanner found will be displayed.

SAPI Error Messages

===================

'Manufacturer driver not found!  SAPI NOT loaded.'

This message is displayed when the scanner driver can not find the "helper"

driver (provided by the scanner maufacturer).  The ZSoft SAPI scanner

driver requires that this driver be loaded before we can load.

'Scanner not responding (do you have the right address?)'

This message is displayed at initialization time if the scanner driver cannot

successfully communicate with the scanner.

'Error communicating with manufacturers driver (????)'

This message is displayed when an error occurs while sending or receiving

information from the manufacturer supplied driver.  An error code is

included in the message to help identify the problem.

'Scanner hardware not configured for interrupts!'

The SAPI.EXE driver requires interrupts to function.  The scanner

hardware interface must be configured to use interrupts.  With most

manufacturers drivers the interrupt can be set with a command line

parameter to the driver (e.g. /I=7, sets interrupt 7).