SpeedSTAR VGA - Windows 3.0 and 3.0a Display Drivers

and

PRERELEASE Windows 3.1 BETA 061d

The instructions below apply to Windows 3.0 as supplied on 5 1/4" HD diskette

media. The included drivers will support Windows 3.1 FINAL BETA 061d release,

for additional information see the README.W31 file.

INSTRUCTIONS - FIRST TIME WINDOWS 3.0 and 3.0a INSTALLATION

Follow the instructions contained in the WIN30DRV.DOC file to install the

Diamond SpeedSTAR display drivers.

INSTRUCTIONS - WINDOWS 3.0 or 3.0a ALREADY INSTALLED

To use the Diamond SpeedSTAR Windows drivers you need to copy the files on the

supplied SpeedSTAR driver diskette to the \WINDOWS\SYSTEM subdirectory.

After you copy the files, change the display configuration from within Windows

itself to one of the extended drivers.

See the Windows documentation for further information.

IMPORTANT

If you have Windows 3.0, you also need to copy the SETUP.W30 file as SETUP.INF

before you run the SETUP program.

If you have Windows 3.0a, you also need to copy the SETUP.W3A file as SETUP.INF

before you run the SETUP program.

When you run the Windows Setup to change to a different driver Windows may

prompt you for some of the Windows Installation diskettes. The drivers use

display fonts which are contained on the standard Windows distribution

diskettes.

The drivers have various display memory requirements and of course your display

monitor must be able to support the resolution you select.

The drivers support the following:

Resolution    Mode      Minimum display     Driver name         Windows Font

(HEX)   memory required

640x480x2     11h       256kb              VGAMONO.DRV  **     VGA

640x480x16    12h       256kb              VGA.DRV      **     VGA

800x600x16    29h       256kb              VGA464.DRV          8514

800x600x16    29h       256kb              VGA464S.DRV         VGA

1024x768x16    37h       512kb              VGA474.DRV          8514

1280x1024x16   3Dh       1 meg              VGA414.DRV          8514

640x480x256   2Eh       512kb              VGA448.DRV          VGA

640x480x32k   2Eh xx    1 meg              VGA443.DRV          VGA

640x480x65k   2Eh xx    1 meg              VGA446.DRV          VGA

800x600x256   30h       512kb              VGA468.DRV          8514

800x600x256   30h       512kb              VGA468S.DRV         VGA

800x600x32k   30h xx    1 meg              VGA463.DRV          8514

800x600x32k   30h xx    1 meg              VGA463S.DRV         VGA

800x600x65k   30h xx    1 meg              VGA466.DRV          8514

800x600x65k   30h xx    1 meg              VGA466S.DRV         VGA

1024x768x256   38h       1 meg              VGA478.DRV          8514

** uses Windows 3.0 or 3.0a supplied VGA driver

xx requires a Sierra Semiconductor Hi-Color DAC

IMPORTANT

It is important that the drivers be installed properly, replacement

grabbers (GR2 extension) and Virtual-Display-Device handler (386 extension) are

included to properly support the extended feature set of the SpeedSTAR.