PHOTOPLOTTING PRINCIPLES

What's a Photoplotter?

A photoplotter is just what the name implies: an plotter that

writes using light.  A plotter has to be told:

Which tool to use.

When to use the tool, and when not to.

Where to go next.

Whether to go there in a straight line or along an arc.

For a photoplotter, "tool" means specially shaped apertures

through which light passes to create a given shape on film.  An

aperture can be used without movement to make a shape (a "flash")

or with movement to make a line or an arc.

There are two major types of photoplotters, "Vector" and "Raster"

(or "laser").  Each handles apertures differently.

Vector Photoplotters

Aperture Wheels

Traditionally, the photoplotter counterpart to a pen plotter's

pen rack has been the aperture wheel.  The aperture wheel is a

disk with 24 or 70 apertures arrayed radially along its

circumference.

When the photoplotter selects an aperture, the aperture wheel is

rotated to place the desired aperture between the light source

and the film.  Apertures are themselves pieces of film and can be

made to any shape required, although in practice this is a time-

consuming process and there is a physical limitation on size.

Flash and Draw Apertures

To achieve constant exposure on a vector photoplotter, apertures

used for flashing pads are filtered differently than those used

for drawing traces.  Therefore, Flash and Draw apertures cannot

be used interchangeably without risk of localized over-exposure

and under-exposure.

Aperture Wheel Setup for Vector Plotters

The setup of an aperture wheel is an exacting and time consuming

process since each aperture in the wheel must be hand-mounted and

aligned.  In order to avoid repeated setup costs, designers have

the photoplotting vendor keep a wheel on file and are forced to

always use that same set of apertures.  This has obvious

drawbacks, both in terms of design flexibility and the ease of

migration to other vendors.

Raster (Laser) Plotters

Aperture Lists

Increasingly, vector photoplotters are being replaced by the

laser photoplotter, which emulates the older style machine in a

raster (bit-map) fashion.  While use of the term "aperture" to

describe a pad or trace shape persists, the term "aperture wheel"

is now being replaced by "aperture list", which implies the

greater flexibility now available to the designer.

There are three principle advantages with aperture lists on

raster plotters:

Aperture shapes can be easily generated in software, thus

eliminating the need to design a physical wheel.

More apertures can be defined on a list.

Allowable apertures sizes are typically (but not always)

greater than those imposed by the physical dimensions of an

aperture wheel.

Flash and Draw Apertures

No distinction need be made between Flash and Draw aperture types

since the light source intensity is constant.

Speed Advantage of Laser Plotters

Laser plotters operate much quicker than vector machines.  A

complex plot that required hours on a vector machine can usually

be performed in ten minutes or less on a laser photoplotter.

This decreases turnaround time and in many markets has driven

photoplotting costs down.

Talking to Photoplotters

The de facto standard for photoplotter data is the Gerber format,

more properly known as RS-274D.  The term Gerber refers to the

Gerber Scientific Instrument company, a pioneer and leader in

photoplotter manufacturing.

RS-274D is a variation on traditional Numerical Control (NC)

machine tool languages.  It differs from traditional NC formats

(i.e. drill data), as far as its use of tool selection codes but

is otherwise compatible.

RS-274D data is organized in "blocks".  A block consists of a

combination of codes:

Tool selection

Setup

Movement

And, an End Of Block (EOB) character, which only follows a

combination of the above codes.

An EOB character is usually an asterisk ('*') or dollar ('$'),

optionally followed by a carriage return and line feed.

An RS-274D code consists of a letter D,G,M,X,Y,I or J followed by

a numerical value.  These codes designate the following:

* - End of Block (end of command)

D - Select aperture, or set aperture use mode

X - Move to X value

Y - Move to Y value

G - Various setup codes

M - Various control codes

I - Relative X location for arc center

J - Relative Y location for arc center

D Codes

D codes have multiple purposes.  The first is to control the

state of the light being on or off. Valid codes for light state

are D01, D02, and D03.

D01 - Light on for next move.

D02 - Light off for next move.

D03 - Flash (Light On, Light Off) after move (effect is limited

to block in which appears, ie non-modal).  You can also think

of a D03 as D02, D01, D02 series of commands linked together.

D codes with values of 10 or greater represent the aperture's

position on the list or wheel.  It is very important to

understand that there is no universal "D10" or "D30".  Unlike the

D01 , D02, and D03 counterparts which have a fixed meaning (draw,

move, flash), D10 and higher values have aperture shapes and

dimensions assigned to them by each individual user.  Hence, one

job's D10 could be a 10 mil Round, when another job's D10 could

be a 45 mil Square.

There are two distinct ways to number an aperture list.  The

traditional 24 aperture system started with D10 - D19, jumping

suddenly to D70 - D71, then back to D20 - D29, ending with D72 -

D73.  This is still a common format for output for CAD packages,

and is still mandatory for old 24 aperture Gerber vector

photoplotters.

It is now common to start with D10, then increase numerically in

steps of 1 (D10, D11, etc.) continuing up to D70 and beyond,

rarely beyond 1000 individual apertures.

X & Y Codes

The X & Y values in the Gerber file determine where the aperture

shape and dimension will be positioned and drawn.  X & Y values

are used as coordinate pairs to determine where the light will be

exposed, using the D codes shapes (i.e. D10) and light exposure

status (i.e. D01, D02, D03) for drawing lines and arcs, as well

as moving between drawing entities.

Here are a few examples of using X & Y codes with D codes.

D10*  { Select aperture D10}

X1000Y1000D02* { The D02 tells us that the light will be off,

and we move to coordinate position X1000 and Y1000}

X2000Y3000D01* { The D01 tells us that we will draw (light on)

to coordinate position X2000 and Y3000}

X5500Y100D03* { The D03 tells us to move to coordinate position

X5500 and Y100 with the light off, then flash (turn the light

on and off)}

G Codes

G codes are used to configure the photoplotter.  Commonly

implemented codes include:

G01 - Future X,Y commands are straightline moves

G02 - Future X,Y commands are clockwise arcs

G03 - Future X,Y commands are counterclockwise arcs

G04 - Ignore the rest of this block (used for Comments)

G54 - Prepare to change apertures

G74 - Future arcs are quadrant arcs

G75 - Future arcs are Full 360 arcs

G90 - Absolute data

G91 - Incremental data

Typically for laser photoplotters, G54 codes are rarely

necessary.  Older vector plotter controllers may require this

preparatory G codes for changing apertures (i.e. G54D10*).

A common situation where G codes are mandatory for all machines

is when the data is switching from vectors to arcs and vice

versa.  When switching from drawing vectors (G01) to drawing arc

(G02, G03), the controller must be informed of the change of

mode.

Another important case for G codes is when determining if the arc

is a quadrant (G74) or Full 360 (G75).  Quadrant arcs never cross

quadrant boundaries, because the center coordinate offsets (I,J

Codes) are always unsigned (even if they are negative!).

Therefore, it requires at least four G74 arcs to draw one

complete circle.

Center coordinate offsets for 360 arcs (G75) can be positive or

negative, allowing for a single command to draw a complete

circle.

In either case, the center coordinates are given relative to the

start point of the arc.  The most dramatic difference between

Quadrant and Full 360 arcs is that a Quadrant arc with identical

start and end points has a sweep of 0 degrees, whereas a similar

Full 360 arc is a full circle.

The G90 code tells the machine controller that all data following

is absolute data.  Hence, if following X & Y data follows, the

controller will move to the absolute value given by the X & Y

value.

G91 tells the machine controller that all data following is

incremental data.  The machine will move the data by the amount

of the X & Y value, rather than to the absolute coordinate point.

Example:

X1000Y1000D02*

X3000Y3000D01*

In absolute mode (G90), the machine will first move to coordinate

point X1000 and Y1000 with the light off, then draw a line to

coordinate point X3000 and Y3000 with the light on.

In incremental mode (G91)  the machine will first move to

coordinate point X1000 and Y1000 with the light off, then draw a

line to coordinate point X4000 and Y4000 with the light on.  This

was done by adding X1000 + X3000 = X4000 and Y1000 + Y3000 =

Y4000.

Here are some more examples of G code usage in conjunction with

X, Y, and D code values:

G54D10*      { Prepare to change aperture position (G54), then

select aperture D10}

G01X1000Y1000D02*      { Prepare to draw a vector (G01) then

turn off the light (D02) and move to coordinate position X1000

and Y1000}

G90*    { This block (command) and all future commands will be

absolute data}

X2000Y3000D01*    Turn the light on (D01) and move to absolute

coordinate position X2000 and Y3000}

G91*              { The G91 command tells the controller that

this command and all future commands that the data is

incremental}

X5500Y100D03*     { Turn the light off and move incrementally

by a value of X5500 and Y100, then flash (D03) (light on and

off)}

M Codes

M codes are used for machine control.  Here are the most commonly

used:

M00 - Full machine stop.  Commonly ignored by many plotters.

M01 - Temporary machine stop.  Commonly ignored by many

plotters.

M02 - End of Plot.

I & J Codes

When you encounter an I & J code in a Gerber block, you have

found an arc command.  Arc commands come in two flavors, Full 360

or Quadrant.  The Gerber arc command is very complicated, and

this section will only briefly describe usage of the Gerber arc.

Full 360 arcs allow the plotter to draw a full circle (360

degrees of arc) in one single command..

Quadrant arcs only allow for an arc to be drawn through a maximum

of 90 degrees of arc, never crossing a quadrant boundary.  Due to

this restriction, I and J arc center offset codes can get away

with never having a negative value, even if the offsets are

negative!

When in a Full 360 arc (G75), only one command is required to

draw a circle.  In Quadrant mode, the same circle would require

at leaset 4 Quadrant arcs (G74), because a circle goes through

all four quadrants.

Quadrant arcs will always have positively signed I and J values,

even if the center offset is actually negative.  Full 360 arc

center offsets can be signed positively or negatively.  A

negative I or J is a sure indicator of Full 360 arcs.

Modality

It is often the case with Gerber data that when moving from one

XY coordinate point to another XY coordinate point, the X or Y

value will not change.  Likewise, it is likely that if the

plotter is drawing a line with multiple segments, the segments

will be connected and the light stays on from segment to segment.

In both of these cases, there are redundant commands, making the

plot data file larger than necessary.

RS-274D allows you to omit this redundant data.  This example

shows a box being drawn with four corners.

Non Modal Data         Modal Data

X0000Y0000D02*         X1000Y1000D02*

X0000Y1000D01*         Y1000D01*

X1000Y1000D01*         X1000*

X1000Y0000D01*         Y0000*

X0000Y0000D01*         X0000*

From this example, a large amount of data has not been written,

thus reducing the final data file size.

Establishing the Decimal Point

A numerical value in RS-274D data has an integer and a decimal

part, but the decimal point ('.') is not a valid RS-274D

character.  Thus, decimal values are written as a string of

integers.  The implicit position of the decimal point is

determined by three parameters:

Number of integer digits (whole digits)

Number of decimal digits (precision)

Zero suppression.

For example: In a system with integer digits=n and decimal

digits=m (an "n,m" system), a numerical value is written using

(n+m) digits.

For example, in a "2,3" format the value 12.345 is written

"12345".  In a "2,4" format, the same value is written "123450".

Zero suppression comes in three flavors - leading, trailing and

none.  The idea of zero suppression is to reduce data file sizes

by eliminating unneeded 0 characters.  The simplest and most

common form of zero suppression is leading zero suppression.

In a "2,4" format, with no zero suppression, the value 0.0100

would be 00 + 01000, written as "000100", but with leading zero

suppression the same value is written as "100".

With trailing zero suppression the same value 0.0100 would be

written as "0001".

How To Describe Data Formats

Gerber data and other XY languages use a standard method for

describing the data format.  Two examples include:

"2,3 leading inch" or "3,3 trailing metric".  The first number

specifies the whole digits used.  The second parameter states

the precision.  "Leading" and "trailing" pertain to the zero

suppression.  And the last part of the description indicates

the units.  Refer to the above sections if these concepts seem

unclear.