440-3  (C) Douglas K. Heller

Here is another Kenwood TS-440 control program.

The program is pretty much straight forward, but I will

explain how it works in these docs.

You can copy the files from this disk to you hard drive

with the normal DOS command  COPY *.* Next type 440 <enter>.

The program will ask which com 	port 1 or 2.  Com 3 or 4 are

not supported. The program will try to get a code from the

rig, and if successful, you will be told that you are

either using a TS-440 or a R-5000.  If the proper code is not

received. the program will tell you so and PRESS ANY KEY to Terminate

the program.  If the program terminates, check your interface

and cabling.

If all has  worked OK, you should be in the main menu. 	Choose

ALT D to display most of the functions of the TS-440. As

you can see at the bottom of the screen there are a few function

keys that you can do some controlling of the TS-440 like input

a frequency, mode. Turn the SPLIT on and off.

When you rotate your VFO, you will notice that nothing

seems to happen on the display until you stop. This is

because the TS-440 output is rather erratic. Use ALT X to

Exit to the main menu.

Again from the main menu, Choose <ALT M>, This will read

all the memories in the Radio. After the program has read

all the memories, it will display 0 to 44. Use 	the <Down Arrow>

key to display memories 45 to 89, or 	use the <Up Arrow>

to display the Split Memories 90 to 99 usually used for

10 meter FM repeater use.

After paging though the memories you can save the contents

of the memories to disk by hitting <ALT S> key. After

saving the memories to disk use the  <ALT X> 	to get back

to the main menu.

You can use the final feature of this program to restore any

or all the memories to the TS-440 in the unlikely event

you reset the micro processor in the TS-440 or you loose

the contents of the memories and provides a short cut

to re programming the memories 	back into the TS-440.

You must have the file 	DKH440.MEM on the same disk or

in the same directory as this program. You must have saved the

contents of the memories to disk before you use this feature.

Oh yes, to exit the program from the main menu type a

<ALT X> and you will go back to dos.

One final thing, If you like this program and make use 	of it,

I would appreciate it if you would send me 10 	bucks or so.

If you don't, that's fine, enjoy the program and thanks anyway.

If you do it helps motivate me to do more and add new features

to later versions, and will be greatly appreciated!

Doug Heller  KE7NF

1121 W. Obispo

Mesa, Arizona  85210