440 MINI-SPOILER MANUAL

VERSION 2.0

The Kenwood TS-440S is a full-feature amateur-radio transceiver capable of

transmission on  all ham  bands and  reception on  all frequencies  between  30

KiloHertz and  30 MegaHertz.   It is a beautifully designed radio with a single

exception: its  front panel.   All  of the powerful features of the transceiver

must be  accessed through a very small and complex set of bounce-prone switches

and a  miniscule display.   Operating the TS-440S to take full advantage of its

features from  the front  panel  takes  extraordinary  dexterity,  memory,  and

patience.  Such is the problem.

The solution  to the  problem is as easy as controlling the TS-440S from a

Tandy Model  100 or  102 laptop  computer.   The 440  Mini-Spoiler opens up the

full-featured insides  of the  TS-440S so that they are available for efficient

and convenient  use.   In addition,  it makes  remote  operation  (changing  of

frequencies, modes,  memories, split  operation, and  the like)  possible  over

distances of up to 100 feet by hardwire connection and up to thousands of miles

by modem connection.

We at the QS02000 Project hope that you find your amateur-radio operations

more pleasant and productive because of the 440 Mini-Spoiler.

The 440  Mini-Spoiler is  brought to you through shareware marketing.  You

may try  the package  for up  to one  month at  no cost  whatsoever, and if you

during that  time find the package to be of sufficient value to you to continue

its use, we ask you to remit the $16.50 registration cost.

The QSO2000 Project sponsors amateur-radio software development to provide

operators with  the latest  in excellent operating tools.  You help the project

to develop  more high-quality  software at  a more  rapid pace when you support

software from  the project that you are using.  Your contribution also helps to

ensure that  the 440  Mini-Spoiler will  be maintained and further improved for

your use in the future (see the Release Notes at the end of this document).

440 MINI-SPOILER: A SHAREWARE PACKAGE

Version  2  of  the  440  Mini-Spoiler  has  cost  hundreds  of  hours  of

development and  testing efforts, but it is yours for one month to try for your

own purposes.   If  you use  it for more than a month, the QSO2000 Project asks

you to share in the development costs by registering your copy of the 440 Mini-

Spoiler for  the nominal  fee of  $16.50.  Your contribution to the maintenance

and further  development of  the software  package does, on average, determines

whether the  package continues to be developed as a useful product for years to

come.

When you  register the  package, you  may, for  a $2.50 media and handling

charge, receive  the latest  version of  the  440  Mini-Spoiler  on  Model  100

cassette tape, portable-drive 3-1/2 inch disk, or Disk/Video Interface (DVI) 5-

1/4 inch  disk.   This version  includes advanced  features  such  as  Transmit

control and  memory-file save  and  load  provisions,  features  that  are  not

available in the shareware examination copy.

The advanced  features that  are not included in the shareware examination

copy of  version 2  of the 440 Mini-Spoiler are the file commands (function key

6) and  the transmit  command (function  key 7).   These features are available

only to  owners of  registered copies  who remit  an additional $2.50 media and

handling charger for tape or diskette of their registered copy.

Shareware marketing  provides a  way for the QSO2000 Project to distribute

high-quality amateur-radio  software at a minimal cost to operators who use the

package.   When you  register your copy, you provide for further development of

the 440 Mini-Spoiler and other amateur-radio packages written by members of the

QSO2000 project.

LOADING THE 440 MINI-SPOILER INTO THE MODEL 100/102

The 440  Mini-Spoiler is  a Model  100/102 BASIC program in ASCII (text or

.DO) format.   You  may import  it into  your Model  100/102 by connecting your

Model 100/102  and the  computer you  downloaded the  file with together with a

"null-modem" cable  that reverses  the signals on pins 2 and 3 (and maintains a

ground on  pin 7  on both ends) of a standard 25-pin "D" connecter.  Set up the

Model 100/102's  TELCOM program  to download,  and initialize a program in your

other computer to upload.

The 440 Mini-Spoiler program (4MS200.BAS) is a Model 100/102 BASIC program

in ASCII  format.  You may import it into your Model 100/102 by connecting your

Model 100/102  and the  computer you  downloaded the  file with together with a

"null-modem" cable  that reverses  the signals on pins 2 and 3 (and maintains a

ground on  pin 7  on both ends) of a standard 25-pin "D" connecter.  Set up the

Model 100/102's  TELCOM program  to download, and initialize a program (such as

QMODEM) in  your other computer to upload.  Transfer speeds up to 9600-baud may

be achieved  if you  enable XON/XOFF handshaking in both TELCOM and your MS-DOS

program.  When you upload it to your Model 100/102, you must set up your MS-DOS

program to  cancel line-feeds as it sends the file.  At the TELCOM end, specify

the download-file name as MS4200.DO.

When you  have the  program downloaded into Model 100/102 memory, you must

load it into Model 100/102 BASIC.

One of  the most  frustrating features  of Model  100/102 BASIC is that it

will not  run a program which has not been converted into internal, "tokenized"

(slightly compressed)  format.   Therefore the  computer must  convert the  440

Mini-Spoiler to  tokenized  format  before  the  Model  100/102  will  run  it.

Unfortunately, Model  100/102 BASIC  converts the entire file while retaining a

copy of  the ASCII  file in  memory.   This means  that you  must have  a Model

100/102 with  32 kilobytes  of memory  to perform  the conversion.  (Disk Video

Interface--DVI--users have  the memory  requirement relaxed,  because they  can

convert a  text file  stored on  disk into  a tokenized  file in memory without

incurring the huge memory overhead.)

Once you  have MS4200.DO  in memory,  simply enter  "LOAD MS4200.DO"  from

BASIC and  you're ready to run the program.  You will likely find it convenient

to save  the program as a memory file by entering "SAVE MS4200" while BASIC has

the MS4200.DO  program loaded  in tokenized  format.  The program is saved as a

memory file with the name MS4200.BA, and you may use the KILL command to delete

the MS4200.DO file in order to conserve memory.

The simplest  solution to  the loading  problem is  to obtain a registered

copy of  the 440  Mini-Spoiler in BASIC (MS4200.BA) format which has been saved

on cassette tape, 3-1/2 inch portable-drive disk, or 5-1/4 inch DVM disk by the

QSO2000 Project.

HARDWARE

The 440  Mini-Spoiler, written  in optimized BASIC-language code, will run

in a  Model 100/102  with as little as 16 Kilobytes of RAM installed, if you in

accordance with  the cautions  given above can get it loaded in .BA format from

an outside device.

You must  have your  TS-440S completely  outfitted for the standard RS-232

interface;  Kenwood   left  a   couple  of   empty  integrated-circuit  sockets

unpopulated behind  the front  panel,  and  you  need  an  external  RS-232/TTL

converter.   Both Kenwood  and at  least two  independent suppliers market both

items.

Kenwood designates  its pair of IC chips the IC-10 and its external RS-232

level converter the IF-232C product models.

Phil Whitehouse, W1GEE, offers a much more economical alternative in a kit

that includes  both the  TS-440S chip set and an RS-232/TTL level converter for

$50 (assembled  price $60).   Phil's  level  converter  draws  power  from  the

transceiver in  order to  minimize wiring clutter.  You may contact Phil at the

following address.

Mr. Phil Whithouse, W1GEE

P. O. Box 527

Hampden, Maine 04444

Lawrence Anderson,  KA6FQN, offers  a chip  set and  level converter  in a

small package  in assembled  form for  $59.   The level  converter, cables, and

instructions alone  (without the  internal chip set) is priced at $49.  You may

contact Lawrence at the following address

Mr. Lawrence R. Anderson, KA6FQN

35615 Avenue D

Aucaipa, CA 92399

The QSO2000  project has  tested both  Phil's and  Larry's assembled level

converters and found found them both to be extremely reliable, easy-to-install,

and attractive.

STARTING UP THE 440 MINI-SPOILER

When you  have the 440 Mini-Spoiler stored as a .BA file in memory and the

cables connecting  your Model  100/102 to  the RS-232/TTL  converter  and  your

fully-chipped TS-440S,  all you  need to  do is  run  the  program.    It  will

interrogate your  transceiver for frequencies and modes stored in both VFOs and

the 90  non-split memories.   During  your initial installation, you'll have to

wait approximately  30 seconds  amid entertaining beeps from your Model 100/102

for the  memories to  be loaded  and the  data to  be converted  to displayable

format.   After your  initial installation, you have no such wait to be patient

with.  The 440 Mini-Spoiler will assume that you have not altered your memories

manually, and it will load in the previously stored memories from a file called

MEMFLE.DO in Model 100/102 in RAM.

NOTE:     To test  your hardware  link if  you find  that the  440 Mini-Spoiler

fails to  work for  you, break  (ctl-pause) out of the spoiler.  Exit

BASIC and  enter TELCOM.    Choose  communication  parameters  (STAT)

78N2E,  giving   you  the   4800-baud,  8-bit-character,   2-stop-bit

convention that the TS-440S requires.  Enter TERM, and type "IF;" (no

ENTER,  please:   it  will   only  interfere,   because  the  TS-440S

understands the  semicolon to  be an  enter signal).  If your TS-440S

doesn't immediately  respond to  the Model  100/102  screen  with  38

characters beginning with "IF", continuing through many numbers and a

few spaces,  and ending  with ";",  you do have some misconnection or

malfunction in  your hardware.   If  you get the right response, your

communication link  is working  properly and you've may have gotten a

corrupted copy  of the  440 Mini-Spoiler.   Obtain  another copy post

haste.

DATA DISPLAY AND DATA ENTRY

The 440  Mini-Spoiler is  designed to  provide totally separate data-entry

and data-display  tasks.   You may  use either  the TS-440S  front panel or the

Model 100/102  keyboard for  entry at  any time,  and the resulting changes are

reflected on both the transceiver display and the 440 Mini-Spoiler screen.

At least  some TS-440S  transceivers include  front-panel keypads that are

extremely prone  to contact bounce.  An annoying result of erroneous data entry

is that  the transceiver  receives multiple  number entries  from its own front

panel when  you press  a button  only once.   A more serious result is that the

transceiver can  send out  status-update packets  at such  a rapid pace that it

overloads the  440 Mini-Spoiler.   Such  a jam  occurs most often with entry of

memory frequency data on the transceiver's numeric keypad.  If your transceiver

has such  switch-bounce difficulty,  you can  avoid the problem by entering new

VFO and  memory data  through the  440 Mini-Spoiler  keyboard (it's  much  more

convenient and less troublesome to enter it that way otherwise too).

THE 440 MINI-SPOILER SCREENS

The 440  Mini-Spoiler initializes  your Model  100/102 while it displays a

title screen; it reads data from your TS-440S.  It also dallies a little bit to

thoroughly display  the copyright notice to remind you that much work and other

resources have gone into development of the program.

After the  title and  copyright screen  disappears, the  440  Mini-Spoiler

operating display  appears.   The display  contains three areas: (1) an area to

the left  of the  screen for  display of  the VFOs  and the  currently selected

memory, (2)  an area  to the right of the screen for displaying any five of the

90 simplex  memories, and (3) a line across the bottom of the screen to display

command prompts.  Figure 1 shows a typical operating display upon startup.

�����������������������������������������ͻ

��������������������Ŀ������������������Ŀ�

��FQ 28.718.87 USB VA��00  3.933.00 LSB U��

���������������������ٳ01 21.023.00 CW  U��

��������������������Ŀ�02 14.300.00 USB U��

��    7.213.02 LSB VB��03 28.454.00 USB U��

��   14.300.00 USB 02��04  7.085.00 FSK U��

�����������������������������������������ٺ

�Slct Freq Mode Copy Wndw File TRtg  Quit �

�����������������������������������������ͼ

[1]  [2]  [3]  [4]  [5]  [6]  [7]   [8]

Figure 1.- Typical operating display

VFO and Current Memory Area

Your current  operating frequency (or frequencies) and mode (or modes) are

shown at  the top of the VFO and Current Memory Area.  The top box displays the

currently selected  transmission and  reception source(s).   From figure 1, the

source for both reception and transmission (indicated by "FQ" for frequency) is

VFO A at a frequency of 27.71887 MegaHertz in upper-sideband mode.

For single-source,  non-split operation,  the top box contains information

about VFO A, VFO B, or the currently selected memory.  For example, if memory 2

is selected as the source, the display changes to that shown in figure 2.

�����������������������������������������ͻ

��������������������Ŀ������������������Ŀ�

��FQ 14.300.00 USB 02��00  3.933.00 LSB U��

���������������������ٳ01 21.023.00 CW  U��

��������������������Ŀ�02 14.300.00 USB U��

��   28.718.87 USB VA��03 28.454.00 USB U��

��    7.213.02 LSB VB��04  7.085.00 FSK U��

�����������������������������������������ٺ

�Slct Freq Mode Copy Wndw File TRtg  Quit �

�����������������������������������������ͼ

[1]  [2]  [3]  [4]  [5]  [6]  [7]   [8]

Figure 2.- Typical memory-selected display

The 440 Mini-Spoiler supports split-VFO operation, but it does not support

split-memory operation  (memories 90  through 99 only).  During a period when a

memory is  selected, the  top box will contain a single line with memory number

appended to  indicate the  frequency (FQ) and mode of the memory that currently

controls the transceiver.

When a  VFO is  selected, it will contain one or two lines to respectively

indicate the  VFO controlling  the transmit-and-receive frequencies or both the

VFO controlling  the receive  frequency and  the VFO  controlling the  transmit

frequency.

When you  have set  up the  transmitter for split-frequency operation from

the VFOs,  the receive VFO information is always shown on the top line of the a

two-line box  (with an  "RX" prefix).  The transmit VFO is shown on the bottom,

with a  "TX" prefix.   One  of the  advantages of using the 440 Mini-Spoiler is

that you  always be  able to  see your  transmit frequency  before you  press a

microphone button  or tap  a key.   Figure 3 shows a typical display in the VFO

and Current  Memory Area  when the  TS-440S  is  set  for  splif-frequency  VFO

operation.

�����������������������������������������ͻ

��������������������Ŀ������������������Ŀ�

��RX  7.075.00 LSB VA��00  3.933.00 LSB U��

��TX  7.213.00 LSB VB��01 21.023.00 CW  U��

���������������������ٳ02 14.300.00 USB U��

��������������������Ŀ�03 28.454.00 USB U��

��   14.300.00 USB 02��04  7.085.00 FSK U��

�����������������������������������������ٺ

�Slct Freq Mode Copy Wndw File TRtg Quit  �

�����������������������������������������ͼ

[1]  [2]  [3]  [4]  [5]  [6]  [7]  [8]

Figure 3.- Split-VFO display

The frequencies  shown in the VFO and Current Memory Area are the absolute

frequencies of  the VFOs  and the  selected memory.   The  frequencies  do  not

account for  any "increments"  that you have added to them from the transceiver

front panel with the RIT or XIT controls.

Five-Memory Window

Upon initialization,  the Five-Memory  Window displays the frequencies and

modes stored in memories 0 through 4.  One of the memories (initially memory 0)

is highlighted  to preset easy access to it by keyboard for transceiver control

or copying.   You can easily scroll through the entire 90 non-split memories or

select by keyboard any five-memory segment of them for display.

The Command Line

The eight  abbreviated commands  shown above  the function-key labels lead

you through  prompts to enable you to control your TS-440S transceiver and your

440 Mini-Spoiler display.  The commands and the prompts that they further offer

you are described in detail below.

440 MINI-SPOILER COMMANDS

Brief Definitions

Source (F1)............................Select xmit and rcve vfo or memory

Frequency (F2).........................Set frequency in any vfo or memory

Mode (F3).........................Set any VFO or memory to LSB, USB, etc.

Copy (F4)..............................Copy freq and mode between sources

Window (F5).........................Set position of memory viewing window

Files (F6)..............Load or save a Model 100/102 file of 440 memories

TR toggle (F7)...............Toggle the XCVR between transmit and receive

Quit (F8)...................................Return computer to MENU level

The commands  are entered through function keys F1 through F8 just beneath

the Model 100/102 LCD display.

All commands  other than  the Quit  Command provide  you with  prompts  to

direct your  entries.  These prompts include a prompt to return you to the main

level of  commands: a Quit Option.  Entering the first letter of an option name

evokes the option.

Memory-Window Hot Keys

Pressing the  Down-Arrow key moves the highlight in the five-memory window

down one  memory in  the memory  to the  next-highest memory  number.   If  the

highlight is  on the  fifth memory,  the memories will scroll so that the next-

highest memory will appear highlighted at the bottom of the window.

Similarly, pressing the Up-Arrow key moves the highlight up in the window,

and if  the highlight  is at  the top  of the  window the  key will  cause  the

memories to  scroll so  that the  next lower-numbered memory will appear at the

top of the window.

If you  hold down the Shift key while you press the Up- or Down-Arrow key,

memories in the window scroll up or down by five.  The highlight remains in the

same position it was in before you press the key.

You may use function key 5 to select the memories appearing in the window,

as described below.

Select (Slct) Command (F1)

The Select  Command offers  you transceiver  control by either VFO, by the

memory currently  selected by  the  TS-440S,  by  the  memory  which  you  have

highlighted in the five-memory window, or by a memory with the number you enter

by keyboard.   The  split option  is also  available to  toggle  dual-frequency

transmit and receive by VFOs.

The prompts  for the Source Command, which temporarily replace the Command

Line, are shown in figure 4.

�A)B)VFO M)em W)in ## S)plit Q)uit ? �    �

�����������������������������������������ͼ

[1]  [2]  [3]  [4]  [5]  [6]  [7]  [8]

Figure 4.- Source Command prompts

Entering A,  B, or  M immediately  puts VFO  A, VFO  B, or  the  currently

selected memory  in control of the TS-440S.  Entering W puts the memory that is

highlighted in  the Five-Memory Window in control, simultaneously making it the

currently selected  memory.  Entering a number between 0 and 89 puts the memory

with that  number in  control and  makes it  the current memory.  The 440 Mini-

Spoiler will not control split memories 90 through 99.

Entering S  toggles VFO split-frequency operation on and off.  This option

is effective  whether the transceiver is currently controlled by a memory or by

VFO.   With a  memory in  control, for example, you can toggle split on so that

when you  change to  VFO operation  the  transceiver  will  be  set  for  split

transmission and reception.

Pressing Q  cancels the Source Command and returns the Command Line to the

bottom of  the display.  You may also cancel the command by pressing the Return

key.

Frequency (FREQ) Command (F2)

The Frequency Command permits you to change the frequency of any source in

your transceiver (except the duplex sources, memories 90 through 99).

The first  prompt that  replaces the  Command  Line  after  the  Frequency

Command is  entered asks you whether you to enter the source you want to change

the frequency of.  This prompt is shown in figure 5.

�A)B)VFO M)em W)in ## Q)uit ? �           �

�����������������������������������������ͼ

[1]  [2]  [3]  [4]  [5]  [6]  [7]  [8]

Figure 5.- Frequency Command first prompt

With entering a single character, you indicate that you want to change the

frequency of VFO A, VFO B, the currently selected memory, or the memory that is

highlighted in  the Five-Memory  Window.  You may enter a number ranging from 0

to 89  to change the frequency of any memory, regardless of whether it is shown

on the  screen.   You may  enter Q  to terminate  the Frequency Command without

changing the frequency of any VFO or memory.

The second  prompt that  replaces the  Command Line  after  the  Frequency

Command is  entered is  simply "Frequency  value or Q)uit?"; you must enter the

frequency that  you wish  to assign to your selected VFO or memory or enter the

single-character Q option (or Return) to terminate the command.

Within useful  limits, you  may  enter  frequencies  in  KiloHertz  or  in

MegaHertz.   For frequencies  up to 9,999,999 Hertz, you have a choice of entry

methods.   You may enter them in either MegaHertz, with a decimal point, or you

may enter  them in  KiloHertz, suffixed  by a  "K" or "k" to tell the 440 Mini-

Spoiler the  difference.   An entry  of "7.12345"  is identical  to an entry of

"712345K" and "712345k."

If you  enter a  frequency not  formatted in  these two  fashions, the 440

Mini-Spoiler will  beep and  query you again for the frequency.  If you enter a

frequency that  is outside  the range of the TS-440S transceiver, the 440 Mini-

Spoiler will also beep and query you again.

Mode (MODE) Command (F3)

The Mode Command permits you to change the mode (LSB, USB, CW, AM, FM, and

RTTY) of  each transmission  source.  The first of two prompts that the command

brings up is shown in figure 6.

�A)B)VFO M)em W)in ## Q)uit ? �           �

�����������������������������������������ͼ

[1]  [2]  [3]  [4]  [5]  [6]  [7]  [8]

Figure 6.- Mode Command first prompt

With entering a single character, you indicate that you want to change the

mode of  VFO A,  VFO B,  the currently  selected memory,  or the memory that is

highlighted in  the Five-Memory  Window.  You may enter a number ranging from 0

to 89  to change  the mode  of any memory, regardless of whether it is shown on

the screen.  You may enter Q to terminate the Mode Command without changing the

mode of any VFO or memory.

The second prompt that replaces the Command Line after the Mode Command is

entered is shown in Figure 7.

�L)SB U)SB C)W A)M F)M R)TTY Q)uit ? �    �

�����������������������������������������ͼ

[1]  [2]  [3]  [4]  [5]  [6]  [7]  [8]

Figure 7.- Mode Command second prompt

Your response to the prompt selects the mode of the VFO or memory you have

chosen or (with W for Quit) terminates the command without changing mode.

Copy (COPY) Command (F4)

The Copy  Command permits  you to  copy the  contents of any VFO or memory

into another  VFO or  memory.   You are  first prompted for the source that you

want to copy from, and then you are prompted for the destination of the copy of

the frequency  and mode  information.  The prompt for source is shown in figure

8.

�From: A)B)VFO M)em W)in ## Q)uit ? �     �

�����������������������������������������ͼ

[1]  [2]  [3]  [4]  [5]  [6]  [7]  [8]

Figure 8.- Copy Command source prompt

The prompt for destination differs only slightly, as shown in figure 9.

�To: A)B)VFO M)em W)in ## Q)uit ? �       �

�����������������������������������������ͼ

[1]  [2]  [3]  [4]  [5]  [6]  [7]  [8]

Figure 8.- Copy Command destination prompt

You may  terminate the  Copy Command either at the time it prompts you for

the source or at the time it prompts you for the destination.

Window (WNDW) Command (F5)

The Window  Command  lets  you  rapidly  change  the  memories  which  are

displayed in  the five-memory  window.   You are prompted for the number of the

memory that  will be  the lowest  of the  five to appear in the window.  If you

choose memory  38, for  example, you  will cause  memories 38  through 42 to be

displayed in the five-memory window.

The Up  and Down Arrow keys work in conjunction with the Window command to

let you  see and  select any  memory from 0 to 89.  Used without the shift key,

the arrow  keys move the inverse-video selection bar up and down in the window.

Used when  the shift key is pressed, the arrows cause the window to move up and

down on the list of memories in increments of 5, while the highlight remains in

the same  position relative  to the  top of  the window.  Pressing [SHIFT-DOWN]

when the  window is  displaying memories  5 through 9 causes the window to move

further down  the list to display memories 10 through 14.  In this instance, if

memory 5 is highlighted before you hold down Shift and press Down Arrow, memory

10 will be highlighted after your scrolling down the memories.

File (FILE) Command (F6)

The file  command allows  you to  S)ave  current  transceiver  memories  0

through 89  to a  Model 100/102  file, and  it allows you to L)oad your TS-440S

with memories previously saved to a file.

NOTE:     The memory-file  command options  are functional  only in  registered

copies of  the 440 Mini-Spoiler obtained on tape or diskette from the

QSO2000 Project.   In  unregistered copies, pressing F6 will cause an

exciting small  beep to  sound and  the command menu to return to the

last line of the screen.

The file may be a memory file (file name only), a cassette-tape file (file

name preceded  by CAS  as in  CAS:MEMFLE), or a disk/vidio (DVI) interface file

(with a 0 or 1 and a colon preceding a file name).

The memory  file named MEMFLE (MEMFLE.DO) will always be loaded by the 440

Mini-Spoiler during  initialization.   This file  contains the values stored in

TS-440S memories  the last  time the  440 Mini-Spoiler  has been run and exited

with the F8 key.  Anytime the 440 Mini-Spoiler is run, the F6 key may evoke the

S)ave option  in order  to save  a file to memory, tape, or DVI disk, and a new

file of memories can be loaded with the F6 key via the L)oad option.

The non-split  memories in  the TS-440S will be set to the frequencies and

modes set  to the  values specified  by whatever  file you  load from memory or

diskette.   They will  NOT remain as were when you initialized the program with

MEMFLE, and  they will not reflect any changes you have made since initializing

the program.

NOTE:     When you  exit the  440 Mini-Spoiler  with the  F8 key,  the  current

memory file  (the previously  loaded file  altered by  any changes in

frequency and mode and any memory copy operations) is saved as MEMFLE

(MEMFLE.DO) in  order to  keep transceiver  memories and  file-stored

memories identical.   If  you have  loaded another  memory file since

initializing the  program, that file and any changes you have made to

it will  be reflected  in both  your TS-440S  memories and  the  file

MEMFLE.  If you have doubt about whether the transceiver memories are

identical to  the file-stored  memories, press F6 and enter the S)ave

option with  a file  name of  MEMFLE.DO to  refresh both the memories

that  the  Spoiler  currently  recognizes  and  the  stored  file  of

memories.

The file  MEMFLE (MEMFLE.DO) and all other memory files that you save with

the F6 file command and S)ave option are files that you can edit with the Model

100 Text  editor.  You may alter the frequencies and modes of the memories with

TEXT, providing  that you  carefully (1)  maintain memory  sequence numbers and

frequency and  mode fields within each line and (2) use the L)oad option of the

F6 command  to enter  new frequencies  and modes  into the  TS-440S  after  you

initialize the program.

CAUTION:  If you  exit the  440 Mini-Spoiler  through any  means other  than by

pressing F8,  the current  state of memories in your transceiver will

not be  saved to  MEMFLE.   You then  have two options to "reconcile"

transceiver memories  with computer  memories.   F6  with  the  S)ave

option to the file name MEMFLE will reconcile computer-saved memories

to those stored in the transceiver, and F6 with the L)oad option will

reconcile transceiver memories with those stored in Model 100 RAM.

Transmit/Receive Toggle (TRtg) Command (F7)

You can  toggle between  receive and transmit operation with the TR toggle

command.   When the transceiver is in transmit mode, a transmit sign appears to

the right  of the  current source  designation (if  non-split)  or  beside  the

receive source (if split VFO operations is in effect).

NOTE:     The transmit  command is  functional only in registered copies of the

440 Mini-Spoiler  obtained on  tape  or  diskette  from  the  QSO2000

Project.   In unregistered copies, pressing F7 will cause an exciting

small beep  to sound  and the command menu to return to the last line

of the screen.

Quit (QUIT) Command

The Quit  Command does  exactly what  it says:  it causes  control of your

Model 100/102  to return  to the  Menu.  Whenever you want to run the 440 Mini-

Spoiler again,  simply place the inverse-video cursor over MS4200.BA again, and

press the ENTER key.

RELEASE NOTES

Version 2.0  of the 440 Mini-Spoiler is released November 15, 1990 with no

currently known bugs.  Maintenance releases which correct inevitably discovered

bugs (as they are discovered) will be designated by incremented version numbers

(2.01, 2.02, and so on).

Version 2.00 supersedes version 1.01, released March, 1990.

OTHER QSO2000 PROJECT SOFTWARE PACKAGES

Please type  or print  the file  QSO2000.TXT on  this diskette  or in this

archive.

REGISTRATION AND LICENSING INFORMATION

If you  find the 440 Mini-Spoiler to be of sufficient value for you during

your one-month  trial period,  please remit  $16.50 to the QS02000 project.  If

you would  like a  copy of  the 440  Mini-Spoiler  with  enhanced  features  in

tokenized format  to be mailed to you on cassette tape, 3-1/2 inch diskette for

the Tandy  Portable Disk Drive, or 5-1/4 inch diskette for the Tandy Disk/Video

Interface, please  include an  additional $2.50  to cover  copying and  mailing

charges (total $19.00).

Please view  the file  RGSTR4MS for  detailed licensing  information and a

printable registration form.

Bill Brewer, K5KNC, Coordinator

QSO2000 Project

5518 12 Street

Lubbock, TX 79416