-----------------------------------------------------------------

The  following article is a scanned copy from page 52-56  of  the

November 1990 issue of CQ magazine.

-----------------------------------------------------------------

For many of us AMTOR is an abstraction., a simple click away on a

mode switch. What is it? How does it work? How do I use it? These

and many more questions are answered by K9GWT in simple, everyday

terms.

AMTOR For Beginners

BY BILL HENRY*, K9GWT

AMTOR is a relatively new and exciting "RTTY-type" mode. However,

I find that many would-be "AMTORites" suffer from "chirp fright."

At  first,  the alphabet- soup of acronyms,  strange  modes,  and

transmitter pulsing can be very intimidating. Not to worry.  It's

all  quite  simple once you get your feet wet.  This is  not  the

first-nor  the last-beginner's article on AMTOR.  The  references

list  a number of other authors and articles on the  same  topic.

You  may  find that their words are even  more  informative  than

mine. Multiple opinions always help. Take heart in the  knowledge

that AMTOR is actively used by many amateurs and is growing.  You

do not have to be a Ph.D. or computer-whiz to have fun on AMTOR!

What Is AMTOR?

Very  simply, AMTOR is "modern RTTY." It is very similar in  many

ways  to the plain old "teletype" we have used since  the  1950s.

AMTOR is also similar to packet. Some would say that AMTOR is the

"technical  bridge" between RTTY and packet. However you want  to

think  of it, AMTOR is a mode which we use to send text back  and

forth  between radio stations Like RTTY (and packet), AMTOR  uses

an FSK (Frequency Shift Keyed) modulation mode. Unlike RTTY,  but

similar  to  packet,  AMTOR includes error  correction.  On  poor

signals,  receiving  RTTY can become tedious as  the  errors  and

misprints  pile  up. A good RTTY modem can help, but  there  will

still  be  some errors when the other fellow's  signal  fades  to

"zip."  Not generally so on AMTOR. It just keeps repeating  until

it "gets it right"!

AMTOR Modes

AMTOR  is  not just one mode to send text;  it  includes  several

related, but different modes. The major modes of AMTOR are:

ARQ: In ARC mode the sending station sends three characters at  a

time. He then turns his transmitter OFF and the receiving station

sends a one-character control signal to signify "OK, go ahead" or

"Please Repeat Last Group. " I n ARQ mode both stations alternate

between transmit and receive. The station sending text is  called

the  ISS for "Information Sending Station," and the  station  re-

ceiving  text is called the IRS for "Information  Receiving  Sta-

tion." ARQ mode is the normal mode used by amateurs when in  QSO.

ARQ  may also be called "ModeA" on some controllers. This is  the

"chirp-chirp" stuff you hear between 14070 and 14080! ARQ mode is

similar to packet in this respect.

FEC: FEC (Forward Error Correction) mode is a lot like RTTY.  One

station  turns on his transmitter, sends an entire  message,  and

then  stands-by for the other station to transmit  his  response.

FEC does not have the "Acknowledge/Repeat" pulse exchange of  ARQ

mode  for error correction. However, FEC corrects errors  by  the

simple  expedient of transmitting each character twice.  The  re-

ceiving  controller  has the "smarts" to know when an  error  has

occurred and to then look at the repeated character. FEC mode  is

primarily used for calling CQ to set up a QSO in ARQ mode, but it

may also be used for RTTY-like QSOs and is especially useful when

"broadcasting" bulletins to many stations. The W1AW bulletins are

sent  via FEC; commercial shore stations often  transmit  weather

information  using FEC mode. On some controllers FEC mode may  be

called "Mode B" or "Mode F."

SFEC: SFEC (Selective FEC) mode is almost identical to FEC  mode,

but it includes a special call-up code so that it may be directed

10  selective  stations,  your controller won't  lock  onto  SFEC

unless  you have coded the same Selective Call sign as that  sent

by the transmitting station. SFEC has to date seen little use  in

amateur radio. Its major use is for messages to a group of  ships

in the maritime service. SFEC may or may not be included in  your

AMTOR controller. SFEC is sometimes called "Mode S."

STBY:  STBY (Standby) Mode is just what it sounds like.  This  is

the normal "rest" condition of your AMTOR controller when you are

monitoring and waiting for someone to call you. AMTOR controllers

are  intelligent and can distinguish between ARQ, FEC,  and  SFEC

signals.  When a valid AMTOR signal is received (and you  have  a

"SELCAL  match" in ARQ or SFEC mode-more on this in a  bit),  the

con  troller  automatically  switches to the  correct,  mode  and

starts  printing.  The  AMTOR controller should  not  respond  or

produce  receive print when a RTTY, CW, or packet signal  is  re-

ceived.

MON: MON (Monitor) mode allows you to listen to ARQ, FEC, or SFEC

signals.  MONitor  is  also called "LISTEN" mode  on  many  AMTOR

controllers. This is a receive-only mode and therefore you do not

get  the error-correction feature when listening to ARQ  signals.

The  controller does the best it can, but you will  probably  get

some "hits" that would not be noticed if you were "connected"  in

ARQ  mode. In some AMTOR controllers, MONitor mode  automatically

accepts and prints ARQ, FEC, and SFEC signals: in others, MON  or

LISTEN  will only receiver ARQ mode signals and you must go  back

to STBY mode to listen to FEC or SFEC signals. Read your  manuals

on  this one! Monitor is an amateur invention for  AMTOR  (credit

G3PLX   1,2,3); it is not specified by CCIR-476 or CCIR-625,  and

is generally not available in commercial "SITOR" equipment.

What Is This

"SELCAL" Business?

In  ARQ  mode (and SFEC mode) a special "station  identifier"  is

sent  at  the start of the "link."  This  "identifier"  specifies

which station you wish to establish communications with. With out

it,  all AMTOR stations on your frequency might start  "chirping-

back" to you and your controller could not sort out the mess. The

acronym  "SELCAL" stands for SELective CALL. The  SELCAL  letters

can actually be any letters we choose, but amateur convention  is

to  use a contraction of your call sign to make up your own  SEL-

CAL.  Eddie Schneider. (G0AZTP4) has the best table for  figuring

out your own SELCAL.

1 x2 Callsign-Drop the # and duplicate the first letter (example:

K9CW SELCAL = KKCW).

1 x 3 Callsign-Drop the # and use all four letters (example K9GWT

SELCAL = KGWT).

2x 1 Callsign-Drop the # and duplicate the first letter (example:

KS91 SELCAL = KKSI).

2  x  2 Callsign-Drop the # and use all  four  letters  (example:

AB1CD SELCAL = ABCD).

2  x  3 Callsign-Drop the # and second  letter  (example:  WA9YLB

SELCAL = WYLB).

This  is  a very convenient system first pro posed  and  used  by

Peter Martinez, G3PLX. It is logical, easy to figure out, and  in

use by practically all amateurs. However, it is not infallible 1.

Consider the SELCAL for W9CD, "WWCD." The above formulas also say

that  "WWCD';  would  be used by W9WCD,  W1CD,  W6WCD,  and  even

"WW9CD"  (if  we ever get to that call sign).  Fortunately,  this

happens  rarely,  but  will occur more often as  AMTOR  grows  in

popularity. The "solution" is simply to pick another  four-letter

combination.

Common Misconception: The FCC (or any other regulatory body) does

not require that your AMTOR SELCAL be related to your call  sign!

The practice is simply a convenient way to construct a SELCAL. It

is not required and a SELCAL does not relieve us from sending our

full callsign for identification'.

Recently, the FCC added a new version of AMTOR to Part 97 of  our

Rules and Regulations, the "CCIR-625" version. The major  differ-

ence  between CCIR-625 and the original CCIR-476 version is  that

the  new  version has a seven-letter rather  than  a  four-letter

SELCAL.  At  this writing, CCIR-625 is brand new and only  a  few

amateurs  have  made  use of it. The  seven-letter  SELCAL  does,

however, solve the confusion caused by the four-letter  algorithm

discussed  above. To date, there is no one strong  recommendation

for just how these seven letters are chosen. I suggested one idea

in  November 1989 CQ (K9GWT5), but there are many  other  schemes

that will also work. CCIR-625 includes "downward  compatibility",

so  don't  fear,  it will also recognize your  .,  older  format"

signal  and  communicate.  CCIR-625 includes several  other  nice

features  and some much-needed clarifications of "gray-areas"  of

CCIR-476. Use of CCIR-625 will increase as our older AMTOR equip-

ment is upgraded or replaced.

I  should  also mention that this whole business  of  "making-up"

letter-style SELCALs is strictly an amateur radio invention.  The

commercial SITOR systems are required to use number SELCAL  codes

that relate to a permanently assigned ship or land-based  station

identifier.  In theory, each marine ship or shore station  has  a

unique  SELCAL  that can be used to identify the  station  in  an

emergency. In practice, that wasn't the case for CCIR-476,  since

the  available  number pool has long since been  exhausted.  This

was,  in fact, the major reason for commercial adoption of  CCIR-

625.  CCIR 491 -1 defines how these SELCAL numbers are  converted

to  letters  that are actually sent on the radio  link.  CCIR-476

will  support 4- and 5-digit numbers CCIR-625,  9-digit  numbers.

Amateurs are not required to use CCIR-491 procedures to define  a

SELCAL. Be glad,as it's not a simple translation! G3PLX's  scheme

makes a lot more sense.

How Do I Use

This SELCAL            Thing?

Simple!  You  "mash" the magic SELCAL but ton on  your  terminal,

type in four letters, and away you go-hi! Actually, it's a little

more involved than that, but not by much.

In  ARQ mode there are two SELCAL codes to worry about-yours  and

the  other  guy's. Your SELCAL identifies your  station  so  that

other  stations may call you and "fink-up." Assuming that you  do

not change callsigns often, this is something you have to do when

the controller is new and may never have to mess with again  This

SELCAL may be called "LOCAL SELCAL," "MYSELCAL," "MYS," or "MYA."

Whatever  it's called, read your manual, enter four letters,  and

store  it ("perm" on some controllers; save "CONFIG" on the  PCI-

3000).

The  second  ARQ  SELCAL is the one of the  other  station.  This

SELCAL  will  need to be changed each time you call  a  different

station. This SELCAL may be called "REMOTE CALL," "HISSELCAL," or

just  be a prompt when you start to send in ARQ mode.  Enter  the

four letters corresponding to the other station's SELCAL.

There  are actually two more SELCAL codes, both  associated  with

SFEC  (Selective  FEC) mode. These are often  called  the  "GROUP

CALLs"-"local" and "remote." They will only be required when  you

are  receiving  or sending in SFEC mode.  The  "local"  GROUPCALL

corresponds  to the SFEC SELCAL that your station will  recognize

when receiving SFEC messages. The "remote" GROUP CALL is the SFEC

SELCAL  you will send when transmitting in SFEC.  Other  stations

must have this same SELCAL loaded as their "local GROUP CALL"  to

receive your SFEC transmission.

How Do I Call CQ?

We  use  FEC mode when calling CQ in AMTOR. This is a  whole  lot

like RTTY, but with some special recommendations:

1.  Keep  your CQ short-2 or 3 lines. A short CQ is  just  common

sense-and  courteous. Two or three short CQs on a frequency  will

probably  be a lot more productive than a long wordy call. I  may

have gone to sleep by the time you get done calling!

2. Include your SELCAL as part of the CQ text ' What you want  is

for the other guy to call you in ARQ mode. He can't do this if he

doesn't know your SELCAL!

3.  Start each CQ with a blank line. The "official"  requirements

of  CCIR-476  and CCIR-625 say that FEC printing does  not  start

until after receipt of a CR/LF. Not all AMTOR controllers require

this, but it's best to be safe!

4. Do NOT include a string of "RYRYRY, RYRY ... " letters.  AMTOR

uses  a special 7 bit data code. The combination  "RYRYRY..."  is

the  worst bit combination you can imagine for  FEC  synchroniza-

tion. In fact, few (if any) AMTOR controllers will ever  synchro-

nize  to a string of "RYRYRY - . . " and some will lose  sync!  "

RYRYRYRYRY . . . " is a poor idea on RTTY and a disaster for FEC!

I suggest the following AMTOR FEC CQ message:

Transmitter ON, AMTOR controller in FEC mode

(blank line)

CQ CQ CQ DE K9GWT K9GWT K9GWT SEL-

CAL: KGWT KGWT KGWT

CQ CQ CQ DE K9GWT K9GWT K9GWT SEL-

CAL: KGWT KGWT KGWT

CQ CQ CQ DE K9GWT K9GWT K9GWT SEL-

CAL: KGWT KGWT KGWT

CQ DE K9GWT (KGWT), BILL IN URBANA, IL

ZZZZ (HAL PCI-3000)-or-END command

Transmitter OFF, AMTOR controller returns to

STBY mode

Paul Newland (AD716-7) also suggests that we delay between trans-

mitter  ON and the start of the CQ text, sending 5 to 30  seconds

of ".FEC idles" at this start. This is a good idea, since "idles"

are required for FEC receive synchronization. However, many AMTOR

controllers do not include a way to delay text transmission.  The

CCIR  "rules" require that synchronizing "idles" be sent  at  the

end/start of each line, so the first CR/LF (blank line) gives you

a headstart, and there are "idles" sent at the end of each  line.

Some  AMTOR controllers also insert extra idle characters  within

each line. As G0AZT and AD7I both point out, this is a very  good

idea and one we all hope all manufacturers will soon adopt.

How Do I Answer A CQ?

First  we have to assume that you have successfully  received  an

FEC  CQ call and that it is formatted like the example. You  must

know  the other guy's SELCAL to answer his CQ! Assuming that  you

did get his SELCAL, enter it as required and start sending in ARQ

mode.  (The  procedure is different for every  AMTOR  controller.

Read  your manual!) If your answer is successful, the other  sta-

tion will soon start chirping back to you, and "away we go."

I'm Linked-Now What?

Ha!  Wasn't  that easy.? Now all you have to do is  start  typing

text. Assuming that you have all the weird and wonderful computer

buffer  controls set correctly (read your manual), text will  now

be sent to the other station-three characters per "chirp."

Even  though  our FCC regulations are  now  considerably  relaxed

concerning ID, I still like to start each major transmission with

the standard "CW style" ID line-"K9CW DE K9GWT," for example. Old

habits die hard for me! I also must admit that I send the ID line

again  at the " end of each of my transmissions. It's not  neces-

sary, but few have complained'

Over

ARQ mode has a special command to use when you have typed all you

want and it's time for the other guy to beat on the keys. We call

this the "OVER" command. The OVER command is actually the  trans-

mission  of "plus" and "question mark" characters in sequence  (+

?). Once you send (+?), the AMTOR controllers take over and "turn

the  channel around"-you become the "IRS" and the other  guy  be-

comes  the "ISS." On some con trollers there is a special key  to

press  or command to enter for OVER; on others you just type  +?.

Note: If you are still using an old Model 15 or 28 TTY machine as

your  AMTOR terminal, you must type "quote" and "question  mark",

the  U.S. Baudot keyboard has a "quote" where the  CCIR  terminal

has a .. plus." It's the same bit pattern, but different  key-top

labels!

Master/Slave

Note that I referred to an exchange of "ISS" and "IRS." This  has

nothing  to  do  with "MASTER" or "SLAVE. " Since  you  were  the

original  calling  station in ARQ mode, we call  you  the  MASTER

station.  In  ARQ mode the MASTER station is simply  the  station

that  first  started the link. The SLAVE station  is  the  called

station in ARQ mode. The terms "MASTER" and "SLAVE" have  nothing

to  do  with who is sending text and who is receiving  text.  The

only  real distinction between "MASTER" and "SLAVE" is  that  the

"MASTER"  sets the timing standards for the link and will be  the

station  initiating a re-call if conditions get so bad  that  the

link  fails.  In an ARQ QSO the ISS and IRS  designations  change

with each OVER operation ' the MASTER/SLAVE designation does  not

change during an ARQ mode QSO.

Forced Over

The  above describes how a "normal" OVER is you do it  the  other

guy is long-winded ("long-fingered"?) and you want to interject a

comment?  Ha!  We have a solution called "FORCED OVER."  In  this

case the receiving station (IRS) can push another special  button

and  take command-instantly (at least within  three  characters).

Once you do this, you are again the ISS and what you type is sent

to  the other station. Here you had better hope that  the  "flow-

control"  between your terminal and AMTOR controller works  right

(and that of the other station as well).

Properly  configured,  what should happen is that  transmit  text

from  the original ISS will be held at the FORCED OVER. When  you

again  return control to the other station (by typing +  ?),  his

transmit  text  should resume exactly where it  was  interrupted.

Most  computer  systems can do this-,  most  mechanical  machines

cannot do this! A FORCED OVER can be rude and may result in  loss

of  some  text if flow-control is not working  properly  at  both

stations. Most of us now use "computer controlled terminals,"  so

it is rarely a problem, However, beware if the other guy says  he

is using an old TTY machine as a terminal!

End Command

An  ARQ link will go forever it you don't end it. ARQ has a  spe-

cial "END" command that 'is used to shut-down the chirping  link.

How you send the END command varies a lot between AMTOR  control-

lers. On some it's a special CTRL key; on others you type  "Z777"

to  end.  Read  your manual! You may observe some  of  us  saying

quaint  things like "I'LL LET You DOWN THE LINK" or simply  "LINK

DOWN."  Obviously,  we about to end the link! By the  way,  don't

forget to send an ID line before dropping the link.

The  above has been just one example of how to use  AMTOR.  Eddie

Schneider, (G0AZT4) Paul Newland, (AD7I6,7), and CQ's Buck Rogers

(K4ABT8)  have  also written  excellent  step-by-step  beginner's

operating instructions. You will learn other techniques and  tips

with a close reading of their articles.

Won't All That Chirping

Destroy My Radio?

Baloney  and poppy-cock! This is by far the , I greatest  concern

of every AMTOR beginner have met. It was my biggest concern  when

started. In spite of what you hear, AMTOR does not destroy  rigs;

at  least  it shouldn't. Modern transceivers will  take  all  the

click-click  operation  of the TX/RX relays I have been  able  to

give  them.  I have never worn out a relay or lost a rig  to  ARQ

chirping.  In fact, since the duty cycle of ARQ mode is only  47%

at worst, ARQ is a lot easier on the transmitter than RTTY or FEC

mode(100% duty cycle).

I do know of two cases where we thought we had wiped-out a  TX/RX

relay-both on ..? TS-830s. What actually happened is that  multi-

ple  relay operations vibrated the relay out of its  socket!  The

TS-830s TX/RX relay is mounted upside down; judicial placement of

a 1/4  inch piece of foam rubber fixed both cases!

Can I Use My Linear?

Now I am going to "hedge my bet"! First of all, you do not need a

linear  for AMTOR. The error correction feature means that  while

you  may get more repeats on weak signals, you can almost  always

get through with 100 watts or less. Second, with very few  excep-

tions,  linears have large and slow relays. These relays are  not

suited  for  fast  ON/OFF switching required in  ARQ  mode.   The

relays  often switch high voltages and/or currents. Yes, you  can

get  some pretty spectacular arcs and sparks in the  linear,  and

they will cost big bucks to fix!

So unless your linear manufacturer makes a big deal out of  "QSK"

operation  of his linear, forget high power on AMTOR.  You  don't

need it! Actually, we could all probably drop our output power to

10  watts  or less and never notice the difference. If  you  have

TVI, AMTOR with low power may be your solution!

How About Switching Times?

This  is  another supposed "problem" that has been blown  out  of

proportion.  Switching times used to be a concern. It  really  is

not  a major concern with the current and previous generation  of

transceiver  models.  They  can all be used  on  AMTOR.  However,

switching  time  can be a big problem on  older  equipment.  Some

models  are in fact unusable on AMTOR or may require  substantial

modifications to use ARQ mode. Many radio manufacturers now  have

data  sheets  that give switching times and  suggested  modifica-

tions. Active AMTOR operators are also a good source for  modifi-

cation in formation-"Caveat Emptor"! By the way, if your rig  has

a "QSK" mode, use it!

There is also a lot of on-the-air discussion about what values to

use  when setting the programmable delays in your AMTOR  control-

ler.  Some amateurs say "set them as low as they will  go."  Good

luck  if you do! Here again, read your manual and start with  the

manufacturer's  recommended  settings. Messing with delays  is  a

two-edged  sword. Shorter delays extend your longest distance  in

ARQ mode, but may also increase your minimum distance. The number

and  purpose of programmable delays varies with each  AMTOR  con-

troller read your manual! A "delay" time of 20 to 50 Ms will work

with practically all radios-and cover the world. You do not  need

to set delays to "5" or " 10" to work DX! Chances are your  radio

is  not that fast and you may actually be trashing some  of  your

own data while the poor radio is trying to switch! Each  control-

ler/radio combination is slightly different. Do some  experiment-

ing and determine your own parameters.

What About The Shift

For AMTOR?

In my opinion, there is only one shift that AMTOR stations should

use,  and that is 170 Hz SHIFT. This has been the  amateur  radio

RTTY  shift for years, is the shift used in all commercial  SITOR

stations,and is usually the only shift supported in the FSK  mode

of modern transceivers. 170 Hz shift is the standard, and you are

only doing yourself a disfavor if you attempt to run AMTOR  using

another shift.

A  second  shift, 200 Hz, has recently appeared  on  the  amateur

bands.  200 Hz shift originated with the Bell 103  telephone-line

modem  and was adopted for inclusion in packet controllers.  When

packet  moved  from VHF to HF, the 200 Hz shift PLL  modems  came

"for  free."  I think all who run HF packet would have  to  admit

that 200 Hz shift really does not work well for packet. There  is

an  active  and much needed drive for improvement  in  HF  packet

modem  standards. Changing the shift (and modulation type)  is  a

recognized  requirement for improved HF packet operation. It  200

Hz shift doesn't work for HF packet, why use it on AMTOR?

It your controller has 200 Hz shift, you face a double  disadvan-

tage  when talking to the rest of us. First of all, your  200  Hz

shift  transmit signal does not match our 170 Hz  spaced  receive

filters.  This makes our receiver tuning much more  critical  and

increases  the  odds that we will get "hits," many  repeats,  and

eventual link failure. Second, your receive filters do not  match

our 170 Hz transmitted signal. You also have to  "straddle-tune,"

face  tuning  errors, and accept less  than  optimum  demodulator

performance. A 30 Hz difference does not sound like a lot, but it

can make a big difference on marginal links.

If  you  have an AMTOR controller with a 200 Hz  shift  modem,  l

suggest that you consider retuning the receive filters and trans-

mit  tones for 170 Hz shift. Your AMTOR will be much  better  for

the  effort!  By  the way, retune for 2125 Hz MARK  and  2295  Hz

SPACE. That's the standard and what your transceiver FSK  circuit

is  set up for! You will not be "true transceive" with any  other

tone frequencies.

What Is APLink?

APLink is a very nice message-forwarding computer program written

by Vic Poor, W5SMM- APLink serves as a link between HF AMTOR  and

VHF packet networks. APLink is run in an "IBM-Compatible" Person-

al  Computer (PC) and provides two data ports-one for  the  AMTOR

controller and one for the packet controller (TNC). The two ports

share  a  common  message storage; each can  access  the  other's

message files (with some guidance from the control operator). Vic

has  written  APLink drivers for the AEA PK-232, AMT-1,  and  HAL

PCI-3000  AMTOR  controllers. Only a "Gateway" or  message  relay

station needs to run APLink software, so it's a program that many

of us will never need. However, we may all want to use the  long-

haul message relay feature of an APLink Gateway station,

Many Gateway stations also include frequency scanning to  monitor

8 to 16 different bands and/or frequencies. These stations there-

fore avoid many of the problems associated with skip distance and

changing band conditions with time. It's also an efficient use of

radio  spectrum, since the control circuitry automatically  skips

frequencies  which are already in use. The best way to  find  out

more  about  APLink is to tune around 20 meters, find  an  APLink

station,  and watch the fun in MONITOR or LISTEN mode.  You  will

get  the  idea after a couple of listening sessions and  then  be

able  to call and request HELP files for more data. APLink is  an

exciting  new use of RTTY, one that makes full use of  the  error

correcting and robust nature of AMTOR.

Conclusions

AMTOR  is  great mode. It is really not hard to  run.  AMTOR  has

fewer cryptic commands and is actually simpler to use than  pack-

et.  HF  AMTOR  is a whole lot more reliable than  HF  packet  or

"straight  RTTY." AMTOR does not require high transmitter  power.

Practically  all  modern transceivers can run  all  AMTOR  modes.

There are now a lot of amateurs all over the world running AMTOR.

Whether  you like to rag-chew or run traffic, AMTOR is  an  ideal

mode.  I must admit that running ARQ mode in a RTTY contest is  a

lot of work, but it can also be very rewarding! All around, AMTOR

is the HF data mode for many of us long time RTTY fanatics.

References

1  -Peter Martinez, G3PLX, "Amtor, an  improved  radioteleprinter

system,  using  a microprocessor," Radio  Communications,  August

1979, RSBG, Cheimstord, Essex, England, pp. 714 719.

2. Peter Martinez, G3PLX, "Amtor, the easy way," Radio Communica-

tions, June/July 1980, RSBG, Chelmsford, Essex, England, pp. 610-

615,

3.  Peter Martinez, G3PLX, "Amtor, an Im proved  Error-Free  RTTY

System," QST, June 1981, ARRL, Newington, CT 061 1 1, pp. 25-27.

4.  Eddie  Schneider, W6/GOAZT, "AMTOR"  (monthly  column),  RTTY

Journal, April, May/ June, July/August 1990, 9085 La Casita Ave.,

Fountain Valley, CA 92708.

5.  Bill Henry, K9GWT, "New AMTOR Mode! CQ, The  Radio  Amateur's

Journal,   November  1989,  CQ  Communications,  Hicksville,   NY

11801,pp.36-40.

6.  Paul  Newland, AD7], "An Introduction to  Amtor,"  QST,  July

1983, ARRL, Newington, CT 06111,pp.11-13.

7. Paul Newland, AD71, "A User's Guide to AMTOR Operation,"  QST,

October 1985, ARRL, Newington, CT 0611 1, pp. 31-34.

8.  Buck Rogers, K4ABT, "The Newcomer's Guide to AMTOR," CQ,  The

Radio Amateur's Journal, November 1989, CQ Communications, Hicks-

ville, NY 1 1 801, pp. 51 -55.