STARTING (from a boot)

----------------------

Run MBBIOS.  It will go resident.  This step only required for OUTBOARD

TNCs not using the G8BPQ Switch.

Run TNCTSR-S.  It will go resident.  This step only required if you have

PC*PA card and are not using the G8BPQ switch code.  I don't supply

TNCTSR-S.  Its on the diskette that comes with the card.

If you have modified PARMS.BB, run BBSETUP.

Run BBSETTNC.  This will switch your TNCs to the HOST MODE if they are

not there already.  You should see one of two things for each TNC.

* JHOST1

TNC was not in host mode but is now

INVALID COMMAND

TNC was in host mode

For AEA TNCs with AEA NETROMS you will see

(smiley face)OGG(blank)(two-headed arrow)

If any of these lines is not the last line for any COM ports, run

BBSETTNC again.  If still no luck, check the SPEED in the PARMS.BB file,

the TNC is on and ready, etc.  If after an hour or two you still can't

get it to work, take two aspirin and drop me a note.

**** DO NOT PROCEED TO NEXT STEUP UNLESS BBSETUP RAN OK ****

If BBSETTNC was OK, you can now run BB

STARTING (System has been up already)

-------------------------------------

If you have modified PARMS.BB, run BBSETUP.

If you are confident that things are still in HOST MODE, you can skip

running BBSETTNC but it won't hurt.  If your TNCs have been reset or are

in an unknown state, then run BBSETTNC.

Now run BB.

WINDOWS

-------

The screen is broken into four sections

+--------------------+

|     upper          |

|     window         |

+--------------------+

+   status line      +

+--------------------+

+     lower          +

+     window         +

+--------------------+

+    input line      +

+--------------------+

The status line shows the number of tasks running, the free storage, the

status of the current window, and the current connected users.

The upper window can show either connected or monitor data.  The lower

window shows either the operator window or the window not being shown in

the upper section.

The input line shows what the operator is typing.

The current window and input line disappear if the editor is in use.

LINE EDIT KEYS

--------------

These keys can be used to edit the line you are typing.

INSERT -- Toggles insert versus overwrite mode.  Insert mode is

depicted by the large BLOCK cursor

DELETE -- Delets the character above the cursor

BACKSPACE -- Deletes the character to the immediate left of the cursor

LEFT ARROW and RIGHT ARROW -- Moves the cursor one step in the direction

of the arror.

FUNCTION KEYS

-------------

The following function keys are active at all times (except when in the

editor).

F2  -- Erase the input area from the cursor to the end of the line

F3  -- Immediate switch to the MONITOR window.  Will disconnect

a terminal session started with "T" command

F4  -- Clear bottom window

F5  -- Swap monitor and connect windows between top and bottom.

F7  -- Terminate terminal session.

F9  -- Retrive command (clockwise)

F10 -- Retrive command (counterclockwise)

ALT-X -- Shutdown BBS

Up-Arrow -- Scroll the current bottom window back one line

Down-Arrow -- Scroll the current bottom window forward one line

Home -- Scroll the current bottom window back to the top of the buffer.

End  -- Scroll the current bottom window forward to the bottom of

the buffer.

Page-Up -- Scroll the current top window back one line

Page-Down -- Scroll the current top window forward one line

Control-Page-Up -- Scroll the current top window back to the top of

the buffer.

Control-Page-Down -- Scroll the current top window forward to the

bottom of the buffer.

All other function keys should cause a beep and a rewrite of the

status line.

OPERATOR COMMANDS

-----------------

Those available remotely have a "*".  If you are on remote, you must

issue the appropriate "@" command to switch to remote sysop mode before

using anything but the "@" command.  Note that all commands are

available via WAKEUP

*   @  -- Change to remote SYSOP

*   @N -- Change to new user

*   @U -- Change to unregistered user

*   @O -- Change to user

*   @E -- Change to expert

*   @B -- Change to BBS

*   @R -- Change to remote SYSOP (same as @)

@L -- Change to local SYSOP

*   A  -- Reload the action file and execute all actions

*   AC -- Execute CLEAN actions only

*   AF -- Reload the action file and execute MAKE_FILE actions

*   AL -- Reload the action file

*   AM -- Execute MAKE_FILE actions only

B  -- Closes the operator window (same as F3)

*   C  xxxxx -- Set the system clock.  xxxx is the date/time in format

of YYMMDDHHMM.  If the command "C" alone is given, the system

responds with its date/time

*   CF file1 file2 -- Copies a file.  Can also be used to print a file.  To

print, use LPT1 or LPT2 as file2.

*   CLOSE port -- Prevents any future forwarding or connects on the port.

Users receive message # 167.  Port should be either the letter

of the port or "S" for system.

*   CM msgno address -- Copy message "msgno" to "address."  You can also

add news bids if necessary.  Example:

CM 1234 W0RLI @ W0RLI

*   DU -- Display users

*   DU call -- Display a particular user

*   EC port -- Edit control parameters for a port.  See MODE.DOC

Port should be either the letter of the port or "S" for

system.

EF filename -- Edit a file.  Uses BORLAND's BINARY EDITOR (part of

the TURBO PASCAL Editor toolbox).  See EDIT.DOC for details

*   EL -- Same as EL but will automatically do a GL after the server runs.

*   EM msgno -- Edit a message's header

*   EP port -- Edit parameters for a port.  Same format as EC

*   ES server file parms -- Execute a server.  Example:

ES REQDIR D:\PATH\T17.EXE

ES REQFIL DOS BATNAM

Use the second example for DOS commands and BAT files.  See info

in SERVER.DOC.

ET msgno -- Edit text of a message.  Uses same program as EF

*   EU call  -- Edit a user's information (See USER.DOC)

*   EX file parms -- Execute another program.  Example:

EX D:\PATH\T17.EXE

EX DOS BATNAM

Use the second example for DOS commands and BAT files.  See info

in SERVER.DOC

*   EXPORT filename # message_list -- EXPORT to a file the messages indicated.

If the file exists, the messages are appended to it.  Example:

EXPORT A:A # 124 3465 22

EXPORTK instead of EXPORT means kill message after export.

*   EXPORT filename {search}  -- Export files who match a search.

*   EW -- Send any white-pages updates

*   Fx msgno directory filename -- Make file from message.  Directories

are those shown by W. If the command is just F, then include

the X400 header.  If the command is FH, give the X400 header

but show all BBS route headers.  If the command is FN, copy

message asis.  Can also be used to print a message.  To print,

use LPT1 or LPT2 as directory and omit the filename.

*   FORCE task -- Issue an immediate disconnect to the indicated task.

To force a user, get his task name from the status line.

Example: FORCE B3

GB -- Set the SYSOP bell flag. 1 or Y is on.  0 or N is off.  If

nothing specified then the flag will be flipped.

*   GC window filename -- Capture all new lines written to a window to

a file (or printer).  The window name must be OPERATOR,

CONNECT, or MONITOR.  If the file already exists, then the data

is added on the end.  To terminate, issue GC with no parms.

*   GE -- Load a new message text file (usually MESSAGE.BB)

*   GF -- Tell BB that ROUTE.BB had been changed.

*   GH -- Do a lookup for extended addresses.  Command will normally

resume where it left off.  GH 1234 will foce the lookup to

start at message 1234.

*   GI -- Reorganize BID file

*   GK -- Force a wakeup cycle.  Used to tell BB that WAKEUP.BB has

been changed.

*   GL -- Reload the regular message file.

*   GM -- Reorganize the message file.  Will not run if anyone else is

connected or if forwarding is in progress.  Killed files

actually disappear now.

*   GN xxxx yyyyy -- Change a file name. xxxx is the file you want to

change.  Allowed are: PATH, ROUTE, MESSAGE, WAKEUP, HELP,

HLOOKUP, PASSWORD, ACTION.  yyyyy is the name of the new file.

Example:

GN ROUTE \ROUTE.NEW

*   GO -- Mark messages as old.  You can mark messages by bid or by age.

GO BID pattern -- Marks a message old by matching its BID

to a pattern.  See WILDCARD.DOC for

pattern info.  You can have multiple

patterns.

GO TYPE msg_type age -- Mark a message old is its type

matches and it is more than "age" days old.

Example:

GO TYPE B 15 PT 3 * 5

Mark B messages after 15 days, P or

T messages after 3 days and all others

after 5 days.

GO {search} -- Marks a message old that matches the given

search. See SEARCH.DOC

GP -- Set the flag that supresses showing the prefix area.  1 or Y is

on.  0 or N is off.  If nothing specified then the flag will be

flipped.

*   GR -- Same as GM but renumber messages too.  Will restart at 1.

GR 1234 will start renumbering at 1234.

GS -- Set the sysop available flag.  1 or Y is on.  0 or N is off.  If

nothing specified then the flag will be flipped.

*   GW window filename -- Capture the current contents of a window to

a file (or printer).  The window name must be

OPERATOR, CONNECT, or MONITOR.  If the file already

exists, then the data is added on the end.

GZ -- Set the show error during forward script flag.  1 or Y is

on.  0 or N is off.  If nothing specified then the flag will be

flipped.

*   HOLD {search} -- Change messages to hold status.  You can use the

same syntax as "R".  Examples:

HOLD 123 TO 126

HOLD 123 146 156

*   IMPORT filename -- IMPORT from a file

IMPORTE instead of IMPORT means erase file when done.

*   K{search} -- Kill messages who match a search (See SEARCH.DOC for

details).  Some useful examples:

KH -- Kill all held files

KO -- Kill all old files

*   KILL task -- Issue an immediate disconnect to the indicated task

and then destroy the task without waiting for the TNC

To kill a user, get his task name from the status line.

Example: KILL B3

*   L{search} -- List messages who match a search (See SEARCH.DOC for

details).  Some useful examples:

LH -- List all held files

LO -- List all old files

*   Mc filename address -- Make message from file.  Message class is "c".

File name is fully qualified.  Address is who to send it to.

You can also add BIDS.  Example:

MB a:bbb ALL @ ALLCA

*    OPEN port -- Reverses action of CLOSE command.  Port should be either

the letter of the port or "S" for system.

QUIT -- Immediate stop (not safe.  Use only in emergencies)

Files will not be closed nor will disconnects be issued.

*   REL {search} -- Release messages from hold and review.  You can use the

same syntax as "R".  Examples:

REL 123 TO 126

REL 123 146 156

REL H

The last example will release all messages in hold.

*    REVIEW {search} -- Display messages for review.  If search is not

specified then "V" is assumed.  Each message is displayed and

SYSOP is asked to tell what should be done.  Options are

Kill, Leave, Release, Hold, and Quit.  The last action leaves

the message untouched and terminates the review command.

Remote SYSOPs indicate the action by typing the first letter

of the word.  Local SYSOPs indicate the action by pressing

ALT-x where x is the letter of the action to be taken.

Local SYSOPs see the messages in the full screen editor with

the title line showing on the top line of the box.

Uses same program as "EF" and "ET."  See EDIT.DOC

*   SHUT -- Shuts BBS down but only if not busy.  Same as ALT-X

See WAKEUP.DOC for precautions to take when using this

command in a wakeup file.

*    SHUTNOW -- Shuts BBS down.  Same as ALT-X See WAKEUP.DOC for

precautions to take when using this command in a wakeup file.

T portchan -- Talk to indicated portchan.  Puts the operator

in terminal mode.  All WA8DED commands are available.

To leave, use either F3 or F10.  See TERMINAL.DOC

*   UNKILL {search} -- Change messages from kill status.  You can use the

same syntax as "R".  Examples:

UNKILL 123 TO 126

UNKILL 123 146 156

UNKILL K

The last example will "unkill" all killed messages.

*   X -- Start foward now

*   X PORT p -- Start forward to a port now.  p can be a single port

letter or a sub-port designation

*   X path  -- Start forward to a path now

*   X path FORCE -- Start forward to a path and connect even if no messages.

*   XSTOP -- Force current forward cycle to end.  MUST BE IN CAPS!

*   Z directory filename -- Erase file in a directory

REMOTE SYSOP PASSWORDS

----------------------

Now moved to AUTHENT.DOC.

The password will be prompted when the @R command is issued.  Modem users

will not need a remote sysop password.