The program now lets you select and deselect features on a port-by-port

basis.  The idea for this came from the 1989 Loma Prieta earthquake;

during the aftermath, BBS operators spent a lot of time manually

filtering traffic -- the features described in this section would have

made the job much simpler.

The features you can select are:

FORWARD EMERGENCY TRAFFIC ONLY

FORWARD NTS TRAFFIC

ACCEPT EMERGENCY TRAFFIC ONLY

ACCEPT NTS TRAFFIC

ALLOW E-MODE USERS ONLY

ALLOW CALL SIGN CHANGE

There are parameters for each one pertaining to the system and to the

port.

FORWARD EMERGENCY TRAFFIC ONLY

------------------------------

Only traffic defined by the EMERGENCY_TYPES parameter in PARMS.BB will

be forwarded.  The applicable parm is FORWARD_EMERGENCY_ONLY.

ACCEPT EMERGENCY TRAFFIC ONLY

-----------------------------

Only traffic defined by the EMERGENCY_TYPES parameter in PARMS.BB will

be accepted.  A normal user trying to send other traffic will be given

an error message; if a BBS tries to send other traffic, either the

message will be rejected or the station will be disconnected, depending

on the setting of the REJECT_ACTION in PARMS.BB.  The applicable parm is

ACCEPT_EMERGENCY_ONLY

FORWARD NTS TRAFFIC

-------------------

NTS traffic will be forwarded.  The applicable parm is FORWARD_NTS_OFF

ACCEPT NTS TRAFFIC

------------------

NTS traffic will be accepted.  If this switch is off, then if a normal

user tries to send NTS traffic, he will be given an error message.  If a

BBS tries to send NTS traffic, either the message will be rejected or

the station will be disconnected, depending on the setting of the

REJECT_ACTION in PARMS.BB The applicable parm is ACCEPT_NTS_OFF

ALLOW E-MODE USERS ONLY

-----------------------

Only users with the "E" flag in their user directory will be allowed to

use the BBS.  The applicable parm is ALLOW_EMERGENCY_ONLY

ALLOW CALL SIGN CHANGE

----------------------

A user without the "E" flag in his user directory can change his call

sign once he is connected.  The applicable parm in PARM.BB is

ALLOW_EMERGENCY_CHANGE.  The purpose of this is to allow the use of

tactical addresses during an emergency.  A typical scenario would be

as follows:

AA4RE is stationed at the Gilroy Emergency Operations Center.  He

connects to the Emergency BBS but AA4RE is not an "E" mode user.  The

BBS will propmt for a new callsign.  He enters GILROY which is an "E"

mode user.  This will then allow AA4RE to send and receive messages for

GILROY.  Packets will have the proper ham callsign on them for

identification purposes.

The local ARES organizations (SF Bay Area) felt that this method was

preferable to using the tactical call sign as the call sign in the

TNC and having to beacon the ham callsign for ID purposes.

Setting the options

-------------------

Each of these options is controlled by a "system master switch" and a

"port switch".  For an option to be effective, both the MASTER and

PORT switch must have the same value.

You can set these options directly in PARMS.BB or you can use the EC

command:

EC S -- Edit master switch list

EC x -- Edit port "x" switch list

You change switches by number.  Specify the number by itself to invert

the setting of the switch.  Specify the number followed by ON or 1 to

turn the switch on; specify the number followed by OFF or 0 to turn the

switch off.

There are parameters for each option for both the system and the port in

PARMS.BB.   Its the same parameter in both places.  You set the

parameter to either Y or N.