G8BPQ NODE COMMANDS

This document explains the commands available within the Node section

of the G8BPQ switch, and an explanation of the responses.

First the result of inputting an invalid command:

NOTTS:G8BPQ-3} Invalid command - Enter ? for command list

Entering ? produces the following:

NOTTS:G8BPQ-3} BBS CONNECT BYE INFO NODES PORTS ROUTES USERS

Note that BBS is only present if the BBS support is enabled (BBS=1 in

th config file). If you have defined your own applications, they will also

be listed.

The BBS (and your other application names) must be entered in full - all other

commands can be abbreviated to the first character of the command.

Entering BBS (or your own application name) will connect you to the first

free BBS (etc) port, or give an error message if none are available.

The CONNECT command is used to connect you to another node, or to a normal

AX.25 user. To connect to another node, enter C NODECALL or C NODEALIAS.

The system will select the 'best' radio port and neighbour to to use to

reach the required node using its ROUTES and NODES lists.

The format used to connect to another normal user depends on whether you

have more than one radio port. The formats are:

(stuff in [] being optional)

C CALL [via digi1 [digi2...]]		If you only have one port

C P CALL [via digi1 [digi2...]]         Where P is the port number,

if you have more than 1.

(You can use C 1 CALL if you only have 1 port - its just a waste of typing!)

If you miss out the port number where it is needed, you will get an error

message, listing the available ports.

Normally you cannot override the automatic route selection when you connect

to another node, but you can fool the system into thinking a node is a

normal station by connecting to the alias, and adding an SSID. For example,

if you want to connect to NOTTS:G8BPQ-3, you can force a Level 2 connect

on a secified port by entering C P NOTTS-1

It is also possible to instruct the node to remain connected to the

currrent node, when the connection to the next one is closed. This is

achieved by adding an S (for STAY) after the connect command. For

example, if I type

C GB7YDX S

I get

NOTTS:G8BPQ-3} Connected to GB7YDX

If I then enter B, GB7YDX closes the connection, but instead of getting

*** Disconnected, I get

Returned to Node NOTTS:G8BPQ-3}

This is particularly useful if you are exploring a distant set of nodes,

and don't want to have to reconnect over the whole path again and again.

The BYE commnad disconnects you from the switch.

The INFO command sends your INFO text from the config file:

NOTTS:G8BPQ-3} G8BPQ Packet Switch, Mapperley, Nottingham. IO92KX

Commands are basically the same as NET/ROM, but to connect to another

normal station (not another node), you must specify a port number before

the callsign. Use PORTS command to list available ports. The BBS command

connects you to the associated Mailbox.

The PORTS command lists available ports. The descriptions come directly

from the configuration file, and should give at least the frequency and

baud rate used.

NOTTS:G8BPQ-3} Ports:

1 144.650 MHz 1200 Baud (PC120)

2 432.675 MHz 1200 Baud (DRSI)

3 Experimental NET/ROM Link

The NODES command lists all the other NETROM/THENET/BPQ Nodes known to

your node:

NOTTS:G8BPQ-3} Nodes:

BBSTST:G8BPQ-1     DV7:G4RFG-1        LRG7:G0GDR-1       G4RFG-2

LRGBBS:GB7LRG      AAABBS:GB7AAA      #LNX2:G4GOU-1      BM1:G7AXC-1

BM2:G7AXC-2        BM7:G7AXC-7        DV2:G4RFG          DV6:G4RFG-3

BOB432:G8HBE-3     NEC21:G8VPQ-2      FPV7:G4FPV-7       SY4:G3UQH-4

WP4:G0KNR-4        LRG2:G0GDR-2       NEC22:G8VPQ-3      MM2:GB7MM-2

TFONET:G8TFO-8     NEC90:G8VPQ-9      NH:G0HWC           RP2:GB3RP-2

ADH2:G8ADH-2       SY7:GB7SY-7        HX2:GB7HX-2        WORC7:G8TIC-7

WV2:G1RLI-2        TEWKS3:G6CMG-3     WB7:G4DVM-7        BOB650:G8HBE-2

LED:GB7LED         SY8:G3UQH-8        VPQNET:G8VPQ-8     GH2:GB7GH-2

SY2:G3UQH-2        FPV:G4FPV-2        AP2:GB7AP-2        LX2:GB7LX-2

LX4:G6TNZ-1        LX7:GB7LX-7        RAYNET:GB7NRC      SF2:G8POT-2

#NICK:GB7LRG-7     NEC72:G8VPQ-7      WORC2:G8TIC-2      PQ2:GB3PQ

TEWKS4:G6CMG-4     BOB675:G8HBE-4     FPV71:G4FPV-8      TEWKS9:G6CMG-9

TICNET:G8TIC-8     BRX:G4AKZ          SC4:G4AJJ-4        CD2:G6ANN-1

TEWKS7:G6CMG-7     TCPIP:G4GOU        CHELT2:G4MEM-2     DROIT7:G8TFO-7

DROIT2:G8TFO-2     MK6:G4WIM          MV2:G2AFD-2        SERVER:GB7AAA-9

ERA24:G0DXX        WP1:GB7WP-1        WP2:GB7WP-2        WP7:GB7WP-7

MK2:G4WIM-2        MK23:G4WIM-7       LNXBBS:GB7LNX-2    TVB:G6TVB

#GATE0:G6CMG-5     #GATE1:G6CMG-6     WORC22:G8TIC-1     VPQ90:G8VPQ-10

VPQ91:G8VPQ-11

By entering N NODECALL (or N NODEALIAS), you can list the routes that

the system will use to access that node:

n gb7ydx

NOTTS:G8BPQ-3} Routes to: DXYORK:GB7YDX RTT=7 FR=17  B 2

> 87 4 5 GB7YS

0 4 7 G8BPQ-8

0 4 6 G4FIS-2

The bits after the callsign are only shown if some frames have been

send to that node. RTT stands for Round Trip Time, and is a rolling

average of the time taken to get a response from that node (in seconds).

FR means Frames, the number of info frames sent to the node. The B, if

present, indicates the target is a BPQ node, and the number following

the B is the number of hops to the target.

Up to 3 possible routes to the node are listed. The first number

displayed is the 'quality' - the relative desirability of using this

route rather than another. The second is the Obsolesance Count, an

indication of how long it is since the system was last told about (

or successfully used) this route. The number starts at a value set in

the config file (typically 5) and is decremented each time a 'NODES'

broadcast is sent (typically evey hour). The 3rd number is the port.

A > indicates the currently active route.

The Round Trip Times and frame counts for all nodes with a non-zero

count can be displayed by entering N T

n t

NOTTS:G8BPQ-3} Nodes:

G1FYS              RTT=22 FR=5  B

#LDS:G3WNR-3       RTT=17 FR=39  B

AP21:GB7AP-2       RTT=90 FR=866

AYTON:G4HRM        RTT=76 FR=18  B

BADBBS:GB7BAD      RTT=40 FR=2576  B 2

BEDS:G1ZPU         RTT=283 FR=1  B 4

BFD41:G4GIR-4      RTT=31 FR=593

BOSTON:G4LPL       RTT=28 FR=1606  B 2

COV22:G3ZFR-3      RTT=29 FR=14

DXYORK:GB7YDX      RTT=7 FR=17  B

HALFAX:G6KZJ       RTT=30 FR=306  B 2

HF:G4JLB-8         RTT=32 FR=99  B 4

LADY61:G7EQM-8     RTT=68 FR=1754  B 1

NEC:G8VPQ          RTT=113 FR=127  B

NN22:G8AMG-2       RTT=30 FR=2047

NN41:G8AMG-4       RTT=19 FR=250

NN72:G8AMG-7       RTT=19 FR=301

NOTBBS:GB7NOT      RTT=34 FR=5659  B 1

OAK:G0LTN-1        RTT=76 FR=1309  B 3

PBORO:G1ARV-8      RTT=26 FR=230  B 2

RP:GB7RP           RTT=41 FR=1396  B

RUTBBS:GB7RUT      RTT=10 FR=10683  B 1

RUTLND:G4FIS-2     RTT=7 FR=16468  B 1

SHEF:GB7YS         RTT=15 FR=11547  B 1

TLH2:G1TLH-2       RTT=36 FR=5245  B

WN:G7HPM           RTT=52 FR=2281  B 4

XOWPMS:G1XOW-2     RTT=38 FR=58  B

YORK:G1FTA         RTT=37 FR=363  B 3

YORKS:GB7YW        RTT=20 FR=117  B

The ROUTES command lists the stations which this node can hear.

NOTTS:G8BPQ-3} Routes:

4 GB7YS 0 0!

1 G4FIS-2 14 0!

3 G3SDC-8 35 0!

> 6 G4FIS-2 150 58!

3 G0INA-1 90 0!

2 G8BPQ-5 250 0!

4 G1EQT-8 50 0!

> 4 G0INA-3 120 48!

> 5 GB7YS 150 93!

3 G7JGX-3 25 0!

> 4 G4IRX-3 50 9!

7 G8BPQ-8 250 105

4 G7EQM-8 10 1

The > indicates that there is an active link to the node.

The first number is the port. The second is the quality. This may be

derived from the 'default quality' parameter in the PORTS section of the

config file, or may be specified explicitly in the ROUTES section.

A value of zero will prevent the route from being used, and is normally

used when you have a one-way or marginal path. The third number is the

number of NODES list entries which refer to this route. This isn't used

by the software - it is just information for the user. The '!' indicates

a 'locked route' - one entered in the CONFIG file, or via SYSOPH. Other

entries come and go as this node hears NODES broadcasts.

Additional information may be obtained by entering r r

r r

NOTTS:G8BPQ-3} Routes:

4 GB7YS 0 0!  8  0  0%  0  0  18:52

1 G4FIS-2 14 0!  0  0  *  0  0  00:00

3 G3SDC-8 35 0!  0  0  *  0  0  00:00

> 6 G4FIS-2 150 58!  57022  1333  2%  0  0  21:55  0

3 G0INA-1 90 0!  0  0  *  0  0  00:00

2 G8BPQ-5 250 0!  0  0  *  0  0  00:00

4 G1EQT-8 50 0!  0  0  *  0  0  17:13

> 4 G0INA-3 120 48!  19305  819  4%  0  0  21:40  0

> 5 GB7YS 150 93!  27017  6229  23%  0  0  21:44  0

3 G7JGX-3 25 0!  0  0  *  0  0  00:00

> 4 G4IRX-3 50 9!  2025  42  2%  0  0  22:09  0

7 G8BPQ-8 250 105  592  0  0%  0  0  22:02

4 G7EQM-8 10 1  3077  219  7%  0  0  21:37

The extra fields are:

Info frames sent

Info frames retransmitted

Retry Rate - the ratio of the above 2, as a percentage

(or * if both are zero)

Non-standard maxframe (from Config file)

Non-standard frack    (      ditto     )

Time the last NODES broadcast was heard from this node.

The last figure is only present if there is an active link. It

is the number of frames queued to be sent. Up to 4.05e, this only

counts frames queued above the link level - there may be up to

another 8 queued at link level. With version 4.05f and above it

includes all frames queued.

The retry rate gives a good indication of how well the link is running.

Dedicated links should normally be well below 10%. A shared link

will normally have a higher rate, but anything above say 25% is likely

to cause significant delays. Another indication of a poor link is a

high number of frames queued - any nonzero value of 4.05e or below, or

above 8 for later versions which persists for more that a minute or so

is likely to be causing problems.

The USERS command lists the stations currently using the node.

NOTTS:G8BPQ-3} G8BPQ Network System V3.21 (95)

Host6(NOTTS:G8BPQ-3)

Host3(NOTTS:G8BPQ-3)              <-->  Circuit(LRG7:G0GDR-1 G8BPQ-1)

Uplink(G9XXX)                     <~~>  Downlink(G9YYY)

Host is an internal (Normally BBS Port)

Circuit is a link from/to another node.

Uplink is a connection from a normal Ax.25 station.

Downlink is a connection to a normal user.

The <--> indicates an active session. <~~> indicates a session being set up.

The Number on the end of the header line is the number of free buffers.

There are a few commands not given in the menu. These are primarily of

interest to the sysop, (or for me to experiment with). They are LINKS,

STATS, L4T1, and PACLEN.

The LINKS command lists the currently active AX.25 Sessions (Both user

access and node-node links)

NOTTS:G8BPQ-3} Links:

G0GDR-1 G8BPQ-3 S=5 P=2 T=3 V=2

S is the link state (see AX25 protocol spec, but the main ones are

2 (connecting) 4 (disconnecting) 5 (connected)).

P is the port.

T is the link type. 1=Uplink, 2=Downlink, 3=Node-Node link.

V is the AX.25 Version (1 or 2).

The STATS command displays a number of counters.

s

NOTTS:G8BPQ-3}

Time active (mins)               8786

Timer Overruns                     48

Buffers: Max/Cur/Min/Out          100       76       16        0

Known Nodes                       144

L4 Connects Sent/Rxed            1055     1732

L4 Frames TX/RX/Resent/Reseq    73288    96544     1870       42

L3 Frames Relayed               10148

L2 Frames Digi-ed       0        0        0        0        0        0        0

L2 Frames Heard    124393     1870    80601    82124    60600   167575     2650

L2 Frames Rxed      11921        0     2909    66674    59032    94340     1447

L2 Frames Sent      15955     1870     4690    69857    71328   107267     3284

L2 Timeouts          4466        0      984     5437     7697     1154       58

REJ Frames Rxed         7        0        2      460     1681      214        0

RX out of Seq         269        0        3      951     1697      272        0

Underrun/Poll T-O       0        0     5358      233        0        0        0

RX Overruns             0        0    39814        0        0        0       63

RX CRC Errors       42260        0        1        0     2324    27025        0

FRMRs Sent              0        0        0       19        0        0        0

FRMRs Received          0        0        0        0        0        0        0

Frames abandoned        1        0        0        0        0        2        0

Link Active %       9  64    0   0    0   0    0   0   14  21   50  82    0   0

Most are fairly obvious, but a few need a bit of explanation. There is one

set of level 2 counters for each port.

Time Active is the time since the system was loaded.

The Buffer count Maximum, Minimum and Current should be obvious. The

Out figure is the number of times a request to allocate a buffer

failed because there were none available.

The L4 frames resent count is the number retransmitted because an

ACK wasnt received within the L4 timeout period. The Reseq count

is the number of frames received out of sequence, but subsequently

used because the missing frame(s) eventually arrived.

Timer Overruns count the number of times the main code (which is initiated

by the timer interrupt) is still running when the next timer interrupt

occured. If it increments very rapidly (several times a minute), it

indicates that the PC is too slow for the amount of data being processed.

It is normal for an XT class machine running a couple of BBS ports to get

about one overrun per minute. My 10MHz AT clone shows very few.

A large number in the 'REJ received' field may indicate that your Maxframe

is too high. Similarly a large number in 'RX out of sequence' may indicate

the the station taking to you has too large a maxframe.

RX Overruns indicate characters lost because the software didn't process

the interrupts fast enough. If you are running KISS ports, and you get

a lot (ie a significant percentage of L2 frames heard), try reducing the

speed of the link from the PC to the TNC. If you are using an HDLC card

(DRSI or PC120), particularly at high speed, then there isnt much you

can do except buy a faster machine. (But I would like to hear from anyone

having problems running at 9600 baud or above - I may be able to speed up

the routines a bit).

Underuns indicate a similar problem in responding to TX interrupts,

but only apply to HDLC cards. As you cant get a TX underrun on an

async port, this field is also used to count timeouts on a polled KISS

system).

Frames abandoned counts the number of frames discarded because they have

been waiting to be sent (for DCD to clear) for more than 60 secs. If a

lot occur, then either your squelch is a bit dodgy, or the channel is

VERY busy. Only used for HDLC cards.

Link Active shows two values for each port. The first is the % of time

in the last minute that your station was transmitting, and the second

the % the channel was active (sum of Transmitting and DCD active). Only

maintained for HDLC ports. Note that of you are using SOFTDCD, then the

indicated % active may be an overestimate.

The L4T1 command displays or sets the Level 4 timeout used for the current

session. It is primarily for me to experiment with.

PACLEN sets or displays the PACLEN value used for messages generated by

the node (eg command responses). Again it is mainly for me to play with.

John Wiseman, G8BPQ

Revised 5/8/92

##########################################################################################################################