Digi-Comm Transceiver Interface

JOS Logic, Inc

Copyright 1991 (All Rights Reserved)

Version Beta 1.65 Notes

Digi-Comm was developed as a convenient PC based operator interface

for popular transceivers such as the Kenwood products. The current program

is mouse-based and requires virtually no keyboard input.  The final version

will be completely hands-free incorporating an inexpensive proprietary

Analog -> Digital card installed in your PC.  Voice recognition of an

approximately 500 word user defined library will be available along with

Digital Voice recording and playback.  Future integration with your rig's

existing microphone will require minimal modifications.  The voice

recognition system should be available early Fall '91.

This program will eventually be copy-protected.  I know, !&%$#@, what a

pain in the butt.  BUT, much time and money has been invested and having

the intact program appear on some BBS will bring this project to a grinding

halt. Future plans include: a) integrated DX logger and database, b) similar

control for multimode controllers, eg.AEA, Kantronics, MFJ and c) as

permitted by FCC, remote operation of your rig by computer and modem!

This as well as systems for all Yaesu, ICOM, Heath and Ten-Tec transceivers.

The price will be reasonable and certainly (IMHO) dollar for dollar,

better than anything out there now or in the near future.

Start-up operation is relatively straight forward but the certain

guidelines should be followed:

1.  The program MUST be installed on your hard disk drive in a

subdirectory called Digicom (note only 1 m in Digicom).  It is not

neccessary for you to create this subdirectory, the installation program

will do everything.  You must use the install (DEMOINST) program.  Log

onto the floppy or hard drive directory containing the compressed Digi-Comm

files.  If you received the programs on floppy disk then PKUNZIP will be

included.  Type PKUNZIP DGDEMO.ZIP to create the program files.  You can

do this in any directory of your choice.  Then type DEMOINST to install

the programs.  Remember to have your mouse driver loaded before doing

anything installation of files.  You must use DEMOINST first otherwise

the proper subdirectory structure will not be created and Digi-Comm will

ultimately fail.  Select the drive where the Digi-Comm ZIP files are

located and then enter the complete path of the subdirectory which contains

the unzip'ed Digi-Comm files.  For example, if you received the files on a

floppy diskette then no subdirectory path is needed, however, if you

downloaded the ZIP files from a BBS then the path will most likely be the

subdirectory containing your telecommunications program used to access

the BBS where Digi-Comm was downloaded from.

' D:\telecom\procomm '  as an example.

2.  Your can access the program easily in one of two ways.

Please don't be insulted if I'm being too explicit with

examples.

A.)  Add the Digicom subdirectory to your PATH statement

in AUTOEXEC.BAT using either DOS's EDLIN editor or

COPY CON:   (see DOS manual for explanations of both)

Reboot your PC and then you can start Digicom from

any subdirectory by just typing GO

PATH=C:\;C:\DOS;<drive>:\DIGICOM

B.) In the root directory, create a .BAT file called

"anything you like".  Put the following commands

in the file:

<drive>:

cd\digicom

go.bat        (with DOS > 3.3 use CALL go.bat)

cd\

Now you can just type "anything you like" to run

Digicom.  This file is enclosed on your diskette

and if you want to use this technique then copy it

to your root directory and NOT to the Digicom

subdirectory.

3.  After starting Digicom you will be prompted as a first time

user to configure the program for your transceiver.  This basically means

choosing the serial port settings and then letting Digicom determine which

transceiver you are using.  I would initially pick only the default values,

except for determining whether you are connected to COM1 or COM2 , etc.

This is a good time to talk about cables.  I found that some cables wired

for modem use would not work.  The cabling is easy to construct, just

follow the wiring diagram below:

PC-end (Female DB25)               IF232 Interface(MaleDB25)

1  -----  1

2  -----  2

3  -----  3

4  -----  4

5  -----  5

7  -----  7

4.   Changing the main display screen colors is up to you, the

default values are somewhat soothing on the eyes but you can always spice

them up for the visitors.

5.   Transceiver functions are selected by clicking your left mouse button

over an Icon button (see BUGS below).  With functions such as RIT, XIT,

LOCK, etc, the first click turns the function ON and a second click turns

it off HOWEVER with SCAN functions the second or OFF click must not occur

with the mouse cursor over that button.

The second click for OFF can be chosen with the mouse cursor placed anywhere

on the screen EXCEPT over the particular button in question.  This was

programmed in this manner for two reasons. First, turning off a scan function

at just the right moment and frequency shouldn't be impeded by having to

position the mouse cursor over some small button lying somewhere on a big

screen.  That's especially true if you fiddle with mouse as you're working

as I do and the cursor is never where you want it to be or expect it to be

when you need it.  Secondly, this design is necessary for future

implementation of the voice recognition system.

6.  There are several kinds of scan functions in Digicom.  The first and

easiest is a collection of buttons which allow you to scan continuously up

or down in 100, 10, 1, 0.1 or 0.01 increments.  This was included (by

multiple requests) for a fast and clean way of changing frequencies within

a band.  This function is timed, ie the scan increments or decrements

slowly for the first 5 seconds and if you don't turn it off it speeds up.

The scan delay time can be adjusted as an option with Menu button.  Let me

know if you have any suggestions on how this works.

Another scan function is the type of receiver scanning you're used to.

The range can be selected by inputting the upper and lower scan limits into

VFO A and VFO B respectively.  If A = B then scan will proceed UP the band

without an upper limit.  If A not = B then the scan is fixed between these

2 limits.  Scan proceeds DOWN the band to the lowest frequency and loops

until interrupted. Play around and I think you will find it easy.

Scan frequencies can also be saved (with text descriptions) in your

memory channels.  The Memory Channel button (MEM) is somewhat unusual in

operation.

With first time use only, you will be prompted to create a file which

will read and store all existing (if any) memory channels in your

transceiver.  After this occurs a list appears with channel information.

This list has a side-bar marker typical of window applications.  Clicking

the top or bottom arrows of the bar moves you instantly to the top and

bottom of the list.  The highlight marker shows your relative position in

the list.  Clicking above or below this marker moves you 10 channels at a

time,  up or down respectively.

Just below this display list you will see a line showing the current

transceiver frequencies.  The single letter after the frequencies

abbreviates the mode where L = LSB, U = USB, K = FSK, F = FM, etc.

If VFO A = 14,000.00 in LSB

VFO B = 14,500.55 in USB

It will look like:

AA  xx                  14,000.00L  14,000.00L

or

BB  xx                  14,500.55U  14,500.55U

depending on which VFO is active.

If SPLIT is on then it will look like:

AB  xx                 14,000.00L  14,500.55U

If you want to save this frequency(s) to your memory list simply

mouse click on an existing or blank channel on the display list.

Anytime you want to save to a channel, edit or delete a channel, or

activate a channel - just click on it and it will become highlighted.

Options:  (On a highlighted choice)

a. SAVE.MEM  -  takes current transceiver freqs and saves to highlighted

channel.  If not blank channel, this will cause new info to overwrite old.

As explained above, if you WANT to store  a split frequency, you must have

these frequencies in A and B already AND have the split state on before

going into MEM.

b.  EDIT     -  takes highlighted channel and allows adding or editing text.

When done just click the mouse button or hit the return/enter key on the

keyboard.

c.  SEARCH   -  does not require a highlighted channel.  Clicking on this

button will allow you to enter word (or number such as a frequency), part

of a word or character(s) for a rapid search of the entire memory list.

Great way for instantly going to important and frequently used freqs for

AMTOR, DX contests, repeaters, etc.  It will find only the first occurrence

of the characters you enter.  For example, if W2ORJ is listed on channel

10 and W2ORD is on channel 12.  Searching for W2OR will find W2ORJ first

and stop. (I haven't found this to be a practical problem, but if enough

people think so, I can set up some REPEAT function to keep searching

through the list.)

d.  DELETE   - takes highlighted channel and deletes ALL info.  If you just

want to change the text, use EDIT.

e.  PRINT    - prints a formatted list of all USED memory channels.

Blank channels are skipped over for printing.

g.  GET.MEM  - this should be only used if you somehow scramble

your memory file on the computer by trashing the disk, brown-outs, computer

lock-ups, etc.  It will re-read your transceiver's memory channels BUT you

will loose the text descriptions.  The nice thing about the way the memory

button function is now working is that it only depends on your transceiver's

settings the first time.  For all other times it uses a file stored on the

PC disk.  VERY IMPORTANT --- if you use the transceiver to change a memory

channel contents directly you may or may not foul up your ENTIRE list.

That's why we're still beta testing!

On the bright side however, this kind of set up will allow future

versions to have endless multiple lists which they can jump back

and forth from.

h.  SCAN    -  takes a highlighted channel and uses its two frequencies to

start a scan.  All scanning is computer generated and not dependent on the

transceiver scan button.  ( Kenwood transceivers tend to limit the user to

two scanning limits.  The TS-440, for example, uses channels 6-7 and 8-9.

Digicom allows you to use any and all of the channels to set up scan limits.

This has been fully tested on the TS-940 and TS-950, but according to the

Kenwood memory channel specs, it should work on any model.)   As noted

above, if the two frequencies are the same (UN-Split state) then scan goes

UP the band without an upper limit.  If the two frequencies are different

then scan proceeds DOWN the band to the lower limit and then loops.

i.  ACCEPT   - takes the highlighted channel and sets the transceiver to

that memory channel and settings.  If you selected a memory channel in

Split Mode with 2 different frequencies then clicking on the M->VFO button

should automatically place the transceiver in a split mode with the

frequencies in VFOA and VFOB ready for receiving, transmitting or scanning.

7.   Direct frequency input is available by clicking the INPUT  button.

A telephone like keypad appears and you can choose any combination.

Eg.     Click in 3,5      ---->  3,500.00

Click    3,51     ---->  3,510.00

click    3,520.2  ---->  3,520.20

If you like your setting click on the green button.  To erase or

start over click on the red button.

8.  Play around and become comfortable.  As with any computer, glitches

can occur and if this happens the worst conceivable scenario is

inadvertently keying the transmitter and keeping it on while you are

unaware.  In the transmit mode there is a flashing red box around the

Frequency display.  I would suggest first time users to select the MENU

button on the main screen and choose an Xmit time limit of their own choice.

Default is 10 minutes and after 5 minutes of continuous Transmit mode a

flashing red window appears warning you of transmit time.

9.  Kenwood did a so, so job designing the Transceiver interface.  In the

940 and 950 models, in the split mode, there's no way of knowing whether

you're split A/B, A/Mem Ch, B/Mem Ch. The communication time with serial

interfaces can be slow especially at 4800 baud or less.  There is a way of

jumpering the IF232 to 9600 baud and Kenwood has been promising me the

specs for a long time.  Line noise from the serial cable and the PC if

not shielded gives  spurious emissions and can be annoying.  USE GOOD

QUALITY CABLES.

10.  This is a beta version (1.65).  Missing from this version are the

following.

a:  UTC/Local

b:  Tone and Repeater Offsets

c:  Dumb Terminal Mode

d:  940S and 950S Advanced functions

i. integrating S-meter readings with an Auto-scan

ii. VBT and SSB slope tuning

iii. SUB functions

iv. TF-SET - this is not an available Kenwood RS-232

command but I'm working on a computer generated

equivalent.

e:  Finish User selected Window colors

f:  In Configuration, user to select which mouse button to use

g:  Full Windows 3.0 Support with multitasking

h:  There will be faster versions for 286/386/486 PC's

All the above will be in the final beta version

Please don't be embarrassed and give me ANY suggestions or

wish lists.  If feasible, I will implement them.

11.  BUGS:

a:  Quarterdeck's Expanded Memory Manager QEMM v 5.x causes some

glitches however Digicom does not require EMS.  However,

Microsoft's new DOS 5.0 seems to work fine with their EMM386.

If you encounter any glitches running Digi-Comm, first try

disabling any expanded memory.

b:  You tell me !

DISCLAIMER

All attempts have been taken to ensure the accuracy and integrity of this

program.  Digi-Comm and JOS-Logic makes no warranty nor any guarantees,

expressed or implied, regarding the safety or specifications of this product

nor in its use with any other particular product or purpose.

JOS - Logic, Inc

Compuserve: 70023,276