ICR7000

Icom IC-R7000 Receiver Control Program

THIS IS FREEWARE!

by

Bart Wolther

N2PXJ

437 Railroad Avenue

Westbury, NY  11590

IMPORTANT:

To greatly improve the functionality of this program, be sure to use the

Squelch-detect Cable described in this documentation. You can build one from

common, Radio-Shack-available parts.

HARDWARE REQUIREMENTS:

Icom R7000 Radio Receiver (or other CI-V compatible radio), CT-17 or

compatible radio interface, PC-compatible computer with serial port, cables

OPTIONAL HARDWARE:

Squelch-detect Cable (I tell you below how to make one) and parallel port

GENERAL DESCRIPTION OF THIS PROGRAM:

This is an Icom IC-R7000 receiver control program. I designed it to work

with the CT-17 computer interface. It allows you to maintain, on your PC,

databases of frequencies, and to download them to your radio.

TO START THE PROGRAM:

Start the program by typing:

ICR7000

CONFIGURING THE SOFTWARE:

The program has a SETU<P> option which allows you to specify several

parameters.  Be sure to run SETU<P> the first time you run the program.

SETUP PARAMETERS:

COMPORT: the serial port your radio interface is connected to. This

should be 1 (COM1) or 2 (COM2).

BAUD RATE: the baud rate at which your radio interface is set. This

should be 1200 or 9600.

LPT: the parallel port to which you have a Squelch-detect Cable, if any,

attached. This should be 1 (LPT1), 2 (LPT2), or 3 (LPT3). Instructions

for building your own Squelch-detect Cable are given below.

DATABASE FILE NAME: the name of a text file which will serve as a

frequency database. You can use any path/file name. See below for how to

build a frequency database.

EDITOR NAME: the name of a text editor which you will use to modify the

database. You can specify the name of your favorite editor, e.g. EDLIN,

EDIT, etc.

HOW TO BUILD A FREQUENCY DATABASE:

A frequency database is actually a simple text file. You build the file

using your favorite text editor, such as EDIT or EDLIN. Each line of the

file contains a frequency, mode and, optionally, a description of the

channel, separated by commas. For example:

145.0100, FMN, 2 Meter Packet Radio BBS

The line above specifies a frequency of 145.01 MHz, a transmission mode of

narrow FM, and a description: "2 Meter Packet Radio BBS".

The recognized mode designators are AM, FM, FMN, and SSB.

You can use the <E>DIT command to invoke your text editor (specified in the

SETU<P> command) with your database file (also in SETU<P>), and thereby edit

your database. You must use the <G>ET command (see below) to download your

database into the radio.

DOWNLOADING FREQUENCIES FROM A DATABASE TO THE RADIO'S MEMORY CHANNELS:

There is a memory channel <G>ET option which allows you to load frequencies

from the database file into the R7000. It asks you to specify a search

string, and then loads frequencies from all lines in your database file that

contain your search string. You may optionally specify two or more search

strings separated by commas, in which case an AND search will be performed

in which all frequencies containing all of the search strings will be

loaded.

Once channels are loaded from the database, you can use the PageUp and

PageDown keys to switch through memory channels, and the program will

display the corresponding channel descriptions as you switch.

THE SQUELCH-DETECT CABLE:

This program has the ability to recognize the R7000's squelch status which

allows the program to know if the radio is tuned to a coherent transmission.

To enable this capability, you must hook up a special "Squelch-detect

Cable". This cable is a quick and dirty solution to a major shortcoming in

the R7000's computer interface: an inability to detect the radio's squelch

status. The cable works by connecting your radio's REMOTE jack to your

computer's parallel port.

BUILDING A SQUELCH-DETECT CABLE:

To build the cable, you go (if necessary) to Radio Shack and buy three

things:

- a length of computer cable with 2 or more conductors R.S.#278-777

- a male, two-conductor, 1/8" mini-plug (i.e. an earphone plug)

R.S.#274-286

- a male DB-25 connector (25-pin D-Sub), crimp-type, R.S.#276-1429

Using the computer cable, connect the tip of the mini-plug to pin # 2 of the

DB-25 and the base of the mini plug to pin # 23 of the DB-25. Then plug the

mini plug into the REMOTE jack of your R7000, and plug the DB-25 to the

parallel port on your computer. That's all there is to it.