SB-948

TS-440S NOTCH FILTER LOW FREQUENCY IMPROVEMENTS   01/20/89

The low frequency attenuation characteristics of the notch

filter can be improved by reducing the input level to the

notch filter chip (IC1). To compensate for the loss of input

level, the amount of negative feedback to the audio

amplifier (IC7) will need to be reduced. This modification

is already incorporated in all models beginning with serial

number 704XXXX.

REQUIRED PARTS:

120K OHM, 1/6 WATT RESISTOR ........ RD14CB2C124J

390 OHM,  1/6 WATT RESISTOR ........ RD14CB2C391J

1.	Disconnect the power supply and antenna.

2.	Using a #2 Phillips screw driver, remove the 9 screws

from the top cover. Remove the top cover from the

transceiver and unplug the speaker wire.

3.	Locate resistors R86 and R229 on the component side of

the IF board (Figure 1).

4.	Using a #1 Phillips screw driver, remove the 7 screws

that secure the IF unit to the chassis of the

transceiver.

5.	Pull the board up and rotate it towards the front of the

transceiver to expose the foil side of the board.

6.	Desolder and remove both resistors from the board.

7.	Install and solder a 102K ohm resistor in place of R86.

8.	Install and solder a 390 ohm resistor in place of R229.

9.	Assemble the transceiver by reversing steps 1 - 5.

This is an optional change that may not be performed under

warranty.

Time required for this modification is 0.5 hrs or less.

(C) 110788TKC