LAN-LINK VERSION 1.59 PAGE 1

LAN-LINK Copyright (c) 1988, 1989, 1990, 1991 by G3ZCZ

All Rights Reserved

Joe Kasser G3ZCZ

POB 3419

Silver Spring

Md., 20918, U.S.A.

Voice (301) 593 6136

Data BBS (301) 593 9067

Compuserve (CIS) 70531,1405

_______

____|__     | (tm)

--|       |    |-------------------

|   ____|__  | Association of

|  |       |_| Shareware

|__|   o   |   Professionals

-----|   |   |---------------------

|___|___|   MEMBER

LAN-LINK  is  a Personal Packet Terminal Program  for  the  TNC1,

TNC2, KPC-2 and most of all a smart multimode digital  communica-

tions controller for the KAM, MFJ1278 and the PK-232.

The program is distributed as a Shareware product. You may freely

copy  and  share  the product for noncommercial  use,  with  your

friends,  associates and other radio amateurs. If you  decide  to

use  the  product, you are asked to become a registered  user  by

completing  the registration form and sending it, and  $35.00  or

equivalent  in foreign currency to the author. All checks  should

be drawn on US banks. Upon receipt of your registration, you will

receive one free update disk, telephone and mail (electronic  and

regular)  support.   If you are not satisfied  with  the  program

after  registering it, your money will be refunded  no  questions

asked,  for  up to 30 days from the date  your  registration  was

acknowledged.

European Radio Amateur users may register (24.00 Pounds Sterling,

includes VAT) and obtain support from Terry Dansey at  ReadyCrest

Ltd.,  PO  Box 75, Chatham, Kent, ME5 9DL,  England.   ReadyCrest

Ltd.   accepts  credit  cards  (Access,  Visa,   MasterCard   and

Eurocard).  Telephones:  Voice 44 (0)634-687168, FAX  44  (0)634-

687178, Data (BBS) 44 (0)634-200931.

LAN-LINK  may  not be sold or distributed  with  another  product

without the express written permission of Joe Kasser, G3ZCZ.

Joe  Kasser,  G3ZCZ will only support unmodified copies  of  this

software.

Potential Commercial Users please contact Joe Kasser directly for

modifications and/or details of Site licensing.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 2

TABLE OF CONTENTS

1.0 INTRODUCTION

1.1 PROGRAM HIGHLIGHTS

1.1.1 Packet Radio

1.1.2 AMTOR

1.1.3 Morse/BAUDOT/ASCII

2.0 Getting Started

2.1 Bringing LAN-LINK Up the First Time

2.2 Using the Quick Menu

2.3 Connecting to Other Stations

2.4 Upgrading from YAPP

2.5 Using Different TNCs

2.6 Using Different HF and VHF Configuration Files

2.7 Using the AMTOR/PACKET Robot/Automatic Contest Station

3.0 Things You Should Know

3.1 Memory Requirements

3.2 PC to TNC Communications

3.3 Open Loop

3.4 Problems

3.5 The Contents of the Status Window

3.6 Logbooks

3.7 How To Get an Update

4.0 Function Keys

4.1 Capture to Disk Toggle

4.2 Send Brag Tape

4.3 Transmit Callsign Sequence

4.4 Send/talk Brag Tape

4.5 Show Packet MHlist and Point and Shoot Connect

4.6 Auto CQ Call

4.7 Activate Type Ahead Buffer/AMTOR Relink

4.8 Disconnect/Receive

4.9 Put TNC in CMD Mode

4.10 Put TNC in CONVERSE /Transmit Mode /AMTOR FEC

4.11 Alternate Function Keys

4.12 Shift Function Keys

4.13 Control Function Keys

5.0 Main Menu

5.1 Quick Menu

5.2 Auto CQ

5.3 Send Brag Tape

5.4 Call Someone

5.5 Log Menu

5.6 Change Mode

5.7 Call CQ

5.8 Send/Talk File

5.9 QRZ

5.10 Send/Talk LAN-LINK files 001..010

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 3

6.0 BBS Menu

6.1 Download Text Files

7.0 Call Menu

7.1 Alert Call/ARQ Call

7.2 Connect/Call

7.3 Enter Call

7.4 reMember Path

7.5 Packet Cluster

7.6 Call CQ

7.7 Retry

7.8 Show Directory

7.9 Target Call

7.10 aUto CQ

7.11 Toggle DX Flag

7.12 Zap BBS

7.13 Zap BBS on Next Mail Beacon

8.0 Edit Menu

8.1 Edit Brag Tape

8.2 Edit Call Directory File

8.3 Edit any other file

8.4 Answer Ctdsk File (Split Screen)

8.5 Answer Incoming Messages (Split Screen)

8.6 Leave Note for Someone

8.7 Edit Capture-to-Disk File

8.8 Edit Two files (Split Screen)

8.9 Edit LAN-LINK.SYS File

8.10 Edit LAN-LINK.001 through LAN-LINK.010 files

8.11 Pick Capture-to-disk to Edit

8.12 Pick Message file to Edit

8.13 Leave Messages on your local BBS

9.0 Event/Clock Menu

9.1 Set Alarm

9.2 Connect

9.3 Display Event

9.4 Periodic Event

9.5 Turn Robot ON/OFF

9.6 Turn Contest Mode ON/OFF

9.7 Time Display ON/OFF

9.8 Cancel Event

9.9 Zap BBS

10.0 Files Menu

10.1 Send Brag Tape

10.2 File Directory

10.3 Erase File

10.4 Pick Ctdsk file to View

10.5 Rename File

10.6 Send File

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 4

10.7 Send/Talk File

10.8 Send Uppercase File

10.9 View file

10.10 View Ctdsk File

10.11 View Incoming Message File

11.0 Function key Help Information

12.0 Jump to Dos

13.0 LAN-LINK Menu

13.1 :QBU: External Protocols

13.2 :QBM: ASCII File

13.3 :QSM: Message

13.4 :QSP: Message

13.5 :QDB: ASCII File

13.6 :QRU: Messages

13.7 :QRA: Who is "ON-LINE"

13.8 :QTA: Messages

13.9 :QZD: Binary Files

13.10 :QZU: Binary Files

14.0 Log Menu

Log Mode Function Keys

14.0.1 Alt-A Append Entry

14.0.2 Alt-E Edit Log Entry

14.0.3 Alt-F Find an Entry

14.0.4 Alt-S Scan Log by Call

14.0.5 Alt-U Toggle Delete Mark

14.0.6 Alt-Z Pack Log

14.0.7 Ins Toggle Insert Mode

14.0.8 End Show Last Page

14.0.9 Home Show First Page

14.0.10 PgUp Move Up One Page

14.0.11 PgDn Move Down One Page

14.0.12 Up Arrow Move Up One Entry

14.0.13 Down Arrow Move Down One Entry

14.1 Append Entry

14.2 Scan Log by Band

14.3 Change Log

14.4 Find Date

14.5 Edit Log Entry

14.6 Find an Entry

14.7 Import-Convert Log

14.8 Help Function Keys

14.9 Scan Log by Mode

14.10 Print Log

14.11 Scan Log by Call

14.12 Update from Old data

14.13 Exit From Log Menu

14.14 Fuzzy Scan

14.15 Pick a Log file

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 5

14.16 Create a New Log file

15.0 Message Menu

15.1 BBS Command Files

15.2 Current Messages

15.3 View Message

15.4 Leave Note

15.5 Old Messages

15.6 Time Tag

16.0 TNC Menu

16.1 Set TNC Date/Time

16.2 Flush TNC Buffer

16.3 Initialize TNC

16.4 Show Calls Monitored List

16.5 Show Connect Status

16.6 Change CTEXT

16.7 Activate TNC Mailbox

16.8 Deactivate TNC Mailbox

17.0 AMSAT-OSCAR Menu

17.1 UoSAT ASCII Beacon

17.2 Phase 3 RTTY Beacon

17.3 SAREX Menu

17.3 1 Attack Mode ON/OFF

17.3.2 Change SAREX Call

17.3.3 SAREX Packets Capture to Disk ON/OFF

17.4 SAREX Orbiter Features

18.0 Parameter Menu

18.1 Turn Beacon Mode ON/OFF

18.2 Clear Connect Count

18.3 Turn Digipeat Detect ON/OFF

18.4 Turn Beacon Control ON/OFF

18.5 Turn Command Echo ON/OFF

18.6 Change Keyboard Beep Count

18.7 Set Message Count

18.8 Turn NODE Drop Link ON/OFF

18.9 Turn Printer ON/OFF

18.10 Turn QTC Snatch ON/OFF

18.11 Turn :QSP: Relay Flag ON/OFF

18.12 Turn Sound ON/OFF

18.13 Turn Contest Mode ON/OFF

18.14 Turn DCD Flag ON/OFF

18.15 Turn BBS Snatch ON/OFF

18.16 Turn Time Id. ON/OFF

18.17 Turn Automatic Capture-to-Disk Flag ON/OFF

18.18 Change BBS Parameters

18.18.1 Local BBS Callsign

18.18.2 Mail-Snatch Trigger Word

18.18.3 Read Request Command

18.19 Change Callsigns

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 6

18.19.1 Callsign

18.19.2 The SELCAL

18.19.3 The PacketCluster Call

18.20 Change Directories

18.20.1 QBM Download Directory

18.21 Enter Common Calls

18.22 Change File Names

18.22.1 VHF Logbook File (VHF.DBF)

18.22.2 HF Logbook File (HF.DBF)

18.22.3 Path Directory File (LAN-LINK.DIR)

18.22.4 Brag Tape (LAN-LINK.TXT)

18.22.5 QBU Upload File Name (QBU-TX.BAT)

18.22.6 QBU Download File Name (QBU-RX.BAT)

18.23 Change LAN-LINK Parameters

18.23.1 Change CQ Text

18.23.2 Set the Default Band

18.23.3 Set the Default Transmitter Power

18.23.4 Set CQ Delay Time Seconds

18.23.5 Set CQ Delay Time Minutes

18.23.6 Set Number of Lines in a Non Contest CQ Call

18.23.7 Set Shut Down Mode (QRT Flag)

18.23.7.1 QRT

18.23.7.2 QRV

18.23.7.3 QRV + CTEXT TOGGLE

18.23.7.4 BBS

18.23.7.5 SOLO

18.23.7.6 BBS+SOLO

18.23.8 Set Default Users

18.23.9 Set UTC Offset

18.24 Change PC Communications Parameters

18.24.1 The Communications Port

18.24.2 The Terminal Baud Rate

18.24.3 TNC Data Bits

18.24.4 TNC Parity

18.24.5 Number of TNC Stop Bits

18.25 Change Colors

18.25.1 OutWindow Color

18.25.2 StatusWindow Color

18.25.3 PromptWindow Color

18.25.4 LogWindow Color

18.25.5 InWindow Color

18.25.6 ScrollBackWindow color

18.25.7 MHWindow Color

18.25.8 Alarmwindow Color

18.25.9 BottomWindow Color

18.25.10 Emphasis Color

18.25.11 Prompt Color

18.25.12 Option Color

18.25.13 Connected Text Color

18.25.14 Computer Color

18.25.15 VHF Header Color

18.25.16 HF Header Color

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 7

18.25.17 Alert Call Color

18.25.18 Outgoing Text Color

18.25.19 Multi I/O Stream Color[1]

18.25.20 Multi I/O Stream Color[2]

18.25.21 Multi I/O Stream Color[3]

18.25.22 Multi I/O Stream Color[4]

18.25.23 Multi I/O Stream Color[5]

18.25.24 Multi I/O Stream Color[6]

18.25.25 Multi I/O Stream Color[7]

18.25.26 Multi I/O Stream Color[8]

18.25.27 Multi I/O Stream Color[9]

18.25.28 Multi I/O Stream Color[10]

18.26 Change SAREX Parameters

18.26.1 SAREX CallSign

18.26.2 Change SAREX Capture to Disk File

18.27 Update LAN-LINK.SYS

18.28 Verify Changes

18.29 Change Scan Words

18.30 Turn Misc. Flag ON/OFF

18.30.1 All Modes:- Debug or state-sequence numbers

18.30.2 BAUDOT Mode:- Navy MARS Protocol Verify

18.31 Change Band/Mode

18.32 Zap BBS on Next Mail Beacon

18.33 BBS Zap, Don't Disconnect After Sequence

19.0 Q Codes [NC/L]

20.0 Communications Modes

20.1 VHF PACKET

20.2 HF PACKET

20.3 ASCII (RTTY)

20.4 BAUDOT (RTTY)

20.5 MORSE (CW)

20.6 AMTOR (STANDBY RECEIVE)

20.7 AMTOR (ALIST [CHIRPCOPY])

20.8 SIGNAL

20.9 NAVTEX

21.0 Terminal Mode Menu

21.1 Packet Terminal Menu

21.1.1 Change Packet Baud Rate

21.1.2 CQ/Beacons

21.1.3 Everything

21.1.4 Read the Mail

21.1.5 Just My Station (Solo)

21.1.6 All Traffic

21.1.7 Single User Connects

21.1.8 Multiple User Connects

21.1.9 Conference

21.1.10 Conference Bridge On/Off

21.1.11 Turn Metabeacon On/Off

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 8

21.2 AMTOR Terminal Menu

21.2.1 Force Break [ACHG]

21.2.2 Start Mode B [FEC]

21.2.3 Force LETTERS case

21.2.4 Read the Mail [ALISTEN]

21.2.5 Override ROBOT/Mailbox

21.2.6 Standby Receive [ARQ]

21.2.7 Set ROBOT/Mailbox

21.2.8 CW Identification

21.2.9 Set ROBOT infinite Delay

21.3 Baudot Terminal Menu

21.3.1 Set Baud Rate

21.3.2 Select Shift

21.3.3 NAVY MARS Message Protocol

21.3.4 USOS On/Off

21.3.5 SELCAL set/reset

21.3.6 CW Identification

21.4 ASCII Terminal Menu

21.4.1 Set Baud Rate

21.4.2 Select Shift

21.4.3 SELCAL On/Off

21.4.4 CW Identification

21.5 CW Terminal Menu

21.5.1 Lock to Receive Signal

21.5.2 Set Morse Speed

21.5.3 Unlock Morse Speed

22.0 eXit to DOS

23.0 Hot Key Combinations

23.1 Alt-A - Alert Call (Packet)/ARQ Call/Talk (AMTOR)

23.2 Alt-B - Send Break/AMTOR Changeover

23.3 Alt-C - Call/Connect to

23.4 Alt-D - Disconnect or Receive

23.5 Alt-E - Enter Call

23.6 Alt-F - Flush TNC Buffer

23.7 Alt-J - Jump to DOS Shell

23.8 Alt-K - Jump to DOS Shell with Quick Return

23.9 Alt-L - Edit Log

23.10 Alt-M - Message Scan

23.11 Alt-N - Multi Connect Stream Next

23.12 Alt-O - Turn Robot On

23.13 Alt-P - Turn Printer ON/OFF

23.14 Alt-Q - Call QRZ

23.15 Alt-R - Retry Call/Connect

23.16 Alt-S - Scan Log

23.17 Alt-X - Exit to DOS

23.18 Alt-Y - Zap Packet Cluster/(BAUDOT) Send RYs

23.19 Alt-Z - Zap BBS

23.20 Alt-= - Toggle Miscellaneous Flag

23.21 End - > +? [Over]

23.22 PgUp - Scrollback (Scrolls back incoming text)

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 9

23.23 The HOME Key

23.24 The DOWN ARROW Key

23.25 Shift/Baudrate Control keys

23.26 Control-Z (Ctrl-Z or ^Z)

24.0 LAN-LINK Disk files

24.1 LAN-LINK.EXE

24.2 LAN-LINK.SYS

24.3 *.RUN (YYMMDD.RUN)

24.4 *.DBF (VHF.DBF and HF.DBF)

24.5 LAN-LINK.DIR

24.6 CONFIG.SYS

24.7 *.MBX

24.8 *.BBS

24.9 LAN-LINK.001 .. LAN-LINK.010

24.10 QBU-RX.BAT

24.11 QBU-TX.BAT

24.12 LAN-LINK.QTA

24.13 QZ*.BAT

25.0 Mailbox features

25.1 Answering Machine

25.2 Mail Beacon (Annunciator)

25.3 How to leave a message

25.4 Remote Beacon shutdown

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 10

APPENDIX 1. Anomalies and Bugs

A1.1 Command/Converse Mode anomalies

A1.2 Buffer messages in AMTOR Beacon CQ Mode

A1.3 FEC +? responses in AMTOR Beacon CQ Mode

A1.4 Using a CGA card with a monochrome monitor

A1.5 The Packet Conference Mode

A1.6 Packet Conference Mode disconnects

A1.7 Strange Events

A1.8 *** DISCONNECTED

A1.9 *** CONNECTED

A1.10 Greek Characters on Incoming Screen

A1.11 Dumb TNC

A1.12 Intermittent Lockup

APPENDIX 2. Local Area Network Capability

A2.1 LAN Protocol (G3ZCZ Version)

A2.2 Message Format

A2.3 ASCII File Uploading/Downloading

A2.4 Path Determination to a DX Station

A2.5 Binary File Transfers

APPENDIX 3. NC/L Dictionary

A3.1 :QBM: ASCII File Download

A3.2 :QDB: ASCII File Upload

A3.3 :QMH: Monitor Heard List

A3.4 :QSM: Send Repeat of Message

A3.5 :QSP: Send Message

A3.6 :QRA: Who is About?

A3.7 :QRT: Shut Up

A3.8 :QRU: Upload Messages in Batch

A3.9 :QNO: Negative Response

A3.10 :QJG: Task Completed Response

A3.11 :QRV: Ready Response

A3.12 :QSL: Confirmation Response

A3.13 :QTA: Messages

A3.14 :QTC: List of Messages

A3.15 :QBU: External Protocol File Transfer

A3.16 :QZU: Zmodem Binary File Transfer Upload

A3.17 :QZD: Zmodem Binary File Transfer Download

A3.18 :QYU: YAPP  Binary  File  Transfer  Upload  (not   yet

implemented)

A3.19 :QYD: YAPP  Binary  File Transfer  Download  (not  yet

implemented)

APPENDIX 4. USING OTHER TNCs

A4.1 TNC2 (MFJ 1270) Version 1.1.2

A4.1.1 Default Commands

A4.1.2 Program differences

A4.2 TNC1 (HD-4040)

A4.2.1 Default Commands

A4.2.2 Program differences

A4.3 Kaypro KPC-2

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 11

A4.3.1 Default Commands

A4.3.2 Program differences

A4.4 KAM

A4.4.1 Default Commands

A4.4.2 Program differences

A4.5 PK-232 EPROM Versions JUN 1987 and later

A4.6 MFJ1278 EPROM Version 3.3

A4.6.1 Default Commands

A4.6.2 Program differences

A4.7 Heath HK-21

A4.8 PK 87/88

APPENDIX 5. UPDATES/REVISION HISTORY

APPENDIX 6. Registration Form

APPENDIX 7. Problem Reporting Form

APPENDIX 8. Other PC Software by Joe Kasser G3ZCZ

APPENDIX 9. How Shareware Works

ACKNOWLEDGMENTS

Thanks to Milt, 4X1AA. If it wasn't for Milt, Version 1.00  would

never have been written.

Thanks  to  Jim, WB8QJL, who loaned me his  MFJ1278  for  several

months but died in a plane accident before he got to use LAN-LINK

on his MFJ1278.

READ THIS NEXT BIT CAREFULLY

I've  recently  received several messages via packet  radio  that

were  questionable  as to their commercial content,  I've  had  a

couple that definatly were commercial in nature.

As the interpretation of what is comercial and what is not, is  a

personal  matter, it is with regret that I'm going to  adopt  the

following policy.

If  your  message  requires me to take  individual  action,  then

please  use  my  landline  BBS, the  post  office,  telephone  or

Compuserve.

If  you  don't  use packet to send queries  to  manufacturers  of

hardware, why do you send them to me. My packet use is for  hobby

only.  Please let me keep it that way.

With regrets and 73 Joe G3ZCZ@N4QQ

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 12

1.0 INTRODUCTION

LAN-LINK  is a Personal Digital Communications  Terminal  Program

for  a Packet Radio TNC. The program is designed to optimize  the

configuration  of  the  TNC in each communications  mode  and  to

provide  some smart terminal features. It takes advantage of  the

extra   features  of  the  PK-232  which  is  made  by   Advanced

Electronics  Applications Inc. It is designed to allow anyone  to

use and get the most out of their packet TNC as well as from  the

PK-232,  the MFJ1278 and the KAM for Morse, ASCII, BAUDOT,  AMTOR

as well as Packet Radio communications without having to keep the

manual handy.

LAN-LINK is a sophisticated program. In its basic state it allows

you  to use the TNC in an optimal manner. It configures  the  TNC

(it  types the commands) for you to maximize  the  communications

efficiency in the communications mode of your choice. That means,

for  example, when working Packet on HF you need to  program  the

TNC  parameters to different values than you would use on VHF  to

make  maximum use of the mode. One significant difference is  the

length  of the packet itself, for the longer it is,  the  greater

the  probability of QRM destroying it. This program  will  adjust

the packet parameters for you.

Since  the  computer is now involved, other  features  have  been

added  to  simplify  operation, and several  features  have  been

automated.  All  these operations are performed using  menus  and

function keys as documented below. It will take a while to  learn

how  to use this program in a manner which suits you,  read  this

document and have fun, after all, isn't that one of the  purposes

of Amateur Radio.

This  manual is organized by menu. It describes the operation  of

each  menu option and function key in each  Communications  Mode.

THE  MANUAL TELLS YOU WHAT LAN-LINK CAN DO, IT DOES NOT TELL  YOU

HOW TO USE LAN-LINK.

Any  Terminal  Node  Controller  (TNC)  is  a  complex  piece  of

equipment. It can operate in a Command Mode, in which you tell it

to do something, or in a Converse Mode in which you are using  it

to talk to other stations. Many people confuse the two when first

getting on Packet. If you monitor the channels you will recognize

Command  Mode TNC instructions on the air, and when you  use  the

TNC  you will receive the "error" reply when you  type  something

thinking  you  are  in the Converse Mode but are  really  in  the

Command  Mode.  LAN-LINK  is designed to  make  Packet  operation

simple  using  Menus  and  high  level  (Function  and  Hot   key

combinations) commands.

Morse,  ASCII, BAUDOT communications modes are HALF  DUPLEX.  One

station  is  transmitting at any time. You thus have to  use  the

Transmit/Converse  and Receive/Disconnect function keys  to  turn

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 13

the transmitter on and off respectively.

AMTOR is also HALF DUPLEX but has some of the attributes of  full

duplex. In this Mode you control who is talking through software,

only  one  station  can send data at a time,  but  you  can  type

characters  in  to  the  buffer  and  they  will  be  transmitted

automatically  when the other station turns the QSO over to  you.

Once   you  have  linked  with  someone,  you  do  not  use   the

Receive/Disconnect function UNTIL THE QSO IS OVER. Both  stations

are  communicating automatically thanks to the TNC. If  you  want

the  other operator to respond type the +? key sequence  (or  use

the 'End' Key).

Packet  Radio communications on the other hand is a  full  duplex

mode of communications. Once you have connected with someone, you

do not use the Receive/Disconnect function UNTIL THE QSO IS OVER.

Both stations are communicating automatically thanks to the  TNC.

If you want the other operator to respond type the > key followed

by the 'Enter' Key, better yet, use the 'End' Key.

Note   Morse,  AMTOR,  ASCII  and  BAUDOT  are   character   mode

communications  in  that each character you type is sent  as  you

type  them. In Packet Communications, the TNC stores them up  and

sends out a burst or packet when you type the 'Enter' Key or  you

type  enough characters to fill up a packet. Studies  have  shown

that the human attention span when sitting at the keyboard is  of

the  order  of 2 seconds. If you are in a  keyboard  to  keyboard

packet  radio connect, remember this and don't forget to use  the

'Enter' Key frequently, especially before you stop to think about

what to send next.

Note  AMTOR and PACKET are full duplex modes  of  communications.

When  you  connect  with someone, there is no  need  to  use  the

transmit  and  receive  function  keys. In  fact  you  will  have

problems if you do. Do not disconnect or break the link until the

QSO is over.

1.1 PROGRAM HIGHLIGHTS

Function key and Menu driven.

Automatic logbook entries for Packet and Mailbox/Beacon  Mode

AMTOR  Connects,  semiautomatic  logbook  entries  for  other

modes.

Logbook file is dBASE compatible and can be processed by  the

LOGBOOK  Package  of PC-HAM by G3ZCZ  for  indexed  listings,

tracking of DXCC and other AWARDS, etc.

Contest  operation, sends standard message and  automatically

increments QSO count.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 14

Automatic  optimized  configuration  of  the  TNC  for   each

communications mode.

All mode Function key 'OVER' feature (End).

There  are  10 files with fixed names  (LAN-LINK.001  through

LAN-LINK.010) which may be viewed and transmitted by means of

function keys. They may also be edited from the Edit Menu.

Set up of TNC for AMSAT-OSCAR Telemetry reception.

Time display and event scheduler.

ASCII Text Editor.

Customizable Colors.

Access  to the TNC Command Mode is provided in case the  user

wishes to override any defaults.

1.1.1 PACKET RADIO

Automatic  capture  to  disk of all  packet  radio  connects.

Automatic indication of the number of Packet connects.

Local   Area   Network  (LAN)  message  store   and   forward

capability.

Capable of automatic connect attempts to download a QTC  from

another station in the LAN.

Capable  of  automatic connect attempts to a  packet  BBS  to

download  your incoming messages, when your callsign  appears

on the BBS mail beacon annunciator.

Capable  of  automatically requesting Bulletins  on  subjects

that interest you from your local packet BBS.

Digipeat monitoring and capture.

Alert signal to let you know when a predetermined call  shows

up in a packet header on frequency.

Conference and Bridge modes in multiconnect situations.

Path determination to DX station via :QMH:.

Indicator that a specific station designated as the  'target'

call   connected   in  Packet  Mode,  or  linked   to   AMTOR

Beacon/Mailbox while you were away.

Automatic  NET/ROM and KA Node path set up from  LAN-LINK.DIR

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 15

call/path directory file.

Selective  answering machine and MAILBOX using  NC/L  command

dialogue.

Screen  indication  of  connect by  desired  station  (target

call).

Automatic Beacon Mode CQ caller

Automatic contest (DX-pedition) mode

Will  call  CQ repetitively and either work the  connect  and

keep  going  after disconnect or signal you when a  reply  is

received.

Zmodem binary file transfer capability

SAREX special features

:QRA: trigger to determine who else is on channel

Single keystroke Packet Cluster connect

1.1.2 AMTOR

Automatic SELCAL determination.

Automatic CQ caller. Will call CQ repetitively and signal you

when a reply is received.

Local   Area   Network  (LAN)  message  store   and   forward

capability.

Selective  answering machine and MAILBOX using  NC/L  command

dialogue.

Function  key  change from monitoring FEC CQ's  to  QSO's  in

progress (chirpcopy).

Automatic Beacon Mode CQ caller

Automatic contest (DX-pedition) mode

Color changes between incoming and outgoing (echoed as  sent)

text.

1.1.3 MORSE/BAUDOT/ASCII

Automatic CQ caller. Will call CQ repetitively and signal you

when a reply is received.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 16

RTTY SELCAL

NAVY MARS RTTY file transmitting protocols for the PK-232.

2.0 Getting Started

MAKE  A  BACK UP COPY OF THIS DISK, BEFORE YOU REMOVE  THE  WRITE

PROTECT TAB.

If  you don't have a PK-232 then rename the LAN-LINK.SYS file  to

PK232.SYS  and  rename  your TNC.SYS file  as  LAN-LINK.SYS.  For

example,  if  you  are  using  a  TNC2,  perform  the   following

operations in DOS.

RENAME LAN-LINK.SYS PK232.SYS

RENAME TNC2.SYS LAN-LINK.SYS

For details of the rest of the customization possibilities in the

'LAN-LINK.SYS'  file, read Section 23.2. YOU WILL NOT BE ABLE  to

change the TNC Type from within LAN-LINK. YOU MUST rename the SYS

files BEFORE you begin the customization sequence.

2.1 Bringing LAN-LINK Up the First Time.

2.1.1 Turn on the TNC.

2.1.2 If  the  date and time in the PC are not  set  to  UTC

(GMT), set them using the DOS date and time commands.

2.1.3 Load  the  program by typing LAN-LINK in the  way  you

normally load a program.

2.1.4 WAIT until the program says it has initialized the TNC

and  shows the band and power in the status window  at

the top of the screen.

2.1.5 If  there  is no battery in your TNC do what  the  TNC

manual  tells  you to do to synchronize  the  TERMINAL

baud  rate  to 1200 baud. In the case of  the  PK-232,

LAN-LINK  will send the PK-232 three *  characters  to

set the PK-232 to 1200 baud.

2.1.6 After  LAN-LINK  has loaded, use the  Parameters  Menu

option  "2"  to replace G8BTB with your  callsign  and

Selcal information. If your TNC does not support AMTOR

you  still need a dummy entry in that line  (which  is

taken care of automatically). You may also change  the

colors  at this time. Use the "U" option to  save  the

updated  file. After reading the rest of this  manual,

you  may want to change other defaults to set up  LAN-

LINK the way you like it.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 17

2.1.8 Use  the  "N"  and  "I" options in  the  TNC  Menu  to

configure  the  TNC. Then select the desired  mode  of

operation  from  the displayed menu  by  pressing  the

appropriate key.

2.1.9 Exit  from LAN-LINK by using the Alt-X Key or the  Esc

Key followed by the X Key.

2.1.10 Reload  LAN-LINK  the same way you did  before.  This

will  load the edited LAN-LINK.SYS file with  the  new

settings.  You are now ready to use LAN-LINK. If  this

is  your first time, try the Quick Menu (See  Sections

2.2 and 5.1).

2.1.11 If  you plan to use the Zmodem binary  file  transfer

capability  of  LAN-LINK and (you are not  using  Com1

serial  port  on your PC) or (you are not  using  1200

baud  between  the  PC  and the  TNC)  then  you  MUST

customize  all FOUR of the QZ*.BAT files as  described

in Section 24.13 and Appendix 2.5.

2.1.12 If  you  are  updating  from  previous  versions  the

following  applies.  The  LAN-LINK.SYS   configuration

files  are incompatible with any version earlier  than

1.58.  Thus if you are upgrading from 1.56 or  earlier

versions,  use the configuration files  supplied  with

this  version. If you are upgrading from 1.58,  ignore

the  error message you get on the first turn on,  then

bring   up  the  Parameters  Menu,  and   Update   the

configuration file. Review Section 18 then choose  the

LAN-LINK Parameters option and change your GMT offset.

Note EST is 5, PST is 8 etc.

NOTES

If  you have batteries in the TNC, make sure the TNC is  set

for  a  terminal  baud  rate of 1200, and  try  to  run  the

program.  If  you see Greek characters or it  doesn't  work,

remove the batteries and try again (Refer to Appendix 1.10).

You  can  then replace the batteries and from  then  on  the

program will be correct. Make sure you leave the TNC on when

you replace the batteries or your initialization will revert

to  the  TNC default settings - the batteries are  there  to

power the TNC RAM while your primary power is off.

If  you  have trouble with a PK232MBX, then,  with  LAN-LINK

running, turn the PK232MBX off, count to ten and turn it  on

again.  Type one * character. If that fails, repeat  this  a

few times. If it still fails, open the PK232 and remove  the

battery  jumper. count to 10 slowly before replacing it  and

try again.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 18

2.2 Using the Quick Menu

The  Quick Menu is designed for new users and  provides  the

most commonly used functions in a single Menu. You bring  up

the Quick Menu be touching the Escape key to access the Main

Menu and then selecting the 'A' option.

The Quick Menu is described in Section 5.1.

2.3 Connecting to Other Stations

LAN-LINK  treats  incoming connects  and  outgoing  connects

differently.  To connect to another station, Use  the  Alt-C

hot  key, Function key 5, or bring up the Quick  Menu  using

the 'Esc A' character sequence as described in Section  5.1,

or  by bringing up the Call Menu via the 'Esc  C'  character

sequence  as  described in Section 7. DO NOT TYPE  'C  Call

sign' in the command mode. If you do, the connect will still

take place (the TNC won't know if you or LAN-LINK issued the

command)  but LAN-LINK will treat it as an incoming  connect

and issue the "[ZCZ] LAN-LINK 1.59>" handshaking signal  (if

the  LAN-LINK flag is in its default, or enabled  state)  as

described  in Section 18. If you connect to a BBS or a  Node

this way, you will get an error message back from the BBS or

node, since it does not recognize LAN-LINK's handshake.

When  you  use the menu, Alt-C or F5 approach you  can  also

make  use of the 'path memory' feature in  the  LAN-LINK.DIR

file.

2.4 Upgrading from YAPP

A  YAPP.CNF file is provided to facilitate  conversion  from

YAPP to LAN-LINK. YAPP.CNF is a configuration file for  YAPP

Version  2 that will configure the PK-232 for YAPP and  then

back to LAN-LINK.

If you are currently using YAPP, replace your YAPP.CNF  file

with  this one and load YAPP. Then terminate the program  in

the normal manner by using the Alt-X Key. You should then be

able  to  load and run LAN-LINK without any  problems.  Each

time you subsequently want to run YAPP, YAPP will set up the

PK-232,  execute  and then reconfigure the PK-232  back  for

LAN-LINK.

If you are using a YAPP.DIR file, rename it to LAN-LINK.DIR.

2.5 Using Different TNCs

There  are differences between the command dialogue and  the

features  offered by the PK-232, KAM and  MFJ1278.  LAN-LINK

makes use of many of them. For example, as the KAM shows the

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 19

HF  and VHF packets as if they ware separate  streams,  LAN-

LINK  detects  them and displays the  different  headers  in

different colors. As this feature is not present in the  PK-

232, LAN-LINK does not offer it. From time to time, as  each

manufacturer  updates the firmware (EPROM) in the TNC,  LAN-

LINK may be updated and incorporate them.

LAN-LINK  is normally loaded by typing LAN-LINK followed  by

the  'Enter'  key.  In  this  instant,  LAN-LINK  gets   its

configuration  information from the LAN-LINK.SYS file.  LAN-

LINK.SYS is the default SYSTEM or configuration file. If you

have  more  than one TNC you may want to  set  up  different

configuration files for each TNC.

You must set up your different configuration files with  the

filetype ".SYS", examples being TNC1.SYS, TNC2.SYS, KAM.SYS,

PK232.SYS  as  well as the default  LAN-LINK.SYS.  Edit  the

sample  files on the distribution disk to customize them  to

your requirements.

To load LAN-LINK with the optional configuration file,  type

'LAN-LINK'  followed  by the name of the file  (without  the

filetype)  on the command line then press the  'Enter'  Key.

For   example,  if  you  have  a  PK-232  and  a  TNC2   and

occasionally use the TNC2, set up the LAN-LINK.SYS file  for

the  PK-232.  Then  type  'LAN-LINK'  to  run  the   program

normally.  On  the rare occasions when you want to  use  the

TNC2, you may type 'LAN-LINK TNC2' to run LAN-LINK with  the

TNC2.SYS configuration file.

2.6 Using Different HF and VHF Configuration Files

You  can  also  use two SYS files, one for HF  and  one  for

VHF/UHF.  If you name them HF.SYS and VHF.SYS you can  bring

the relevant one up by typing

'LAN-LINK VHF' or 'LAN-LINK UHF'

as desired. You can also use a different one for a  contest,

or for any reason you wish.

2.7 Using the AMTOR/PACKET Robot/Automatic Contest Station

In  order  to use the Robot in the AMTOR Mode, you  need  to

have  a  Brag Tape file. The default name for this  file  is

LAN-LINK.TXT.  If this file does not exist, you will not  be

able to activate the Robot.

You   activate  the  Robot  by  using  the  Alt-O  hot   key

combination.  The  Robot will then call CQ and  wait  for  a

reply. If no reply is received, it will call again. It  will

continue  to  call  at preset intervals  until  a  reply  is

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 20

received  (you  may change the interval via  the  Parameters

Menu).  In AMTOR, the Brag Tape will be transmitted and  one

(and  one  only)  transmission  accepted  from  the  linking

station,  unless the linking station uses a NC/L command  on

the  same  line  as the +?. In that  case,  the  Robot  will

respond to it. All incoming text will be captured to disk in

the Capture-to-disk file (LAN-LINK.RUN). The Robot will then

sign  off  and  drop  the link. If it  has  not  managed  to

recognize a valid call from the linking station it will  ask

for a retransmission of the callsign.

In  the Packet Communications Mode, the CTEXT will  ONLY  be

transmitted  if you have set CMSG to ON (in the TNC  Command

Mode),  the Brag Tape will not be transmitted. NC/L will  be

active.  When the connecting station disconnects, the  Robot

will restart the CQ cycle.

In AMTOR turn the answerback off if the Robot is on. In this

way  other  stations  will  not  be  able  to  mess  up  the

sequencer.

When  you touch the F8, F3, Alt-D, Alt-Q or 'End' keys,  you

turn the Robot off.

The  PK-232's  flush command is used before  each  robot  CQ

message is transmitted to synchronize the text to the  Robot

software. The KAM however, does not have a way to flush  its

internal  transmit  buffer.  Under  QRM  conditions  or   if

stations do not sign on correctly, the sequence can get  out

of order due to the wrong text being transmitted.

To  use the Automatic Contest mode, enable the contest  mode

from  the Parameter menu, then activate the Robot using  the

Alt-O function key. To turn the Automatic Contest Mode  off,

deselect  (turn  off) the contest mode  from  the  Parameter

Menu.

3.0 Things You Should Know

3.1 Memory Requirements

LAN-LINK  1.59 requires a minimum of 450k of RAM.  You  will

need  more  RAM if you wish to use the  Editor.  The  Editor

files  will  each take a maximum of 64K additional  RAM.  If

your  memory is marginal, you may get an error when you  try

to log a station or edit a file/message.

3.2 PC to TNC Communications

All communications between the computer and the TNC normally

use  the 8 BIT NO PARITY and 1 stop bit RS-232  Serial  Port

configuration. You CAN THUS SEND FOREIGN LANGUAGE CHARACTERS

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 21

FROM  THE KEYBOARD OR FILES. If you want to limit the  char

acter  set  to the 7 bit ASCII set, set the  TNC  parameters

accordingly. The baud rate defaults to 1200, but you may set

it  to  2400, 4800 9600 or even 19200 if your CPU  clock  is

fast enough and the TNC will respond correctly.

3.3 Open Loop

IF YOU USE THIS PROGRAM, YOU STILL MUST READ THE TNC MANUAL.

WARNING .... THIS PROGRAM OPERATES IN AN OPEN LOOP MODE.  IT

REMEMBERS WHAT INSTRUCTIONS OR COMMANDS IT GAVE THE TNC.  IF

FOR SOME REASON, THE TNC IS NOT SYNCHRONIZED TO THE  PROGRAM

YOU WILL NOT GET CORRECT DISPLAYS ON THE SCREEN.

3.4 Problems

In case you have problems, always check the default settings

using  the 'Verify Changes' option of the  Parameters  Menu.

Further information is contained in Appendix 1. Problems can

be  caused by incorrect settings of the TNC  parameters.  If

all  else fails, (hard) reset the TNC. Make sure  XFLOW  and

FLOW are OFF.

3.5 The Contents of the Status Window

The indicators in the status line depend on the state of the

LAN  Link and are described in the sections that  deal  with

the  functions they area associated with. This section is  a

summary of the displays in the Status Window.

Some typical status window displays are shown below.

2 Meters 2 Watts PACKET 1200 W [R] TRFC ->N4QQ

20 Meters 100 Watts MORSE TEST[4]

20 Meters 100 Watts PACKET 300 N TRFC

CAPTURE TO DISK PACKET 300 N [C1][Z] TRFC ->N4QQ

If the automatic Capture-to-disk is turned off, a yellow bug

will be flashing on the left side of the line.

If the Printer Flag is set, a yellow [P] will be flashing on

the left side of the line.

If  the miscellaneous flag is set, an asterisk '*'  followed

by two numbers and a ']' will be displayed. The first number

is  the  BBS  Sequence  State, the  second  is  the  Connect

Sequence State. These numbers reflect the internal state  of

LAN-LINK and should be quoted when describing problems.

If  a message transfer is taking place, the callsign of  the

station involved will be displayed.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 22

When  the Capture-to-Disk is turned on, a 'CAPTURE TO  DISK'

will  be  shown,  if it is turned off,  the  operating  band

(Meters) and power (Watts) will be shown. If a ZapBBS is  in

operation, the Bulletin Request Count will then be shown  in

square brackets ('[]').

The communications mode will be shown next, it may be one of

the following;

PACKET AMTOR AMTOR-(MAIL) AMTOR-FEC

BAUDOT ASCII MORSE NAVTEX

SIGNAL ???? DEFAULT {boot up}

A flashing yellow musical note following the  Communications

Mode indicates that the sound has been inhibited.

The  next  number to be displayed is the  RF  communications

baudrate.  The  letter 'W' or 'N' indicates  that  the  Wide

shift or the Narrow shift has been selected. A 'down  arrow'

will  be shown if the shift is normal. If the shift  is  re

versed a flashing yellow 'up arrow' will be displayed.

In the BAUDOT mode, 'USOS' will be displayed if 'Unshift-on-

space' is selected.

If  the Contest Mode is selected, the word 'TEST' will  then

be displayed followed by the Contest QSO number in '[]'.

In  certain  Robot  and  AutoCQ  states,  namely  while  the

computer  is waiting for a reply, a flashing  cumulative  CQ

count will be displayed next.

If  the  RTTY  SELCAL is active the word  'SELCAL'  will  be

displayed next.

If  the Navy MARS BAUDOT RTTY protocol is enabled,  a  '[n]'

will be displayed.

In the packet and AMTOR Robot modes, if at least one station

has  connected  or linked, a flashing yellow '[C]'  will  be

displayed  next. The number in the square brackets with  the

'C' is the number of stations that have connected/linked. If

the  target call station was one of those station,  a  happy

face will also be displayed in the square brackets.

If at least one QTC snatch has taken place a flashing yellow

'[Q]'  will  be  displayed next. The number  in  the  square

brackets  is  the  number of QTC Snatches  that  have  taken

place.

In the packet communications mode, if a ZapBBS, a QTC Snatch

or a PacketClusterZap is in progress a flashing yellow '[Z]'

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 23

will be displayed next. However, if LAN-LINK is set to  stay

connected  when  the sequence ends, the  flashing  indicator

will be a {Z} instead of a [Z].

In the packet or AMTOR communications mode, a '[R]' will  be

displayed  next  if LAN-LINK is configured  for  'store  and

forward' (:QSP:).

In the packet and AMTOR communications modes, if the  beacon

mode  is enabled a '[B]' will be displayed next. if  someone

has  connected and sent a :QRT: sequence, the '[B]' will  be

flashing. The '[B]' will be followed by a '[M]' if there are

messages pending. The number in the square brackets with the

'[M]' is the number of messages (*.OUT files) pending.

In  the  packet communications mode, if the  SAREX  mode  is

enabled,  a  '[S]'  will be shown next. The  '[S]'  will  be

flashing  while  capture to disk is in  progress.  If  SAREX

packets  have been copied a happy face will be displayed  in

the  square  brackets together with the 'S'.  If  the  SAREX

Orbiter robot is active a flashing '>>' will be displayed.

In  the packet communications mode, if the  digipeat  Detect

mode is enabled a '[D]' will be displayed next.

In  the packet communications mode, the  terminal  sub-modes

(see Section 21) are then displayed as follows;

SOLO      This lets you only see messages addressed to you.

CQ/B      This lets you see beacons and CQ calls as well.

TRFC      This  lets you see packets containing messages  as

well.

EVRY      This lets you see everything on channel, including

the link control packets.

MAIL      This  lets  you monitor packets  from  a  specific

station.  In  the PK-232, you can  do  it  without

displaying the headers.

The  next  item  to be displayed is a  callsign,  under  the

following  conditions;  the call of  the  station  connected

with, will be prefixed by a '*>' when connected, and by a '-

>' when a connect request is in progress. if a disconnect is

in progress, the callsign will be replaced by a '$$$$'.  The

callsign  will  be  seen as flashing when a  connect  is  in

progress.

In  the MAIL mode, the callsign of the station that you  are

"reading the mail" on will be displayed, otherwise the  call

sign displayed will be that of the one you are connected to.

If  an  Alert Call has been entered, it  will  be  displayed

next,  similarly  for  a target call. The  display  will  be

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 24

slightly  different of the calls have been  heard/worked.  A

!!>  before  the  flashing call  indicates  that  the  Alert

function is active. A '<->' before the Alert Call  signifies

that  packets  to  or  from that call  have  been  heard  on

frequency.  A smiling face in front of the  call,  signifies

that  it has been worked.  An '->' is displayed  before  the

Target Call.

The  next  items to be displayed are the packet  multi  user

mode  displays. If the conference bridge is up,  a  flashing

yellow bridge will be shown. If only the conference mode  is

set,  the word 'C will be flashing, if the basic multi  user

mode is set, the word 'M will be seen. The callsign will  be

prefixed  by the I/O channel number/letter, and will  be  in

the  color  allocated to the traffic on  that  channel.  The

number  shown  after the flashing letter is  the  number  of

streams that LAN-LINK thinks that you are connected on.

In the non packet modes, the callsign displayed will be that

of the station being worked (entered by the AMTOR robot, the

automatic CQ call recognition, or by the operator using  the

Alt-C  or  Alt-E keys). In AMTOR, the SELCAL  of  the  other

station  will  also be displayed sometimes, usually  if  you

entered the call.

An  '[L]' will be displayed in the non packet modes  when  a

callsign is entered. The '[L]' will flash to remind you  (to

log it) until the QSO is logged.

3.6 Logbooks

LAN-LINK  uses two dBASE 3 compatible logbook files. One  is

used for VHF packet logging (VHF.DBF), the other for all  HF

modes (HF.DBF). See Section 14 for more information.

3.7 How To Get an Update

There  are two ways to get an updated version,  as  follows.

When you register your copy of LAN-LINK, if this version  is

still  current, your registration will be acknowledged,  and

the  following upgrade will be sent to you automatically  at

the  time it is released. If a later version exists  at  the

time  that  you register, it will be sent to you  when  your

registration is acknowledged.

If  you are not on the list for an automatic  upgrade  (ie.,

you  received  a  disk  in the mail  as  a  result  of  your

registration), then when you hear that a new release is out,

collect  together  at least 300 kBytes of  traffic  on  your

local  vhf/uhf LAN by running LAN-LINK with the  capture-to-

disk  activated when you are not connected.  If you can  add

to  that,  connect to your local BBS and  dump  all  current

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 25

bulletins  with  full headers showing  date/time  into  your

capture-to-disk  file. If necessary compress the  file  (ZIP

preferred), and send in the disk in a mailer together with a

label  addressed to you, and sufficient return postage,  and

the  upgrade will be yours. Alternatively, you  may  perform

the  data transfers electronically to the BBS  phone  number

listed on page 1.

4.0 Function Keys

Function  keys  are  used to perform  operations  common  to  all

communications modes, or to allow fast performance of a  function

without having to go through several layers of menus. The  action

of  each  of  the function keys is  described  in  the  following

sections.  The function key number corresponds to  the  paragraph

number, namely 4.1 refers to Function Key 1 (F1), etc.

4.1 Capture to Disk Toggle (F1)

This  key  toggles the "capture-to-disk" command.  When  the

"capture-to-disk"  is active, all incoming  text  (including

'Echo as sent' outgoing text) is stored on a disk (floppy or

hard) in a file called 'YYMMDD.RUN' where YY is the last two

digits  of  the  year,  MM the month and  DD  the  day  (EG.

900824.RUN). If this file does not exist the first time that

LAN-LINK  is loaded, it is created. From that time  on,  for

the rest of that day, successive activations append new data

to the contents of the file.

Each time the command is activated a datestamp (or  timetag)

will be written to the file.

When  a  Packet station connects "capture-to-disk"  will  be

activated automatically unless you instruct LAN-LINK not  to

do so with the Capture-to-disk flag (Section 18.17).  Packet

connects   will   thus  be  recorded   in   their   entirety

automatically  unless  you turn  off  the  "Capture-to-disk"

during  the  course of the QSO.  "Capture-to-disk"  will  be

stopped at disconnect time.

You  may later edit the file at your convenience  with  your

favorite word processor or the included Editor. However,  if

your word processor can only handle files smaller than  64k,

make  sure that the 'capture-to-disk' file remains  smaller.

You can do this by renaming the file to something like 'LAN-

RUN.001'. LAN-LINK will automatically close the "capture-to-

disk" file when you terminate the program and return to DOS.

You  can make Capture-to-disk active when you  scroll  back.

You  can  use  this feature to save data  that  has  already

scrolled by the window.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 26

4.2 Send Brag Tape and Talk (F2).

This  command lets you transmit the Brag Tape (Default  LAN-

LINK.TXT.  In  the  non  packet modes,  it  will  leave  the

transmitter on after sending the file. In the packet mode it

will  send  a  ^Z  after sending  the  file  to  notify  the

recipient that the file is complete.

4.3 Transmit Callsign Sequence (F3)

This command lets you transmit the following sequence <other

call>  DE <your call>. For example if you are 4X6AA and  you

are  working  G3ZCZ  then this command  would  transmit  the

sequence 'G3ZCZ de 4X6AA'. You use this at the start and end

of a transmission in any mode except Packet.

4.4 Send Brag Tape (F4)

This  command lets you transmit the Brag Tape (Default  LAN-

LINK.TXT.  In  the  non  packet  modes,  it  will  turn  the

transmitter off after sending the file.

4.5 Show Packet 'MH' list and Point and Shoot Connect (F5).

This  command  lets  you see what calls the  TNC  has  heard

recently on the Packet channels. You can see up to 18,  with

the data and time they were last logged by the TNC. Refer to

the  'MH' command in the TNC manual for the  particulars  of

the display.

The  MH list shows up in a special window and freezes  until

you  depress any key. While the list is frozen, LAN-LINK  is

not monitoring received data, so don't freeze it too long.

If  the MH list is blank, or information comes in  from  the

TNC  between the time you entered the command and  the  time

the  list  is sent back, subsequent text from the  TNC  will

show up in the MH window until the next 'Cmd:' appears or 18

lines of text have been received.

You  can also use this command in a point and  shoot  method

for  attempting  a connect with a station  you  have  heard.

Bring  up the MH Window and move the cursor to  the  desired

callsign. When the cursor is positioned on any character  in

the  call,  press  the 'Enter' Key and  that  call  will  be

recognized  as  the callsign. If the call is joined  to  the

word "Cmd:" or the cursor is positioned on a space or blank,

the connect attempt will be inhibited.

This command does not work on the TNC1.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 27

4.6 Auto CQ Call (F6)

This  command will send a CQ sequence, wait a  preset  delay

and then repeat the CQ sequence until it receives your  call

back  from someone else. When it does so, it will alert  you

with  a  Connect  Alarm signal, reset the  delay  timer  and

revert  to  the normal or listening mode. In AMTOR  it  will

revert  to the mode in which it is waiting for an  ARQ  call

and is able to copy FEC signals on frequency.

When   the  Automatic  CQ  sequence  is  in  operation,   an

indication will be shown in the status window. The indicator

will be steady while the transmitter is sending the CQ text,

and will blink while the receiver is listening for a reply.

The time delay between calls is set using the 'Q' command in

the Parameters Menu (Set CQ delay time). It is also set as a

defined  default  by you when you  setup  your  LAN-LINK.SYS

file.

The  automatic  CQ  command will only work in  the  TNC  non

packet  modes if the 'Echo as Sent Mode' is set to 'ON'  per

the  LAN-LINK.SYS file. In other words, you must be able  to

see  the CQ text slowly echo in the incoming window for  the

command to work. The reason for this is that the "K K K" (or

"+?" in AMTOR) sequence is used to arm the software to  look

for a reply.

In  AMTOR  you  will get the connect  alarm  only  when  the

replying station transmits the '+?' character combination.

The check for the detection of your callsign and the connect

alarm  sequence only occur after a line feed  character  has

been received.

Under  most conditions in AMTOR, many in RTTY, and  some  in

MORSE,  LAN-LINK will recognize the callsign of the  station

answering  your CQ and display it in the status  window.  In

the  event  that it fails completely, the  sequence  '--?--'

will  show  up in the status window. Since  the  recognition

algorithm  used just looks for the first word following  the

'DE space' sequence and assumes that it has the callsign, be

careful to check it when operating under QRM conditions. You

can  override  the automatic call selection with  the  Enter

Callsign command in the Call Menu.

The automatic CQ sequence is canceled when an incoming  call

is detected (only if the Beacon or Robot/Mailbox Mode is not

selected), if you push the disconnect command key, or if you

change communications modes.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 28

4.7 Activate Type Ahead Buffer/AMTOR Relink (F7)

This key is not available in the Packet Communications Mode.

It  also  functions somewhat differently in  the  AMTOR  and

other  communications  modes as described in  the  following

sections.

4.7.1 Activate Type Ahead Buffer

The  use of this key sets LAN-LINK to the "Type  Ahead"

Mode.  This is necessary for the PK-232 only,  the  KAM

and  the  MFJ1278  have  the  feature  built  in.   Any

characters  then typed at the keyboard will  be  placed

into  the  TNC  transmit buffer. The  contents  of  the

buffer  will  be  transmitted over  the  air  when  the

transmit command is invoked with Function Key 10 [F10].

This  feature  allows you to 'type-ahead'  as  you  can

enter  text  into  the transmit  buffer  while  viewing

incoming  words in the INPUT window in the screen.  Any

characters  typed from the keyboard will be  echoed  in

the OUTGOING window on the screen.

4.7.2 AMTOR Relink

If  you  return  to the Command  Mode  when  linked  to

another  station in AMTOR, use this key to get back  to

the  Converse Mode. If the link times out, you can  try

to relink with this key if you have entered the  SELCAL

of the other station.

4.8 Disconnect, Disengage or Receive (F8)

This  function  key  returns  the  station  to  the  receive

communications  (TNC command) mode. It may also be  used  to

abort the automatic CQ sequence. If you use it to abort a CQ

sequence while the transmission is in progress, you may also

want to flush the PK-232 transmit buffer by using the  Flush

command in the TNC Menu or the Alt-F hot key combination.

In  a multiconnect situation the program will prompt you  to

tell it which I/O channel to disconnect.

In  AMTOR  this  command key toggles  LAN-LINK  between  the

Chirpcopy  Mode  (listening  to  a  QSO  in  progress)   and

monitoring FEC transmissions (CQ calls).

4.9 Put TNC in CMD Mode (F9)

This  command  puts the TNC into the Command Mode.  At  this

time, the program is transparent and acts as a dumb terminal

(except  for  placing  incoming and  outgoing  text  in  the

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 29

relevant  windows). You use this Mode for overriding any  of

the TNC default conditions. (SEE I TOLD YOU that YOU HAD  TO

READ THE TNC MANUAL).

4.10 Put TNC in CONVERSE /Transmit Mode /AMTOR FEC (F10)

This  command places the TNC into the transmit  or  Converse

Mode.  The transmitter is turned on and the contents of  the

transmit buffer are transmitted. In the non packet  communi

cations  modes, the outgoing characters will be echoed  back

from the TNC and displayed in the INCOMING window on the CRT

as they are transmitted on the air.

In the AMTOR modes this key puts the TNC in the FEC Mode and

turns the transmitter on.

4.11 Alternate Function Keys

These  keys  transmit the files with the  fixed  names  LAN-

LINK.001 through LAN-LINK.010 (the suffix number corresponds

to  the  function  key) which must be located  in  the  same

directory  as the LAN-LINK.EXE program. The  transmitter  is

left on when the file has been sent.

4.11.1 Send/Talk file LAN-LINK.001

4.11.2 Send/Talk file LAN-LINK.002

4.11.3 Send/Talk file LAN-LINK.003

4.11.4 Send/Talk file LAN-LINK.004

4.11.5 Send/Talk file LAN-LINK.005

4.11.6 Send/Talk file LAN-LINK.006

4.11.7 Send/Talk file LAN-LINK.007

4.11.8 Send/Talk file LAN-LINK.008

4.11.9 Send/Talk file LAN-LINK.009

4.11.10 Send/Talk file LAN-LINK.010

In  the non packet communications modes, the transmitter  is

left  on when the file has been sent. In AMTOR, the  change

over character pair (+?) is not sent.

4.12 Shift Function Keys

These function keys show the contents of the files with  the

fixed  names LAN-LINK.001 through LAN-LINK.010  (the  suffix

number  corresponds  to  the function  key)  which  must  be

located in the same directory as the LAN-LINK.EXE program.

4.12.1 Show contents of file LAN-LINK.001

4.12.2 Show contents of file LAN-LINK.002

4.12.3 Show contents of file LAN-LINK.003

4.12.4 Show contents of file LAN-LINK.004

4.12.5 Show contents of file LAN-LINK.005

4.12.6 Show contents of file LAN-LINK.006

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 30

4.12.7 Show contents of file LAN-LINK.007

4.12.8 Show contents of file LAN-LINK.008

4.12.9 Show contents of file LAN-LINK.009

4.12.10 Show contents of file LAN-LINK.010

4.13 Control Function Keys

These  function keys show the first lines of the files  with

the  fixed  names  LAN-LINK.001  through  LAN-LINK.010  (the

suffix number corresponds to the function key) which must be

located in the same directory as the LAN-LINK.EXE program.

5.0 Main menu

When  you  press the Escape Key, the Main Menu  appears  on  your

screen  and provides a number of choices which are  described  in

the following sections.

A Quick menu

B BBS Menu

C Call Menu

D eDit Menu

E Event Menu

F Files Menu

H Help for function keys

J Jump to DOS Shell

K LAN-LINK Menu

L Log Menu

M Message Menu

N TNC Menu

O AMSAT-OSCAR Menu

P Parameter Menu

Q Q Codes [NC/L]

S communicationS Mode Menu

T Terminal Mode Menu

X eXit LAN-LINK

Type in the single letter to make your choice.

NOTE:  Different options will show up in each menu  depending  on

the  connect  state (in packet) or the type of TNC that  you  are

using.  For  example, if you are using a TNC that  only  supports

packet,  the non packet menus will not be seen. An other  example

is  that  the BBS Menu is only active when you are  connected  to

another station.

5.1 The Quick Menu

The  Quick Menu is designed for new users and  provides  the

most  commonly  used functions in a single Menu.  The  Quick

Menu Provides the following options.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 31

A Auto CQ

B Send Brag Tape

C Call Someone

L Log Menu

M Change Communications Mode

Q Call CQ

S Send/Talk File

Z QRZ

1..0 Send/Talk LAN-LINK files 001..010

Type  in the single letter to make your choice. Each  option

is described below.

5.2 Auto CQ

This  option  initiates the automatic CQ sequence  in  which

LAN-lINK  sends out a CQ call, listens for a reply,  and  if

none  is received, sends out another CQ call, and so on,  ad

infinitum.  It is the same as using Function Key 6 (F6)  and

is described in Section 4.6.

5.3 Send Brag Tape

This  option  transmits the Brag Tape is the same  as  using

Function  Key  2 (F2) and is described in  Section  4.4  and

10.3.

5.4 Call Someone

This  option  is used when you want to call  or  connect  to

another station. It is described in Section 7.2.

5.5 Log Menu

This option invokes the logbook, described in Section 14.

5.6 Change Communications Mode

This option invokes the Communications Mode Menu,  described

in  Section  20.  You use it to  change  TNC  communications

modes.

5.7 Call CQ

This  option described in Section 7.6 is used to initiate  a

single CQ call.

5.8 Send/Talk File

This  option is used to send a file. When the file has  been

sent, the transmitter will stay on. This option performs the

same function as does Function Key 4 (F4).

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 32

5.9 QRZ

This  option is the same as using Alternate Q  Function  Key

(Alt-Q) and is described in Section TBS.

5.10 Send/Talk LAN-LINK files 001..010

This option is used to transmit the contents one of the LAN-

LINK buffer files. You select the number, LAN-LINK does  the

rest.  The  option  is  the same as  that  provided  by  the

Alternate Function Keys and is described in Section 4.11.

6.0 The BBS Menu

This menu gives you smart file transfer capabilities with a  BBS.

The menu is only active when you are connected to another  packet

station. The BBS Menu provides the following capability.

D Download Text Files.

Type in the single letter to make your choice.

6.1 Download Text Files

This option lets you download a text file from the BBS  into

a  separate  text  file on your disk.  Enter  the  full  BBS

download command at the prompt line.

To download a file from a W0RLI BBS such as FILENAME.TYP  in

the  A  subdirectory  you'd  enter  'DA  FILENAME.TYP'.   To

download  the  same file from the PACKET subdirectory  of  a

WA7MBL BBS you'd enter the command 'D \PACKET\FILENAME.TYP'.

If you use the 'D' command to download a text file, it  will

end up in a separate file on your disk ONLY IF a file of the

same  name does not exist on your drive. If a file with  the

selected name already exists on your disk drive you will get

an error message.

7.0 The Call Menu

The call menu is used to enter calls, either to call/connect/link

a  station  or  to remember a call or path.  Each  time  you  are

prompted for a call, LAN-LINK will remember the previous 15 calls

and  you may use the Up and Down arrows to move the cursor  to  a

remembered call. If you wish to capture a call from the  incoming

text  window, push Function Key 10 (F10) to enter the window  and

move  the  cursor  to the desired callsign. When  the  cursor  is

positioned on any character in the text, push the 'Enter' Key and

that  word  will be recognized as the  callsign.  Any  characters

joined  to the call (such as a '.') will be picked up as part  of

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 33

the  call. If the cursor is positioned on a space or  blank,  the

connect attempt will be inhibited.

In  AMTOR, whenever you are asked to enter a  callsign,  LAN-LINK

will  prompt you with its guess at the SELCAL. If you agree  with

it,  just  press  the 'Enter' Key, if you  disagree,  enter  your

choice over the prompt version.

The Call Menu provides the following choices.

A Alert Call (packet) or ARQ Call Talk (AMTOR)

C Connect/Call

D Show Directory

E Enter Call

M reMember Path

P Packet Cluster

Q Call CQ

R Retry

T Target Call

U aUto CQ

X Set DX Flag On/Off

Z Zap BBS

! Zap BBS on next Mail Beacon

Type in the single letter to make your choice.

7.1 Alert Call/ARQ Call

7.1.1 Alert Call

The   Alert  call  feature  is  used  in   the   Packet

Communications Mode when disconnected and the  terminal

set  for 'TRFC' or 'CQ/BEACON'. LAN-LINK will scan  the

packet headers received from the TNC, and, when it sees

a   packet  originated  (or  digipeated  if  the   MRPT

parameter  in the TNC is set to 'ON'), by  the  station

whose  call  you have entered as the 'Alert'  call,  it

will sound an alarm at the console. The line containing

the  packet  header  will also be  displayed  in  light

green.

This option toggles the Alert call feature on and  off.

If  it  is  off, you are prompted for the  call  to  be

monitored,  and,  if it is on, it is  turned  off.  The

toggle  is set when you enter a callsign,  and  cleared

when you depress the 'Enter' Key instead of a callsign.

The  Alert  call will be shown flashing in  the  status

window when in the disconnected state.

NOTE: If the terminal is set for 'CQ/BEACON' the  Alert

will  only sound if the specified station transmits  CQ

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 34

or BEACON packets.

7.1.2 ARQ Call

In AMTOR, this option will attempt to send an ARQ  call

to the desired station, transmit the callsign  sequence

when linked and allow you to continue typing text (Note

that  the  C option sends the '+?' after  the  callsign

sequence).

7.2 Connect/Call

This  option  works in almost the same manner  in  both  the

packet  and non packet communications modes as described  in

the following sections.

7.2.1 Non packet Modes

You  use  this option to call a station  heard  on  the

band. In AMTOR the link request will be sent in the ARQ

Mode to the SELCAL of the desired station.

7.2.2 Packet Mode

You  use this option to attempt a connect with  another

packet  station. In a multiconnect situation  the  LAN-

LINK  will automatically select a free I/O  channel  to

try the connect.

7.2.2.1 Names, Handles and Paths

If  you  so  desire, you can create  a  directory  file

(default name is LAN-LINK.DIR) and keep a list of names

and  calls. The computer will look up a name  and  call

the station by its callsign. For example if you put

Joe G3ZCZ

as  a line in the file, when you tell the  computer  to

connect to Joe, it will try to connect to G3ZCZ. If you

enter

4X4HF 4X4HF V 4X1AA, 4Z4ZB, 4X4IL

you  only  need to type '4X4HF' to set up  the  correct

connect path.

You  can  also use this feature to automate  a  NET/ROM

and/or a KA Node path connection.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 35

7.2.2.2 NET/ROM and TheLink

In  the manual NET/ROM path setting up mode, you  would

first  connect  to your local node, then when  you  are

connected with it, connect to the next node, and so  on

through  any intermediate nodes within the  whole  path

before  you  finally  connect  with  your   destination

station.  You may also have digipeaters in any  of  the

paths.

For  example if you wish to connect to K1HTV who is  in

range  of the SSCT node, and you are in Washington  DC,

you  might first have to connect to your local  NET/ROM

node   (ELK)  via  a  digipeater,  then  each  of   the

intermediate nodes, SNJ3, NNJ, WMA before you could try

to connect with K1HTV.

Consider  the manual sequence to make  the  connection.

K1HTV  is  the station the contact is  being  attempted

with,  and  he  is located within  range  of  the  SCCT

NET/ROM  node.  The station attempting the  connect  is

located  in  the Washington DC area one  digipeat  away

from the ELK NET/ROM node.

The  connection to the local node is  performed  first.

The  operator  has to type 'C ELK v WB4APR-5'  to  make

that link. When the connect with the node is  achieved,

the operator must type 'C SNJ3' to connect to the  next

node.  Each time the connection is made,  the  operator

must  type in the command to make the next  one,  until

finally the last node is reached and a connect  request

can  be made to the destination station (either  direct

or  via  a  digipeater).  Sometimes  there  may  be  an

intermediate   non-NET/ROM   connection   between   two

stations somewhere in the path.

This  procedure is tedious and time consuming,  because

it  may take several seconds to complete each stage  of

the link. LAN-LINK automates this sequence by  scanning

the  line in the LAN-LINK.DIR file associated with  the

callsign  entered and transmitting each section of  the

line  when it recognizes the 'Connected to' reply  from

each NET/ROM node in the path. When it finally gets  to

the destination it sounds a chime to alert you that the

connect has been achieved.

The  LAN-LINK.DIR  file  entry  for  this  path  is  as

follows:

K1HTV ELK v WB4APR-5!SNJ3!NNJ!WMA!SCCT!K1HTV

where the '!' separates each stage of the link.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 36

The key word is K1HTV. LAN-LINK then scans the line  to

see if there is a '!' in it. If there is it knows  that

it  is dealing with a NET/ROM path. It then  tells  the

TNC to attempt a connect with the characters  following

the key word up to the first '!'. In this case it tells

the  TNC to connect to ELK via WB4APR-5. When  the  TNC

sends  back the connected message, LAN-LINK  tells  the

TNC  to  try to connect to the next node in  the  path,

namely  in this example, SNJ3. The  sequence  continues

each  time  a  connection is made  until  the  link  is

complete.

To  set up a directory entry just use the format  shown

above.  Place  a  '!' sign after the  uplink  path  and

different  node  segments. Do not place one  after  the

downlink path.

Depending  on the configuration, LAN-LINK will issue  a

'disconnect'  command  to  the TNC  in  the  event  the

NET/ROM or KA-Node connect attempt fails. This  feature

is  designed  for  known paths  not  for  general  path

determination.

7.2.2.3 KA-Nodes

You can connect or cross connect via KA-Nodes using the

directory file. You must still use the ! separators  as

well  as two new symbols % and &. Use % for 'C'  and  &

for 'X' in KA-Node links..

For example, given that a path to W9TNN-2 is first to a

local  KA node [JRW] then to a second node on the  same

VHF frequency [W3EAX-7] and lastly via a cross  connect

on  40 Meters to W9TNN-2, the line in the  LAN-LINK.DIR

file would be as follows.

w9tnn-2 jrw!%w3eax-7!&w9tnn-2

You can mix NET/ROM and KA-Node paths as in:

n7dva!g3zcz-9!&WA6IEL-4!n7dva v kl7vz-3

The different symbols are used for the KA node to  tell

LAN-LINK  which replies to look for as well as what  to

prefix the call with.

Depending  on  the Node Drop Link  Flag  configuration,

LAN-LINK  will issue a 'disconnect' command to the  TNC

in  the  event the NET/ROM or KA-Node  connect  attempt

fails. This feature is designed for known paths not for

general path determination.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 37

7.2.2.4 Loop Backs

In the Packet Communications Mode, if you want to  loop

back through someone else you would have to type

C <yourcall> V <hiscall>.

For  example if I (G3ZCZ) wanted to loop  back  through

4X6AA, I would have to type the command

C G3ZCZ V 4X6AA

to the TNC.

In  LAN-LINK you just have to enter the  '/'  character

followed  by the call of the station you want  to  loop

back  through. For example if I (G3ZCZ) wanted to  loop

back through 4X6AA, I just need to enter

/4X6AA

as  the call to connect to at the correct  prompt,  and

LAN-LINK  will tell the TNC to try to connect to  G3ZCZ

via  4X6AA, namely issue the command 'C G3ZCZ V  4X6AA'

to the TNC.

7.3 Enter Call

This  option lets you enter the call of any station  calling

you into the program. If you type the Enter Key in  response

to  the  prompt,  you can later use  Function  Key  (F3)  to

transmit the sequence "DE <yourcall>". You may also use  the

Retry feature to call that station.

In  AMTOR,  you use the feature to enter  the  callsign  and

SELCAL  of the station you are listening to and may want  to

call later.

7.4 ReMember Path

You may use this option to put an entry in the  LAN-LINK.DIR

file.

7.5 Zap Packet Cluster

This  option  firsts prompts you with the  callsign  of  the

PacketCluster. If you enter a callsign, the entered callsign

will become the new PacketCluster call. LAN-LINK then issues

a connect request to the PacketCluster. When the  connection

is achieved, and the PacketCluster sends you a '>' character

at the end of a line of text, LAN-LINK will issue a

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 38

'SHOW/WWV' command followed by a 'SHOW/DX'

command to the PacketCluster and return to manual operation.

Note:  PacketCluster  is  a  specialized  BBS/conference  by

Pavilion Software for DX chasers.

7.6 Call CQ

You  use  this  command  key  to  call  CQ.  In  the  Packet

Communications  Mode, the TNC will transmit a line  of  text

along with the CQ packet. The line of text is set up in  the

LAN-LINK.SYS file described in Section 22.

In  the  AMTOR  Communications Mode,  your  SELCAL  will  be

transmitted along with the CQ call in the FEC Mode.

7.7 Retry

This  option  (also  activated by the  Alt-R  function  key)

allows  you to retry a connect that for some reason did  not

go  through the first time. It saves wear and tear  on  your

fingers  particularly if the desired connect path is  via  a

number  of  digipeat relays or through a number  of  NET/ROM

NODES.

In the non packet modes, it allows you to retransmit a  call

to another station without having to reenter the call.

If  you  have  previously used the Enter feature  to  put  a

callsign in the program when you hear that station, you  can

then use the Retry command to call him later when the QSO in

progress terminates.

7.8 Show Call Directory

This option displays the contents of the call directory file

(LAN-LINK.DIR).

7.9 Target Call

The Target call feature is used in the Packet Communications

Mode  when disconnected. LAN-LINK will monitor the  stations

connecting  to  you. When it sees a connect by  the  station

which you have entered as the 'Target' call, it will set the

flashing Connect Count display to show a 'happy face'.

This  option toggles the Target call feature on and off.  If

it is off, you are prompted for the call to be monitored, if

it is on, it is turned off. The toggle is set when you enter

a  callsign,  and  cleared when you press  the  'Enter'  Key

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 39

instead of a callsign.

The Target call will be shown flashing in the status  window

when in the disconnected state.

7.10 Auto CQ

This option is the same as Function Key 6 (F6).

7.11 Toggle DX Flag

LAN-LINK allows you to call either 'CQ' or 'CQ DX'. If  this

flag  is  set,  all CQ calls will be to "CQ DX".  If  it  is

cleared, all CQ calls will be plain CQ's.

7.12 Zap BBS

The BBS Mail Snatch can be initiated in two ways. It can  be

initiated when your incoming screen displays a "Mail" beacon

(received  from your local BBS) with your call in the  first

line  following  the  header,  or  by  LAN-LINK's  Zap   BBS

function. The automatic BBS Zap will not take place if  LAN-

LINK is in the multi-user mode.

In the event of a BBS Mail-Snatch, if the *.BBS file  exists

for  that BBS, the file will be processed and  the  contents

sent to the BBS. Thus if you don't get a chance to send  the

mail  before  LAN-LINK downloads the mail from the  BBS,  it

will send it for you.

When the BBS has accepted the commands the name of the  file

is  changed  from '*.BBS' to '*.n', where 'n'  is  a  number

increasing sequentially each time the function is performed.

For  example,  the  file name KE8X.BBS will  be  changed  to

KE8X.001  the first time it happens. In this way,  you  will

have  a  copy of the outgoing file on your  disk  until  you

choose to delete it.

In the real world however, message formats are  nonstandard,

and  can  contain  any  set  of  characters.  The  algorithm

developed  here  transmits the next line in the  *.BBS  file

when  LAN-LINK sees a 'trigger' character-string unless  one

of the 'inhibitor' character-strings is also present on  the

line of the message text received by LAN-LINK.

These  'trigger'  character-strings  are '>'  (as  the  last

character  of the line of text ), 'Enter', 'Subject',  'Sj:'

and 'Msg:'. The 'inhibitor' character-strings are 'R:',  '<'

and 'Message'(R: as in routing headers and < and Message  as

in Message-Id: <message number> in message headers).

This should cover most simulated manual BBS message  reading

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 40

(WA7MBL and W0RLI/VE3GYQ) and avoid false triggering due  to

'>'s showing up in message and routing headers.

NOTE that when LAN-LINK finishes processing the *.BBS  file,

it  will  send  a 'B' command to the BBS  to  terminate  the

session.  This feature is designed to minimize the time  you

spend  connected  to  the BBS by allowing you  to  scan  the

message list when automatically receiving your mail, prepare

your  replies and download requests off-line and then  later

read  the  incoming messages or files without tying  up  the

BBS. It thus positively disconnects you from the BBS.

In the Packet Communications Mode, the Alt-Z key initiates a

BBS  Mail Snatch. You use this key if you connect to  a  BBS

before you hear its beacon, or if you have disabled the  BBS

Mail Snatch feature. If you are connected to a BBS, the  key

must be used BEFORE the first '>' is received. The first '>'

triggers  the  sequence. If you have already  received  that

first '>', send the BBS any command, and the sequence should

begin when the BBS signals that it has completed  processing

that command. If you are not connected, LAN-LINK will  first

try to connect to the BBS, then perform the sequence.

When  you use this command you will be asked if you want  to

upload  or download. If you want to download, just push  the

'Enter'  Key.  A Download (Mail from the BBS  to  you)  will

automatically  be  followed by an Upload if the  *.BBS  file

exists.

The download sequence performed by the BBS Mail Snatch is as

follows:

RM  To read your messages, (Configurable)

L   To let you know what is new on the BBS,

LM  To list ALL your messages on the BBS,

B   To log you off the BBS.

LAN-LINK will wait for the '>' as the last character of  the

line of text being received from the BBS, before issuing the

next  command in the sequence. Your messages will be  placed

in a message file <yourcall>.OUT as if someone else had left

a message for you.

The 'RM' may be customized in the LAN-LINK.SYS file to  'RN'

or 'VM' depending on the software in your local BBS and your

preference.

During  the  time  that the BBS is  responding  to  the  'L'

command  with a list of new messages, LAN-LINK  is  scanning

the  lines  containing the messages to see if it  finds  one

containing  a Scan Word. If it does it will queue a  request

to download that message.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 41

If  you wish to add a 'KM' to the sequence, place it in  the

BBS.BBS file. If you use 'KM' as you may delete mail  BEFORE

reading it.

If you use the 'D' command to download a text file, the file

will  end up in a separate file on your disk ONLY IF a  file

of the same name does not exist on your drive.

Do delete 'read' mail from your local BBS. Don't clutter the

BBS's  disk with your read mail. The 'LM' command  is  built

into LAN-LINK to remind you of what you have out there.

The BBS 'S' command (including its variations such as  'SP')

is the only multiple line command allowed. You can thus  put

all  the single line commands such as D, H, ?, L, and  R  in

the BBS.BBS file.

A  'multiple  command'  is defined in  this  instance  as  a

command which allows you to enter multiple lines before  the

BBS  will send back a '>' character (i.e. one that  the  BBS

tells you to type in text and terminate it with the  Control

Z character).

Each  message must be in the format expected by the BBS  and

must  end with the " :EOF: " character string, but there  is

no need for any special end of BBS file character.

A typical message could be as shown below.

SP G3ZCZ @ N4QQ

LAN-LINK

I think it is great. However, why don't you change  the

following...... (add your comments here).

:EOF: {last line may also be '/EX'}

7.13 Zap BBS on Next Mail Beacon

This option causes LAN-LINK to Zap your local BBS next  time

it  beacons with a list of calls for which it has Mail.  You

set  this  option active when you want to upload  a  set  of

messages, or are not sure if there are any messages  waiting

for you on the BBS. A typical situation is where there are a

lot  of NTS messages stacked up in the BBS.BBS file  waiting

to  go. This parameter will be reset when the  connect  goes

through, so if the Zap attempt failed, it will try again  on

the following Mail beacon. This will not work in the  Multi-

user mode.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 42

8.0 Edit Menu

This menu provides the following choices.

B Brag Tape

D Call Directory File

F Any other File

K Answer Ctdsk File

L Leave messages on your local BBS

M Answer Incoming Mail

N Note

O Pick *.OUT (Message) File to Edit

P Pick Ctdsk File to Edit

R Capture to Disk File

S LAN-LINK.SYS

T Two Files

1,2 - 0 LAN-LINK.001 - 010

Type in the single letter to make your choice.

LAN-LINK  contains an ASCII text editor suitable for  files  less

than  64k  in size. It is built based on Borland's  Turbo  Pascal

Editor Toolbox and the commands are compatible with Sidekick  and

Wordstar. A summary of the editor commands is shown below.

F10 Switch Windows (if more than one window open)

Backspace Delete left char

Del Delete Char

Enter New line

Ins Toggle insert mode

PgUp Move cursor up one page

PgDn Move cursor down one page

^A Move cursor left one word

^C Move cursor down one page

^D Move cursor right one character

^E Move cursor up one line

^F Move cursor right one word

^G Delete one character

^H Delete left one character

^I Tab

^L Find/replace next occurrence

^P Insert a control character into the text

^M New line

^N Insert line

^R Move cursor up one page

^S Move cursor left one character

^T Delete one word after cursor

^V Toggle insert mode

^W Scroll up

^X Move cursor down one line

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 43

^Y Delete line at cursor

^Z Scroll down

^K^B Mark beginning of block

^K^C Copy block to position of cursor

^K^D Save file and exit edit

^K^H Hide block marker

^K^K Mark end of block

^K^Q Abandon file and exit edit

^K^R Read file into screen

^K^T Mark single word as block

^K^V Move block to position of cursor

^K^W Write block to disk file

^K^Y Delete block

^K 1..9 Set marker 1 .. 9

^Q^A Find text and replace

^Q^B Move to beginning of block

^Q^C Move to end of file

^Q^D Move to right of line

^Q^E Move to top of window

^Q^F Find text

^Q^I Toggle autoindent mode

^Q^K Move to end of block

^Q^R Move to top of file

^Q^S Move to left of line

^Q^X Move to bottom of window

^Q^Y Delete to end of line

^Q 1..9 Jump to marker 1..9

The  following options can be set in the Find/Find  and  Re

place (^Qf and ^Qa) operations.

# locates #th occurrence

G global replace

N replace without Y/N question

U ignore upper case/lower case

W match whole words only

Notes

1 The ^ key in front of a character identifies that character  as

a  'control' character. To activate it, hold down  the  Control

key AND the character key.

2 Some of the commands require two keystrokes.

3 Use  ^P to embed a control character in the text. For  example,

if  you are creating a file containing commands to be  sent  to

the TNC, to enter a control-C character into the file, use  the

^P^C sequence of keystrokes.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 44

WHILE  USING  THE  EDITOR, LAN-LINK  can't  receive  and  process

characters  from  the TNC. All other features  ARE  INHIBITED  OR

LOCKED  OUT. THUS IF SOMEONE CONNECTS TO YOU, THE TNC  WILL  KNOW

BUT LAN-LINK WILL NOT. BE WARNED !!!

The Edit menu allows you to call up the various files for editing

as follows.

8.1 Edit Brag Tape

This option allows you to create and edit the Brag Tape File

(LAN-LINK.TXT).

8.2 Edit Call Directory file

This option allows you to create and edit the Call Directory

File (LAN-LINK.DIR).

8.3 Edit any other file

This option allows you to create and edit any other file.

8.4 Answer Ctdsk File (Split Screen)

This option allows you to edit the capture-to-disk file  and

reply to incoming bulletins/messages in a split screen mode.

The contents of the incoming bulletins and messages will  be

displayed in the top window, and the reply file in the lower

window.  The reply file is assumed to be named according  to

the Zap convention (*.BBS file). You will have to set up the

replies/outgoing messages in the form shown below.

SP Call @ BBS

Subject of the message

content lines

End the message with either

':EOF:' , '/ex' or '/EX'

as  the first word on a separate line (without the  ').  For

example,

SP G3ZCZ@N4QQ

Test

This is a test

73

/ex

Use  Function key 10 (F10) to switch windows. At  this  time

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 45

you cannot transfer text between windows.

8.5 Answer Incoming Messages (Split Screen)

This  option  allows you to answer incoming  messages  in  a

split  screen  mode. The contents of the  incoming  messages

will  be displayed in the top window, and the reply file  in

the  lower  window. The reply file is assumed  to  be  named

according to the Zap convention (LOCAL-BBS.BBS file).  Reply

messages are set up as described in Section 8.4.

Use  Function key 10 (F10) to switch windows. At  this  time

you cannot transfer text between windows.

8.6 Leave Note for Someone

In  all  modes  you can leave a  brief  note  (message)  for

another  station. When you invoke this option, you  will  be

prompted for the call of the station that you want to  leave

a  note for. The date/time header is written to the  message

file, then as you enter the text it is stored on the disk in

the usual format.

After you have left the note and received the 'QSL'  message

from LAN-LINK, if you are in the Packet Communications Mode,

press  Function Key 8 (F8) or Alt-D to load the callsign  of

the recipient into the beacon.

When  you  leave a note, leave it for the root call  of  the

connectee, for example to leave a note for G3ZCZ/W3 leave it

for  G3ZCZ, similarly to leave a note for WB8WKA-1 leave  it

for  WB8WKA.  LAN-LINK is smart enough to extract  the  root

call from any callsign it links/connects with.

8.7 Edit Capture-to-Disk file

This  option allows you to edit the Capture-to-Disk file  as

long as it is smaller than 64k.

8.8 Edit Two Files (Split Screen)

This option allows you to create and edit any two files. Use

Function  Key 10 (F10) to switch windows. At this  time  you

cannot transfer text between windows.

8.9 Edit Configuration file

This option allows you to edit the Configuration File  (LAN-

LINK.SYS).

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 46

8.10 Edit LAN-LINK.001 through LAN-LINK.010 files

This option allows you to create and edit the 10 files (LAN-

LINK.001  to LAN-LINK.010). Use the suffix number to  access

the  respective LAN-LINK.* file, use the number 0 to  access

the LAN-LINK.010 file .

8.11 Pick Capture-to-disk to Edit

This option presents you with a directory window  containing

a  list  of  capture-to-disk files  (YYMMDD.RUN).  Move  the

cursor  down to the desired file and touch the 'Enter'  key.

If you don't want to edit any of them, then move the  cursor

down to a blank line. If you have more files than one screen

full,  use the Pgdn (Page Down) key to show the next  screen

full.

8.12 Pick Message file to Edit

This option presents you with a directory window  containing

a list of message files (*.OUT). Move the cursor down to the

desired message file and touch the 'Enter' key. If you don't

want  to  edit any of them, then move the cursor down  to  a

blank line. If you have more files than one screen full, use

the Pgdn (Page Down) key to show the next screen full.

8.13 Leave Messages on your local BBS

This option allows you to edit your LOCAL-BBS.BBS file in  a

single window.

9.0 Event/Clock Menu

LAN-LINK contains some time dependent features which allow you to

schedule events. These features are controlled from this menu and

are listed below.

A Set Alarm

C Connect/Call

D Display Event

P Periodic Event

R Turn Robot ON/OFF

S Turn ConteSt ON/OFF

T Turn Time Display ON/OFF

X Cancel Event

Z Zap BBS

Type in the single letter to make your choice.

To set a scheduled event, first select the event, then enter  the

time. You may set the event to occur once a day, or once an hour.

If you wish the event to occur once an hour, enter an hour  value

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 47

greater than 23 when asked to enter the hour. Each time the event

occurs, the feature will be cleared.

9.1 Set Alarm

This option allows you to set an alarm clock.

9.2 Connect

This  option allows you to set LAN-LINK to issue a  call  or

connect request at some later time.

9.3 Display Event

This option allows you to display the scheduled event.

9.4 Periodic Event

This option allows you to set a periodic connect request  or

call event. You can set the start time, the end time and the

time  interval between the connect/call attempts. The  first

connect that goes through will clear the event scheduler.

9.5 Turn Robot ON/OFF

This option allows you to turn the AMTOR/Packet Robot on  or

off at a fixed time.

9.6 Turn Contest ON/OFF

This  option allows you to turn the  AMTOR/Packet  Automatic

Contester  on or off at a fixed time. You will  be  prompted

for  the  time,  a  starting  QSO  number  (in  case  you  a

restarting  the contest) and the Contest Text (in  case  you

wish to change it).

9.7 Turn Time Display ON/OFF

This option allows you to turn the clock display on or off.

9.8 Cancel Event

This option allows you to cancel a scheduled event.

9.9 Zap BBS

This  option  allows  you  to  schedule  a  BBS  Zap  at   a

predetermined  time (like in the middle of the  night).  You

may  for example set it to attempt to connect to a BBS at  a

fixed  time or hourly at any desired minute after the  hour.

Should  the  connect be made, the event  scheduler  will  be

cleared and no further connect attempts will be made.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 48

10.0 Files Menu

The Files Menu provides the following choices.

B Send Brag Tape

D File Directory

E Erase File

P Pick Ctdsk File to View

R Rename File

S Send File

T Send/Talk File

U Send Uppercase File

V View file

Y View Ctdsk File

Z View Messages

Type in the single letter to make your choice.

During  the file transfer, the outgoing text will appear  in  the

OUTwindow.  If the text is long enough and fills the TNC  buffer,

you will see it stop, and then continue when the TNC is ready  to

accept more text, the message sequence will resume. NOTE hardware

flow control NOT ^S/^Q software is used.

When you transmit a file (B or S) you are returned to the receive

mode  after the file is sent except in the Packet  Communications

Mode.  In the Packet Communications Mode, when the file has  been

loaded into the TNC the computer will add the ^Z character to the

outgoing text to signal to the recipient that the file  transmis-

sion is over.

The computer will signal you (using the bell character) when  the

file  has been loaded into the TNC, Note that is not the same  as

the  time that the TNC has transmitted the file, because the  TNC

incorporates a character buffer.

10.1 Send Brag Tape

This  command  allows you to transmit a standard  text  file

historically known as a "Brag Tape". The default name of the

brag  tape  file is LAN-LINK.TXT, and it should  be  created

with a word processor in the non-document mode.

10.2 File Directory

This command shows you the directory of the disk drive.

10.3 Erase File

This command allows you to erase or delete a file.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 49

10.4 Pick Ctdsk file to View

This  command  allows  you to view a *.RUN  file.  When  you

exercise  this command you will be presented with a list  of

capture-to-disk files (YYMMDD.RUN). Move the cursor down  to

the one you want to view and touch the 'Enter' key to select

it.   Use this option, if you must, on long files. On  files

with less than 64k characters, use the Edit Menu.

10.5 Rename File

This  command  allows you to rename a file.  You  could  for

example rename a document file to a "<callsign>.OUT" file so

that  someone will receive it automatically next  time  they

connect.

10.6 Send File

This  option  lets you transmit an ASCII file.  In  the  non

packet modes, it will turn the transmitter off after sending

the  file.  In the packet mode it will send a  ^Z  character

when the file is complete.

10.7 Send/Talk File

This  option  lets you transmit an ASCII file.  In  the  non

packet modes, it will leave the transmitter on after sending

the file.

10.8 Send Uppercase File

This  option  lets you transmit an ASCII file  and  converts

each   line  of  text  in  the  file  to  UPPERCASE   before

transmitting it. The file itself is not changed. In the  non

packet modes, it will turn the transmitter off after sending

the file.

10.9 View file

This option lets you view the contents of a file. Use it  if

you  must  on  long  files. On  files  with  less  than  64k

characters, use the Edit Menu.

10.10 View Ctdsk File

This  option lets you view the contents of  the  capture-to-

disk file (YYMMDD.RUN) which you specify by name. Use it  if

you  must,  on  long  files. On files  with  less  than  64k

characters, use the Edit Menu.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 50

10.11 View Incoming Message File

This option lets you view the contents the Incoming  message

file  (yourcall.OUT). Use it if you must, on long files.  On

files with less than 64k characters, use the Edit Menu.

11.0 Help with function keys

This  option  will activate the HELP display which  presents  you

with  a  (very) brief reminder summary of the  function  and  hot

keys.  This  window will remain on display until a  character  is

typed at the keyboard.

12.0 Jump to DOS Shell

This  option  allows  you to enter a DOS shell.  If  you  do  not

restore  the  system  to the directory  which  contains  LAN-LINK

before you EXIT from the shell, LAN-LINK will do so for you.

You  may use the DOS shell while in QSO when you have loaded  the

computer/TNC output buffers and output will continue to flow.

To  exit from the DOS shell back to LAN-LINK, enter  the  command

'EXIT'.  LAN-LINK  will automatically return you to  the  correct

subdirectory.

13.0 LAN-LINK Menu

The LAN-LINK menu provides different options depending on whether

you  are  connected  to another station  or  not.  The  following

options are available when connected.

B :QBU: External Protocols

D :QZD: Download Binary File Using Zmodem

M :QBM: ASCII File

P :QSP: Message

Q :QDB: ASCII File

S :QSM: Message

T :QTA: Messages

U :QRU: Messages

Z :QZU: Upload Binary File Using Zmodem

The following option is available when not connected.

W Who is "ON-LINE"

Type in the single letter to make your choice.

Each  of  the features (except the :QBU:) may also  be  performed

manually by typing the full sequence of Network Control  Language

(see Appendix 3) character strings.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 51

13.1 The :QBU: Command

This is a Sequence for external operations. It requires that

the  batch file 'QBU-TX.BAT' be present in the  transmitting

(initiating)  station,  and the batch file  'QBU-RX.BAT'  be

present  in the receiving station. When the QBU sequence  is

executed the following occurs:-

The transmitting station sends a :QBU:

The receiving station sends a :QRV: and an amount of  memory

left on disk, enters the transparent mode, jumps to dos  and

executes the batch file 'QBU-RX.BAT'.

Upon  receipt  of the :QRV:, the transmitting  station  then

puts  the TNC in the transparent mode and jumps to  Dos  and

executes the batch file 'QBU-TX.BAT'. The batch file  should

exit and return to LAN-LINK. LAN-LINK then exits the  trans

parent mode.

When  the receiving station batch file finishes and  returns

to  LAN-LINK. LAN-LINK then exits the transparent  mode  and

transmits  a :QSL: if the DOS Error level was 0 and a  :QNO:

error if it wasn't.

At this time both stations are back in the converse mode.

You  may  use this technique for  transmitting  pure  binary

files  using  external  (to LAN-LINK)  protocols).  You  may

change  the  default names of the batch files  in  the  LAN-

LINK.SYS configuration file.

13.2 :QBM: ASCII File

You  use this option to download an ASCII file from  another

LAN-LINK station. It is the same as manually typing

':QBM: FILENAME'.

13.3 :QSM: Message

You use this option to request a repeat of a message. It  is

the same as manually typing ':QSM:'.

13.4 :QSP: Message

You  use this option to store a message on another  LAN-LINK

station. It is the same as manually typing

':QSP: <callsign>'.

LAN-LINK contains store and forward routines for messages in

a Local Area Network (LAN) as described in Appendix 2.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 52

When connected to another LAN-LINK station, you may use  the

NC/L :QSP: command to leave a message at that station.  This

QSP command automates that feature for you. It lets you just

enter  the  callsign  of the station you  wish  to  leave  a

message  for.  It  will then send the  text  sequence  :QSP:

<callsign>.  The  other LAN-LINK HOST  station  will  either

reply  :QNO: followed by a number or :QRV:  <callsign>.  The

'QNO'  message means that the other computer  cannot  accept

the message, the :QRV: message means that it is waiting  for

you to send it.

If  you get the :QRV: reply, go ahead and type the  message,

or  send the text file containing the message. If  you  type

the  message,  use a control Z (^Z) character  and  carriage

return  to terminate the message. (In AMTOR,  LAN-LINK  will

translate the ^Z to :EOF: +? to terminate the message).  You

will  then  receive either a :QSL: from  the  other  station

meaning that the message is accepted, or a :QNO: followed by

a message that something went wrong and the message was  not

stored.

13.5 :QDB: ASCII File

You use this option to upload an ASCII file to another  LAN-

LINK station. It is the same as manually typing

':QDB: FILENAME'.

Using this option saves you typing the file name twice.

13.6 :QRU: Messages

The  NC/L  QRU  command allows you to  upload  a  number  of

messages  to  the other station. You use it  when  you  have

messages  pending on your system and want to pass them to  a

third  computer for storage in the LAN, while you take  your

computer off-line.

You use this option to begin the upload sequence to transfer

messages  for  a  number of stations to  another  LAN-  LINK

station. It is the same as manually typing :QRU:.

You  can only use this option with a station which you  have

previously  designated as a store and forward mail box,  for

specific  stations, namely one for which you have set  up  a

<callsign>.MBX  file. This command may be triggered  by  you

using this choice or by the connecting station sending you a

:QRU: command.

When the sequence is over, the computer sending the messages

will transmit the NC/L command word :QJG: to signal that the

sequence  is complete. Should you receive it before you  get

any messages, then there weren't any pending.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 53

If  you  want  to download messages  from  another  LAN-LINK

system,  connect to it and type the sequence :QRU:  followed

by  the 'Enter' character. that system will then respond  to

you as if the operator there had initiated a QRU command.

Appendix 2 contains more information on the LAN capabilities

of LAN-LINK.

13.7 Who is "ON-LINE"

You  use  this option (when unconnected and  monitoring  the

traffic on a channel) to see who is 'ON-LINE' on the LAN. It

is  the  same as going manually into the Converse  Mode  and

transmitting an unconnected packet containing the  character

string :QRA:.

13.8 :QTA: Messages

If you have a bad path (See Section 24.12) and your messages

are  not  automatically deleted by the other  station  after

they  have been sent to you, you use this option  to  delete

your message. It is the same as manually typing ':QTA:'.

13.9 :QZD: Binary Files

This  command  uses the external batch files to  download  a

Binary  file from another LAN-LINK Station using the  Zmodem

Protocol.  See  Appendix 2.5 for further  information  about

LAN-LINK's  binary file transfer capability. If you  attempt

to  download a file which exists in your computer,  LAN-LINK

will  use ZModem's "restore after interrupt"  capability  to

pick  up  the transfer at the point when  (it  assumes)  the

transfer  was  interrupted.   You must  configure  all  four

QZ*.BAT files before trying to :QZD: as described in Section

24.13.

13.10 :QZU: Binary Files

This  command  uses  the external batch files  to  Upload  a

Binary  file  to another LAN-LINK Station using  the  Zmodem

Protocol.  See  Appendix 2.5 for further  information  about

LAN-LINK's  binary file transfer capability.  LAN-LINK  will

not  let  you upload a file to the other system, if  a  file

with that name already exists on the system.  This stops you

overwriting existing files. If the link drops out while  you

are  uplinking  a  file, and you wish to  try  again  later,

rename  the  file you are sending, then  reconnect  and  try

again.  Don't forget to :QSP: the other station and tell the

operator what happened.  You must configure all four QZ*.BAT

files before trying to :QZU: as described in Section 24.13.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 54

14.0 Log Menu

LAN-LINK performs automatic logbook entries for Packet and  AMTOR

Beacon mode Connects, and semiautomatic logbook entries for other

modes.  The Log files are in dBASE 3 format and can be  processed

by  the  DBASE Logbook Package in PC-HAM  for  indexed  listings,

tracking of DXCC and other AWARDS, etc. This is ideal for special

event  stations and DX-peditions, for the whole QSL  process  and

further statistical analyses of their operation. The structure of

the logbook files are as shown below.

Field Field Name   Type     Length

1    DATE      Character      8

2    TIME      Character      4

3    BAND      Character      3

4    CALL      Character     10

5    RX        Character      3

6    TX        Character      3

7    MODE      Character      4

8    POWER     Character      4

9    QSLSENT   Character      1

10   QSLRX     Character      1

11   COMMENTS  Character     20

-------------------------------

** Total **                  62

The  size of the COMMENTS field is a compromise. Each field is  a

fixed  size  so  that each log entry takes up  a  minimum  of  62

characters  worth of space on the disk even if all the  character

space  is  unused. You may change the size of the  field  if  you

wish, but to do that you will have to use dBASE. Note also,  that

if  you  do make the comments field longer, the display  will  be

screwed up.

For  the  logging feature to work, the logbook file  VHF.DBF  (or

HF.DBF) must be present on the disk.

When  an AMTOR beacon mode or packet connect occurs, the  connect

is automatically logged. In the Packet Mode, the connect path  is

placed  in the comments section of the logbook entry. In the  non

packet  modes, the log data entry is set up for you with  default

599 values for the reports exchanged.

When you bring up the Log Menu you will be shown the last  screen

or  page of log entries. The last entry will be  highlighted.  At

this  time  a  number of function keys can be used,  or  you  may

depress  the  'Escape'  key to bring up the Main  Log  Menu.  The

following are the Function Keys are active in the Log Menu.

Alt-A      Append Entry

Alt-E      Edit Log Entry

Alt-F      Find an Entry

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 55

Alt-S      Scan Log by Call

Alt-U      Toggle Delete Mark

Alt-Z      Pack Log

Ins        Toggle Insert Mode

End        Show Last Page

Home       Show First Page

PgUp       Move Up One Page

PgDn       Move Down One Page

Up Arrow   Move Up One Entry

Down Arrow Move Down One Entry

The operation of each function key is described below.

14.0.1 Alt-A Append Entry

This function key allows you to manually append an entry  to

the log. You can also use this command to enter the odd  SSB

contact into the logbook file. If you do, use the characters

'SSB'  or  'FM' for those voice modes to  remain  compatible

with the rest of the DBASE logbook package.

When   appending   or  editing  an  entry,   LAN-LINK   will

automatically  enter the date, time, callsign of  the  other

station, band, mode and QSL information in upper case.

The   comments  field  in  the  LAN-LINK  logbook  file   is

restricted  to  a maximum of 20 characters. If you  try  and

enter more than 20, the surplus will be ignored.

14.0.2 Alt-E Edit Log Entry

This function key allows you to edit the highlighted  entry.

Move  the  cursor  with the arrow keys,  and  end  the  edit

process by depressing the 'Escape' key.

You  may use the 'QSL' entry to tag the fact that  you  have

written out or received a QSL card.

14.0.3 Alt-F Find an Entry

This  function  key  allows  you to find  an  entry  by  the

(record)  number  assigned to it by LAN-LINK.  This  is  the

number displayed in the first column.

14.0.4 Alt-S Scan Log by Call

This function key allows you to scan the logbook by callsign

prefix.

14.0.5 Alt-U Toggle Delete Mark

This function key allows you to mark an entry for  deletion,

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 56

or  un-mark it in case you change your mind. Entries  marked

for deletion will be flagged by an '*' character next to the

record number.

14.0.6 Alt-Z Pack Log

This function key allows you to pack the logbook files.  All

entries  marked for deletion will really be deleted at  this

time.

14.0.7 Ins Toggle Insert Mode

This function key allows you to turn the insert mode ON  and

OFF for the edit/append functions.

14.0.8 End Show Last Page

This function key moves you to the last page of the logbook.

14.0.9 Home Show First Page

This  function  key  moves  you to the  first  page  of  the

logbook.

14.0.10 PgUp Move Up One Page

This  function  key  moves the display up one  page  of  the

logbook.

14.0.11 PgDn Move Down One Page

This  function  key moves the display down one page  of  the

logbook.

14.0.12 Up Arrow Move Up One Entry

This  function key moves the highlight up one entry  in  the

logbook.

14.0.13 Down Arrow Move Down One Entry

This function key moves the highlight down one entry in  the

logbook.

When  you  bring  up  the Main Log  Menu,  it  provides  the

following choices.

A Append Entry

B Scan Log by Band

C Change Log

D find Date

E Edit Log Entry

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 57

F Find an Entry

I Import-Convert Log

H Help Function Keys

K Pick Log file

M Scan Log by Mode

N Create New Log File

P Print Log File

S Scan Log by Call

U Update from Old data

X eXit From Log Menu

Z fuZzy scan

Type in the single letter to make your choice.

14.1 Append an Entry

This option lets you append an entry. It is the same as  the

Alt-A logbook mode function key.

14.2 Scan Log by Band

This option lets you scan the logbook file and display  QSOs

made on a particular band.

14.3 Change Logbook File

This option lets you temporarily change the logbook file. If

you  want  to change it permanently, change it in  the  LAN-

LINK.SYS  file. When you enter the new name,  LAN-LINK  will

assume  that it is a filetype DBF. For example, if you  have

different logbooks for different operations on-line, such as

4X.DBF or W8.DBF, you only need answer 4X in response to the

question to change to the 4X.DBF logbook. In accordance with

the dBASE convention, you cannot use any filetype other than

DBF.

14.4 Find Date

This  option  lets  you locate and  move  the  highlight  to

entries  for a particular date. If no entries are found  for

the  desired date, the first entry after that date  will  be

highlighted. Note you can also scan for parts of a date such

as "90/01" for the first entry in January 1990.

14. 5 Edit Log Entry

You  invoke this option to edit the highlighted entry.  This

function is the same as that invoked with the Alt-E function

key.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 58

14.6 Find an Entry

This option lets you find an entry by its number. It is  the

same as the Alt-F logbook mode function key.

14.7 Import-Convert Log

This  option lets you import logs from earlier  releases  of

LAN-LINK (up to and including Version 1.56). It can also  be

used  to  import  a  log generated  by  the  PC-HAM  CONTEST

program.

14.8 Help Function Keys

This  option brings up a menu listing the log mode  function

keys.

14.9 Scan Log by Mode

This option lets you scan the logbook file and display  QSOs

made using a particular mode.

14.10 Print Log

This  command prints the contents of the logbook file  in  a

formatted manner to the printer.

14.11 Scan Log by Call

This command displays the contents of the logbook file in  a

formatted manner on the screen. It requests the callsign  of

the  log entry to be displayed. If you want to  see  entries

for  particular calls or parts of call, enter those call  or

parts  of  calls. Valid entries are G, G3, G3Z etc.  If  you

want to see every entry in the log, don't use this  command,

use the Page Up and Page Down keys from the Main Log menu.

14.12 Update from Old data

This  option is identical to the Append option  except  that

the  date  and time are not entered for  you  automatically.

You'd  use this option to manually enter old data  into  the

logbook.

14.13 Exit From Log Menu

This option lets you exit from the Log Menu. It provides the

same function as the Alt-X key.

14.14 Fuzzy Scan

This  option allows you to scan the callsign or the  comment

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 59

field (your choice) for a character string. For example  you

could scan the comments field for all entries with the  word

"Joe".  You could scan the callsigns for all calls with  the

suffix of "ZCZ".

14.15 Pick a Log file

This  command  allows  you to pick a *.DBF  file.  When  you

exercise  this command you will be presented with a list  of

log  files (*.DBF. Move the cursor down to the one you  want

to use and touch the 'Enter' key to select it.

14.16 Create a New Log file

This  command allows you to create a blank  logbook  (*.DBF)

file.

15.0 Message Menu

The Message Menu provides the following choices.

B BBS Command Files

C Current Messages

N leave Note

O Old Messages

T Time tag

V View Message

Type the single letter to make your choice.

15.1 BBS Command Files

This option shows you a list of the BBS command files on the

default  directory. In other words, all files with the  file

extension of '.BBS'.

15.2 Current Messages

This  option shows you a list of the outgoing messages  that

have not been sent. In other words, all files with the  file

extension of '.OUT'.

15.3 View Message

This  feature allows you to view messages in  the  computer.

After  depressing the key you will be prompted for  INcoming

or  OUTgoing (I/O) messages. If you request  'I'  (Incoming)

messages  you  will  see  the  messages  addressed  to  your

callsign.  If  you request 'O' (for OUTgoing)  you  will  be

presented  with  a  directory window containing  a  list  of

messages.  Move the cursor down to the desired message,  and

touch  the  'Enter' key. If you don't want to  view  any  of

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 60

them, then move the cursor down to a blank line. If you have

more messages than one screen full, use the Pgdn (Page Down)

key to show the next screen full.

15.4 Leave Note

This feature allows you to leave a note for another  station

on the LAN in your system. It works in the manner  described

in Section 8.

15.5 Old Messages

This feature shows you a list of the messages that have been

sent.  In other words, all files with the file extension  of

'.OLD'.

15.6 Time Tag

This option is used to place a date time header on a message

file  previously prepared by a word processor. The  name  of

the file must be in the format 'CALLSIGN.OUT' (For  example,

G3ZCZ.OUT). It requests the callsign of the message file. It

then opens a new file, writes the header to the new file and

copies the contents of the original '*.OUT' file to the  new

one.  When all is done, the original filename is changed  to

'*.BAK' and the new one becomes '*.OUT'.

16.0 TNC Menu

The TNC Menu provides the following choices.

D Set TNC Date/Time

F Flush TNC Buffer

I Initialize TNC

M calls Monitored List

Q Change CQ Text

S Show Connect Status

T Change CTEXT

X Activate TNC Mailbox

Y Deactivate TNC Mailbox

Type in the single letter to make your choice.

16.1 Set TNC Date/Time

This  feature puts the TNC into the Command Mode  and  loads

the date into it from the computer time of day clock.

This command does not work on the TNC1.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 61

16.2 Flush TNC Buffer

This  option  is  used to flush the  program's  incoming  or

outgoing text buffer and that of the PK-232. When used  with

other  TNCs  it  will only flush contents (if  any)  of  the

buffers  in the PC.  If you flush the incoming  buffer,  you

will  only flush the buffer in the PC, thus  any  characters

still  in the TNC will still come through. If that  happens,

bring up the function and wait a while before activating it.

16.3 Initialize TNC

This feature sets up the defaults for the TNC from the  file

LAN-LINK.SYS.  If  you have put batteries in  the  TNC,  you

should normally only have to run this command when you first

power  up  the  TNC or change the  batteries.  However,  the

batteries must be removed initially since they are there  to

retain the default settings of the TNC (particularly the PK-

232).  Once the initialization is complete, you can  replace

the  batteries  but  leave  the TNC  on  since  the  battery

provides  power  to the RAMs after the TNC power  is  turned

off.

It  will also set the date and time within the TNC from  the

computer  clock, and, when it has loaded  the  configuration

file,  may prompt you to set the TNC Terminal mode.  If  you

are  using a TNC1 or a TNC2, it will not prompt you  because

it assumes that they only operate on VHF packet. If you  are

using a KPC-2, you may select 'H' or 'V', and of course,  if

you have a PK-232, you may choose any of the options.

16.4 Show Calls Monitored List

This  feature performs the same operation as Function Key  5

(F5).

16.5 Show Connect Status

When you are in the Packet Communications Mode, this command

lets  you see the state of the connection. It will show  you

if  there  are any unacknowledged packets en  route  to  the

other  station.  If  you  have  been  sending  text,  it  is

advisable  to  use this command before  you  disconnect,  to

ensure that all the data has been received and  acknowledged

by  the other party. You may use this command in either  the

TNC Command or the TNC Converse Mode.

In a multiconnect situation, it will show you the status  on

all the I/O channels.

You  may  also use this command to reset the  status  window

display  after  a relink or any other event  which  was  not

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 62

reflected in the status window.

16.6 Change CTEXT

This  feature allows you to temporarily change  the  message

that  will  be received by another packet  station  when  it

connects  to  you. Refer to the 'CTEXT' command in  the  TNC

manual.

16.7 Activate TNC Mailbox

This  option  allows you to activate  the  personal  mailbox

built into your TNC. You use it to read your mail, or before

shutting down, if you have not set the shut down mode to 5.

You will have to interrogate the mailbox in the command mode

(F) to view any messages.  The mailbox in your TNC  provides

a convenient way for people to leave you a message when your

PC  is  switched off or is in use for something  other  than

packet  radio. If you do choose to make use of the  personal

BBS in the mailbox, make sure that you have a battery in the

TNC.  When reading your TNC Mailbox mail, you must type  the

commands listed in your TNC manual.

To  save your local full service BBS SYSOP some hassles,  it

is  strongly  recommended  that you  let  LAN-LINK  do  your

outgoing  and  incoming  message  and  bulletin  store   and

forwarding. Use the 'L' option in the Edit menu to enter any

messages off-line, the 'M' and 'K' options in the Edit  menu

to answer any incoming messages or comment on any bulletins.

Once the data are in the LOCAL-BBS.BBS file, let the BBS Zap

take care of transferring the messages to the BBS.

16.8 Deactivate TNC Mailbox

This  option allows you to deactivate the  personal  mailbox

built into your TNC.

17.0 AMSAT-OSCAR Menu

This menu allows you to configure the KAM, MFJ1278 or the  PK-232

to  copy signals from AMSAT-OSCAR Spacecraft  telemetry  beacons.

The following options are available.

A UoSAT ASCII Beacon (PK-232 only)

B Phase 3 RTTY Beacon

C SAREX Menu   (only   if  you  are   in   the   Packet

Communications Mode)

Type in the single letter to make your choice.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 63

17.1 UoSAT ASCII Beacon

This  option is only available on the PK-232. It  configures

the  PK-232 for receiving UoSAT 1200 Baud ASCII  signals  in

the following manner.

ASCII

AB 1200

VHF ON (Bell 202 tones)

RXR OFF

TXR OFF

WI ON (850 Hz shift)

Signals   from  UoSAT-OSCAR  11  may  be  copied   in   this

configuration.  Note that although UoSAT-OSCAR 11  transmits

Bell  202  compatible tones, the data sense is  reversed.  A

hardware  modification is required in the PK-232 to  reverse

the  sense of the data at the modem output BEFORE it  passes

to the digital section of the unit.

17.2 Phase 3 RTTY Beacon

This  option configures the KAM, the MFJ1278 and the  PK-232

for  receiving  the  AMSAT-OSCAR 10 and  13  RTTY  telemetry

beacons in the following manner.

BAUDOT

WI OFF (170 Hz shift) [ KAM : SH 170]

TXR OFF

RXR OFF

RB 50 (50 baud)

USOS OFF (Unshift on space off)

17.3 SAREX Menu

This menu is only active in the Packet Communications  Mode.

It  is  designed  for use with  the  Shuttle  Amateur  Radio

Experiment  (SAREX), MIR and the packet radio  Microsats  or

any terrestrial station you want to connect with as soon  as

they appear on frequency. Apart from the Attack Mode, it can

also  be  used as a diagnostic LAN tool for  monitoring  LAN

traffic.

17.3.1 Attack Mode ON/OFF

If the Attack Mode is set, this option will cause  LAN-

LINK  to  issue  a connect request to  the  SAREX  Call

whenever a packet sent to or from it is heard. The mode

is cleared when the connect is made (and does not retry

out) or when the 'A' option is selected a second  time,

or  when  the Alert Call is cleared. If  this  mode  is

enabled,  the  Alert/SAREX  Call prefix  shown  in  the

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 64

Status Window will indicate accordingly.

A  happy face will be displayed next to the '>'  before

the call once the connect has been achieved.

Be careful using this feature, as it has the  potential

to cause a great deal of QRM. It can also be cleared by

another  station connecting to you and telling  you  to

':QRT:'.  In the SAREX configuration you are  listening

on one channel while transmitting on an other. If  this

feature  was  used  on  a  simplex  channel   (everyone

transmits  and listens on the same channel) aimed at  a

DX-pedition,  there  is a potential  lockout  mode,  in

which any one station sending a connect request to  the

desired/SAREX  call  will  trigger the  other  ones  on

frequency  who will in turn trigger the first. If  this

pile  up situation occurs, I hope the  DX-station  will

QSY and leave the automatic stations to do their thing.

IN  any event, if you use this feature and  cause  QRM,

anyone  can connect to you with the callsign of the  DX

station (as a pirate) and shut you down. You will  then

not get the coveted DX QSL.

17.3.2 Change SAREX Call

This option lets you temporarily change the SAREX call.

17.3.3 SAREX Packets Capture to Disk ON/OFF

This option allows you to configure LAN-LINK to capture

to  disk  any packets addressed to or  from  the  SAREX

callsign. A [S] will be displayed in the status  window

when this option is active. The capture to disk file is

opened  by a packet header containing the  SAREX  call,

and closed by another packet header not containing  the

call. When the file is open, the [S] will blink. Packet

headers are considered to be lines with a '>' character

in  them.  LAN-LINK thus considers both  of  the  lines

below as packet headers.

N4QQ*>G3ZCZ

N4QQ BBS>

If you use this feature for monitoring BBS traffic, the

BBS  prompt  lines  ('E.G.  N4QQ  BBS>')  will  not  be

captured  to disk. Do not get your SYSOP to remove  the

'>' from the prompt line, or the ZAP feature will  then

not work.

A  happy  face will be displayed in the [S]  after  the

SAREX call has been heard.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 65

If you want to see an ORBITER menu, temporarily  change

the SAREX call to be the same as your call and bring up

the AMSAT-OSCAR Menu.

17.3.4 Edit SAREX Capture-to-disk File

This  option  lets you edit the  SAREX  Capture-to-disk

file as long as it is less than 64 kBytes.

17.3.5 Zap SAREX On/Off

This  option combines the 'Set SAREX Attack  Mode'  and

the  'SAREX Packets Capture to Disk ON/OFF' options  in

one fast operation.

17.4 SAREX Orbiter Features

You need the SAREX ROBOT EPROM for this menu to actually  do

anything  to  the TNC. The EPROM may be available  from  The

Radio Amateur Satellite Corporation (AMSAT) for  DX-pedition

use. Other options which control LAN-LINK parameters may  be

used without the SAREX ROBOT EPROM.

In  the Orbiter mode, LAN-LINK does not scan  incoming  text

for  the  'CONNECTED' and 'DISCONNECTED' messages  when  the

SAREX  Robot  is  active. This inhibits the  bells  and  the

capture-to-disk.  It also means that the Robot must  be  OFF

for any :Q: code uploads or downloads to occur.

17.4.1 Status Window Display

A flashing '>>' appears in the status window when the  SAREX

robot  is active. The SAREX Configuration boots up  assuming

that the SAREX Robot is active (Default).

17.4.2 Alt-O Function key.

The Alt-O Function Key brings up the Orbiter menu. This is a

faster route than the 'Esc O' sequence.

17.4.3 The Metabeacon

The  Metabeacon  is loaded from the contents of one  of  the

META.00n  files. These files may be created by the  LAN-LINK

editor. When loading, LAN-LINK will insert a pass  character

(^V)  into the text before a carriage return, so as  not  to

end the packet before at least 240 characters, and not  more

than 255 characters have been output to the TNC. The maximum

number of characters that can be loaded into the  Metabeacon

is 1776. The maximum number of packets in a Metabeacon is 7.

You can also force a hard carriage return or 'end of packet'

in   the  Metabeacon  by  inserting  the   '\'   (backslash)

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 66

character, which will not be transmitted, into the text.

17.4.4 SAREX Orbiter Menu

The following commands are active in the Orbiter Menu.

17.4.4.1 Activates Beacons/SAREX Robot.

This  option activates the SAREX Robot and  loads  Meta

Beacon. This command is designed for use when the SAREX

is returned to the regular frequency.

17.4.4.2 Set QRZ/QSL Beacon Interval

This command allows you to change the beacon timer. The

default value is 120.

17.4.4.3 Set SAREX Call.

This command allows you to change the SAREX call. Don't

use this command on orbit (its for ground use only)

17.4.4.4 Turn SAREX ROBOT OFF

The  Robot  must  be  turned On and  Off  this  way  to

synchronize  LAN-LINK  to the TNC. When  the  Robot  is

turned  OFF the following commands are sent to the  TNC

by LAN- LINK.

USERS 1

|A

RO OFF

CMSG OFF

NO OFF

17.4.4.5 Set SAREX ROBOT Heard Count

This command lets you set the Heard Count number.

17.4.4.6 Show I/O Status

Shows who is on each stream, the SAREX Robot state, the

number of USERS, the beacon timer value, the metabeacon

timer value and the CTEXT.

17.4.4.7 ChecK Count

Shows both heard and worked counts.

17.4.4.8 List Heard Calls

Lists calls of the last few stations heard.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 67

17.4.4.9 Load Metabeacon

This command loads the Metabeacon. You will be prompted

for a text file number in the META.00n series. Enter  a

number  1-9 corresponding to META.001-META.009.  The  0

value  is file META.010. Note the delays after  loading

the file to allow the TNC to stabilize.

17.4.4.10 Show Worked Calls

Shows the last 35 or so calls worked.

17.4.4.11 Turn SAREX ROBOT ON

The SAREX Robot must be turned On and Off via the menu.

When the Robot is turned ON the following commands  are

sent to the TNC by LAN-LINK.

USERS 9

CONOK ON

CMSG ON

RO ON

NO ON

17.4.4.12 Stop Metabeacon

This command stops the Meta beacon. When activated, the

contents  of  the  beacon are  dumped  to  the  UNPROTO

address  (default  SAREX)  by the TNC,  rather  like  a

regular  BBS  dumping packets when a  user  disconnects

abnormally.

17.4.4.13 Set Metabeacon Interval

This  command allows you to set the Meta Beacon  timer.

It  will be loaded into the TNC the next time that  the

Meta Beacon is loaded. Even though loaded into the  TNC

immediately, it does not take effect till then.

17.4.4.14 Set SAREX ROBOT Worked Count

Lets you set the number.

17.4.4.15 Set Connect TeXt

This is the text (CTEXT) that will be transmitted to an

incoming  station  when  the ROBOT mode  is  off.  This

option lets you set up a new text string to go with the

QSO number.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 68

17.4.4.16 Selects Silent Mode

This command turns off the beacons, the SAREX Robot and

also stops the Meta beacon. This command is designed to

be used just before any prearranged packet activity.

17.4.4.17 Restart TNC

This command RESTARTs the TNC, loads the date from  the

computer's  clock and turns the 'EC OFF'. It  seems  to

have  no  effect on the SAREX ROBOT and  counters,  but

does kill the Meta beacon.

17.4.4.18 Capture QSL/QRZ Calls

This  command initiates the log-to-disk  operation.  It

turns the capture-to-disk 'on' and sends the  following

commands  to the TNC :- 'DA', 'SERW',  'SERH','MW',MH'.

When the calls have stopped scrolling, use Function key

1 to turn the capture-to-disk off.

18.0 Parameter Menu

The Parameter Menu provides the following choices.

B Turn Beacon Mode ON/OFF       0 Change Band/Mode

C Clear Connect Count           1 Change BBS Parameters

D Turn Digipeat Detect ON/OFF   2 Change Callsigns

E Turn Command Echo ON/OFF      3 Change Directories

F Stay Connected after BBS Zap  4 Enter Common Calls

I Turn Beacon ON/OFF            5 Change File Names

K Change Keyboard Beep Count    6 Change LAN-LINK Parameters

L Turn Ctdsk ON/OFF             7 Change PC Communications Params

M Set Message Count             8 Change Colors

N Turn NODE Drop Link ON/OFF    9 Change SAREX Parameters

P Turn Printer ON/OFF           U Update LAN-LINK.SYS

Q Turn QTC Snatch ON/OFF        V Verify Changes

R Turn :QSP: Relay Flag ON/OFF  W Change Scan Words

S Turn Sound ON/OFF

T Turn Contest Mode ON/OFF

X Turn DCD Flag ON/OFF          ! Zap  BBS on Next Beacon

Y Turn BBS Snatch ON/OFF        + Turn Misc. Flag ON/OFF

Z Turn Time Id. ON/OFF

The colors used for the option letter differentiate between those

options that are "remembered" in the SYS file and those that  are

only set temporarily for the session.

Type in the single letter to make your choice.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 69

18.1 Turn Beacon Mode ON/OFF

This command toggles the beacon option for the automatic  CQ

caller in the AMTOR and Packet Communications Modes. When it

is  active a [B] will be displayed in the status window.  In

this  mode, the Automatic CQ sequence will resume after  the

AMTOR  link  is  dropped or  a  Packet  Communications  Mode

disconnect.

In  the  Beacon Mode, the last few logbook entries  will  be

displayed in the logpage window which overlays the  outgoing

text window at the bottom of the screen.

In   the  Packet  Mode,  the  CTEXT  message  is  the   only

transmission  from you that the other station will  receive,

so make it good and don't forget to set 'CMSG ON'.

In  the AMTOR Mailbox/Beacon Mode, the contents of the  LAN-

LINK.TXT file will be transmitted to the station who  called

you.  The capture-to-disk will then be turned on  to  record

one  transmission  from that station. The QSO will  then  be

logged  to  the  logbook file. Don't  forget  to  check  the

entries  in  the LAN-LINK.LOG and text in  the  LAN-LINK.RUN

files in case the computer made a mistake in recognizing the

call sign of the other station.

THE  BEACON MODE IS DESIGNED TO ENCOURAGE PEOPLE TO QSY  OFF

THE  CROWDED PACKET HF CHANNELS. DO NOT USE IT ON A  CROWDED

HF CHANNEL OR ON AN ACTIVE VHF CHANNEL. IF YOU DO, YOU  WILL

SOON FIND that OTHER STATIONS HAVE SET THEIR TNCS TO  REJECT

YOUR  PACKETS, AND YOU WILL NOT WORK ANYBODY, YOU WILL  JUST

BE A SOURCE OF QRM AND WILL BE KNOWN AS SUCH.

THE  MODE IS ALSO DESIGNED FOR ROBOT BEACONS FOR THE  HIGHER

FREQUENCIES. FOR EXAMPLE IF YOU LEAVE IT RUNNING ON 6 OR  10

METERS YOU NEVER KNOW WHEN A SHORT OPENING WILL ALLOW A  QSO

TO  SOME  PART  OF THE WORLD, A DX CONTACT  YOU  WOULD  HAVE

OTHERWISE MISSED. DO NOT LEAVE IT RUNNING ON 20 METERS  WHEN

THE BAND IS OPEN. FURTHER APPLICATIONS ARE IN  METEORSCATTER

AND OTHER "EXOTIC" MODES OF COMMUNICATIONS.

If you are using this Mode in a special event station or DX-

pedition  you may type characters into the transmit  buffer,

while  the other station is sending to you. When he, she  or

even  it  turns  it back to you,  you  will  transmit  those

characters before the sign-off sequence.

In both the Packet and AMTOR communications modes, the robot

may  be shut down by stations connecting to you and  telling

you to QRT.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 70

18.2 Clear Connect Count

You  use  this option to reset the PACKET and  AMTOR  Beacon

Mode  "connect  has taken place indicator"  and  QTC  Snatch

count. These are flashing [C] and [Q] displays in the status

window.  Whenever someone else connects or links up  to  you

the  [C] display will be incremented. The number shown  with

the  'C'  is the number of connects that  have  taken  place

since  the  Indicator was last reset. If a 'happy  face'  is

flashing along with the connect count, your "target station"

was  amongst those connecting. The [Q] indicator alerts  you

to  the  fact  that LAN-LINK tried  to  connect  to  another

station  after recognizing your call in a mail beacon.  Both

are  reset  by  this option. If you  are  monitoring  Packet

traffic on a channel, you may not notice that a connect  has

previously taken place. These indicators alert you to  check

the contents of the Incoming mail file ('yourcall.OUT')  for

any incoming messages.

If the 'Q' count is much greater than the 'C' count you  may

be  copying a beacon from a station that you cannot  connect

with directly.

18.3 Turn Digipeat Detect ON/OFF

This option toggles the digipeat display command on and off.

If  the Digipeat Display is active, any  signals  digipeated

through  you will be stored to the LAN-LINK.RUN file  and  a

'beep' will be sounded for each packet being digipeated. The

status display window will also contain a [D].

18.4 Turn Beacon Control ON/OFF

This  option toggles the LAN-LINK Handshake  features.  When

they are on (default), the Connect handshake message

[ZCZ] *** LAN-LINK 1.59>

will  be transmitted on incoming connects. The  beacon  text

and time will be controlled by LAN-LINK as a function of the

messages you have posted on your system. When they are  off,

LAN-LINK  does not do anything to your beacon, nor  does  it

transmit the handshake.

18.5 Turn Command Echo ON/OFF

This  option toggles the 'Command ECHO ON/OFF' flag. If  the

Command  Echo  flag is set, the commands and  (contents  of)

files  that LAN-LINK sends to the TNC will be  displayed  in

the  outgoing  window.  Set this flag to  see  how  LAN-LINK

commands the TNC in the different modes.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 71

18.6 Change Keyboard Beep Count

LAN-LINK  counts the number of characters entered on a  line

at  the  keyboard. When it gets to a preset  value  it  will

beep, signaling you to start a new line. This option changes

the value that the keyboard beep counter is compared with to

sound the audio signal.

18.7 Set Message Count

As  an  alternative to Function Key 8 (F8) or  Alt-D,  using

this  option  will  also load a QTC 'Mail'  list  into  your

Packet  Communications  Mode Beacon and  initiate  a  beacon

transmission every 30 minutes (Refer to the BTEXT command in

the  TNC  manual). If no messages are present, or  the  only

message   is   to  you  (incoming   or   snatched),   beacon

transmissions are inhibited. This conforms to good operating

practice on crowded Packet channels.

18.8 Turn NODE Drop Link ON/OFF

When connecting via NET/ROM, the normal way of operating  is

to  disconnect  if  the  connect  attempt  fails.  You   may

configure the NET/ROM connect attempt flag to stay connected

at the node where the failure occurred. Use the normal  mode

when  trying  to get through on a known path, and  the  stay

alive mode when exploring NET/ROM circuits.

18.9 Turn Printer ON/OFF

This option toggles the printer on and off. When the printer

is  turned  on, any incoming data will be  printed  in  real

time. If you intend to print files received at 1200 baud  in

the  Packet Communications Mode, use a print spooler or  you

may  lose data. The printer is automatically turned  off  in

the Packet Mode when a disconnect occurs.

18.10 Turn QTC Snatch ON/OFF

This  option toggles the QTC Snatch flag on and off. If  the

QTC  Snatch flag is enabled, LAN-LINK scans the contents  of

packets  appearing in the Inwindow on the screen  while  the

terminal is disconnected. Should it recognize the three text

strings  'LAN-LINK',  ':QTC:', and 'your  callsign'  in  the

contents  of a packet, it will then issue a connect  request

to the callsign of the LAN-LINK station that originated  the

beacon  transmission.  If  the connect is  made,  the  other

station  will send you the message automatically which  LAN-

LINK  will "capture to disk". The connect will time out  and

terminate a few minutes later.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 72

The  normal  connect  request will be  made  DIRECT  to  the

station.  If you expect messages from stations you can  only

connect  with via a digipeater, place the path in your  LAN-

LINK.DIR  directory file. It should be there anyhow  if  you

connect to the station regularly. For example, if you expect

messages from 4Z4ZB whom you can only work if digipeated via

4X1AA then place an entry like

4Z4ZB 4Z4ZB V 4X1AA

in the LAN-LINK.DIR file.

The  QTC  Snatch  leaves your messages  in  a  message  file

<yourcall>.OUT.  It  is  triggered by the  first  >  in  the

initial  connect response of LAN-LINK 1.59R> (note the  '>')

and  terminated by the 'over' at the end of the message  ea.

G3ZCZ/W8>  (note the '>'). If you train stations  not  using

LAN-LINK who want to leave you messages to trigger your QTC-

Snatch, by transmitting the following unaddressed packet

LAN-LINK :QTC: <yourcall>

your  system will connect to them. (For example to  leave  a

message for me, they would send

LAN-LINK :QTC: G3ZCZ

as  an  unaddressed  packet  [in  Converse  Mode  when   not

connected, or in their beacon text].)

If they then send you a '>' on a line by itself, followed by

the  message  itself, and terminated by another '>'  as  the

last  character  of the line, the message will end up  in  a

message  file rather than in the LAN-LINK.RUN file  and  you

can scan it fast using Alt-M (Message Scan Menu Options). It

goes  without  saying  that the message  itself  should  not

contain the '>' character. If it does, by the way, the first

part  of the message will be in the message file, while  the

remainder will be in the LAN-LINK.RUN file.

18.11 Turn :QSP: Relay Flag ON/OFF

This  option  toggles the Store and  forward  Message  Relay

Capability  of your system on and off. If the Message  Relay

Capability is active (toggled on), any one who connects  can

store a message in your system using the ':QSP:  <callsign>'

NC/L command. If the Message Relay Capability is not  active

anyone  who  attempts to ':QSP: a message'  will  receive  a

':QNO:  1' "error" message. The status display  window  will

show a [R] when the Message Relay Capability is active.  The

default  state  for the Message Relay Capability  is  loaded

from the LAN-LINK.SYS file.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 73

When the Store and forward capability is active, the  suffix

'R' will be added to the

'[ZCZ] *** LAN-LINK 1.59' ie., '[ZCZ] *** LAN-LINK 1.59R'

message  received  by anyone connecting to  your  system  to

notify  them  that they can store messages. In  this  manner

they  don't  have  to try to leave a message  and  then  get

':QNO:'  error  message  to find out  that  the  feature  is

toggled off.

Remember,  if  you  configure your  computer  to  store  and

forward  messages,  you have a moral obligation  to  deliver

them.

18.12 Turn Sound ON/OFF

This  option toggles the sound on and off. If the  sound  is

off,  a flashing indicator will be displayed in  the  Status

Window.  When  the sound is toggled on again, a  chime  will

sound to verify the audio.

18.13 Turn Contest Mode ON/OFF

This  option first toggles the Contest Mode. If the  Contest

Mode  is active, the word 'TEST' and the number of the  next

QSO will be displayed in the status window. When the Contest

Mode is toggled on, this command then prompts you to set the

starting  QSO number and the standard message that  will  be

transmitted  as  part of the exchange of messages  during  a

contest.  The QSO number will be incremented each  time  you

log a contact.

There are two special character sequences that you can enter

in the contest text string. If you enter a double '#'  sign,

as  in '##', it will be replaced by the QSO number when  the

text is transmitted. You may also use '#1' to represent  the

callsign  of  the station being worked in the  contest  text

string.  For  example,  use the following  entries  for  the

following contests;

'##1 UR ##A' ... in the Sweepstakes,

'5905' ... in the WAZ,

'59 MD' ... in the ARRL DX, and

'59 ##' ... in the WPX.

As  an example, if I (W3/G3ZCZ) was in the Sweepstakes  con

test, and my Contest text line was,

'##1 UR ##A W3/G3ZCZ 68 MDC'.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 74

If  the next QSO number is 345, and I am working KW3C,  LAN-

LINK  would  transmit the following line when  I  touch  the

'End' key;

'KW3C UR 345A W3/G3ZCZ 68 MDC'.

The contest feature is toggled off the next time you  select

this feature.

Note  that  while  this does  give  you  some  semiautomatic

contest  operation  by  using  command  keys,  there  is  no

checking of callsigns for duplicates built into the program.

You may do that by using the Alt-S hot key.

In  a  contest, you enter the callsigns and call  the  other

station  in  the normal manner, then use the  'End'  Key  to

transmit  the exchange (QSO number and standard text).  LAN-

LINK will only transmit the contest text in this manner.  If

you  want to add a 'K' or 'KN', you will need to put  it  in

the  contest  text  string. It is also advisable  to  use  a

different  log file for each contest to cut down the  search

time.  The contest QSO number increments each time  you  log

the QSO (Alt-L).

You  may  put  the Contest Mode into automatic  if  you  are

operating AMTOR or packet. To do so, use the Alt-O  function

key.

18.14 Turn DCD Flag ON/OFF

If you are using a KAM or a PK-232 then this flag should  be

set  to ON. If you are using another TNC, if you can  jumper

the TNC so that pin 8 on the RS-232 interface changes  state

when connected, do so and set this flag to ON. If you can't,

then set this flag to OFF. If this flag is OFF, the multiple

connect modes will not work so don't attempt to invoke them.

If  this  flag is OFF, the words '***  CONNECTED'  and  '***

DISCONNECTED'  when  seen  in the  LAN  may  cause  spurious

apparent connects and disconnects in LAN-LINK.

18.15 Turn BBS Snatch ON/OFF

If the BBS Mail Snatch is enabled, when a BBS sends a packet

announcing 'Mail', if your callsign is in the first line  of

that  message, LAN-LINK will issue a connect request  (again

via  the path in LAN-LINK.DIR) to that BBS. When  connected,

it  will act as if you have initiated a ZapBBS command.  The

Zap counter will be incremented each time a BBS Mail  Snatch

takes place.

A  typical KAM display which triggers a BBS Mail Snatch  for

any of the calls listed below follows.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 75

N4QQ>MAIL/V [09/03/89 18:49:24]:

LstMsg# 19428 Mail: N4FG KL7VZ WA6IEL N7DVA N9BE G3ZCZ

Note if the BBS Mail beacon has an extra line in it (between

the  header and the list of calls) as follows, the BBS  Mail

snatch will not work.

N4QQ>MAIL/V [09/03/89 18:49:24]:

BBS Serving Silver Spring Md.

LstMsg# 19428 Mail: N4FG KL7VZ WA6IEL N7DVA W4TNX G3ZCZ

If a message is posted for any call containing the character

string  "N4FG" such as N4FGA, N4FG will respond to the  mes

sage. This gives rise to a lock out situation, because  N4FG

will  respond  to the beacon, go through  the  sequence  de

scribed  in  the Zapbbs section and disconnect.  The  beacon

message will not be cleared because it was not read, so  the

next  mail beacon from the BBS will contain the call  N4FGA,

and N4FG will once again respond to it.

Should  you be unlucky and happen to be in  this  situation,

disable the BBS Snatch flag and set a Periodic Event to  Zap

your BBS sometime in the middle of the night. Alternatively,

when you come into the shack and the BBS is in use, set  the

'Zap  BBS on Next Mail Beacon' parameter to ON (See  Section

7.13).

18.16 Turn Time Id ON/OFF

This  option is only active in the PK-232. It allows you  to

determine   if  the  callsign  sequence  transmitted   using

Function  key 3 (F3) will contain a date/time stamp. If  the

Id is set ON a date/time stamp will be transmitted.

18.17 Turn Automatic Capture-to-Disk Flag ON/OFF

This option allows you to turn the automatic capture-to-disk

feature ON and OFF.

18.18 Change BBS Parameters

This  sub-menu allows you to change the BBS  Parameters,  as

described in the following paragraphs.

18.18.1 Local BBS Callsign

This option lets you change the call of the local  BBS.

This is the call that LAN-LINK will show in the  prompt

for the 'Zap' command.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 76

18.18.2 Mail-Snatch Trigger Word

You  can  set the Mail-Snatch Trigger in the  BBS  Mail

beacon.  Most  BBS's use word 'Mail' and  a  string  of

calls to indicate the presence of messages. If your BBS

uses something else, you may customize LAN-LINK on this

line to tell it what word your BBS uses.

Note that if your BBS uses 'QTC' you may end up calling

yourself if you disconnect while traffic is flowing  on

the LAN.

18.18.3 Read Request Command

This is the command LAN-LINK issues to the BBS during a

Zap sequence to request your mail. The default value is

RM for Read Mine. On a WA7MBL BBS you may change it  to

VM for a Verbose Read of the full packet headers. On  a

W0RLI  BBS you may change it to RN to only request  new

messages.  If you do change it, use the 'V'  option  in

the  Parameters Menu to make sure it is correct. If  it

isn't correct, the Zap BBS feature will not work.

18.18.4 Read Bulletin Request Command

This is the command LAN-LINK issues to the BBS during a

Zap  sequence to read bulletins identified by the  scan

words. The default value is R for Read. On a WA7MBL BBS

you  may change it to V for a Verbose Read of the  full

packet headers. On a W0RLI BBS you may change it to RH.

18.19 Change Callsigns

This  sub-menu allows you to change the Callsign  and  AMTOR

SELCAL, as described in the following paragraphs.

18.19.1 Callsign

The default callsign is G8BTB. Please do not use G8BTB.

Replace  G8BTB with yours. You may use / characters  if

you  wish. This is the call sign that will be  used  in

the  non  packet  modes. You  may  thus  use  "portable

designators"  such  as G3ZCZ/A, G3ZCZ/W3,  W3/G3ZCZ  or

G3ZCZ/4X, which you cannot do in Packet.

This  is  the call that will be monitored  for  in  the

Digipeat  Monitoring  Mode.  In  this  Mode,  which  is

toggled  on  and off by the selection of the  D  Option

included  in  the  Parameter  Menu  (PK-232  only),  if

someone  uses  you as a digipeater while  you  are  not

connected  to anyone else, the event will be logged  to

disk.  This  Mode will only work if  your  callsign  as

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 77

stored in this line is a valid (uppercase only)  packet

callsign.

For  example, in the PK-232, if the callsign  is  G3ZCZ

then  the  sequence  >G3ZCZ> will be  recognized  as  a

digipeat and the event logged. Thus  4X6AA*>G3ZCZ>4Z4ZB

would be logged as a digipeat event.

Store  your callsign using UPPERCASE characters if  you

want the DIGIPEAT DETECT option to work (EA. G3ZCZ). If

you use lower case (ea. g3zcz), recognition will  never

take  place.  The TNC automatically  converts  case  to

upper case in the non packet communications modes.

If  you are using an SSID (e.g. G3ZCZ-8) in the  Packet

Communications  Mode, you should enter the  whole  call

INCLUDING the SSID.

18.19.2 The SELCAL

This  is your AMTOR SELCAL. LAN-LINK will try to  guess

it  and  prompt you with a SELCAL configured  for  your

callsign.  If you want a different one,  overwrite  the

prompted version.

If  you are not using a PK-232 or KAM or  the  MFJ1278,

the SELCAL must still be in the Configuration file.

18.19.3 The PacketCluster Call

This option allows you to change the PacketCluster Call

without connecting to it.

18.20 Change Directories

This  sub-menu allows you to change the Directories used  by

LAN-LINK, as described in the following paragraph.

18.20.1 QBM Download Directory

This is the directory the :QBM: command will search  to

find  a  file to be downloaded. The  default  value  is

\LAN-LINK\FILES\.  Set  them up using DOS. If  you  are

using   a  hard  disk,  put  LAN-LINK  in  a   LAN-LINK

subdirectory. Note: that last '\' MUST be present.

18.21 Enter Common Calls

Each  time you call another station, LAN-LINK  remembers  it

(up  to  the last 16). When you update the SYS  file,  these

calls that you use most commonly will be saved to disk.  You

can use this option to add calls, delete calls or edit calls

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 78

in the list. Use the 'Delete' key to delete a call.

18.22 Change File Names

This  sub-menu allows you to change the default  file  names

used by LAN-LINK, as described in the following paragraphs.

18.22.1 VHF Logbook File (VHF.DBF)

This  is  the name of the file to be used as  your  VHF

Packet logbook.

18.22.2 HF Logbook File (HF.DBF)

This  is  the name of the file to be used  as  your  HF

logbook. It is used for all modes, including HF packet.

18.22.3 Path Directory File (LAN-LINK.DIR)

This  is  the  name  of the  file  used  as  your  call

directory.

18.22.4 Brag Tape (LAN-LINK.TXT)

This is the name of the file used as your brag tape.

18.22.5 QBU Upload File Name (QBU-TX.BAT)

This  is the name of the batch file used to initiate  a

QBU.

18.22.6 QBU Download File Name (QBU-RX.BAT)

This  is the name of the batch file used when  a  :QBU:

command is received from a remote station.

18.23 Change LAN-LINK Parameters

This sub-menu allows you to change the LAN-LINK  Parameters,

as described in the following paragraphs.

18.23.1 Change CQ Text

This  feature allows you to change the contents of  the

CQ text message string. This is the message string that

is  transmitted along with the CQ call. Think  of  this

message as a CQTEXT line.

18.23.2 Set the Default Band

The  default  value is 2 for 2 Meters.  Use  the  Meter

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 79

value  for the band to keep the display correct. It  is

displayed in the status window. If you normally operate

on  another  band change the default value  to  reflect

your usual default state.

18.23.3 Set the Default Transmitter Power

The  default value is 10 for 10 watts. It is  displayed

in  the  status  window. If  your  transmitter  uses  a

different  power  level  change the  default  value  to

reflect your usual default state.

18.23.4 Set CQ delay time Seconds

This  option  allows you to change the  time  (seconds)

that the LAN-LINK waits for a response after a CQ  call

until  the  following one is transmitted.  The  default

value (in minutes and seconds) is loaded from the  LAN-

LINK.SYS file.

18.23.5 Set CQ delay time Minutes

This  option  allows you to change the  time  (minutes)

that the LAN-LINK waits for a response after a CQ  call

until  the  following one is transmitted.  The  default

value (in minutes and seconds) is loaded from the  LAN-

LINK.SYS file.

18.23.6 Set Number of lines in a Non Contest CQ Call

This option allows you to change the number of lines in

a CQ call from the default value of 3 to any number you

wish. Note that in the contest mode, this option has no

effect. This parameter is not remembered when LAN- LINK

is terminated, so if you want to use a value other than

3,  you will have to set it each time you operate  this

program.

18.23.7 Set Shut Down Mode (QRT Flag)

This  option  allows  you to customize  the  shut  down

sequence  of the TNC. You have the choice of  QRT,  QRV

and  QRV + CTEXT Toggle as described in  the  following

sections.

18.23.7.1 QRT

Turns  the  TNC into a VHF  Digipeater  and  turns

CONOK  OFF  (CFROM NONE for the PK-232).  In  this

condition the TNC will reject connect requests.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 80

18.23.7.2 QRV

Allows you to stay alive on turnoff. It will  send

MON  OFF  (MON  0 for the PK-232) to  the  TNC  at

shutdown  without touching  CONOK/CFROM  {PK-232},

and  at turn on. You will then have to  initialize

the terminal and Communications Mode after reading

your  mail. Connects should then be logged at  the

time you read the mail, not at the time they  took

place.

18.23.7.3 QRV + CTEXT TOGGLE

This  is  the same as the QRV state  but  CMSG  is

toggled on/off. When stations connect to you while

LAN-LINK  is  active, they will  not  receive  the

CTEXT message. When other stations connect to  you

while  LAN-LINK is not active, they  will  receive

the CTEXT message

18.23.7.4 BBS

This option lets you activate the PBBS in the  TNC

when you close LAN-LINK. It inhibits the PBBS when

you bring LAN-LINK up the next time.

18.23.7.5 SOLO

This option puts the TNC in the SOLO mode when you

shut  down,  so as not to fill the buffer  in  the

TNC.

18.23.7.6 BBS+SOLO

This option combines the two previous ones.

Set this value as follows,

0 = QRT,

1 = QRV,

2 = QRV + CMSG toggle,

3 = BBS,

4 = SOLO, and

5 = BBS+SOLO.

18.23.8 Set Default Users

This  option  allows you to set the maximum  number  of

multiple users that can be connected to your TNC at the

same  time, and to configure the default start up  mode

of the program. If this parameter is set to 1, LAN-LINK

will configure itself to the single user mode, and  set

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 81

the maximum number of users to 9. If this parameter  is

set  to any number other that 1, LAN-LINK  will  assume

that  value  to  be  the  maximum  number  of   allowed

connects,  and  configure itself in the  Multiple  user

Mode (equivalent to the TNC Menu Option 0).

18.23.9 Set UTC Offset

This parameter contains the difference in hours between

UTC  and  your  local time. For  example,  if  you  are

located on the east coast of the USA, and it is winter,

you are in the EST which is 5 hours behind UTC. In this

case,  the  value  of the UTC Offset  parameter  is  5.

Don't  forget  to  change  this  when  the  clocks  are

adjusted each spring and autumn.

18.24 Change PC Communications Parameters

This  sub-menu  allows  you  to  change  the  Communications

Parameters  between the PC and the TNC, as described in  the

following paragraphs.

18.24.1 The Communications Port

This  is the default Communications port.  The  default

value  is 1. You may set it at any value between 1  and

4. If you only have one serial port in your PC, set the

parameter  to  '1' even if your card is  configured  as

Com2.  LAN-LINK  will ALWAYS assume Com1 for  a  single

card.  If you use a different port instead of Com1, and

plan to do binary transfers, don't forget to  customize

the QZ*.BAT files.

The Com3 and Com4 port addresses are as follows.

Port Address   IRQ

Com3  3E8       4

Com4  2E8       3

18.24.2 The Terminal Baud Rate

This Parameter tells LAN-LINK what baud rate the TNC is

set  to.  This  is the baud rate  on  the  RS-232  line

between the TNC and the computer. Valid baud rates  for

LAN-LINK are 300, 1200, 2400, 4800, 9600 and 19,200. If

you  specify  something else LAN-LINK will  default  to

1200.

You  may  experience  problems when  using  baud  rates

greater than 1200. It depends on the CPU clock rate and

the  I/O  card in the PC. LAN-LINK scans  the  incoming

text constantly, something which requires time. My  PK-

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 82

232, when used with my XT Clone at 10MHz, does not like

baud  rates  faster than 2400, and sometimes  not  even

that.  The PK-232 seems to drop the first character  of

some lines at 4800 baud. Use 1200 until you have tested

and are sure that a faster one will work.

This  program has been tested on a TNC2 and an  MFJ1278

using  a  terminal baud rate of 9600  baud  and  worked

correctly.  It also works correctly with a KAM at  2400

Baud.

If  you use a different rate instead of 1200, and  plan

to  do binary transfers, don't forget to customize  the

QZ*.BAT files.

18.24.3 TNC Data Bits

You can set the number of data bits on the RS-232  line

between  the computer and TNC to 7 or 8. Use 7  with  a

TNC 2 if you have problems with Greek characters.

Set this value to either 7 or 8.

18.24.4 TNC Parity

Use this parameter to set the Parity bit on the  RS-232

line between the computer and TNC as described below.

0 = None, 1 = Odd, 2 = Even, 3 = Mark, 4 = Space.

18.24.5 Number of TNC Stop Bits

You can set the number of stop bits on the RS-232  line

between  the computer and the TNC to 1 or 2. Use  1  at

all terminal (not packet) baud rates except 300.

Set this value to 1 or 2.

18.25 Change Colors

This option lets you change the default colors. A display of

all  of  the combinations is shown on the left side  of  the

screen.  You will then be prompted to sequence through  each

window/message  color  that  can be  customized.  Enter  the

number  shown  in  the combination window  followed  by  the

'Enter'  key  to set a value. If you don't wish  to  make  a

change, just press the 'Enter' Key.

The following identifications are made to the color choices:

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 83

18.25.1 OutWindow Color

This  is the color of the text (typed at the  keyboard)

in the outgoing window.

18.25.2 StatusWindow Color

This is the color of the Status Window.

18.25.3 PromptWindow Color

This is the color of most of the menus.

18.25.4 LogWindow Color

This is the color of the text in the log window.

18.25.5 InWindow Color

This is the color of the text in the window showing the

incoming characters.

18.25.6 ScrollBackWindow color

This  is  the  color of the text  in  the  Scroll  back

window.

18.25.7 MHWindow Color

This  is  the color of the text in  the  monitor  heard

calls (MH) window.

18.25.8 Alarmwindow Color

This  is the color of the text in the Error  and  Alarm

window.

18.25.9 BottomWindow Color

This is the color of the text in the prompt line at the

bottom of the screen.

18.25.10 Emphasis Color

This  is the color of the emphasized text in  the  Menu

windows.

18.25.11 Prompt Color

This  is  the  color of the prompt  text  in  the  Menu

windows.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 84

18.25.12 Option Color

This  is the color of selected option text in the  Menu

windows.

18.25.13 Connected Text Color

This  is the color of the text in the  incoming  window

when connected in the packet communications modes.

18.25.14 Computer Color

This is the color of the computer generated text in the

outgoing  window when the Command Echo Flag  option  is

enabled.

18.25.15 VHF Header Color

This  is  the  color of the VHF  Header  lines  in  the

Incoming window when not connected.

18.25.16 HF Header Color

This  is  the  color  of the HF  Header  lines  in  the

Incoming window when not connected.

18.25.17 Alert Call Color

This is the color of the packet header lines containing

the  Alert  call  in  the  Incoming  window  when   not

connected.  It is also the color of  the  non-permanent

Parameter Menu prompts.

18.25.18 Outgoing Text Color

This  is the color of the outgoing text echoed  in  the

Incoming window in the AMTOR Communications Mode.

18.25.19 Multi I/O Stream Color[1]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 1.

18.25.20 Multi I/O Stream Color[2]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 2.

18.25.21 Multi I/O Stream Color[3]

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 85

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 3.

18.25.22 Multi I/O Stream Color[4]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 4.

18.25.23 Multi I/O Stream Color[5]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 5.

18.25.24 Multi I/O Stream Color[6]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 6.

18.25.25 Multi I/O Stream Color[7]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 7.

18.25.26 Multi I/O Stream Color[8]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 8.

18.25.27 Multi I/O Stream Color[9]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 9.

18.25.28 Multi I/O Stream Color[10]

This  is  the color of the text lines in  the  Incoming

window  in the Multiconnect Mode when connected to  I/O

stream 10.

NOTE that LAN-LINK does not overwrite all existing  windows.

Thus  if you want to be sure of seeing the  changed  colors,

save  the  color changes using the Update option,  and  then

exit from and reload LAN-LINK.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 86

18.26 Change SAREX Parameters

This sub-menu allows you to change the SAREX Ground  Station

parameters, as described in the following paragraphs.

18.26.1 SAREX CallSign

This is the callsign of the SAREX station.

18.26.2 Change SAREX Capture to Disk File

This  option  lets  you change the name  of  the  SAREX

Capture-to-disk  file.  Don't change file  names  while

capture-to-disk  is  in progress (flashing [S]  in  the

status  window), or the old one will stay open and  may

not close properly when LAN-LINK is terminated, result

ing in data loss.

18.27 Update Configuration File

Use  this  option to save the changes (permanently)  in  the

configuration file (LAN-LINK.SYS). Activate it twice to  see

most of the changes.

18.28 Verify Changes

This feature lets you display how LAN-LINK was configured by

the  LAN-LINK.SYS  file. It also displays the state  of  any

flags  you have set/reset if they are not already  shown  in

the Status window or Parameter Menu. YOU SHOULD USE IT AFTER

EDITING  THE LAN-LINK.SYS FILE TO MAKE SURE that  EVERYTHING

IS CORRECT.

18.29 Change Scan Words

In  the ZAP BBS feature you can ask the program to scan  the

lines read from the BBS and if it sees any of the scan words

in  the line, it will queue a request for that  bulletin  by

number.  You  can  only set 16 such Scan  words  (should  be

enough) and request up to 63 messages/bulletins at a time.

Use the 'Delete' key to delete a word from the list.

NOTE  that  words that show up in the BBS  prompt  line  are

ILLEGAL. For example if your BBS prompts you with

N4QQ BBS>

then 'N4QQ' and 'BBS' should not be used as Scan Words.

DON'T  LEAVE  A BLANK LINE IN THE SCAN WORD SECTION  OF  THE

CONFIGURATION FILE.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 87

18.30 Turn Misc. Flag ON/OFF

The Miscellaneous Flag performs several different  functions

in the different Communications modes as follows.

18.30.1 All Modes:- Debug or state-sequence numbers

Shows  debug  or state-sequence numbers  in  left  hand

corner  of the status window. These are used to  figure

out what went wrong in the automatic sequences.

18.30.2 BAUDOT Mode:- Navy MARS Protocol Verify

When  the flag is set, and the BAUDOT Navy MARS  option

is  enabled,  the  character  string  ':CR:'  will   be

displayed  in the outgoing window each time a  carriage

return  character is received in the  incoming  window.

Similarly,   every  time  a  line  feed  character   is

received,  a ':LF:' will be displayed in  the  incoming

window.

You may use this feature to verify that another station

is sending files using the correct protocol. If you do,

make  sure that you tell the PK-232 not to add  a  line

feed to incoming carriage returns by setting 'ALFD OFF'

in  the  Command Mode. You will have to set  'ALFD  ON'

again  when  you  change communications  modes  or  all

incoming lines will overwrite each other.

18.31 Change Band/Mode

This  option is used to set the band and radio mode  without

invoking the Log Menu. You use it when changing from  packet

vhf to an hf mode.

18.32 Zap BBS on Next Mail Beacon

This  parameter causes LAN-LINK to Zap your local  BBS  next

time it beacons with a list of calls for which it has  Mail.

It is the same as the option described in Section 7.13.

18.33 BBS Zap, Don't Disconnect After Sequence

This  parameter  stops LAN-LINK from issuing  the  automatic

disconnect  when  the Zap sequence is over. You can  set  it

during  the Zap sequence. Use it to stop the  disconnect  if

the sequence gets out of sync with the messages, or if there

is  something  interesting you wish to do to the  BBS.  This

parameter resets when the Zap sequence terminates.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 88

19.0 Q Codes [NC/L]

This  option  displays  help information about  the  use  of  the

Network Communications Language (NC/L) "Q" codes.

20.0 Communications Modes

This  menu  allows you to change the communications mode  of  the

TNC.  If  your  TNC hardware cannot support  those  modes,  don't

bother  to try them. This is software not hardware, and in  spite

of  what  people  tell you, software  can't  do  everything.  The

following communications modes are available.

V VHF PACKET

H HF PACKET

A AMTOR (STANDBY RECEIVE)

B BAUDOT (RTTY)

C MORSE (CW)

M AMTOR (ALIST [chirpcopy])

S ASCII (RTTY)

G SIGNAL

N NAVTEX

Type  in the single letter to make your choice. Don't  forget  to

select the correct Radio port on the PK-232.

20.1 VHF PACKET

This  is  the  1200  baud VHF  Packet  Mode.  LAN-LINK  will

optimize the TNC link parameters for this mode.

20.2 HF PACKET

This is the 300 baud HF Packet Mode. LAN-LINK will  optimize

the TNC link parameters for this Mode.

20.3 ASCII (RTTY)

This is the ASCII RTTY Mode. LAN-LINK will configure the TNC

parameters for this Mode at 110 baud.

20.4 BAUDOT (RTTY)

This  is the BAUDOT RTTY Mode. LAN-LINK will  configure  the

TNC parameters for this Mode at 45 baud.

20.5 MORSE (CW)

This  is  the  CW  Mode. LAN-LINK  will  configure  the  TNC

parameters for this Mode at 20 wpm (10 wpm for the KAM).

In this Mode the ALT-D Key will disengage the receive  speed

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 89

lock  inside the PK-232 to facilitate locking  onto  another

signal  and  Function Key 8 (F8) will turn  the  transmitter

off.

LAN-LINK  will  set the XMITOK parameter in  the  PK-232  to

"OFF" when you enter the CW communications mode, and turn it

"ON"  again  when you call up any other  communication  mode

FROM THE COMMUNICATIONS MENU.

20.6 AMTOR (STANDBY RECEIVE)

When  you select the AMTOR Standby Mode you are placed  into

the  'Standby  Receive' Mode. If you are  in  AMTOR  Standby

Mode,  you  will  not  be able to  display  signals  of  the

Chirpcopy  Mode  (which means that you will not be  able  to

display a QSO in progress).

The  Alt-D  Key or Function Key 8 (F8) toggles  between  the

AMTOR ARQ and the ALIST communications modes.

20.7 AMTOR (ALIST [chirpcopy])

If  you  want to read the mail on an existing AMTOR  QSO  in

progress, select the AMTOR ALIST or Chirpcopy Mode.

The  Alt-D  key or Function key 8 (F8) toggles  between  the

AMTOR ARQ and the ALIST communications modes.

20.8 SIGNAL

Use the 'G' option to select the 'SIGNAL' Mode. Read the PK-

232  documentation  carefully to understand  how  this  mode

works. Once the PK-232 has identified the digital signal  it

is  tuned  to,  use the Communications Mode  Select  Menu  a

second  time  to select the relevant mode and then  set  the

baud rates and shift to display the data if necessary.

20.9 NAVTEX

Use the 'N' option to select the 'NAVTEX Mode. Read the  PK-

232  documentation  carefully to understand  how  this  Mode

works.

21.0 Terminal Mode Menu

This menu allows you to set the Terminal Configuration Mode.  The

Terminal  Menu  provides  different choices  depending  on  which

communications mode you have selected.

21.1 Packet Terminal Menu

The  Packet Terminal Mode of operation will be shown in  the

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 90

Status  window at the top of the screen. If you are  reading

the mail on a station, the callsign of that station will  be

shown  next  to  the word "MAIL". If you  are  connected  to

someone,  the call (and path) will be shown prefixed by  the

"*>" sequence.

In  the Packet Communications Mode, you are  presented  with

the following choices.

B Change Packet Baud Rate

C CQ/Beacons

E Everything

O Turn Metabeacon ON/OFF

M Read the Mail

S Just My Station (Solo)

T All Traffic

0 Multiple User Connects

1 Single User Connects

F Conference

R Conference Bridge On/Off

Type in the single letter to make your choice.

21.1.1 Change Packet Baud Rate

In  the  PACKET Mode you may change the  RF  baud  rate

between 300, 600, 1200 and 2400 baud.

21.1.2 CQ/Beacons

In this Mode, you will see CQ and BEACON packets on the

channel.  (This  corresponds to 'MONITOR 1'on  the  PK-

232).

This Mode only works on the PK-232 and the KAM.

21.1.3 Everything

You  can  set  the TRAFFIC  Mode  to  'Everything'.  It

corresponds to 'MONITOR 6' on the PK-232.

21.1.4 Read the Mail

You can read the mail on a BBS or other station and get

bulletins without connecting to that station  yourself.

You  use this Mode to do so. You will be  prompted  for

the call whose packets you wish to monitor. Remember to

enter the call of the station sending the packets,  not

the  call of the one receiving them. (This  corresponds

to  'MONITOR  0' and 'MBX' callsign).  This  Mode  only

works without showing the packet headers on the PK-232.

On the other TNCs the headers will also be displayed.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 91

You  can  also use this command to monitor  both  sides

(read the mail) on a QSO on a busy packet channel.

You may want to turn the Digipeat Data Display Mode off

when reading the mail.

21.1.5 Just My Station (Solo)

In  this Mode, you will only see messages addressed  to

you. You will only get messages from people who connect

to  you. (This corresponds to 'MONITOR 0'  or  'MONITOR

OFF').

21.1.6 All Traffic

In  this  Mode  you will see all  of  the  data  packet

traffic on channel. You can use this Mode to check that

the TNC is working. (This corresponds to 'MONITOR 4' or

'MONITOR ON').

21.1.7 Single User Connects

This is the normal one-on-one Mode of operation ('USERS

1').

21.1.8 Multiple User Connects

When  you are in a Multiple Connect Mode,  an  asterisk

will  be  shown in the status window next to  the  word

'PACKET'. Each I/O stream will show up in the  Inwindow

in a different color.

LAN-LINK   supports   two   multiple   connect   modes,

Individual and Conference.

An  extract  of  an actual 'off the  air'  CRT  display

(showing  received text only) in a multi-user  mode  is

shown below. Note each line of text as received by  the

TNC  is prefixed (by the PK-232) with the  I/O  channel

number and callsign of the station its connected to.

1:4X4IL: *** CONNECTED to 4X4IL via 4X4IC, 4X6JP

0:4X4IC: joe do u read?

2:4Z4ZB: *** CONNECTED to 4Z4ZB

1:4X4IL: Look for me on PR via 4X6JP 73s and Shalom.

2:4Z4ZB: GE JOE I WILL DIS THEN U CAN GO VIA THE REP>

1: *** DISCONNECTED: 4X4IL via 4X4IC, 4X6JP

0:4X4IC: UNPROTO CQ

2:4Z4ZB: GO AHEAD BOYS...DE ALON..>

0:4X4IC:

2: *** DISCONNECTED: 4Z4ZB

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 92

This  is the normal Multiconnect Mode as  described  in

the  TNC  manual. Here you are connected to  up  to  10

stations  and  will send different traffic to  each  of

them.  Each  time  you  wish to  send  something  to  a

particular station, you must select the I/O channel the

station  is  connected  on before typing  the  text  or

sending the file.

The  stream  change character for all TNCs in  the  VHF

multiple connect modes is '|' ($7C). In the KAM, it  is

'\' on HF and '|' on VHF. They are both on the same key

but  one is in the shift case. Be careful on HF as  the

'\'  shows  up  in file directories  and  confuses  the

computer. THESE VALUES ARE HARD CODED INTO LAN-LINK. IF

YOU  USE SOMETHING ELSE, THE COLOR AND  OTHER  MULTIPLE

CONNECT FEATURES WILL NOT WORK CORRECTLY.

You  select  the I/O channel using the  Alt-N  key  and

moving  the cursor to the desired stream, or by  typing

the '|' character followed by (in the TNC) the  channel

letter/number.  For example to do it manually,  if  you

are connected to G8BTB on I/O channel 0 and to G3ZCZ on

I/O  channel  1, then to talk to G8BTB you  would  type

'|0'  before any text, and similarly to send  something

to  G3ZCZ you would type a '|1' before the text  you're

going  to send me. If you are not using a PK-232,  read

the  TNC  manual to determine if the  I/O  streams  are

identified by numbers or by letters. The TNC2  standard

configuration uses letters (|A etc.).

Use  the Alt-N key and move the cursor up and  down  to

select the I/O channel automatically.

21.1.9 Conference

In  the Conference Mode on the other  hand,  everything

that   you  type  at  the  keyboard  is   automatically

transmitted  to  each station that  you  are  connected

with. Thus if you are linked to two stations each  line

will  be packeted twice by the TNC. You don't  have  to

worry  about  sending  the wrong  thing  to  the  wrong

person, as they will all get the stuff.

You enter this Mode by selecting the 'F' option in  the

Terminal Menu. You terminate this Mode by selecting the

'1' option.

You  connect  in the usual manner using  Alt-C  or  the

Connect  Option in the Call Menu. You MUST WAIT  FOR  A

CONNECTION  TO  BE  ESTABLISHED  OR  TIME  OUT   before

attempting  to connect to the next station. If  you  do

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 93

not, you will get an error message back from the TNC.

In the Conference Mode, as the same text is transmitted

on  all I/O channels, the throughput will  be  reduced.

Slow  down  your  typing speed or  use  short  packets.

Should  you  check the link status  or  call  monitored

list, you may not be able to get back into the Converse

Mode if you have too many packets outstanding. If  that

happens, wait a while and then try to set the  Converse

Mode with Function Key 10 (F10). You may have to try it

a few times, depending on how good your links are.

DON'T TRY TO SEND A FILE IN THE CONFERENCE MODE, IT  IS

NOT DESIGNED FOR IT. KEEP THE PACKETS SHORT.

BE ALSO WARNED that IF YOU USE THE CONFERENCE  (BRIDGE)

MODE  ON  BAD LINKS (SUCH AS CROWDED  HF  CHANNELS  AND

OTHERS WITH LOTS OF RETRIES), THE KEYBOARD MAY SEEM  TO

LOCK  UP,  WHILE LAN-LINK WAITS FOR SPACE  IN  THE  TNC

BUFFER TO OUTPUT THE TEXT.

The  Conference  Mode will only work in the  PK-232  if

DCDCONN  is  set to ON. So if you are using  a  PK-232,

LAN-LINK  will  do  it  for  you  when  you  enter  the

Conference  Terminal  Mode in case  you  forgot  or/and

haven't  set the LAN-LINK.SYS file to include the  line

'DCD  ON'.  At the same time set  LAN-LINK.SYS  default

states  to  show the calls of other  stations  on  each

incoming  line in the Multiconnect modes (read the  TNC

manual on how to do this).

The  Conference  Mode  will only work in  the  TNC2  if

DCDCONN  is  jumpered  so  that Pin  8  of  the  RS-232

Interface  changes  state when you connect  to  someone

else. If you get error messages and see the  characters

'|J'  on  your screen, then the jumper  probably  isn't

set.

The Conference Mode does not work on the TNC1.

In  the  Conference Mode, if any one  of  the  stations

disconnects,  the TNC will return to the Command  Mode,

and you must use Function Key 10 (F10) to return to the

Converse  Mode  before you type  another  character  of

text.

21.1.10 Conference Bridge On/Off

This  option toggles the Conference Bridge on and  off.

In  the  Bridge Multiconnect Mode you are acting  as  a

central switch. You can link together up to 10 stations

in  one multiway connection. In this Mode  any  packets

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 94

arriving on any stream are echoed to all other  streams

that  you have 'connects' on except the one  that  they

came  in  on.  Anything you type will be  sent  to  all

connected streams.

In  this  manner, all stations will not see  their  own

text echoed back. Each line of text output by  LAN-LINK

in  this Mode will contain the callsign of the  station

originating the packet. Your packets will contain  your

call  as  entered in line 1 of the  LAN-LINK.SYS  file.

Thus  for  example,  if  your  call  was  KB8RP/4X  and

normally shows up in the packet header as KB8RP here it

will show in the data as KB8RP/4X.

Just  remember  that  the  Bridge puts  out  a  lot  of

packets, so do multiway connects on a quiet  frequency.

You  must be in the Conference Mode to turn the  bridge

on,  and  any  disconnect will take  the  bridge  down.

Should  that happen, toggle it on again. A bridge  will

be  shown in the status window whenever the  Conference

Bridge is up.

21.1.11 Turn Metabeacon On/Off

This  option lets you turn the LAN-LINK  Metabeacon  On

and Off.

Lan-LINK provides you with a Metabeacon, that gives you

the  capability  to transmit a long beacon  message  at

periodic intervals.  The Metabeacon is loaded from  the

contents of one of the LAN-LINK.00n files. These  files

may  be created by the LAN-LINK editor.  When  loading,

LAN-LINK  will  insert a pass character (^V)  into  the

text  before  a carriage return, so as not to  end  the

packet  before  at least 240 characters, and  not  more

than  255 characters have been output to the  TNC.  The

maximum  number of characters that can be  loaded  into

the  Metabeacon is 1776. The maximum number of  packets

in  a  Metabeacon is 7. You can also end force  a  hard

carriage return or 'end of packet' in the Metabeacon by

inserting the '\' (backslash) character, which will not

be transmitted, into the text.

The  frequency of Metabeacon transmissions is the  same

as that of the automatic CQ timer.

The MetaBeacon is designed for special event  stations.

PLEASE do not abuse it.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 95

21.2 AMTOR Terminal Menu

In the AMTOR Communications Mode, you are presented with the

following choices.

C Force Break [ACHG]

F Start Mode B [FEC]

L Force LETTERS case

M Read the Mail [ALISTEN]

O Override ROBOT/Mailbox

S Standby Receive [ARQ]

R Set ROBOT/Mailbox

W CW Identification

Type in the single letter to make your choice.

21.2.1 Force Break [ACHG]

This  option forces a changeover or 'break in'. If  for

any reason you want to interrupt the other station  you

send a 'BREAK' signal in software this way.

21.2.2 Start Mode B [FEC]

This  option places the terminal into the FEC  Transmit

Mode. This puts you on the air, anything you type  will

be  transmitted.  Function Key 10 (F10) does  the  same

thing.

21.2.3 Force LETTERS case

This  option lets you force the PK-232 receiver to  the

letters case if for some reason you appear to be in the

numbers case. It will return you to the Converse Mode.

21.2.4 Read the Mail [ALISTEN/LAMTOR]

This   option  lets  you  read  the  Mail  on  a   QSO.

Unfortunately, when you tune in a chirping AMTOR QSO in

the  ARQ or Standby Mode you won't get any copy on  the

QSO  in progress. You can however read the mail on  one

by entering this Mode.

21.2.5 Override ROBOT/Mailbox

This  option  lets you temporarily override  the  ROBOT

BEACON  and  chat to a station that has linked  to  the

ROBOT.  Use  it after the message in  LAN-LINK.TXT  has

been  sent, namely when the other station is sending  a

message to you.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 96

21.2.6 Standby Receive [ARQ]

This  option returns you to the ARQ Standby  Mode.  You

must  be in this Mode to receive signals  from  another

station who is calling you. When you tune for CQ calls,

they sound somewhat like fast RTTY with a ringing sound

similar to the connect alarm used by LAN-LINK (but at a

faster ringing rate).

21.2.7 Set ROBOT/Mailbox

This  option  lets  you turn on the  Mailbox  during  a

regular AMTOR QSO so that the other station can leave a

message or download a file remotely. You must only  use

it when the other station is sending text.

21.2.8 CW Identification

This  option is used to send a CW  identification.  The

CWID is sent in the FEC Mode.

The  identification consists of the sequence 'DE  <your

callsign>'  where your call sign is that  entered  into

the PK- 232 as 'MYCALL'.

You  use this feature if your license requires  you  to

identify  in  Morse code as well as  in  the  operating

mode.

21.2.9 Set ROBOT Infinite Delay

You  use  this option to prevent the  Robot  from  ever

transmitting when not linked. In the normal manner, the

Robot counts down the CQ delay parameter, and, when  it

gets  to 0 puts a CQ call on the air. If  the  Infinite

Delay  is  set,  the CQ call will  be  inhibited.  This

option thus puts the Robot into an indefinite wait mode

where  it  thinks  it has sent out a  CQ  call  and  is

waiting for a reply.

You would use this Mode if you want to set up a MailBox

on a known frequency for some group or organization.

21.3 Baudot Terminal Menu

In  the Baudot Communications Mode, you are  presented  with

the following choices.

B Set Baud Rate

F Select Shift

N NAVY MARS Message Protocol

U USOS Off

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 97

S SELCAL On/Off

W CW Identification

Type in the single letter to make your choice.

21.3.1 Set Baud Rate

This feature allows you to set a desired communications

baud  rate directly. You will be prompted to enter  the

desired rate. Use this command to avoid having to shift

through  too many intermediate states, if for  example,

you want to make a direct change from 45 to 100 baud.

21.3.2 Select Shift

This  feature lets you select Wide (850 Hz)  or  narrow

(170  Hz) shift directly. The PK-232 will copy  170  Hz

signals  using  its internal 200 Hz circuitry.  If  you

have  a KAM, you may also select the  Commercial  Shift

(425 Hz).

The shift is displayed in the status window at the  top

of the screen. It is shown as 'W', 'N' or 'C' for wide,

narrow or commercial respectively.

21.3.3 NAVY MARS Message Protocol

The Navy Mars option toggles the Navy MARS Mode on  and

off.  This feature is only present on the PK-232.  When

active, the feature provides the following protocol.

21.3.3.1 Any  Carriage  Return or Enter (CR)  from  the

keyboard is sent as CR, CR. (two sequential CR

characters.

21.3.3.2 In a file transmission lines are terminated by

a CR,CR,LF (Line Feed) sequence.

21.3.3.3 In  a file transmission, blank lines are  sent

as a LF (without a CR).

21.3.3.4 In a file transmission, a plus sign '+' in the

file is replaced by a sequence of 12 "letters"

characters. There can be more than one +  sign

on a line.

21.3.3.5 In a file transmission, the sequence 'NNNN' is

transmitted  followed  by  a  sequence  of  12

"letters"  characters. There can only  be  one

'NNNN' sequence on a line.

When  the  NAVY MARS feature is active a  [N]  will  be

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 98

displayed in the status window.

21.3.4 USOS On/Off

This option toggles the Unshift on space (USOS) feature

on and off. When the USOS is on, the TNC reverts to the

letters   shift  each  time  a  'space'  character   is

received. It helps when displaying noisy copy.

If  USOS is active, 'USOS' will be shown in the  status

window at the top of the screen.

21.3.5 SELCAL On/Off

This command key toggles the SELCAL feature on and off.

When  the SELCAL is activated, as shown in  the  status

window, the incoming text from the TNC is scanned. When

either  the  letters  "ZCZC"  or  'your  callsign'  (as

entered in line 1 of the LAN-LINK.SYS file) is received

the "capture-to-disk" is turned on. All subsequent text

will  be  stored in the LAN- LINK.RUN  file  until  the

letters "NNNN" are received. At that time the "capture-

to-disk" is turned off.

You  may  use this feature when  monitoring  commercial

RTTY  broadcasts to record news and other  information,

or when calling CQ to start recording all incoming text

and keep the QSO on disk.

You may use Function Key 1 (F1) to turn the capture  to

disk  permanently on to override the automatic  turnoff

if you wish.

21.3.6 CW Identification

This  option is used to send a CW  identification.  You

use  this if your license requires you to  identify  in

Morse Code as well as in the operating mode.

The  identification consists of the sequence 'DE  <your

callsign>'  where your call sign is that  entered  into

the PK- 232 as 'MYCALL'.

21.4 ASCII Terminal Menu

In the ASCII Communications Mode, you are presented with the

following choices.

B Set Baud Rate

F Select Shift

S SELCAL set/reset

W CW Identification

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 99

Type in the single letter to make your choice.

21.4.1 Set Baud Rate

This feature allows you to set a desired communications

baud  rate directly. You will be prompted to enter  the

desired rate. Use this command to avoid having to shift

through  too many intermediate states, if for  example,

you want to make a direct change from 110 to 1200 baud.

21.4.2 Select Shift

This  feature lets you select Wide (850 Hz)  or  narrow

(170  Hz) shift directly. The PK-232 will copy  170  Hz

signals  using  its internal 200 Hz circuitry.  If  you

have  a KAM, you may also select the  Commercial  Shift

(425 Hz).

The shift is displayed in the status window at the  top

of the screen. It is shown as 'W', 'N' or 'C' for wide,

narrow or commercial respectively.

21.4.3 SELCAL On/Off

This command key toggles the SELCAL feature on and off.

When  the SELCAL is activated, as shown in  the  status

window, the incoming text from the TNC is scanned. When

either  the  letters  "ZCZC"  or  'your  callsign'  (as

entered in line 1 of the LAN-LINK.SYS file) is received

the  capture-to-disk is turned on. All subsequent  text

will be stored in the LAN- LINK.RUN disk file until the

letters "NNNN" are received. At that time the  capture-

to-disk is turned off.

You may use Function Key 1 (F1) to turn the capture  to

disk  permanently on to override the automatic  turnoff

if you wish.

21.4.4 CW Identification

This  option is used to send a CW  identification.  You

use  this if your license requires you to  identify  in

Morse code as well as in the operating mode.

The  identification consists of the sequence 'DE  <your

callsign>'  where your call sign is that  entered  into

the PK- 232 as 'MYCALL'.

21.5 CW Menu

In  the CW Communications Mode, you are presented  with  the

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 100

following choices.

L Lock to Receive Signal

S Set Morse Speed

U Unlock Morse Speed

Type in the single letter to make your choice.

21.5.1 Lock to Receive Signal

This  option  locks  the PK-232 to  the  speed  of  the

incoming CW signal.

21.5.2 Set Morse Speed

This option lets you set the CW speed.

21.5.3 Unlock Morse Speed

This  option  unlocks the PK-232 from the  incoming  CW

signal.  It performs the same task as the Alt-D  option

keys. It is used to speed up capture of a new signal or

to  break  the  lock when the PK-232  appears  to  have

locked up at the wrong speed.

22.0 eXit to DOS

When  you  exit the program and return to DOS, if the  Shut  down

(QRT) flag is set to 0, the TNC is put into the VHF default  mode

in  which it rejects calls from any other stations.  (Read  about

the  'CFROM'  command in the TNC manual). You can  thus,  if  you

wish,  leave  it  powered, and if you also leave  the  VHF  radio

powered  up, you will be active as a digipeater, but at the  same

time  anyone trying to connect with you will receive a busy  sig-

nal.  The MBX command (Reading the Mail) is also cleared at  this

time.

If you want to leave the TNC set up to accept messages when  LAN-

LINK is not active, customize the shut down state as described in

the Section 18.23.

23.0 Hot Keys

LAN-LINK  also  contains a number of "Hot" keys to allow  you  to

perform common operations with a minimum of key strokes. The keys

perform   related   but   slightly  different   tasks   in   each

communications mode. A list of those available is shown in  Table

23.  Should you require any more hot keys, I suggest you  use  an

external  program  with  a  Keyboard  Macro  capability  such  as

Borland's Superkey.

Consider the action of each key in detail.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 101

Table 23 Summary of Hot Keys

____________________________________________________________________

Hot Key                  Communications Mode

_______________________________________________________________

Packet            AMTOR          BAUDOT/ASCII    CW

____________________________________________________________________

A    Set ALERT Call      ARQ Call/Talk      ------        ------

____________________________________________________________________

B    Send 'Break'        Force Changeover   ------        ------

to TNC

____________________________________________________________________

C    Connect             ARQ Call            Call          Call

____________________________________________________________________

D    Disconnect     Toggle Receive           Receive   Disengage

Modes                   Mode    Speed Lock

____________________________________________________________________

E     Enter  Call     Enter Call and           Enter  Call  Enter

Call

Selcal

____________________________________________________________________

F    Flush TNC Buffer (all communications modes)

____________________________________________________________________

H    Display function key Information Screen

____________________________________________________________________

J    Jump to DOS (all communications modes)

____________________________________________________________________

K    Jump to DOS quick return (all communications modes)

____________________________________________________________________

L    Edit Log Entry (all communications modes)

____________________________________________________________________

M    Scan Message (all communications modes)

____________________________________________________________________

N    Next I/O Stream

____________________________________________________________________

O    Turn Robot ON  Turn Robot ON            ------         -----

-

____________________________________________________________________

P    Turn Printer ON/OFF (all communications modes)

____________________________________________________________________

Q    call QRZ       call QRZ in FEC          call QRZ        call

QRZ

____________________________________________________________________

R    Retry Connect  Retry last Call (entered).

(not ZAP)

____________________________________________________________________

S    Scan Logbook (all communications modes)

____________________________________________________________________

X    Exit to DOS (all communications modes)

____________________________________________________________________

Y    Zap PacketCluster   ----------------    Transmit RY's    ---

--

____________________________________________________________________

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 102

Z    Zap BBS    Drop Link after         Receive mode after

sending contents        sending contents

of buffer               of buffer

____________________________________________________________________

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 103

23.1 Alt-A - Alert Call (Packet) and AMTOR ARQ Call/talk

In  the  Packet Communications Mode,  this  key  combination

performs the same task as the Alert Call option in the  Call

Menu.

In  the  AMTOR  Communications Mode,  this  key  combination

performs the same task as the AMTOR ARQ call/talk option  in

the Call menu.

23.2 Alt-B - Send Break/AMTOR Changeover

In  the  Packet  Communications Mode,  activating  this  key

combination sends a 'BREAK' signal to the TNC. Use this  key

combination to get out of the TNC's transparent mode.

In  the AMTOR Communications Mode, use this key  combination

to  force a changeover. You can type text into the  outgoing

buffer and then force a changeover without losing that text.

23.3 Alt-C - Call/Connect to

This  key  combination performs the same task  as  the  Call

option in the Call Menu.

23.4 Alt-D - Disconnect or Receive

This key combination performs the same task as Function  Key

8 (F8) EXCEPT in MORSE and AMTOR.

In  Morse this key disengages the receive speed lock in  the

PK-232 to unlock the PK-232 from the last signal copied.  It

lets you lock onto another signal faster.

In  AMTOR this key toggles the receive mode between ARQ  and

FEC signals (Qso's in progress and Cq calls).

23.5 Alt-E - Enter Call

This  key  combination performs the same task as  the  Enter

Call option in the Call Menu.

23.6 Alt-F - Flush TNC Buffer

This key combination performs the same task as the Flush TNC

Outgoing buffer option in the TNC Menu.

23.7 Alt-J - Jump to DOS

This  key  combination lets you "Jump to a  DOS  Shell".  It

performs  the  same function as the 'J' option in  the  Main

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 104

Menu.

23.8 Alt-K - Jump to DOS, Quick Return

This  key  combination lets you "Jump to a  DOS  Shell".  it

performs  the  same function as the 'J' option in  the  Main

Menu.  When the command is executed, control is returned  to

LAN-LINK without the 'type exit to quit the shell sequence'.

23.9 Alt-L - Edit Log

This  key  combination performs the same task  as  the  Edit

option in the Log Menu.

23.10 Alt-M - Message Scan

This  key combination performs the same task as the  Message

Scan choice in the Message Menu.

23.11 Alt-N - Next Stream

In  the  Packet Communications Mode, if you  are  using  the

multiple   connect   capability  of   LAN-LINK,   this   key

combination  prompts you to change I/O streams, and  display

the  state of the I/O streams showing which ones  were  last

used  to send or receive something. To change  I/O  streams,

move the cursor to the desired stream then push the  'Enter'

Key.  The  calls  in  the Next  Stream  window  update  when

stations connect and disconnect. In the event of the  window

not  being correct, you may manually enter or  delete  calls

using the 'Insert' Key to enter a call, and the 'Delete' Key

to remove a call.

The data about the streams will be shown in the same  colors

as the multistream text colors.

23.12 Alt-O - Turn Robot On

This  key combination performs the same function as the  Set

ROBOT option in the AMTOR Terminal Menu. The hot key  combi

nation  however,  is  active in both the  Packet  and  AMTOR

communications modes.

This  function  key also puts the Packet and  AMTOR  Contest

operation into automatic.

23.13 Alt-P - Turn Printer ON/OFF

This  key  combination  performs the same  function  as  the

toggle printer on/off option in the Parameter Menu.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 105

23.14 Alt-Q - Call QRZ

This key combination transmits the sequence 'QRZ QRZ QRZ  DE

YOURCALL'  and  turns the transceiver back  to  receive.  In

AMTOR, it sends the sequence in FEC and adds the SELCAL.

23.15 Alt-R - Retry Call/Connect

This  key  combination performs the same task as  the  Retry

choice in the Call Menu.

23.16 Alt-S - Scan Log

This key combination performs the same function as the  Scan

option in the LOG Menu.

23.17 Alt-X - Exit to DOS

This  key combination performs the same task as the Exit  to

Dos  choice  in  the Main Menu. It  exits  the  program  and

returns you to DOS.

23.18 Alt-Y - Zap PacketCluster/RTTY Transmit RYs.

In  the  Packet Communications Mode,  this  key  combination

performs  the same task as the PacketCluster choice  in  the

Call  Menu. It connects you to the PacketCluster and  issues

the 'SHOW/WWV' and 'SHOW/DX' instructions when connected.

In the RTTY communications mode, this key causes a  sequence

of  RYs  to  be transmitted.  Note  that  with  solid  state

equipment  in use in most stations, there should  be  little

need to send RYs during a QSO. They made be used as a tuning

signal.

23.19 Alt-Z - Zap BBS

This  key combination performs the same task as the Zap  BBS

choice in the Call Menu.

23.20 Alt-= - Toggle Miscellaneous Flag

This  key  combination performs the same function as  the  +

option in the parameters menu.

23.21 End - > +? [Over]

In  the Packet Communications Mode this command  sends  your

callsign followed by the > and Line feed characters. You may

use this to signal 'OVER' in a packet contact.

In the AMTOR Communications Mode this command sends the '+?'

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 106

sequence of characters which is the software 'OVER' signal.

In  the  MORSE and RTTY (Baudot  and  ASCII)  communications

modes, this command lets you transmit the 'KN' sequence  and

returns  you  to the Receive Mode. For example  if  you  are

4X6AA and you are working G3ZCZ then you would use  Function

Key  3  (F3) to transmit the sequence 'G3ZCZ de  4X6AA'  and

this key to send 'KN' and revert to receive. You use this at

the END of a transmission in any mode except Packet.

If  you  have typed something into the 'type  ahead  buffer'

then  press  this key, the contents of the  buffer  will  be

transmitted followed by the turnover command.

23.22 PgUp - Scrollback (Scrolls back incoming text)

This  key lets you repeat the display of up to 255 lines  of

incoming text which has scrolled out of the incoming window.

You may print it or capture it to disk during the repeat.

A  small menu displays your options when showing the  scroll

back  information. You may move the menu window up and  down

with  the  left and right arrow keys. Use the 'ESC'  key  to

terminate the Scrollback Mode.

You can save the contents scrollback buffer displayed on the

screen by turning the "Capture-to-Disk' on, and then writing

text  to the window. Anything written will be  captured.  If

the  Capture-to-disk is ON before or during scrollback,  you

will get two copies of the text in the file.

23.23 The Home Key

The  Home Key will clear the window screen that  the  cursor

happens to be in at the time that the key was depressed, and

will then position the cursor at the top left hand corner of

that window.

23.24 The Down Arrow Key

The  Down  Arrow  Key toggles  the  Normal/Reverse  Sideband

command in all PK-232 communications modes except MORSE. The

sideband is depicted in the status window by the Up and Down

arrows. If the Down Arrow is showing, you are set for  Lower

Sideband (LSB) which is the one normally used.

If you have reversed the sidebands a flashing Up Arrow  will

be displayed.

If you set your transceiver to LSB you will be set correctly

for all modes.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 107

For CW operation you may set your rig to the CW position and

ignore  the sideband setting. If you use FSK just set it  so

that it works for you.

23.25 Shift/Baudrate Control Keys

The  following keys are used to control the Shift  (Wide  or

narrow) and the Radio link Baudrate in the BAUDOT and  ASCII

communications modes. In the BAUDOT and ASCII communications

modes  all  baud  rates  supported  by  the  PK-232  may  be

selected.

23.25.1 RIGHT ARROW speeds up the baud rate.

23.25.2 LEFT ARROW slows down the baud rate.

23.25.3 UP ARROW toggles the shift from Wide shift (850

Hz) to Narrow shift (170 Hz) and back.

23.25.4 DELETE toggles the USOS command (RTTY only).

In  the  CW  communications mode, the  left  and  right

arrows change the CW speed by 1 wpm.

The  state of each of the above commands are  displayed

in the status window at the top of the screen. If  USOS

is active, 'USOS' will be shown. The shift is shown  as

'W' or 'N' for wide or narrow respectively.

23.26 Control-Z (^Z)

This  key  combination is used in the  Packet  Communication

Mode  to signal the end of a file or message,  when  loading

messages into another LAN-LINK system or into a BBS. In  the

AMTOR  Communications Mode, it is  automatically  translated

into  the  sequence  '  :EOF: +? '  which  can  be  used  to

terminate  a message being stored in another LAN-LINK  AMTOR

mailbox.

24.0 LAN-LINK Disk Files

The following disk files are associated with the program.

LAN-LINK.COM

LAN-LINK.SYS

*.RUN

LAN-LINK.LOG

LAN-LINK.DIR

CONFIG.SYS

*.MBX

*.BBS

LAN-LINK.001 .. LAN-LINK.010

QBU-RX.BAT

QBU-TX.BAT

LAN-LINK.QTA

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 108

QZ*.BAT

Consider each of them in turn.

24.1 LAN-LINK.EXE

This is the program. You activate it from DOS by typing  the

word  LAN-LINK  followed by ENTER in the  normal  manner  of

invoking DOS commands.

24.2 LAN-LINK.SYS

This  file contains the configuration information  for  both

the program and the TNC. YOU MUST EDIT IT before you run the

program  on  the air the first time. Use  the  customization

options in the parameters menu to make the changes.

The contents of first few lines of the default file together

with an item number are shown below.

1. Callsign

2. Selcal

3. CQ Text

4. Band

5. Power

6. CQ Delay (seconds)

7. CQ Delay (minutes)

8. :QSP: Message Relay Flag

9. PC Serial Communications Port Number

10. TNC Type

11. PC to TNC Baud Rate

12. VHF Log Book File

13. HF Log Book File

14. GMT Offset

15. Path Directory File name

16. Brag Tape File name

17. BBS Snatch Mail Trigger Word

18. QTC Snatch Flag

19. BBS Snatch Flag

20. Shut Down Mode

21. PC to TNC Data Bits

22. PC to TNC Parity Bits

23. PC to TNC Stop Bits

24. DCD Flag

25. QBM Download Subdirectory

26. BBS Read Request Command

27. Local BBS Callsign

28. SAREX Callsign

29. SAREX Capture to Disk File name

30. PacketCluster Call

31. Digipeat Detect Flag

32. Node Drop Link Flag

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 109

33. Sound Flag

34. Display Computer Generated Commands Flag

35. Digipeat PK-232 Date/timestamp in Call Sequence Flag

36. Keyboard Character Beep value.

37. :QBU: Receiving Station Batch file (QBU-RX.BAT)

38. :QBU: Originating Station Batch file (QBU-TX.BAT)

39. Capture-to-disk Flag

40. Beacon Control/LAN-LINK handshake Flag

41. Spare

42. Start of Scan Words (MUST HAVE AT LEAST ONE)

43. * end of Scan Words

44. Start of Common Calls (MUST HAVE AT LEAST ONE)

45. * end of Common Calls

46. Default users

47. Start of Custom Colors

48. Remaining TNC start up Commands.

The  contents of each line are described in the  section  of

the  Parameters menu which deals with customizing them.  The

remaining  lines  contain commands to the TNC.  For  further

information  about  them refer to the TNC  manual.  LAN-LINK

assumes different defaults in some instances to those set in

the TNC. The SYS file contains only those that are different

to the TNC default reset state.

NOTE: If you are not using a TNC1 you MUST set the  HEADERLN

parameter  to  ON. The packet headers and the  text  of  the

packets  must show up on your screen on separate lines.  You

must  also have the "Echo as Sent" feature set ON ('EAS  ON'

for PK-232 and 'XM' ON ' for the KAM).

24.3 *.RUN

These  files  contain incoming data from  the  TNC  recorded

while  the 'capture to disk' function was  active.  LAN-LINK

names  them  as YYMMDD.RUN automatically (YY =  Year,  MM  =

Month, DD = Date, i.e 900903.RUN is a file for 3 Sept 1990).

24.4 *.DBF (HF.DBF and VHF.DBF)

These  file contain the log entries. They are Dbase 3 and  4

compatible.  The format of the logbook file is the  same  as

that  used  by the LOGBOOK package of  the  PC-HAM  software

package, with more space for comments.

24.5 LAN-LINK.DIR

This  is the directory file as described in  Section  6.1.2.

You  use it to store connect, NET/ROM, TheLink,  KA-Node  or

digipeat paths as well as name/callsign relationships.

Examples are

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 110

Joe G3ZCZ

Alon 4Z4ZB V 4X6AA

Milt 4X6AA

LR 4X6LR

hf-il 4x4hf v 4z4zb 4x4il

hf-rj 4x4hf v 4z4zb 4z4rj

K1HTV ELK v WB4APR-5! SNJ3! NNJ! WMA! SCCT! K1HTV

w9tnn-2 jrw !% w3eax-7 !& w9tnn-2

You  create this file with the editor or by using your  word

processor in its non- document mode. You must leave AT LEAST

one  space  character between the first (key) word  and  the

connect  path.  When you type in the key word,  the  program

does not care about upper or lower case.

You may also use the 'reMember Path' option in the Call Menu

to make entries in this file.

If  you already have a YAPP.DIR or a PK232COM.DIR file  just

rename it to LAN-LINK.DIR and add any NET/ROM and/or KA-Node

paths.

NOTE:  All path entries in LAN-LINK.DIR must be on a  single

80 column line.

24.6 CONFIG.SYS

You must have a CONFIG.SYS file on the boot disk if you  are

going  to  use  the disk logging features.  This  file  must

contain the line

FILES = 20

See the DOS manual for more information about the CONFIG.SYS

file.

24.7 *.MBX

These  are the files associated with the stations  you  have

designated  as  LAN  store  and  forward  mailboxes.   Other

stations   can   only  :QRU:  with  you  if   you   have   a

'theircall.MBX'  file on your disk. Thus if for example  you

want  to use 4Z4ZB as a store and forward mailbox, you  will

have to create an ASCII file called 4Z4ZB.MBX using  Editor,

or  the  non-document mode of your word processor.  In  this

file  you will put a list of the call signs of the  stations

4Z4ZB can connect to reliably. If for example you want 4Z4ZB

to  be  able  to store messages from you  to  4X6AA,  4X6LR,

G3ZCZ, ON8IK, F0WN,and G8BTB, you would put those call signs

in the file, one call on a line as follows.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 111

4X6AA

4X6LR

G3ZCZ

ON8IK

F0WN

G8BTB

When  the QRU function is invoked either by you manually  or

by  4Z4ZB  connecting  to you and sending  you  the  command

:QRU:, any message files present on your system addressed to

any of those stations (E.G. G3ZCZ.OUT and ON8IK.OUT) will be

transferred  from you to 4Z4ZB just as if you had  tried  to

:QSP:  the  messages manually or as if G3ZCZ  or  ON8IK  had

connected  directly  to you. The file names on  your  system

will be changed to *.OLD.

You  must  create  one file for each  designated  store  and

forward station, ie. 4Z4ZB.MBX, G3ZCZ.MBX etc.

24.8 *.BBS

These are LOCAL-BBS.BBS AUTO-Upload Command files.

Each file is named for the callsign of the BBS that you wish

to access. It must have exactly the name as the callsign  of

the  BBS.  Thus if the BBS is W9TNN-1 the name of  the  file

must  be W9TNN-1.BBS, or if the BBS is 4Z4ZB then  the  file

must  be called 4Z4ZB.BBS. The file must be an  ASCII  file.

that  means  the  word processor must be used  in  the  non-

document mode to create it.

This file contains the command sequence that you would  have

typed  at  the  keyboard  if  you  were  accessing  the  BBS

manually.  The ONE difference is that the text of a  message

you are uploading with the S command must be followed by the

character  string :EOF: (on a separate line) as  illustrated

by the example below.

The  contents of A TYPICAL *.BBS file are shown below as  an

example.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 112

k 1234

d \net\lan-link.doc

s g8btb

Test Message

This is a test message.

:EOF:

sp G3ZCZ @ N4QQ

Greetings

The text of this message is greetings from LAN-LINK.

/ex

lm

r 567

r 593

24.9 LAN-LINK.001 - LAN-LINK.010

These  are 10 different ASCII files with fixed  names.  They

can be transmitted using the Alternate function keys.  Their

contents can be viewed using the Shift function keys. In all

cases  the  function  key  to  use  for  a  particular  file

corresponds  to the number in the file suffix. For  example,

use Alt/Shift Function Key 3 (Alt-F3) to transmit/view  file

LAN-LINK.003. You may also use the Edit Menu to  edit/create

the files.

In  case you forget what you put in each of the files,  LAN-

LINK allows you to take a peek into all of them at the  same

time. Use Control Function key 1 (Control-F1) to look at the

first line of all the LAN-LINK.001 - LAN-LINK.010 files  and

the Brag Tape.

24.10 QBU-RX.BAT

This  is  the  command  or batch  file  used  in  the  :QBU:

originating station.

24.11 QBU-TX.BAT

This  is  the  command  or batch  file  used  in  the  :QBU:

receiving station.

24.12 LAN-LINK.QTA

You can leave a message on your system for someone else in a

file named CALLSIGN.OUT. When the station with that callsign

connects to you, the message will be sent, and the  filename

will  be changed from CALLSIGN.OUT to CALLSIGN.OLD. This  is

fine  as long as the path between the stations good. If  the

path  is  marginal, it is possible that  your  station  will

transmit  the message and change the filename, but the  link

will  drop out before the message is received at  the  other

station.  If the other station does not connect to  you  and

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 113

:QSM:,  the message may get lost. If this situation  occurs,

you  can tell LAN-LINK that there is a bad path between  you

and  that station, and not to change the file names  without

an  explicit :QTA: instruction from that other station.  You

do  this by putting the call of the station(s) having a  bad

path  in an ASCII file called LAN-LINK.QTA. Put one call  on

each line in the form

N9BE

G8BTB

You  must use the root call of the station without an  SSID.

that  means, if the station is N9BE-4, just put N9BE in  the

file.

Incoming  messages  from stations in the  LAN-LINK.QTA  file

will automatically be acknowledged by a :QTA:.

24.13 QZ*.BAT

As  any LAN-LINK station can either receive or  originate  a

:QZD: and a :QZU: command, LAN-LINK uses four batch files to

control Zmodem transfers. These files are as follows.

QZU-RX.BAT     Batch file implemented after receipt of :QZU:

QZU-TX.BAT     Batch file implemented after transmission  of

:QZU:

QZD-RX.BAT     Batch file implemented after receipt of :QZD:

QZD-TX.BAT     Batch file implemented after transmission  of

:QZD:

Each  LAN-LINK  user must configure the  QZ*.BAT  files  for

their  systems. The port number in the batch file should  be

set to the PC I/O Port used. The data rate in the batch file

should  be set to the data rate used between the PC and  the

TNC, NOT the data rate used on the radio link.

For example, the default QZU-RX.BAT file of

'pcz 1 1200 rz %1' should be changed to

'pcz 2 1200 rz %1' if you are have interfaced your TNC to

Port 2 on your PC, and if, the baud rate between the TNC  is

2400 baud, the file should be changed to

'pcz 2 2400 rz %1'.

Each of the four batch files have to be customized for  each

user who is not using PC Port Com1 and 1200 baud between the

PC and their TNC.  The high level of customization  required

here  is  because different Zmodem  implementations  require

different  sequences of instructions and LAN-LINK  does  not

want to limit you to a specific one.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 114

25.0 Mailbox features

This section describes the mail box and message handling features

built into LAN-LINK.

25.1 Answering Machine

LAN-LINK  contains an AMTOR (Robot beacon CQ mode only)  and

PACKET  Mode  SMART "answering machine"  facility.  You  can

leave  messages on your disk (in the same directory  as  the

LAN-LINK.*  files,  if  you  are using  a  hard  disk),  for

different  stations.  When someone connects to you,  if  you

left  a message for him, he (or she or even it as  the  case

may be) and only that station will receive it automatically.

No one else will normally be able to download that message.

25.2 Mail Beacon (Annunciator)

To ensure that people know that you have left a message  for

them a 'MAIL for' list is loaded into your Packet Beacon and

transmitted every 30 minutes (Refer to the BTEXT command  in

the  TNC manual) as ':QTC:' followed by a list of calls.  If

no  mail is pending, or the only message in your  system  is

one   addressed  to  you,  the  beacon   transmissions   are

inhibited.  This  conforms  to good  operating  practice  on

crowded channels (at least inhibiting the beacon does).

The mail beacon in the AMTOR Mode is transmitted as part  of

the beacon AUTOCQ message. It is automatically updated  when

a  message is transmitted. If you don't set the beacon  (see

below)  you  will not transmit a mail beacon  text  in  your

AMTOR CQ message.

The  Mail  beacon text is set up either when  you  tell  the

computer  to set it up or when you disconnect a  packet  QSO

from another station.

In this manner you may update the list while the program  is

running,  for  example  by using Sidekick  or  the  included

Editor, to write a message, or preferably the NOTE feature.

Any   station   using   LAN-LINK  can   be   configured   to

automatically  attempt  a connect (QTC-Snatch)  when  seeing

their callsign in someone else's LAN-LINK :QTC: beacon  mail

message list.

If  for some reason the message got lost (e.g.  because  the

link  was  marginal), :QSM: will get you a  repeat  of  that

message.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 115

25.3 How to leave a message

You  can use the note feature or type the message as a  non-

document  file on Wordstar, the included Editor, or  if  you

use another word processor, you must leave the message as an

ASCII text file.

If  you  use the Note feature, the message will be  given  a

header as if it came in from someone else. if you use a word

processor  or the included Editor to write the message,  you

will  have  to write in a header or show  somehow  that  the

message is not 'live'. You may then later time tag a message

header  on message files created by the word  processor.  In

this way people will realize that it is an automatic message

by  seeing  the header. On the other hand, you may  wish  to

fool people into thinking that you are home.

You  just  name  the message file as  the  callsign  of  the

station  to whom you wish the message to be sent at  connect

time. You must also give it the filetype of 'OUT'.

For example a message for G3ZCZ would be stored on the  disk

as  a  file named 'G3ZCZ.OUT', and similarly  a  message  to

4X6AA would be stored on disk as '4X6AA.OUT'.

Once the message has been sent, an entry to that effect will

be  made in the LAN-LINK.RUN data capture file. The  message

file  will  then  be renamed and given  the  file  extension

'.OLD'. At that time any previously existing mail file  with

the  file extension '.OLD' will be deleted. This  stops  the

addressee  receiving the message on successive  connects  if

you  can't delete it for some reason (you forget)  while  at

the same time, the message is still available to you in case

you need it.

You  should use a separate utility program if you  want  the

calls in the mail beacon sorted. You may for example wish to

sort them alphanumerically or in the order of the date  that

they were left in your answering machine.

Incoming  messages triggered by the QTC-Snatch will be  left

in a message file '<yourcall>.OUT', which can be scanned  by

using  the Alt-M Key combination. If you have  non  LAN-LINK

friends, train them to trigger the QTC-Snatch by  simulating

a  beacon,  and prefixing and ending their text  with  lines

containing  the '>' as the only character. Better yet,  give

them a copy of LAN-LINK and tell them to register it.

25.4 Remote Beacon Shutdown

I recognize that some stations may abuse the beacon facility

of  this  program.  I have thus built  into  the  program  a

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 116

'remote  shutdown' capability. If someone  running  LAN-LINK

has their beacon timer set too often, connect to them in the

AMTOR or Packet Communications modes, and tell them to :QRT:

which will clear their beacon mode and take them off the air

for a while at least.

In the Packet Communication Mode, the CQ beacon is shut down

but  the 30 minute mail beacon is not touched. In the  AMTOR

Communication  Mode, the CQ delay time is set  to  infinity.

The  LAN-LINK  Mailbox  will thus still be  active  in  both

modes, but will not call CQ on the air.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 117

APPENDIX 1. ANOMALIES and BUGS

A1.1 Command/Converse Mode anomalies

Since  the Program has no way to check the command/converse  mode

status  of the TNC, you may see various Cmd: and  error  messages

from  the  TNC  when using the menus or  function  key  commands.

Ignore  them for now. If you get an error message, try using  the

function key again.

A1.2 Buffer messages in AMTOR Beacon CQ Mode

In the AMTOR Beacon CQ Mode, if the other station drops the  link

while  you are recording the text, BEFORE the callsign  has  been

recognized  as  a valid callsign, LAN-LINK will time  out  before

logging  the QSO and restarting the CQ sequence. If someone  else

calls  in  before the time-out has occurred, the text  from  both

stations  should be captured to disk, but the log entry  may  not

show  both  stations, thus the 'LOG' must be  compared  with  the

'RUN' file for QSO purposes.

LAN-LINK in the Beacon/Mailbox Mode, normally logs the contact at

the  end  of the line in which it recognized the  callsign  as  a

valid callsign.

A1.3 FEC +? responses in AMTOR Beacon CQ Mode

In the AMTOR Beacon CQ or Mailbox mode, if another station  sends

a  +? in the FEC Mode, the beacon will attempt to send  the  LAN-

LINK.TXT file. The PK-232 however is not in the Converse Mode  at

that time and will interpret the text of the file as commands. If

by  some chance, the text corresponds to a valid command the  PK-

232  will execute it and put the PK-232 in an unknown  state  and

you  may  not notice that it did. As such I  recommend  that  the

first  character of each line in the file LAN-LINK.TXT be  a  non

valid  command  character  such as a number or a  period  (.)  or

another punctuation sign.

A1.4 Using a CGA card with a monochrome monitor

If  you are using a CGA card with a monochrome monitor, the  text

you  type  into the OUTWINDOW will be fainter than  the  incoming

text.  This  is because of the colors used. If you  are  using  a

Hercules  compatible  card you will not  notice  any  difference.

Change the colors using the Parameter Menu.

A1.5 The Packet Conference Mode

The  Conference  Mode has been tested on a PK-232, KAM,  a  SANYO

MBC-675  which  has a clock speed of 4.7MHz and other  PC  clones

with faster clock speeds. If you have problems with  intermittent

selection of I/O channels try slowing down your CPU clock.  There

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 118

is a software delay loop in the program that is used to allow the

TNC time to switch I/O streams after receipt of the stream change

command. This delay is there because LAN-LINK tests for a connect

on  each channel before outputting any text, so that it does  not

send unaddressed packets on non connected I/O channels.

A1.6 Packet Conference Mode disconnects

A  Packet  conference  Mode disconnect returns  the  TNC  to  the

Command  Mode, even if you remain connected to one or more  other

stations.

A1.7 Strange Events

If  the TNC does not use an interrupt driven serial port you  may

experience problems when using baud rates greater than 1200.  The

PK-232  DOES  NOT LIKE LAN-LINK to use RS-232 baud  rates  faster

than  2400, and sometimes not even that. Use 1200 until YOU  have

tested and ARE SURE that a faster one will work.

IF YOU HAVE ANY STRANGE PROBLEMs AND ARE USING an RS-232 TERMINAL

BAUD RATE to the TNC WHICH IS GREATER THAN 1200, set the Terminal

Baud Rate to 1200 and see if the problem goes away.

Make sure that your configuration is set the way you think it is.

Verify  it  using the 'display deFault' (and the  Misc.  Flag  if

necessary) option(s) in the Parameter Menu.

You  must have at least one common call or scan word in your  SYS

file, or you will get errors when you update the SYS file.

A1.8 *** DISCONNECTED

In  the  Packet Communications Mode, when you  are  connected  to

someone and the TNC sends this text string to the computer,  LAN-

LINK thinks you have been disconnected and resets. This  normally

only happens when a true disconnect occurs. However, if the  text

shows up in the middle of a line being sent to you, it WILL  HAVE

THE SAME EFFECT AND THINK that YOU HAVE DISCONNECTED if the  DCD-

Flag is not set in the SYS file, and the TNC does not use the DCD

signal on pin 8 of the RS-232 interface.

A1.9 *** CONNECTED

In  the  Packet  Communications Mode,  when  you  are  monitoring

traffic  on  the  LAN someone sends this  text  string,  LAN-LINK

thinks you have been connected if the DCD-Flag is not set in  the

SYS file, and the TNC does not use the DCD signal on pin 8 of the

RS-232 interface.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 119

A1.10 Greek Characters and other garbage in the Incoming Screen.

If  you see Greek characters in the Incoming screen you  probably

have  the wrong parity set in the TNC. first make sure  that  the

RS-232 baud rate is correct. If it is, take the batteries out  of

it  and reset it. If that fails, play with the  parity.  LAN-LINK

sets  the serial port as configured in the TNC.SYS file for  each

TNC. For example, for the PK-232 it is 8 bits, no parity, for the

TNC2  it  is 7 bits. If you still can't get it to work,  set  the

interface to 7 bits in the configuration file.

A1.11 Dumb TNC

If  the  TNC  seems to respond to commands  but  is  not  sending

anything  back, you may have one of two problems. You may have  a

bad  RS-232 cable with on wire disconnected. If the computer  can

operate  the TNC in another communications program, try to  reset

the TNC, take the batteries out, etc. You may also have a not-so-

compatible PC.

A1.12 Intermittent Lockup

Watch out for flow control problems on the cable between the  TNC

and the PC and between the PC and the Display.

Versions  of  LAN.LINK  upto and  including  1.55  used  software

handshaking.  Anytime  the  program  received  a  Control-S  (^S)

character from the TNC it waited until it subsequently received a

Control-Q  (^Q).  These  characters can appear on the  LAN  in  a

binary packet, either when binary file transfers are in progress,

or  in  a  Inter NET/ROM message. To avoid this  lockup,  a  line

containing  'MFILTER 19' was added in your LAN-LINK.SYS  file  so

make  the  TNC  filter the ^S character out and not  pass  it  to

LAN.LINK.  LAN-LINK 1.56 and onward use hardware  handshaking  so

this  problem  should not appear. However if the FLOW  and  XFLOW

parameters are not set to OFF in your TNC, LAN-LINK can appear to

lockup. Make sure that they are both set OFF in your TNC.

APPENDIX 2. Local Area Network Capability

VHF  Packet  radio systems can be considered as part of  a  Local

Area  Network (LAN) in which messages can be left by one  station

in the computer belonging to a second station. At HF the same  is

true,  but  the area becomes greater. The basic problem  here  is

that  people  can only send and receive messages to or  from  you

when  you are on-line. To compensate for this, BBS stations  were

developed which allowed both messages and bulletins to be  stored

by anyone for later retrieval.

LAN-LINK  allows you to store messages in your computer so as  to

use it as an automatic answering machine. By storing the messages

in a file called by the name of callsign, with the file extension

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 120

'.OUT', anyone connecting to the computer will get their  message

without having to request it. LAN-LINK also extends that  concept

to  cover  the eventuality of you wanting to take  your  computer

off-line  for some reason. You may now load the message into  any

other  computer you can connect to using elements of the  Q  code

adapted to the High Level Network Communications Language  (NC/L)

proposed  in  my book Software for Amateur  Radio,  (Book  number

1560) published by TAB Books, Blue Ridge Summit, Pa. 17214 U.S.A.

A2.1 LAN Protocol (G3ZCZ Version)

The  protocol  is as follows. When connected to someone  who  has

their  LAN-LINK  configured  as a host, if you want  to  store  a

message  you send the following instruction to the other  station

:QSP: <callsign> where <callsign> is the call of the station that

the message is for, not the callsign of the host station in whose

computer  you are storing the message. [Note use only  one  space

character  after  the :QSP:]. LAN-LINK sends  the  :QSP:  message

automatically using QSP option in the Message Menu.

For  example if you want to store a message for 4Z4ZB in  4X6AA's

computer  which is configured as a Store and Forward system,  you

would first connect to 4X6AA and then send the command as

:QSP: 4Z4ZB .

Better  still use :QSP: option in the Lan-Link Menu  to  automate

the sequence.

The computer at 4X6AA will respond either with a statement saying

that  it is ready for you to go ahead, or send a  message  saying

that  it  can't comply. If it is ready you get a  positive  reply

which will take the form :QRV: <callsign> which if you know the Q

code, means " I am ready to accept a message for <callsign>".

At  this time you may go ahead and send the message. If you  type

the message at the keyboard, you may use either a control Z  (^Z)

character or the character sequence :EOF: followed by a  carriage

return  (the  ENTER key) to terminate the message.  If  you  have

first  written the message into a text file you may then send  it

using  the  'Send  File'  option  of  the  Files  Menu  and   the

termination sequence is automatic.

Once  you have completed the message, the other  (host)  computer

will  either reply that the message has been successfully  stored

or give you an error message.

If  the  message  is stored and ready to be sent  next  time  the

addressee  connects  to that computer, you will see  the  message

:QSL: on your screen. If something went wrong, you will get  back

a  negative message taking the form :QNO: followed by  a  number.

The number tells you why the operation failed.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 121

A2.2 Message Format

The  message is stored just as if you had left it in your  system

(except that a header is added identifying the time of  reception

and  the call of the sending station). It will contain  a  header

showing  the  date and time it was received  and  your  callsign.

Should a message for that station already be in the system, yours

will be appended to it. In the event the your upload is  aborted,

the  amount  of text received before the abort occurred  will  be

stored as the message.

When  you  disconnect from the other LAN-LINK host  station,  its

mail beacon will be updated.

Once the message is loaded in the host, it can only be deleted by

the operator of the host station. When the addressee connects  to

the host and receives the message, the file type will change from

'.OUT'   to   '.OLD'.   Note  QTA   stations   require   positive

confirmation  of  receipt  of message for the  renaming  to  take

place.

A2.3 ASCII File Uploading/Downloading

There  comes a time when you want to leave a file on your  system

for  someone to download later. You can do this by using NC/L  to

control  the transfer. You must leave the file in a  subdirectory

defined in the LAN-LINK.SYS FILE called \LAN-LINK\FILES. You must

first  create it if it does not exist. The station who wants  the

message just asks for it using :QBM: <filename>, note they do not

need to use the full subdirectory path. They must however leave a

single  space character between :QBM: and the filename. In  AMTOR

the Mailbox must be turned on for remote functions.

For  example, you have the latest ARRL DX bulletin, and you  want

to  pass it on. You could pass it to selected people  by  copying

the  file to individual messages with the filetype  '.OUT',  e.g.

4X6AA.OUT, 4Z4ZB.OUT or even G3ZCZ.OUT if you want me to get  it,

which wastes a lot of disk space.

On  the  other  hand  you could tell people  that  the  file  was

available  for downloading, either in the CTEXT  connect  message

line  which everyone gets when connecting to you  by  configuring

LAN-LINK accordingly, or in individual ('.OUT') messages.

If you include the following in your CTEXT message

"ARRLDX.015 now available"

people will know that to download it, they just have to type  the

line  ':QBM: ARRLDX.015' (one space character between  :QBM:  and

the  file  name) to get the file. LAN-LINK is not designed  as  a

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 122

BBS,  however,  if you copy the contents of  the  \LAN-LINK\FILES

subdirectory  into  a  file called DIR (USE DIR >  DIR  in  DOS),

anyone  can  get a list of the files you have stored  in  it  for

downloading  by typing :QBM: DIR. Since they do not get the  real

directory, just what was in it at the time you copied it into the

file 'DIR', you may keep "hidden" files in this subdirectory. You

may thus leave a file for someone to download, tell them that its

there   by   leaving  them  a  message  (which  they   will   get

automatically when they connect) and no one else connecting  will

know that it is there.

:QDB: allows you to upload text or ASCII files.

If  you  use the 'lan-linK' option of the Packet  Terminal  Menu,

LAN-LINK will automate the QDB and QBM features for you.

A2.4 Path Determination to a Dx station

If  you want to establish a digipeat path to a  station  somewhat

out of your direct range, you need to know which of the  stations

that you can connect to can hear that desired DX station. If  you

could  get a call monitored (MH list) from the stations that  you

connect  to,  you  would be able to see if the  station  you  are

connected to has heard your desired DX station.

LAN-LINK uses the :QMH: command to request such a list. When LAN-

LINK  receives a :QMH: command, it drops back to the TNC  Command

Mode and issues a 'MH' command to the TNC (except to a TNC1 which

does not support it). It stores the list received in a  temporary

buffer,  and then returns to the Converse Mode and transmits  the

list back to the station it is linked to.

By  judicious  use  of :QMH: you can  determine  paths  to  other

stations.  Note however, that just because one station  can  hear

another  station,  it  does not mean that it  can  work  it.  For

example,  the station you are connected to may be using  a  power

level  of 1 watt or so, while the station 200 miles away that  it

heard was using 100 watts. Test the path yourself, or/and leave a

message  asking how reliable the connect path between  those  two

stations is.

If  you and a small group use a non standard packet  channel  for

inter-group  communications, you may also use the :QMH:  function

to  find  out who the station you are connected with,  has  heard

recently, when you first sign on the LAN channel.

A2.5 Binary File Transfers

A2.5.1 Introduction

In the PC world most binary file transfers seem to be taking

place  using  the  YAPP protocol built into  the  YAPP  user

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 123

terminal  program  and  the BBS  software  written  by  Jeff

Jacobsen, WA7MBL. As Jeff stated in the documentation,  YAPP

was written to provide a terminal program for use in testing

the WA7MBL BBS code.  However for many radio amateurs,  YAPP

was their first terminal program.

A2.5.2 Transfer of Data

The  YAPP binary transfer protocol is similar to the  Xmodem

protocol used on landlines. In order to transmit a file, the

sending  station  breaks  the  file  down  into  blocks   of

characters.  Each block is transmitted in sequence. After  a

block  has been transmitted, the sending station  waits  for

the  receiving station to acknowledge that block and  verify

that it was received without errors.

When  two  stations  are  alone on  a  frequency,  they  can

transfer  data quite quickly. So back in 1986 when YAPP  was

written, it provided the few hams using packet radio with  a

binary  file  transfer  capability.   Today,  however,   the

situation  is  different. In most urban areas,  each  packet

radio  channel  is  occupied by several  stations,  and  the

requirements  for speedy binary file transfers are  somewhat

different.

A2.5.3 Requirements for Transfer Protocols

The  AX.25 protocol provides for a basic  verification  that

packets  have been received error free. Thus if  the  binary

file  transfer  protocol could start sending  blocks,  after

establishing  the connection, then keep sending  data  until

one  of  three  things  happen;  the  link  drops  out,  the

receiving  station  signals  that  some  of  the  data  were

received   with  errors  in  it,  or  the  whole   file   is

transmitted.

If  the whole file is transmitted, then the sending  station

just  has to wait for an acknowledgment from  the  receiving

station that the file was received.  If the circuit is  busy

with  several  stations  on the channel, or  the  path  goes

through  one or more nodes, there will be varying delays  in

the  delivery  time  of  the  packets.  By  not  having   to

acknowledge  each  block  of  data  in  the   communications

software  running in the PC, the file can get  through  much

faster  than it would have in the case in which  each  block

has to be transmitted and individually acknowledged.

If  the link drops out, it would be nice if  the  computers,

could determine how much of the file was received, then pick

up the transmission at that point next time.

If  the  receiving  station  signals  that  some  data  were

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 124

received in error, the sending station should then begin  to

retransmit from the start of the error.

A2.5.4 The Zmodem Protocol

This  situation  is not unique to the amateur  radio  packet

network.   It  is  also present  in  the  commercial  packet

switching environment, and the Zmodem protocol was developed

to  provide the capabilities described above.   Zmodem  thus

seems  to be an almost ideal binary file  transfer  protocol

for   use   on  radio  amateur  packet  channels,   and   is

incorporated in LAN-LINK 1.59.

LAN-LINK  uses the :QZD: and :QZU: commands to  provide  the

capability  to  remotely download and upload  binary  files.

:QZD:  allows you to download a file from  another  station,

while  :QZU: allows you to upload a binary file.   :QZD:  is

configured  so  that  if  the link  drops  outs  during  the

transfer,  when the connection is once again established,  a

subsequent  :QZD:  for  the same filename,  will  cause  the

transfer to pick up where it left off when the link  dropped

out.   This  feature  is not allowed  in  the  upload  :QZU:

command  to inhibit people from injuring files that  already

exist  on the other computer. Each command may be  initiated

from  either end of the link, but to gain the most from  the

recovery  capability,  files  should be  downloaded  by  the

receiving station.

A2.5.5 The PCZ Package

LAN-LINK  implements the Zmodem protocols using batch  files

in DOS that call up external programs. This approach  allows

stations to try different versions of Zmodem and  experiment

to find the optimal one for their area. LAN-LINK comes  with

the  (PCZ)  Public  Domain  implementation  of  the  Zmodem,

Ymodem,  Xmodem,  Xmodem-1K and Sealink  protocols  by  Drue

Kennon and Gary Smith.

A2.5.6 QZ*.BAT Files

While  each  LAN-LINK user can set up binary  file  transfer

batch  files  to implement any of  the  protocols,  LAN-LINK

comes with four default batch files for Zmodem which must be

configured by each user as described in Section 24.3.

As  any LAN-LINK station can either receive or  originate  a

:QZD: and a :QZU: command, LAN-LINK uses four batch files to

control Zmodem transfers. These files are as follows.

QZU-RX.BAT Batch file implemented after receipt of :QZU:

QZU-TX.BAT Batch  file  implemented  after  transmission  of

:QZU:

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 125

QZD-RX.BAT Batch file implemented after receipt of :QZD:

QZD-TX.BAT Batch  file  implemented  after  transmission  of

:QZD:

Each  pair of users can experiment with other protocols  and

Zmodem  implementations using the :QZD: and  :QZU:  commands

after reading the PCZ documentation.  The :QBU: command  can

also be used if the corresponding QBU-RX.BAT and  QBU-TX.BAT

DOS Batch files are created.

A2.5.7 Buffers

LAN-LINK  contains  an  output buffer to  store  data  being

output  to the TNC. The TNC contains a buffer  which  stores

data  about  to  be sent on the radio link.  In  the  normal

course of events, the two buffers fill up so that the Zmodem

package  used  by LAN-LINK may think that all the  data  has

been  transmitted, when in fact there are  still  characters

left in the buffers, or the last packet has not yet made  it

to  the  destination station. In this case, you  may  see  a

message on your screen that the system has sent all the data

and  is waiting for the final acknowledgement, while at  the

same  time  you can see from the LEDs on the  TNC  that  you

still  have outgoing data in your system. Have patience  and

wait a while to let the two systems synchronize.

In  the  event that the link is bad and errors  occur,  this

buffering  may  cause  problems which require  some  of  the

Zmodem   timing  parameters  to  be  changed.  Should   this

situation arise, read the Zmodem package documentation.

A2.5.8 Summary

The  flexibility  offered by the Zmodem  protocol  has  many

advantages  when  used in a packet radio  network.  LAN-LINK

1.59  implements  Zmodem by the use of  an  external  public

domain  protocol  package  which provides  for  the  maximum

degree of user customization.This capability is provided  at

the cost of having the individual user customize four  batch

files using the text editor built into LAN-LINK.

APPENDIX 3 BASIC NC/L DICTIONARY

Messages can be transferred into any other LAN-LINK computer  you

can connect with using elements of the Q code adapted to the High

Level Network Communications Language (NC/L) proposed in my  book

Software  for Amateur Radio, (Book number 1560) published by  TAB

Books, Blue Ridge Summit, Pa. 17214 U.S.A.

NC/L  command words are summarized in the  following  paragraphs.

There is no command word to request your message. To receive your

message, you do nothing, you will receive your mail automatically

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 126

when  connecting/linking  with  a LAN-LINK  station.  You  cannot

normally read messages addressed to another person.

A3.1 :QBM:

To download an ASCII file, send

:QBM: filename.type

The filename.type is the file you want. For example

:QBM: dir

:QBM: help

:QBM: LAN-LINK.doc

A3.2 :QDB:

To upload ASCII files to another LAN-LINK system send

:QDB: filename.type

Files  will be uploaded into the same sub-directory as  LAN-LINK,

and will have a message header inserted at the top of the file to

allow the operator to know who uploaded the file. If the operator

then wants to make the file available for downloading, he or  she

can  move  it to the \LAN-LINK\FILES  sub-directory.  The  '.TYP'

following the filename is optional.

If  you attempt to upload a file which has the same name  as  one

that  already  exists on the remote system, you will  receive  an

'error' message.

This command is the reverse of :QBM:.

A3.3 :QMH:

To request a call monitored list ('MH') from the LAN-LINK station

that you are connected with, send

:QMH:

A3.4 :QSM:

To  request a repeat of a message from a station using  LAN-LINK,

send

:QSM:

This  command will be valid as long as the host station  has  not

deleted the <yourcall>.OLD file on his disk. If the file does not

exist, you will receive a :QNO: 2 'error' response.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 127

A3.5 :QSP:

To leave a message, send

:QSP: callsign

The  callsign is that of the station you wish to leave a  message

for.

A3.6 :QRA:

When  disconnected, to see what stations (using  LAN-LINK  1.53+)

are on the LAN, or which LAN-LINK stations have messages pending,

send

:QRA:

All LAN-LINK stations will reply with a mail beacon within a  few

seconds.

A3.7 :QRT:

To shut down an AMTOR/Packet mode Mailbox/beacon station which is

causing QRM, link or connect to the station and send

:QRT:

A3.8 :QRU:

To  upload messages from one LAN-LINK/PK232COM (1.38+) system  to

your computer, send

:QRU:

You  may  only use the QRU function with stations  designated  as

Store and Forward mailboxes. Put a list of stations that you  can

connect  to reliably in your <callsign>.MBX file and send  it  to

other stations in your LAN also using LAN-LINK. They will then be

able to dump their mail messages on you and you on them. If  they

do  the  same and send you their file, then you will be  able  to

send messages via them for stations that they can work.

While  QRU  gives you the capability to bulk upload  messages  to

another  station in your local area, when you take  your  machine

off  line, it may also be used to transfer messages  between  two

LANs via well sighted gateway digipeaters.

A3.9 :QNO:

':QNO:' and error message text is a response to a request.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 128

A3.10 :QJG: is a response to :QRU:

It confirms that the QRU sequence is complete. There are no  more

messages pending.

A3.11 :QRV: callsign is a response to :QSP: callsign.

The computer is ready to send the message. End the message with a

control Z (^Z) character, or the sequence :EOF:

A3.12 :QSL: is a response to a command

It  confirms receipt of message to that callsign or that  a  file

has  been  successfully uploaded and stored to disk. It  is  also

used to acknowledge receipt of a ':QRT:' command.

A3.14 :QTA: message

If  LAN-LINK  receives  :QTA:  and  a  message  exists  for   the

connecting station, LAN-LINK will change the filetype from  'OUT'

to  'OLD' and respond with a 'QSL'. If a message does not  exist,

it will respond with a 'QNO'.

A3.14 :QTC: message list

If  LAN-LINK receives :QTC: it will respond with the beacon  text

showing the mail queue.

:QTC:  precedes a list of callsigns for whom messages are  stored

up  on a computer. It is used in Packet Beacon  transmissions  or

AMTOR Beacon mode CQ calls.

A3.15 :QBU: External Protocols

This  feature is supplied to allow experimentation with  external

protocols.

A3.16 :QZU: Zmodem Binary File Upload.

:QZU:  is  used  to implement an upload of  a  binary  file  from

another station using an external Zmodem protocol via the QZ*.BAT

files.

A3.17 :QZD: Zmodem Binary File download.

:QZD:  is  used to implement a remote download of a  binary  file

from  another station using an external Zmodem protocol  via  the

QZ*.BAT  files.  In the event that the link drops out during  the

transmission, when the link is remade, and the :QZD: retried, the

transfer will pick up from the point at which it was interrupted,

if the batch files provided are used.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 129

PROPOSED EXTENSIONS

A3.18 :QYU: YAPP format file upload.

A3.19 :QYD: YAPP format file download.

APPENDIX 4. USING OTHER TNCs

The  PACKET Mode capabilities can be used on all TNCs. There  are

minor differences in the command dialogue between different TNCs.

The dialogue differences affecting the operation of LAN-LINK  are

built into the program and set when the TNC type is read from the

LAN-LINK.SYS file at the program load time. The non packet  modes

will obviously result in error messages if attempted on a  packet

mode only TNC.

The   following  notes  apply  to  different  TNCs   or   digital

communications controllers.

A4.1 TNC2 (MFJ 1270) Version 1.1.2

A4.1.1 Default Commands

Rename  the  LAN-LINK.SYS  file  as  PK232.SYS.  Rename  the

TNC2.SYS  file as LAN-LINK.SYS and edit it using  your  word

processor  in its non document mode to customize it to  your

station.

A4.1.2 Program differences

The following functions DO NOT WORK because the TNC does not

support them.

All NON PACKET modes.

Software controlled HF/VHF switching.

Software controlled baudrate switching.

CQ/BEACON Packet terminal mode.

TNC Transmit Buffer flush.

A4.2 TNC1 (HD-4040)

The Packet Communications Mode header colors are not supported in

LAN-LINK for this TNC.

A4.2.1 Default Commands

Rename  the  LAN-LINK.SYS  file  as  PK232.SYS.  Rename  the

TNC1.SYS  file as LAN-LINK.SYS and edit it using  your  word

processor  in its non document mode to customize it to  your

station.

Ignore error messages at start up.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 130

A4.2.2 Program differences

The following functions DO NOT WORK because the TNC does not

support them.

All NON PACKET modes.

Day/Time functions.

'MH' and remote :QMH: functions.

MAIL Packet Terminal Mode.

CQ/BEACON Packet Terminal Mode.

TNC Transmit Buffer flush.

Conference and multiconnect modes.

A4.3 Kaypro KPC-2

The Packet Communications Mode header colors are not supported in

LAN-LINK for this TNC.

A4.3.1 Default Commands

Rename  the  LAN-LINK.SYS  file  as  PK232.SYS.  Rename  the

KPC.SYS  file  as LAN-LINK.SYS and edit it using  your  word

processor  in its non document mode to customize it to  your

station.

A4.3.2 Program differences

The following functions DO NOT WORK because the TNC does not

support them.

All NON PACKET modes.

MAIL Packet Terminal Mode.

CQ/BEACON Packet Terminal Mode.

TNC Transmit Buffer flush.

A4.4 KAM

LAN-LINK switches colors when it sees the /V or /H following  the

callsign  in the packet header, or in the calls monitored  ('MH')

list.  If you are monitoring both HF and VHF simultaneously,  the

colors  will show you which radio you heard which signal  on.  If

the  Mail-Snatch or the QTC-Snatch are triggered,  LAN-LINK  will

change  modes  and attempt the connect on the correct  radio  but

will  not  change the band/power information or  the  logbook  to

reflect  that change in the logbook or in the status window  dis-

play.

A4.4.1 Default Commands

Rename  the  LAN-LINK.SYS  file  as  PK232.SYS.  Rename  the

KAM.SYS  file  as LAN-LINK.SYS and edit it using  your  word

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 131

processor  in its non document mode to customize it to  your

station.

A4.4.2 Program differences

The following functions DO NOT WORK because the TNC does not

support them.

TNC Transmit Buffer flush.

The SIGNAL Communications Mode.

The NAVTEX Communications Mode.

The Alt-D key in the MORSE Communications Mode.

The AMSAT-OSCAR Menu does not allow UoSAT option, as the KAM

does not have 1200 baud ASCII capability.

The BAUDOT NAVY MARS Protocol is also not available.

A4.5 PK-232 EPROM Version 30 DEC 1988 and subsequent ones.

Turn   the  'BBS  OFF'  if  you  want  the  Alert  Call,   packet

communications mode header colors and Digipeat Detect features to

work otherwise turn it on.

A4.6 MFJ 1278

LAN-LINK  assumes  that  the Radio 1 port  is  connected  to  the

VHF/UHF packet transceiver, and the Radio 2 port is connected  to

the HF transceiver with the multi mode communications capability.

To transmit CW turn the VOX on.

The MFJ1278 suffers from one major deficiency as far as  LAN-LINK

is  concerned. The Echo-As-Sent only works in the AMTOR  Communi-

cations Mode. that means that you will not see the contents of  a

file  echo in the incoming window as the file is being  transmit-

ted.

A4.6.1 Default Commands

Rename  the  LAN-LINK.SYS  file  as  PK232.SYS.  Rename  the

MFJ1278.SYS file as LAN-LINK.SYS and edit it using your word

processor  in its non document mode to customize it to  your

station.

A4.6.2 Program differences

The  following functions DO NOT WORK because either the  TNC

does  not  support them or does not do so  in  a  consistent

manner.

TNC Transmit Buffer flush.

The SIGNAL Communications Mode.

The AMSAT-OSCAR Menu does not allow UoSAT option.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 132

The BAUDOT NAVY MARS Protocol is not available.

CW Identification.

The NAVTEX communications mode. (MFJ1278 allows, but not set

in LAN-LINK).

A4.7 Heath HK-21

K3WGF  noticed that his Heath HK-21 TNC didn't seem to  have  the

proper  circuitry  to  support true DCD on pin  8  of  the  RS232

connector.  Even though the pin was labeled DCD it didn't  signal

properly.  Set  the DCD function in LAN-LINK to OFF to  make  the

Capture-to-Disk close out normally rather than continue to remain

engaged.

A4.8 PK 87/88

Use the same LAN-LINK.SYS file as for a PK-232 and ignore the non

packet communications modes and functions.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 133

APPENDIX 5 UPDATES/REVISION HISTORY

1.56 RTTY and Navy Mars File transfer bug fixed. Command Echo bug

fixed. Disconnect Capture-to-Disk bug fixed. Alt-K and Alt-Y

added.  :QBU: added. J option in parameter menu  deleted,  I

and L options added. BBS Bulletin read command customizable.

Automatic   packet/AMTOR  contest  mode   added.   Callsigns

converted  to upper case. CW speed shown in  status  window,

left/right arrows change CW speed by 1 wpm. Sound flag kills

^G  bell on incoming text. /EX or /ex as well as  :EOF:  can

now be used in *.bbs file column 1. 'RY's deleted from  RTTY

CQ  call.  PC  to TNC now  uses  hardware  handshake  (^S/^Q

deleted).   SAREX  Orbiter  menu/functions  changed.   SAREX

Orbiter  Metabeacon interface changed. Logs split to HF  and

VHF. PacketCluster added. Update changed.

1.57 dBASE compatible logbook files. AutoCQ bug, 1.56 Non KAM vhf

PACLEN  bug, Packet 'End' key bug and Editor F10 bug  fixed,

changes  in order of colors in SYS file. K option  added  to

Edit  menu. ! F and 0 options added in Parameters Menu.  PK-

232 Mailbox commands changed at start-up and shut-down.  QSM

added to LAN-LINK Menu. Com 3/4 addresses/IRQs changed  from

PS-2  to XT. Capture-to-disk files automatically named  each

day as YYMMDD.RUN. Quick Menu added. Kam/KPC Shut-down state

4/5 turns PBMON and NDMON off.

1.58 Default  users  added. Editor stack/ 1.57  lockout  bug  and

other anomalies fixed, X and Y options added to TNC Menu. O,

P, Y and Z options added to Files Menu. Pick option added to

message Menu. Com port set up changed. Multi-user operations

changed. :QTA: Added.

1.59 F10,  First  menu character, RTTY/CW file transfer  and  QBU

transparent  mode bugs fixed. QTA ack, left/right arrows  in

log  prompt, Local time display, QZU/QZD Zmodem,  Metabeacon

in  packet,  MCON control in SAREX Mode, L  option  in  Edit

menu,  ! in Call menu, K and N in Log Menu, and Z  in  SAREX

Menu added. B removed from call menu. Alt-Y now does RY's in

RTTY. XMITOK ON/OFF toggle in PK-232 CW mode. Scrollback not

written to capture file.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 134

APPENDIX 6 LAN-LINK 1.59 REGISTRATION FORM

To: Joe Kasser G3ZCZ, P O BOX 3419, SILVER SPRING, MD 20918.

CALL ________________ TODAY'S DATE _____________

NAME ______________________________________________

ADDRESS ___________________________________________

___________________________________________

CITY ___________________________________________

STATE _________ POSTCODE ____________ TELEPHONE ____________

TNC TYPE _________ FIRMWARE REV __________ HOME BBS ___________

DISK SIZE 5.25 _____ 3.5 ____ Other ? _____________

I  enclose  a check for $35.00. Please register me as a  user  of

LAN-LINK.  I am currently using Version ______ which  I  obtained

from  _________________________________________.  Please send  me

the  latest version of LAN-LINK or if a more recent one does  not

exist  at this time, QSL my registration and add my name  to  the

list to receive a free update when it becomes available.

I also enclose an additional $___ for evaluation copies of WHATS-

UP  _, PC-HAM _ and Startrek TCP _ ($5 for 1 program, $10.00  for

the set). If I like them, I plan to register them in due course.

Signature

My favorite operating modes are ______________________________.

Additional  Features I'd like to see in future releases  of  LAN-

LINK.

Keep Conference Bridge Up  __ YAPP Binary File compatibility __

Expanded Contest Operation __ More AMTOR (explain below)     __

NTS capability __             Picture Transfer capability    __

Telephone Modem capability __ PacketCluster Menu (Like BBS)  __

Kenwood Radio Control Menu __ Printed Manual ($15.00)        __

Dumb Terminal (CP-1, ST-5 etc.) Baudot/ASCII RTTY operation  __

NOTE:  some  additions  come into LAN-LINK as a  result  of  user

suggestions, so here's your chance to get some input in.

Comments, likes, dislikes, wish list, etc.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 135

APPENDIX 7 LAN-LINK PROBLEM REPORT

Name ______________________________________ CALL _______________

Street __________________________________________________________

City _______________________________ State _____ ZIP __________

Tel. ____________________ LAN-LINK version ____

1. Describe your computer system.

Manufacturer ____________________ Model ___________TNC __________

Firmware _____

DOS Version ________________ Clock Frequency _________ MHz.

List peripheral equipment and cards installed in your computer.

Video Herc/EGA/CGA_____ _______________________

_______________________ _______________________

_______________________ _______________________

What  memory  resident programs were installed when  the  problem

occurred?

_______________________ _______________________

_______________________ _______________________

2. Describe  the  problem  in detail. What were  the  Misc.  flag

state-sequence numbers when the problem was happening?

3. Describe the sequence of events or keystrokes that led to  the

problem.

4. Any other information that may help in locating the problem.

Use  additional  sheets of paper if necessary. ATTACH a  copy  of

your LAN-LINK.SYS file.

Mail to: Joe Kasser, P O Box 3419, Silver Spring, MD. 20918.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 136

APPENDIX 8 Other PC Software by Joe Kasser G3ZCZ

8.1 PC-HAM 3.52 BY G3ZCZ

LOGBOOK

Full  blown  logging package. With automatic check  of  logs  for

awards such as DXCC. Allows you to recall any entry by call  sign

within  seconds. Indexed displays, QSL'ing, Contest mode  QSL'ing

(prints  the lot) and lots more. Although written in  dBASE3  the

package  contains a compiled version (LOGBOOK.EXE), so you  don't

need dBASE to run it. The source code is ONLY given to registered

users.  It  can convert your exported LAN-LINK.LOG file  to  LAN-

LINK.DBF  to put this package to work. Ideal for DX-peditions  or

for DX robot users to handle QSLing and log statistics.

CONTEST

Keeps Dupes in memory, logs QSO's to disk in format which can  be

processed  by the LOGBOOK package. Now compiled in  Turbo  BASIC,

source  code  is supplied so that you can modify the  program  to

meet your requirements.

CQSS

Sweepstakes   game  compiled  in  Turbo  BASIC.  Work  the   ARRL

Sweepstakes  contest  on  your computer.  You  are  located  just

outside  Washington  DC. A propagation model is built in  to  the

program.  This program is REQUIRED training for  all  sweepstakes

operators. Earlier version of the program is described in  detail

in 'Software for Amateur Radio' by Joe Kasser G3ZCZ, published by

TAB Books, Blue Ridge Summit, PA. 17214.

WHATSON

Predict HF Propagation for given days. Contest mode with printout

to whole world at hourly intervals. Needs BASIC.

This  software  AND MORE comprises PC-HAM which is  available  as

Shareware  from G3ZCZ. Send a blank formatted disk, SASE,  and  a

QSL card for a copy or save time and money and register as a user

for  $36.50 (half of $73). Request PC-HAM and specify  disk  size

(3.5 or 5.25 inches).

8.2 STARTREK The Computer Program

An  ideal  task  for the beginner to learn a  language  on  is  a

simulation  game  which is written around the computer  that  the

beginner  has  available.  For in that case,  there  is  complete

control of all inputs and outputs, This kind of game in which the

player makes decisions based on the information available to  him

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 137

or  her  available  at  the  time),  can  be  made   sufficiently

sophisticated  and complex so as to make writing it  an  adequate

challenge for anybody.

The  techniques  used in writing a good game are  the  same  that

programmers  use in professional activities. Writing a good  game

poses  a  challenge that allows you to develop  good  habits  and

techniques  for  programming  and  also allows  you  to  learn  a

language in an interesting manner. By taking an orderly  approach

to the game design, complex operations may be clearly  understood

and  converted  to  computer  code with the  aid  of  a  language

reference manual, irrespective of the language being used.

This product teaches the techniques for writing such a game using

the  STARTREK game as an example, and the BASIC language  as  the

programming language in which to write the code.

Registration fees.

Single Copy $15.00

10 - 50 Copies $12.00 per registered copy.

50 - 100 Copies $10.00 per registered copy.

100 + Copies $8.00 per registered copy.

This is available as Shareware from G3ZCZ. Send a blank formatted

disk  and an SASE for a copy or save time and money and  register

as  a user by sending a check for the registration fee  according

the  schedule  posted  above. Request  Startrek  -  The  Computer

Program and specify disk size (3.5 or 5.25 inches).

8.3 WHATS-UP 1.00

WHATS-UP  is a tool for experimenting with orbital  dynamics  and

Telemetry  Decoding and display Program for the  UoSAT-OSCAR  11,

AMSAT-OSCAR  13, Fuji-OSCAR 20 and the AMSAT Microsat  Spacecraft

(OSCARs  16,  17,  18  and  19).  It  is  table  driven  via  the

configuration files to allow maximum flexibility.

WHATS-UP contains the following features:

* Can  automatically  set  Kenwood Radios  to  spacecraft  beacon

frequency when satellite comes over your horizon.

* Can  automatically read the frequency from Kenwood Radios to  a

disk file for Doppler frequency measurements.

* Can  automatically configure a PK-232 for each spacecraft  data

format.

* Reads 2 Line and AMSAT format orbit element data files.

* Provides antenna azimuth and elevation pointing data.

* Real time, Playback and Dumb Terminal mode.

* Automatic Capture-to-disk of raw telemetry.

* Extracts  telemetry channel data to a database  or  spreadsheet

readable file for further analysis.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 138

* Link quality measurement on packet telemetry.

* Capability to print the raw telemetry as it is received.

* Up to 16 user configurable display pages (screens). You set the

position  on  the page (width of engineering  unit  field,  and

number of decimal places) that a parameter is displayed at.

* Wild card page (parameter shows up on all pages).

* Selectable  display of Engineering units or raw byte  for  each

display page.

* Display of raw packets (i.e. STATUS)

* Color  changes if a parameter value changed between  successive

frames.

* Audio  and  visual alarms if a telemetry value  exceeds,  falls

below or falls outside a preset limit value(s).

* Dumb split screen terminal mode (a la LAN-LINK).

* Customizable colors, PC to TNC baud rate, data parity and  stop

bits.

* Default spacecraft configuration files.

* Time of day clock display (in HH:MM:SS format)

Real time analysis requires a receiver and demodulator.  Analysis

of  playback  data can be performed with just  the  computer  and

WHATS-UP.

If  you  would  like a copy of WHATS-UP  and  have  captured  any

telemetry  to  disk, especially daytime  or  southern  hemisphere

passes,  please  put  them on a disk and send  it  in  with  your

request,  a mailer and return postage. Don't let  the  individual

data files become too long. I suggest that you rename them  every

day in the format YYMMDD.SC, such as 900824.D17, 900824.F20, etc.

IF  YOU  ARE INTERESTED IN OSCAR AND SPACECRAFT  TELEMETRY,  THEN

THIS PROGRAM IS A MUST.

Suggestions for improvements and additions are always welcome.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 139

APPENDIX 9 HOW SHAREWARE WORKS:

The Association of Shareware Professionals (ASP) has  established

standards for its members and for any organization which has "ASP

Approved"  status.  The  ASP wants to  make  sure  the  shareware

principle  works for you. If you are unable to resolve a  problem

with  an  ASP  member  or  organization  (other  than   technical

support), the ASP may be able to help. Please write to

The  ASP  Ombudsman, 545 Grover road, Muskegon,  MI.  49442-9427,

USA.

You  are encouraged to copy the floppy disk and share  it  freely

with  others.  You have the luxury of trying out the  product  at

your own pace and in the comfort of your own home or workplace.

After  you  have used the material for  a  reasonable  evaluation

period  (30  days),  you should either  discontinue  use  of  the

material  or  register your copy. Your support is  important  and

greatly appreciated. With it, Shareware authors are encouraged to

design  and distribute new products. Without it, a great deal  of

high quality, low cost software will cease to be available.

Why pay at all?

* You receive support from the author.

* You receive a CURRENT copy of the program.

* Your input and ideas help shape future products.

* A sense of pride and ownership in having honestly participated

in the Shareware revolution.

* You  help  to  keep  software  prices  down  by  supporting  a

distribution   method  which  doesn't  depend   on   expensive

advertising campaigns.

Be  aware of the following restrictions, designed to protect  the

community  of Shareware users and to prevent greedy  people  from

taking unfair advantage of the trust, hard work and good will  of

Shareware authors.

1. No  price  or consideration may be charged for  the  material.

However,  a distribution cost may be charged for the  cost  of

the diskettes, shipping and handling, not to exceed $6.

2. The  files  and programs on the disks may not be  modified  or

deleted.

3. The  material  cannot  be  sold as part  of  some  other  more

inclusive package.

4. The material cannot be "rented" or "leased" to others.

5. The end user must be told clearly in writing on the outside of

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK VERSION 1.59 PAGE 140

the  package and in all advertising that the  diskette(s)  are

"Shareware."

6. The  package must contain a written explanation that the  disk

is   for   evaluation  purposes,  and   that   an   additional

"registration fee" is expected by the author, if the  material

is used beyond an initial evaluation period.

7. In  the case of distribution via any telecommunications  link,

the following must be done:

An error checking protocol must be used.

The individual files must be combined into, and transferred in

a library or archive format.

8. Shareware distribution is permitted only in the United States,

Canada, England, and Australia.

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 141

Accepts, 1

Accordance, 57

ACHG, 8, 95

Ack, 133

Acknowledgement, 125

Acknowledgment, 123

Adjusted, 81

Adopt, 11

Advantages, 125

Alarm, 3, 27, 33, 46, 47, 83, 96

Alarmwindow, 6, 83

ALFD, 87

ALIST, 7, 88, 89

Alt-A, 4, 8, 54, 55, 57, 103

Alt-B, 8, 103

Alt-C, 8, 18, 24, 92, 103

Alt-D, 8, 20, 45, 71, 88, 89, 100, 103, 131

Alt-E, 4, 8, 24, 54, 55, 57, 103

Alt-F, 4, 8, 28, 54, 55, 58, 103, 112

Alt-J, 8, 103

Alt-K, 8, 104, 133

Alt-L, 8, 74, 104

Alt-M, 8, 72, 104, 115

Alt-N, 8, 92, 104

Alt-O, 8, 19, 20, 65, 74, 104

Alt-P, 8, 104

Alt-Q, 8, 20, 32, 105

Alt-R, 8, 38, 105

Alt-S, 4, 8, 55, 74, 105

Alt-U, 4, 55

Alt-X, 8, 17, 18, 58, 105

Alt-Y, 8, 105, 133

Alt-Z, 4, 8, 40, 55, 56, 105

Amateurs, 1, 123

AMSAT-OSCAR, 5, 14, 30, 62, 63, 65, 131, 137

AMTOR, 2, 7, 8, 10, 12, 13, 14, 15, 16, 19, 20, 22, 23, 24, 27, 28,

29, 33, 34, 37, 38, 47, 52, 54, 69, 70, 74, 76, 77, 84, 88, 89,

95, 96, 101, 103, 104, 105, 107, 114, 116, 117, 121, 127, 128,

131, 133, 134

AMTOR-FEC, 22

Antenna, 137

APPENDIX, 10, 11, 17, 21, 50, 51, 53, 117, 119, 125, 129, 133, 134,

135, 136, 139

Applies, 17

Areas, 123

Arise, 125

ARQ, 3, 8, 27, 33, 34, 89, 95, 96, 101, 103

Arrow, 4, 9, 22, 55, 56, 106, 107

Arrows, 32, 106, 107, 133

ASCII, 2, 4, 5, 7, 8, 10, 12, 13, 14, 15, 21, 22, 42, 49, 50, 51,

52, 62, 63, 88, 98, 101, 106, 107, 110, 111, 112, 113, 115, 121,

122, 126, 131, 134

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 142

Attack, 5, 63, 65

AUTO-Upload, 111

AutoCQ, 22, 114, 133

Automatic, 2, 5, 13, 14, 15, 19, 20, 21, 24, 27, 28, 31, 39, 47,

54, 64, 69, 74, 75, 87, 94, 98, 99, 104, 115, 119, 120, 133, 136,

137

Automatically, 13, 14, 16, 24, 25, 34, 40, 49, 50, 53, 54, 55, 58,

71, 77, 92, 107, 109, 113, 114, 120, 122, 125, 133, 137

Autumn, 81

AX, 123

Azimuth, 137

Banks, 1

Batteries, 17, 61, 119

Battery, 16, 17, 61, 62

Baud, 6, 7, 8, 16, 17, 21, 63, 71, 81, 82, 88, 89, 90, 96, 97, 98,

99, 107, 108, 113, 118, 119, 131, 138

BAUDOT, 2, 7, 8, 12, 13, 15, 22, 63, 87, 88, 96, 101, 106, 107, 131,

132, 134

Baudrate, 9, 22, 107, 129

BBS, 1, 3, 5, 6, 7, 8, 9, 11, 14, 18, 21, 24, 25, 30, 32, 33, 38,

39, 40, 41, 42, 44, 45, 46, 47, 59, 62, 64, 67, 68, 74, 75, 76,

80, 86, 87, 90, 102, 105, 107, 108, 111, 119, 122, 123, 131, 133,

134

Beacon, 3, 5, 7, 9, 10, 13, 14, 15, 23, 27, 33, 34, 39, 40, 41, 45,

54, 62, 63, 66, 67, 68, 69, 70, 71, 72, 75, 76, 87, 90, 94, 95,

109, 114, 115, 116, 117, 121, 127, 128, 129, 130, 137

Beep, 5, 68, 70, 71, 109

Behind, 81

Bell, 48, 63, 133

BIT, 11, 20, 21, 82

Bits, 6, 82, 108, 119, 138

Blink, 27, 64

Blocks, 123

BottomWindow, 6, 83

Breaks, 123

Bridge, 7, 14, 24, 90, 93, 94, 134

BTEXT, 71, 114

Buffer, 2, 5, 8, 10, 13, 20, 28, 29, 32, 48, 60, 61, 69, 80, 93,

101, 102, 103, 106, 117, 122, 125, 129, 130, 131

Buffering, 125

Bug, 21, 133

Bugs, 10, 117, 133

Bulletin, 22, 62, 76, 86, 121, 133

Bulletins, 14, 25, 44, 62, 76, 86, 90, 119

Call, 2, 3, 4, 5, 6, 7, 8, 14, 15, 18, 19, 20, 22, 23, 24, 26, 27,

30, 31, 32, 33, 34, 36, 37, 38, 39, 42, 44, 45, 46, 47, 54, 55,

57, 58, 63, 64, 65, 66, 69, 70, 74, 75, 76, 77, 78, 79, 84, 89,

90, 92, 93, 94, 96, 98, 99, 101, 103, 104, 105, 108, 109, 110,

113, 116, 118, 120, 121, 122, 124, 126, 131, 133, 134, 135, 136

Calls, 5, 6, 23, 24, 26, 27, 28, 32, 34, 39, 41, 58, 59, 60, 61,

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 143

66, 67, 68, 74, 75, 76, 77, 83, 87, 93, 96, 100, 103, 104, 109,

114, 115, 117, 128, 130

Callsign, 2, 5, 6, 7, 14, 16, 20, 21, 23, 24, 26, 27, 32, 33, 34,

35, 37, 38, 39, 45, 49, 51, 52, 55, 58, 59, 60, 64, 71, 72, 73,

74, 75, 76, 77, 86, 90, 91, 94, 96, 98, 99, 105, 108, 109, 111,

112, 114, 115, 117, 119, 120, 121, 127, 128, 130

Callsigns, 5, 59, 68, 74, 76, 128, 133

Capture-to-Disk, 3, 5, 20, 21, 22, 24, 25, 44, 45, 46, 49, 65, 68,

69, 75, 86, 98, 99, 106, 109, 132, 133, 137

CFROM, 79, 80, 100

CGA, 10, 117, 135

Character, 13, 17, 18, 26, 27, 29, 32, 37, 39, 40, 41, 42, 43, 48,

49, 50, 52, 53, 54, 56, 59, 64, 65, 66, 72, 73, 75, 82, 87, 92,

93, 94, 98, 109, 110, 111, 115, 117, 119, 120, 121, 128, 133

Chatham, 1

Checks, 1

Chime, 35, 73

CHIRPCOPY, 7, 15, 28, 88, 89

Circuit, 123

CIS, 1

Clocks, 81

CMD, 2, 26, 28, 117

CMSG, 20, 66, 67, 69, 80

Coast, 81

Collect, 24

Color, 6, 7, 15, 24, 82, 83, 84, 85, 91, 92, 138

Colors, 6, 14, 16, 19, 68, 82, 85, 104, 109, 117, 129, 130, 131,

133, 138

Column, 55, 110, 133

Comercial, 11

Command, 5, 10, 12, 14, 15, 18, 19, 20, 25, 26, 27, 28, 29, 32, 35,

36, 37, 38, 40, 41, 48, 49, 50, 51, 52, 53, 55, 58, 59, 60, 61,

62, 66, 67, 68, 69, 70, 71, 72, 73, 74, 75, 76, 77, 78, 84, 87,

91, 93, 97, 98, 99, 100, 104, 105, 106, 107, 108, 111, 112, 113,

114, 117, 118, 120, 122, 124, 125, 126, 128, 129, 133

Commands, 10, 11, 12, 16, 21, 39, 41, 42, 43, 62, 66, 67, 68, 70,

107, 108, 109, 117, 119, 124, 125, 129, 130, 131, 133

Comments, 41, 54, 55, 59, 109, 134

Common, 6, 25, 68, 77, 100, 109, 118

Compress, 25

Compuserve, 1, 11

Computers, 123

Conference, 7, 10, 14, 24, 38, 90, 91, 92, 93, 94, 117, 118, 130,

134

CONFIG, 9, 107, 110

Configuration, 2, 12, 14, 17, 18, 19, 20, 36, 45, 51, 61, 63, 64,

65, 77, 86, 89, 92, 108, 118, 119, 137, 138

Connect, 2, 3, 5, 8, 13, 14, 15, 18, 21, 23, 24, 26, 27, 30, 31,

32, 33, 34, 35, 36, 37, 38, 40, 41, 46, 47, 49, 53, 54, 60, 61,

63, 64, 67, 68, 70, 71, 72, 74, 79, 80, 91, 92, 93, 96, 100, 101,

103, 104, 105, 109, 110, 112, 114, 115, 116, 118, 120, 121, 122,

125, 127, 130

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 144

Connected, 6, 10, 13, 14, 22, 23, 24, 30, 32, 35, 36, 40, 50, 52,

65, 68, 71, 72, 74, 76, 80, 84, 85, 90, 91, 92, 94, 105, 111, 118,

120, 122, 126, 131

CONOK, 67, 79, 80

Contest, 2, 3, 5, 6, 13, 15, 19, 20, 22, 46, 47, 58, 68, 73, 74,

79, 104, 133, 134, 136

Control-C, 43

Control-F, 112

Control-Q, 119

Control-S, 119

Control-Z, 9, 107

Convenient, 62

CONVERSE, 2, 10, 12, 28, 29, 51, 53, 61, 72, 93, 95, 117, 122

Copy, 1, 16, 24, 27, 39, 43, 62, 95, 97, 98, 99, 115, 122, 135, 136,

137, 138, 139

Couple, 11

CP, 134

CQ, 2, 3, 6, 7, 10, 15, 19, 20, 22, 23, 24, 27, 28, 31, 33, 38, 39,

60, 69, 78, 79, 90, 91, 94, 96, 98, 103, 108, 114, 116, 117, 128,

129, 130, 133

CQTEXT, 78

Creating, 43

Credit, 1

Ctdsk, 3, 4, 42, 44, 48, 49, 68

CTEXT, 5, 6, 20, 60, 62, 66, 67, 69, 79, 80, 121

Ctrl-Z, 9

Customize, 17, 19, 76, 79, 81, 82, 100, 125, 129, 130, 131

CW, 7, 8, 88, 89, 95, 96, 97, 98, 99, 100, 101, 107, 131, 132, 133

CWID, 96

Dansey, 1

DBF, 6, 9, 24, 54, 57, 59, 78, 109, 136

DCD, 5, 68, 74, 93, 108, 118, 132

DCD-Flag, 118

Deactivate, 5, 60, 62

Defaults, 14, 16, 21, 61, 109

Definatly, 11

Degree, 125

Delivery, 123

Demodulator, 138

Digipeat, 5, 14, 23, 35, 38, 68, 70, 76, 77, 91, 108, 109, 122, 131

Digipeated, 33, 70, 72

Digipeater, 35, 72, 76, 79, 100

Digipeaters, 35, 127

DIR, 6, 9, 14, 18, 34, 35, 36, 37, 38, 44, 72, 74, 78, 107, 109,

110, 122, 126

Directories, 6, 68, 77, 92

Directory, 3, 6, 15, 29, 30, 33, 34, 36, 38, 42, 44, 46, 48, 50,

59, 72, 77, 78, 108, 109, 114, 122

DISCONNECTED, 10, 33, 38, 39, 65, 71, 74, 91, 118, 119, 127

Display, 3, 14, 23, 26, 27, 38, 46, 47, 50, 54, 56, 57, 58, 61, 65,

70, 72, 74, 79, 82, 86, 89, 91, 101, 104, 106, 109, 118, 119, 130,

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 145

133, 137, 138

Displayed, 17, 21, 22, 23, 24, 29, 33, 44, 45, 55, 58, 64, 69, 70,

73, 79, 87, 90, 97, 98, 99, 106, 107, 138

Displaying, 23, 98

Displays, 19, 21, 24, 38, 39, 58, 70, 86, 88, 106, 136

DL, 1

Domain, 124, 125

Doppler, 137

Downlink, 36

Download, 3, 6, 10, 14, 32, 40, 41, 50, 51, 53, 77, 78, 96, 108,

114, 121, 122, 124, 126, 128, 129

Drawn, 1

Drue, 124

DUPLEX, 12, 13

Dynamics, 137

EAS, 109

East, 81

EAX, 36, 110

Echo, 5, 25, 27, 68, 70, 84, 109, 131, 133

Echo-As-Sent, 131

Edit, 3, 4, 8, 14, 19, 20, 25, 30, 42, 43, 44, 45, 46, 49, 50, 54,

55, 56, 57, 62, 65, 77, 101, 104, 108, 112, 129, 130, 131, 133

Editor, 14, 20, 25, 42, 44, 65, 94, 110, 114, 115, 125, 133

EGA, 135

Electronically, 25

Element, 137

Elevation, 137

Embed, 43

End, 4, 8, 13, 14, 20, 26, 32, 37, 41, 43, 44, 47, 55, 56, 65, 72,

74, 76, 94, 105, 106, 107, 109, 117, 124, 128, 133, 139

Enter, 3, 6, 8, 13, 19, 26, 27, 28, 32, 33, 34, 37, 38, 39, 40, 41,

42, 43, 45, 46, 47, 49, 50, 52, 53, 55, 57, 58, 59, 62, 67, 68,

73, 74, 77, 82, 89, 90, 92, 93, 97, 99, 101, 103, 104, 108, 120

Environment, 124

EOF, 41, 44, 52, 107, 111, 112, 120, 128, 133

Erase, 3, 48

Error, 12, 17, 18, 20, 32, 51, 72, 73, 83, 93, 117, 120, 123, 124,

126, 127, 129, 140

Errors, 118, 123, 125

Escape, 18, 30, 54, 55

EST, 17, 81

Establishing, 123

Eurocard, 1

European, 1

Events, 10, 46, 118, 125, 135

Exercise, 49, 59

Experiment, 63, 124, 125

Experimenting, 137

Explicit, 113

FAX, 1

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 146

Features, 5, 9, 12, 15, 18, 44, 46, 50, 65, 70, 92, 110, 114, 122,

131, 134, 137

FEC, 2, 8, 10, 15, 27, 28, 29, 38, 95, 96, 101, 103, 105, 117

Files, 2, 3, 4, 5, 9, 14, 16, 17, 19, 20, 21, 23, 24, 25, 29, 30,

31, 32, 40, 42, 44, 45, 46, 48, 49, 50, 51, 53, 54, 56, 59, 60,

65, 69, 70, 71, 77, 81, 82, 87, 94, 107, 109, 110, 111, 112, 113,

114, 115, 120, 121, 122, 124, 125, 126, 128, 133, 137, 138, 139,

140

Filetype, 19, 57, 115, 121, 128

Final, 125

Fine, 112

Firmware, 19, 134, 135

Flashing, 21, 22, 23, 24, 33, 38, 39, 65, 70, 73, 86, 106

Forwarding, 62

FOUR, 17, 53, 113, 124, 125

FSK, 107

Function, 2, 4, 12, 13, 14, 15, 18, 20, 24, 25, 28, 29, 30, 31, 32,

37, 38, 39, 44, 45, 50, 54, 55, 56, 57, 58, 61, 65, 68, 70, 71,

74, 75, 89, 93, 95, 98, 99, 101, 103, 104, 105, 106, 109, 111,

112, 117, 122, 127, 132

Gain, 124

Gary, 124

Gateway, 127

Greek, 10, 17, 82, 119

Grover, 139

Handshake, 18, 70, 109, 133

Happened, 53

Hassles, 62

Header, 6, 14, 33, 39, 45, 60, 64, 75, 84, 94, 115, 121, 126, 129,

130, 131

HEADERLN, 109

Headers, 19, 23, 25, 33, 39, 40, 64, 76, 90, 109

Hercules, 117

HF, 2, 6, 7, 9, 12, 19, 24, 34, 54, 69, 78, 84, 87, 88, 92, 93, 108,

109, 110, 119, 129, 130, 131, 133, 136

Highlight, 56, 57

Highlighted, 54, 55, 57

Hobby, 11

Horizon, 137

Hours, 81

Implement, 124, 128

Implementation, 124

Implementations, 113, 125

Implements, 124, 125

Includes, 1

Incompatible, 17

Indicator, 14, 23, 27, 70, 73

Individually, 123

Inhibit, 124

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 147

Inhibitor, 39

Injuring, 124

Inserting, 65, 94

INTERESTED, 138

Interfaced, 113

Intermittent, 10, 117, 119

Interpretation, 11

Interrogate, 62

Interrupted, 53, 128

InWindow, 6, 71, 83, 91

IRQ, 81

IRQs, 133

Jacobsen, 123

Jeff, 123

KA-Node, 36, 109, 110

KA-Nodes, 36

KAM, 1, 11, 12, 18, 19, 20, 28, 62, 63, 74, 77, 82, 88, 90, 92, 97,

99, 109, 117, 130, 131, 133

Kaypro, 10, 130

KBytes, 24, 65

Kennon, 124

Kent, 1

Kenwood, 134, 137

KM, 41

KN, 74, 106

KPC, 1, 10, 61, 130, 133

Label, 25

Labeled, 132

LAMTOR, 95

LAN, 10, 14, 15, 19, 21, 24, 25, 26, 29, 34, 45, 46, 51, 52, 53,

57, 60, 63, 66, 69, 72, 74, 76, 79, 98, 99, 110, 112, 118, 119,

120, 122, 127

LAN-LINK, 1, 2, 3, 4, 6, 7, 9, 11, 12, 14, 16, 17, 18, 19, 20, 21,

23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38,

39, 40, 41, 42, 44, 45, 46, 47, 48, 50, 51, 52, 53, 54, 55, 57,

58, 61, 62, 63, 64, 65, 66, 67, 68, 69, 70, 71, 72, 73, 74, 75,

76, 77, 78, 79, 80, 81, 85, 86, 87, 88, 89, 91, 92, 93, 94, 95,

96, 98, 99, 100, 104, 107, 108, 109, 110, 112, 113, 114, 115, 116,

117, 118, 119, 120, 121, 122, 124, 125, 126, 127, 128, 129, 130,

131, 132, 133, 134, 135, 136, 138

Landline, 11

Landlines, 123

LEDs, 125

Left, 21, 29, 40, 42, 43, 45, 51, 82, 87, 106, 107, 114, 115, 119,

121, 124, 125, 133

Link, 5, 13, 19, 20, 21, 23, 26, 28, 29, 32, 34, 35, 36, 45, 46,

51, 52, 53, 57, 66, 68, 69, 71, 72, 74, 79, 88, 93, 98, 99, 102,

107, 108, 112, 113, 114, 117, 119, 123, 124, 125, 127, 128, 138,

140

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 148

Listing, 58

Little, 105

LM, 40, 41, 112

LOCAL-BBS, 45, 46, 62, 111

Lockout, 64, 133

Lockup, 10, 119

Log-to-disk, 68

Logbook, 6, 13, 24, 31, 54, 55, 56, 57, 58, 59, 69, 78, 101, 109,

130, 133, 136

LogWindow, 6, 83

Ltd, 1

Mail, 1, 3, 7, 8, 9, 14, 22, 23, 24, 33, 39, 40, 41, 42, 52, 62,

70, 71, 74, 75, 76, 80, 87, 89, 90, 91, 95, 100, 108, 114, 115,

116, 121, 125, 127, 128, 130, 135

Mail-Snatch, 5, 39, 76, 130

Mailbox, 5, 8, 9, 13, 14, 15, 27, 60, 62, 69, 95, 96, 107, 110, 114,

116, 117, 121, 127, 133

Mailer, 25, 138

Manufacturers, 11

MARS, 7, 8, 16, 22, 87, 96, 97, 131, 132, 133

MasterCard, 1

Matter, 11

MBX, 9, 17, 52, 90, 100, 107, 110, 111, 127

MCON, 133

Measurements, 137

Menu, 2, 3, 4, 5, 7, 8, 12, 13, 14, 16, 17, 18, 20, 21, 27, 28, 30,

31, 32, 33, 42, 44, 46, 48, 49, 50, 54, 56, 57, 58, 59, 60, 62,

63, 65, 66, 67, 68, 72, 76, 81, 83, 84, 86, 87, 88, 89, 92, 95,

96, 98, 99, 103, 104, 105, 106, 108, 109, 110, 112, 117, 118, 120,

122, 131, 133, 134

Menus, 12, 25, 30, 83, 117

Message, 3, 4, 5, 8, 9, 10, 11, 13, 14, 15, 17, 18, 20, 21, 30, 32,

36, 39, 40, 41, 42, 44, 45, 46, 48, 50, 51, 52, 53, 59, 60, 62,

68, 69, 70, 71, 72, 73, 74, 75, 78, 80, 82, 93, 94, 95, 96, 97,

101, 104, 107, 108, 111, 112, 113, 114, 115, 117, 119, 120, 121,

122, 125, 126, 127, 128, 133

META, 65, 66, 67, 68

Metabeacon, 7, 65, 66, 67, 90, 94, 133

METEORSCATTER, 69

MFILTER, 119

MFJ, 1, 10, 11, 12, 18, 28, 62, 63, 77, 82, 129, 131, 132

MH, 26, 68, 83, 122, 126, 130

MHWindow, 6, 83

MI, 139

MIR, 63

Mode, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 15, 17, 18, 19, 20, 21,

22, 23, 24, 26, 27, 28, 29, 30, 31, 33, 34, 35, 37, 38, 39, 40,

41, 42, 43, 44, 45, 48, 49, 51, 53, 54, 55, 56, 57, 58, 60, 61,

62, 63, 64, 65, 67, 68, 69, 70, 71, 72, 73, 74, 76, 77, 79, 80,

81, 84, 85, 87, 88, 89, 90, 91, 92, 93, 94, 95, 96, 97, 98, 99,

100, 101, 102, 103, 104, 105, 106, 107, 108, 110, 111, 114, 116,

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 149

117, 118, 122, 127, 128, 129, 130, 131, 132, 133, 136, 137, 138

Morse, 2, 7, 8, 12, 13, 15, 21, 22, 27, 88, 96, 98, 99, 100, 103,

106, 131

MRPT, 33

Multi-user, 39, 41, 91, 133

Muskegon, 139

MYCALL, 96, 98, 99

Nature, 11

NAVTEX, 7, 22, 88, 89, 131, 132

NDMON, 133

NET, 14, 34, 35, 36, 38, 71, 109, 110, 112, 119

Nice, 123

NNNN, 97, 98, 99

NODE, 5, 12, 14, 18, 34, 35, 36, 68, 71, 108

NTS, 41, 134

Obtain, 1

Occupied, 123

Office, 11

Offset, 6, 17, 81, 108

Orbital, 137

Originate, 113, 124

OSCAR, 138

Outs, 124

OutWindow, 6, 48, 83, 117

Overwriting, 53

Packet, 1, 2, 3, 7, 8, 10, 11, 12, 13, 14, 15, 19, 20, 21, 22, 23,

24, 25, 26, 27, 28, 29, 30, 32, 33, 34, 37, 38, 40, 45, 47, 48,

49, 53, 54, 61, 62, 63, 64, 65, 68, 69, 70, 71, 72, 74, 76, 77,

78, 82, 84, 87, 88, 89, 90, 91, 94, 101, 103, 104, 105, 106, 107,

109, 114, 116, 117, 118, 119, 122, 123, 124, 125, 127, 128, 129,

130, 131, 132, 133, 138

PacketCluster, 6, 37, 38, 77, 101, 105, 108, 133, 134

PacketClusterZap, 22

PACLEN, 133

Patience, 125

PBBS, 80

PBMON, 133

PC-HAM, 13, 54, 58, 109, 134, 136

Pcz, 113, 124, 125

PgDn, 4, 42, 46, 55, 56, 60

PgUp, 4, 8, 42, 55, 56, 106

Phone, 25

Pick, 3, 4, 42, 46, 48, 49, 53, 57, 59, 123, 124, 128, 133

PK, 1, 11, 12, 16, 17, 18, 19, 20, 23, 28, 61, 62, 63, 74, 75, 76,

77, 79, 80, 82, 87, 88, 89, 90, 91, 92, 93, 95, 96, 97, 98, 99,

100, 103, 106, 107, 109, 110, 117, 118, 119, 127, 129, 130, 131,

132, 133, 137

Plan, 17, 81, 82, 134

PO, 1

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 150

Pointing, 137

Policy, 11

Possible, 112

Post, 11

Postage, 25, 138

Pounds, 1

Preferred, 25

Presents, 46, 50

Prompt, 6, 28, 32, 33, 37, 61, 64, 75, 77, 83, 86, 133

PromptWindow, 6, 83

PST, 17

Public, 124, 125

Putting, 113

QBM, 4, 6, 10, 50, 51, 77, 108, 121, 122, 126

QBU, 4, 6, 10, 50, 51, 78, 109, 112, 125, 128, 133

QBU-RX, 6, 9, 51, 78, 107, 109, 112, 125

QBU-TX, 6, 9, 51, 78, 107, 109, 112, 125

QDB, 4, 10, 50, 52, 122, 126

QJG, 10, 52, 128

QJL, 11

QMH, 10, 14, 122, 126, 130

QNO, 10, 51, 52, 72, 73, 120, 126, 127, 128

QRA, 4, 10, 15, 53, 127

QRM, 12, 20, 27, 64, 69, 127

QRT, 6, 10, 23, 64, 69, 79, 80, 100, 116, 127, 128

QRU, 4, 10, 50, 52, 53, 110, 111, 127, 128

QRV, 6, 10, 51, 52, 79, 80, 120, 128

QRZ, 2, 8, 31, 32, 66, 68, 101, 105

QSL, 10, 45, 51, 52, 54, 55, 64, 66, 68, 120, 128, 134, 136

QSM, 4, 10, 50, 51, 113, 114, 126, 133

QSO, 13, 22, 24, 25, 28, 38, 47, 50, 67, 69, 73, 74, 89, 91, 95,

96, 98, 105, 114, 117

QSP, 4, 5, 10, 23, 50, 51, 52, 53, 68, 72, 108, 111, 120, 127, 128

QSY, 64, 69

QTA, 4, 9, 10, 50, 53, 107, 112, 113, 121, 128, 133

QTC, 5, 10, 14, 22, 68, 70, 71, 72, 76, 108, 114, 128

QTC-Snatch, 114, 115, 130

Queries, 11

Questionable, 11

Questions, 1

Quickly, 123

Quite, 123

QYD, 10, 129

QYU, 10, 129

QZ, 9, 17, 53, 81, 82, 108, 113, 124, 128

QZD, 4, 10, 50, 53, 113, 124, 125, 128, 133

QZD-RX, 113, 125

QZD-TX, 113, 125

QZU, 4, 10, 50, 53, 113, 124, 125, 128, 133

QZU-RX, 113, 124

QZU-TX, 113, 124

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 151

Radio, 1, 2, 11, 12, 13, 14, 62, 63, 65, 87, 88, 100, 107, 113, 119,

120, 123, 124, 125, 130, 131, 134, 136

Radios, 137

Reads, 137

ReadyCrest, 1

Recognize, 12, 18, 20, 27, 71, 115

Recommended, 62

Reconnect, 53

Recovery, 124

Refunded, 1

Registering, 1

Regret, 11

Regrets, 11

REJECT, 69, 79

Release, 24

Released, 24

Remade, 128

ReMember, 3, 13, 32, 33, 37, 73, 90, 94, 110

Reminder, 50

Repeat, 10, 17, 27, 51, 106, 114, 126

Replacing, 17

Restart, 20, 68

Retried, 128

RETRIES, 93

Review, 17

RH, 76

RM, 40, 76

RN, 40, 76

Road, 139

Robot, 2, 3, 8, 19, 20, 22, 23, 24, 27, 46, 47, 65, 66, 67, 68, 69,

95, 96, 101, 104, 114, 136

Route, 61, 65

RTTY, 5, 7, 16, 22, 27, 62, 63, 88, 96, 98, 105, 106, 107, 133, 134

RXR, 63

RY's, 101, 133

RYs, 8, 105

Rz, 113

SAREX, 5, 7, 15, 23, 62, 63, 64, 65, 66, 67, 68, 86, 108, 133

Satisfied, 1

Scheduler, 14, 47

Screen, 3, 10, 15, 16, 21, 28, 30, 39, 43, 44, 45, 46, 54, 58, 60,

69, 71, 82, 83, 90, 93, 97, 98, 99, 101, 106, 107, 109, 119, 120,

125, 138

Screwed, 54

Scroll, 25, 42, 43, 83, 106

Scrollback, 8, 106, 133

ScrollBackWindow, 6, 83

Sealink, 124

SELCAL, 6, 8, 15, 16, 22, 24, 28, 33, 34, 37, 38, 76, 77, 97, 98,

99, 101, 105, 108

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 152

Semiautomatic, 13, 54, 74

Service, 62

Shift, 2, 8, 9, 22, 29, 63, 89, 92, 96, 97, 98, 99, 107, 112

Shut-down, 133

Shutting, 62

Sideband, 106, 107

SIGNAL, 7, 8, 14, 15, 18, 22, 27, 48, 52, 71, 88, 89, 95, 100, 103,

105, 106, 107, 118, 130, 131, 132

Signifies, 24

Smiling, 24

Smith, 124

Solid, 105

SOLO, 6, 7, 23, 80, 90, 91

Speedy, 123

ST, 134

Stack, 133

STANDBY, 7, 8, 88, 89, 95, 96

Stated, 123

Status, 2, 5, 16, 21, 27, 33, 39, 60, 61, 62, 64, 65, 66, 69, 70,

72, 73, 79, 83, 86, 87, 90, 91, 93, 94, 97, 98, 99, 106, 107, 117,

130, 133, 138, 139

StatusWindow, 6, 83

Sterling, 1

Stream, 7, 8, 66, 84, 85, 91, 92, 94, 101, 104, 118

Streams, 19, 24, 92, 94, 104, 118

Strongly, 62

Sub-directory, 126

Sub-menu, 75, 76, 77, 78, 81, 86

Sub-modes, 23

Subject, 39, 44

Sufficient, 25

Suffix, 29, 30, 46, 59, 73, 112

Switched, 62

SYS, 3, 7, 9, 16, 17, 19, 27, 38, 40, 42, 45, 51, 57, 61, 68, 72,

77, 79, 86, 93, 94, 98, 99, 107, 108, 109, 110, 118, 119, 121,

129, 130, 131, 132, 133, 135

TCP, 134

Telephones, 1

Terrestrial, 63

Terry, 1

Testing, 123

TheLink, 35, 109

Time-out, 117

Timer, 27, 66, 67, 94, 116

Timestamp, 109

Timing, 125

TNC, 1, 2, 5, 6, 8, 10, 12, 13, 14, 16, 17, 18, 19, 20, 21, 26, 27,

28, 29, 30, 31, 33, 36, 37, 38, 43, 44, 48, 50, 51, 60, 61, 62,

65, 66, 67, 68, 70, 71, 74, 77, 79, 80, 81, 82, 88, 91, 92, 93,

94, 98, 99, 100, 101, 103, 108, 109, 113, 114, 117, 118, 119, 122,

125, 129, 130, 131, 132, 133, 134, 135, 138

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 153

Today, 123

Traffic, 7, 24, 53, 63, 64, 70, 76, 90, 91, 92, 118

Transferring, 62

Trigger, 5, 15, 39, 64, 72, 76, 108, 115

Trouble, 17

Tuning, 105

TXR, 63

TXT, 6, 19, 26, 44, 48, 69, 78, 95, 117

Type-ahead, 28

Un-mark, 56

Unique, 124

UNPROTO, 67, 91

Unshift, 63, 98

Unshift-on-space, 22

Updating, 17

Upgrade, 24, 25

Uplinking, 53

Upload, 6, 10, 40, 41, 50, 52, 53, 78, 121, 122, 124, 126, 127, 128,

129

Uppercase, 4, 48, 49, 77

Urban, 123

USOS, 8, 22, 63, 96, 98, 107

Varying, 123

VAT, 1

Verbose, 76

Verification, 123

Visa, 1

Watch, 119

Window, 2, 16, 21, 25, 26, 27, 28, 29, 32, 33, 39, 42, 43, 44, 45,

46, 50, 59, 61, 62, 64, 65, 69, 70, 72, 73, 79, 82, 83, 84, 85,

86, 87, 90, 91, 94, 97, 98, 99, 104, 106, 107, 130, 131, 133

Windows, 29, 42, 44, 45, 83, 84, 85

Winter, 81

Wpm, 88, 107, 133

XFLOW, 21, 119

XMITOK, 89, 133

Xmodem, 123, 124

YAPP, 2, 10, 18, 110, 122, 123, 129, 134

Yellow, 21, 22, 24

Ymodem, 124

Yourcall, 37, 40, 50, 70, 72, 105, 115, 126

YYMMDD, 9, 25, 46, 49, 109, 133, 138

Zap, 3, 7, 8, 33, 37, 39, 41, 44, 45, 46, 47, 62, 64, 65, 68, 74,

75, 76, 86, 87, 101, 102, 105

ZapBBS, 22, 74, 75

ZCZC, 98, 99

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991

LAN-LINK Index Page 154

Zmodem, 10, 15, 17, 50, 53, 113, 124, 125, 128, 133

ZModem's, 53

LAN-LINK.DOC (c) G3ZCZ 1988, 1989, 1990, 1991