The RoundTable server for the G8BPQ switch.

Requirements:

Runs in a 20K DESQview window.

Uses it's own set of BPQ switch ports.

Has it's own node command.

Getting started:

First, configure the switch for RT.

In the file BPQCFG.TXT, add the new application RT.

Note that it MUST be the fourth or later application.

Example: APPLICATIONS=BBS,,,RT

Add some ports to the TNCPORTLIST.

RT ports cannot be shared with other applications.

Run BPQCFG and re-boot your system.

Second, configure RT by editing RT.CNF.

Change the callsign, name, and qth to your own.

Change the port list to correspond to the ports you added in BPQCFG.

If you did not configure RT as the fourth application, change the

application number to the one you used.

Sample RT.CNF :

W0RLI	    <-	Station callsign

Hank	    <-	Sysops name

West Linn, OR  <-	Station QTH

4		    <-	Application number

480	    <-	Timeout in seconds

10 11 12 13    <-	TNCPORTLIST ports to use

Startup RT in a 20K DESQview window.

Now anyone that wishes to use it can just connect to your node

and then issue the RT command.