(*****************************************************************************)

(*                                                                           *)

(*             MSATCOM(tm) - MicroSat Ground Station Terminal                *)

(*                                                                           *)

(*                              Version 1.0                                  *)

(*                                                                           *)

(*            (c) Copyright 1990 by Claudio M. Zanella LU4AEY                *)

(*                     Released For Public Distribution                      *)

(*                           ALL RIGHTS RESERVED                             *)

(*                                                                           *)

(*****************************************************************************)

USE

Digital communications protocol FTL0 (File Transfer Level 0)

SOFTWARE REQUIRED

- MSATCOM.EXE    Main program (english version)

- README.COM     Program to read file THUNDER.DOC

- MSATCOM.DOC    User documentation in Spanish

- MSATENG.DOC    User documentation in English (this document)

- MSATCOM.CNF    System configuring file

HARDWARE REQUIRED

MSATCOM can be used on PC, XT, AT and Compatibles. It requires for its

functioning TNCs with TAPR type commands and the complete connection of

the RS232 port, fit 1,2,3,4,5,6,7,8,20.  Take special care in fitting:

8 (Data Carrier Detect), 4 (Request to Send), 5 (Clear to Send).

With a PacComm TNC 220 & 320 set JP-C to CONN (Default is HIGH)

The addition of the following in your CONFIG.SYS file is recommended:

FILES = 20

BUFFERS = 20

MSATCOM was tested with the following HARDWARE:

TNC types:

PACCOMM TINY 2

PACCOMM TNC 200

PACCOMM TNC 220

PACCOMM TNC 320

MFJ-1270/1278

AEA PK 232

AEA PK88

Computer types:

PC AT 80386   20/25 MHz

PC AT 80286   16/20 MHz

PC AT 80286   12/16 MHz

PC XT 80286    8/12 MHz

PC XT 8088     4/10 MHz       PC XT NEC V20 10/12 MHz

SETTING THE TNC

This programm uses Hardware Handshaking.   Set its TNC on NO PARITY and

WORD LENGTH OF 8 BITS.

CPactime OFF

CR       ON

SEnd     $0D

DCD      ON  <-- External Carrier Detect in PK232

Enable the Hardware Handshaking of your TNC as follows:

START  0

STOP   0

XFLOW  OFF

XON    0

XOFF   0

The following parameters are recommended:

TXDEALY 20

AXDELAY 20

PRIOACK ON <-- Release TAPR 1.1.7 only

FLOW    ON

SUMMARY OF COMMANDS

The following commands can be used in MSATCOM:

F1         - Request reception of directories.

F2         - Request reception of files (DOWNLOAD).

F3         - Request send files (UPLOAD).

F4         - Activate/De-activate the printer in Directory mode.

F5         - Review downloaded directories by screens.

F6         - Adjust time of General directories request (HIGHTIME..PG).

F7         - Adjust time of Personal directories request (HIGHTIME.MY).

F8         - Adjust time of Bulletin directories request (HIGHTIME.BUL).

F9         - Change the current satellite.

F10        - Gateway to DOS.

Alt-B      - Send of break signal to TNC to put it into command mode.

Alt-D      - Change level of Data Carrier Detect (DCD pin 8 v24).

Alt-H      - Help screen (HELP).

Alt-R      - Re-organize directory files.

Alt-T      - Toggle modes: Terminal or Server.

Alt-X      - End MSATCOM (Return to DOS).

DESCRIPTION OF USE OF COMMANDS

KEY F1 (DIRECTORY):

By pressing F1 you get to DIRECTORY options.  There are four options

for the mode:

ALL      -  Receive the File Headers since last request.

PERSONAL -  Receive the File Headers sent to 'mycall' since last request

BULLETIN -  Receive the File Header sent to 'ALL' since last request

SEARCH   -  Receive the File Header that contains the search criterion

specified by you taking as reference the time since the

last request to ALL.

To correct the times since the last request (hightime.xx), see options

F6, F7 and F8;  these are contained in files MSTIME.ALL, MSTIME.MY and

MSTIME.BUL in UNIX Hexadecimal format.

KEY F2 (DOWNLOAD):

Through this key we DOWNLOAD.  Its use is very simple and consists of

indicating the HEXADECIMAL number of the file we are interested in

receiving.

This will be memorized until the DOWNLOAD concludes, satisfactorily or

not.  If by any chance the DOWNLOAAD of the file did not end and the

contact with the server is lost, the next time the satellite is in view

press F2 and the programm will automatically reinitiate the DOWNLOAD

from the offset where it lost contact.

ATTENTION: this will repeat as long as the program doesn't receive the

COMMAND DATA END or the DOWNLOAD_ERR_RESP that gets you off the PDL

status to the DOWNLOAD file.  If you want to abort this download

request, ERASE from your DOS the current DOWNLOAD file.

File status:

xxxxxx.PDL   : File being downloaded (This hasn't ended).

xxxxxx.DL    : File downloaded successfully

xxxxxx.XDL   : Rejected file (An error occurred during the download).

xxxxxx       : Hexadecimal number of requested file.

KEY F3 (UPLOAD):

By pressing this option you can UPLOAD -- sending of FTL0 format

files -- to the server of the Micro satelite.  As with DOWNLOADs, the

option UPLOAD remains memorized until it concludes satisfactorily or

not.

If the sending is incomplete because the contact with the server was

lost, we can reinitiate the upload from the offset where the process

has been interrupted the next time the satellite is in view.

The UPLOAD is immediate.  Once thithe programm s option is chosen,

initiates the connection procedureif files to be

sent are found, .

File status:

mmmmmm.OUT   :  File in initial process of sending.

xxxxxx.PUL   :  File in process of sending (It has been interrupted).

xxxxxx.UP    :  File has been uploaded successfully.

xxxxxx.XUL   :  Rejected file, an error occurred during uploading.

mmmmmm       :  Initial name of file.

xxxxxx       :  Hexadecimal number assigned by the server

KEY F4 (PRINTER TOGGLE):

This option enables to put the printer on or offline.  The same will be

used during the DIRECTORY procedure, this means we can obtain a copy on

paper of the directory of the server while we are receiving it, by this

the DOWNLOAD operation is greatly speeded up.

KEY F5 (RELOAD/REVIEW DIRECTORY):

The function of this option is to enable to load and sort  (Reorganize)

on memory the directories received from the current server, at the same

time they are shown on screen from the newest to the oldest.

By the keys PageUp and PageDwn you can skip these directories pagewise.

By the keys UP and  DOWN we can go up  of down by lines.   The HOME key

positions the screen at  the beginning of the  directory.  The End  key

positions the screen at the end of the directory.  The W key enables to

save the screen directories on a file.

The messages directed  to our licence  will be highlighted  to indicate

the presence of these on list.

A subprocess  is executed  while running  this option.   It consists in

loading the diretories on a sublist.

This  subprocess  assures  that  the  File  Header  received during the

DIRECTORY process are  not repeated, so  that no duplicate  directories

are generated.  If the  presence of a duplicate directory  is detected,

this is indicated on screen but is not saved on the master file.

KEYS F6, F7, F8 (TIME ADJUSTMENT):

From these options depend the times of request of directories.  MSATCOM

performs all directory requests taking as reference the present time in

the MSATCOM.BUL, MSATCOM.ALL or MSATCOM.MY files.

These  files  will  be  updated  after  having  concreted  a request of

directory, for this the time of  the last File Header in the  server is

taken as reference.

Once updated, this time can be  altered by these options.  It  consists

in indicating the program the difference in hours of our GMT hour.

The format  of these  files is  HEXADECIMAL and  corresponds to time in

UNIX.  The  same will be  created for the  first time by  the programm,

taking as reference the present time of the programm.

KEY F9 (CHANGE COURRENT SATELLITE):

This  option,  of  immediate  access,  enables  to change instantly the

current satelite of the program.

In this way the configuration of the program can be changed for the new

satelite without  leaving it  to perform  changes to  the configuration

file by loosing time and making it's use complex.

KEY F10 (DOS GATEWAY):

Enables  to  performa  a  DOS  shell  to  execute  external commands of

programs to MSATCOM.

KEY ESC (ABORT & DISCONNECT):

This key enables to abort any operation started immediately  performing

a total resetting of the status of variables of the program.

KEY ALT-B (SEND BREAK):

Enables to send a break signal of  350 ms when we are in terminal  mode

with the target of putting the TNC in COMMAND mode.

KEY ALT-D (DCD LEVEL):

Enables  to  change  the  logical  state  of  the  DCD, this option was

introduced to enable the TNC's use which use as VALID DCD a high state,

being the Low state the commonly encountered in the TNC's.

KEY ALT-H (HELP SCREEN):

This option gives us access to a small onscreen help of the commands of

the programm.

KEY ALT-R (REBUILD DIRECTORY):

Used to reorganize the directory file of the current satelite.

This reorganization consists in cleaning the file directory of possible

duplicate directories by erasing at the same time the old ones after an

amount of days we indicate.

KEY ALT-T (TERMINAL/SERVER MODE):

By  this  option  we  can  switch  from  mode  SERVER  to mode TERMINAL

instantly and quickly.

KEY ALT-X (EXIT TO DOS):

End the execution of MSATCOM (Return to DOS).

--- ** ---

Configuration of file MSATCOM.CNF

-------------------------------------

As we saw at the beginning,  this file is in charge of  configuring the

system, it is  divided in four  sections (Serial SetUP,  Misc Function,

Top_Cmd_Start/End_Cmd_Setup  y  Top_Cmd_End/End_Cmd_Setup).   The first

and second one serve to send different commands to the proper  programm

and the others have as mission to send commands to the TNC to set it in

its initial and by leaving the program, the last ones can be configured

by the user.

Typical configuration

---------------------

In the first  section (Serial SetUP)  we find as  start the setting  of

speed for  communication of  the computer  to the  TNC, it is necessary

that both coincide in this speed, the  TNC in this case must be set  on

9600 Baud.  The length of  word, parity and the stop bit  must coincide

with the previous setting of the TNC.  There are three options for  teh

Handshake  and  these  are  the  following:   H (hardware, RTS/CTS) , S

(software, XON/XOFF) y N (none).  The recommended is the Hardware.

It is also possible  to set the communications  port to be used  in our

computer by the option Com_Port and it is necessary to do it indicating

an hexadecimal number, attaching in the following setting (IRQ_Com) the

corresponding interruption number, for example:

Port 1 = $03f8  IRQ4

Port 2 = $02f8  IRQ3

Port 3 = $03e8  IRQ4

Port 4 = $02e8  IRQ3

#

#              MSATCOM version 1.0  --  Configuration FILE  --

#

# Serial SetUP

#

#

Baud_Rate   = 9600

#

Word_Lenght = 8

#

Parity      = N

#

Stop_Bit    = 1

#

Handshake   = H

#

Com_Port    = $03F8

#

IRQ_Com     = 04

#

#

In the second section (Misc Function), we have both first  instructions

that correspond to the  tipe of video plate  that is used normally  Not

and Yes  are the  most used,  but sometimes  it is  necessary to change

NOT/YES for a better functioning depending on the video plate.

SYSOP_NAME:  as the word says, it is necessary to indicate the name  of

the system operator.

MYCALL: operator license.

GMT_ZONE:  thought to be used by the program with universal coordinated

time, taking for it the computer  hour, therefore we must take care  to

maintain the hour of our machine  and in this case we must  indicate if

our station is east of west of Greenwich as follows:  EST o WES

GMT_HOURS:  this command operated parallel to the aforementioned and it

is necessary to  indicate the time  difference that exists  between our

QTH and the Greenwitch time allways in positive numbers.

SATELLITE:  The  license of the  satelite to which  we want to  connect

must be  put, without  omitting the  number that  corresponds after the

dash  and  in  this  case  it  is  number  12,  that for all satellites

identifies the BBS during the  execution of the program can  be changed

of  satellite  (F9)  withouth  changing  the  configuration  DCD_LEVEL:

logical level of the DCD L (low) or H (high)

SORT_TYPE:  This instruction  is used by the  option F6 to perform  the

classification of the directory, in  this case it will be  performed by

time (TIME), this means it will be seen on screen since the most recent

date to the oldest.  It is also possible to make the classification  by

another parameter, for example SIZE (size).

# Misc Function

#

#

Snow_Check    = Not

#

Direct_Video  = Yes

#

#

Sysop_Name    = Ruben

#

Mycall        = LU6DYD

#

GMT_ZONE      = EST

#

GMT_HOURS     = 3

#

SATELLITE     = SIMSAT-12

#

DCD_LEVEL     = L

#

SORT_TYPE     = TIME

#

#

In the third section we have  possibility to put the TNC parameters  we

think adecuate to send at starting the program.  These parameters  will

be sent automatically to the TNC and can be modified by the user for  a

better operation with the satellite

*** Top_Cmd_Start

#

#

FULLD ON

FRAC 2

CONOK OFF

MAX 7

PAC 255

TXD 25

AXD 25

UN SIMSAT V SIMSAT-1

BT SIMSAT Earth Simulator of Micro Satellites - Buenos Aires ARGENTINA.

B E 5

M ON

CONMODE TRANS

#

*** End_Cmd_Setup

#

In the fourth section it is  identical to the third but the  parameters

are  sent  to  the  TNC  when  we  get  off  the  program (ALT-X) These

parameters can also be modified by  the user mainly those that use  the

same TNC for satellites operation and earth

#

*** Top_Cmd_End

#

#

FULLD OFF

CONMODE CONVERSE

MCOM ON

UN CQ

B E 0

BT %

CONOK ON

MON OFF

#

*** End_Cmd_Setup

#

--- ** ---

ERROR REPORT

Due to that the programm is  still being developed, it may be  possible

that some error occurs.  If  this happens, please pay attention to  the

following:

Some error in the momento of execution make that the program visualizes

an error message and ends.

Run time error: nnn at xxxx:yyyy

nnn       : Number of error.

xxxx:yyyy : Location of error (segment and displacement).

The errors in time of running are divided in 4 types:

Errors of D.O.S.               : Del 1 al 99.

Errors of Entrance/Exit (I/O)  : Del 100 al 149.

Critical Errors                : Del 150 al 199.

Fatal Errors                   : Del 200 al 255.

Please take note of this information and send it so as to evaluate  and

correct the possible error.

--------------------------------------------------------

*** ATENTION ***

Ham Radio:

-----------

For the distribution no charge is asked.  If you are owner of  MSATCOM,

please let me know so as to keep a statistic of use as well as any type

of error you may detect or suggestions to improve it.

Therefore, please mail via Packet Radio to:

LU4AEY @ LU4AEY.#VDP.ARG.SA Claudio M. Zanella

By satellite:

LU4AEY @ UOSAT3, PACSAT, LUSAT

By mail to:

Claudio  M.  Zanella

Av. Alvarez Jonte 3353 Piso 1 Dto  2  Villa del Parque

CP (ZIP) 01407 Buenos Aires - ARGENTINA. TE: 001-01-567-1239.

* * * * * * * *

ENJOY IT!

...  Claudio

#