===========                                         NOSview [137]

FORWARD.DOC

===========

Introduction

------------

This section of the NOS docs deals with the intricacies of mail

forwarding.  You should read and understand this documentation

thoroughly before attempting to forward mail through your NOS box

to the AX.25 BBS world, otherwise you might grossly misconfigure

your system and be the unhappy recipient of flames from BBS

sysops.

This section does NOT deal with the minutiae of the mailbox and

its various commands; it assumes that you understand concepts

such as user areas (both public and private) and how to list and

send mail. If you need help with these, please look elsewhere in

the NOS docs.

Apart from the usual DOMAIN.TXT and other files necessary for

ordinary functionality of NOS, three files are important in the

mail forwarding process. These are: N:\SPOOL\FORWARD.BBS,

N:\ALIAS and N:\SPOOL\REWRITE.  The contents of these will now be

addressed individually.

FORWARD.BBS

-----------

This file describes the actions taken by NOS in forwarding to

AX.25 BBSs.  The file contains a series of forwarding records,

each record being separated by a line containing two or more

hyphens.  The template for a forwarding record is:

-----------------------------------------------------------------

BBS callsign

Connection route

Connection commands                <zero or more lines>

List of areas to be forwarded      <one per line>

------------                       <end of record>

-----------------------------------------------------------------

BBS callsign

------------

This is simply the ordinary call of the remote BBS.  A typical

(but not random!) entry might be simply the line:

-----------------------------------------------------------------

sm0rgv

-----------------------------------------------------------------

The callsign may be followed, on the same line, by a comma-

separated list of valid intervals when forwarding is to take

place.  Each valid interval is a four digit number: the first two

digits are the beginning hour of the valid interval, the last two

digits are the final hour of the valid interval.

For example, if the first line of a forwarding record looks like:

-----------------------------------------------------------------

sm0rgv 0006,1414

-----------------------------------------------------------------

then forwarding to sm0rgv will take place only during hours

numbered 00, 01, 02, 03, 04, 05, 06 and 14.  Ticks of the mbox

timer outside of these times will not cause mail to be forwarded

to sm0rgv.  The default interval for forwarding is 0023.

Connection route

----------------

This is the method by which communication is to be established

with the remote BBS.  The first token on the line is the type of

protocol to be used.  This is one of ax25, netrom or tcp.

Following this is whatever further information the chosen

protocol requires to make the connection.

An example connection route for a simple ax25 connection on

interface tnc0 is:

-----------------------------------------------------------------

ax25 tnc0 g3dlh

-----------------------------------------------------------------

Connection commands

-------------------

Connection commands may, optionally, follow the connection route.

These take the form of a dot (period), followed by the command

which will be transmitted once the connection defined in the

first line of the connection route is established.

For example, suppose that we wish to establish a NET/ROM

connection with sm0rgv-2, through the netrom node #sth67. Then

the connection route and connection command portion of the record

would look like:

-----------------------------------------------------------------

netrom #sth67

.c sm0rgv-2

-----------------------------------------------------------------

Note that the dot would be placed at the beginning of the line;

it is placed here indented by one column simply so that gateways

which handle this message do not complain at having a line

beginning with a full stop; this convention is followed

throughout this documentation.

If the station is reached through digipeating, then the

digipeater callsigns should be in the ax25 route to the

destination callsign.  That is, if you wish to forward traffic to

w0ljf, using k2na as a digipeater, then you should have the line:

-----------------------------------------------------------------

ax25 route add w0ljf k2na

-----------------------------------------------------------------

in AUTOEXEC.NOS.

List of areas to be forwarded

-----------------------------

This is a list, one per line, of entries in the N:\SPOOL\MAIL

directory which will be forwarded to the remote BBS.  An entry of

the form:

-----------------------------------------------------------------

callsign

-----------------------------------------------------------------

will cause the file N:\SPOOL\MAIL\callsign.TXT to be scanned for

unread messages.  Any such messages are sent to the remote BBS

and deleted from the file.

One can also forward user areas using this mechanism.  To do

this, simply place a line containing the name of the area in the

record.  So, to forward amsat bulletins to the BBS, one would

have a line:

-----------------------------------------------------------------

amsat

-----------------------------------------------------------------

This will search the N:\SPOOL\MAIL\AMSAT.TXT file; any messages

contained therein which have not been forwarded to the BBS in

question will be forwarded.  They will NOT be deleted.

The determining factor as to whether or not entries are deleted

is that if the filename is present in the AREAS file, then there

is NO deletion, otherwise there is.

Please note that ONLY files in N:\SPOOL\MAIL are checked.  In

particular, the outbound SMTP mail queue is NOT checked.

Changing the recipient address

------------------------------

Normally, NOS uses the information in the To: header line to

determine the parameters used by the "S" command during BBS

forwarding.  As the To: header is unchanged by all ALIAS and

REWRITE machinations, the mail will be sent to the BBS addressed

precisely as the originator of the message typed it.

Occasionally, one might want to change this behaviour.  In this

case, a line of the form:

-----------------------------------------------------------------

area  newaddress

-----------------------------------------------------------------

in the list of areas to be forwarded will replace the originally

typed destination with the string newaddress instead.

ALIAS

-----

The alias file is used to map LOCAL names to other names, which

may be either local or remote.  Additionally, from a single input

message, the alias file permits one to produce multiple output

messages.

Thus, typical uses for the ALIAS file are: converting one local

name to another, converting a local name to a remote name, and

exploding a mail message so that it is passed on to several

recipients.

The format of a record in the alias file is very simple:

-----------------------------------------------------------------

aliasname       recipient1 recipient2 recipient3

<tab> or <SP>   recipient4 ... recipientN

-----------------------------------------------------------------

There is no separation between records in the ALIAS file other

than a newline.

The aliasname is a local username; that is, it does not contain

an "@" symbol.  When the alias file is processed, if the

destination of the message matches precisely the aliasname, then

the mail is redirected to ALL of the aliased recipients.

Scanning of the ALIAS file is performed by the SMTP server.  The

SMTP timer (which controls the SMTP client) is kicked whenever

the mailbox or SMTP server queues something for delivery by SMTP.

Mail transport within a single NOS system is performed through

the SMTP client/server mechanism.

The result of these facts is that as soon as a piece of mail is

entered to the mailbox, the SMTP client is kicked and attempts to

deliver the mail (which has already been scanned by the rewrite

mechanism - see below).  If the mail is local to the NOS system

(i.e. no "@" sign in the address), then the ALIAS file will be

scanned and the name mappings take place.

A few lines in the ALIAS file might look something like:

-----------------------------------------------------------------

bdale   bdale@n3eua

local   fred@k0yum bdale@n3eua bill@ai0c.co.usa.na

n5op@n5op jim@k0jtz n0esg@n0esg

g4bki   g4bki@gb7bil.2712.gbr.eu

-----------------------------------------------------------------

The system must know how to deliver traffic to each of the

individual addresses in the style in which they are entered in

the ALIAS file.  If the system does not know how to deliver one

of the new addresses, then it will send it to the SMTP gateway

station defined by the 'smtp gateway' command.

Note that it is reasonable, and sometimes desirable, to have

alias records of the form:

-----------------------------------------------------------------

area    area dest1 dest2 ...

-----------------------------------------------------------------

As the ALIAS file is scanned only once (see below), this does not

result in an infinite recursion.

REWRITE

-------

The rewrite file is used to perform a one-to-one mapping between

destination addresses as received by NOS and destination

addresses as actually used by NOS.

Each record within the rewrite file comprises a single line,

containing either two or three entries separated by spaces.

The first field is the template field; if a destination address

matches the template, it is replaced by the second field.

The third field, which is optional, is the single letter "r",

which, if present, tells NOS to rescan the rewrite file, using

the new destination address to attempt to match against the

templates.

A template may contain asterisks.  These stand for a match of any

number of characters (including zero).

In the second field, the character "$", followed by a single

digit in the range 1 to 9, represents the string that matched the

respective asterisk in the template.

By way of example, suppose that there is a line in the rewrite

file which looks like:

-----------------------------------------------------------------

*@* $1%$2@g1emm.ampr.org

-----------------------------------------------------------------

Then, any traffic reaching the system through the mailbox or the

SMTP server, but which is supposed to go to a remote system, will

be redirected to go through g1emm.ampr.org.

Suppose that a user logs on, and sends a message to n0gbe@nq0i.

Then the rewrite file attempts to match "n0gbe@nq0i" against the

entry *@*.  It matches, and assignes $1 the value n0gbe, and $2

the value nq0i.

The mail file as written to the disk will no longer be to

n0gbe@nq0i, but, rather, to n0gbe%nq0i@g1emm.ampr.org.  [The

nomenclature "station1%station2@station3" means the final

destination is station1@station2, and this traffic is to be

routed through the gateway station3].

As soon as a template match is found, the conversion is performed

and scanning is stopped, unless the third "r" field is present,

in which case scanning restarts from the top of the file.

N.B. It is a good idea to have a line of the form:

-----------------------------------------------------------------

*@*.ampr.org $1@$2.ampr.org

-----------------------------------------------------------------

at the beginning of your rewrite file.  This will cause all

amprnet traffic to be caught early in the rewrite scan, and no

further scanning (and, hence, no unexpected substitutions) will

take place.

Scanning procedure

------------------

The two files which are used to determine the disposition of

traffic are scanned under slightly different circumstances.  Note

that neither the ALIAS nor the REWRITE scan makes any actual

changes to the contents of the traffic.  In particular, the To:

field remains exactly as it was first entered into the system.

There are four possible entry routes for traffic into the system:

1.  SMTP

2.  through the mailbox by a user

3.  through the mailbox by a BBS

4.  via an external program (like BM) or creation of the files

manually.

NOS determines if a piece of traffic was entered into the system

by a BBS by looking for a BBS system ID (like the "[NET-H$]"

block issued by NOS) on the incoming connection prior to messages

being uploaded.

Traffic received by SMTP server

-------------------------------

1.  The rewrite file is scanned and any changes applied (unless

the traffic was received through the local mailbox; in that case,

this step does not occur);

2. If the traffic appears to be local then the alias file is

scanned and any changes or explosions applied.

3. Any copies local to the system are delivered; copies for

remote delivery are placed in the SMTP queue.

Traffic received by mailbox from user

-------------------------------------

1. The rewrite file is scanned and any changes applied;

2. The traffic is passed to the SMTP client.

Traffic received by mailbox from BBS

------------------------------------

1. The rewrite file is scanned and any changes applied;

2. The traffic is passed to the SMTP client.

Traffic entered by external mechanism

-------------------------------------

1. No scanning occurs;

2. The traffic is passed to the SMTP client.

Headers

-------

Appropriate RFC-822 headers are added to all incoming traffic.

Traffic entering through the mailbox receives a full complement

of RFC-822 headers; traffic coming through the SMTP server has

only a "Received:" header applied.

On forwarding to a BBS, if an item of traffic contains BBS R:

headers, the RFC-822 header is converted to an appropriate R:

line at the time that NOS forwards the message. (This change only

occurs for BBS forwarding; forwarding by SMTP retains the RFC-822

headers).

Bulletin Identifiers (BIDs)

---------------------------

The AX.25 BBS system has evolved a reasonably efficient way of

reducing overhead when forwarding bulletins.  When a bulletin is

originated on a BBS, it is given a unique bulletin identifier

(BID).  This BID should (theoretically) travel with the bulletin,

and should never be changed during the distribution of the

bulletin.

Each system keeps track of all received BIDs.  If a forwarding

station wishes to forward a bulletin to a BBS, then the receiving

station checks its local list of known BIDs and informs the

transmitting station if it already possesses the bulletin in

question.

The NOS mailbox conforms to this protocol.  Received BIDs are

stored in the file N:\SPOOL\HISTORY, and are encoded in the

Message-ID: header line of the message by NOS.  Messages

forwarded from areas listed in the AREAS file will have their BID

(re)generated from the Message-ID: line.

Note that ALL messages from public areas are forwarded with a

BID, whether or not the message was produced with the "SB"

command.

Like other BBSes, NOS will inform a transmitting station not to

transmit a bulletin if it is one that NOS already has locally;

likewise, it understands similar messages from other stations to

which it tries to forward.

Note that the BID mechanism is not a part of the SMTP world.  If

you are forwarding bulletins through SMTP, there is no mechanism

by which the receiving station can reject the attempted delivery

of a bulletin, even if it already exists on the recipient system.

(Note that a possible workaround is to deliver bulletins to

TCP/IP stations using TCP instead of SMTP.  Alternatively, one

could use NNTP, as NNTP commands utilise the Message-ID: line,

from which the BID is derived.)  The BID is preserved no matter

which mechanism is used to deliver the bulletin.

Traffic in practice

-------------------

Now, the big question is, how does one set up these various files

to perform intelligent manipulation of mail?  A number of

examples follow.  Note that, often, there is more than one way to

accomplish an objective.  The following are merely examples (and

not necessarily the most efficient method possible for any given

case).

The format used will be:

-----------------------------------------------------------------

typed destination -> intended destination

-----------------------------------------------------------------

followed by the necessary entries in the ALIAS, REWRITE and

FORWARD.BBS files.

Using familiar names - SMTP destination

-----------------------------------------------------------------

bdale -> bdale@n3eua.ampr.org

alias:

bdale   bdale@n3eua.ampr.org

rewrite:

forward:

-----------------------------------------------------------------

Exploding local mail

-----------------------------------------------------------------

sysops -> nq0i, n5op@n5op.ampr.org

alias:

sysops  nq0i n5op@n5op@ampr.org

rewrite:

forward:

-----------------------------------------------------------------

Using familiar names - BBS forwarding

-----------------------------------------------------------------

g4bki -> g4bki@gb7bil.2712.gbr.eu, to be forwarded by ai0c

alias:

rewrite:

forward:

ai0c

ax25 ax1 ai0c

g4bki g4bki@gb7bil.2712.gbr.eu

ai0c

-----------------------------------------------------------------

Handling incoming bulletins by subject

-----------------------------------------------------------------

tcpip@* -> nq0i, tcpip, bdale@n3eua.ampr.org, ai0c@ai0c [a BBS]

alias:

tcpip   nq0i tcpip bdale@n3eua.ampr.org ai0c

rewrite:

tcpip@* tcpip

forward:

ai0c

ax25 ai0c

ai0c

-----------------------------------------------------------------

Let's walk through the above example.  An incoming item comes in

addressed to TCPIP@ALLUS.  A scan is made through the REWRITE

file, and a match is found.  The item is redirected to tcpip.

The ALIAS file is scanned; a total of four copies of the item

exist after this, three in local areas tcpip, nq0i and ai0c, and

one on the SMTP queue (for bdale@n3eua.ampr.org).  When the

mailbox timer next ticks, the mail in the local ai0c area will be

forwarded on the ax1 interface to ai0c.

Routing based on Hierarchical addressing

-----------------------------------------------------------------

Wyoming -> KE7VS (SMTP)

Nebraska -> AG0N (BBS over the NETROM, NETROM ID WNBBS)

Europe -> W0LJF (BBS over AX.25)

alias:

rewrite:

*.noam            $1.na r

*.us              $1.usa.na r

*.usa             $1.usa.na r

*.ne              $1.ne.usa.na r

*.wy              $1.wy.usa.na r

*@*.*.wy.usa.na   $1%$2.$3.wy.usa.na@ke7vs

*@*.wy.usa.na     $1%$2.wy.usa.na@ke7vs

*.ne.usa.na     ag0n

*.eu            w0ljf

forward:

ag0n

netrom tnc0 wnbbs

ag0n

----------

w0ljf

ax25 ax1 w0ljf

w0ljf

----------

-----------------------------------------------------------------

Why is the example REWRITE file apparently so complicated?  This

is to handle poorly constructed hierarchical addresses in a

reasonable way.

A full US hierarchical address has the form:

callsign@BBS.#localid.state.usa.na.

Many states have no #localid field.  In the example REWRITE file

above, the first three lines convert non-standard, but frequently

used, US designators to the more standard format.

It is common for users not to use a full hierarchical address if

the destination is relatively local.  For example, a user might

easily use only .wy instead of the full .wy.usa.na if he is

geographically close to Wyoming.

The second grouping of two lines handles this problem.  Note the

third, "r", field in all the entries so far.

The remainder of the file handles properly formatted hierarchical

addresses.  The two Wyoming entries handle the cases with and

without a #localid field.  Differentiation between these cases is

not necessary for BBS forwarding.

General bulletin handling

-------------------------

The details of bulletin handling will vary somewhat from place to

place, as there are several distinct styles of bulletin handling

currently in use in the AX.25 BBS world.  In general, it is

necessary to arrange one's system so that it accepts bulletins

from BBSes, forwards them to one or more stations, and also

handles intelligently bulletins input by users into NOS.

Suppose that we wish to handle bulletins @JUNK.  We are to

deposit them locally in the junk area, and also forward to BBS

g4bki.  We also know that we generally receive @JUNK bulletins

from g4amj, a local BBS which handles much bulletin traffic.

-----------------------------------------------------------------

alias:

rewrite:

*@junk   junk

forward:

g4bki

ax25 ax1 g4bki

g4bki

junk

----------

g4amj

ax25 ax1 g4amj

g4amj

junk

----------

-----------------------------------------------------------------

All incoming @JUNK traffic is written to the junk area (which

should be an explicit entry in the AREAS).

Each tick of the mailbox timer, NOS scans the junk area for

traffic not forwarded to g4bki or g4amj and attempts to deliver

unforwarded bulletins.  Usually, g4amj will respond with a "Have

it" message and the bulletin will not be forwarded.  Any

bulletins @JUNK deposited locally by users will automatically be

sent to both g4bki and g4amj.

Questions and Answers

---------------------

Q.  Under what circumstances does NOS request reverse forwarding

from a BBS?

A.  NOS requests a reverse forward after completing any forwards

of its own to the BBS.  If no traffic was queued for a given BBS,

then no connection is attempted, so no reverse forward request is

issued.

Q.  What kinds of message types does the NOS mbox support?

A.  Basically, NOS supports all two letter commands starting with

an "S".  If the mailbox has not received an SID banner (the

"[NET-H$]") from a connected station, then an SF command will

send a followup to the address specified on the command line.

The SR command will send a reply to the current message. One can

also issue the command "SR <number>", where <number> is the

number of the message to which you want to generate a reply.

All other variations cause an X-BBS-Msg-Type: header to be added

to the message.  When a message with such a line is forwarded to

a BBS, it is sent to the BBS with the appropriate message type as

the second letter in the "S" command to the BBS.

If NOS has received a valid SID, then ALL S commands are handled

by the X-BBS-Msg-Type: mechanism outlined above.