N  R  D

A Control Program / Database for use with Japan Radio NRD525/535 Radios

by Tom Whiteside

NRD is a control program / database for NRD525 and NRD535 receivers and

requires an IBM or compatible computer.  This program is distributed as

"Freeware" as a token repayment for the valuable assistance received over the

years from other DXers in the form of articles and log reports.  Included also

is source code (for Turbo Pascal) so you can adapt this to your own style (or

convert existing computer database files over to NRD).

Note:  The enclosed database files are intended as examples, not definitive

Short Wave databases by any means!  This is intended as a tool to

enhance your own logging productivity and enjoyment.  This program

can import data from Tom Sundstrom's English language database for

subscribers to this (described later) and his IS a definitive database.

Note:  There are many commands and features in NRD that take some practice.

I recommend printing this and trying the various commands in the

documentation with the program.

PHILOSOPHY:

I got tired of using paper for logging many moons ago.	With paper logs, its

tough to go back and find where radio xyz is, etc.  Much continues to be

written on ways to organize paper records -- no thanks!!!  The first step was

logging on paper, then entering the data in a Spreadsheet.  Now, things could

be found but this is tedious!  This program eliminates the middle person!  The

next step was achieving the right balance between controlling the radio

manually and from the keyboard.  Basically, you can do either with this

program.  This program does not use the radio's memory channels (essentially

the database can be thought of as memory channels only with comments).	Sloth

is the watch-word throughout.  For example, to log a station, hit the "L" key

and the program will automatically grab the frequency, date, time (and the rest

of the receiver state) leaving you to enter only the ID and Location.  While

you don't have to use the program to search for stations, keys like "+" and "-"

can be used to tune in 5khz increments.  The program will follow along pointing

at records for this frequency.	If you've been tuning manually, hit the "*" key

to find the entry for this frequency.  Other keys bump the bandwidth or mode.

SETUP:

The program defaults to COM1:  for the serial port.  If you need to change this

delete the CONFIG.DAT file and rerun the program.  This causes NRD to request

the desired COM port and other information described below.  If you specify "0"

for the COM port, the program will run in demo mode.  You will be able to see

the database but none of the radio commands will operate.  The program uses

1200 baud for the 525 and 4800 baud for the 535 to communicate with the radio.

(To determine the baud setting on the radio, hold down the "MEMO" key and hit

the "8" key on the Radio keypad.  The radio should display "1200" on the

frequency readout.  If it shows "300", press "MEMO" "8" again.

Note:  The time offset from GMT is also contained in CONFIG.DAT.  Delete

CONFIG>DAT if you are not on Central Standard Time.  NRD will prompt you

for the COM port, etc.  and rebuild this file.

Note:  When you run the program after deleting CONFIG.DAT, you will be asked

if you have a KIWA MAP Unit.  If you don't have one, simply say no.

Saying "yes" will enable a "K" for "KIWA" command described later. (This

feature is only available with the NRD525.

Note:  NRD assumes you have a color monitor.  To use with a monochrome display,

delete the file @MONITOR.DAT.

RUNNING NRD:  Type:  NRD and hit enter.

COMMANDS:

These will be described in two sections:  CONTROLLING THE RADIO and DATABASE

COMMANDS.

CONTROLLING THE RADIO:

This program allows you to control the radio either manually or with the

computer keyboard.  To do this, mostly the radio is kept "unlocked" by the

program.  This philosophy means that when you change the radio, the program

does not know about it and the receiver display will not automatically reflect

changes.  (Note:  for the 535, this is not true, The 535 allows the program's

view of the receiver status to be continuously updated without locking out the

receiver controls.) However, all radio and database commands that involve

knowing the status of the radio will automatically check the contents of the

radio before proceeding.  For example, the "+" key described below that

increments the radio's frequency by 5 khz will check the actual radio frequency

before proceeding to increment it.  If you wish to update the radio status

display at any time, hit the SPACE BAR.  Changes since the last update are

always highlighted in RED.

CONTROLLING THE RADIO COMMANDS:

"+" : increments the frequency by 5khz.  If the mode is USB or LSB, this is

first done off a khz to assist in hearing weak station heterodynes.

Note: if the database is sorted by frequency, the highlighted database

entry will automatically follow the frequency.

"-" : decrements the frequency by 5khz.  If the mode is USB or LSB, this is

first done off a khz to assist in hearing weak station heterodynes.

Note: if the database is sorted by frequency, the highlighted database

entry will automatically follow the frequency.

"<" : decrements the receiver "mode".  eg LSB to USB.  "," works as well so

no shifting is needed.

">" : increments the receiver "mode".  eg USB to LSB.  "." works as well so

no shifting is needed.

"[" : decrements the bandwidth.

"]" : increments the bandwidth.

" " : (space) refresh the receiver display.  Useful if radio has been changed

manually. (Not necessary for the 535 except for channel changes).

"K" : for "Kiwa" - If in LSB or USB, puts the radio in AM with WIDE bandwidth

(and detunes by 2 khz).  If in AM, puts radio on nearest 5 khz center

and puts in LSB or USB (depending on the offset) and sets the bandwidth

to INTER.  Used to toggle high fidelity listening for use with KIWA MAP

unit.  Unless you answer "yes" to has KIWA MAP unit in building

CONFIG.DAT file, this command will not appear or work.	This feature is

not supported for the NRD535.

"R": for "s-meteR" - applies only for NRD535 users.  Toggles mode of

displaying the S-Meter reading in the receiver status.  Default is OFF.

This mode causes an annoying synthesizer (I assume) induced noise in

USB/LSB/CW/RTTY modes hence the option.

GRAPHICS COMMANDS:

For users with NRD535s and with graphics hardware, there is a G(raphics

option.  Most graphics hardware should work.  Testing included VGA, EGA,

and CGA.  The following adapters should work but were NOT tested:  MCGA,

EGA64, EGAMONO, HERCMONO, and PC3270.	This code requires reading the radio's

S-Meter which is not possible on the NRD525 and hence does not show up on the

menu for the 525.  From the 535 command menu, select:

"G" : for "Graphics.  Selecting the Graphics option produces the following

command menu:

GRAPHICS:	C(lear, T(ime plot, S(pectral plot, Q(uit

These commands are as follow:

"C" for "Clear"

Clears the graphics screen.

"T" for "Time plot"

Produces a time domain plot of the S-Meter output.  This is useful

for watching signals fade in and out over time.  Units are in dB

and calibrated to the NRD535 S-Meter.  Hitting the SPACE BAR will

stop the plotting and return control to the Graphics menu.

"S" for "Spectral plot"

Produces a frequency domain plot of the S-Meter output over a user

specified frequency range.  You will be prompted to enter the

start and stop frequencies.  The program defaults to -/+ 5khz off

the current frequency if you wish to hit enter without typing a

number for the start and stop frequencies.  The sampling is done

in CW mode using the bandwidth control (BWC) set to 500Hz. The

following command prompt is displayed when doing Spectral plots:

COMMANDS: A(verage, C(lear, Quit

Hitting "A" for "Average" will plot the average value for each

frequency.  Typically, after sampling a while, this is useful

to see signals that might otherwise be in the mud and to filter

out lighting and modulation effects.  A running average is

maintained that wieghts each sample equally.  "C" for "Clear"

clears the screen but does not stop the sampling or reset the

statistics used in Averaging.  "Q" for "Quit" returns to the

graphics menu.

The radio will be returned to the settings it had prior to using

this command when either the plot finishes or when you hit a key

to stop it.

"Q" for "Quit"

Returns control to the main menu and leaves graphics mode.

DATABASE COMMANDS:

This program allows for a set of up to 20 separate databases.  These are

selected with the Journal command (described later).  One database is displayed

at a time.  The status line at the bottom of the display will show the "active"

log.  When you switch logs, the program remembers the previous log which is

shown as the "inactive" log.  Commands described below allow hot keying between

these logs and copying data from the inactive log.  A highlighted line shows

the position in the database.  Cursor control keys move about in the database

as do "pageup" and "pagedn" keys.  "Control-pageup" and "control-pagedn" keys

page by 10 pages at a time.  The "home" key takes you to the top of the

database.  Likewise the "end" takes you to the end.  The following commands

affect the "active" log.

"L" for "Log":

"Logs" a new entry in the "active" database.  Time stamp and radio contents

are provided automatically.  User is placed in "Edit mode" at the call

sign location.  Enter the call sign and hit the "tab" key to enter the

location.  Hit the "tab" key again to enter a comment.

Note: This comment field is contained on a different "page".  (See the

"Page" command).

"C" for "Confirm":

"Confirms" and existing log entry.  Date is updated to current date.  If

time is outside "Start" and "End" time fields, these are changed.  The

contents of the radio are polled and this data is updated.  (Particularly

valuable for drifting stations).

Note: The time algorithms are rather simple minded.  Mostly they do the

right thing.

"T" for "Tune":

"Tunes" the radio from the highlighted database entry.

"*"  This command finds the first database entry with the same (or higher)

frequency that the radio is set to.  Sort of the opposite of the "Tune"

command.

"E" for "Edit":

Because the program uses commands, you cannot just type over database

entries without using this mode.  The edit field will be highlighted and

the entry can be changed.	"Home" takes you to the beginning of the field;

"End" takes you to the end.  The "insert" and "delete" keys work.  When

finished, hit "enter" or use a "tab" key to bounce to the next field.

"P" for "Page":

Each database entry is displayed horizontally over three "pages".  Hitting

the "P" key toggles between these three.  Cursor and Tab keys can be used

as well.

"D" for "Delete":

This removes an entry from the database.

"N" for "uNdelete":

Reverses accidental deletions.  Since subsequent "loggings" use up deleted

entries, this is best used immediately after an accidental delete.

"S" for "Sort":

The database can be sorted in ascending order with any two fields.  For

example, by frequency and by time.  In this example, the database will

be sorted by frequency with all stations with a common frequency listed

in time order.

Note:  For automatic searching on the receiver frequency (see "*", "+",

and "-") to work properly, the database should be in frequency

order.

Note:  This only sorts the active log.  Different logs can be sorted in

different ways.

"M" for "Mark":

Marks the current highlighted line.  Displayed in YELLOW.	Moving the

cursor to another line and pressing "M" again marks a range of lines.

See the Journal command for usage.

"U" for "Unmark":

Cancels any marked lines.

"H" for "Help":  Type for help on using this program.

"/":  Selects alternate command prompt.  All commands are always active.

"A" for "Alternate":

This command allows hot keying between the active and inactive database.

This will have no effect if there is no inactive database.  (The active

and inactive databases are displayed on the status line at the bottom

of the screen).  This is particularly useful for switching between

a station log and a target log.  The "A" key was selected for its

keyboard position.

"W" for "Write":

This command will "write" the log data from the highlighted line for the

"inactive" (not seen) log to the highlighted area for the active log.

To illustrate how the "A" and "W" commands might be used, consider this

example:  It is dawn and you are using your "INDO TARGET" log to search

for new stations.	You find one!  You use the "Confirm" command to update

the time and date in the target log and use the Journal command to bring

up the log you use to keep confirmed catches.  To save time, you don't

bother to find the "proper" place in this log for this station (you can

sort it later).  You hit the "W" for "Write" key and the entry appears.

Now hit the "A" for "Alternate" key and you are back in "INDO TARGET".

Find another one?	Confirm it and just hit "A" for "Alternate" again to

toggle back to the main log and use "W" again.

Note: This creates a new entry and does not overwrite data.

Note: The Journal "Read", "Write" and "Move" commands may also be used.

"J" for "J(ournal"

This command will display the various log choices and its own command

menu:

Journal: S(elect, C(reate, D(el, I(mport, R(ead, W(rite, M(ove, P(rint, Q(uit

These commands will now be described:

"S" for "Select":

Select a new active database.	You will be prompted to enter the

NAME of the database you which to view.  (Note: Enter the name for the

database, NOT the number).   This new database becomes the active

database and the last one becomes the inactive one.

"C" for "Create":

Creates a new, empty database.

"D" for "Delete":

Deletes a database.

"I" for "Import":

Imports data from another database:  Currently, only one supported is

Tom Sundstrom's English language database.  To accomplish this, copy

the file SWSKED.DBF into the NRD subdirectory and select "J" for

"Journal" then "I" for "Import".  This file is NOT included and must

be purchased from TRS Consultants.  To receive more information on

Tom's  program, send a #10 SASE to Thomas R. Sundstrom PO Box 2275,

Vincentown, NJ 08088-2275, USA or FAX (609) 859-3226 or call the

Pinelands BBS on (609) 859-1910. .  Tom offers an update service.  MY

PROGRAM ASSUMES THAT YOU HAVE COMPLETED THE UPDATE PROCEDURE HIS

DOCUMENTATION CALLS FOR PRIOR TO USE.

"W" for "Write":

Writes marked lines to database you select.

"M" for "Move":

Like "Write" but deletes the marked lines from the source log after

copying them to the new log.

"P" for "Print":

Prints a copy of the database in compressed print to LPT1:

ABOUT THE DATABASE SAMPLE:

I include my personal log (called, somewhat unimaginatively "LOG") as a sample.

In practice, I use this to record stations as I hear (or rehear) them and use

other logs for specific target areas.  You can very quickly tune through a list

of INDOs at dawn by creating a special target file, for example and it's easy

to copy entries from one database to another.  You will probably want to start

over with your own log.  If you do some programming and have a computer log in

some other format, it's not hard to map over using the included source code.

(My original database was Lotus 1-2-3).

MICROSOFT WINDOWS:

NRD can be used with Microsoft Windows.  I use NRD in one window and the

wonderful GEOCLOCK program in another.	(See NASWA Listener's Library May 1990

for John Bryant's review on GEOCLOCK -- get this program and register it in

order to get all the extra maps!!!) The file NRD.ICO is an icon for use with

windows.

PERFORMANCE HINTS:

Using Disk Cache programs really help.	As you delete and add entries over

time, the database will become fragmented and you may notice some degradation.

To remedy, first back up the data files, then mark all entries (put the cursor

on the top line and hit "M" for "Mark", go to the last entry and hit "M again

-- all lines should be Yellow now if you have a color monitor).  Next hit "J"

for "Journal", Create a new database ("C" for "Create), and select "W" for

"Write".  When the program asks for "Log to write to:", enter the name of the

database you just created.  This process copies the old log to the new one and

eliminates any fragmentation.

OTHER RADIOS:

All routines for the receiver control are isolated in the NRDIO.PAS file and

the intention was to minimize receiver dependent code to simplify the task of

porting this to other receivers.

FEEDBACK PLEASE:

I'm interested in feedback on the program and ideas for how to make it better.

If you make changes to the program, please comment your changes well and upload

with a name that distinguishes it from NRD so we don't get confused.  I can be

contacted at:

Tom Whiteside (512) 258-5924

11505 Oak View

Austin, TX 78759

I will also be on the Pineland's BBS (609) 859-1910 from time to time.  Hope

you enjoy NRD!

Fixes:

Rev 2.1  12-02-90  Oops, original upload had the offset to GMT hardwired

for Central Time Zone.  Offset is not stored in the

Config.dat file.  Fixed a minor bug where homing

left the cursor at the bottom of the page rather than

the top.

Rev 2.2 12-25-90   Added feature to allow com port 0 --- allows demoing the

program if you don't have an NRD525 or a working serial

link to it.

Rev 2.3 12-26-90   Improved error recovery if receiver is off

Rev 2.4 03-01-91   Embarrasment preventing use with monochrome monitors

fixed.

Rev 2.5 03-23-91   Added Import command to Journal.  Allows subscribers

to "import" Tom Sundstrom's English Language data

to an NRD journal entitled: "SWSKED"

Rev 2.6 03-30-91   Added concept of "Active" and "Inactive" logs and the

ability to write between them.  "*" command added.

Fixed potential hang condition in comreadln (com port

read line).  Deleted non-useful and dangerous Read

command from Journal.

Rev 3.0 04-26-91   modified for 535

Rev 3.1 04-27-91   Fixed program hang if user incorrectly enters the db

name in "Journal".  Added continuous update of receiver

status display for the 535

Rev 3.2 05-12-91   Added S-meter to 535; changed mode order to AM -> ECSS_U;

Added graphics command for 535; misc bug fixes for 535;

Rev 3.3 05-19-91   Enhancements to graphics - commands for spectral and

time displays.  Performance enhancement to spectral plot