PCTRACE

Version 3.002

Users Manual

July 29, 1990

by

Douglas E. Ehlers

Copyright (C) 1989, 1990

2

Table of Contents

Notice of Copyright.......................................3

Introduction..............................................4

Installation..............................................5

System Requirements..................................5

Software Setup.......................................5

Input/Output Menu.........................................6

Board Definition Menu.....................................7

Chip Definition......................................8

Chip Placement.......................................9

Interactive Router.......................................13

Mouse use...........................................16

Autorouter...............................................16

Board Statistics.........................................18

Printer Menu.............................................18

Exiting the program......................................18

Brief Tutorial...........................................19

Advanced PCTRACE design..................................21

RPRINT Subprogram........................................23

Keyboard control summaries...............................24

Order Form...............................................25

3

Notice of Copyright

This  program is  protected by  a copyright  held by  me and

filed with the United States  Government.  Although this  program

is  being distributed as Shareware,  that does not  make it free.

You are  granted limited use of  this program to test  it out and

see if it will fulfill your  needs.  If you find that  your needs

can be  satisfied by this program  and you decide to  use it, you

are  obligated to pay the 75 dollars  to register your copy.  You

are  allowed and  encouraged  to give  unaltered  copies to  your

friends to test.  This same agreement applies to them.   Business

organizations  and  education  institutions  must  pay  for  this

software if it is in use.

Note:     This  program  was originally  marketed under  the name

PCROUTE  but  for  copywrite  reasons  has  been  named

PCTRACE.

4

Introduction

PCTRACE  is  a computer  aided  design  program for  printed

circuit boards.  It contains board layout routines that should be

quite  adequate for  most  purposes.   It  also contains  a  full

featured autorouter  that averages around a  93% completion rate.

It  also supports  four  of the  more  popular printers  for  its

output.

This program is, to my knowledge,  the first of its type  to

be  released  as shareware.   Although  it  lacks several  of the

features of similar products  that are commercially available, it

also lacks a $1000 price tag.

Key features include:

1.   Autorouting

2.   Graphic interactive routing

3.   Board layout functions

4.   Flexible pinouts for devices

5.   2X and 1X output for use in making PC boards

6.   Disk functions built into the software

7.   Sophisticated component and connection

entry systems

8.   General statistics

9.   Up to 300 components and 1800 connections

10.   Mouse support in the interactive router

11.   Variable pad and trace sizes

12.   Single and double sided boards

13.   Manual placement of traces

14.   The authors continued support

5

Installation

Contents of Archive/Disk:

PCTRACE .EXE -  The main program

RPRINT  .EXE -  The graphic printer output program

PCTRACE .DOC -  The documentation for PCTRACE.EXE

TEST    .CDT -  Brief tutorial example

EDGE    .CDT -  Advanced board in second example

DIP24V6 .DEF -  24 PIN DIP chip definition-vertical .6 wide

DIP20V3 .DEF -  20 PIN DIP chip definition-vertical .3 wide

DIP40V6 .DEF -  40 PIN DIP chip definition-vertical .6 wide

DIP40H6 .DEF -  40 PIN DIP chip definition-horiz. .6 wide

PAD     .DEF -  Definition of a single pad

2PAD50V .DEF -  2 PADS 0.5 inches apart-vertical

works well for 1/4 watt resistors

2PAD15V .DEF -  2 PADS 0.15 inches apart-vertical

2PAD10V .DEF -  2 PADS 0.10 inches apart-vertical

PRINTDOC.BAT -  Prints the documentation stored on disk

System Requirements:

To run this program requires:

1. CGA, EGA or better graphics

2. 640k of memory with about 540k free

3. 1 floppy disk drive

4. One of the following printers:

a. IBM PC or compatible printer

b. EPSON LQ-2500 or compatible printer

c. Hewlett Packard Laserjet II or compatible

d. Adobe Postsciprt Compatible printer

e. HP Deskjet or Deskjet +

5. Hard-Disk (not essential but recommended)

6. Mouse (supported but not essential)

A fast processor and math coprocessor are also beneficial.

Software Setup:

If  you are  going to  run this  program from a  floppy disk

system, simply  copy all the files that came in the archive or on

the  distribution disk  onto an  empty formatted  floppy  disk by

using the following command:

COPY A:*.* B:

Where A: contains the  original PCTRACE disk and B:  contains the

empty  formatted floppy disk.   All  the files  that came  on the

diskette  are required to run the  program except the PCTRACE.DOC

file.  The PCTRACE.DOC file may be deleted from the run disk.

If you will be  using a hard disk to run the program, make a

directory  in  which to  place the  program.   Use  the following

6

commands to accomplish this:

1.   cd \

2.   md pctrace

3.   cd pctrace

4.   copy a:*.*

Where A: holds the original PCTRACE files.

For PCTRACE to run  reliably, you must  also have a line  in

your  config.sys file setting files=20.   If you  are running any

major applications program this  command probably exists, but you

should check.  To execute  the program type PCTRACE  from the DOS

prompt.

Input/Output Menu

1. Load Data:

This  function  is  used  to  retrieve  from  disk  the

necessary  information to  design a  printed circuit  board.

Select this function  from the menu and when prompted, enter

the file name.   No extension is used, the  program will add

the .CDT extension for you.   The drive and path should  not

be used on  the filename itself.   There is a  function that

will  be discussed  later to  set the  drive and  path name.

After entering the filename, press Return.  The program will

ask if this is the correct name, enter Y or N to answer.  If

the file is not found you will have a chance to re-enter the

file name.  You can also use the ESC key to jump back to the

main menu instead of entering the file name.

2. Save Data:

This function  will save a complete  description of the

design layout, component list, and connection list.  It does

not save  the information  used for  deleting traces  in the

interactive  router which  will  be discussed  later.   When

prompted, enter the filename.  Again do not specify the file

extension, drive  designation, or  path name.   PCTRACE will

ask  after entering the file  name if everything is correct,

enter a Y  or N.  If the file exists the program will ask if

you want  to overwrite the  file.   If you say  no, it  will

prompt for a new file name.  ESC also works to return you to

the I/O menu instead of entering the file name.

3  Write Autocad file:

This function will  write a  file that can  in turn  be

read  into and edited with Autocad.  The process is one way,

you can not  read an autocad file  back into PCTRACE.   Call

this  function  from  the I/O  menu  and  give  a name  when

prompted.  Here as usual, specify no path or file extension.

7

The extension  .scr will  be added  to the  file.  Once  the

program  is done writing the  file, you may  run Autocad and

use  the  command  "SCRIPT"  from   the  "Command:"  prompt.

Autocad will ask for the script file name.  Specify the same

name  (with path if needed)  as the one  you used in PCTRACE

and Autocad will read in the file.  It may take awhile so be

patient.    Autocad  will  display  three  layers:   solder,

component, and silkscreen.   You may  edit them however  you

like and save  them as  Autocad drawing files.   Doing  this

greatly increases the  number of printers you  can output to

as Autocad supports mega-hardware.

4. Erase Data:

This  routine clears  the memory  of the  program.   It

erases the boards, component list, and connection list.  The

program will  ask if you are sure that you want to erase the

data in  memory.  Answer Y  or N.  The  program will respond

with "Erasing..." and after  a few seconds drop you  back to

the I/O menu.

5. Disk Directory:

This function displays a directory of the current drive

and path.  Use the space bar  to view twenty file names at a

time.  When  no more files exist, the space  bar will return

you to the I/O menu.

6. Change drive/path:

This function  changes the  current drive and  or path.

The  ESC key  will  back you  out  of the  function  without

changing the  path.  The  function will display  the current

drive and  path and then prompt  for a new one.   Just enter

the  new one and press Return.   You will be returned to the

I/O menu.

Define/Edit Board Menu

1. Component Edit/Entry:

This  function is used to enter  components into the system.

Once  you have  entered  this function  you  will placed  in  the

entry/editing environment.   If  no components have  been entered

you  will see only two possible commands, if there are components

in memory you will  have the option  to delete, copy and  perform

other functions.

New Component Entry:

Hitting an "A" will allow you to enter  new components.  The

program will display the component number being entered  and then

prompt for the  name.  The name  is up to 15  characters long and

serves only  to remind the designer  of what the device  is.  ESC

can be  used at  this point  to return  you to  the entry/editing

environment.

8

The  next  question is  for the  designation.   This  is the

unique  name that  you will  give the  component and  use in  the

future to refer  to it.  Typical names include  IC1, R1, IC2, and

A1.  The only reserved designation  is for vias. (A via is a  pad

with no device  attatched to it.  It is just  to connect a  trace

from one side of the board to the other)  Designations that start

with "V"  and then have a  number after that, are  not allowed in

the system.   So V1 would not  be accepted by VI1 would  be.  ESC

may be  used to back  up while entering  the designation  name to

reenter the component name.

Third the program will prompt for  the definition file name.

This is the name of a file that tells the program how the pins on

the device being entered are situated.  The program first shows a

list  of possible definition  file names at the  lower right.  At

the end of the  list the program will prompt for a  name.  Do not

enter a file extension, the program will add it for you.   If the

name  of the file entered exists, it  will read the file for you,

and display the chip definition for  you to see if it is correct.

If you answer  yes that  it is  the correct  definition you  will

continue to the  next question.   If the file  did not exist  you

will  be asked  if you  wish to  create the  definition.   If you

answer no,  the program  will re-prompt for  the definition  name

after  showing you the possible  files. Otherwise it  makes a new

definition.  ESC may be used to back up and reenter the component

designation.

Creating New Definition Files:

The program needs to know  the exact pin layout of

any  component  placed on  the  board.   This  function

allows you to create these definitions.  Upon entry you

will  see a graphic screen with some information at the

bottom  and a green square in the middle of the screen.

In the top left of the box is your cursor.  You use the

cursor  keys to move  the cursor  around to  define pin

locations.  The pins are entered in the order from 1 to

up to 70.  Move the cursor to the position of the first

pin and  hit return  to  place it.   Then  move to  the

second position and hit return to place pin 2.  You may

also  remove pins in the reverse  order you placed them

by positioning  your cursor  over the last  pin entered

and  hitting the  backspace key.   This  key  will only

delete the  last entered pin.   The information  at the

bottom of the  screen will  tell you what  pin you  are

entering,  as  well as  its  relative  location on  the

screen.  When you  have placed 70 pins or  are finished

with the component definition, hit  ESC to finish.  The

program  will ask if you want to reedit.  Answering yes

will  clear  the screen  and you  can  start over.   By

answering  no you will receive the prompt asking if you

want to keep the definition or not.  Answering yes will

save the definition for later use, no will give you the

list of  existing definitions on the  previous screen.

9

All components are moved to  the upper left most corner

for  saving.   This  is for  internal reasons;  you can

design  the chip anywhere in the square.  You will need

to  make definition files  for all your  components.  I

would suggest  a  meaningful system  for  naming  them.

Maybe something like DIP16V and  DIP16H for 16 pin Dual

Inline   Package   Vertical  attitude   and  Horizontal

attitude respectively.

***Note:   Most chip  dimensions are available  in data

books such as RCA cross reference manuals.  Radio Shack

also  puts  device  dimensions  on the  back  of  their

packages.

The fourth prompt  is for the  pad size for  the device.   A

small window will  pop up and prompt for a  pad size between 0.05

inches  and 0.07  inches.   All  pads  are circular.    Enter the

corresponding number to the pad size you wish.

The next question the  program will ask is if  everything is

correct.    Answering  no  will restart  this  process  with  re-

prompting for the name.  Answering yes will get the next prompt.

The  next prompt  is  if  you  wish  to  keep  this  device.

Answering  no will skip  the next prompt  and ask if  you wish to

enter another device.  Answering yes gives you the next prompt.

This  prompt asks  if  you wish  to  place the  device  now.

Entering no  will  allow you  to  delay this  until later.    All

devices must be placed somewhere on the board before they  can be

connected.  Placing a chip goes as follows:

Placing Chips:

When  this graphic screen comes up  you will see a

representation of the  current circuit  board with  all

other placed  components and ground planes  shown.  The

chip you are entering  will be shown in the  upper left

corner of the  board.   The arrow keys  will move  this

chip  around  on  the  board. By  using  a  shift-arrow

combination  the chip  can be  moved faster.   Move the

chip to where you want it and press return.  You cannot

overlap  chips.    The  chip  is  now  placed  in  this

location.  If you change your mind you can press ESC to

exit without placing the chip.  You will be prompted if

you are sure you want to abandon chip placement or not.

Yes  will go to the next  prompt, no will put you right

back where you were before hitting ESC.

The last prompt is if you wish to enter another chip or not.

Answering yes  will put you back  up at the name  prompt to enter

another  chip.   No  will  put  you  back  in  the  entry/editing

environment.

Editing Component Definitions:

Selecting E from the entry/editing environment will give you

the ability to change any part of each definition.  Simply select

10

the  letter of the  item to change.   For example,  to change the

name, hit A after hitting E and  you will be prompted for the new

name.   Either enter the new name, or  you can press ESC to leave

it  unchanged.   When you are  done editing,  press the  key that

corresponds to exiting the edit.

If  you  are  editing  the  placement  of  chips,  the  chip

definition  file must be present in the working directory for the

placechip  function to work.  The only  time this is a problem is

if the  board you are editing is not your own, such that the chip

definition files have different names from your own.

Deleting Component Definitions:

This function  allows  you  to  delete  a  component.    The

component  that will be deleted is the one currently displayed on

the screen.  Press D and you will be asked if you are sure if you

want to delete this  component or not.  Answer  accordingly.  All

connections  that contained  the  deleted function  will also  be

deleted.

Copying Components:

This  function will let you make copies of a chip definition

to speed  the entry of chips.   It will copy  everything from the

displayed  definition   to  a  new  definition   except  for  the

designation  from  which it  will take  the  alpha part  and then

increment the numerical part to the next available part.  It also

does not put the chip in the same location.  You will be asked if

you wish to place the chip now or later.

Searching for a Component:

Sometimes  finding the component you wish  to edit or delete

can take some time on a large board.  This function allows you to

search for components  by the designation.   When prompted  enter

the designation, PCTRACE will  change the case to upper  for you.

If  it exists the program will display  that device's number.  If

it doesn't exist, you will continue right where you were.

Incrementing and Decrementing the displayed Component:

Using  + and  -  will change  the  displayed component.    +

increments  and - decrements.  The display is circular in nature,

therefore hitting  + at the  last component  will put you  at the

first and hitting - at the first will put you at the last.

Exiting the Entry/Editing Environment:

Use the N key to end these functions and return to the board

definition menu.

2. Connection Edit/ Entry:

This  system  allows  for  entry  and  editing  of  all  the

connections  that need to be  made between the  components in the

system.    The  functions  are  very  similar  to  the  component

functions.

11

New Connection Entry:

Typing an "A" will allow you  to enter new connections.  The

program will display the connection number being entered and then

prompt for the  name of the  source designation.   The name  must

exist in memory as a component.  ESC can be used at this point to

return you to the entry/editing environment.

The next question is for the  source chip pin number.  Enter

the number of the pin that you want the trace to connect to.  ESC

will back you up to the previous prompt.

The  program will now prompt for the name of the destination

designation.  The name must exist in memory as a  component.  ESC

can be used at this point to return to the previous prompt.

The  next question if  for the destination  chip pin number.

Enter  the number of the  pin that you want the  other end of the

trace  to connect  to.   ESC  will back  you up  to the  previous

prompt.

The program will now ask if this is a priority route or not.

By answering yes, this trace will be routed before all other non-

priority  routes  no matter  how long  it may  be and  what other

routes it may block.  This might be used for making sure a memory

access bus is  all done in copper instead of  using jumpers for a

vital connection.

Next, the program will want to know the desired trace width.

Traces  from 0.01  to  0.07 are  supported.   Select  the  number

corresponding  to the size  required.   The following  table will

tell you  what size trace will  go between two pads  of some size

that are spaced on 0.1 inch centers.

Pad Size (inches)

Trace width              0.05      0.06      0.07

0.01                Yes       Yes       No

0.02                Yes       No        No

0.03                Yes       No        No

0.04                No        No        No

0.05                No        No        No

0.06                No        No        No

0.07                No        No        No

These tolerances  can be overridden using  the interactive router

which will be discussed later.

The next question the  program will ask is if  everything is

correct.  Answering no will  restart this process by  reprompting

for the source designation.  Answering yes will bring up the next

prompt.

The  next prompt  is if  you wish  to keep  this connection.

Answering no will  skip the next  prompt and ask  if you wish  to

enter  another connection.    Answering yes  gives  you the  next

prompt.

The   last  prompt  asks  if   you  wish  to  enter  another

connection.   Answering yes, will put  you back up at  the source

designation prompt to enter another  connection.  No will  places

12

you back in the entry/editing environment.

Editing Connection Definitions:

Selecting E from the entry/editing environment will give you

the ability to change any part of each connection.  Simply select

the letter  of the item  to change.   For example, to  change the

source  designation,  hit  A after  hitting  E  and  you will  be

prompted  for  the  new  designation.    Either  enter  the   new

designation, or  press ESC to  leave it unchanged.   When you are

done editing, press the key that corresponds to exiting the edit.

Deleting Connection Definitions:

This  function  allows you  to  delete  a  connection.   The

connection that will be deleted is the one currently displayed on

the screen.   Press D and you  will be asked if you  are sure you

want to delete this connection.  Answer accordingly.

Searching for a Connection:

Sometimes finding the connection you  wish to edit or delete

can take some time on a large board.  This function allows you to

search  for connections by the designation.   When prompted enter

the designation sought.  PCTRACEwill change the case to upper for

you.  PCTRACE then will search from the current connection to the

end.   It looks at  both the source  and destination designations

for a  match.  If  it finds  the search string  the program  will

display that connection's information.  If it doesn't exist,  you

continue right where you were.

Incrementing and Decrementing the displayed Connection:

Using  +  and -  will change  the  displayed connection.   +

increments  and - decrements.  The display is circular in nature,

therefore hit + at the last connection will put you  at the first

and hitting - at the first will put you at the last.

Exiting the Entry/Editing Environment:

Use the N key to end these functions and return to the board

definition menu.

3. Ground Plane:    This  function is  used  to set  up a  ground

plane.  The program will display a picture of

the  board.  It will  prompt for the width of

the ground  plane on  all four sides  on both

the  component side  and solder  side of  the

board.  Each digit entered corresponds to .05

actual inches.  You  may come back and change

the ground plane sizes at any time.  ESC will

also back  you out  of the first  prompt only

(top-solder side).

***   Please note that  this function  erases

any  work  done  in  the  interactive  router

(discussed later).

13

4. Define Board Size:    This  function  is  the  first  function

used.   It defines the dimensions of the

board.  The maximum board size supported

by PCTRACE  is 11" X 8".   When prompted

for  numbers, you must enter the numbers

as   multiples  of   5.     Each  number

represents 0.05" of actual size.  So  an

entry  of  100 would  correspond  to 5".

You may change  board sizes at  any time

during the design process by calling the

routine.  If you make the board smaller,

the  chips  that  were outside  the  new

border will  have to  be replaced.   ESC

will  back up to the previous prompt, or

jump  out  of  the routine  leaving  the

board size unchanged.

5. Spider Web: This function is useful  for checking to see if  a

chip  would  be  more  logically  placed somewhere

else.   By using the +  and - keys, you can toggle

through  the chips.  As each  chip is selected, it

as  well  as  all   traces  connected  to  it  are

highlighted in  red.  Once a  chip is highlighted,

you can select M to move the chip.  After pressing

M,  the arrow keys  become active and  the chip as

well as all its  connections will be rubber banded

around the board.  You can hit RETURN to place the

chip at  the current location,  or hit ESC  to not

place the chip  and put  it back  at its  original

location.  The  ESC or  E key will  exit you  from

this function.

***  Please note, if you  move any chips, the work

done by the interactive router is erased.

Interactive Router

This board editing system allows you to interactively design

a board. It can  be very useful for finding possible  routes that

the autorouter  was unable to  find.  Upon entry  to this section

the program generates the borders and places the pads of all  the

chips.  It will also ask you to place all the chips that have not

been placed  yet.  If you  intend to route the  entire board this

way, place  your chips and then  leave them.  Moving  them around

after placing the chip will  distort your board.  PCTRACE has  no

way of connecting traces up to  a chips new location.  The screen

will show  all the  chips with their  designations centered  amid

each components pins.   This system has many keys that are active

while in this environment.  An explanation of each follows.

14

S - switch sides:   This function  changes the side of  the board

displayed.   When  you enter  S, you  will be

shown  the  other side  of  the  board.   The

program supports  only  2 sides,  solder  and

component.

G - grab location:  The key is only active at certain  time.  The

F1, F2,  F4, F5, F6, and F7  function all use

this key.

E or ESC - end:     Pressing  E or  ESC  will exit  you from  the

interactive router.

Cursor Keys:   The cursor keys move the solid block around on the

board.   Each time  you press  the arrow  key, the

cursor  will move  1 space.   Relative coordinates

are displayed at the bottom of the screen.   Using

Shift-Arrow moves  the cursor  10 spaces.   If you

come to the edge of the screen but not the edge of

the board,  the screen  will shift the  board over

for you.

Function Keys:

The function keys numbered F1-F8 do the majority of the work

in  PCTRACE.  They  allow you to  place vias,  run traces, delete

traces, resize traces, and delete vias.

F1 - Set Source:    By pressing F1, the bottom of the screen will

say to  position the cursor over  the pad you

wish  to  designate as  the  source and  then

press  G for  grab.   The bottom  status line

will  tell you  exactly  what  chip  and  pin

number you have selected.

F2 - Set Dest.:     This  function  works   exactly  as  the   F1

function but selects the destination pin.

F3 - Run Trace:     Once   you  have   defined   a   source   and

destination,  pressing  F3  will  attempt  to

connect the two pins with a trace 0.01 inches

wide.  If  it is successful, you will see the

trace  appear.   If  it cannot  complete  the

route it will notify you  at the bottom.   At

this point  you might  try placing a  via and

routing from the source to it on one side and

from the via to the destination on the other.

F4 - Delete Trace:  This  function will  delete any  part of  any

trace that you define. Pressing F4 enters the

delete selection mode.  After pressing F4 the

screen will say to use G to grab the parts to

be deleted.  As you  press G on valid  traces

15

the  pieces will  be  highlighted.   You  can

change screens by  moving the cursor  off the

screen  without  losing the  already selected

traces.   Once  you  have  selected  all  the

pieces you wish to delete, press F4 again and

you will be prompted to make sure you wish to

delete  the selected  traces.   Answer  Y  to

finish the delete operation.

F5 - Add Via:  This function key  will add a via  at the location

of the cursor when G is pressed.  It will be V and

then  the next  available number.   Use  these for

multilevel routes or just  extra pads needed.  You

cannot place  a via  adjacent to an  existing pad.

You can place one diagonally from one though.  You

also may not place a via in the middle of a trace.

All vias are deleted by the autorouter when it  is

invoked.

F6 - Del Via:  This  function  deletes  the  via  at  the  cursor

location when G is pressed if one exists there.

F7 - Resize trace:  This  function  will resize  a  trace to  any

valid trace size.  Select F7 and the use G to

select the  traces to resize.   Once you have

selected  the traces to  resize press  F7 and

you  will  be asked  if  you  really wish  to

resize the traces, answer Y  if you do.  Next

the program will prompt for the trace size. 1

corresponds to 0.01 up to 7 which corresponds

to 0.07  inches.   Be careful not  to overlap

other traces or pads with this function.

F8 - Check Route:   This function will check to see if a selected

pair  of  pads can  be  connected.   It  will

report back  as to  whether the route  can be

completed.  This  is useful for  doing multi-

layer  routes  using vias.   Checking  for an

available  route  before  laying  the  actual

trace will save a  number of deletes if later

your path is blocked.

F9 - Manual Route:  This  function allows  you to  manually route

traces.  Place the cursor on the starting pad

or  trace and press F9.  If the cursor is not

on  a  valid  starting  point,  nothing  will

happen.   Now use the cursor  control keys to

route your trace.   Every time you move, your

trace is lengthened.   When you are done, you

can either  stop in and open  space and press

F9 or you can stop on  a trace or pad and hit

F9,  the  correct  connecting  piece  will be

16

added to  make the  connection.   Please note

that  if  you  go  over  an  existing  trace,

PCTRACE will  cut that trace in  favor of the

new path.  This  works well for putting short

jumpers on  the board.   This is  very useful

for routing traces  that would normally block

other  routes if  they  were not  placed this

way.  With the interactive router you can run

any  size trace  anywhere  you wish.   It  is

possible to over-ride the autorouter  and run

a 0.01 inch trace between two  0.07 pads.  If

your board making process is accurate to 0.01

inches you can complete many low power traces

this way.   The manual router is also used in

many advanced "tricks" for making specialized

boards.

Using a mouse in the interactive router:

The  mouse  allows you  to  do all  the  functions mentioned

previously  except for the manual  router.  You  must install the

mouse before running  the PCTRACE  program.  When  you enter  the

interactive program you will  see four arrows in the  lower right

corner of the screen.  By  using the mouse, putting the cursor on

one of these arrows, and pressing the left button the screen will

be  moved  by  half  in  that  direction.    The  same thing  was

accomplished by moving the  cursor off the board using  the arrow

keys.

Pressing  the  right button  will pop  up  a menu  along the

bottom of the screen.  The menu  corresponds to function keys F1-

F8.  The  exit command  at the  far right,  exits the  menu.   By

placing  the  cursor anywhere  on  the command  desired,  you can

execute  that command  by  pressing the  right  button.   If  the

command selected is F1, F2, F4, F5, F6, or F7,  you will be using

the "Grab" method.  The  left button works the same as the  G key

in  these cases.   Use it to highlight  sections and locations on

the board  to work  with.   When  using the  delete and  resizing

functions  (F4 and F7) you must press and release the left button

for each piece of trace.

Autorouting

Selecting  this from the menu will enable you to connect all

or most of the  connections you desire.   The program will  first

draw  borders, place pads,  and set up  the ground planes.   If a

component  has not been  placed, you will  be asked  to place it.

The program requires all chips to be placed before it  can route.

There  are four  routing strategies  that can  be employed.   You

select each strategy  by hitting Y  or N as  the cursor moves  to

each  option.  If you hit Y  for single or double via routing you

will also be prompted  for the maximum number of tries  for each.

17

Just  enter a  number.   Bigger  numbers  are better  for  bigger

boards, smaller number for small boards.

The  PCTRACE  system  is  totally  re-entrant.    The  first

question  you  are asked  is  if you  wish  to clear  all traces.

Answering yes clears anything  currently on the board.   No would

keep what was  on the board such as work  done by the interactive

router.  You can hit ESC during any routing strategy to exit that

routine and the autorouter.  The program  will then show the list

of all unconnected routes still existing.  If the  router has not

completed one run-through and you  have routed before, traces may

have been completed by the earlier route and still be reported as

unconnected.   You can do a  design check to see  which routes in

you connection list have not been  connected either by hand or by

autorouter by running  the autorouter and not erasing the current

traces.  Just select the double or single sided route system.  It

will report any routes  that are incomplete.  Any  route that was

completed  before will  be marked as  complete and  not reported.

The program will report whatever routes weren't done and the ones

it couldn't complete.

Single Sided Route: This function  will attempt to  place all the

routes on the  solder side of the board.   It

is  the least efficient strategy but is great

for small projects  where ease of making  the

board itself  is a  priority.  This  function

cannot  be used in  conjunction with the next

three strategies.

Double Sided Route: This  function will  do  its  routing on  two

sides  of  the  board.     It  has  a  higher

completion percentage than  the single  sided

strategy.

Single Via Route:   This  function requires that the double sided

strategy be  select first.  This  function is

used after the  double sided strategy  to try

to  complete  any  uncompleted  routes.    It

places a via and tries to do the single route

with  a trace on each  side of the  board.  A

number such as 50  is good for maximum number

of tries.

Double Via Route:   This  function  also  requires  double  sided

routing  be selected  to work.   It  uses two

vias  to attempt  to complete  the route.   A

good  maximum number  of tries  is 30  or so.

This function  can take a great  deal of time

so be patient.

18

Board Statistics

This function displays 5 statistics that may be of some interest.

Equivalent Integrated Circuit Count:    This   function   is  the

total   number  of   pins

divided by 16.

Board Density: This is the amount  of space total that each  chip

has to  occupy.   It is  calculated by  taking the

area of  the board and dividing  by the Equivalent

IC count.

Total Trace Length: The function  gives the total  length of  all

the  traces on  the board.   The  smaller the

number, the better the design.

Board size:    The board dimensions in inches.

File name:     This is the file name currently in memory.

Printer Menu

1. Print Component List: This  function will  generate a  list of

all the components.  It can be sorted by

name or designation.

2. Print Connections:    This  function will  generate a  list of

all the connections  in memory.   It can

be  sorted by the  source or destination

designation.

3. Print a Silkscreen Representation:

This function will generate a pseudo-silkscreen.  It is

not the same scale as the  actual board but it is useful for

remembering where components  are to be placed.   Select the

printer  to be used and watch it go.  The postscript driver,

however,  does  output a  scaled  silkscreen  and will  also

prompt for an output destination of file or printer.

Exiting the Program

Selecting  7 from the main  menu will drop  you from PCTRACE

back to DOS.  If you have data in memory you will be asked if you

wish  to save it. Answering Yes will  prompt you for a file name.

Enter the file name as you would in the save data function in the

Input/Output  menu.  You  can hit ESC  to abort the  save, but it

will  not return  you to the  program.   Answering N  to the save

query, will drop you immediately to DOS.

19

Brief Tutorial

The  following is  intended as  a method  of getting  up and

running with the program.  It will give a general idea of how the

program works.

1.   Begin  by running PCTRACE.   This  information can  be found

back in the section on setting up PCTRACE for your machine.

2.   Once  at the main menu,  press 3 for  define/edit board, and

then select 4 for define board dimensions.  For our purposes

we will use a board size of 80 x 80, which translates into a

4 inch square board.

3.   Select function 1  to enter  new components.   You will  see

that no data has been  entered yet.  Press A to  enter data.

We  will put  three chips  on the  board and  two resistors.

Answer each  prompt below  with the  following  data.   When

asked to place  each chip answer Y  and place the chip  with

the  cursor keys.   After  you have  entered the  last item,

answer N to the "Enter another component?" prompt.

Name: 8051 uP                 Designation:  IC1

Definition file: DIP40V6      Placement: Left side of board

Pad Size: 0.05

Name: 74ls373                 Designation:  IC2

Definition file: DIP20V3      Placement: Upper right

Pad Size: 0.05

Name: 2716                    Designation:  IC3

Definition file: DIP24V6      Placement: Lower right

Pad Size: 0.05

Name: 100k resistor           Designation:  R1

Definition file: 2PAD50V      Placement: Upper middle

Pad Size: 0.06

Name: 10k resistor            Designation:  R2

Definition file: 2PAD50V      Placement: Lower middle

Pad Size: 0.07

N exits you to the board definition menu.

4.   Next, we want to  enter some connections to be  made between

the chips.   Select 2 from the menu to  enter the connection

entry section.  Enter the following connections.  All traces

should be 0.02 inches wide.   Answer N to each prompt of "Is

this a  priority connection?".   When you  have entered  the

last connection  answer N  to the "Enter  another connection

prompt?".

20

Source Device       Pin       Desitination Device      Pin

IC1            2              IC3                 2

IC1            3              IC2                 16

IC3            22             IC2                 6

R1             1              IC2                 4

R2             1              IC1                 8

R2             2              IC3                 6

R1             2              IC1                 31

IC2            8              IC3                 17

IC1            21             IC1                 15

IC2            8              IC3                 8

IC2            9              IC3                 9

IC2            10             IC3                 10

IC1            14             IC2                 9

N returns you to the menu.

5.   Ok, lets autoroute the board.   Go back to the main menu and

select  4  to autoroute  the board.    When prompted  to use

single sided routing or not, answer Y.  Now hit  the ESC key

and watch  the  machine route.    When it  is  done is  will

display the  list of uncompleted connections  to the printer

or  screen.   Select whichever  you wish.   We  will  now be

dropped back  to the main menu.  We can  take a look at what

we have done by selecting  number 5 from the menu.   You can

cursor around and see  a representation of the board  on the

screen.  Hit ESC when done.

Now lets  re-route the board using  double sided techniques.

This time  say N to single  sided and Y to  double sided and

then hit ESC.  The machine will route double sided and again

give  a list of incomplete connections if any exist.  Select

printer or  screen for this list.   Lets take a  look at the

board again.  This time we may also want to use the S key to

change sides of the board that we are viewing.  When you are

done hit ESC.

6.   From the main  menu, hit  6 for statistics.   Displayed  are

five statistics that  tell you how good your design is.  Hit

space when you are done with this.

7.   Last of all we'll  print this baby out.   Follow instuctions

in the section on the RPRINT Subprogram to do this.

21

Advanced PCTRACE Design

By using  the interactive  router before running  the autorouter,

and using a few tricks, different board shapes, cutout areas, and

edge connectors can be  created.  The following is  an example of

how to do this:

The objective here is to make a four inch square  board with a 10

pin edge connector on  one side.   There also needs  to be a  one

inch square hole in the center of the board.

1.   Define a board size of 5x5 inches.

2.   Place a via at coordinates 0.15,0.15.  A  via must be placed

so that the manual router has somewhere to start.

3.   With the cursor still at 0.15,0.15 press F9 (manual  router)

to draw the outside edge.

a.   Right arrow to 4.20,0.15

b.   Down arrow 5 times

c.   Right arrow 5 times

d.   Down arrow 22 times

e.   Left arrow 5 times

f.   Down arrow 44 times  (4.2,4.2)

g.   Left arrow to 0.15,4.2

h.   Up arrow to 0.15,0.15

4.   Delete Via 1 at 0.15,0.15

5.   Put the cursor at 0.15,0.2, press  F9, go Up 1, Right 2, and

press F9 to close the path.

6.   Move to 2.0,2.0 and place a via here.

7.   Start the manual  router at  2.0,2.0 go right  20, down  20,

left 20  and up  20.  Delete  the via and  close the  box as

before.

8.   Move to 4.45,1.0,  start the  manual router and  go left  6.

Press F9 again.  Repeat this command at 4.45,1.1 , 4.45,1.2.

The last one  goes at 4.45,1.9.   There  will be ten  traces

when you are done.

9.   Use  the  resize  function  (F7)  to  resize  each  of   the

previously drawn traces (#8) to 0.05 inches in width.

10.  Add vias at  4.15,1.0 ,  4.15,1.1 down to  4.15,1.9.   There

should now be 10 vias on the board.

11.  Delete the  traces at 4.45,.95 4.45,1.05  down to 4.45,1.95.

This  is necessary as the  autorouter will get  stuck on the

outside loop that we created  and never complete the  route.

22

You must  be careful not to allow the router to see a source

or  destination network that is circular.  It will cause the

router to hang the computer in an infinite loop.

12.  Exit to the component entry screen and add a 40 pin DIP that

is horizontal (DIP40H6)  to the top of the board.  Make sure

that it is  opposite the vias shown will placing  the 40 pin

device.  Also make sure that it is within the  bounds of the

outside edge we drew.

13.  Enter the following connections.

IC1-40 to V1-1

IC1-22 to V7-1

IC1-21 to V10-1

14.  Run the autorouter but  say no to clearing the  traces.  Use

single sided  mode.  When the autorouter  has completed this

process, go back to  the interactive router and notice  that

all the traces went around the middle  box.  The router will

not cross that boxed area.

This  system will allow you  to get the  general board shape

and  edge connector dimensions correct.   You will  still have to

cut and file  the edge connector to get  it to fit nicely.   When

doing double sided boards the same technique will work.  You will

have  to be  careful that  as you  add vias,  you bring  the edge

connector trace out far enough to offset them so that you can get

individual pads for each side.

Instead of using  edge connectors  you could  also define  a

single pad definition and  use numbers other than V1, V2,  and so

forth.  If you are designing a card for an IBM PC, use A1-A31 and

B1-B31.    The  pads would  have  to be  placed  first  using the

component entry system and then go in with the interactive system

to put in the connections to the pads.

One  method of marking the  boundaries of the  board so that

you  can see where  you are placing chips,  relative to the board

edge and  connector, might be to put a via  at any corner so that

it will show up  as a single pad  in the placechip function.   It

will also give you the general outline of the edge connector.

23

RPRINT.EXE Subprogram

Execute  this program by typing RPRINT  from the dos prompt.

Upon entry into this program you will be prompted for you printer

type.  Enter the corresponding number.  You will then be prompted

for the board  name.  Type the name with no  extension.  The main

control  panel  will  now  explode  onto  the  screen.    A  math

coprocessor is particularly useful in this program.

1. File name:  Selecting 1 will  allow you to change  the file to

be printed.  Enter the filename with no extension.

2. Board side: This selects  the board side to  print.  Selecting

this  number will ask if  you wish to change board

sides  to print.   Answer  Y or  N.   Single sided

boards are done on the solder side.

3. Mirroring:  This selection will allow  you to mirror the board

left to right for whatever reason you might have.

4. Negative:   This  function will  allow  you to  toggle between

printing a  negative or  a positive of  the actual

board.

5. Actual size:     Laser printers and 24 pin dot matrix printers

can  output the  images at  actual size.   If

your  printer will  not  do  this, this  menu

choice  will not  exist.   You  can use  this

selection to toggle  between actual size  and

double size printing.

6. Print Board:     This  selection  starts the  printing  of the

board.

7. Exit program:    This selection exit  the RPRINT program  back

to DOS.

Note:     The postscript driver also has the ability to output to

a file.   You will be  prompted after pressing 6  as to

whether you wish to print to the  printer or to a file.

If you select  a file,  the program will  prompt for  a

file name.  Do not enter an extension.  ".PST" will  be

appended  automatically to  the filename.   This allows

you to use someones  (god forbid) MAC Laserwriter or  a

high quality typesetter.

24

Keyboard Summaries

Chip Placement Screen:

ESC                 - Option to Abandon Placement

RET                 - Place chip at current location

Cursor Keys         - Moves chip one space for each

depression

Shift Cursor Key    - Moves chip 10 spaces for each

depression

Define Chip Definition:

ESC                 - Option to Abandon Definition

RET                 - Place pin at current location

Cursor Keys         - Moves cursor one space for each stroke

BACKSPACE           - Removes last placed pin

Spider Web System:

M*                  - Set system to move highlighted chip

Cursor keys    - Move chip one space on board

RET            - Place chip at current location

ESC            - Abandon placement

+,-*                - Increment and Decrement through chips

E*                  - Exit Web System

* - Denotes that this selection is displayed on screen.

Interactive Router:

S                   - Display other side of board

G                   - Grab a piece of trace at cursor

E,ESC               - Exit interactive router

F1                  - Select source pad

F2                  - Select destination pad

F3                  - Attempt to connect source and

destination

F4                  - Delete traces

F5                  - Place via at cursor location

F6                  - Delete via at cursor location

F7                  - Resize selected trace

F8                  - See if route is possible between

source and destination pads

F9                  - Enters and exits the manual router

25

Order Form

Please enclose this form when ordering.

Name:     __________________________________________

Address1: __________________________________________

Address2: __________________________________________

City:     _________________   State:    _______   Zip:_________

Phone Number:  (________) ________-______________

Serial number on opening screen of PCTRACE: _________________

Computer name: _____________________________________

Processor: 8086  8088  80186  80188  80286  80386  80386SX

Printer type:  _____________________________________

Suggestions/comments/other information:

____________________________________________________

____________________________________________________

____________________________________________________

____________________________________________________

____________________________________________________

____________________________________________________

Please also  enclose the $75 check  or money order,  (do not send

cash) made payable to:

Douglas E. Ehlers

4520 S. 58th

Lincoln, NE  68516

(402) 435-8440