Don's Original Designs

Oceanside, Ca

Don Rasmussen  WB8YQJ/6

Version 2.0

Don's Original Designs proudly introduces ProCat/HF

user  customized  software.  What you get  is  computer  keyboard

control  of your transceiver, user defined command keys,  contest

callsign duplicate checking, and transceiver memory loads for the

Yaesu FT747GX. The system  works  with  Kenwood, Icom, or   Yaesu

transceivers  that have provisions for computer control. You need

an IBM compatible computer with hard drive,  serial port, and EGA

or VGA graphics card. An interface is  required  to  connect  the

computer serial port to the transceiver, which is available  from

the   transceiver   manufacturer.  Interfaces   can   be   easily

constructed, and are generally compatible between  manufacturers.

A  Yaesu FIF232 interface works great with Icom  transceivers  by

connecting Yaesu DIN #1 to Icom remote connector center, and  DIN

#3 to Icom remote connector ground. Once interfaced, the  program

allows  instant  QSY  using the numeric keypad  on  the  computer

keyboard.  The  control  keys  are grouped  to  make  tuning  the

transceiver EASIER than it would be by using the transceiver  VFO

knob.  You  can also add other commands to the  transceiver,  and

assign  any  key you wish to perform that command. You can change

the on screen colors to suit  your preferences, they are saved to

disk,  and  the on screen  GMT clock is adjustable to  your  time

zone. I hope you  find  ProCat/HF useful in  your shack, as  many

hours  were  spent  in  design, programming, and debugging.

Best 73's

Don

Special Thanks To:

Edward J. Coan

Yaesu Musen Co. Ltd.

Tokyo,  Japan

Roy Kessler

Janet Traylor

PC Globe Incorporated

Tempe, Arizona

A Quick Demonstration

All programs and files should be copied onto your hard disk.

To  run the program, key in PROCAT and then press the enter  key.

Make sure your transceiver and interface are turned on and ready.

Make sure the NumLock light on the keyboard is on.

The  program  will  show  you  the  opening  screen,  with  the

station  clock  in  the  upper  left    corner,  and  ten  VFO

frequencys  listed along the right border of the screen.  One  of

these VFO frequencys will have a highlighted background. You  can

change the highlighted frequency by pressing the cursor up or the

cursor down key, these keys are  located just to the left of  the

numeric keypad.

NOTE: The transceiver that is currently configured is listed just

below the time display. See configuration instructions to  change

the current transceiver.

You can set the transceiver display to the highlighted  frequency

by  pressing the numeric keypad minus (-) key. You can raise  the

highlighted frequency by pressing the keypad plus (+) key, the  6

key, the 5 key, or the 4 key. When you press the plus key (+) the

frequency will only be raised by 20 hertz, so you'll have to keep

it depressed for a few seconds to hear a noticable change in  the

received  signal.  You can lower the frequency  by  pressing  the

keypad  (enter) key, the 3 key, the 2 key, or the 1  key.  Again,

you'll have to hold down the (enter) key for a couple seconds  to

notice  any  change.  Transceiver mode can  also  be  changed  by

entering the first letter of the mode.  Notice how all tuning  of

the transceiver can be easily done with one hand over the numeric

keypad, and very little looking at the keys.

If  you  want to change the highlighted  frequency  to  something

completely  different,  press  the "b" key to  enter  a  new  VFO

frequency.

The TEN VFO's on the screen have a description in the lower right

corner of the screen. You can change this description by pressing

"e"  then pressing "l". Or, you can clear this group of VFO's  by

pressing "e" then pressing "e" again.

There  are  up  to 20 groups of TEN VFO's, the  group  number  is

listed just to the left of the description. A different group  of

VFO's  can  be called by pressing "r" then entering a  new  group

number (from 1 to 20).

Main Menu Key Definitions

F2 - adjusts on screen window color.

F3 - adjusts on screen text color.

F4 - adjusts the station clock.

F5 - starts contest callsign checking.

F6 - finishes contest callsign checking.

F9 - clears the contest callsign file.

b - enter a new base frequency in highlighted VFO position.

r - request a different page of VFO's.

e - edit the label or erase a page of VFO's.

o - call the configuration options program.

u, l, c, a, f, n - used to set transceiver mode.

KEYPAD KEYS

4 - Up   100 KHZ

1 - Down 100 KHZ

5 - Up 10 KHZ

2 - Down 10 KHZ

6 - Up 1 KHZ

3 - Down 1 KHZ

(plus key) - up 20 HZ

(enter key)  - down 20 HZ

(minus key) - set displayed frequency, no change.

(cursor keys) - select displayed VFO.

Duplicate QSO Checking

This feature is an  aid to the  operator that is not  planning on

sending his contest results  into the  contest officials.  Simply

press  F5  to start  contest  duplicate  callsign  checking.  All

keys other than numeric keypad keys are disabled. Use the  keypad

to tune in a contest station. Key in the station's  callsign  and

press return. If the  call begins  with  the  number 1 through  6

(this rarely happens) preceed the call with an asterisk. The call

will  be added to the list, and displayed in large letters if  it

is a good call. Then, try to work the station, use the  displayed

QSO  number  in the exchange. If you can't work  him,  press  the

backspace  key  to  drop  him from the list. If  the  call  is  a

duplicate, it is not added to the list, and the WARNING LEMON  is

displayed. Over 900 calls can be saved. Press F6 to save the  log

and return to ragchew mode. Press F9 to clear the duplicate log.

Configuration and Customization

Configuration  can  be as simple as defining  your  graphics

card  and transceiver brand. Or, you can  define custom  keys  by

looking  up the computer command  data in your  Kenwood or  Yaesu

transceiver manual or Icom CI/V interface manual. An  example  of

this would be to define the "S" key. First specify a label for the

key,  like "SPLIT ON". Then, look in your transceiver  manual  to

find  the  command bytes that perform this function, and  key  in

that data. When you finish, this will be displayed on the  screen

S - SPLIT ON    . You can also position this label anywhere  on

the  screen you want. From then on, whenever you press the  upper

case S, the split function on your transceiver will be turned on.

You may want to define the lower case "s" as SPLIT OFF. There are

20 keys available for definition.

The options screen is available by pressing "o" from the

main menu.  Then press return to select  configuration  from  the

options  menu. Here is  the list  of options  and  the  responses

required.

Change Basic Configuration? - enter Y to specify a different brand

of transceiver than that listed under the time on the  main menu.

Configure Icom Transceiver? - enter Y to select an Icom transceiver.

Enter Icom Address - enter 2 digit rig address, 00 if you dont know.

Ex. IC735 - 04,IC751 - 28,IC761 - 30. Call Icom for other addresses.

Configure Kenwood Transceiver? - enter Y to select a Kenwood.

Did you modify your rig for 9600 BPS operation? - All Icoms  come

from  the factory configured for 1200 BPS. There is a  jumper  in

the  transceiver  that  can be moved to  enable  a  higher  speed

transfer. Enter Y if you've enabled this option, its a very good idea.

Command  Data  is 5 Bytes Long, is the command  byte  sent  last?

(Yaesu Only) - All Yaesu commands are 5 bytes long. Depending  on

your model the command byte may be the first byte, or more often,

the last byte of the string.

Frequency set command byte? (Yaesu Only) - is usually 0a, look in

your manual and remember to enter zero and not the letter "O"  in

these hexadecimal responses.

Setup  Operating Modes? - enter y to define the keys used to  set

transceiver mode.

Mode set command byte? - Kenwood and Icom users enter 00. Yaesu's

that have one command byte and four dummy bytes for the mode  set

command (this is rare) enter 00. For all other Yaesu's enter  the

value that will be first or last in the 5 byte string (this  byte

is also called the "instruction byte" by Yaesu.).

Configuration and Customization

(Continued)

AM  mode  set command byte? - Icom users enter  02.  Yaesu  users

enter  the AM command byte, also called the mode  set  "parameter

byte" by  Yaesu.  Kenwood's mode  parameter byte is n , where the

mode command listed in the Kenwood  manual is  MDn;  and  n  is a

number between 1 and 6 that specifies the mode.

CW  mode  set command byte? - Icom users enter  03,  Yaesu  users

enter the CW narrow parameter byte. Kenwood users enter n value.

FM  mode  set command byte? - Icom users enter  05,  Yaesu  users

enter the FM narrow parameter byte. Kenwood users enter n value.

LSB  mode  set command byte? - Icom users enter 00,  Yaesu  users

enter the LSB parameter byte. Kenwood users enter n value.

CWN  mode  set command byte? - Icom user enter 04  (RTTY),  Yaesu

users enter the CWN parameter byte. Kenwood users enter n value.

USB  mode  set command byte? - Icom users enter 01,  Yaesu  users

enter the USB parameter byte. Kenwood users enter n value.

Suggested values were listed for Icom because all Icom radios use

the  same  values, Kenwood and Yaesu values  should be taken from

the transceiver manual.

OTHER CONFIGURATION OPTIONS

Adjust memory loading delay? (Yaesu FT747GX only) - enter a large

enough value in this delay to ensure that memorys are not  loaded

into  the  FT747GX too fast, when this happens memorys  are  lost

after the transceiver is powered down.

Use auxillary serial port? - enter Y to use Comm Port 2.

Use  VGA graphics? - enter Y if you have VGA graphics.

Change  the  programmable keys? - enter Y to see a  list  of  the

programmable  keys that have been defined, and to change them  if

you wish.

Programmable keys available 20  - Press  an  alphabetic key. You

Press Esc to finish            will redefine the key if  it is

on  the  custom  key  list that

borders  the left side of the screen. You can  define a  new  key

if  it does not appear on the list. Remember that upper case  and

lower case are different keys. Press Esc to finish.

Enter  key label - Enter a short description of the key you  want

to  define, such as SPLIT ON, SPLIT OFF, WWV, CLARIFIER ON,  etc.

This  description will be seen on the main menu. The letters  are

small, using upper case improves readability.

Configuration and Customization

(Continued)

Enter  Command  (10 digits) - this is where you  will  enter  the

actual byte string from your transceiver manual that will perform

the  function you described in the (Enter key  label)  procedure.

For  example, in the Yaesu FT747GX manual the  SPLIT  ON  command

consists of these  5  bytes ( 00 00 00 01 01 ). In  this example,

these  are  the  exact  10 digits  to  enter  (there  are  always

two  digits in one byte). Remember that although the  values  you

are entering are in hexadecimal, there is no conversion  required

by you!. Simply plug the ten digits in the way they are listed in

the  manual.  All Yaesu commands are 10 digits (which is  also  5

bytes)  long.  Whatever you put in this ten digit field  will  be

sent directly to your transceiver, with no changes in byte order.

Icom commands are not always ten digits long,  here is  the

format  for Icom commands. In the Icom CI/V manual  (available  to

anyone,  free  from Icom) on page 13 there is a  sample  FM  Mode

command.  It  looks  like  this  ( FE FE 04 F1 01 05 FD ).  Every

command  in Icoms manual starts with FE FE 04 F1 , and ends  with

FD. Since that is true, the program will provide these characters

to  the transceiver for you. In this example, if you  ignore  the

characters  that the program adds for you, ( 01 05 ) is all  that

is left. These are the digits for you to enter. When the  program

asks for ten digits, key this in ( 01 05 FD FD FD ). Always use FD

to fill out the ten digit field. As an added example, the command

on page 12 looks like this ( FE FE 04 F1 00 10 32 54 21 FD ) for a

frequency set of 21.543.21 MHZ. You enter this ( 00 10 32 54 21 ).

Kenwood commands are  entered as characters from the Kenwood

manual. Six characters  must entered, up to five command characters,

and the sixth character must be a semicolon. If the command consists

of less than five characters, the  field  must be  filled  with the

semicolon character. The TS440 command that  steps the rigs memory

channel down by one is listed in the Kenwood manual as DN; This is

counted as two characters and would be entered as DN;;;; The TS440

command for select memory channel is MCnnn where nnn is the channel

number. Enter this as MC001; to select channel one.

Arrange  key  labels onscreen? - enter Y if you want  to  display

your  key  labels  in another part of  the  screen.

Move  which  key? - find a custom defined key  somewhere  on  the

screen that you want to move. Press that key to select it.

Use  the cursor keys to position the label - move the  key  label

wherever it looks good, then press Esc to paste it in position.

FINAL NOTES

If  the  main file CATBASE.DAT is damaged, etc. Just  remove  the

damaged file and execute ProEdit.exe A new file will be built.

ProCat.EXE - main program    ProEdit.EXE - options program

Catbase.dat - database       ProCatHF.DOC - documentation

EGAPCA.DAT - EGA SCREEN 1    EGAPCB.DAT - EGA SCREEN 2

VGAPCA.DAT - VGA SCREEN 1    VGAPCB.DAT - VGA SCREEN 2

TT747.EXE  - FT747 program   PT747.EXE - prints the database

I can accept no responsibility for damages caused either directly

or  indirectly  from the use of this free program.  If  you  like

ProCat/HF please pass it on to a friend in it's ZIP format.

Version 2.0 Notes

I hope Kenwood guys like this program too. I view ProCat/HF as a

sensible approach to computer control of HF radio gear. I don't

see any reason to transfer control from the rig to the keyboard

unless it makes operating easier or more fun. The bottom line on

the program is that it makes tuning around a band or various bands

instantly available with only a keystroke or two. It also provides

a no hassle way to keep from being embarrased by calling the same

big gun contest station twice. Try the program and it's variations

with whatever rig you use, and if you think its the computer control

program you want to use, send me a letter. Thats all I ask in return

for this program.

If you would like your callsign displayed on the  screen

where my  call (WB8YQJ/6) is now, or anywhere else on the screen

that you like, send me $25 along with your request, whether you

want an EGA or  VGA graphics  screen, and  your diskette  format

(3.5 or  5 1/4 floppy). I'll  send you  updated an  screen file,

customized for your station.

Inquirys may be addressed to

A.R.S. WB8YQJ/6

Donald Rasmussen

2316 214 Paseo De Laura

Oceanside, Ca 92056################################################################################