RADIO REMOTE CONTROL PROGRAM

By Lydia Rosa

Copyright 1990 by Lydia Rosa

RADIO REMOTE CONTROL PROGRAM

WHAT'S NEW IN THIS RELEASE

Version 0.7

Fixed problem that would cause occasional hang with COM2. Also

fixed profile not recognizing COM2 in lower case.

Version 0.6

Documentation cleanup. No code change.

Version 0.5

Fixed a bug that caused garbage to display if function key pressed

for an ANY KEY TO CONTINUE keypress.

Fixed sort problem that caused replication of channel info.

Version 0.4

Fixed a bug that caused COM port not to respond.

Version 0.3

Added the TXADR command to allow addresses other than 8 for

the ICOM radios.

Fixed a bug in the SEARCH function.

Version 0.2

Initial Release

WHAT'S NEW IN THIS RELEASE                                      1

RADIO REMOTE CONTROL PROGRAM

DISCLAIMERS

IBM is a trademark of IBM corporation.

ICOM is a trademark of ICOM incorporated.

KENWOOD is a trademark of KENWOOD corporation.

DBASE III is a trademark of Ashton-Tate.

The  author  takes no  responsibility for  any damages that  may

occur due to the operation of this program.

This program is being distributed as SHAREWARE. This means  that

voluntary  contributions  are the  only  source  of  remuneration

obtained  for this work. A contribution of  $20.00 gives you  the

following:

1.  Automatic upgrades to the program.

2.  Fixes for any bugs.

3.  Schematics and instructions for the interfaces.

4.  A clear conscience.

Please mail all contributions and problem reports to

Lydia Rosa

P.O. Box 414

Marlboro, N.Y.

12542

Thank you for your support.

DISCLAIMERS                                                     2

RADIO REMOTE CONTROL PROGRAM

INTRODUCTION

The  RADIO REMOTE CONTROL PROGRAM will allow  you  to  control

your ICOM-R7000, or KENWOOD R-5000 radio from any  IBM compatible

PC,XT or AT.  With  the  standard  interface adapters provided by

the  manufactures of these radios, you can store and recall banks

of  100 channels.  Using a special  interface  circuit,  you  can

scan, search, and log  active channels.  This circuit is provided

free of charge to all registered users.

The program allows you to select various baud rates, as well as

COM  port 1 or 2. Commands are buffered,  and  an interface  to a

DBASE data file is provided.

Although  it has not been  tested, this  program may work  with

other radio models  produced by the manufactures of the supported

radios.

INTRODUCTION                                                    3

RADIO REMOTE CONTROL PROGRAM

REQUIREMENTS

In order to  run this program  you will need an IBM PC,XT or AT

running DOS 3.0  or higher. The  program requires  about 128k  of

available RAM to run.

If you are controlling the R7000, you will need either the ICOM

CI-V interface adapter, or the  modified RS232 interface.  If you

wish  to use  the signal detection feature  of this  program, you

will need to use the modified interface.

For  the  R-5000,  you  will  need  the  IC-10  option  kit.  A

modification must be made to  the R-5000  if you  wish to use the

signal detection feature of this program.

It  is highly recommended that the  modified interface  be used

for all radios.  Most of the features of this program make use of

the detection of an available  signal  (SQUELCH). These radios do

NOT  currently  have  squelch detection as  part of  the standard

interface.  Although  this program  will work fine  without  this

signal detection, operations such as SCAN and SEARCH will have no

usability.

The interface circuit is a inexpensive, easy  to build project.

All the parts  are readily  available  at any  electronic  store.

Estimated cost for each interface is  $15. Plans and instructions

for these  interfaces  are  provided to you  when  you  become  a

registered user.

REQUIREMENTS                                                    4

RADIO REMOTE CONTROL PROGRAM

STARTUP

To start the program,  make sure the program and data files are

in  the  default drive. If running  the R-7000, enter ICOM at the

DOS prompt. If running the R-5000, enter KENWOOD.

The program will  load, and  then execute  any  existing profile.

Once the program loads, the first page panel will be displayed.

STARTUP                                                         5

RADIO REMOTE CONTROL PROGRAM

THE DISPLAY

The display  panel shows you the  status of your radio, as well

as the status of the program.  At the top left hand corner of the

screen is displayed the current BAUD rate and COM port being used

to talk  to the radio. At the Bottom of  the screen  is  a status

line showing the various parameter settings. If there is a signal

present  on  the radio, the  word "SIG" will be displayed in  the

lower  left  hand corner. This  display  is  only  valid  if  the

modification has been made to detect squelch.

In  the center  of the screen, the status of memory channels is

displayed.  These channels  reflect  the  current  setting in the

computer's memory. They can be used  to set the radio's memory if

so desired. There are 100 memory slots available in this program.

Since only 20 of them can be displayed at  a time, the  PgUp  and

PgDn  keys  are  used  to display the rest.  Information  can  be

directly entered into most of the fields displayed.

Toward  the bottom of  the screen  is the COMMAND line. This is

where  commands to  the program are entered. All command  data is

automatically  converted to upper  case.  To  the  right  of  the

command line is  the LAST LOGGED status. This  indicates the last

information (frequency, modulation) sent by the radio.

THE DISPLAY                                                     6

RADIO REMOTE CONTROL PROGRAM

DATA FIELDS

Each memory channel displayed  has  several  fields associated

with it.  Most  of these fields  can be modified just  moving the

cursor to the field and typing.

VALID

The VALID field determines whether or not SCAN and SEARCH will

use the frequency currently entered into that  channel. The field

is set ON  by entering an "X"  while  the cursor is positioned on

that field. The  space  bar  turns  the VALID  flag OFF. The SCAN

operation  will  NOT scan that memory channel  on the ICOM if the

VALID field corresponding  to that memory channel is OFF. For the

SEARCH operation,  any frequency entered in a memory record  that

has the VALID flag off will NOT cause the search to stop and log.

Entering a "P" in the  valid  field  will cause the channel to

become the current priority channel.

DELAY

The DELAY  field is use during the SCAN  operation. If delay is

set  ON, scanning will continue one second  after the signal ends

on  that  channel.  If  delay is  set  OFF,  then  scanning  will

continue immediately  after the signal ends.  The field is set ON

by entering an "X" while the cursor is positioned on  that field.

Any  other key turns  the DELAY flag OFF. Delay affects only  the

specified memory channel.

FREQUENCY

The FREQUENCY field is used to record or set the current memory

channel frequency. When a SCAN  or SEARCH  stops on a  channel or

frequency, the frequency currently set  in the ICOM  is logged in

the appropriate display channel. If a frequency is entered into a

specific channel, the memory channel on the ICOM corresponding to

the  channel  on  the  display  is  set  to that  frequency.  The

frequency  is entered by positioning the cursor to  the FREQUENCY

field  of  the  desired channel,  and  entering  in  the  desired

frequency.  Only number input is accepted.  The  decimal point is

DATA FIELDS                                                     7

RADIO REMOTE CONTROL PROGRAM

always assumed,  and never entered.  The  LOAD  command will also

set the frequency to correspond with the radio memory channel.

MODE

The  MODE  field is used to display  or set  a specified memory

channels modulation. If a memory channel  is  logged  by the SCAN

operation,  the  selected  modulation  of  that  channel  will be

displayed  in this field. A channel's  modulation  can be set  by

moving the cursor  to the MODE field and entering in the  desired

modulation. MODE recognized by the program are as follows:

radio                valid modulation

ICOM                  AM, FM, FMN, SSB, CW ,LSB ,USB

KENWOOD               AM, FM, FSK, CW, LSB, USB

Note that the ICOM will accept  CW, LSB and USB. These modes have

no  meaning  for  the  R-7000,  but  are  accepted  by  the  ICOM

interface.  They are provided  for compatibility with  other ICOM

radios.

COUNT

This field displays the number  of times a signal was detected

on a  specific channel. This  field cannot be  altered.  It  can,

however be reset. The RESET command  will cause this value  to be

reset  to  0.  Ctl-END will also reset the COUNT, as well as  all

other fields on the selected information line.

DURATION

This field  displays the  average  number  of  seconds  that a

signal was detected on  a specific channel. This field cannot  be

altered.  It can, however  be reset. The RESET command will cause

this  value to  be  reset  to  0.  Ctl-END  will  also  reset the

DURATION, as well as all other fields on the selected information

line.

DATA FIELDS                                                     8

RADIO REMOTE CONTROL PROGRAM

SERVICE

This field is  used  to  record  the  service for  a  specific

channel.  It is not modified by any of the commands except DBASE.

This is a  scratch  pad  location to record information  about  a

specific  frequency. The  DBASE  command will  fill in this field

from  the frequency database if it  finds a  match of the current

FREQUENCY.

DATA FIELDS                                                     9

RADIO REMOTE CONTROL PROGRAM

COMMANDS

All  commands  are entered from  the  COMMAND line. A  command

stack is provided,  so that a command can  be  entered before the

previous command  is complete. The number  of commands stacked is

limited only by the available memory in your computer.

When a command is stacked,  the  status line  (on the bottom of

the screen) is updated  to  show  the number  of commands queued.

The "Q" indicator shows  the number of stacked commands. Pressing

ESC  will terminate  the  command in process, and cause the  next

command in the queue to be processed.

If a command is not valid when entered, the system will "BEEP"

and the invalid command will be left in the COMMAND line, so that

it  can be changed. This will also happen if there  are  too many

commands in the command stack.

BAUD  {value}

The communication baud rate  between the RS232  port  and  the

radio is set with  this command. The default  is  1200 baud.  Any

valid baud rate can be set. However, for the program to work, the

radio  and the computer  MUST  be set to the same baud rate.  The

ICOM supports 9600, 1200  and 300 bauds ONLY. The  ICOM baud rate

is selected by a jumper inside the unit. It is set at the factory

to 1200 baud. It is recommended that 9600 be used, for efficiency

in communications  between  the computer  and  radio. This is the

default baud rate for the ICOM program. The KENWOOD is hard-wired

for a 4800  baud  rate.  This is the default  baud  rate  for the

KENWOOD program.

CHANNEL {channel-number}

The CHANNEL  command  sets the  radio to a  specific  channel.

Example: CHANNEL 3 will set the selected  radio to memory channel

3.

CLEAR

The  CLEAR command will clear all  memory  channels  from  the

computer's memory. All FREQUENCY and SERVICE  fields are reset to

blanks. The DELAY fields are set  off, and the COUNT and DURATION

fields are  reset  to 0. The first 20  frequencies are marked  as

COMMANDS                                                       10

RADIO REMOTE CONTROL PROGRAM

VALID, and the rest of the VALID fields are set off.

This  command only affects the  COMPUTER memories.  It has  no

affect on the radio channels.

COM1

This  command  sets the  communications port  for the radio  to

COM1.  This is the default setting. After this command  executes,

all communication  between  the computer  and  the radio will  be

through the COM1 port.

COM2

This  command  sets  the  communications port  for the radio to

COM2.  After this command executes, all communication between the

computer and the radio will be through the COM2 port.

DBASE

A  sample DBASE compatible file  is provided  to  hold  details

about  various  frequencies.  This  file  can  be  updated  using

DBASE-III  or  any clone capable of updating  a DBASE type  file.

The  file  is  called  FREQLIST.DBF.  The  sample  file  contains

information on nationally allocated frequencies.

When the DBASE command is issued, this database is searched for

matching  frequencies. If a match  is found between the FREQUENCY

field,  and the database, the  SERVICE  field is copied from  the

database. This is an easy way to update the SERVICE field. Please

note that the DBASE  command can  take  a long time  to complete,

based on the size of the database file and the available memory.

When  processing the DBASE command, the program will attempt to

buffer as much information  in  free  memory  as possible.  Since

processing from  disk  takes a  lot  longer than processing  from

memory, this will speed  up the DBASE search. The memory for this

buffering comes from the  free memory left over after the program

is  loaded. The more free memory  available, the faster the DBASE

search will finish.

It is very  important that  the structure of the provided DBASE

file be maintained. This program will not be able to process this

file if certain field structures are altered. See the section  on

data file formats for the details on this file.

COMMANDS                                                       11

RADIO REMOTE CONTROL PROGRAM

DIR  {filespec}

When  issued,  this  command  displays  files  on  the  current

drive/directory. If issued by itself, ALL files are listed.  If a

filespec  is included, only those  files  matching  the  spec are

listed. This command CANNOT list files on a drive/directory other

than the current one.

FILENAME  {filespec}

The  FILENAME command  will set the  current file used for SAVE

and RECALL commands.  Please  note  that  only  the  filename and

extension should  be entered.  Paths should NOT be included. This

program can only save or load from the current directory.

FKn {command}

The FK command allows setting of function keys 1-10. "n" is the

function  key you wish to set.  Any  valid program command can be

set to the selected function key.  The program  comes with  a few

of the keys already defined. These can be overridden using the FK

command.

Example: FK2 QUIT

After execution  of this command,  pressing Function key  2 will

cause the program to terminate.

DEFAULT settings:

F1 - HELP, F3 - QUIT, F5 - SCAN, F7 - HALT

HALT

If the program is SCANning, SEARCHing, or LOADing, this command

will terminate that function. It  is  equivalent to  pressing the

ESC key.

COMMANDS                                                       12

RADIO REMOTE CONTROL PROGRAM

HELP

The HELP command will display a list of ALL valid commands, and

a short description of each. The current setting of function keys

is also displayed.

INC {value}

The INC command sets the increment value  used for  SEARCH. The

value  entered  will be added to the  current frequency for  each

step of the SEARCH routine.  The increment value is assumed to be

in kilohertz, and must be in the range of .1 to 999.9.

LOAD

This  command  loads  the computer memory  channels  with  the

contents  of  the  radio's  memory  channels. All the COUNT,  and

DURATION  values are set to 0. The SERVICE fields are clears, and

all DELAY  flags are turned off. The first 20 channels are marked

as  valid. This command  is  useful  initializing  the  program's

memory channels.

LPT1

LPT1 sets the default printer port to LPT1. The printer port is

used by the PRINT command. This is the  default printer  port for

this program.

LPT2

LPT2 sets the default printer port to LPT2. The printer port is

used by the PRINT command.

COMMANDS                                                       13

RADIO REMOTE CONTROL PROGRAM

NOSIGNAL

This command turns off  the signal  sensing  for the radio. The

signal sense is the CARRIER DETECT line of the RS232 port. If the

modified interface is not installed, this signal does not reflect

the state of the radio's squelch.  Issuing  the NOSIGNAL  command

tells the  program  to ignore the state of this line. When signal

detection is off, the signal status field  displays the word  OFF

constantly.

PRINT

When issued, this command prints the contents of the computer's

memory channels to the default printer.

PRIORITY  {channel-number}

Any of  the 100 channels can be selected as a priority channel.

During SCAN,  the  selected  priority channel  is  polled  once a

second.  If there is a signal on  the priority channel,  scanning

will stop on  the priority channel until the signal is no  longer

present.  Scanning will then  resume with the  last  channel that

was scanned before the priority channel activated.

To turn  off  the priority scan, enter PRIORITY with no channel

number.

QUIT

This command terminates  the program and returns to DOS (or the

calling environment).

RECALL {filename} {SET}

RECALL loads the computer memories with the values specified in

the  current file. If a channel is not included in the file, then

it will not be set. The radio's memories are  not affected unless

the SET option is included in command. If  a filename is included

with  the  command, the file specified  will  be  read,  and  the

current filename will be set to the  file  entered. If a filename

is NOT included in the command, then the current filename will be

COMMANDS                                                       14

RADIO REMOTE CONTROL PROGRAM

used.

NOTE:If the filename you wish to recall is called SET, and you do

NOT wish to set the  memories, you will need to first  issue  the

FILENAME command,  then  the  RECALL  with  no  options.  This is

because the program will not be able to tell that the SET keyword

is a filename and NOT a command option.

RESET

When this command is issued, all COUNT and  DURATION values are

reset to 0.

RESUME {value}

The value  entered here controls how  long SCAN or SEARCH  will

remain on a channel after  a  signal is  detected.  A  value of 0

causes scanning to continue ONLY after  the signal is  no  longer

present. Any  other value will cause scanning to resume after the

number of seconds entered have elapsed.

For example  if the command RESUME 3  was entered, and a SCAN

operation detected a squelch on channel 1, scanning would stop on

channel 1 for 3 seconds, then continue on to the next channel.

SAVE  {filename}

This command will save the contents  of  the computer memory to

the current filename specified. If the filename is not specified,

the current filename will be  used. If a file already  exists, it

is updated with the current settings.

SCAN

The  SCAN command starts  the radio  scanning through  selected

memory channels. Memory channels that are flagged with  the VALID

field will be scanned.  Channels with the VALID flag  off will be

bypassed. When a  signal is detected, scanning will stop, and the

memory channel being  scanned will be displayed in inverse.  When

the signal stops, scanning will continue.

COMMANDS                                                       15

RADIO REMOTE CONTROL PROGRAM

Several  parameters and  memory  channel  settings  affect  the

operation of SCAN. Each  channel has a VALID flag associated with

it. If this flag is set off, the  corresponding memory channel in

the  radio  will  NOT  be  scanned.  If  the  DELAY  field of the

corresponding  memory  channel  is set, the program will  delay 1

second after the signal stops before resuming scan.

The SPEED setting determines  how  fast each  channel  will  be

scanned. In  the  event  that  a scan is  too fast to respond  to

certain frequencies, it can be slowed with the SPEED settings.

The  RESUME  setting determines  the  action  after the squelch

breaks. If RESUME is set to 0, scanning  will not continue  until

the signal  ends. If  RESUME is  any other  value,  scanning will

resume after the number of seconds specified by RESUME.

If  a  channel is set to be  a  PRIORITY channel,  it  will  be

scanned  once every  second.  If  a  signal is  detected  on  the

priority channel normal  scanning stops,  and the radio is set to

receive the  priority channel.  Normal scanning  will resume when

the signal is no longer present on the PRIORITY channel.

Note  that any of the parameters or memory channel settings can

be set either before the scan is started, or during the scan. The

new parameters will take effect immediately in either case.

SEARCH

SEARCH allows you  to scan  a range of frequencies for action.

The range is selected by the START and STOP values. The INC value

determines the step size  for each frequency increment. The SPEED

value determines how fast the next increment will be selected.

During a SEARCH  operation, a frequency is set  into the radio,

and  a check is made for a  signal.  If  no signal is found,  the

frequency is  incriminated  by the  INC  value  and the operation

repeated. When the  frequency reaches the  STOP value, the search

starts again at the START value.

If  a  signal is found  during SEARCH,  the memory channels are

searched  for the active frequency.  If  the  active frequency is

found in one of  the computer memory channels, and the channel is

marked as VALID, the COUNT and  DURATION  fields will be updated.

If the memory channel is NOT marked as VALID, searching continues

with the next frequency.

If a signal is found during SEARCH, and the active frequency is

NOT found in  one of the  computer  memory  channel,  the  active

frequency is logged in  the first  clear memory channel. If there

are no  free  memory channels, the FULL status indicator  will be

displayed,  and the log of the current  active  channel  will  be

lost. You can free a memory  channel  during SEARCH by moving the

cursor to the row of the  selected  channel and pressing CTL-END.

Since  there are only 100  channels available,  this mean that  a

COMMANDS                                                       16

RADIO REMOTE CONTROL PROGRAM

search can only locate a maximum of 100 active frequencies.

The RESUME value controls SEARCH in the same manner as SCAN. If

RESUME  is set to 0, searching will resume AFTER the signal is no

longer present. If  RESUME is  set  to  a number greater  than 0,

searching will resume  that  number  of seconds  after signal has

been detected.

SET

SET causes the values in the computer memory channels to be set

into the radio's memory channels. All 100 channels are set.  Note

that it may take a minute or so for all the memories to be set.

SIGNAL

SIGNAL turns the sense for the signal detection on. This is the

default condition for the program. It is the  opposite command of

NOSIGNAL.

SORT

SORT will sort the memories by frequency in ascending order. It

is most useful to do a sort during or after a SEARCH operation.

SPEED {value}

This command sets the scanning  and  searching speed. The value

should be  a number between 0 and 32768. A Speed value  of 0 will

result in  the  fastest  scan possible.  Higher  numbers  cause a

slower scan. The number  is used as a countdown in the scan loop.

The larger the number, the slower the scan/search rate.

START {value}

The value set in this  command controls  the starting frequency

for  the  SEARCH  operation. The value must be  frequency  in the

range of the radio.

COMMANDS                                                       17

RADIO REMOTE CONTROL PROGRAM

STOP  {value)

The value set in this command controls the ending frequency for

the SEARCH operation. The value must be frequency in the range of

the radio.

TXADR {value)

ICOM  radios can be set to respond to  various  addresses. This

allows multiple radios to be connected on the ICOM C-IV bus.  The

default  address of the R-7000  is 8. This is the default setting

for this  program. The TXADR allows the program to talk to radios

with a different  address.  The  value entered must be an integer

between 1 and  255.  This command has no  affect when running the

KENWOOD program.

NOTE:If the program does not appear to be communicating with your

radio, make sure that the TXADR value is  matched with the remote

control address of  your radio. This  is  set  by a jumper inside

ICOM radios.

COMMANDS                                                       18

RADIO REMOTE CONTROL PROGRAM

SPECIAL KEYS

ENTER

The ENTER key finalizes input to the program. If pressed  while

on the COMMAND line, the entered command is  executed (if valid).

If pressed while on the FREQUENCY field, the entered frequency is

set  into the radio's memory, and the cursor is moved to the MODE

field. If pressed while on the MODE field, the entered modulation

is checked for validity.  If  the modulation  is valid, it is set

into the radio's memory,  and  the  cursor moves  to  the SERVICE

field.

If the cursor  is on any other field,  ENTER will move it to the

next field in the current row.

ESC

Pressing ESC terminates the current operation. It is equivalent

to the HALT command.

Backspace

The destructive backspace key only works  on the command  line.

When  entered, the current character  is deleted, and the  cursor

moves to the previous character.

HOME

The  HOME key  will  move  the  cursor to the  beginning of the

current field.

SPECIAL KEYS                                                   19

RADIO REMOTE CONTROL PROGRAM

END

If pressed while on the SERVICE field, the  cursor  is moved to

the last entered character in that field. If pressed while on the

command line, the  cursor is moved to the last  entered character

in the command.

ctl-END

This key clears all information from  the current channel.  The

current channel  is the line on which the cursor is currently on.

When  pressed,  the  VALID  and  DELAY  flags  are  cleared,  the

FREQUENCY field is set to blanks, the MODE field is set to '???',

the COUNT and DURATION  fields are zeroed, and the  SERVICE field

is set to blanks. This key only affects  the computer memory. The

radio is left unaffected.

ctl-HOME

This key will move the cursor between  the command line and the

memory  channel  fields. It allows  for a hot-key  switch between

field editing and command editing.

Cursor keys

Cursor  keys  allow  movement  around the  screen  to  edit the

various fields, and  contents of fields. The left and right  keys

move within a field. The up  and  down arrows move between  rows.

The  program will  only allow  the  cursor  to  be  placed  on  a

changeable  field.

PgDn/PgUp

These keys move between the pages  of the  display. There are 5

pages of channel displays. Each page  displays 20  channels. PgDn

selects the next page, PgUp displays the previous page.

SPECIAL KEYS                                                   20

RADIO REMOTE CONTROL PROGRAM

INS

INS   selects  or  cancels  insert  mode.  Insert  mode  allows

inserting of characters in a field.

DEL

The DEL key deletes the characters at the cursor location.

Function keys

Function  keys  F1-F10 can  be set  by the user  using  the FK

command.  When one of these  keys is pressed, the current command

assigned to  this key is  executed. Function keys that  have  not

been set do not cause any action to be taken.

SPECIAL KEYS                                                   21

RADIO REMOTE CONTROL PROGRAM

PROFILE

The profile  is a file named progname.PRO, where  "progname" is

the name of the program you started. EXAMPLE: KENWOOD.PRO  is the

profile  that would be executed when the KENWOOD program was run.

The profile contains a collection of valid commands.  The primary

use  of the profile  is to set up the program for  your  specific

setup.  Any  valid command  can be  included in  the  profile.  A

sample profile  for each  of  program  has  been included on  the

distribution disk. If you do  not wish any profile to  be run, it

may be deleted.

PROFILE                                                        22

RADIO REMOTE CONTROL PROGRAM

DATA FILE FORMATS

There are several  files  that this program makes use of. These

files are used to store and retrieve data used by the program.

FREQLIST.DBF

This  is  a  standard  DBASE  III+ type data file. It  contains

frequencies and  allocations for  the  radio spectrum. There  are

three fields that are used by this program.

field name     allocation       description

-----------------------------------------------------------

FREQUENCY      decimal 9.4      holds the frequency in Mhz

DESCRIPT       character 50     usage of the frequency

ACTION         logical          flags the frequency in use

If the ACTION field is not present, the  program will  use  the

DESCRIPT field  to  determine  ACTION.  A  blank  DESCRIPT  field

signals an inactive channel.

The length of the DESCRIPT field can be changed, if so desired.

This program only uses the first 38 chars of this field.

Any other fields can be added or deleted from this file without

affecting the  operation of the program.  However, if either  the

FREQUENCY  or  DESCRIPT field is not present in the database, the

DBASE function will not operate.

frequency data files

The  SAVE  command creates a file with the current  settings of

memory channels, and various scan parameters. The file created is

an  ASCII  readable  file.  Each record is terminated by  a CR/LF

sequence. One  record represents  one memory  channel or one scan

parameter.  Each  field  is  blank delimited.  The  file  can  be

altered (or created) using any standard text editor.

If  the first  non-blank character in a  record  is a  '*', that

record is ignored. This  provides a way to imbed  comments in any

data file.

The format for a memory channel entry is as follows:

field   name       value range      description

DATA FILE FORMATS                                              23

RADIO REMOTE CONTROL PROGRAM

----------------------------------------------------------------

1   CHANNEL     0-100           channel number to set

2   FREQUENCY   0-999.9999      frequency in Mhz

3   MODE        3 chars         valid modulation for radio

4   COUNT       0-999999        count of squelch breaks

5   DURATION    0-999999        total duration of squelch

6   VALID       VALID/NOTVALID  indicator of channel valid

7   DELAY       DELAY/NODELAY   indicator of channel delay

8   SERVICE     38 chars        service for channel

The service field follows the first blank after DELAY. All chars

up to the CR/LF are taken. If there are more then 38 chars in the

service field, they are ignored.

The format for a scan parameter entry is as follows:

parm   value

The valid parameters and there values are as follows:

parameter          value       description

START              0-999.999   start frequency in Mhz

STOP               0-999.999   stop frequency in Mhz

INC                .1-99.9     increment in Khz

SPEED              0-32768     scanning speed

RESUME             0-32768     resume value

profile

The  profile  is  a file with valid commands that gets executed

when the program starts. The profile name is

progname.PRO  where  "progname" is the name of the

program running.

Any  command  that can be executed  on  the command  line can be

placed in  the profile. Each record in the  profile contains  one

command  (and  parameters)  followed  by  a CR/LF.  If the  first

non-blank character in  a record is a '*', the record is ignored.

This allows the user to comment the profile.

DATA FILE FORMATS                                              24

RADIO REMOTE CONTROL PROGRAM

RS232 INTERFACE

This program is designed  to use the serial communications port

(RS232) of the IBM  computer. In  order  to  interface a radio to

this  port,  an  interface adapter  is  needed  designed for  the

specific radio.  Each  radio manufacturer  makes such an adapter.

However, none  of the  adapters  supplied from  the  manufacturer

provide for squelch detection. Without this detection, the signal

sensing functions of this program will not work.

For  each  radio  supported, a  circuit  and  instructions  are

available.  This circuit provides the squelch detection needed by

this  program.  The interface  circuits are  easy  to build,  and

require few parts.  All  of  the parts can be obtained cheaply at

any electronics supply store. The schematics and instructions are

provided to all registered users.

RS232 INTERFACE                                                25