��������                ��

��������              ���

��������            ���

��������          ���

��������        ���

��������      ���

��������    ���

��������  ���

������������     ���������

�����������       9  4  0

�������� ���     ���������

��������  ���

��������   ���

��������    ����

��������     �����

��������      ������

��������       �������

��������        ��������

��������  ���������������������

��������   S  p  o  i  l  e  r

��������  ���������������������

THE 940 SPOILER: VERSION 2.00

USER'S MANUAL

The 940  Spoiler provides  you with  the full  operating capability of the

Kenwood TS-940S  amateur radio  transceiver in  the easiest and most convenient

manner possible.   It  also provides  you with a way to control the transceiver

from across  the room, across the house, down the tower, and across the nation.

The 940  Spoiler will  also operate  (within the limits of the 40 memories that

the TS-940S contains) the Kenwood TS-440S transceiver.

The 940  Spoiler is  provided to you through shareware marketing, a method

which gives  you the  finest software available at a minimum price by providing

you a  means to  try out  what you  might find  useful enough  to  support  its

maintenance and  further development.   You  are invited to use the 940 Spoiler

for a  period of  up to  one month.   If  you decide  to use  the  940  Spoiler

thereafter, you  must obtain  a license for the software by registering it with

the QSO2000  Project, 5518  12th Street,  Lubbock, TX,  79416 for a nominal fee

of $39.  A convenient registration form is included at on the last page of this

manual and in the file REGISTER.TXT.

Registering your  software provides  you with  significant benefits.   You

receive a diskette copy of the most-recent revision of the Spoiler.  Registered

copies of  the package  contain the  latest  enhancements,  latest  maintenance

revisions, and  no shareware notices.  They also include enhancements which are

unavailable in  shareware examination  copies.   Your period of registration is

one year,  and enhancements  which are made to the package within that year are

yours for merely a copying and mailing charge $3.00).

OVERVIEW

The 940  Spoiler provides  you with the computing power that will help you

to make  better and  more enjoyable  use of  the Kenwood  TS-940S amateur radio

transceiver in two ways: (1) by overcoming operating difficulties inherent in a

very capable  radio with  a very restrictive front panel, and (2) by permitting

you to  control the  radio remotely  from wherever you can establish two duplex

voice channels.

OVERCOMING OPERATING DIFFICULTIES

The Kenwood  TS-940S is  by itself  a  very  sophisticated  high-frequency

transceiver.  It contains 40 memories.  It contains a general-coverage receiver

and transmitter and a unique receive and transmit increment-tuning system.  The

operating potential of the transceiver is tremendous.

Because  of  the  operating  capabilities  of  the  transceiver,  however,

operation from the front panel is of necessity complex.  Storing, altering, and

using all  of the  information into TS-940S memories through the front panel is

time-consuming  and   often  confusing.     In   addition,  in  some  operating

environments, the 40-memory capacity of the transceiver is insufficient.

Memory management is difficult with the TS-940S in two important respects.

Use of  more than 10 memories requires that you slide open a door on top of the

transceiver and  select another  of four  "banks" of  10 memories.   Also,  you

cannot enter  new values  into any  memory while  it (or  any other)  memory is

selected.

Limitations TS-940S  display also  present operating problems.  You cannot

see the  frequency and  mode of  a memory  or VFO  unless  you  select  it  and

therefore cease  to receive  or transmit  on your  current frequency  and mode.

Moreover, you  cannot  see  both  your  transmit  frequency  and  your  receive

frequency at  the same  time.   If you  are using  increment tuning  on  either

transmit or  receive, the  display shows you the result of your using it but it

does not  show you  the actual frequency of the VFO or memory that is currently

selected.

The 940  Spoiler overcomes  the problems  presented  by  the  front  panel

switches and  the display.   Your MSDOS computer keyboard provides considerably

more convenient  and logical control of the transceiver.  With a minimal number

of keystrokes, you can select any of the operating parameters (frequency, mode,

etc.) stored  in the  TS-940S, and  you can  conveniently alter  the  operating

parameters stored  in the  transceiver.   From your keyboard, you can select or

update any of the TS-940S memories (numbered 0 to 40) regardless of bank-switch

position, even when it (or another memory) is selected.

Display

The Spoiler  displays all  operating frequencies, modes, frequency splits,

tuning increments,  and other  operating information  on  the  screen  of  your

computer in one of two ways that you select by keyboard toggle.

In a  standard operating screen, all of the memories stored in the TS-940S

are displayed on the screen at all times; you can toggle between the lower half

and the  upper half  at any  time with  a single  keystroke.  In a high-density

operating screen,  the  memories  of  the  940  are  displayed  along  with  40

additional "virtual"  memories, frequency  and mode  sources that  may be  used

almost as easily at the 40 memories which are internal to the TS-940S.

The Spoiler screens always display the current operating conditions of the

TS-940S, regardless  of whether  you have  used the front panel or the computer

keyboard to  change them.   If  you change  frequency, mode,  split,  increment

tuning, or  memories from  the front  panel, the Spoiler updates its display to

show the remotely entered data.

Function key 9 toggles between the standard and high-density screens.

Control

The Spoiler  makes transceiver  control extremely  simple, convenient, and

fast as prompts direct you through a minimum of key strokes.

Memory Management

The 940  Spoiler contains a complete memory-management system that enables

you to store up to tens of thousands of frequency and mode memories on computer

disk.

Stored in  files of  40 memories, the memories may be loaded into your TS-

940S with  a few  key strokes.   Two  of these  files may  be loaded at once to

provide you  instantaneous access  to eighty  memoriies.    You  may  customize

individual memory  files for different modes of operation, for different bands,

for DX  or traffic-net  operation, for  monitoring broadcasts,  for  monitoring

citizens  band  transmissions,  or  for  whatever  your  specialized  interests

include.   Moreover, the  Spoiler manages  VFOs and memories in a way that does

not disturb your current operation.

While you  may load  and store  an unlimited  number of  memory files  for

active use  with the  940 Spoiler,  you also  have use  of a  reserve file that

effectively gives  you a  second group  of 80  memories to  work with.  You may

exchange this  file with  the active  file with a couple of keystrokes, and you

can copy and exchange individual memory contents between the active and reserve

memories with another couple of keystrokes.

You can  enter new  frequencies and modes into unselected memories or VFOs

without missing  a word  on a  net or  a calling  frequency you  are  currently

monitoring.

With the  940 Spoiler, you never have to worry about memory-bank switching

with the  toggle switch  inside the  sliding door  on top  of the TS-940S.  The

Spoiler renumbers  all memories,  for the sake of convenient program operation,

from 0  through 39  and disables  the memory-bank  switch while  the program is

running.

Simplicity of  operation is  the most  important design feature of the 940

Spoiler.   From your  screen, you see prompts to lead you through any operation

you care  to perform.   If  you want to change the operating mode of a vfo or a

memory, you press the MODE function key; you press SOURCE, FREQUENCY, INCREMENT

TUNING, COPY, and FILES keys with similar ease to instantaneously control every

feature of your transceiver.

Keypad Operating Control

The numeric keypad (or the corresponding enhanced arrow and other function

keys) provides  for truly  real-time remote  operation.   Arrow  keys  let  you

instantaneously alter  the main  operating frequency  or receive  and  transmit

increment tuning.

Keypad tuning  of the  main  operating  frequency  is  effective  for  all

memories as  well as  both VFOs.   In  effect, when you control tuning from the

keypad, you  have 42  VFOs to  work with  rather than two VFOs and 40 memories.

When you  have finished  tuning, you  have  the  choice  of  leaving  your  new

frequency in the VFO or selected memory or reverting to the frequency which was

in effect before you began tuning.

Arrow keys  do the  same work as front-panel knobs.  Press a key in one of

the two  pairs  of  arrow  keys  to  begin  incrementing  or  decrementing  the

frequency; the  keys in  the other pair serve as accelerator and decelerator to

control the rate of frequency change.  Pressing one of the original keys pauses

tuning, and  pressing the  home key while tuning is in process reduces the rate

to its  slowest value.   While  tuning is  paused, pressing one of the opposite

pairs of arrow keys halts the tuning process.

Of course, changes in main tuning are instantaneously shown on the Spoiler

display so that you know your frequency to 10-Hz accuracy at all times.

Transmit can  be activated  and terminated  from the  keypad with the Page

Down key (a feature that can be disabled on installation).

Note:  Shareware  examination   copies  of  the  940  Spoiler  do  not  provide

accelerator, deceleratior,  and transmit  functions.  These are features

reserved for registered copies.

Scanning

The Spoiler  will continuously cycle the transceiver through six groups of

contiguous memories  and six  ranges  of  frequencies  specified  by  pairs  of

memories.  Groups or ranges may be locked out, just as any single memory may be

locked.   Pause length  is set  independently for  each of  the six groups, and

scanning speed independently for each of the six ranges.

Note:  Shareware examination  copies of  the 940  Spoiler are  provided with  a

"standard" set  of scan  specifications (memory  groups,  ranges,  pause

lengths, and  speeds) which may be altered while the program is running.

The changes, however, are not saved for use after the program is exited.

Between-session editing is a feature reserved for registered copies.

Shelling to Another Program

The 940  Spoiler also  contains provisions for accessing a log-maintenance

program, a digital-controller program, or other software while you maintain the

940 Spoiler  resident in  memory for  quick access.   You  may shell out of the

Spoiler and re-enter it instantaneously.

OPERATING REMOTELY

With two  duplex voice  channels connecting  your transceiver location and

your (and  your computer's)  location, the  Spoiler will help you with complete

remote operation of the TS-940S.

The simplest  case is  at a single QTH where you run rs-232 cable from one

room of  your house  to another  to connect the serial port of your computer to

the serial  input of  your TS-940S.   Another  cable carries  audio between the

transceiver and  a speaker  and microphone  at your  computer desk  (a cordless

telephone link  is easier  to implement,  if you  have  the  equipment  already

available).

For longer  links via  telephone channels, a modem is required at each end

of the  computer-to-transceiver line.   The  modems, for  simplest lashup, must

operate at  4800 baud  with 8N1  or 8N2  byte  setup,  and  the  modem  at  the

transceiver site must be initialized for auto answer with no result codes to be

issued to  the TS-940S.   A  simple telephone  patch activated  by  ring-detect

circuitry can  provide the  remote transmit/receive  audio link  on the  second

line.

The QSO2000  Project  does  not  recommend  such  lashups  except  at  the

beginning stages of experimenting with remote operation.  An interface computer

at the remote site provides many advantages in convenience and security in your

exclusive control  of the  transceiver.  The interface computer can convert any

telephone-line data  rate to  the 4800-baud  rate required  by the TS-940S, and

with minor  programming effort  provide for password-screened access to control

of the  transceiver.   Although the  data rate  for standard  copies of the 940

Spoiler, version  2, is  fixed at  4800 baud,  a special copy that incorporates

selectable data  rates (300-19200 baud) and a 40-character modem-initialization

string is  available to  previously registered  users at  a minimal  $3.00 copy

charge.   (Such a  special copy is available only after operators have received

and familiarized themselves with the standard 4800-baud version).

It is,  of course, possible with standard copies of the Spoiler to fix the

rate of  transfer between the computer and modem at 4800-baud while setting the

telephone-line data  rate  to  a  different  speed.    Such  preliminary  setup

operations must be carried out before running the Spoiler.

Note:  Responsibility for  failure or  abuse over  such long  remote links lies

totally with those of us who use the Spoiler within the wider context of

establishing and  maintaining the  links.  Spoiler software provides you

with the  opportunity to  engineer long  links within which the software

may be  of use  to you,  but the  responsibility  for  establishing  and

maintaining the  integrity and  fail-safe  operation  of  the  links  is

entirely your own.

In no  event will  the  QSO2000  project  be  responsible  for  material

damages,  operational   failure,  or   improper  or   illegal  operation

(equipment used  is the responsibility of its owners and procedures used

are the  responsibility of amateur radio operators whose licenses attest

to their  having the  knowledge and care to operate within the limits of

the licenses they have obtained).

In other  words, remote-control  operation is  always full of complexities

that amateur  operators must  understand before they commit their equipment and

licenses to supporting it.

HARDWARE REQUIREMENTS

The 940  Spoiler requires an MSDOS or PCDOS computer with at least 256K of

memory, one  diskette drive,  and one  serial (COM)  port.   Performance of the

program is improved, of course, with your running it from hard disk rather than

floppy.  The faster your CPU speed the more responsive the program.

Your TS-940S must be equipped for serial digital communications.  You must

have an interface kit such as the ones Kenwood sells, designated as the IC-10B,

as well  as a  TTL/RS-232C level converter such as Kenwood sells as the IF-232C

adapter.   It is  not necessary  for you to use the parts that Kenwood markets,

although duplicating the IC-10B kit is likely not a worthwhile project even for

the most  experienced and  well-equipped operator.   If  you are experienced in

digital-circuit design,  you might build your own level converter, or you might

purchase one from an alternate source (see Hardware Requirements, below).

The serial  interface  in  your  computer,  your  cable,  and  your  level

converter must  support control  of transmit-data, receive-data, clear-to-send,

and ready-to-send signals.

OPERATION

The opening screen of a shareware examination copy of the Spoiler contains

a reminder  that the  software depends on user registration.  The screen, shown

in figure  1, is  displayed for  10 seconds:  after the initial period, you may

bring up the standard operating screen by pressing any key.

������                ��      ����������������������������������������������͸

������              ���       �                THE 940 SPOILER               �

������            ���         �             Shareware Version 1.00           �

������          ���           �                                              �

������        ���             � We hope that you enjoy this examination copy �

������      ���               � of the 940 Spoiler, a program that lets  you �

������    ���                 � control your Kenwood  TS-940S  conveniently, �

������  ���                   � rapidly, and completely.                     �

����������     ���������      �                                              �

���������       9  4  0       � You may  support your use of this program by �

������ ���     ���������      � registering your copy with a contribution of �

������  ���                   � $39 to support further development and main- �

������   ���                  � tenence.  Your registration will provide you �

������    ����                � with a  diskette copy  of  the full-featured �

������     �����              � 940 Spoiler package as well as  free upgrade �

������      ������            � privilege.  Type  or  print the accompanying �

������       �������          � file REGISTER.TXT for further information.   �

������        ��������        �                                              �

������  ��������������������� �     >Bill Brewer, K5KNC, QSO2000 Project<    �

������   S  p  o  i  l  e  r  ����������������������������������������������;

������  ����������������������������������������������������������������������

����������������������������������������������������������������������������ͻ

� ******************  Press any key to continue to run  ******************** �

� ***************** your examination copy of the Spoiler. ****************** �

����������������������������������������������������������������������������ͼ

Figure 1.- Examination copy opening screen

Registered copies  of the  Spoiler display  an opening screen for only the

length of  time it  takes you  to press  a key.  The opening screen contains no

shareware notice:  instead, it contains the name and amateur-radio call sign of

its registered owneras shown in figure 2.

���������                ��

���������              ���

������            ���

Q ������          ���

S ������        ���

O ������      ���

2 ������    ���

0 ������  ���

0 ����������     ���������           This Registered Copy of the 940 Spoiler

0 ���������       9  4  0                          Version 1.00

������ ���     ���������                          belongs to

P ������  ���

R ������   ���                                     Bill Brewer

O ������    ����                                      K5KNC

J ������     �����

E ������      ������

C ������       �������

T ������        ��������

������  ���������������������

���������   S  p  o  i  l  e  r

���������  ��������������������������������������������������������������������

�����������������������������������������������������������������������������ͻ

� ******************  Press any key to continue to run  ********************* �

� *****************  your registered copy of the Spoiler. ******************* �

�����������������������������������������������������������������������������ͼ

Figure 2.- Registered copy opening screen

OPERATING SCREEN

The operating  screen shows  every parameter  that enters into transceiver

operation, regardless  of whether  changes in  operation occur  from the  front

panel or  from the  computer keyboard.   It  also  shows  commands  which  make

computer-keyboard control of the TS-940S incredibly easy and fast.

The Spoiler  always displays  the standard  operating screen upon startup.

The display  may be  toggled  to  the  high-density  operating  screen  anytime

thereafter.

THE STANDARD OPERATING SCREEN

The standard screen, shown in figure 3, consists of five display areas for

each of  the following: (1) current operating sources, (2) command options, (3)

slope and  variable-bandpass tuning, (4) 40 memories, and (5) commands.  Figure

2 shows  a typical  operating screen  (which has been shortened horizontally to

fit within  the margins  of this documentation file.  Current operating sources

are shown  in the  box in the upper left.  Command options are shown in the box

which contains  the program  logo when  no command  has been issued.  Slope and

variable-bandpass  tuning   are  shown   to  the  right  of  the  command  box.

Transceiver memory  contents are  shown in  the large  box to  the right of the

screen.  Commands are shown on the bottom line.

����������������������������������������ͻ�����������������������������������ͻ

�      RECEIVE       �RX  3.971.80 LSB VA��             MEMORIES              �

�    3.971.80 LSB    �TX  3.934.64 LSB VB������������������������������������Ķ

�        VA          �������������������ĺ�00   7.012.00 CW �20   7.156.00 USB�

��������������������͹TR  7.030.00 CW  16��01   7.012.00 CW �21   7.165.00 LSB�

�      TRANSMIT      �������������������ĺ�02   7.012.00 CW �22   7.185.00 LSB�

�    3.934.64 LSB    �No Increment -0.08K��03   3.779.93 LSB�23   7.195.00 LSB�

�        VB          �                   ��04   3.790.00 LSB�24   7.205.00 LSB�

����������������������������������������͹�05   3.905.00 LSB�25   7.213.00 LSB�

�Time 10:02:46�DEFAULT.MRY �Date 07-27-90��06   3.910.00 LSB�26   7.221.00 LSB�

����������������������������������������ͼ�07   3.928.00 LSB�27   7.225.00 LSB�

�����������������������ͻ���������������ͻ�08   3.933.00 LSB�28   7.235.00 LSB�

�     ����       �      ��      VBT      ��09   3.936.60 LSB�29   7.240.00 LSB�

�     ����     ��       ����������������Ķ�10   3.940.00 LSB�30   7.245.00 LSB�

�     ����   ��         ����������������ܺ�11  14.234.56 CW �31   7.249.00 LSB�

�     ���� ��  ���      ����������������ۺ�12   7.012.00 CW �32   7.290.00 LSB�

�     ������   940      ����������������ۺ�13   7.015.00 CW �33   2.500.00 AM �

�     ���� ��  ���      ����������������Ϲ�14   7.020.00 CW �34   5.000.00 AM �

�     ����  ��          ��     SLOPE     ��15   7.025.00 CW �35  10.000.00 AM �

�     ����   ���        ����������������Ķ�16   7.030.00 CW �36  20.000.00 AM �

�     ���� ���������    ����������������ܺ�17   7.045.00 CW �37  20.000.00 AM �

�     ����  Spoiler     ����������������ۺ�18   7.060.00 CW �38  14.005.00 CW �

�     ���� ���������    ����������������ۺ�19   7.075.00 CW �39  14.010.00 CW �

�����������������������ͼ���������������ϼ�����������������������������������ͼ

1)Select 2)Freq 3)Mode 4)Incrmt 5)Copy 6)Files 7)Scan 8)Shell 9)Display 10)Quit

Figure 3.- Standard operating screen

CURRENT-SOURCES DISPLAY BOX

The display box in the upper left of the operating screen displays current

sources: the  two VFOs,  the memory  selected by the TS-940S, and the increment

tuning.   This box  shows every  detail about  the  current  operation  of  the

transceiver.  An example of the current-sources box is shown in figure 4.

����������������������������������������ͻ

�      RECEIVE       �TR 21.005.00 CW  13�

�   21.005.00 CW     �������������������ĺ

�         13         �TR 27.285.00 AM  VA�

��������������������͹TR  3.921.44 LSB VB�

�      TRANSMIT      �������������������ĺ

�   21.005.00 CW     �No Increment..-0.05�

�         13         �                   �

����������������������������������������͹

�Time:14:30:11�ZEROS1.MRY  �Date:05-01-90�

����������������������������������������ͼ

Figure 4.- Current-sources display

The box  contains areas  for receiving  and  transmitting  frequency,  for

detailed source information, and for current Spoiler operation.

Receive and Transmit Areas

Overall transceiver  operation is  summarized in  the receive and transmit

areas to  the left  of the  box.   These areas  show, irrespective of operating

components that  go into  transceiver  operation,  the  current  receiving  and

transmitting frequencies,  and modes.   The  frequency and  mode that  you  are

hearing on  the transceiver,  as  well  as  the  frequency  and  mode  you  are

transmitting on, are displayed to an accuracy of 10 KHz in these two areas.

Receive  and transmit  areas show  the combined  effects of VFO or memory

frequency selection,  split-frequency sources,  and currently  active increment

tuning.   The displayed  frequencies are the actual frequencies of transmission

and reception.   The  receiving frequency  accounts for  any receive  increment

tuning (RIT),  and the  transmit frequency  accounts for any transmit increment

tuning (XIT).

When your  are receiving,  the receive  area is  highlighted; when you are

receiving, the transmit area is highlighted.

When you  use keypad  tuning, an additional symbol appears in these areas.

The tuning  symbol, unless  you are  operating split frequency, appears in both

areas to  indicate that  other Spoiler  functions are  locked out until you are

finished with  keypad tuning.   The  actual direction  of tuning is shown by an

arrow symbol in the top line of the source-detail area

These tuning  symbols also  appear when  keypad  increment  tuning  is  in

progress, and  the arrow  symbol indicating  the direction of tuning appears in

the increment-tuning line of the source-detail area.

The symbols appear as shown in figure 5.

����������������������������������������ͻ

�      RECEIVE       �TR#21.005.00 CW  13�

�   21.005.00 CW     �������������������ĺ

�        13         #�TR 27.285.00 AM  VA�

��������������������͹TR  3.921.44 LSB VB�

�      TRANSMIT      �������������������ĺ

�   21.005.00 CW     �No Increment..-0.05�

�        13         #�                   �

����������������������������������������͹

�Time:14:30:11�ZEROS1.MRY  �Date:06-01-90�

����������������������������������������ͼ

Figure 5.- Current-sources during keypad tuning

Source-Detail Area

To the  right of  the receive  and transmit  areas is  an area  containing

detailed information  about the  components of the current sources: frequencies

and modes  of the  two  VFOs  and  the  selected  memory,  the  split-frequency

operating status, and the status of increment tuning.

In the  source-detail box to the right, the upper partition identifies the

currently active  source(s) and shows frequency and mode.  The middle partition

shows the  sources "on  reserve" for your use at the press of the TS-940S VFO/M

and A/B switches.  The lower partition shows current increment-tuning frequency

and status

Unlike the  receive and transmit areas to the left or the TS-940S display,

the detail-area source display does not reflect the effect of increment turning

on the currently-selected source.  If you turn on increment tuning for transmit

or receive,  for example,  the area  to the  left will  show the  result of the

increment tuning, but the detail area will continue to show the exact frequency

of the  source without the effect by increment tuning.  The last two characters

of the  line identify  the source with VA for VFO A, VB for VFO B, and a number

between 00  and 39  for the  selected memory.   The first two characters of the

line identifies  the  role  that  the  source  contributes  to  split-frequency

operation.

The upper  partition of  the detail-source display may contain one line to

indicate a  non-split source  operating on both transmit and receive (therefore

prefaced by TR), as it does in figure 2, or it may contain two lines, as in the

display of split-VFO operation shown in figure 6.

����������������������������������������ͻ

�      RECEIVE       �RX  7.209.04 CW  VA�

�    7.209.04 CW     �TX  3.959.23 LSB VB�

�         VA         �������������������ĺ

��������������������͹TR  3.525.00 CW  02�

�      TRANSMIT      �������������������ĺ

�    3.959.23 LSB    �No Increment..-0.03�

�         VB         �                   �

����������������������������������������͹

�Time:14:43:54�GENPUR1.MRY �Date:06-07-90�

����������������������������������������ͼ

Figure 6.- Split VFO display

If keypad  tuning is  active, an  up or  down  arrow  appears  beside  the

current-source receive designation (RX or TR).

The currently active source is highlighted.  For split operation, both the

receive line and the transmit line are highlighted.

The middle  partition of the detailed display area shows alternate sources

that are currently selected: these are the sources that you can use by toggling

between VFO A and VFO B or between VFO and memory.

The lower  partition shows  the frequency  and status of increment tuning.

The frequency is shown to the right of the line, and the status is shown to the

left.   Status may  be No  Increment, RX Increment (receive only), TX Increment

(transmit only), or TR Increment (both transmit and receive).

You should notice carefully that if you toggle between VFO A and VFO B any

increment tuning  that is  in effect  remains in effect.  If you toggle between

either VFO and memory, however, any increment turning loses its effect.

If you  have receive  increment tuning, transmit increment tuning, or both

applied, the  increment-tuning line  is highlighted.  If neither is applied the

"No Increment"  shows the  frequency increment  that will  be in  effect if you

choose to use it.

If you have keypad increment tuning in effect, an up or down arrow appears

beside the  designations RX, TX, TR, or No.  During keypad tuning other Spoiler

functions are disabled.

Current Spoiler Operation

The current time, the active file of memories stored in your computer, and

the current  date are  displayed in  the area across the bottom of the current-

sources box.   Both  the time and date are set with the DOS level TIME and DATE

commands

MEMORY DISPLAY

The contents of TS-940S memory sources are displayed in the largest box on

the screen.  Figure 7 shows the memory-display box.

�����������������������������������ͻ

�             MEMORIES              �

�����������������������������������Ķ

�00   7.012.00 CW �20   7.156.00 USB�

�01   7.012.00 CW �21   7.165.00 LSB�

�02   7.012.00 CW �22   7.185.00 LSB�

�03   3.779.93 LSB�23   7.195.00 LSB�

�04   3.790.00 LSB�24   7.205.00 LSB�

�05   3.905.00 LSB�25   7.213.00 LSB�

�06   3.910.00 LSB�26   7.221.00 LSB�

�07   3.928.00 LSB�27   7.225.00 LSB�

�08   3.933.00 LSB�28   7.235.00 LSB�

�09   3.936.60 LSB�29   7.240.00 LSB�

�10   3.940.00 LSB�30   7.245.00 LSB�

�11  14.234.56 CW �31   7.249.00 LSB�

�12   7.012.00 CW �32   7.290.00 LSB�

�13   7.015.00 CW �33   2.500.00 AM �

�14   7.020.00 CW �34   5.000.00 AM �

�15   7.025.00 CW �35  10.000.00 AM �

�16   7.030.00 CW �36  20.000.00 AM �

�17   7.045.00 CW �37  20.000.00 AM �

�18   7.060.00 CW �38  14.005.00 CW �

�19   7.075.00 CW �39  14.010.00 CW �

�����������������������������������ͼ

Figure 7.- Memory display box

For 940  Spoiler operation,  TS-940S memories  are numbered  from 0 to 39.

Memory 0  is transceiver  memory 0 of bank 1, memory 10 is transceiver memory 0

of bank  2, memory  39 is  transceiver memory 9 of bank 4, and so on.  With the

940 Spoiler in opertion, you can conveniently forget about bank selection.

SPOILER COMMAND LINE

The bottom  line of  the Spoiler  display is  a prompt  for command entry.

Figure 8 shows this line.

-------------------------------------------------------------------------------

1)Select 2)Freq 3)Mode 4)Incrmt 5)Copy 6)Files 7)Scan 8)Shell 9)Display 10)Quit

-------------------------------------------------------------------------------

Figure 8.- Spoiler command line

HIGH DENSITY SCREEN

The high-density screen displays all TS-940S parameters on a single screen

in addition  to 40  virtual memories  from a  memory file.   Figure 9 shows the

high-density screen.

�������������ͻ��������������������������������������������������������������ͻ

�    RX VA    �� CURRENT MEMORY FILE  DEFAULT  � RESERVE MEMORY FILE  DEFAUT1 �

� 3.971.80 LSB���������������������������������������������������������������Ķ

�������������Ķ�00  7.012.00 CW�20  7.156.00 US�00  7.290.00 LS�20  7.156.0 US�

�    TX VB    ��01  7.012.00 CW�21  7.165.00 LS�01  7.290.00 LS�21  7.165.0 LS�

� 3.934.64 LSB��02  7.012.00 CW�22  7.185.00 LS�02  7.249.00 LS�22  7.185.0 LS�

�������������Ķ�03  3.779.93 LS�23  7.195.00 LS�03  7.012.00 CW�23  7.195.0 LS�

�Time 10:03:06��04  3.790.00 LS�24  7.205.00 LS�04  5.000.00 AM�24  7.205.0 LS�

�������������Ķ�05  3.905.00 LS�25  7.213.00 LS�05 10.000.00 AM�25  7.213.0 LS�

�Date 07-27-90��06  3.910.00 LS�26  7.221.00 LS�06 15.000.00 AM�26  7.221.0 LS�

�������������ͼ�07  3.928.00 LS�27  7.225.00 LS�07 20.000.00 AM�27  7.225.0 LS�

�������������ͻ�08  3.933.00 LS�28  7.235.00 LS�08 14.005.00 CW�28  7.235.0 LS�

�      VA     ��09  3.936.60 LS�29  7.240.00 LS�09 14.010.00 CW�29  7.240.0 LS�

� 3.971.80rLSB��10  3.940.00 LS�30  7.245.00 LS�10  3.940.00 LS�30  7.245.0 LS�

�������������Ķ�11 14.234.56 CW�31  7.249.00 LS�11 14.234.56 CW�31  7.249.0 LS�

�      VB     ��12  7.012.00 CW�32  7.290.00 LS�12  7.012.00 CW�32  7.290.0 LS�

� 3.934.64tLSB��13  7.015.00 CW�33  2.500.00 AM�13  7.015.00 CW�33  2.500.0 AM�

�������������Ķ�14  7.020.00 CW�34  5.000.00 AM�14  7.020.00 CW�34  5.000.0 AM�

�      16     ��15  7.025.00 CW�35 10.000.00 AM�15  7.025.00 CW�35 10.000.0 AM�

� 7.030.00 CW ��16  7.030.00 CW�36 20.000.00 AM�16  7.030.00 CW�36 20.000.0 AM�

�������������Ķ�17  7.045.00 CW�37 20.000.00 AM�17  7.045.00 CW�37 20.000.0 AM�

�     INCR    ��18  7.060.00 CW�38 14.005.00 CW�18  7.060.00 CW�38 14.005.0 CW�

�   No-0.08K  ��19  7.075.00 CW�39 14.010.00 CW�19  7.075.00 CW�39 14.010.0 CW�

�������������ͼ��������������������������������������������������������������ͼ

1)Select 2)Freq 3)Mode 4)Incrmt 5)Copy 6)Files 7)n/a 8)Shell 9)Display 10)Qiit

Figure 9.- The high-density screen

The command  line, with  one exception (scan), lists the same options that

the command  line for  the standard screen does.  There is, however, no command

box to explain commands; therefore you should use the high-density display only

after you  have familiarized  yourself with Spoiler operation with the standard

display.

SPOILER COMMANDS

The Kenwood  TS-940S is  a completely  self-contained transceiver, and the

front panel,  although sometimes  cumbersomely, provides  ways to  control  its

operation.   Your MSDOS  computer provides you with an alternative to the front

panel.   (The Spoiler  displays provide  current information  about transceiver

operating parameters,  regardless of  whether you  change the parameters at the

front panel or at the keyboard. )

Spoiler commands  are invoked  by function  keys 1  through 10.    Command

number 10,  labeled QUIT  on the  command line,  causes the  Spoiler  to  cease

operation and  control of  the computer  to return to the disk operating system

(DOS).

The commands  are listed  on  the  command  line  by  abbreviation.    The

abbreviations correspond  to the  PC function  keys and  the brief descriptions

shown in table 1.

Source selection (F1)....................Select a VFO, VFO split, or a memory

Frequency selection (F2)..Change the frequency of a VFO,a memory, or passband

Mode selection (F3)......................Change the mode of a VFO or a memory

Increment Tuning (F4)...Add frequency increment to transmit of receive source

Copy sources (F5)..................Copy or exchange between VFOs and memories

Memory management (F6)..................Use computer files for memory storage

Scan frequencies (F7)..........Cycle through groups and ranges of frequencies

Shell (F8).................Run another program while the Spoiler is in memory

Display toggle (F9).........Toggle between standard and high-density displays

Quit (F10)...........................Quit Spoiler operation and return to DOS

TABLE 1.- Brief descriptions of Spoiler commands

When you  press a  function key  to invoke  a Spoiler  command, you open a

window behind  the Spoiler  logo in  the box at the lower left of the operating

screen.   The window  prompts you  for information  that you need to provide to

complete any control operation for the transceiver.

Arrow keys  and the Home, Pg Up, and Pg Dn also control Spoiler functions.

The Pg  Up key  works in  standard display mode to toggle between the lower and

upper half  of the 110 memories.  The arrow and Home keys provide for frequency

control for both main tuning and increment tuning.

The following  sections of  this documentation  file describe  each of the

Spoiler commands in detail.  Each command option is also described.

SOURCE SELECTION

The source-select  command invoked  by F1 determines the primary frequency

and mode source for TS-940S reception and transmission.  The frequency and mode

of your  reception and  transmission are always from a certain source, the VFOs

or the  memories, as  you determine by issuing a source-select command.  Figure

10 shows  the options you are presented with in the command-options box for the

source-select command.

�����������������������ͻ

�   Source Selection    �

� VFO A...............A �

� VFO B...............B �

� Memory..............M �

� Split...............S �

� Mem Num............## �

� Quit................Q �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 10.- Source-select command options

As figure  11 shows, you may choose VFO A, VFO B, or the current memory to

be your current reception and transmission source.  You may choose transmission

and reception  to be  split, with  thhe currently  selected VFO  being used  on

receive and the alternate VFO being used on transmit.

The option  box is  a simple reflection of this powerful command.  You may

press A,  B, or M to select the VFO A, VFO B, or current memory.  You may enter

the number  of any  other memory  to cause it to become both the current memory

and the  selected source.   And  you may toggle the VFOs between split and non-

split operation.

Your pressing  a single  key (A,  B, or  M) causes  VFO A,  VFO B,  or the

current memory  to become  the current  source.  If the source is a VFO and the

VFOs were last used in split-source operation, your reselecting them will bring

them up  for split  operation (regardless  of which of them you select for your

primary (receive) VFO.

Your pressing the S key while either VFO is selected toggles between split

and non-split  operation.  Pressing the S key while a memory is selected has no

effect.

For the  VFOs, the  status of  split-frequency operation  remains constant

until your  change it.   If you switch from a VFO to a memory source, while the

VFOs are  linked for  split-frequency operation,  you'll find  the  VFOs  still

linked when you switch back from memory to VFO operation.

You do  have an  alternative to  pressing alphabetic  keys to  change  the

current sources  for your  transceiver.   While  the  source-selection  box  is

displayed, the  command letters  are displayed  across the  last  line  of  the

screen.   Pressing a  function key  accomplishes the same result as pressing an

alphabetic key.   Pressing  F2, for  example, selects  VFO B  as  your  current

source.

Figure 11 shows the last-line display of source-selection command options.

------------------------------------------------------------------------------

1)VFO A  2)VFO B  3)Memory  4)Split Toggle  5)Quit

------------------------------------------------------------------------------

Figure 11.- Source-selection command options

Response to your pressing the A, B, M, or function keys is immediate.  The

Spoiler does  not wait  for you  to press  enter.   If you  mistakenly enter an

unrecognized command  character, the  Spoiler will  prompt you again for a good

command option.   If  you want  to abort the source-selection operation, simply

press either "Q" or the enter key at the prompt.

The "Q"  entry represents  the Quit  command.  It appears in prompt lines,

but it  does not appear in the command-help tables.  It is always equivalent to

pressing enter when you have no character entered.

FREQUENCY SELECTION

You have  two methods  of frequency  selection available  to  you.    Your

primary method  is using  a frequency-selection command to change the frequency

of any source.  An alternate method permits you to tune your currently selected

source from the numeric keypad.

Frequency-Selection Command

The frequency-selection  command invoked  by F2 provides you with the same

basic options  that you  have with current-source selection.  You can choose to

enter a  frequency for  VFO A, VFO B, the current memory, and any memory stored

in the  transceiver.   Three other options permit you to select upper and lower

passband slope  frequencies for  SSB operation  and select  variable  bandwidth

tuning for CW operation.

Figure 12  shows the options you are presented with in the command-options

box for the frequency-selection command.

�����������������������ͻ

�   Source Frequency    �

� VFO A...............A �

� VFO B...............B �

� Memory..............M �

� Mem Num............## �

� Slope High..........H �

� Slope Low...........L �

� VBT.................V �

�  Freq of (ABM##Q):    �

�                       �

�                       �

�����������������������ͼ

Figure 12.- Frequency-selection source options

A powerful  feature of  the Spoiler is that your entry of information into

an unselected source does not require you to select the source before you enter

it.   If you  choose to enter a new frequency into an unselected VFO or memory,

you do  so without  interrupting your  listening to  and transmitting from your

currently selected  source.   If you  are operating  with VFO  A or  VFO B, for

example, you  can change  the contents  of all  memories and  the alternate VFO

without missing  a single  word of  a net  or a  single  baud  of  your  CW  DX

monitoring.

You may  select the  source sources with the function keys as well as with

the alphabetic  keys.   Pressing F3,  as figure  9 shows,  selects the  current

memory for frequency entry, just as pressing M does.

Memories other than the current one may be selected by number.

After you  have selected  the source that you want to change the frequency

of, the  command-option box  prompts you  for the  new frequency,  as shown  in

figure 10.   The  prompt verifies  the source  you have  selected (VFO  A,  for

example, in figure 13).

�����������������������ͻ

�   Source Frequency    �

� VFO A...............A �

� VFO B...............B �

� Memory..............M �

� Mem Num............## �

� Slope...............Q �

� VBT.................Q �

�  Freq of (ABM##Q): B  �

� (KHz/MHz): 14.336     �

�                       �

�                       �

�����������������������ͼ

Figure 13.- Frequency-selection prompt

If you want to abort the frequency-selection command at the prompt, simply

press enter.

Frequency  may   be  entered  in  either  MegaHertz  or  KiloHertz  units.

MegaHertz is  the default.  KiloHertz values must be suffixed with a "K," as in

"3515K" for an entry that is equivalent to 3.515 MegaHertz.  The entry shown in

the figure  above is  equivalent to  "14336K" (should  anyone care for abstruse

expression).

A special  circumstance occurs  when a  TS-940S memory is completely empty

(with no previous frequency or mode stored in it).

One of  the characteristic  of the TS-940S memories is that they can store

into an  empty memory  neither frequency  nor mode  without storing  the  other

parameter.    Because  the  Spoiler  stores  frequency  and  mode  in  separate

operations, the  program must store a "boilerplate" mode into a memory whenever

you store  a frequency.   This  mode is  LSB.  If you enter a frequency into an

empty memory,  the USB  mode is  stored simultaneously  in memory.  If you want

some other  mode for  the memory, you can change it quite conveniently from the

keyboard with the mode-selection command invoked by F3.

Tuning with the Keypad

You may tune the frequency of your currently selected VFO by pressing your

UP and  DOWN arrow  keys.   Tuning is  inhibited while you are entering command

options, and command options are inhibited while tuning is in progress.

When you  use the  arrow keys  to tune  your currently  selected VFO,  the

resulting change  in frequency  is  always  applied  to  your  current  receive

frequency.   If you  have not  selected split-frequency operation, it will also

alter your  transmit frequency.   If split-frequency operation is selected, the

transmit frequency remains unchanged.

Left and  right arrow  keys function,  respectively,  as  decelerator  and

accelerator keys.  Pressing the right arrow key after you have initiated tuning

with a  vertical arrow key increases the rate of frequency change; pressing the

left arrow  key subsequently  decreases the rate to its original value.  At any

time that tuning is in progress, pressing the Home key restores the rate to its

lowest value.

Note:  Acceleration and  deceleration  of  keypad  tuning  rate  is  a  feature

available only  in registered  copies of  the 940  Spoiler.   It is  not

available in shareware examination copies.

While tuning  is in  progress, either  vertical arrow  key may  be used to

pause it.  Pressing a vertical key subsequently resumes tuning in the direction

indicated by the key.

To exit  keypad tuning, you must pause it with a vertical arrow key.  Then

you have  two choices.  When you subsequently press one of the horizontal arrow

keys, you  exit with  the source  frequency retaining  its new value.  When you

press the Home key, the source frequency reverts to the value it had before you

began tuning.

In summary,  the  vertical  arrow  keys  initiate  tuning  and  pause  it.

Horizontal arrow  keys, when  tuning is  not paused, control tuning rate.  When

tuning is paused, horizontal keys cancel the tuning process while retaining the

current frequency.   The  Home key,  when tuning  is not paused, reduces tuning

rate to  a minimum.   When  tuning is  paused, the  Home key cancels tuning and

restores the frequency to the value it had before tuning began.

Passband Tuning and Selectivity Choices

You may  control both  slope tuning and variable-bandwidth tuning from the

frequency-selection command-option box.

For upper and lower slope tuning (active on SSB modes only), you provide a

number ranging  from 0 to 10, with lower numbers specifying a narrower passband

on each side of the center frequency.  Entering the number 0 for both upper and

lower slopes  brings both  skirts  in  as  close  as  possible  to  the  center

frequency,  and   entering  10  for  both  values  provides  you  with  maximum

symmertrical bandwidth.

The effects  of slope  tuning are  displayed graphically  inside the SLOPE

TUNING box.   Within  the graph,  the upper  and lower  tic marks  indicate the

center frequency.

For  variable-bandwidth   tuning  (VBT)  on  all  modes  except  SSB,  the

bandwidth-selection process is similar; the number entered ranges from 0 to 10,

with 0  representing the  narrowest bandwidth  and 10  representing the  widest

bandwidth.   The graphic  display for  VBT, located just above the slope-tuning

display, shows relative selectivity about a center line.

When the  940 Spoiler is used with a TS-440S, slope and variable-bandwidth

tuning commands have no effect.

MODE SELECTION

The mode-selection  command invoked by F3 provides convenient selection of

one of  the six  operating modes  for either  VFO, the  current memory,  or any

unselected memory.   All  of these  sources can  receive and  transmit  in  the

following modes.

Lower sideband (LSB)

Upper sideband (USB)

Continuous wave (CW)

Frequency modulation (FM)

Amplitude modulation (AM)

Frequency-shift keying (for RTTY)

To select  a mode  for a  source, you  first select  the source within the

command options box, as shown in figure 13.

�����������������������ͻ

�      Source Mode      �

� VFO A...............A �

� VFO B...............B �

� Memory..............M �

� Mem Num............## �

� Quit................Q �

�  Mode of (ABM##Q): A  �

�   Mode (LUCFAR):      �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 13.- Mode-selection source options

The prompt  for mode  selection shown  in figure 15 indicates that you are

selecting the mode for VFO A.  If you want to abort the selection, simply back-

space and press enter at the source prompt.

To finish selecting the mode, press "L" for LSB, and so on.

A special  circumstance occurs  when a  TS-940S memory is completely empty

(with no previous mode or frequency stored in it).

Because the  Spoiler stores mode and frequency in separate operations, the

program must  store a "boilerplate" frequency into an empty memory whenever you

store a  frequency.   This frequency  14.250 MHz.  You may change the frequency

conveniently with the Frequency Selection command invoked by the F2 key.

INCREMENT TUNING

You may  use TS-940S  increment tuning  to alter  the receive  or transmit

frequency of  your selected VFOs or memories.  You may tune above or below your

primary source  frequency by  up  to  9.99  MHz.  (When  used  with  a  TS-440S

transceiver, the  940 Spoiler  limits increment tuning to the 1.27 KHz range of

the TS-440S.)

You may  invoke increment  tuning with  the F4  key, or  you may alter the

increment tuning frequency from the keyboard.

The Increment-Tuning Command

The increment-tuning  command, invoked by F4, provides you with options to

apply a tuning increment to your current receive frequency, transmit frequency,

or both  frequencies.  You may set the increment frequency to any value between

-1.27 KHz  and 1.27  KHz, or you may clear the increment (set it to 0).  Figure

15 shows the options box for the increment-tuning command.

�����������������������ͻ

�    Increment Tune     �

� RX..................R �

� TX..................T �

� Clear...............C �

� Frequency.......�###K �

� Quit................Q �

�  Change (RTCQ):       �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 15.- Increment-tuning command options

The results of toggling receive and transmit increment tuning is displayed

in the  lowest partition  of  the  source-detail  area  of  the  screen.    The

possibilities are No Increment, RX increment, TX increment, and TR Increment.

The source-detail  area of  the screen  shows the  frequency of  increment

tuning even  when it  is not  applied to receive and transmit frequencies.  You

may alter  this frequency  from the  keyboard or  the front  panel at any time,

regardless of  whether it  is being  applied to  a current frequency.  You will

notice that  when you alter increment frequency from the keyboard, the position

of the  RIT knob  on  the  TS-940S  no  longer  corresponds  to  the  increment

frequency; when  you next  alter increment  tuning from  the front  panel,  the

position of the control will once more correspond to the increment frequency.

Whenever you  enter an  increment frequency at the command-options prompt,

you will  notice that the response to your entry is not immediate.  The Spoiler

must, in  fact, send  discrete commands  to the  TS-940S for  each 10 Hz of the

increment, so  full-range excursions may take up to 5 seconds (depending on the

speed of  your computer).   Clearing  the increment  (setting it  to 0) occurs,

however, instantaneously.

Increment frequency entry has a default of Hertz, and a "K" suffix is used

to indicate  KiloHertz.   For  example,  the  entries  "1200"  and  "1.2K"  are

functionally identical.

Keypad Increment Tuning

You may  use the  right and left arrow keys to alter the frequency of your

increment tuning.   Pressing  the keys,  however, while a command option box is

displayed, has  no effect.   The command options have no effect while increment

tuning is in progress.

After you  initiate increment tuning with the right or left arrow key, you

may use  the up  and down  arrow keys as accelerator and decelerator keys.  The

home key returns the rate of frequency change to its lowest value.

When increment  tuning is  active (either  running or  paused),  an  arrow

symbol which  indicates the direction of tuning appears in both displays to the

immediate right  of application  expression (No,  RX, TX, TR).  In the standard

operating screen,  the tuning  symbol (#)  appears in the lower right corner of

the receive source box, the transmit source box, or both as appropriate.

Pressing either  horizontal arrow  key  pauses  increment  tuning.    When

increment tuning  is paused,  it may be exited by your pressing either vertical

arrow key.

COPYING SOURCES

You may, by using the source-copy command, copy the contents of any source

into any  other source.  You may also exchange the contents of any two sources.

This command,  invoked by  F5, provides you with a convenient way to reorganize

the frequencies and modes stored in your VFOs and memories.

The option box for the source-copy command is shown in figure 16.

�����������������������ͻ

�      Source Copy      �

� Copy................C �

� Xchange.............X �

� Quit................Q �

�    Choose (CXQ): C    �

�    From (ABM##): A    �

�      To (ABM##):      �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 16.- Source-copy command options

If you select the copy option, you are prompted for origin and destination

sources, as shown in figure 17.

�����������������������ͻ

�      Source Copy      �

� Copy................C �

� Xchange.............X �

� Quit................Q �

�    Choose (CXQ): C    �

�    From (ABM##): A    �

�      To (ABM##):      �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 17.- Copy option

You abort  the copy operation if you press enter at either prompt.  If you

select the Exchange option, you are provide similar prompts, as shown in figure

18.

�����������������������ͻ

�     Copy Sources      �

� Copy................C �

� Exchange............E �

� Quit................Q �

�    Choose (CXQ): C    �

� Src 1 (A,B,M,##): 32  �

� Src 2 (A,B,M,##): _   �

�                       �

�                       �

�                       �

�                       �

�����������������������ͼ

Figure 18.- Exchange option

You may  exchange or  copy memories  within the current-memory box, within

the reserve  memory box, or between both boxes.  When you exchange or copy into

the active group of memories, the transceiver memory is, of course, immediately

changed.

MEMORY FILE MANAGEMENT

The 40  memory channels  in the  TS-940S  provide  for  a  great  deal  of

flexibility for  most applications, but they may be insufficient for the varied

uses that a single transceiver may be put to.  They are probably sufficient for

most people's  use in  traffic-net participation,  DX monitoring,  SW broadcast

monitoring, CB  monitoring, and  the like, but they are likely insufficient for

combinations of these activities.  Reorganizing and altering 40 memory channels

is a tedious task that no one undertakes with relish.

To help  you with  memory management,  the Spoiler  lets you store TS-940S

memories in  computer disk  files.   With the  file-management command, invoked

with the  F6 key,  you can store all 40 memory channels to disk, restore all 40

channels from disk, and clear all 40 channels (setting them off).  In addition,

the Spoiler  provides you  with the  ability to  maintain a  file of 40 reserve

memories which  you may  exchange with  the active  memories with  a couple  of

keystrokes; you may also exchange and copy single memories from one file to the

other.

Figure 19 shows the memory-management command option box.

�����������������������ͻ

�     Memory Files      �

� List................L �

� Open................O �

� Save As.............S �

� New As..............N �

� Clear XCVR..........C �

� Quit................Q �

�  Choose (LOSNCQ):     �

�                       �

�    redo to show reserve

�                       �

�����������������������ͼ

Figure 19.- Memory management options

The LIST  option displays  the names of all TS-940S memory files stored on

disk (in the current drive and directory).  On disk, these files always have an

MRY extension;  the list,  however shows  the file names without the extension.

The list  is displayed  in the command-option box.  When the number of files on

disk exceeds  the number  of lines  in the  box, the display is paused for your

reading a  partial list until you press any key.  Figure 20 shows the beginning

and end of a list that is too long to fit into the option box.

�����������������������ͻ  �����������������������ͻ

�     List of Files     �  �     List of Files     �

�        CBRADIO        �  �        DEFAULT        �

�        TRAFFIC        �  �        SKEDS          �

�        CWNETS         �  � End of MRY file list  �

�        DXFONE         �  � Press a key . . .     �

�        SWL            �  �                       �

�        JACKS          �  �                       �

�        40CONTIN       �  �                       �

�        75CONTIN       �  �                       �

�        10SCAN         �  �                       �

� Press a key . . .     �  �                       �

�����������������������ͼ  �����������������������ͼ

Figure 20.- Initial and terminal displays of a file list

The OPEN  option reads  a file  from disk into both Spoiler memory and the

memories of the TS-940S.  The option box prompts you for the name of your file.

Note that  when you  initialize the  Spoiler for  your  TS-940S  it  reads  the

contents of  the transceiver  memories into  a file  called DEFAULT in order to

preserve the memories for you.  This prevents you from losing the data that you

already have  stored in  the transceiver when you download other files into the

transceiver.

Note:  Shareware examination  copies of  the Spoiler  will not open files other

than  the  one  opened  automatically  by  its  being  specified  during

installation.   Opening files  other than  the file  that  is  initially

specified is a feature reserved for registered copies of the Spoiler.

The file that is open is continuously updated by the Spoiler as you change

the memories  of your  transceiver.  New memory sources are automatically saved

as you  enter them.  Also, when you leave the Spoiler to run any other program,

you will  find when  you again  run the Spoiler that it automatically opens the

file you last opened before quitting.

The Reserve  option opens  a reserve-memory  file of  your choice.  The 40

additional memories  appear in  the high-definition screen and may be exchanged

either as  a group  or  individually  with  the  active  memories,  effectively

providing you with access to 80 memories for the TS-940S.

The Save  As option  writes the  currently selected  file to  disk under a

another name  and then  opens the  new file.   This  option provides you with a

convenient way  to edit  the contents  of an  old  file  without  automatically

storing the changes in the file.  The changes are stored only in the new file.

The New  As option reads all TS-940S memories into a disk file with a name

you provide,  and then  it opens  the file  for current use.  This is an option

that comes  in handy when you have changed the contents of transceiver memories

external to Spoiler operation.

Note:  The option  name "New"  was used  in version  1.01 of the 940 Spoiler to

initiate exactly  the same  function now  initiated  by  the  "Save  As"

option.   The revised  nomenclature is  more standard and therefore less

confusing.

The CLEAR  XCVR option lets you erase all of the memories in your TS-940S.

You may  want to  use this  option whenever you wish to begin a new memory file

from scratch  or you wish to maintain security about your operating frequencies

while other  operators are  using your transceiver.  The option requires you to

enter a  new file name; after the transceiver is cleared, new file by that name

containing zeroed  sources is  opened and  maintained  on  disk.    Of  course,

thereafter you may by using the OPEN command load your transceiver with sources

from any other file you have saved.

SCANNING

Scanning is  controlled directly  by the Spoiler and your PC.  It does not

rely on  the scanning  methods used  with the  TS-940S front  panel in order to

achieve more  versatility.   You may program scanning to step through groups of

memories you  select ("memory  scan"), through  memory-bounded frequency ranges

("programmed scan"),  or through  both groups  and ranges simultaneously.  Step

rate for  groups and  tuning rate  for ranges  are yours to choose, and you can

select downward or upward stepping and scanning.

The scanning  menu, shown  in figure 21, provides access to the individual

(groups or  ranges) or  combined (both) scanning methods.  It also provides for

editing the  group and  range data,  as well as for toggling memory lockout for

group scanning.

�����������������������ͻ

�         SCAN          �

� Groups..............G �

� Ranges..............R �

� Both................B �

� Mod groups..........M �

� Altr ranges.........A �

� Tog Lock............T �

� Quit................Q �

�  Choose (GRBMATQ):    �

�                       �

�                       �

�����������������������ͼ

Figure 21.- Scan command box

When group  scan is  selected, the  command  box  is  replaced  by  a  box

displaying group  information as  well as  instantaneous scan parameters.  Each

group is defined by a number (0 through 5), a group-lockout function (Locked or

Unlocked), a  beginning memory number, an ending memory number, and a delay (in

seconds).  Figure 22 shows the group-information box.

�����������������������ͻ

�      Scan Groups      �

�Grp  Begin   End    Dly�

� 0L   00     05     03 �

� 1L   06     06     02 �

� 2U   08     09     05 �

� 3L   22     26     02 �

� 4L   35     37     15 �

� 5L   20     10     03 �

�����������������������ĺ

�      Memory: 00f      �

�      3.905.06 LSB     �

�����������������������ͼ

Figure 22.- Group-information box

A similar  range-information box appears when range scan is selected.  The

parameters for  each scan  range are  identical to  those for  each scan  group

except for  the specification  of a  scan speed  rather than a scan delay.  The

scan-speed number range is 0 (slowest) through 99 (fastest).  Figure 23 shows a

range-information box.

�����������������������ͻ

�      Scan Ranges      �

�Rng  Begin   End    Spd�

� 0U   08     09     05 �

� 1L   09     08     50 �

� 2L   05     06     99 �

� 3L   35     38     00 �

� 4L   10     19     40 �

� 5L   25     26     70 �

�����������������������ĺ

�Scan range [08] to [09]�

�       3.933.05 LSB    �

�����������������������ͼ

Figure 23.- Range-information box

When combined  (Both) scanning  is selected, the group and range boxes are

displayed during the respective times that each method is in effect.

The invariable  scan sequence  begins with  scanning group  0, followed by

range 0, followed by group and range 1, and so on through group and range 5.

A group  or a  range will  not be  scanned if  it is  locked.   Individual

memories in  the TS-940S  may be locked out during Spoiler operation, (although

the  lockout  will  not  be  effective  during  subsequent  scanning  with  the

tranceiver's built-in basic scan capability).

Scanning may  be paused  at any  time with  the Escape  key.  Subsequently

pressing Escape  again ends  scanning and  brings back  up the  scan menu  with

transceiver parameters  unaltered.   Pressing Enter  while scanning  is  paused

causes the  current scan  parameter to  become the active source of transceiver

operation.   Pressing any  other key  after scanning  begins causes scanning to

resume at  the point  at which  it left off.  The command line provides prompts

for these operations during scanning.

The group  and range  parameters are  edited by  selection of  the  Modify

Groups and  Alter Ranges  commands in  the scan menu.  Selecting Modify Groups,

for example,  brings group  information into  the box  and prompts  you for the

group you want to edit information about, as shown in figure 24.

�����������������������ͻ

�      Edit Groups      �

�Grp  Begin   End    Dly�

� 0L   00     05     03 �

� 1L   06     06     02 �

� 2U   08     09     05 �

� 3L   22     26     02 �

� 4L   35     37     15 �

� 5L   17     18     03 �

�                       �

�       Group #:        �

�                       �

�����������������������ͼ

Figure 24.- Edit-groups selection box

Once you  have selected  the  group  number,  you  are  prompted  for  the

parameter in  that group  which you  want to  edit (lockout,  beginning memory,

ending memory,  and delay),  as shown  in figure  25.    You  are  subsequently

prompted for  the letter  or number  you want  to replace the current parameter

with.

�����������������������ͻ

�      Edit Groups      �

�Grp  Begin   End    Dly�

� 0L   00     05     03 �

� 1L   06     06     02 �

� 2U   08     09     05 �

� 3L   22     26     02 �

� 4L   35     37     15 �

� 5L   13     19     03 �

�                       �

�   Group 3 selected    �

� L)ck,B)gn,E)nd,D)ly:  �

�����������������������ͼ

Figure 25.- Edit-groups options

Editing ranges  involves an identical procedure, with the exception that a

speed value,  rather than  a delay value, may be replaced.  Figure 26 shows the

Edit-ranges selection box.

�����������������������ͻ

�      Edit Ranges      �

�Rng  Begin   End    Spd�

� 0U   08     09     01 �

� 1L   09     08     10 �

� 2L   05     06     30 �

� 3L   22     24     20 �

� 4L   16     16     15 �

� 5L   25     26     05 �

�                       �

�       Range #:        �

�                       �

�����������������������ͼ

Figure 26.- Edit-ranges selection box

Note:  In the  shareware examination  copies of the 940 Spoiler, changes to the

group and  range parameters  are effective only during a single session.

The parameters  are  saved  between  sessions,  however,  in  registered

copies.

For the  TS-440S only,  individual memories may be locked out and unlocked

from the scan menu through the use of the Toggle Lock option.  You are prompted

for a  memory number.  If the memory is unlocked, it will become locked out; if

it is  already locked  out, it will become locked. Individual memory lockout is

indicated in  the memory  windows of  both screens  by a dot between the memory

number and  the frequency.   Memory  lockout affects  only group  scanning.   A

locked out memory may still be used to specify the beginning frequency and mode

or the ending frequency of a range.

RUNNING ANOTHER PROGRAM

The F8 key provides you with a convenient way to run another program while

the 940  Spoiler remains  in memory  for instantaneous  use after  you exit the

other program.  The program that F8 will cause the Spoiler to "shell to" is the

one you've named during installation.

If you  have installed  the name "COMMAND.COM" (the default offered by the

installation program),  you will drop to the DOS command prompt, and from there

you may run any program that you have enough memory to run while the Spoiler is

resident.   After you  exit the  program run  from the  command prompt  and the

prompt once  again appears,  your entering  EXIT will return you immediately to

the Spoiler as you left it.

TOGGLING THE SCREENS

Function key  9 toggles  between the standard operating screen and the the

display high-density screen.  F9 is active except during tuning and transmit.

QUITTING THE SPOILER

Pressing F10  causes you  to exit  the Spoiler  program,  unload  it  from

memory, and  return to  DOS control of your computer.  Your TS-940S is restored

to normal  operation with  the memory-bank  switch  controlling  the  currently

selected group of 10 memories.

TOGGLING TRANSMIT

If you  have enabled  the feature  during installation, you may toggle the

TS-940S transmitter  on and  off by pressing the Page Down key.  If you use the

Page Down  key on  the numeric  keypad, be certain that the Num Lock key is not

toggled on.

Note:  Transmit toggling is unavailable in shareware examination copies of the

940 Spoiler.  It is a feature reserved for registered copies.

USING THE 940 SPOILER

There is  only one  task that you have to undertake to get the 940 Spoiler

working for  your radio-operating  pleasure.    You  must  connect  your  MSDOS

computer to  your TS-940S.   After your transceiver and computer are connected,

you are  at your leisure to learn how to use the Spoiler by trial and error, by

reading, or by a most-efficient combination of both activities.

COMPUTER/TRANSCEIVER CONNECTION

The easiest  way to  physically connect  your TS-940S transceiver and your

computer is  for you  to obtain an RS-232 level converter, an interface kit for

installation inside  the TS-940S,  and cables to connect your transceiver, your

level converter, and a serial port in your computer.  The sole practical source

for the internal interface kit (IC-10B) for the TS-940S is your Kenwood dealer.

Your dealer  also has  the IF-232C  level converter, which includes a cable for

connecting the  converter to  your transceiver  (but  not  a  standard  serial-

communications cable for connecting your computer to the converter).

Phil Whitehouse,  W1GEE, offers a much more economical alternative in both

kit and  assembled form  that includes an RS-232/TTL level converter and cables

for under  $50.   Phil's level  converter draws  power from  the transceiver in

order to  minimize wiring  clutter.   You may  contact Phil  at  the  following

address.

Mr. Phil Whithouse, W1GEE

P. O. Box 527

Hampden, Maine 04444

Larry Anderson,  KA6FQN, offers  a level  converter and  cables in a small

package in assembled form.  The level converter, cables, and instructions alone

are priced at $49.  You may contact Larry at the following address

Mr. Lawrence R. Anderson, KA6FQN

35615 Avenue D

Aucaipa, CA 92399

Optionally, you  might also  want to  obtain a  switch so that your RS-232

port for  devices other  than control  of your TS-940S at the flip of a switch.

Connectors and  people can  stand only  so much  plugging and unplugging, but a

simple switch solves both problems.

When you have installed the chip and the connector in your transceiver and

have the  interface box  and cables  in hand,  plug them  up.    Turn  on  your

transceiver and your computer, and run the Spoiler.

If something  is amiss  in the hardware (the transceiver, the cabling, the

interface box, or the computer), the Spoiler will tell you upon installation.

Caution: The 940  Spoiler is  an interrupt-driven  program that  maintains  the

highest-possible RS-232  transfer speeds  between your PC and your TS-

940S.   All four  of the  lines maintained  by the Kenwood transceiver

interface must  be maintained:  RD (receive data), TD (transmit data),

CTS (clear  to send),  and RTS  (ready to  send).  Failure to have the

last pair  of lines  connected MAY result in transmission of erroneous

data between your PC and the transceiver.

INSTALLING THE 940 SPOILER

You should copy 94SPL200.EXE and  onto a blank, formatted working diskette

or a  newly created,  empty directory on your hard disk.  Your directory should

have no  other files  but these  two.   This diskette  or directory must be the

currently selected one for you to properly install the 940 Spoiler.

The file  named   is an  installation program  that must be run before the

Spoiler program will itself run.  If you try to run 94SPL200.EXE before running

the installation  program, the  Spoiler will automatically run the installation

program before  it continues to run.   must be present in the current directory

with 94SPL200.EXE  for automatic  installation  to  succeed.    Otherwise,  the

Spoiler ships you back to DOS so that your can place  and  both in your current

directory.

The function  of the  940INS100.EXE is twofold.  It permits you to install

Spoiler features,  and it permits you to read the initial information from your

TS-940 memories  into a  disk file.  If you have any doubt about which of these

functions to use, use them both.  Install both features and memories.

Note:  Before you  begin any  installation or reinstallation, reinitialize your

TS-940S communications  interface by  turning the  transceiver off for a

few seconds prior to turning  it back on.

The Spoiler  will automatically  perform  the  installation  procedure  by

automatically running   whenever it does not find a file named 94SPL200.DEF and

a currently  specified memory file (*.MRY) in the current directory.  After you

install both  features and  memories,   passes control  back to 94SPL200.EXE so

that you can continue running the 940 Spoiler.

While 940INS20B.EXE  is running,  the program asks you whether you want to

perform feature  installation, memory-file  installation, or  both.  You should

not attempt  to install  a memory file until you have your TS-940S connected to

your computer.   The  Spoiler itself will not run until you have installed both

features and memories.

FEATURE INSTALLATION

The program  asks you  a  number  of  questions  about  hardware,  desired

features, and  your initial  memory file.   You  need to  provide number of the

serial port (1 or 2) that you have connected your TS-940S to.

It then  asks you  to  provide  a  name  for  the  file  that  will  store

information from  your initial  TS-940S memories.   If  you press  [ENTER], the

program selects the name "DEFAULT" for the file.

You need  to  provide  information  about  your  display  type  (color  or

monochrome), and  you need  to  provide  color  preferences  from  the  several

selections offered you.

If you  select the monochrome display adapter and black and white screens,

the Spoiler  will automatically  avoid attempting  to switch  pages of  adapter

memory.   This means  that screens will be painted within your view rather than

painted on  an alternate  page of adapter for instantaneous  switching into the

display.   Do not  install the  940 Spoiler for any colors other than black and

white if you are using a monochrome adapter.

Note:  Most LCD displays show "the reverse" of CRT displays.  A color choice of

"White on  Black," for  example, will provide an LCD display of black on

white.

You are  asked for  a highlighting  preference, and  you may select either

intensity highlighting  or reverse-video highlighting.  If you are using an LCD

display, intensity  highlighting may  have no  effect,  and  for  some  of  the

Spoiler's display features to be available to you must select the reverse-video

method of highlighting.

Your preferences  are summarized,  and you  are asked  whether you want to

save them,  redo them,  or exit the installation.  When you save them, the file

94SPL200.DEF is written to your current directory.

If you  have not chosen to perform a memory-file installation, the program

will terminate

MEMORY-FILE INSTALLATION

Memory-file installation  can be  performed all  by itself  or along  with

feature installation.  You are asked for a file name for storing TS-940S memory

information into  a disk  file.  The default is the name "DEFAULT," but you may

choose to use the same name you have already placed into your 94SPL200.DEF file

during feature installation, or you may choose to enter another file name.

You may, of course, once you have an initial memory-file installed, create

subsequent memory  files from within the Spoiler itself with the New As command

from the Memory File menu.

Note:  Shareware examination copies of the Spoiler permit opening only the file

name chosen  in the  installation program,  although other  files may be

created with  the Save  As and New As commands.  The full-feature option

to open  files so  saved (without going through the installation program

again) is  reserved to  registered copies  of the  Spoiler.  You may, of

course, at  any time  reinstall Spoiler  features so  that the  name  of

another file  of memories  is  the  one  that  the  Spoiler  works  from

(provided that  memories in your transceiver have not been altered since

you have issued a Save As or New As command when you run .

FAMILIARIZING YOURSELF WITH THE 940 SPOILER

You should  initially consider  the Spoiler  a  display  window  into  the

operation of  your TS-940S.   All operating parameters are displayed in the box

to the  upper-left of  your screen,  and all memory information is presented in

the J-shaped  (largest) box  to the  right and bottom of your screen.  When you

change any  operating parameter  (frequency, mode,  split, RIT, XIT, source, or

memory) from  the TS-940S  front panel,  the Spoiler  updates its  display  and

current memory  file immediately  to reflect  the change.   The  updates  occur

regardless of  whether the  change is  initiated from  the front  panel or  the

keyboard.

Later, you  should work  with the  Spoiler commands  that are initiated by

your pressing  the function  keys listed at the bottom of the Spoiler operating

screen.   Using the Spoiler, you'll find that you have little use for the front

panel of  your TS-940S  except for  such "analog" functions as your controlling

volume, passband  tuning, antenna  tuning (if  your transceiver  has an antenna

tuner installed), and the like.

The 940  Spoiler, combined  with the  operating capability of the TS-940S,

provides you  with a  quantum leap  in radio communications ability.  You'll be

able to  operate many traffic nets simultaneously, and you'll quickly find your

DX tallies  among only  those possessed  by other  operators who  are using the

Spoiler.   If you  are looking  for world  news  from  international  shortwave

broadcasts during  whatever international crisis the future brings, you'll find

it much  faster with  the Spoiler.   If  you're  looking  for  music  broadcast

throughout the world, you'll find your selection to be both wide and quick.  If

you want  to know  what the CB citizenry has further devolved into, the Spoiler

will help you find out with convenience that rewards your curiosity.

REGISTERING THE 940 SPOILER

As with  all shareware,  the 940  Spoiler is supported by those who obtain

registered copies  of the  software and  documentation.  The QSO2000 project is

not Microsoft  Corporation, and  nobody in  the project wants it to become more

than a cooperative endeavor among the avant garde communicators of the world so

that the world of amateur-radio communications can more rapidly become better.

Several hundreds  of hours  of  research,  trial-and-error,  and  software

development have  gone into  development of  the 940  Spoiler, and  before  the

useful life of the program is over several hundreds of additional hours will go

into its  maintenance and  enhancement.  We urge you to consider the value that

the 940  Spoiler represents  to you  and to appropriately register your copy of

the software  by your printing the form provided below and mailing it with your

remittance to the QSO2000 Project.

Your registration  of the  940 Spoiler  brings you  by return mail a full-

featured registered copy the latest version of the 940 Spoiler on diskette.  It

also entitles you to free upgrades of the 940 Spoiler software for a year after

your registration.  You will be notified of the upgrades by mail for the entire

year following your registration.

Registered copies  include your  name and call sign on the opening screen,

no shareware  notice with  its attendant  delay, no  closing-screen reminder of

unregistered-use limitations, and the following reserved features.

Keypad tuning acceleration and deceleration

Keypad control of transmit function

Automatic saving of edited scan paramaters

Memory-file opening while the Spoiler is running

Shareware examination  copies of  versions of  the  940  Spoiler  will  be

maintained and  enhanced throughout  the like of the package to a lesser extent

than registered  copies are.   Rewards for sharing in the resources expended by

further development of the package will be significant.

Please distribute  shareware examination  copies of  the latest version of

the 940  Spoiler to  other operators  as widely as you care to so that they are

invited to  share in  our progress.  Please include all of the files in the 940

Spoiler package.   If  you charge  a fee for distributing shareware examination

copies on  diskette, you  must charge  no more  then $3.00.   You  may not,  of

course, distribute  copies of  your registered  version of  the 940  Spoiler in

accordance  with   the  licensing   agreement  on  the  form  provided  in  the

accompanying file RGSTR940.TXT.

Licensees of  registered versions  of the Spoiler are welcome to write the

QSO2000 Project  with comments  and questions.   Licensees  are also welcome to

communicate directly  with the authors of the 940 Spoiler on the bulletin board

listed below.

Bulletin Board ECHO (Education)

806-741-1570

1200-2400 baud, 19200 baud USR-HST

For further information, please view or print the file RGSTR940.TXT.