�����������������������������������������������������������������������

*** VBBS MODEM DOCUMENTATION ***

�����������������������������������������������������������������������

VBBS comes preconfigured to work correctly with MOST modems.

If you are configuring for a 2400 baud modem in particular, it is

likely that the extent of your modem setup will be limited to chosing

the correct modem port in VCONFIG's channel.cfg.

Some modems, particularly some of the newer manufacture high

speed modems may require a bit more configuration and for that reason,

exclusive setup and initialization information is provided in this

document for many of the more popular high speed modems.

------------------

High Speed Modems:

------------------

Recent developments in the high speed modem market has found

the price of high speed modems dropping substantially over the past

several months. This is due primarily to a particular communications

chipset designed and manufactured by the Rockwell corporation. These

modems are "V.32bis" (14.4k baud) modems and most as well contain a FAX

facility as well. The price of these modems are currently ranging from $199

which is SUBSTANTIALY less expensive than high speed modems of lesser

speeds were priced only 1 year ago.

Many modem manufacturers offer SysOps of BBSs a special

discount off the User retail price of their modems. Information on current

SysOp deals on modems as well as extended help on setting up all

modems with VBBS can be found on the VirtualNET modem message base.

--------------------------------------------------------------------------

Additional Modem Set-Up assistance is available from any VBBS Support BBS.

--------------------------------------------------------------------------

VBBS and the Modem Initialization Setup

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

----------------------------------

The VCONFIG/Channel Configuration:

----------------------------------

VBBS presently allows up to 64 phone lines to be configured, even though in

most cases only the first 1 or 2 channels will be used by the average hobby

sysop.  Explainations will lean more towards this, for it is sufficient

enough if you were to connect more lines.

For each Data line in VBBS, a channel (phone line) configuration must be

set to define the Serial Com port that the individual line(s) will be

running from.  Below is a example layout.

1.) Com Port :

2.) Base Address (Hex):

3.) IRQ (Hex) :

4.) Baud Rate :

5.) Init String :

6.) Locked Baud Rate :

7.) Hardware (RTS/CTS) Handshake :

8.) Minimum Baud allowed :

1.) Com Port:

--------------

Define Commmunications port 1-8 per channel.  If by chance you use more

than eight different channels in the board, it is possible to use a Com

Port more than once, but MUST be addressed differently in the Base Address

field to prevent an IRQ conflict.  (See Section on IRQ's).

2.) Base Address:

------------------

This is were you define the Base address that DOS understands as the Port

assignment for the channel. (example: COM1 is 3F8, COM2 is 2F8).

3.) IRQ:

---------

Assignment for IRQ (Interupt Requests) that must be set for further

definition of the Com port channel.  (See Section on IRQ's for more

technical reference).

4.) Baud Rate:

---------------

Define your Modem's baud rate speed, from 300 to 57600.

Note: 300/1200 and 2400 baud modems must be set for thier Proper speed.

High Speed modems use the Locked Baud Rate that your High speed can handle,

for instance, if you have a 14.4 modem, you would define 14400, 19200,

38400 or 57600, whatever your modem allows, and also whatever works

accordingly to the type of computer equipment it is handling.

5.) Init String:

----------------

This is the Modem initialization string used by VBBS for the particular

channel to define the reset and operation of the channel with your Modem.

(see section on Modem NRAM's and INIT Strings for further definition).

6.) Locked Baud Rate:

---------------------

Toggle Yes/No for Locked Baud Rate on High Speeds.  This should be set to

no for 2400's and below.

7.) Hardware (RTS/CTS) Handshake:

----------------------------------

Three Options can be toggled for this, and is only used for High Speed

modems.  No, Yes(Normal), and Yes(Slow).  Most likely, you will want to set

this for Yes(Normal), because it behaves the best for High Speeds and error

control.

8.) Minimum baud rate allowed:

-------------------------------

This option is new to 5.50 and allows the sysop to configure what the

minimum speed for the channel/modem shall be.  This choice is beneficial to

sysops not wanting 300 or 1200 baud callers, or the sysop may set it for a

minimum baud of 9600 for a particular Channel if they just want thier High

Speed Modem Line available only to High Speed callers.

Serial Cards and Com Ports:

UART Chips and Recommendations:

What is the 16550 and how can it benefit communications?

--------------------------------------------------------

The chip, formally known as the NS16550AFN chip, is the heart of your

asynch board.  It is the UART chip, or Universal Asynchronous Receiver/

Transmitter.

The advantage of the 16550 over the older 16450 and 8250 UARTs is that

it has a 16 byte buffer on it.  Now, sixteen bytes doesn't sound like a

whole heck of a lot but it is SIXTEEN TIMES the size of the buffer on

the older UARTs which only had one a one byte buffer.

The advantages of the 16-byte buffer are two fold:

1) It makes high speed communications more reliable.

On the older chips, with their one-byte-buffer, you would lose data

if a second byte came in from the modem before the CPU had a chance

to pull the first byte out.  The 16550, with its 16-byte-buffer,

gives the CPU up to 16 chances to pull the data out before a

character is lost.

To realize what this means you can figure that at 19200 bps you are

expecting the CPU to service the comm port 1920 times each second or

once every .0005 seconds.  If the CPU happens to take .0006 seconds

to get around to servicing the comm port then in a one-byte buffer

UART that first byte is lost.  On the 16550 chip, with 16 bytes of

buffer space, you have up to .008 seconds to service the comm port.

2) It helps make a multitasking system more efficient.

When VBBS is transmitting data it has to stop the CPU and fill the

UART's transmitter buffer.  That means that if the caller in the

background is doing a directory scan his scan will take longer while

VBBS attempts to send data out to the first caller.

In the one-byte-buffer UARTs at 19200 bps VBBS must stop the CPU

1920 times each second just to send data out the comm port.  With

the 16550, however, it can stuff up to 16 bytes into the buffer at

a time and therefore interrupts the CPU only 120 times each second.

That drops the performance hit on the CPU, even at high speeds, to

almost that of a 1200 to 2400 bps modem!

To top it off... the older 8250 chips were buggy, making them less

reliable, and they were never designed for the high speeds that we

expect out of our modems today.  The NS16550AFN, on the other hand, is

designed with high bus speeds and high modem speeds in mind.  When

multitasking, even at slower baud rates, the 16550 can be very helpful

in providing smooth operation for the entire system.

--------------------------------------------------------------------------

Modem Initialization Settings:

Modem Setups and Init Strings:

For all practical purposes 300/1200/2400 Baud modems are probably the

easiest to setup and run, because they have no internal VCHIPS in them like

v.32, v42, etc.  and for VBBS should require only a workable Init String to

maintain operation.  Below are Init strings gathered over time from sysops

on the VirtualNET, and should function well depending on the brand that you

have.

Modem AT command settings

--------------------------------------------------------------------------

Standard 2400 bps or 1200 bps Hayes Compatibles (NON-MNP):

If your modem supports the extended "&" command set, then try:

ATX4Q0E0V1&C1&D2S2=251S7=40

X4     Enables extended result codes

Q0     Modem returns Result Codes

E0     Modem does not echo commands back to the computer

V1     Modem returns Verbal Result Codes

&C1    Modem asserts Carrier Detect based on connection status

&D2    On DTR high-to-low transition, Modem Hangs Up.

S2=251 Disable Modem Escape Sequence

S7=40  Time Modem waits for a carrier when initiating a new connection

If your modem does not support the & command set, try using:

ATX4Q0E0V1S2=251S7=40

Modems which do not use the & command set usually have DIP switches of some type

to set their mode of operation. Verify that your modem is set to:

a) Assert CD (carrier detect) only when conencted to a remote modem

b) Hang Up on drop of DTR (data terminal ready)

--------------------------------------------------------------------------

Exclusive High Speed Modem NRAM Setups.

Below is a collection of workable NRAM Setups, some with INIT strings that

have proven usable with VBBS for Channel assignments.

--------------------------------------------------------------------------

US Robotics Courier HST / Courier Dual Standard:

Since the HST utilizes so many settings, it is almost impossible to include

them all in one initialization string. Therefore, you may wish to use the

NRAM feature to store the settings which are common to most applications,

and use the VBBS init string for VBBS-specific settings.

NRAM Settings Recommended:

X7     Return full result codes

&H1    Turns on hardware handshaking for output from DTE (CTS line) *

&R2    Turns on hardware handshaking for input to DTE    (RTS line) *

&B1    Fixes the DTE to DCE rate **

&N0    Negotiates highest link rate with fallback/fallforward enabled

&M4    Enables ARQ if remote modem has ARQ

&C1    Modem asserts carrier detect based on connection status

&D2    On DTR high-to-low transition, modem hangs up

&A3    Give FULL EXTENDED Result Codes

(choose one of these 3 &K commands based on your preference)

&K1    Enables data compression if remote modem requests compression

&K3    Enables V.42 bis data compression but not MNP5

&K0    Disables all compression

ATI4

USRobotics Courier 14400 HST Dual Standard Settings...

B0  C1  E1  F1  M1  Q0  V1  X7

BAUD=38400  PARITY=N  WORDLEN=8

DIAL=HUNT   ON HOOK   TIMER

&A3  &B1  &C1  &D2  &G0  &H1  &I0  &K3  &L0

&M4  &N0  &P0  &R2  &S0  &T5  &X0  &Y1  %R0

S00=000  S01=000  S02=251  S03=013  S04=010

S05=008  S06=002  S07=045  S08=002  S09=006

S10=007  S11=050  S12=050  S13=000  S14=000

S15=008  S16=000  S17=000  S18=000  S19=000

S20=000  S21=010  S22=017  S23=019  S24=150

S25=000  S26=000  S27=000  S28=008  S29=020

S30=000  S31=000  S32=001  S33=000  S34=000

S35=000  S36=000  S37=000  S38=000

- Or use this -

ATI4

USRobotics Courier 14400 HST Dual Standard Settings...

B0  C1  E1  F1  M1  Q0  V1  X7

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=HUNT   ON HOOK   TIMER

&A3  &B0  &C1  &D2  &G0  &H1  &I0  &K3  &L0

&M4  &N0  &P0  &R2  &S0  &T5  &X0  &Y1  %R0

S00=000  S01=000  S02=251  S03=013  S04=010

S05=008  S06=002  S07=045  S08=002  S09=006

S10=007  S11=070  S12=050  S13=000  S14=001

S15=008  S16=000  S17=000  S18=000  S19=000

S20=000  S21=010  S22=017  S23=019  S24=150

S25=000  S26=000  S27=000  S28=008  S29=020

S30=000  S31=000  S32=001  S33=000  S34=000

S35=000  S36=000  S37=000  S38=000

and for your VBBS initialization string, use:

ATX7Q0E0V1S2=251S7=25&H1&R2&B1&M4&N0&K1&A3

- or, at least -

ATX7Q0E0V1S2=251S7=25&H1&R2&B1&A3

--------------------------------------------------------------------------

USR 9600 HST:

Use the init string above, stated for the USR/DS.

USRobotics Courier 9600 HST Settings....

C=1  E=1  F=1  M=3  Q=0  V=1  X=7  B=1

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=TONE   ON HOOK    TIMER

&A1  &B1  &G0  &H1  &I2  &K0

&M4  &N0  &P0  &R2  &S1  &Y1

S00=000  S01=000  S02=251  S03=013

S04=010  S05=008  S06=002  S07=025

S08=002  S09=006  S10=007  S11=040

S12=050  S13=000  S14=001  S15=000

S16=000  S17=000  S18=000  S19=005

S20=000  S21=010  S22=017  S23=019

NVRAM Settings

USRobotics Courier 9600 HST NRAM Settings....

DIAL=TONE   M=3  X=7  F=1  B=1

BAUD=19200  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H1  &I2  &K0

&M4  &N0  &P0  &R2  &S1  &Y1

S02=251  S03=013  S04=010  S05=008

S06=002  S07=025  S08=002  S09=006

S10=007  S11=040  S12=050  S13=000

S15=000  S19=005  S21=010  S22=017

S23=019

--------------------------------------------------------------------------

Compucom High Speed Modems:

AT&F2*H1%C0\N0S25=2M0QV1ES11=50&C1&D2H&W

Compucom STAR:

AT&F2*H2&D2S7=75&Y0&WX4M0L0S11=50

This may be too long to fit in the init string space provided by VBBS,

If so, just assign all these values to one of the NRAM settings, and init

string will actually read:

ATZ0

To assign an NRAM setting (2 are available, 0 and 1) type in the init string

above, in whole or in parts, and then type:

AT&W0

To disable the CCSP (Compucoms proprietary protocol) type

AT*H0

Baud rate should be locked and set to 19200, or 38400 for those with 16550

UARTs.

--------------------------------------------------------------------------

TwinCom V.32:

(No NRAM settings submitted)

ATE0H0M0Q0V1X4S0=0S2=1W0S95=145

--------------------------------------------------------------------------

Cardinal 9600:

(No NRAM settings submitted)

ATW2Q0V1E0&D2&C1S2=251M0&W2S95=1235

- or -

ATQ0V1E0&D2&C1S2=251S7=45M0

--------------------------------------------------------------------------

Supra/Fax 14.4:

NRAM settings can be run as default factory....

AT&F2  and then AT&W to write the set.

AT&K3\N3\T5E0H0M0Q0V1X4S0=0S2=1W2S95=3

--------------------------------------------------------------------------

Lightning Communications:

AT&FE0&D2&C1&Q9S7=65W2M0

Locked Baud Rate:  38400

RTS/CTS         :  Slow

(No Current Information provided on NRAM)

--------------------------------------------------------------------------

Zoom 14400 Fax/Modem:

Locked Baud Rate:  38400

RTS/CTS         :  Slow

AT&C1&D2E0H0Q0V1X4W0S95=3\T2

--------------------------------------------------------------------------

Zoom 2400 V.42bis:

ATX4Q0V1E0&D2&C1S2=251S7=45

ACTIVE PROFILE:

B1 E1 L2 M1 N1 P Q0 V1 W1 X4 Y0 &C0 &D0 &G0 &J0 &K3 &Q5 &R0 &S0 &T4 &X0 &Y0

S00:000 S01:000 S02:043 S03:013 S04:010 S05:008 S06:002 S07:045 S08:002 S09:006

S10:014 S11:095 S12:050 S18:000 S25:005 S26:001 S36:005 S37:000 S38:020 S44:003

S46:138 S48:000 S49:008 S50:255

STORED PROFILE 0:

B1 E1 L2 M1 N1 P Q0 V1 W0 X4 Y0 &C0 &D0 &G0 &J0 &K3 &Q5 &R0 &S0 &T4 &X0

S00:000 S02:043 S06:002 S07:030 S08:002 S09:006 S10:014 S11:095 S12:050 S18:000

S25:005 S26:001 S36:005 S37:000 S38:020 S44:003 S46:138 S48:007 S49:008 S50:255

STORED PROFILE 1:

B1 E1 L2 M1 N1 P Q0 V1 W0 X4 Y0 &C0 &D0 &G0 &J0 &K3 &Q5 &R0 &S0 &T4 &X0

S00:000 S02:043 S06:002 S07:030 S08:002 S09:006 S10:014 S11:095 S12:050 S18:000

S25:005 S26:001 S36:005 S37:000 S38:020 S44:003 S46:138 S48:007 S49:008 S50:255

--------------------------------------------------------------------------

ZyXel High Speed:

this is saved as Profile 0.  Init string is just ATZ0.

Be aware that register S20 sets the DTE speed, set to 19200, so you

may want to change this to S20=2 or S20=1.

Current   Setting.............

B0   E0   L4   M0   N5   Q0   V1   X6

&B1  &C1  &D2  &G0  &H3  &J0  &K4  &L0  &M0  &N0  &P0  &R1  &S0  &X0  &Y0

*B0  *C0  *D0  *E0  *I0  *M0  *P9  *Q2  *S0

S00=000   S01=000   S02=251   S03=013   S04=010

S05=008   S06=003   S07=060   S08=002   S09=006

S10=007   S11=070   S12=000   S13=000   S14=002

S15=002   S16=000   S17=018   S18=000   S19=000

S20=003   S21=176   S22=000   S23=112   S24=133

S25=000   S26=000   S27=156   S28=064   S29=000

S30=000   S31=017   S32=019   S33=000   S34=030

S35=000   S36=000   S37=000   S38=016   S39=000

ZyXEL 14400 V.32 V.32bis V.42 V.42bis Send/Recieve Fax/Modem:

AT&FX6E0&C1&D2&G2H0M0|

Locked Baud: 19200

harware handshaking normal

---------------------------------------------------------------------------

DESCRIPTION OF COMMUNICATION STANDARDS.  INCLUDES MODEM & FAX.

---------------------------------------------------------------------------

STANDARD NAME    DESCRIPTION

---------------------------------------------------------------------------

CCITT v.21       Standard for 300 bps.  (FSK).

Bell  103J

Bell  202        Old standard for 1200 bps, half duplex. Leased lines.

CCITT v.22       Standard for 1200 bps. (DPSK).

Bell  212A

CCITT v.22bis    Standard for 2400 bps. (DPSK).

CCITT v.26ter    Standard for 2400 bps with echo cancellation.

Bell  208        Standard for 4800 bps. Used on leased lines.

CCITT v.27ter    Standard for G3 FAX at 4800 bps with fall back to 2400 bps.

CCITT v.29       Standard for G3 FAX at 9600 bps with fall back to 7200 and

4800 bps.  Also v.27ter/v.29 is used by CompuCom modems.

Half duplex.  Reverse direction bps (slow?).

CCITT v.32       Standard for 9600 bps with echo cancellation.  Fall back

to 4800 bps.  Both direction simultaneously.  (QAM).

CCITT v.32bis    Standard for 14400 bps with echo cancellation.  Fall back

to 12000, 9600, and 7200 bps.  (TCM).  Both directions

simultaneously.

CCITT v.33       14400 with fall back to 12000 bps.  Used for what?

USR HST14400     US Robotics HST modulation.  14400 bps in one direction

and 450 bps in reverse direction.  Fall back to 12000,

9600, and 4800 bps.

Trailblazer      Proprietary and fast!  Up to 18000 bps.  ???

Hays V-Series    Proprietary full-duplex 9600 baud.  Hays its own CCITT LAP-B

error correction and its own scheme for data compression.

EIA Class2 FAX   Fax standard called G2.  ???

CCITT v.FAST     Standard for 19200 bps or slightly higher.  Uses echo

CCITT v.32bisbis canellation and is not FINALIZED.  The first modems will be

available in late 1993.  The name of the standard has not

been determined yet.  It will have a much greater capability

to adapt modem speeds to meet existing conditions (more than

any existing modem).  Theory sets a limit in the low 20000

bps range.  This will probably be the LAST analog modem

standard.

ISDN             Integrated Services Digital Network.  This uses fiber optic

cable.  It is currently available in larger cities within

3 miles (has been lengthen) of the central office. Available

in two flavors: Primary Rate Interface service and Basic

Rate Interface service.  Basic, also known as 2B+D provides

users with two 64Kbit/sec channels for voice and data and

one 16Kbit/sec channel used by the carrier for call setup

and network management.  The primary, also known as 23B+D

provides 23 64Kbit/sec channels and one 16Kbit/sec channel.

Some recent west coast subscribers will be paying a one

time $75 fee for a network connection device and $97 per

month for service, which includes two voice lines.

SMDS             Switched Multi-Megabit Data Service.  Uses fiber optics.

???

HDSL             High-rate Digital Subscriber Link.  Bell Labs and Bellcore

have recently developed this technology.  It is capable of

carrying 1.5 megabits/sec over copper phone lines!  Limited

to about 3 miles from the central office.  Benificial since

it provides high speed data rate without running new fiber

optic cable to the customer.  This is still in the labs.

---------------------------------------------------------------------------

MODE          BPS       BAUD      MODULATION      NOTES

---------------------------------------------------------------------------

v.33         14400      2400       128-TCM

v.33         12000      2400        64-TCM

v.32bisbis   19200      2400       512-TCM        TBD ???

v.32bis      14400      2400       128-TCM

v.32bis      12000      2400        64-TCM

v.32bis       9600      2400        32-TCM

v.32bis       7200      2400        16-TCM

v.32          9600      2400        32-TCM

v.32          9600      2400        16-QAM        Uncoded

v.32          4800      2400         4-DPSK

v.29          9600      2400        16-QAM

v.29          7200      2400         8-QAM

v.29          4800      2400         4-DPSK

v.27ter       4800      1600         8-PSK

v.27ter       2400      1200         4-DPSK

G3 FAX         ->        ->           ->          According to v.29/v.27ter

v.22bis       2400       600        16-QAM

v.22          1200       600         4-DPSK       Same as BELL 212A

ISDN         65536     65536       FiberOptic     Digital

The baud rate over the phone line means that more than two tones are used at

the baud frequency to achieve the bps rate.  4-DPSK means four tones...

---------------------------------------------------------------------------

NAME      DESCRIPTION OF ERROR CORRECTION AND DATA COMPRESSION

---------------------------------------------------------------------------

MNP1      Hardware error correction.

MNP2      Hardware error correction.

MNP3      Hardware error correction.  Strips off start and stop bits during

transmission over phone line.

MNP4      Hardware error correction.  Fastest way to send compressed files.

Strips off start and stop bits during transmission.  Also adapts

to line conditions.

MNP5      Hardware error correction and data compression (up to 2 time

compression on TEXT).  Do NOT use on compressed files!

MNP6      Additional Universal Link Negotiation and Statistical Duplexing

to MNP5 service

MNP7      Additional Enhanced Data Compression with MNP4.

MNP8      Nothing available ???

MNP9      Additional Enhanced Data Compression with v.32 modems  Also adds

Enhanced Universal Link Negotiation, which allows connection to

both MNP and non-MNP modems at the highest performance level.

MNP10     Hardware error correction and data compression.  Special!

v.42      CCITT error correction. LAPM.  Fastest way to send compressed

files.  Strips off start and stop bits during transmission.

v.42bis   CCITT data compression (up to 4 time compression on TEXT).

Uses a version of the Lempel-Ziv data compression algorithm.

---------------------------------------------------------------------------

baud  means Actual rate of symbols trasmitted per second.

bps   means Bits Per Second.

MNP   means Microcom Networking Protocol.

CCITT means International Telephone and Telegraph Consultative Committee.

FSK   means Frequency Shift Keying.

DPSK  means Phase Shift Keying.

QAM   means Quadrature Amplitude Modulation.  Combination of PSK and AM.

TCM   means Trellis Coded Modulation.

LAPM  means Link Access Procedure for Modems.

--------------------------------------------------------------------------