VFOSSIL.DOC

Using a Fossil Driver with VBBS

The fossil driver can open up a whole new world to VBBS.

The "standard" serial driver which comes with VBBS is

not able to allow IRQ sharing between COM ports.

This is not so much a limitation in the serial driver, as it

is a limitation of the underlying operating environment.

A FOSSIL driver, on the other hand, can handle 2 or more

serial ports using the same IRQ line. In fact, a FOSSIL

can be used to control up to 8 ports. It can be "programmed"

to use almost any serial board in existance.

For the purposes of this document, we shall refer to the

X00 and BNU fossil drivers. If you're new to fossil drivers,

they are a "standard" way for programs to communicate with

serial ports, and you only need one installed on your system

in order to make use of it.

X00 is one type of fossil driver that loads thru CONFIG.SYS.

BNU is a another type of fossil driver that loads thru AUTOEXEC.BAT.

Both programs do roughyl the same thing, and yu have your choice of

which one you wish to use.

** Number of simultaneous open ports **

***************************************

The "basic" CONFIG.SYS install line for X00 looks like:

DEVICE=X00.SYS

The "basic" AUTOEXEC.BAT install line for BNU is:

BNU

By default, a fossil will assume you wish to have only

ONE COM port open at any given moment.

This doesnt specify WHICH port (COM1, COM2, COM3, etc),

but specifies the maximum number of simultaneous open

ports.

Each Port you request is like a "data channel," corresponding

to your VBBS data channels.

It is possible to request that the fossil make more ports

available to you.

For more detailed information on the driver, consult

the fossil documentation.

* VBBS Set Up *

***************

Set up your VBBS Data Channels as you normally would.

***********************************************************

VBBS - 2 user system example

----------------------------

Ch 1 to be on COM1

Ch 2 to be on COM2

------------------

X00:

DEVICE=X00.SYS 0=COM1 1=COM2

BNU:

BNU /P=2

---------------------------------------

VCONFIG: Set Channel 1 to COM1

Set Channel 2 to COM2

***********************************************************

VBBS - 2 user system example

----------------------------

Ch 1 to be on COM2

Ch 2 to be on COM3

------------------

X00:

DEVICE=X00.SYS 0=COM2 1=COM3

BNU:

BNU /P=2

---------------------------------------

VCONFIG: Set Channel 1 to COM2

Set Channel 2 to COM3

***********************************************************

VBBS - 4 user system example

----------------------------

Ch 1 to be on COM1

Ch 2 to be on COM2

Ch 3 to be on COM3

Ch 4 to be on COM4

------------------

X00:

DEVICE=X00.SYS 0=COM1 1=COM2 2=COM3 3=COM4

BNU:

BNU /P=4

---------------------------------------

VCONFIG: Set Channel 1 to COM1

Set Channel 2 to COM2

Set Channel 3 to COM3

Set Channel 4 to COM4

***********************************************************