Printer Driver Editor

If your printer is not directly supported by FormGen, you may set up a .DRV

file for your printer using a pair of utilities included on your FormGen

diskettes.

The DRV program will load and allow you to edit the control sequences sent to

the printer during the printing process.

The DRVPRINT program will print a complete list of the .DRV file contents to

allow you to check the sequences you've entered or as reference when you're

working on a file.

Using the DRV Editor

The DRV Editor is started by typing:

DRV filename

where filename is the name of an existing .DRV file

ie.

DRV HP (to edit the driver file HP.DRV)

The editor will load the .DRV file and present the printer contoll sequences

in format similiar to the one shown below:

>>> FormGen .DRV Editor <<<

------------------------------------------------------------------------------

Printer description: Epson FX series                  Driver #  4

------------------------------------------------------------------------------

Font Description         Sp Start Sequence            End Sequence

Double Condensed Pica    50 ^O^[G                     ^R^[H

Double Elite             50 ^[M^[G                    ^[P^[H

Proportional Pica        50 ^[p1                      ^[p0

Emphasized Pica          50 ^[E                       ^[F

Double Enlarged Elite    50 ^[M^[^N^[G                ^[P^[T^[H

Empasized Enlarged Pica  50 ^[E^[^N                   ^[F^[T

------------------------------------------------------------------------------

Fillin text       ^[E                      |end       ^[F

Line Graphics     ^@                       |end       ^@

Floating Text     ^[S^@^[j^T               |end       ^[T^[3^T^J

Printer setup     ^[@                      |Reset     ^[@

Landscape seq     ^@                       |Portrait  ^@

Form Feed         ^L                       |Transpare ^@

Row Prefix        ^@                       |suffix    ^@

Dot Col Prefix    ^@                       |suffix    ^@

# Copies prefix   ^@                       |suffix    ^@

Rel Dot Row Prefi ^@                       |suffix    ^@

Foreign Translation Type  3                 Line Draw Type  1

------------------------------------------------------------------------------

Each of the shown fields has a specific purpose and may be tailored to your

precise requirements.  You simply press [Enter] after reviewing (and

modifying, if necessary) each field to move to the next.  Once each field has

been reviewed, the editor saves the updated file and exits to DOS.

When setting up ypur own printer driver, it is good idea to make a copy of

the closest possible driver and then make the modifications to the copy.

Printer Description

This is the description of the printer that will be displayed on the printer

configuration list in the FormGen Configuration Module.  Make sure you

provide enough detail here to differentiate it from other drivers for the

user to properly select it.

Driver #

This selectes the basic printer type and determines which of the internal

print driver subsystems FormGen will use to control the printer.  The driver

types are:

1-300 dpi PCL laser printers

for 300 dpi laser which user the HP PCL command set and will be

using internal or cartridge-based fonts

use this option for HP LaseJet + and compatible printers (for

the original LaserJet, see option 13)

2-300 dpi Xerox laser printers

for Xerox 4045 or Xerox 2700 type laser printers

3-line printers with IBM extended set

for daisywheel or dotmatrix printers which can directly print

the IBM extended character set

use this option for printers like the IBM Proprinter or the

Diablo 630 ECS

4-dot graphics and n/72" line spacing

for dot matrix printers that do not directly support the IBM

extended character set and set spacing in 72nds of an inch

this is for printers that recognize ESC L as the beginning of a

8 bit dot graphics command and on which ESC A directly sets

n/72" line spacing (ESC 2 is NOT required)

use this option for Epson MX/RX/FX compatibles

5-dot graphics and n/180" line spacing

for dot matrix printers that do not directly support the IBM

extended character set and set spacing in 180ths of an inch

this is for printers that recognize ESC L as the beginning of a

8 bit dot graphics command and on which ESC 3 sets n/180" line

spacing

use this option for Epson LQ/SQ compatibles

6-dot graphics and n/216" line spacing

for dot matrix printers that do not directly support the IBM

extended character set and set spacing in 216ths of an inch

this is for printers that recognize ESC L as the beginning of a

8 bit dot graphics command and on which ESC 3 sets n/216" line

spacing

7-Tandy dot printers (native mode text)

this driver uses the IBM code ESC L to print lines and graphics

and the Tandy native mode for printing text, giving access to

such items as color printing

can be used only with Tandy printers that can switch between

Tandy and IBM modes using ESC !

8-Tandy dot printers (IBM emulation mode)

this driver is used with Tandy printers that are set to IBM mode

9-HP DeskJet

for the Hewlett-Packard DeskJet which doesn't accept the PCL

command for mutiple copies, but is compatible in other respects

11-Dot Matrix (6 lpi)

for dot matrix printers which directly support the use of the

IBM character set and will print the characters at their full

height allowing the printer to use standard 6 lpi spacing

13-Original HP LaserJet

for the original HP laser which has a limited command set and

MUST have a cartridge installed with a line draw character set

Font Description

The description of the fonts which will be displayed at the time a text

window is opened.

Sp (line spacing)

The distance between lines that should be used when printing window text (in

300th of an inch measured form baseline to baseline).  This is usually the

same as the height of the font.

Start Sequence

The control sequence which will activate the font. (see section Control Codes

for more info)

End Sequence

The control sequence which will deactivate any special attributes turned on

in the start sequence.  (not necessary for laser printers where complete

fonts are selected for each operation)

Fillin text

Selects the font to use when printing data entered in FormFill.  This should

be set to a 10cpi font.

Line Graphics

Selects the font to use when printing line characters.  This should be set to

a 10cpi font.

Floating Text

These sequences are sent just before and just after floating text is printed.

Ith sequence should raise the text in superscript or reverse the page a

fraction of a line and then restore it.

Printer Setup

This sequence is sent to the printer at the beginning of a print job and

should set it for 6lpi, 10cpi printing.

Reset

This sequence is sent to the printer at the end of a print job and allows you

to leave the printer in a known state.

Landscape

This sequence sets the printer to landscape mode. (laser printers only)

Portrait

This sequence sets the printer to portrait mode. (laser printers only)

Form Feed

Sequence to eject a page from the printer.

Transparent

Sequence to inform the printer that the next character is a control-type

character, but its ASCII representation is to be printed.

Row Prefix/Suffix

Start and end of sequence for direct row addressing.  FormGen inserts an

ASCII numeric value between these.  (laser printers only)

Dot Col Prefix

Start and end of sequence for direct dot column addressing.  FormGen inserts

an ASCII numeric value between these.  (laser printers only)

# Copies prefix

Start and end of sequence to set number of copies to print.  FormGen inserts

an ASCII numeric value between these.  (laser printers only)

Rel Dot Row Prefi

Start and end of sequence for relative dot row addressing.  FormGen inserts

an ASCII numeric value between these.  (laser printers only)

Foreign Translation Type

This parameter allows you to determine how foreign characters will be

produced on the printer.

0-no foreign character support

for printers that do not support foreign characters

1-no translation required

for printers that directly support foreign characters using the

IBM extended character set

2-HP Roman 8 Extention

for printers using the HP Roman 8 symbol set mapping for foreign

characters

3-Epson Foreign Sets

for Epson type printers that allow selective remapping of groups

of foreign characters into the regualar ASCII set

4-Xerox ISO

for printers that allow the addition of accents to characters

using the Xerox ISO set

5-use table in driver

activates the character remapping facility in FormGen

6-overprint build

FormGen assembles representatyions of foreign characters by

adding accents to regular ASCII characters

ie.

() e accent acute is printed by printing e, backing up one

space and then printing '

for fixed fonts only - not likely to produce desirable results

with proportional fonts

7-use transparent sequence for 128 - 159

for printers that have foriegn characters mapped into ASCII

locations 128 to 159 but usually interpret them as control codes

Line Draw Type

0-directly supported

for printers that directly support the line drawing cahracters

in the IBM extended character set

1-dot graphics

for printers that must use dot graphics to print lines

2-HP Line Draw set

for older laser printers that do not support the line drawing

characters in the IBM extended character set and have no facilty

for FormGen to downloaded a line drawing font

this setting will allow the use of a font cartridge with the HP

line draw set

3-daisy wheel

uses the | and - characters to draw lines on a daisywheel

printer

4-Xerox forms font set

uses the line characters in the Xerox forms font

5-direct addressing

sends specific row and column addresses ahead of each line

character in order to prevent line alignment problems on certain

printers

Table Maintenance

A number of data tables are contained in the .DRV file.  These may be

modified to suit the particular printer as required.

Foreign Character Translation Table

The foreign character translation table allows you to remap the character set

in the PC to suit the character set in your printer.  Each entry from 128 to

175 is displayed on the screen.  You may enter the appropriate ASCII value

for the matching character in your printer.  This table must be activated by

setting the Foreign Translation Option to #5.

Font Width Tables

There are a set of six font tables in each .DRV file - one for each font

supported.  These tables contain the width (in 300ths of an inch) for each

character in each font.

If the corresponding font is not proportional, a single value (in 300ths of

an inch) may be entered here and it will be automatically replicated through

the entire table.  If the font is proportional, enter a zero.  The editor

will then prompt for the desired unit of measure.  If your manual describes

the character widths in 180ths of an inch enter 180 here and it will be used

as a factor to convert all your entries to 300ths.

Control Codes

Control sequences are entered by typing them into the various field areas.

Control codes and the escape character are represented using a common

shorthand method.

null char (or ASCII 000) is entered as ^@

control A (or ASCII 001) is entered as ^A

control B (or ASCII 002) is entered as ^B

.

.

.

control Z (or ASCII 026) is entered as ^Z

escape    (or ASCII 027) is entered as ^[

ie.

You want to set up font 6 for double wide, emphasized printing on your dot

matrix printer.  Your printer manual tells you that double width printing is

turned on using ESC W 1 and off with ESC W 0.  It also tells you that then

emphasized print mode is activated using ESC E and deactivated with ESC F.

First, you type the description under the heading "Font Description":

Empasized Double Width

Second, you set the line spacing:

50   (has no effect on a dot matrix printer)

Third, you enter the codes to turn on the double width and the emphasized

print:

^[W1^[E  (escape, W, 1, escape, E)

Finally, you enter the codes to turn off the emphasized and double print

^[F^[W0  (escape, F, escape, W, 0)

The line in the editor will look like:

Font Description         Sp Start Sequence            End Sequence

.

.

Empasized Double Width   50 ^[W1^[E                   ^[F^[W0

Editing

You may edit you entries using the [Ins], [Del], and [Ctrl][End] keys

[Ins] - inserts a blank at the cursor position

[Del] - deletes the character at the cursor position

[Ctrl][End] - clears the current entry

[Enter] - saves the definition and moves to the next field

[Esc] - abandons any changes and move to the next field