Date: 03/31/92

Status: Beta 2.0.01

Revised README.DOC file (04/07/92)

Revised README.DOC file (04/08/92)

Revised README.DOC file (04/14/92)

Revised README.DOC file (04/16/92)

[Usenet/Internet Distribution:

Revised May 2, 1992, by Timothy F. Sipples, netmail

sip1@ellis.uchicago.edu.]

*Last minute change:

WINOS2 drivers are not included.  They can be downloaded

from Trident BBS.  These drivers are regular Windows 3.0

VGA.DRV drivers.  If you already have these drivers, you

may not want to download them again.

[Usenet/Internet Distribution:

Suitable Win-OS/2 drivers are included (equivalent to Windows 3.0 drivers).

Note that these Win-OS/2 drivers work only with 8900C/9000 chipset Trident

adapters.  For suitable drivers for other chipsets use those supplied with

the adapter.]

Trident

TVGA 8900/9000 PM Display Driver Package

for OS/2 Version 2.0

IMPORTANT: Before installing any of the programs in this package, make

---------  sure that you read through this document once.

Download Info

-------------

Instead of downloading the whole package listed below, we

recommend that you choose the one or two drivers that fits your current

needs and system configuration.

After downloading any of the files in this package, please

use PKUNZIP utility to unpack it.  All the files listed below are

available from Trident BBS:

W800.ZIP        - 800x600-16 Seamless DISPLAY.DLL

and SWINVGA.DRV (W800SL.DRV)

W1024.ZIP       - 1024x768-16 DISPLAY.DLL

T640.ZIP        - 640x480-256 DISPLAY.DLL

T768.ZIP        - 768x1024-256 DISPLAY.DLL (portrait)

T800.ZIP        - 800x600-256 DISPLAY.DLL

T1024.ZIP       - 1024x768-256 DISPLAY.DLL

T8514.ZIP       - 1024x768-256 DISPLAY.DLL with 8514 type

fonts and icons

T640V512.ZIP    - 640x480-256 Virtual Desktop DISPLAY.DLL

(1024x512-256 for 512 KB memory)

T640V.ZIP       - 640x480-256 Virtual Desktop DISPLAY.DLL

(1024x1024-256 for 1MB memory)

T800V.ZIP       - 800x600-256 Virtual Desktop DISPLAY.DLL

(1024x1024-256 for 1MB memory)

[Usenet/Internet Distribution:

These files, and the Win-OS/2 drivers, have been bundled in a slightly

different fashion.]

The above files may also be available from the following

BBSes or Internet FTP archives (maybe different file names):

IBM NSC BBS:    (404) 835-6600

Fernwood:       (203) 483-0348

OS/2 Shareware: (703) 385-4325

CompuServe      IBMOS2 forum

hobbes.nmsu.edu                 (anonymous FTP)

novell.com (not day time)       (anonymous FTP)

mtsg.ubc.ca                     (anonymous FTP)

archive.latrobe.edu.au          (anonymous FTP)

usenet news system: comp.binaries.os2

Newer releases of OS/2 drivers will be posted on Trident

BBS later.  Please check the date and status of the driver files.

Descriptions:

-------------

( The following descriptions assume that OS/2 2.0 is installed on

the target system, drive C:. )

The file W800.DLL is a customized PM display driver which supports

the 800x600-16 color mode PM Desktop session.  This file (if 800x600 16

color mode is chosen as the desired display mode) will replace the default

DISPLAY.DLL (640x480-16 color standard VGA mode) file in directory

C:\OS2\DLL.  This driver is fully compatible with 2.0 display driver spec

and is capable of running seamless Windows applications.  To run seamless

Windows apps on 800x600-16 workplace shell, you need also install

W800SL.DRV.

The file W1024.DLL is a customized PM display driver which supports

the 1024x768-16 color mode PM workplace shell.  This file will replace the

default DISPLAY.DLL file in directory C:\OS2\DLL.

The file T640.DLL is a customized PM display driver which supports

the 640x480-256 color mode PM workplace shell.  This file will replace the

default DISPLAY.DLL file in directory C:\OS2\DLL.

The file T768.DLL is a customized PM display driver which supports

the 768x1024-256 (portrait mode) color mode PM workplace shell.  This file

will replace the default DISPLAY.DLL file in directory C:\OS2\DLL.

The file T800.DLL is a customized PM display driver which supports

the 800x600-256 color mode PM workplace shell.  This file will replace the

default DISPLAY.DLL file in directory C:\OS2\DLL.

The file T1024.DLL is a customized PM display driver which supports

the 1024x768-256 color mode PM workplace shell.  This file will replace the

default DISPLAY.DLL file in directory C:\OS2\DLL.

The file T8514.DLL is similar to T1024.DLL, which supports the

1024x768-256 color mode PM workplace shell.  However, it contains 8514

compatible type of fonts and icons.  This file will replace the default

DISPLAY.DLL file in directory C:\OS2\DLL.

The file T640V512.DLL is a Virtual Desktop PM display driver which

supports 1024x512-256 color PM workplace shell through a physical 640x480

256 color display screen.  This file will replace the default DISPLAY.DLL

file in directory C:\OS2\DLL.  (For more info on Virtual Desktop, see

Appendices below.)

The file T640V.DLL is a Virtual Desktop PM display driver which

supports 1024x1024-256 color PM workplace shell through a physical 640x480

256 color display screen.  This file will replace the default DISPLAY.DLL

file in directory C:\OS2\DLL. (For more info on Virtual Desktop, see

Appendices below.)

The file T800V.DLL is a Virtual Desktop PM display driver which

supports 1024x1024-256 color PM workplace shell through a physical 800x600

256 color display screen.  This file will replace the default DISPLAY.DLL

file in directory C:\OS2\DLL. (For more info on Virtual Desktop, see

Appendices below.)

The file W800SL.DRV (zipped in W800.ZIP) is a customized WINOS2

display driver which supports Windows applications running on PM workpalce

shell seamlessly.  The PM workplace shell should run in 800x600-16 mode

(using W800.DLL as DISPLAY.DLL).  This file will replace the default

SWINVGA.DRV in directory C:\OS2\DLL.

BVH driver support, which is a nice feature with our 1.x drivers,

is no longer included in this package since OS/2 2.0 has build-in Trident

SuperVGA support.

PMFIX for 1.x should not be executed under 2.0.

OS2CRTC, which is used to adjust video parameters for 1.2/1.3

OS/2 drivers is not available at this time.  However, there is an

alternative.  See descriptions in "Appendices" below.

Installation:

-------------

[Usenet/Internet Distribution:

Make sure that IOPL=YES in CONFIG.SYS.]

0. Install OS/2 2.0 GA version with TVGA card on system

Assume the startup drive is C:

1. Make a backup copy of the following OS/2 drivers:

C:\OS2\DLL\DISPLAY.DLL

(PM display driver)

C:\OS2\MDOS\WINOS2\SYSTEM\VGA.DRV

(WINOS2 full screen driver)

C:\OS2\MDOS\WINOS2\SYSTEM\SWINVGA.DRV

(WINOS2 seamless driver)

We recommend that the backup be put on a floppy

2. Unzip the drivers to be installed

3. Go to a DOS Full Screen session, run system command

SVGA ON

A file called SVGADATA.PMI will be generated in C:\OS2

Save this file on a floppy disk

4. Switch to OS/2 protected mode session and start

DDINSTAL

5. Follow the instructions to finish the rest of the procedures

6. You need to shut down and re-boot the system before the

driver takes into effect

7. Before installing T768.DLL, T640V512.DLL, T640V.DLL T800V.DLL,

read the special discussions below

Appendices:

----------

1.  Files in the package:

README   DOC    -- Read this first

W800     DLL    -- 800x600-16 color PM driver

W1024    DLL    -- 1024x768-16 color PM driver

T640     DLL    -- 640x480-256 color PM driver

T768     DLL    -- 768x1024-256 color PM driver

T800     DLL    -- 800x600-256 color PM driver

T1024    DLL    -- 1024x768-256 color PM driver

T8514    DLL    -- 1024x768-256 color PM driver

with 8514 compatible resources

T640V512 DLL    -- 640x480-256 color PM driver

with 1024x512 virtual desktop

T640V    DLL    -- 640x480-256 color PM driver

with 1024x1024 virtual desktop

T800V    DLL    -- 800x600-256 color PM driver

with 1024x1024 virtual desktop

W800SL   DRV    -- 800x600-16 seamless WINOS2 driver

W800     DDP    -- Device Driver Profile (for DDINSTAL)

W1024    DDP

T640     DDP

T768     DDP

T800     DDP

T1024    DDP

T8514    DDP

T640V512 DDP

T640V    DDP

T800V    DDP

2.  On board video memory requirements (8900/9000)

driver        resolution      min. mem (KB)   chipset

W800.DLL      800x600 -16          256        8900/9000

W1024.DLL     1024x768-16          512        8900/9000

T640.DLL      640x480-256          512        8900/9000

T768.DLL      768x1024-256         1024       8900

T800.DLL      800x600-256          1024       8900

T1024.DLL     1024x768-256         1024       8900

T8514.DLL     1024x768-256         1024       8900

T640V512.DLL  640x480-256 (1Kx.5K) 512        8900/9000

T640V.DLL     640x480-256 (1Kx1K)  1024       8900

T800V.DLL     800x600-256 (1Kx1K)  1024       8900

Fail to comply with the above requirements may cause

system hangup during OS/2 boot.  In case this happens,

use a bootup diskette to boot from drive A, restore OS/2

default DISPLAY.DLL driver (the saved one), then boot

OS/2 again.

[Usenet/Internet Distribution:

It appears that T640.DLL and T640V512.DLL actually require 1024K video

memory.  This problem is being worked on.]

3.  Some useful tips for trouble shooting

. Use TVGA 8900C/9000 chips or newer.

8900B chipset works in most cases.

[Usenet/Internet Distribution:

Note:  The archive "8900bfix" provides additional support for this

level chipset.]

. Use OS/2 2.0 GA version only.

OS/2 installation should be done after installation

of TVGA card.

. Do not forget to backup original OS/2 drivers

. Do not forget to run SVGA ON system command from

DOS FULL SCREEN session.

. Alt-F1 on system boot time will erase SVGADATA.PMI

Back up the .PMI files before trying Alt-F1

. Make sure your TVGA board has enough memory for

the designated PM display mode.

. Expierenced users may choose to manually copy the

driver files to C:.  Make sure DISPLAY.DLL and

Seamless WINOS2 drivers use the same resolution

and color.  Full screen WINOS2 driver may use

different display mode.

. Whenever the TVGA board configuration is changed,

such as adding or removing display RAM, setting

jumpers, you need to run SVGA ON again to replace

.PMI file.  The best way for this is to run SVGA OFF,

restore the original VGA (640x480-16) DISPLAY.DLL,

erase old .PMI file first.  Change the hardware

configuration.  Then boot and run SVGA ON from DOS

full screen session.  Install the SVGA driver

and reboot.

. T768.DLL (portrait mode) needs a special .PMI

entry that currently can not be generated by

SVGA.EXE.  If you have problem installing this

driver, contact our technical support.

. If abnormal display occurs when running apps,

please try to duplicate the problem.  You may

also want to try the same apps under the original

DISPLAY.DLL which comes with your OS/2.  If the

problem is still there, there is a high chance

that this is an application's bug.  Otherwise,

please report the problem to our technical

support.

. Bugs can usually be fixed within days.  But for

stability, the drivers may not be available to

the public immediately until the next major driver

release.  Private build can be obtained on individual

bases for evaluation.

. If you cannot get TVGA driver for WINOS2 working,

check to see if the driver is compressed.  A regular

size (just like the W800SL.DRV) is about 73000 bytes.

If yours is considerably small, you need to uncompress

it first.  Try the EXPAND.EXE program in GA Disk 9 to

uncompress the .DRV file.

[Usenet/Internet Distribution:

Note:  The Win-OS/2 drivers supplied with the OS/2 drivers have been

pre-EXPANDed for convenience.]

4. Advanced topic: adjusting video parameters

In most cases, TVGA card matches display monitors

perfectly. However, some monitors cannot sync under

default system settings.  Other monitors support high

refresh rates.  To get the best display result, you

may want to go through the following procedures to

adjust video parameters.  Unless this is necessary,

and you fully understand how this works, we recommend

that you skip this operation.

. Boot OS/2 with the default DISPLAY.DLL (640x480-16).

. Use the TVGACRTC.EXE utility (on TVGA Driver/Utility

Diskettes, also available in Trident BBS) under OS/2

DOS FULL SCREEN.

It will generate TVGACRTC.SYS file and insert a line

DEVICE=TVGACRTC.SYS

in the CONFIG.SYS.

. Shutdown OS/2 and reboot the system.

. Open a DOS full screen and run

SVGA ON

This will generate a "corrected" .PMI file.

. Remove DEVICE=TVGACRTC.SYS statement from CONFIG.SYS

. Shutdown OS/2 and reboot again.

5. Advanced topic: Vertual Desktop Display Driver

Virtual Desktop Drivers (T640V512.DLL, T640V.DLL and

T800V.DLL) are a new type of OS/2 display drivers.

The driver enables user to work on a larger desktop

which cannot be supported directly by the video display

hardware.  For example, your VGA monitor does not support

high resolution, or your VGA adapter is not able to set

video resolution higher than 1024x768 256 color.  Also on

a smaller monitor (say 14"), you may not want to use the

highest resolution mode but still want a bigger dimension

desktop.

Typical 256 color Virtual Desktop dimensions on TVGA

chipset are 1024x1024 for 1MB RAM, 1024x512 for 512KB

RAM.  Larger sizes may be supported on future TVGA

chipsets.

Since the physical screen (the viewport) is smaller than

the virtual desktop, there must be a way to move the

viewport around the desktop to display different parts

of the desktop.  Current driver design enables the user

to move mouse pointer beyond the boundary of the viewport,

to the direction of the part of desktop to be viewed.

The display driver will follow the mouse pointer and

adjust the position of viewport accordingly.

Some users may not like Virtual Desktop Drivers, for

various reasons.  They can use the T640.DLL or T800.DLL

instead.

To install T640V512.DLL, T640V.DLL or T800V.DLL, you need

to edit the .PMI file.  Make sure that you understand the

structure of .PMI file before editing it.  First make a

copy of the current .PMI file (in case you want to go back

to T640.DLL or T800.DLL).  Then use an editor to open

C:\OS2\SVGADATA.PMI.

If you are installing T640V512.DLL, find the mode entry

for 640x480-256 color mode from SVGADATA.PMI file:

[comment]

Graphics Mode: 640 x 480 x 256 colours.

and change the [modeinfo] section to:

[ModeInfo]

ModeAttributes   = 0x18

BytesPerScanLine = 80

XResolution      = 1024

YResolution      = 512

TextRows         = 30

BitsPerPixel     = 8

NumberOfPlanes   = 1

PageLength       = 524288

SaveSize         = 524288

InterlaceMode    = 0

BufferAddress    = 0x0a0000

All other steps are similar to installing T640.DLL.

If you are installing T640V.DLL, find the mode entry

for 640x480-256 color mode from SVGADATA.PMI file:

[comment]

Graphics Mode: 640 x 480 x 256 colours.

and change the [modeinfo] section to:

[ModeInfo]

ModeAttributes   = 0x18

BytesPerScanLine = 80

XResolution      = 1024

YResolution      = 1024

TextRows         = 30

BitsPerPixel     = 8

NumberOfPlanes   = 1

PageLength       = 1048576

SaveSize         = 1048576

InterlaceMode    = 0

BufferAddress    = 0x0a0000

All other steps are similar to installing T640.DLL.

If you are going to install T800V.DLL, find the mode

entry for 800x600-256 color mode from SVGADATA.PMI file:

[comment]

Graphics Mode: 800 x 600 x 256 colours.

and change the [modeinfo] section to:

[ModeInfo]

ModeAttributes   = 0x18

BytesPerScanLine = 100

XResolution      = 1024

YResolution      = 1024

TextRows         = 37

BitsPerPixel     = 8

NumberOfPlanes   = 1

PageLength       = 1048576

SaveSize         = 1048576

InterlaceMode    = 0

BufferAddress    = 0x0a0000

All other steps are similar to installing T800.DLL.

Trident Microsystems, Inc.

205 Ravendale Drive

Mountain View, CA 94043

Phone: (415) 691-9211

BBS:   (415) 691-1016   (Original distribution site for this package)

Line 0: 9600 bps

Line 1: 2400 bps

Parity: NONE

Data:   8

Stop:   1

* This package if provided by Trident Microsystems, Inc.  It can be

uploaded to any other electronic media, provided that no charge is

made from it.