E P A S M

Microprocessor assembler

for the Intel 8749H

Joseph L. Hora

January 12, 1992

Version  1.3

EPASM  Assembler     October 11, 1990

Table of Contents

I.  Introduction  . . . . . . . . . . . . . . . . . . . . . .   2

II.  Loading EPASM on your system . . . . . . . . . . . . . .   4

III.  Creating an assembly language program . . . . . . . . .   6

IV.  EPASM assembler directives . . . . . . . . . . . . . . .   8

Description of assembler directives  . . . . . . . . . .   8

; . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   8

ORG . . . . . . . . . . . . . . . . . . . . . . . .   8

"label"   . . . . . . . . . . . . . . . . . . . . .   8

CONST   . . . . . . . . . . . . . . . . . . . . . .   9

INCLUDE   . . . . . . . . . . . . . . . . . . . .    10

V.  MCS-48 command notes  . . . . . . . . . . . . . . . . .    12

VI. Assembling your program . . . . . . . . . . . . . . . .    15

command line options . . . . . . . . . . . . . . . . .    15

Running EPASM  . . . . . . . . . . . . . . . . . . . .    16

VII.  Error messages  . . . . . . . . . . . . . . . . . . .    18

VII.1  Command, file, and DOS errors . . . . . . . . .    18

VII.2  EPASM assembly errors . . . . . . . . . . . . .    19

VIII.  Using the EPROM programming device . . . . . . . . .    22

INVOICE . . . . . . . . . . . . . . . . . . . . . . . . . .    24

1

EPASM  Assembler     October 11, 1990

I.  Introduction

The EPASM program is a one-pass assembler that was written for

the Intel 8749H EPROM as described in the Intel Microcontroller

Handbook, using the MCS-48 instruction set.  This assembler can

also be used for other chips using this instruction set (8748H

EPROM, 8048AH/8049AH/8050AH ROM, and 8035AHL/8039AHL/8040AHL CPU)

as far as their memory structures are similar to the 8749H EPROM.

This manual is not intended to be a reference for the MCS-48

instruction set or the above mentioned devices.  Please refer to

the Intel Microcontroller handbook for a complete description.

The input to the EPASM program is a file containing "source code"

which is the EPROM program.  This contains the MCS-48 command

mnemonics, labels, assembler directives, and comments.  EPASM

converts it into the machine code of the processor, the actual

command numbers that will direct the processor to carry out the

steps the programmer has specified.  This machine code is saved

to a file which then must be transferred to the processor using a

EPROM "burner" or programming device.

The output of EPASM is an object code module file.  In this file

are the encoded commands that were specified by the assembler

language program.   This program was written to construct an

ASCII file suitable for the PROMPRO 7 EPROM Programming Unit,

made by Logical Devices, Inc.   The files have also been used to

program EPROMs using the B&C Microsystems 1409 Universal (E)EPROM

programmer.  However, the file can also be passed to many other

EPROM programming devices that support the same format.  The file

could also be converted by a program of your own to the proper

format for your device.

This program is distributed as shareware.  It may be freely

distributed to others, as long as it is in the form of the

archived .ZIP file which contains all the separate files

necessary for EPASM, and the files are not modified in any way.

If you find this program useful, register your use of the program

by sending $15 to

Joseph Hora

46-1063 Emepela Way, Unit D

Kaneohe, HI  96744-3975

When registering, you will receive the latest version of EPASM

(without the annoying registration screen), a printed copy of the

manual, and the source code for EPASM (written in Turbo Pascal

2

EPASM  Assembler     October 11, 1990

5.0).  You will also be notified when updates of the program

occur in the future.  Questions and technical support can be

obtained by mail at the above address, or by phone at (808) 956-

9848 weekdays 8am-5pm HST.

Your registration entitles you to use this software on a single

computer and to make as many copies of this software as you wish

for backup purposes.

Note:  MCS-48 and the various chip names mentioned above and

throughout this manual are registered trademarks of the Intel

Corporation.  The EPASM program and associated files are

copyright 1990-92 by Joseph Hora.

3

EPASM  Assembler     October 11, 1990

II.  Loading EPASM on your system

Before doing anything else with your distribution disk, make a

backup copy first.

The approved EPASM release is archived using the PKZIP file

archiving utilities, and is named "EPASM.ZIP" .  This includes

the following files:

EPASM.EXE  - the assembler program

EPASM.DOC  - this document

INSTR.SET  - the MCS-48 instruction set

SAMPLE.ASM - a sample assembly language program

INVOICE.DOC - the registration form for EPASM

To install the assembler, these files must be extracted and

placed in a directory of your choice.  The following are

instructions for various system options.

Single floppy drive systems:

First, make a backup of the distribution disk.  Place the

original EPASM (source) disk in drive A: and type

diskcopy a: a:

Wait until the computer is done reading the disk and asks you for

the target disk.  Remove the original disk and place a blank disk

in drive A:, then type <RETURN>.

If you have PKUNZIP and EPASM.ZIP on the same 360K or larger

floppy disk with nothing else, there will be enough room to store

all the original plus extracted files.  At the DOS prompt, type

pkunzip epasm

and the files will be extracted to the default directory.

Dual Floppy users:

First, make a backup of the distribution disk.  Place the

original EPASM (source) disk in drive A: and a blank formatted

disk (target) in drive B: and type

diskcopy a: b:

4

EPASM  Assembler     October 11, 1990

Wait until the computer is done reading the disk and writing to

the target disk.

You can make an EPASM disk by doing the following:  First, make

sure you have PKUNZIP and EPASM on one disk, and place it in the

A: drive.  Place a blank formatted disk in the B: drive and type

"B:" to change default to that drive. Then type

a:pkunzip a:epasm

and the files will be extracted and written to the disk in the B:

drive.

Hard disk users:

See the above instructions to backup your copy of EPASM before

you install the program on your hard drive.

Place the disk with the files EPASM.ZIP and PKUNZIP in the A:

drive.  Make a subdirectory on your hard drive to place your

EPASM files into, for example, type

C:\>mkdir epasm

(Type the commands above that are after the >).  Change directory

to that subdirectory

C:\>cd epasm

C:\EPASM>

Then type

C:\EPASM>a:pkunzip a:epasm

and the files will be extracted and written to the directory you

are currently in on the hard drive.

5

EPASM  Assembler     October 11, 1990

III.  Creating an assembly language program

You must create the source code for EPASM in a text editor

program.  This can be any program that allows you to save the

file in "ASCII" or "DOS" format.  This means just text, with no

page breaks, printer commands, formatting codes or commands

specific to the word processing program.  For example, in

WordPerfect, you can use the Ctrl-F5,1,1 command to save the

document as a DOS text file.

EPASM will create various output files, based on the text file

containing commands it is given and the command line options (see

Chapter VI.)  These output files can be viewed using the "TYPE"

command from the DOS command prompt.  The files can also be read

into the word processing program and viewed in the same way the

source code can.  This is most useful when the user is trying to

track down errors in the source code.

There are two element types in the program, assembler directives

and MCS-48 commands.  Assembler directives are commands that are

processed during the assembly by the EPASM assembler.  The second

type are commands that are converted directly into machine code

for the EPROM processor.  These two types are discussed in the

following chapters.

Below is a short excerpt from a sample program written for this

assembler:

;

;

;       IDLE routine:  remain here until interrupted

;

IDLE    mov       R3,#80        ;select expander 0, disable chop

mov       a,#8F         ;address   "     "

outl      P2,a          ;   "      "     "

movd      P7,a          ;output slave idle state

call      riff          ;reset interrupt-in-progress

jmp       IDLE          ;jump to IDLE

;

;   End of IDLE routine

;

This short excerpt illustrates a number of characteristics of the

source code.  First, the simplest command is the semicolon

character ";", the comment command.  This simply instructs the

assembler to ignore all the characters following that one on the

6

EPASM  Assembler     October 11, 1990

current line.  It can be used at the beginning of the line to

designate the entire line as a comment, or it can be at the end

of a valid command.  In the above example, it is used to space

out the routine with blank lines, hold the title of the routine,

and annotate each command line with a comment.  This is usually a

good practice, since even a small assembly language program is

difficult to follow without a few comments explaining the code,

even for the person who wrote the program!

Note that there is a mix of uppercase and lower case characters

in this example.  EPASM treats upper and lower case letters to be

equal.  Therefore, the commands "mov" and "MOV" are treated in

exactly the same way.

The next assembler directive used is in the first non-comment

line, the text "IDLE".  This is being used as a "label", or a

marker designating that line or address in the program.  This

allows the programmer to reference commands in the program

without knowing the actual address of the command in the program

memory.  Notice at the end of the routine there is a "jmp IDLE"

command, which will cause the EPROM to loop back to the beginning

of the routine when it runs the program.  The EPASM program

determines the address of "IDLE" as it assembles the program.

On the same line as the "IDLE" label is the MCS-48 command for

the processor.  Note that these words are all separated by a

number of spaces.  Any combinations of spaces and tabs may be

used to separate the words of each line.  The spacing can be

chosen arbitrarily to make the program easy to read.  In the

above example, the first column is reserved for labels.  This

makes the labels easy to see and therefore easier to follow the

flow of the program.  The next two columns hold the command

arguments, and the third column holds a comment for that line.

This is a fairly standard format for assembly language programs.

Another equally valid format would be to have the commands be in

one column, as in the following:

;

;

;       IDLE routine:  remain here until interrupted

;

IDLE    mov R3,#80         ;select expander 0, disable chop

mov a,#8F          ;address   "     "

outl P2,a          ;   "      "     "

movd P7,a          ;output slave idle state

call riff          ;reset interrupt-in-progress

jmp IDLE           ;jump to IDLE

;

;   End of IDLE routine

;

7

EPASM  Assembler     October 11, 1990

IV.  EPASM assembler directives

This chapter describes the valid assembler directives for the

EPASM program.  An assembler directive is a command that does not

directly generate machine code in the output file, but affects

the way EPASM compiles the program.   The following are the

recognized assembler directives, and a description of each:

;        ORG       "label"       CONST     INCLUDE

Description of assembler directives:

;

the comment character.  Directs EPASM to ignore all other

characters following this one on the same line.

ORG

specifies the location  of the start of the program in the

processor's program memory.  The default location if this

directive is not used is zero.  The correct syntax is

ORG addr

where "addr" is the desired starting address, in

hexadecimal.

"label"

any word more that three characters long and less than nine

characters which is intended to mark a location in memory.

I have enclosed the word "label" in quotes, since there is

no actual command that you invoke by typing "label", you

simply use a word that is not a command and the assembler

will interpret it as a label.  The label cannot be the same

as another of the assembler directives (such as ORG or

CONST), or be the same as one of the MCS-48 commands (such

as MOV or JMP).  Note that since no upper and lower case

distinctions are made, the label "IDLE" is treated the same

as the label "idle".

When the label appears as the first element of a line, that

equates the label with the memory position of that line.

8

EPASM  Assembler     October 11, 1990

When the label appears in any other line after a valid MCS-

48 command, that means that label is being referenced.  When

the EPROM program is assembled, the address corresponding to

the line where the label appears will be placed in that

command.  The label can be defined only once, but referenced

many times.  Below is a program excerpt where labels are

used.  The routine checks the test pins of the 8749H

processor and takes actions based on their state.  Notice

the use of labels: the first label is "Main" which is

referenced in a number of instructions.  Other labels allow

control of the program flow based on the conditions

encountered.

Example IV.1

Main     JT0 Stat1           ;If T0 set, goto Stat1

JT1 Stat2           ;If not T0 and T1, go here

Mov R1,#20          ;Neither T0 or T1, BankA<<<<

Jmp StatEnd         ;Exit Stat section

Stat1    JNT1 Stat3          ;

Mov R1,#00          ;If T1 also set, then WAIT

Jmp Main

Stat3    Mov R1,#40          ;T0 and not T1, so Bank B<<<<

Jmp StatEnd         ;

Stat2    Mov R1,#60          ;not T0 and T1, so Bank C<<<<

StatEnd  Mov A,R1            ;Get current base address

XRL A,R2            ;Compare to last base address

JZ  TakDat          ;If same, go inc. counters

MOV A,R1            ;Different bank this time

MOV R2,A            ;set "old" base address here

IN  A,P1            ;set old port reading equal

MOV R3,A            ;     to current one

IN  A,P2            ;same for port 2

MOV R4,A

JMP Main            ;End of Main Program loop

The processor running this program would repeat the loop,

testing T1 and T2.  If the current base address as the last

time, the program jumps to the line designated by the

"StatEnd".

CONST

This assembler directive is useful for equating hex

constants with words.  It is exactly like defining a label

that references a constant, not a memory location.  The

syntax of the directive is

CONST  name   value

9

EPASM  Assembler     October 11, 1990

For example, the program in example IV.1 above considers the

memory as being divided up into 3 "banks": A, B, and C, at

locations 20, 40, and 60 (hex).  Instead of the statements

above which explicitly specify these memory locations, one

could define constants to make this more readable.  If the

following lines are included in the program file:

;

;        Constants

CONST    BankA    20        ;Bank A: 32-63 decimal

CONST    BankB    40        ;Bank B: 64-95 decimal

CONST    BankC    60        ;Bank C: 96-127 decimal

;

Then example IV.1 could be written as follows:

Example IV.2

Main     JT0 Stat1           ;If T0 set, goto Stat1

JT1 Stat2           ;If not T0 and T1, go here

Mov R1,#BankA       ;Neither T0 or T1, so Bank A<<<<

Jmp StatEnd         ;Exit Stat section

Stat1    JNT1 Stat3          ;

Mov R1,#00          ;If T1 also set, then WAIT

Jmp Main

Stat3    Mov R1,#BankB       ;T0 and not T1, so Bank B<<<<

Jmp StatEnd         ;

Stat2    Mov R1,#BankC       ;not T0 and T1, so Bank C<<<<

StatEnd  Mov A,R1            ;Get current base address

XRL A,R2            ;Compare to last base address

JZ  TakDat          ;If same, go increment counters

MOV A,R1            ;Different bank this time, reset

MOV R2,A            ;set "old" base address here

IN  A,P1            ;set old port reading equal

MOV R3,A            ;     to current one

IN  A,P2            ;same for port 2

MOV R4,A

JMP Main            ;End of Main Program loop

The main reason for having the CONST directive is for

program clarity.  It is much easier to understand what you

are doing if you write data to "BankC" instead of "80".

INCLUDE

This directive causes EPASM to stop reading from the current

file being assembled, open the file specified and assemble

commands from that file into the program until the end of

that file is reached, then return to the original program

10

EPASM  Assembler     October 11, 1990

and continue assembling.   It is effectively the same as

having those commands in the INCLUDE file in your program.

The command syntax is

INCLUDE [path]filename

The file must be in the current directory, or the full path

specified, or else EPASM will not be able to find the file

and an error will be generated.  This command is useful if

you have a number of standard definitions or routines that

you use in many programs.  Then you can have a single file

containing them and all the other programs you write can

INCLUDE this file, so you don't have to have multiple copies

of the routines in every file.  Also, when editing the

standard routines, you can then recompile the routines and

the changes will be effective in all programs that use that

file.

The only limitation on the include file is that you cannot

"nest" the include files.  That is, you cannot have an

"INCLUDE" file that has another "INCLUDE" command for yet

another file.

Also, when there is an error in the program after the

include file, the line number reported will be the actual

line number of the program file plus the number of lines in

the include file.  If the error is in the include file

itself, the line number reported will be the line number of

the error in the include file plus the number of lines ahead

of it in the main program.  To be certain of the line

numbers, the best way is to look at the .WRK file created by

EPASM (see chapter VI).  This file will show all lines from

the main program and the include file together, in the order

assembled.

11

EPASM  Assembler     October 11, 1990

V.  MCS-48 command notes

The MCS-48 commands are stored in the file INSTR.SET.  This

should, for the purposes of EPASM, be the reference for the

correct syntax of the commands, since they will be expected to be

exactly as they are spelled in this file.  Note that all the

commands in the file INSTR.SET must be in upper case, since any

file assembled will be converted to upper case before being

compared to these commands.  The commands in the program to be

assembled can be in either upper or lower case, as discussed

above.

The file INSTR.SET is an ASCII file, with each line being the

definition for one command.  Comments can also be included, if

they are preceded by a semicolon.  Below are a few sample lines.

;

;

JF0  $    B6   2

JF1  $    76   2

JMP  !    04   2

MOV  A,#  23   2    ;this command moves in the immediate word

MOV  A,PSW C7  1

MOV  A,R0 F8   1

MOV  A,@R0 F0  1

MOV  A,@R1 F1  1

MOV  A,T  42   1

The format of the line is first the command word(s), then an

address or data indicator, if any, then the hex machine code for

the command, then the number of machine cycles for that command.

The data or address indicators are the "#" and the "$"

characters, respectively, and they show that the command includes

a byte of data or an address.   For example, the command to move

immediate data to the accumulator is given by the line

MOV A,#  23  2

Since it contains the "#" character, it indicates that a second

byte is required; namely, the data to be moved into the

accumulator.  So the correct usage of this command would be

MOV A,#1C        ;Move the value 28 (1C hexadecimal) to acc.

Similarly, the "$" character indicates that an address is needed

in the command.  So in the command

JF1  $    76   2

12

EPASM  Assembler     October 11, 1990

the program will jump to the address given if flag 1 is set.  The

correct usage of this command is

JF1  DONE          ;if flag set, jump to DONE

where DONE is a label that is defined elsewhere in the program.

Note that when the address is given by an eight-bit value, the

location of DONE must be on the same memory page as the JF1

command, or else the jump will not be performed properly.

The "!" character is used for the JMP and the CALL commands,

which have a 11-bit address (in order to cover the entire 2K

address space).  The upper three bits of the address are encoded

into the first byte of the command.  This differs from the

address byte in commands using the "$" character, which is just

the second byte and therefore is limited to eight bits.  EPASM

treats these two commands specially and encodes the address

automatically.

Other commands have no data or address information, and are

completely specified in the INSTR.SET file.  For example,

MOV  A,@R1 F1  1

specifies the command to move data memory contents into the

accumulator (register R1 has been previously loaded with the

desired address).  The correct usage of this command is

MOV A,@R1           ;Move register-addressed data into acc.

almost precisely as it appears in INSTR.SET.

Because the INSTR.SET file is in ASCII format and read into the

EPASM program every time it is executed, the INSTR.SET file can

be edited to change the syntax of the commands, if desired.  This

is not usually recommended, since it is good to keep the command

names similar to the MCS-48 commands for clarity.  However, if

you wish to program for a different EPROM with a different

command set, one could simply change the file INSTR.SET to

contain the commands in the alternate instruction set.  The only

restriction is that the CALL and JMP commands cannot be changed,

since they are handled in a special way.  Also, new commands with

the "!" character cannot be added, since it requires changes in

the EPASM program.  Of course, if you have registered your copy

of EPASM and have the source code, you can alter the assembler

program for different devices.

Another feature of EPASM is the ability to do simple arithmetic

within the statement with hexadecimal numbers.  The allowed

operations are addition and subtraction.  For example, say that

you wanted to move the value 8C into the accumulator.  The

command would be

13

EPASM  Assembler     October 11, 1990

MOV A,#8C      ; fill accumulator with value

Now say that you wanted to move a number that was 7 larger than

8C.  The correct command would be

MOV A,#8C+07    ;fill accumulator with 7 greater than 8C

This expression is evaluated during the assembly, and the correct

constant value (147, or 93 hexadecimal) is placed in that memory

location.  So the command is equivalent to

MOV A,#93        ;fill accumulator with 93 hex

Since the expression is evaluated before the .OBJ output file is

written, the statement will not add to the object code file

length, or slow the program down.  This feature is useful when

you don't feel like doing the math operaation, or when you want

to show the relationship between constants.

14

EPASM  Assembler     October 11, 1990

VI. Assembling your program

Once the program has been written and saved to disk as an ASCII

or "DOS" format text file, it is ready to be assembled by EPASM.

The following is the command syntax for the assembler:

epasm progname [-w -e -b -l -h] [filename]

The "progname" above should be the file name of the ASCII file

which contains the assembly language program you wish to

assemble.  It can be just the file name, or the full path if the

file is not in the default directory.  The extension ".ASM" is

assumed if none is used.  Be sure not to use the extension .TMP,

because EPASM uses this for a temporary file, and later erases

it.  If you have another file with the same root name as your

program and this extension, it will be erased.

The command line options are:

-w  : create a "work" file, showing the program memory

address, the assembled commands, and the original

command from the input file source code.  This is

extremely useful when debugging the program, to see

where the error is occurring and what the EPASM program

is trying to do with the commands it is given.  The

default file name is the original name, with the

extension ".WRK".

-e  : create an error listing file.  This is a file

containing all the error messages generated when

running EPASM.  These messages are also printed to the

screen, but may come too fast to see them all.  The

default file name is the original file name with the

extension ".ERR".

-b  : in addition to the ASCII object module created whenever

EPASM successfully assembles a file, a binary file

containing the machine code can also be created using

this option.  The binary file is a non-printable file

which is 2K bytes in length and contains no format

characters, checksums, etc, just the machine code in

binary format.  This file may be useful when a EPROM

programming device may require a different format than

the default .OBJ module normally created by EPASM.

This binary file can be read easily into a conversion

program that the user would write to output the proper

format.  The default name for this file is the original

name and the extension ".BIN".

15

EPASM  Assembler     October 11, 1990

-l  : this option creates a label listing output file that

may also be useful for debugging purposes.  This file

contains a list of the program addresses and the

machine code to be placed in that address.  Labels are

indicated by a carat (^) followed by an ASCII character

for identification.  At the end of the program will be

a listing of all the labels used and the characters

used to identify them in the listing above it.  This

output was mainly used during the development of EPASM,

but it can also be useful as a debugging tool.  The

default name for this file is the original file name

plus the extension ".LAB".

-h :  This option creates a "Hex" output file, which is an

ASCII file with the assembled commands listed in

hexadecimal form, 16 bytes per line (32 characters).

This is basically the .OBJ format without the header

information and the checksum.  See chapter VIII.

Note that when using command line switches, either upper case or

lower case can be used, so that "-E" is the same as "-e".  Also,

the DOS switch character can be used (e.g., "/E").

Running EPASM

When EPASM is run without any arguments, the program simply lists

the command format and gives a short reminder of the command line

options.  To do this, just type

C> EPASM

(Note: in this example and those below, the "C>" represents the

DOS prompt and should not be typed.)  The EPASM program must be

in the default directory, or a directory that is accessible via

the current DOS PATH defined.  The file INSTR.SET must be in the

default directory or the same directory as EPASM, otherwise the

error "Instruction set not found" will be generated.  To assemble

a program, type EPASM plus the file name to assemble.  For

example, if you wish to assemble the file SAMPLE.ASM which is

included with this documentation, type

C> EPASM SAMPLE

The extension ".ASM" is assumed, and need not be entered.  If you

wish to assemble a file called SAMPLE.TST, just type

C> EPASM SAMPLE.TST

If you use command line switches, separate them with spaces from

the rest of the command and from each other.  For example, if you

want to create a "work" file and an error listing (if there are

16

EPASM  Assembler     October 11, 1990

any errors in the program), type

C> EPASM SAMPLE -E -W

If there are errors, the files SAMPLE.ERR and SAMPLE.WRK will be

created.  If there are no errors, the files SAMPLE.WRK and

SAMPLE.OBJ will be created.  If you wish to specify different

file names for the output files, the names can be included after

the switch.  For example, if we want to assemble the file

SAMPLE.ASM, but want the work file name to be WORK.SMP, and

create in addition to SAMPLE.OBJ a binary output file, we would

type

C> EPASM SAMPLE -W WORK.SMP -B

While the EPASM program is running, it will notify you of errors

it encounters by printing a message to the screen.  If errors are

encountered, no .OBJ, .HEX or .BIN output files are generated.

If no errors are encountered, an .OBJ file will be created, along

with a .BIN or .HEX file if the proper command line switch is

used.  An .ERR file will not be created, even if one is specified

using the command line switch.

If you wish to stop the assembly at any time, just press any key.

The program will suspend the assembly and ask if you wish to

abort.  Type "Y" if you want to stop.  This feature means that

you can't "type ahead" the next DOS commands after EPASM, but the

ability to terminate the assembly is very useful, mostly when

EPASM finds errors and you do not want the assembler to bother

with the rest of the file until you have gone back and corrected

the errors.

The file SAMPLE.ASM is included with this release to mainly

illustrate the proper syntax and structure of an .ASM program,

not to show good or useful programming technique.

17

EPASM  Assembler     October 11, 1990

VII.  Error messages

The first section below summarizes some of the errors that may

occur in the command used to start EPASM from DOS, and errors in

finding the proper files necessary to run EPASM.  The second

section describes the error messages given by EPASM when it finds

errors in the program it is assembling.  Tips are given as to

what to do to correct the errors.

VII.1  Command, file, and DOS errors

Bad command or file name

Make sure that you typed EPASM correctly, and that the

EPASM.EXE program is in the current directory, or accessible

via the PATH you have currently defined.

Instruction set file INSTR.SET not found

The file INSTR.SET is not in the default directory or cannot

be found in the directories defined in the current DOS PATH.

Make sure you have copied it from the distribution disk to

the proper directory.

Error in command line parameter

You used an invalid combinations of switches and file names

in the command line when you ran EPASM.  See chapter VI for

instructions on using the command line switches and file

names.

File not found

Path not found

The file name you specified is not correct, or it is not in

the directory you specified, or that directory does not

exist.  Check the file names used in the command line, and

in the INCLUDE files used, if any.

Disk is write-protected

If your files are on a floppy drive, make sure the write

18

EPASM  Assembler     October 11, 1990

protect tab is not on (for a 5 1/4" disk) or in the proper

position (covering the hole in the 3.5" disk).

Drive not ready

Make sure the disk is in the drive specified, and the disk

is locked in for reading.

VII.2  EPASM assembly errors

The following are error messages generated by EPASM when it finds

errors in the program it is assembling.  The error message will

be printed out, followed by some of the offending text, and a

line number.  Often if a line has an error in it, two different

error messages will be created, as EPASM tries to interpret the

code in different ways.  When the problem is solved, both

messages will go away.

The error detection in EPASM can only detect certain errors.  It

cannot, of course, diagnose logical errors that will cause your

program to not work as you expect after you have "burned" the

program into your EPROM.

Invalid constant character

The text encountered is being interpreted as a hexadecimal

constant, and there was an error in converting the number.

You may be trying to define a label that is two or fewer

characters in length (a label must be three to eight

characters long), or you mistyped the constant (you entered

8L when you intended 8C).

Hex constant overflow

The assembler is expecting a hexadecimal constant in that

location, but the number there is over two digits in length

(constant limits are 00 to FF hexadecimal).

Invalid jump command

The jump command entered is not a valid MCS-48 jump command.

19

EPASM  Assembler     October 11, 1990

Syntax error

The command used on the line is not a valid MCS-48 command.

Most likely you have misspelled the command, or the

assembler is attempting to evaluate a label as a command.

Make sure there are spaces or tabs separating the elements

in a line.

Call instruction cannot be located at [address]

The CALL instruction cannot be located at addresses 2046-

2047 or at 4094-4095.  You must change your program so that

these statements do not fall on these addresses.

Conditional jump at [address] cannot reach destination

The conditional jump commands (e.g., JC, JF0, etc.) can only

jump to addresses on the same 256-byte memory page, since

the address used in the command is only 8 bits.  The program

is trying to jump to a label (address) on another page.  You

must change the program so that either the conditional jump

and the destination are on the same page, or rewrite the

code so that you jump across the page boundary (the CALL and

JMP commands are not restricted to one page, they can jump

directly to any location within the 2K program memory).

Unresolved label

You have referenced a label somewhere, but never defined its

location (you define a label by putting it as the first

element in some line).  Either you forgot to define it, or

misspelled it, either here or where it was defined.

Warning: cannot nest INCLUDE files

Your INCLUDE file contains another INCLUDE file.  This is

not allowed: call the second INCLUDE file from the main

program.  The second INCLUDE file will be ignored, and the

assembly will continue.

Warning: error opening INCLUDE file

The INCLUDE file you specified could not be found.  The

INCLUDE file must be in the default directory, or must be

specified by the full path name. The assembly will continue,

but without the information in the INCLUDE file.  Check the

location of the file.

20

EPASM  Assembler     October 11, 1990

Work file error

There was an error opening or deleting the work file

specified (or the default work file name, if none explicitly

chosen).  EPASM first creates a work file with the extension

.TMP, then at the end, erases this and makes a work file

with the extension .WRK, or whatever you have specified.  Be

sure not to have any other files in the directory with the

extension .TMP, or they may be erased!

21

EPASM  Assembler     October 11, 1990

VIII.  Using the EPROM programming device

The procedure that you use to program or "burn" the EPROM will

vary greatly, depending on the device being used.  In most cases

you will be able to send the programming device a file of

commands, which will direct the device to program the EPROM.

This file is one of the output files from the EPASM program which

contains your assembled code, either the default .OBJ file (which

is the most common case), or the hex or binary files containing

the same program in different format.

Some programming devices (such as the PROMPRO-7)  require that

you communicate with it using a terminal emulator program.  In

this case, the emulator program has to have an "upload" command

to send a file to the device.

Other devices come with their own software drivers, such as the

B&C Microsystems 1409 Universal programmer.  With this device,

the file can be read in and sent directly using commands from the

driver program.

There are three output file options in the EPASM program.  The

default .OBJ files are in Intel hex format.  Each line of the

file has the following information (all numbers in hexadecimal):

INTEL  hex  format

# of ASCII

Characters     Description

1         the colon character (:)

2         number of data bytes N in record (00 for end record)

4         the starting address of where the data bytes are to

be located

2         00=program data in line, 01=end of file record

N*2        the program data to be loaded into the EPROM

2         checksum (two's compliment of the sum of the bytes

after the colon character, modulo 256)

EPASM always sets N above equal to 10H (16 decimal).

Another format option is the binary file, which contains the

program to be loaded into the EPROM with no formatting

information, just the bytes starting at address 0 and going to

the end of the program memory, for a total of 2048 8-bit bytes of

data.  The locations from the end of the assembled program to the

22

EPASM  Assembler     October 11, 1990

end of the program memory space will be filled with zeros.

A final format is the ASCII hex file, which contains the ASCII

representation of the hex data to be loaded into the EPROM.  This

is just the same as the Intel file, except only the program data

is in the file, there is no header information or end record.

The data for program starting at address 0 going to the end of

the program are included in the file.  Sixteen program memory

values are written per line (32 ASCII hex digits).

23

EPASM  Assembler     October 11, 1990

INVOICE

Remit to:

Joseph Hora

46-1063 Emepela Way, Unit D

Kaneohe, HI  96744-3975

Questions and technical support number:

(808) 956-9848

internet: hora@galileo.ifa.hawaii.edu

From:

________________________________________________

________________________________________________

________________________________________________

________________________________________________

Enclosed is $15.00 ( Check ___   Money Order ___  P. Order  ___)

for registration of EPASM.  Please send me the latest version of

the EPASM program and example files, a printed version of the

manual, and the source code for the EPASM program (in Turbo

Pascal 5.0), and put me on the mailing list for future updates

and extensions to the EPASM program.

24