The Issue of the Reliable Modem Escape Sequence

Modems allow personal computers to communicate with other computers over

telephone lines by transmitting data and files.  There are two states of

communication between an intelligent modem and the computer to which it is

connected, the Command Mode and the Receive/Transmit Mode.  The Command Mode

allows that modem to receive instructions or commands from the computer, such

as directing the modem to dial the phone, change modem settings, and hang up

the telephone after a transmission is completed.  The other state of

communication is the Receive/Transmit Mode.  It is during this state of

operation that the modem sends or receives data or files which are exchanged

between a personal computer and a remote computer.

When the modem dials the telephone number and makes the connection, it goes

from Command Mode to the Receive/Transmit Mode.  The mechanism that causes the

modem to go from the Receive/Transmit Mode back to the Command Mode is called

an "Escape."  The reliability of the escape mechanism is vital to the integrity

of the system and the ability to predictably deliver the data or file being

transmitted.

When the escape occurs in the middle of a data transmission an aborted

transmission results.  This unintentional escape causes the interruption of the

data flow, loss of time, increased  telephone toll charges because the

telephone line stays open even though no data is being transmitted, the

inability of the software to use the modem until it has been manually reset,

and most importantly, the file not being transferred.  In computer jargon, the

transmission "blows up" or "crashes" and the file transfer "aborts" or "bombs."

For the personal user, this creates at the very least a significant

inconvenience.  Most users wonUt recognize the problem for what it is when it

occurs.  In businesses with large numbers of modems, this can create

significant problems in operations for any company which relies on the

transmission of data by modems to conduct its operations and make money.

Preventing this inadvertent escape is a major design goal in reliable modem

design.

The Problem

Prior to 1981, modems used a sequence of characters to escape from

Receive/Transmit Mode to Command Mode.  The state-of-the-art at the time used a

series of control characters, sometimes "QUIT," to tell the modem to escape.

Because these characters could appear in the data stream, this escape mechanism

did not provide the user with a fail-safe method of escape.  It is easy to see

how a modem like this could inadvertently escape, creating just the problems

described.  The best idea at the time was to increase the length of the escape

code and reduce the probability of it appearing inadvertently in the data.  You

could never be sure a file you were about to send would not contain the

sequence of escape characters which would block the transmission.  In order to

alleviate this problem of inadvertent escape, it is necessary that the escape

mechanism be transparent to data.  That is, the possibility of the data alone

triggering the escape should not exist.

The Solution Was Found in 1981

Designing a modem completely transparent to data was the goal behind Dale

HeatheringtonUs efforts during the development of the first Hayes Smartmodem in

the early 1980s.  Dale was not satisfied with an escape mechanism which caused

some data to be UNSENDABLE.  Dale's belief was that it was unacceptable to

build a system where the modem was working as designed, i.e. NOT BROKEN, and

the computer was not broken, and the software was not broken, but at some time,

eventually, regardless of how improbable, the modem would fail to do its job.

In effect, Dale redefined the problem as understood by the experts at that time

and, by understanding the problem a different way, was on the path to a new

solution.

As part of his intensive research in the development of the original Hayes

Smartmodem, Dale Heatherington solved this problem by surrounding the escape

code, a sequence of characters, with guard times on both sides to alert the

modem that the sequence is distinguished from a typical string of characters in

a file transmission.  This escape sequence

<guard time>    <escape code>    <guard time>

eliminates the problems inherent in a data-dependent escape sequence because

of its use of time and because it does not depend on the probability of

character occurrence in a stream of data.  It is virtually impossible for the

Hayes escape sequence with guard time to appear in a file transfer and cause an

unintentional escape using the common file transfer protocols.  Computers just

donUt work that way.  The problem of inadvertent escapes was solved.

Dale Heatherington's invention led to the issuance of United States Patent

Number 4,549,302, the Improved Escape Sequence With Guard Time, often called

the Hayes '302 Patent, and corresponding patents in a number of countries.  The

Hayes '302 Patent ensures that modems escape or change to the Command Mode of

operation reliably and without the possibility that data alone could ever

trigger the escape.  In over eleven years of use of the Hayes '302, Hayes has

never received a complaint about an unintentional escape.  In addition, this

method was copied by almost everyone in the industry making it one of the most

widely adopted defacto standards.

With the Improved Escape Sequence With Guard Time you have reliable modem

control.  With reliable modem control, you donUt have to know how to:

o     Understand and know how to modify file and document data formats.

o     Understand the details of how your communications software controls

your modem.

o     Learn AT commands, syntax and escape sequence parameters.

o     Bypass your communications software and directly communicate with

your modem.

o     Deal with serial cable pin outs and control signals.

The New/Old Problem

In September of 1991, another modem manufacturer requested that Hayes examine

and test a particular modem to determine if the escape mechanism incorporated

by the modem was covered by the Hayes '302 Patent.  During the testing process,

Hayes discovered that the modem incorporates a new escape sequence that is

based purely upon a series of characters which appear in the data stream.

This new escape mechanism is called Time Independent Escape Sequence or

TIES.  The name appears to derive from the way in which the escape sequence

works because it does not make use of time as the Hayes '302 does.  TIES

depends entirely upon the appearance of the escape sequence in the stream of

data being received by the modem.  The TIES escape mechanism is very similar to

the escape mechanism in use at the time of the invention of the Hayes '302

because an escape can be triggered by the data being sent as part of a file

transfer.  Historically, that old escape sequence was unreliable, and it is

very likely that TIES will fall to a similar fate.  TIES cannot be considered

by anyone who understands it to be progress in the ongoing development of modem

design.

TIES - What Is It?

The simplest escape sequence for TIES is "+++AT<CR>" where "+++" stands for

any escape character and "<CR>" represents carriage return or the character set

in the modem registers by the AT command set which designates the end of the

command. Whenever that series of characters appears in the data stream, the

modem "escapes" or changes from Receive/Transmit Mode to Command Mode of

operation.  In effect, what happens at that point in the transmission is that

the flow of data stops!  The flow of data has halted simply because the

characters which make up the escape sequence have appeared in the data being

transmitted.

The appearance of these characters does not necessarily mean that you can pick

up a file and read until you find the escape characters.  The escape characters

may appear as a mixture of file data and file transfer protocols which would

not appear in printed text.  In either event, the receipt of those characters

by the modem will cause it to escape.

Since software compatibility was an important criteria, the TIES escape

sequence was selected to use the same sequence of characters that a computer

sends to a Hayes modem to cause it to first escape and then execute a command.

However, the TIES escape mechanism does not use guard times.  The TIES modem

appears to work with most existing software, but by disregarding the guard

times the data transparency of the TIES modem is fatally flawed.  In essence,

the TIES escape sequence was designed to fool computers into thinking that they

were communicating with a Hayes modem or a modem incorporating the Hayes '302

Patent, but TIES modems cannot fool the data being sent to prevent the TIES

block from stopping the data transmission.

TIES - What does this mean to you?

Depending on where the escape characters appear in the data, the receipt of

those characters by the modem could create unintentional escapes, prematurely

halting the flow of data.  For instance, if the file being transferred has been

completely transferred and the TIES escape characters appear at the end of the

data, there is no problem.  The receipt by the modem of the escape sequence at

that time would not interrupt the flow of data as all the data in that file has

already been transmitted.  However, if the TIES escape characters appear at any

place during the transfer of a file other than at the end of the data, when

those characters are received by the modem, the modem will escape, blocking the

transmission of the file Q the TIES block.  At that time, the modem may need to

be reset, and the transmission of that file must begin again.

Every time the file reaches the point where the TIES escape characters occur,

the modem will experience a TIES block and escape.  As long as those characters

appear in the data, at any point other than at the end of the data, the file

can never be successfully transmitted through a TIES modem.  No matter how many

times you try to transmit the file containing those escape characters in the

data stream, there is a 100% probability of a false escape.

When the TIES block occurs, the typical modem user operating that modem will

not be able to determine why the transmission of the data has stopped.  Each

and every time that the user attempts to send the file containing the escape

sequence characters, the transmission of data will halt, and the user will be

unable to determine the problem.  In some instances, depending on the data

following the inadvertent escape, the modem may actually have to be turned off

and back on again before the modem will be able to operate.

Further, a modem incorporating TIES cannot be repaired because it is not

broken.  There is an inherent limitation engineered into the product, i.e. the

absolute inability to transmit the sequence of data that TIES defines as its

escape sequence.  The only practical solution to this problem of unreliability

is to replace the TIES modem with one that does not incorporate the TIES escape

sequence, unless of course you are willing to stand by the modem and reprogram

the escape character and retransmit the data every time the TIES block occurs.

Since many modems are used in unattended operation, this is completely

unacceptable.  And why would anyone who knows about this want to take the risk?

TIES - Why You Haven't Heard About it?

In December of 1991, Hayes Customer Service received a telephone call from a

user of a modem manufactured by another modem company.  This individual was

using Hayes Smartcom software and was having a transmission-related problem.

While attempting to assist this individual in solving this problem, Hayes

learned that the modem was using the TIES escape sequence.

This was the first time that Hayes had actual knowledge that the TIES escape

sequence was being introduced into the marketplace.  Manufacturers who are

incorporating this technology have not been publicizing their use of TIES.

There have been few articles written about this new escape sequence, and the

majority of those articles are the result of Hayes expressions of concern over

the reliability of TIES.  Hayes was surprised to learn of the commercial

implementation of this escape sequence because of those reliability questions.

In fact, TIES appears to be the reincarnation of the problem that was solved by

the invention of the Hayes '302 patented technology in 1981.

Why are modem manufacturers not publicizing their use of this new technology?

If there are no inherent reliability problems, why does it seem that modem

manufacturers using this technology are hiding it and not touting its benefits

from the rooftops?  Why is it that modem manufacturers who use TIES want to

keep consumers in the dark about their adoption of this new escape sequence?

The answers to all these questions are rooted in financial concerns.  Hayes, on

the other hand, knows that consumers want to be completely and accurately

informed about the reliability of the communications products they purchase.

Modem manufacturers who use TIES should know that informed consumers will not

knowingly want to buy products with TIES.

The Philosophy of Reliable Modem Design

Data alone should never cause a modem to escape.  Users rely on the

manufacturers of computer equipment and modems to build the equipment in such a

way that users can count on its operation to conduct business and increase

personal productivity.  When the data being sent by a computer user can cause

an interruption of use, the user does not get the performance expected.  For a

manufacturer to intentionally include a mechanism in a modem where data

transmission alone can cause the escape is a great disservice to the computer

industry, the public using the products, and eventually the manufacturer.  The

manufacturer should be concerned that dissatisfied customers who learn about

this defect will at least have some serious questions about buying such a

product.

Once customers understand the issue they will no more want a modem that

escapes when data is being sent than they would want a computer which

miscalculates when processing a particular program sequence.  The defect is

simply unacceptable if there is a readily available, well-understood way to

avoid the problem.  Manufacturers who care about their responsibility to their

customers will avoid products which contain deliberate defects since their

products may create legal liability for that manufacturer.

Development of the TIES Test

Anticipating that there would be other telephone calls to Hayes Technical

Support about the first known TIES modem and realizing that there may be other

modems which incorporate this new escape sequence, Dr. John Copeland of Hayes

was asked to formulate a test for use by Hayes Technical Support to determine

if a modem being used by a caller was a modem incorporating the TIES escape

mechanism.

The test was easy to develop because TIES will cause a file to abort whenever

a certain sequence of characters is present, and this sequence of characters is

contained in the data stream.  In fact, the characters themselves are nothing

more than data.  The test data file developed by Dr. Copeland includes all

possible sequences of the form "xxxATy" (the TIES escape sequence).  The

character "x" can be any one of 128 different data characters.  The character

"y" similarly can be any one of 128 different data characters.  This results in

16,384 possible sequences which are each repeated twice in the test file.

The TIES test data file was developed only to determine whether a modem would

escape on a TIES data sequence.  Contrary to the statements of some members of

the press, the file is not a software package.  The test file is simply an

ASCII data file to assist users in determining if their modem implements TIES.

Some people have charged that the test data file is rigged.  This is not a

rigged test.  It is necessary that the test contain all possible combinations

of characters in order to be effective because some software packages you use

may set the escape code to be of different values.

Not "Hayes Compatible"

Since the introduction and overwhelming success of the first Hayes Smartmodem,

other modem manufacturers have touted their products as "Hayes compatible" and

have assured purchasers that the modems they buy are "Hayes compatible."

Consumers have come to rely on "Hayes compatibility" as a standard

consideration in their purchases of communications equipment.  Consumers who

buy a TIES modem might assume that the modem is "Hayes compatible" because it

uses AT commands, only to learn later that the modem might have been designed

with a serious defect.  TIES modems are not "Hayes compatible."

Why Is This Issue So Important?

Hayes believes that the introduction of TIES into the installed base of modems

will seriously compromise the integrity of the industry.  The degree of damage

to the industry and to the confidence in the installed base of modems depends

on the frequency that this problem will occur if the use of modems with TIES

becomes widespread.

There is no way of accurately predicting the probability of inadvertent

escapes occurring in a TIES modem.  One manufacturer of a chip set

incorporating the TIES technology has admitted that when using a 9600 bps modem

in continuous operation, there is a probability of an inadvertent escape every

seven years while transmitting random data.  The problem with these

calculations is that data in the real world is never random.  We know from

research in data compression that data is not random.  The fact the data is

compressible demonstrates that it is not random.  As a result the statistical

probability is even greater that the ASCII characters used in the TIES escape

sequence would occur in data.  Further, once a byte sequence has special

meaning, it appears much more frequently than would a truly random sequence of

the same length.  For example, Dr. Copeland scanned the hard disk in his office

and found three files that would escape a TIES modem: a communications program

EXE file, an Internet message discussing modems, and a log file from a

telephone-line security program.  Based on these results, the mean time to

blockage at 9600 bps for a TIES modem would only be 14 hours for Dr. Copeland's

data.

The probability of an inadvertent escape may either increase or decrease

depending on the data being transmitted.  Probability figures are easily

manipulated to reflect the outcome desired by the entity doing the

calculations.  For this reason, Hayes has not calculated the probability of

inadvertent escape by a TIES modem, and we believe the calculations made by the

modem chip manufacturers selling TIES in their chip sets represent the absolute

minimum under the best conditions.

For those who require an answer to the question of "how often," they can turn

to the figures provided by the chip manufacturer itself, in the attempt to

minimize the importance of this problem.  Placing that calculation in terms of

the "real world," in a company using 700 TIES modems communicating at 9600 bps,

there would be an inadvertent escape twice a week.  Twice a week a data file

important to the business would not be sent because it could not get through

the TIES modem.  Hayes maintains that any inadvertent escape is unacceptable.

Two inadvertent escapes each week is outrageous!  And if the data is being sent

at 38.4 kbps, as many new modems are now capable of doing, this TIES block will

occur more than once per day, only you cannot predict which of the systems will

be hit.

The following chart represents the minimum frequency of TIES blocks that you

should expect with a TIES modem depending on the number of modems and the speed

at which they are operating:

If Data Was Random

Type of Use     Hours/Day   Modem Speed     Number of Modems   TIES Aborts/Year

Single User         1          2400                 1                    0.001

Small BBS          12          9600                 2                    0.12

Large BBS          12         14400                12                    1.1

Small Company       2         14400                50                    0.8

Medium Company      2         14400               500                    7.6

Large Company       2         14400              5000                   76

Dialup WAN         24         14400               200                   36

Dialup WAN         24         38400               200                   97

Data with 1 TIES per 50 Mbytes

Type of Use     Hours/Day   Modem Speed     Number of Modems   TIES Aborts/Year

Single User         1          2400                 1                    6

Small BBS          12          9600                 2                  605

Large BBS          12         14400                12                5,449

Small Company       2         14400                50                3,784

Medium Company      2         14400               500               37,843

Large Company       2         14400              5000              378,432

Dialup WAN         24         14400               200              181,647

Dialup WAN         24         38400               200              484,393

TIES Impact on Business - From Personal Frustration to Corporate Chaos

Fortunately, TIES blocks have yet to show up in great volume.  However, as

more and more TIES modems are sold, it is predictable that TIES blocks will

start to occurr with ever-increasing frequency in businesses where numbers of

modems exist.

Personal Frustration.  Take for example, your remote sales staff.  What is the

traveling salesperson going to do when he or she tries to log an order into the

system at 9:00 at night and suffers a TIES block?  Is that person going to

attempt to adjust the RS232 serial cable pin outs or reconfigure the modem

control from a hotel room?  No, that is unreasonable to expect.  Or say, for

instance, sales support is attempting to send the monthly sales forecast

spreadsheet to the regional offices.  Only this time the transmission keeps

crashing because one of the fields happens to include a number that when

translated for transmission happens to be the TIES escape sequence.   No matter

how many times the transmission is attempted, it will never go through.

Paralysis.  TIES could also have a dramatic impact on entire computer systems

that had incorporated software which is incompatible with the TIES escape

sequence.  Imagine the scenario where over the weekend your company upgrades

its software using a program that includes some data that is incompatible with

TIES.  On Monday morning, when employees try to send material to remote offices

of the corporation, the transmissions all fail. Your entire company will suffer

a TIES block.  Think of the time and money required to correct such a problem,

not to mention the lost productivity while the problem is being found and fixed.

Sabotage.  The introduction of TIES into the installed base of modems has

far-reaching consequences beyond TIES blocks.  TIES is a threat to the security

of your business.  Its implementation  makes it easier for an individual to

sabotage the work of a company.  If a company is using TIES modems to transmit

data in its day-to-day operations, it would be very easy for a disgruntled

employee to insert the TIES escape mechanism into the companyUs data.  The

employee could even change the software used by the company to insert this

sequence of characters in every file being created at the company.  Even more

problematic is the ability of an employee to sabotage the employerUs business

by the insertion in spreadsheets of a few numbers which will cause the TIES

modem to fail.  The TIES escape sequence can appear in a transmission of either

binary numbers or ASCII characters.  There are many number combinations in data

that will cause an inadvertent escape in a TIES modem.

Chaos.  TIES creates a window of opportunity for a new type of computer virus.

In the last year, we have seen the business world shaken by the existence of a

number of computer viruses that attack data directly in the computer.  TIES

offers illegal hackers the ability to attack the integrity of data by inserting

a character string in unused parts of the data file to prevent its future

transmission.  Because TIES relies entirely on the existence of its escape

characters in the data stream, it would be possible for a hacker to introduce a

virus that would carry those escape characters into files without changing the

program data itself.  The file would be incapable of transmission because the

TIES modem would inadvertently escape when it reached those characters every

time it was transmitted.

With this perspective in mind it is clear that TIES is a step backward that

also could potentially create chaos in computer communications if it is

deployed to any great extent.  On many levels, TIES could actually affect

productivity throughout the business community.  There is an installed base of

modems estimated at over 25 million.  Assuming that these modems were TIES

modems and using the probability calculations provided by the TIES chip

manufacturer, there would be almost 3 million inadvertent escapes a year.  Even

assuming typical modem usage and a mixture of modem speeds,  there would be

more TIES blocks in one year than the number of people killed on U.S. highways.

As modems with this inferior technology are introduced into the marketplace,

their effect on productivity has the potential to be far-reaching.  Keep in

mind these calculations of probability are based upon what we believe to be the

theoretical absolute minimum. Hayes fully believes that calculations using

random data grossly underestimate the problem presented by the introduction of

TIES modems into the installed base of computing equipment used around the

world.

Arguments for Acceptable Levels of Failure

Because manufacturers have thus far refused to tell the public that they are

using this new technology, it is very difficult to identify who is using TIES.

However, the manufacturers of the chip sets necessary to implement TIES and the

modem manufacturers who admit that they use this new escape sequence concede

that "it is not as reliable as the technology offered by the Hayes '302."

There are repeated instances of these companies making statements to this

effect.  The question remains; what level of reliability is acceptable?  Hayes

believes that the industry should not accept any reduced level of performance

if there is readily-available technology that offers proven reliability.

Why is Hayes Interested In Educating the Public About Escape Sequence

Reliabity?

Hayes developed an education campaign targeted to corporate and business

America because we know this is where the greatest damage is likely to occur if

TIES control modems become widely installed.  Without action the problem

eliminated by Dale Heatherington in 1981 will be in the future of the modem

industry.  Data alone should not cause a transmission to abort!  Modem

manufacturers who have made the decision to begin implementing TIES have not

informed the consuming public about TIES, their decision to use this escape

mechanism, nor the inherent problems associated with the use of data alone as

an escape signal.  We believe that many of the manufacturers who are using TIES

do not even understand the true nature of the problem and some just donUt care

and want the public to believe it doesnUt matter.

Since business, government, and industries increasingly rely on information

technology infrastructure to conduct their business with applications like

electronic mail, electronic data interchange for orders and invoicing,

telecommuting to reduce traffic and fossil fuel consumption, and the rapid

growth of information services for education and entertainment, it is becoming

a regular part of our everyday lives.  When major events occur that affect the

infrastructure like the telephone system crashing in the Northeast U.S., the

major scare over computer viruses, or similar threats to the use, availability

or reliability of this strategic and tactically critical resource, the public

confidence severely diminishes.

Since its founding in 1978, Hayes has sought to educate consumers about many

topics related to modems and their usage.  TIES is just one more of these

issues.  Hayes has learned through the years that consumers want to know about

these issues and appreciate any information provided that will enable them to

make informed choices in their purchase of communications products.  If someone

wants to buy a modem with TIES, they should be in a position to be an informed

consumer and should be prepared to deal with the consequences when the TIES

block happens to them.