STAC ELECTRONICS

TECHNICAL SUPPORT BULLETIN

NUMBER:     015

SUBJECT:    DOS5.0

DATE:       June 28, 1991

TIME:       7:24 AM

SOURCE:         Tech Support, MS DOS 5.0 Documentation

SUPERCEDES/

SUPERCEDED BY:

RELEASE STATUS: Public

TITLE:

Loading STACKER into upper memory ("loading high") with MS-DOS 5.0

OVERVIEW:

MS-DOS 5.0 comes equipped with a memory manager which may be used to load

device drivers into the upper memory area between 640K and 1MB.  STACKER

is one such driver.

ISSUES/COMMENTS:

1) DEVICE=C:\DOS\HIMEM.SYS provides access to extended memory and ensures

that no two programs use the same portion of it at the same time.

2) DOS=UMB tells DOS to maintain a link to the upper memory area;

DOS=HIGH,UMB also loads the kernel of DOS into the high memory area

(HMA).

3) DEVICE = C:\DOS\EMM386.EXE NOEMS causes extended memory to simulate

expanded memory so that programs and device drivers may be loaded into

the upper memory area.  Specifying RAM instead of NOEMS will establish a

64K page frame for expanded memory, and the reduced number of upper

memory blocks may be insufficient to allow the driver to be loaded high;

in this case, a commercial memory manager (QEMM386, 386MAX) may be

needed.  If the STACKER coprocessor card is installed, it will also be

necessary to exclude its address range (e.g., x=CC00-CFFF).

4) DEVICEHIGH=C:\STACKER\STACKER.COM... does the actual loading of the

STACKER device driver into the upper memory area.  If there is

insufficient memory available, DOS will load the driver into conventional

memory with no warning message.

5) The DOS command MEM/C lists the order and size in which drivers and

programs have been loaded.

RECOMMENDED ACTIONS:

1) Ensure that the statements are executed in CONFIG.SYS in proper order.

(Note that they need not be consecutive, and the paths may differ from

system to system):

DEVICE=C:\DOS\HIMEM.SYS

DOS=HIGH,UMB

DEVICE=C:\DOS\EMM386.EXE (NOEMS or RAM) (X=exclude range

if applicable)

DEVICEHIGH=C:\STACKER\STACKER.COM...

2) If boot drive is SSWAPped, ensure that a copy of HIMEM.SYS and

EMM386.EXE reside in the \DOS directory on the host volume; for instance,

if C: and D: are SSWAPped, then a copy of the files must reside in

D:\DOS.