............................................................................

SPEEDRAM.EXE                  David Foley                      SHAREWARE

Version 1.10             Foley Hi-Tech Systems

............................................................................

Purpose:     Increases the speed of your system CPU by changing the refresh

rate of your systems 8253 timer chip.

Syntax:      [d] [path] SPEEDRAM [parameter]

where valid parameters are:

+  Enable SpeedRam.  Entering a + will enable SpeedRam and set

the optimum refresh rate.

-  Disable SpeedRam.  Entering a - will disable SpeedRam and set

the refresh rate to stock values.

r:nn  Set refresh rate to nn microsec's.  Entering r:nn will

set the refresh rate to nn microseconds.  Only use this if

have a good understanding of the refresh rates.

Setting too high a rate will cause memory parity errors or hang

your system.  If your find that SpeedRam + causes parity errors or

hangs your system than you will want this option.  If you do not

specify a parameter then SpeedRam will list the available parameters

for you.

Remarks:    All Personal Computers, including the original IBM PC were

designed with a very high refresh rate.  The rate used by all

personal computers is far beyond the rate required by the RAM

chips used in these machines.  SpeedRam sets the refresh rate to

match closer with RAM manufacturers specifacations andin turn

frees up CPU cycles.  These free cycles can be used by your

software rather than going to wase by refreshing the RAM too often.

The change in refresh rate matches within specifications of most

DRAM manufacturers.  Depending on your system SpeedRam should

increase your CPU's performance from 2% to 10%.  You can measure

this in many ways such as LANDMARK'S CPU SPEED Test or

Norton Utilities SI program.

Notes:      SpeedRam is not a TSR. It does not require any RAM or take any

away from your system.  SpeedRam ony needs to be loaded once and

it will perform until the machine is rebooted.  Be sure to save

any critical data in memory before experimenting with SpeedRam,

as you may experience crashes before you get it set up correctly

for your system.

Trouble Shooting

SpeedRam has been thoroughly tested on many systems before its

release but due to the complexity and variety of equipment

available there can be no guarantee that it will work on every

PC.

If SpeedRam causes any problem, it will cause memory problems.

If you notice memory errors, parity errors, or that your machine

is hanging after running SpeedRam + then the rate chosen for your

system is too high causing the DRA< chips to fail.  This is in

no way a permanent failure, you will just have to reboot and pick

a lower rate at which to run SpeedRam.  To use a manual rate type:

SPEEDRAM r:nn

where nn is the refresh rate you wish to use.  Start with 60 and

retest your system.  If you continue to have problems keep

decrementing nn by 5 until you  reach a value that your system works

with.  If the value is less than 18 then you will no benefit from

SpeedRam.

SPEEDRAM.EXE is Copyright 1987-1991 by Foley Hi-Tech Systems.

This program is not public domain but is "Shareware" and a part of the

Foley Hi-Tech Systems ExtraDOS collection.  To register send a check or

money order for $19.00 to:

Foley Hi-Tech Systems

ExtraDOS Registration

172 Amber Drive

San Francisco, CA 94131

(415) 826-6084