Tseng ET4000 - Windows 3.0 and 3.0a Display Drivers

The instructions below apply to Windows 3.0 as supplied on 5 1/4" HD diskette

media.

INSTRUCTIONS - FIRST TIME WINDOWS 3.0 and 3.0a INSTALLATION

Follow the instructions contained in the WIN30DRV.DOC file to install the

Tseng ET4000 display drivers.

INSTRUCTIONS - WINDOWS 3.0 or 3.0a ALREADY INSTALLED

To use the Tseng ET4000 Windows drivers you need to copy the files on the

supplied ET4000 driver diskette to the \WINDOWS\SYSTEM subdirectory.

After you copy the files, change the display configuration from within Windows

itself to one of the extended drivers.

See the Windows documentation for further information.

IMPORTANT

If you have Windows 3.0, you also need to copy the SETUP.W30 file as SETUP.INF

before you run the SETUP program.

If you have Windows 3.0a, you also need to copy the SETUP.W3A file as SETUP.INF

before you run the SETUP program.

When you run the Windows Setup to change to a different driver Windows may

prompt you for some of the Windows Installation diskettes. The drivers use

display fonts which are contained on the standard Windows distribution

diskettes.

The enclosed drivers are generic and should work with most ET4000 based boards.

The drivers have various display memory requirements and of course your display

monitor must be able to support the resolution you select.

The drivers support the following:

Resolution	Mode 	Minimum display     Driver name         Windows Font

(HEX)   memory required

640x480x2	11h          256kb	    VGAMONO.DRV  **     VGA

640x480x16	12h          256kb          VGA.DRV      **     VGA

800x600x16	29h	     256kb          VGA464.DRV          8514

800x600x16	29h	     256kb          VGA464S.DRV         VGA

1024x768x16	37h	     512kb          VGA474.DRV          8514

1280x1024x16	3Dh	     1 meg          VGA414.DRV          8514

640x480x256	2Eh	     512kb          VGA448.DRV          VGA

640x480x32k	2Eh xx	     1 meg          VGA443.DRV          VGA

640x480x65k	2Eh xx	     1 meg          VGA446.DRV          VGA

800x600x256	30h	     512kb          VGA468.DRV          8514

800x600x256	30h	     512kb          VGA468S.DRV         VGA

800x600x32k	30h xx	     1 meg          VGA463.DRV          8514

800x600x32k	30h xx	     1 meg          VGA463S.DRV         VGA

800x600x65k	30h xx	     1 meg          VGA466.DRV          8514

800x600x65k	30h xx	     1 meg          VGA466S.DRV         VGA

1024x768x256	38h	     1 meg          VGA478.DRV          8514

** uses Windows 3.0 or 3.0a supplied VGA driver

xx requires a Sierra Semiconductor or compatible HICOLOR DAC

The 65k color drivers require a Sierra MODE2 capable DAC, if your display

board can NOT support this mode Windows will not run with these

drivers and return you to the DOS prompt after the the driver displays a

message "video driver unable to set desired mode". You can try the 65k

drivers and if your board is unable to support them, simply select another

driver running Windows Setup from the DOS prompt.

IMPORTANT

It is important that the drivers be installed properly, replacement

grabbers (GR2 extension) and Virtual-Display-Device handler (386 extension) are

included to properly support the extended feature set of the ET4000 VGA chip.