---------------------------------------------------------------------------

_________ __

/___  ___// /

CHRISTIAN BUCHNER                     / /   / /

/ /   / /__   _______

PROUDLY PRESENTS               / /   / ___ \ / /__/_/

/ /   / /  / // /

/_/   /_/  /_//_/____

____                      __    ____                       __     __

/ __ \                    / /   / __ \                     / /__  / /

/ /_/ /                   / /   / /_/ /                    / //_/_/ /_

/ _  _/ ____/\ __  __ __  / /   / _  _/ ____/\ /\___   __  / /__ /  __/

/ / \ \ / __/ // / / // _\/ /   / / \ \ / __/ // ___ \ / _\/ // / / /

/ /__/ // /_/ // /_/ // /_/ /   / /__/ // /_/ // /  / // /_/ // / / /

/______//_____//_____//_____/   /______//_____//_/  /_//_____//_/ /_/

THE CHEAPEST WAY TO USE YOUR SERIAL PORT AT LIGHT SPEED

---------------------------------------------------------------------------

BaudBandit Documentation, Release Version, 12th July 1992, 02:51

---------------------------------------------------------------------------

WHAT IS THE BAUD BANDIT?

When  Commodore  developed the serial.device some years ago, they could not

have  foreseen  the  development  of  relatively  cheap  modems  using data

compression  at incredibly high baud rates like the US Robotics HST and the

new  v32bis  modems.   So  the  serial.device  soon became too slow for the

requirements of modern telecommunication standards.

The BaudBandit is a replacement for the original serial.device allowing you

to  drive  your serial port at baud rates up to 115200 Baud with a standard

68000 Processor in full multitasking.  Without any doubt the BaudBandit can

be  called  "a little revolution" within the telecommunication scene on the

Commodore Amiga.

It  is especially designed for use with high speed modems and supports full

bidirectional RTS/CTS handshake and receive buffering.  This device will no

longer  bring  your  computer  to  a standstill when transferring a file at

14400 baud or more.  Compared to the serial.device, this program provides a

speed increase of about 500% percent.

HOW TO USE THE DEVICE

Simply  copy  the BaudBandit.device to your DEVS:  directory.  Now you have

to  configure  your  software  to  the  new device.  Most telecommunication

programs offer a configurable device name and unit.  Here you have to enter

"BaudBandit.device".  The device name is case sensitive, so be sure to type

it right as shown above!

If  your  software  does not allow to use a different serial device because

the name is hardcoded in your software or whatever, you may use the program

QuickFix to patch the OpenDevice() vectors.  Any attempt to open the serial

device  will  open  the BaudBandit device now.  The patch may be removed at

any  time  with a CTRL-C or the break command from CLI.

Owners  of  the  Commodore  A2232  multiserial  card can easily replace the

oldser.device  with the faster BaudBandit.device by using the patch program

A2232Fix.  It works exactly like QuickFix.

WHAT ABOUT THE COMPATIBILITY?

The  code  is designed to work under OS1.2/1.3 as well as OS2.0.  It should

run on all processors from 68000 to 68040.

In  order  to  get  the  best speed results, I dropped some features of the

original serial.device which are usually not needed in telecommunication.

I  completely  dropped  the  xOn/xOff protocoll, as well as the termination

array  and  parity  checks.   Breaks are currently not supported but may be

included in later versions.  Characters must consist of 8 bits.  The device

only supports plain 8N1 RTS/CTS transfer.  But this is all you need!

The  BaudBandit.device  should be able to replace the serial.device in most

situations.   It  has  been tested with Fido mailers like TrapDoor and many

terminal programs like Term or NComm.  Up- and downloads are no problem and

especially  at  high  speed  transfers you should notice that your computer

will NOT stand still anymore.

HARDWARE REQUIREMENTS

The  device  supports  a very common handshake method for high speed modems

using  the  RTS/CTS  wires.  The RTS/CTS wires prevent a buffer overflow on

the  computer  and  on  the  modem  side.   This  handshake method is often

referred to as "flow control".

THIS  IS  IMPORTANT:  In order for the device to work properly, you have to

connect  these  wires to your computer, even if your modem does not support

this  handshake method.  Without the RTS/CTS wires connected to your serial

port, the device will not be able to send anything!

WHAT IS INCLUDED IN THIS PACKAGE?

This  archive  contains  an  fully  functional  version of the device and a

complete  documentation.   If you decide to use this program regularly, you

have to register.  Please note that the contents of this archive may not be

altered in any way.

WHY IS IT SHAREWARE?

The  development  of  this  programm  has taken many weeks of hard work and

frustration.   Now  I  am  able  to present a reliable and fully functional

program which will be of great use for many people.  Please understand that

I do not want to give this program away for free.  I ask you to pay a small

shareware  donation.   The  more  people  register,  the  more support this

product  will  get.  The shareware fee will help me to support my customers

and to continue my work on this project.

HOW TO REGISTER

Before  you decide to register, please check whether this program satisfies

all your requirements or not.  You are allowed to evaluate this program for

two weeks.  If you continue using it, you have to register.

Fill  in the registration form and send it to me.  I think, it is justified

to  ask  for a shareware donation of DM 20.  There are many ways of sending

the money to me, so don't tell me that you did not know how to do it.

a) Enclose it cash.

b) Send me a cheque over DM 20.

c) Let your bank transfer it to me

d) Transfer it with a postal money order.

Please don't transfer any money before sending the registration form.

The registration form must be mailed to the following address:

Christian Buchner

Ganghoferstrae 2

8034 Germering

Deutschland

-------------------------

AND DON'T FORGET TO PAY

-------------------------

Really!!!

SO THIS IS THE END.

Yes, looks like that.