KAM.10

I have experienced problems using a KAM with the TR78xx series of transmitters.

The problem was due to overdriving of the audio from the KAM LO outout which is

about 10 mv p-p. While this worked OK on the older TR7400 rig it caused many

repeats and time-outs with the 7800,7830 and 7850. The change is as follows..

Replace R11 on the KAM board with a 200-500 miniature multu-turn pot and with

the help of another station with a VU meter set the pot for a level of zero DB

or to the same level as a reference station in your area.The approximate level

for the KAM audio out is about 3 mv but you may find minor differences.

73 .. Fred @ VE3NUU.