TH25AT.1

The following information is for the modification of the Kenwood

TH-25AT 2 meter handheld. Thanks go to Brad Killebrew, N5LJV for

providing this modification. I have not tried this modification

so I can't verify that this will work.

TH-25AT MARS/CAP MODIFICATION

The following modification will allow the TH-25AT to transmit

from 141.000 Mhz to 162.995 Mhz. Specifications are guaranteed

for the Amateur band only. The transceiver may require

realignment of the PLL circuits.

1. Disconnect the battery pack and antenna.

2. Remove the VOLUME, SQUELCH, and TUNING control knobs by

pulling them straight up from the top panel.

3. Using a 7mm spanner wrench, remove the nut from the VOLUME

control and the nut from the TUNING control.

4. Remove one screw located by the PTT switch.

5. Remove one screw located by the speaker jack.

6. Remove two screws from the battery terminal plate.

CONTINUED IN FILE TH25AT.8