TH25AT.6

On the TH-25AT:

(All frequencies given in MHz.)

R22 in

R28 out

This is how the radio is delivered in the USA.  TX 144-1, 141-163

(I think).

R22 out

R28 in

The radio tunes from 142-151.  This may be the modification given

to US MARS members.  I don't remember where the unit will transmit.  It

may or may not transmit outside the range from 144-148.

R22 in

R28 in

The radio tunes only from 144-148.

R22 out

R28 out

Frequencies may be selected from 100-200 MHz (on the display only - your

PLL will not lock up in this entire range).  In addition, TX is possible

where your PLL locks up.

R25 out

Removing R25 disables automatic offset selection.

R23 and R24 are used for selecting the step size for tuning.  I can't

remember which positions are for which step sizes, and alas I didn't

write down what I found.  If you want to play with this, go ahead.

CONTINUED IN FILE TH25AT.11