TH45AT.1

On the TH-25AT:

(All frequencies given in MHz.)

R22 in

R28 out

This is how the radio is delivered in the USA.  TX 144-1, 141-163

(I think).

R22 out

R28 in

The radio tunes from 142-151.  This may be the modification given

to US MARS members.  I don't remember where the unit will transmit.  It

may or may not transmit outside the range from 144-148.

R22 in

R28 in

The radio tunes only from 144-148.

R22 out

R28 out

Frequencies may be selected from 100-200 MHz (on the display only - your

PLL will not lock up in this entire range).  In addition, TX is possible

where your PLL locks up.

R25 out

Removing R25 disables automatic offset selection.

R23 and R24 are used for selecting the step size for tuning.  I can't

remember which positions are for which step sizes, and alas I didn't

write down what I found.  If you want to play with this, go ahead.

On the TH-45AT:

(All frequencies are given in MHz.)

R18 in

R28 in

This is how the radio is delivered in the USA.  The radio covers

438-450 MHz.

R18 in

R20 out

The radio is prohibited from tuning outside 440-450 MHz.

R18 out

R28 in

The radio will only tune from 215-230 MHz.  Note that the PLL would not

lock up!  (What did you expect?)  Could it be possible that Kenwood

originally planned a 220 version of this radio, but then scrapped their

plans?

R18 out

R28 out

The radio will tune from 200-500 MHz (on the display only - your PLL

will not lock up over this entire range).  Transmitting is possible

anywhere your PLL will lock up.

CONTINUED IN FILE TH45AT.2