TH75A.2

From: WA6TKV@N6IIU

The Kenwood approeed MARS mod for the TH-75A is very

simple.  Remove the 3 exterior screws.  There are 4 screws

where the battery pack slides on.  Remove the 2 screws toward

the face of the radio.  You now have access behind the front

panel.  You will see a green wire near a square chip.  Cut the

wire.  You will now have transmit from 142-152 and receive from

136-174.  Transceive from 420-450.  Although you can actually

key in 385-512.

This is truly a fantastic radio!  You can take the mod two

steps further.  You wyll definitely need a service manual and a

very delicate touch.  (Not for the weak!)  Locate and remove D4.

It is in the same vicinity of the green wire.  The radio will

now transceive from 136-174, some VCO adjustment may be required

for the entire range.  It is also now capable of transceiving

from 385-512.  The UHF VCO's have approximately a 50 MHz band-

width so you will only get a 50 MHz wide portion of 385-512.

The second step is adjusting the VCO's.  There are separate

adjustments for transmit and receive for both VHF and UHF.

This calls for complete dis-assembly of the radio.  Refer to

your service manual.

I have found these mods extremely valuable.  Now I have

a dual band ham rig and a police scanner in one.  Have fun and

please use EXTREME caution in transmitting only on your amateur

and MARS frequencies.